ATNP/WG2

WP - 414

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL(ATNP)

Working Group 2

27. - 30. October 1997

Redondo Beach, USA

Experimental ATN Topology and ATN Addresses allocated in Germany

- Information Paper -

Presented by A. Herber

Prepared by Thomas Schade

Summary

This information paper provides an overview of the Experimental ATN topology established by DFS in its ”Demonstrator and ATN Research Testbed“ (DART) with the focal point on NSAP addressing. Purpose is to facilitate interconnection with potential partners for future experiments.

# INTRODUCTION

On the basis of a cost-benefit- and an user requirement-analysis, DFS will decide on a transition to ATN in Germany.

To obtain the necessary information for such analysis, the DFS program ”Demonstrator and ATN Research Testbed (DART)” which is supported by the CEC under the name ATIF (ATN Trials Infrastructure), was launched.

Part of this programme is the integration of different ATN compliant subnetworks. For the next months, it is planned to integrate also the following air/ground subnetworks into the DART:

- End of 1997:				AMSS/Satcom

- End of 1997/Beginning of 1998:	Mode S

Joint ATN trials will be performed together with further experimental ATN sites.

To show the benefits of data link applications and services in an international ATN environment, it is recognized that an adequate infrastructure is necessary for this to take place. The DFS therefore fully supports the concept of ATIF for this purpose and has already interconnected with other ATIF sites.

With this paper, a brief overview should be given of the DFS ”Demonstrator and ATN Research Testbed“ with the focal point on NSAP addressing to facilitate interconnection with potential partners for future experiments.

# TOPOLOGY

Focal point of the DART concept is to provide an flexible infrastructure which enables his users

- to integrate various systems and subnetworks with less expenditure,

- to interconnect with other Euro-ATIF partners (via different X.25 subnetwork, e.g. Datex-P, Transpac, PSN),

- to configure different ATN scenarios and environments, for example one or more routing domains (including one or more airborne domains).

- to use the facility „Fast Select“ on all systems

To achieve these objectives, the DART routers were installed on two locations. One is the academy of the DFS in Langen (three ground routers plus one airborne router in the end of 1997), the other one is the former research and development center in Frankfurt (one router).

The ground routers are connected to the DFS owned Packet Switched Network (PSN), which is a private X.25 network.

Furthermore, an direct access to a public X.25 network, the Datex-P of the Deutsche Telekom, as well as a gateway between the PSN and the Datex-P allow a multiple resp. flexible connectivity to other ATN sides and an extensive investigation of the collaboration of the ATN with X.25 subnetworks of different providers.

In addition to the X.25 connections, an Ethernet backbone connects the three ground routers of the ground routing domain in Langen.

All systems currently used are TAR-TTS systems developed by Vertel under an Eurocontrol contract and are based on HP 9000 work stations.

In the scope of Satcom subnetwork integration in the end of 1997, a combined air/ground BIS, based on a PC TAR, will be installed in the airborne laboratory of DART.

An other combined BIS/End System, a combination of TAR-TTS and the Trials End System (TES), which was developed under an Eurocontrol contract as well, is already in use.

- Planned infrastructure of the Demonstrator and ATN Research Testbed (DART)

- Present physical configuration of the DART

It has to be mentioned that the environment of the EEC Bretigny does consist of more than one router as shown in figure 2-1. It is reduced in these figure to the BIS which provides the connection to the DART.

# NSAP ADDRESSES

System name definition and address allocation followed with keep to the naming and addressing scheme, defined in the ATN Guidance Material and to the arrangements of Euro-ATIF sides.

All NSAP addresses are, as defined on the last ATNP WG2 meeting in Langen, registered on the ATNP NSAP Address Repository.

## Routing Domains defined for DART:

- Routing domains and routing areas of the DART (with two connections to the EEC Bretigny)

RD_DFS1 (ground):	4700278144455500465331

AFI=47

IDI=0027

VER=81

ADM=444555 (DEU)

RFD=00

ARS=465331 (FS1)

RD_DFS2 (air):	47002781444555003C9A61

AFI=47

IDI=0027

VER=81

ADM=444555 (DEU)

RFD=00

ARS=3C9A61 (German 24Bit ICAO Aircraft address)

## NSAP addresses of involved systems:

Ground-Ground Systems:

DFS_GR1:	{+RD_DFS1}000144465347523101

DFS_GR2:	{+RD_DFS1}000144465347523201

DFS_GR3:	{+RD_DFS1}000244465347523301

Air-Ground-End Systems:

DFS_EAG1:	{+RD_DFS1}000144465341473101

Airborne Systems:

DFS_EA1:	{+RD_DFS2}000144465345413101

# CONCLUSION

To attain the objective to be well prepared for a decision in the end of 1998 with regard to a transition to ATN in Germany, DFS will integrate, as mentioned before, a Mode S subnetwork in the testbed very soon. Beyond this, work has already started to connect the ATN environment with the ATC simulator in Langen. This will optimize further trials and investigations with respect to realistic surroundings. This complex research system will obtain an, as it where, real ATC environment.

In this connection, DFS is interested in a co-operation with other partners (aircraft or ground) within the ATN validation resp. evaluation, because an global, international ATN environment will substantiate the basis on which the decision will made and synergy effects resulting from an international co-operation, will optimise the way to this goal.

ATN NSAP Registration Form

| Administrative Authority |  |  |  | DFS Deutsche Flugsicherung GmbH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Country |  |  |  |  | City |  |  |  |  |  |  | Facility |  |  |  |  |
| Geographical |  |  |  | Germany |  |  |  |  | Langen |  |  |  |  |  |  | DFS Academy |  |  |  |  |
|  |  |  |  | Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location |  |  |  | Aircraft type Aircraft Type |  |  |  |  |  |  |  |  |  | Registration/Tail Number |  |  |  |  |  |  |
| Address Status |  |  |  | Operational |  |  |  |  | Trials |  |  |  |  | x | Reserved |  |  |  |  |  |
| Description/ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFS ground router #1 of DART Infrastructure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address |  |  |  | NSAP |  |  |  | NET |  |  |  |  | RDI |  |  |  |  | RDCI |  |  |
| Type |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address Length (octets) |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| System |  |  |  | Class 1 IS |  |  | Class 2 IS |  |  |  |  | Class 3 IS |  |  |  |  |  | Class 4 BIS |  | x |
| Type |  |  |  | Class 5 BIS |  |  | Class 6 BIS |  |  |  |  | Class 7 BIS |  |  |  |  |  | ES |  |  |
|  | Fields | Nb Octets |  | Authorised Values |  |  |  |  |  | Hexadecimal Value |  |  |  |  |  |  |  |  |  |  |
|  | AFI | 1 |  | 47 |  |  |  |  |  | 47 |  |  |  |  |  |  |  |  |  |  |
|  | ID | 2 |  | 0027 |  |  |  |  |  | 0027 |  |  |  |  |  |  |  |  |  |  |
|  | VER | 1 |  | 01-41-81-C1 |  |  |  |  |  | 81 |  |  |  |  |  |  |  |  |  |  |
| Fields | ADM | 3 |  | n/a |  |  |  |  |  | 44 45 55 |  |  |  |  |  |  |  |  |  |  |
| Values | RDF | 1 |  | 00 |  |  |  |  |  | 00 |  |  |  |  |  |  |  |  |  |  |
|  | ARS | 3 |  | n/a |  |  |  |  |  | 46 53 31 |  |  |  |  |  |  |  |  |  |  |
|  | LOC | 2 |  | n/a |  |  |  |  |  | 00 01 |  |  |  |  |  |  |  |  |  |  |
|  | SYS | 6 |  | n/a |  |  |  |  |  | 44 46 53 47 52 31 |  |  |  |  |  |  |  |  |  |  |
|  | SEL | 1 |  | n/a |  |  |  |  |  | 01 |  |  |  |  |  |  |  |  |  |  |

| Associated Contact Person |  |  |  |
| --- | --- | --- | --- |
| Name | Firstname | Address |  |
| Schade | Thomas | Paul-Ehrlich-Straße 37-39 |  |
| City | Postcode | Region / State | Country |
| Langen | 63225 | Hessen | Germany |
| Phone Number | Fax Number | E-Mail |  |
| (+49) 6103/707-783 | (+49) 6103/707-742 | schade@se.dfs.de |  |

ATN NSAP Registration Form

| Administrative Authority |  |  |  | DFS Deutsche Flugsicherung GmbH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Country |  |  |  |  | City |  |  |  |  |  |  | Facility |  |  |  |  |
| Geographical |  |  |  | Germany |  |  |  |  | Langen |  |  |  |  |  |  | DFS Academy |  |  |  |  |
|  |  |  |  | Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location |  |  |  | Aircraft type Aircraft Type |  |  |  |  |  |  |  |  |  | Registration/Tail Number |  |  |  |  |  |  |
| Address Status |  |  |  | Operational |  |  |  |  | Trials |  |  |  |  | x | Reserved |  |  |  |  |  |
| Description/ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFS ground router #2 of DART Infrastructure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address |  |  |  | NSAP |  |  |  | NET |  |  |  |  | RDI |  |  |  |  | RDCI |  |  |
| Type |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address Length (octets) |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| System |  |  |  | Class 1 IS |  |  | Class 2 IS |  |  |  |  | Class 3 IS |  |  |  |  | x | Class 4 BIS |  |  |
| Type |  |  |  | Class 5 BIS |  |  | Class 6 BIS |  |  |  |  | Class 7 BIS |  |  |  |  |  | ES |  |  |
|  | Fields | Nb Octets |  | Authorised Values |  |  |  |  |  | Hexadecimal Value |  |  |  |  |  |  |  |  |  |  |
|  | AFI | 1 |  | 47 |  |  |  |  |  | 47 |  |  |  |  |  |  |  |  |  |  |
|  | ID | 2 |  | 0027 |  |  |  |  |  | 0027 |  |  |  |  |  |  |  |  |  |  |
|  | VER | 1 |  | 01-41-81-C1 |  |  |  |  |  | 81 |  |  |  |  |  |  |  |  |  |  |
| Fields | ADM | 3 |  | n/a |  |  |  |  |  | 44 45 55 |  |  |  |  |  |  |  |  |  |  |
| Values | RDF | 1 |  | 00 |  |  |  |  |  | 00 |  |  |  |  |  |  |  |  |  |  |
|  | ARS | 3 |  | n/a |  |  |  |  |  | 46 53 31 |  |  |  |  |  |  |  |  |  |  |
|  | LOC | 2 |  | n/a |  |  |  |  |  | 00 01 |  |  |  |  |  |  |  |  |  |  |
|  | SYS | 6 |  | n/a |  |  |  |  |  | 44 46 53 47 52 32 |  |  |  |  |  |  |  |  |  |  |
|  | SEL | 1 |  | n/a |  |  |  |  |  | 01 |  |  |  |  |  |  |  |  |  |  |

| Associated Contact Person |  |  |  |
| --- | --- | --- | --- |
| Name | Firstname | Address |  |
| Schade | Thomas | Paul-Ehrlich-Straße 37-39 |  |
| City | Postcode | Region / State | Country |
| Langen | 63225 | Hessen | Germany |
| Phone Number | Fax Number | E-Mail |  |
| (+49) 6103/707-783 | (+49) 6103/707-742 | schade@se.dfs.de |  |

ATN NSAP Registration Form

| Administrative Authority |  |  |  | DFS Deutsche Flugsicherung GmbH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Country |  |  |  |  | City |  |  |  |  |  |  | Facility |  |  |  |  |
| Geographical |  |  |  | Germany |  |  |  |  | Frankfurt |  |  |  |  |  |  | TEZ |  |  |  |  |
|  |  |  |  | Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location |  |  |  | Aircraft type Aircraft Type |  |  |  |  |  |  |  |  |  | Registration/Tail Number |  |  |  |  |  |  |
| Address Status |  |  |  | Operational |  |  |  |  | Trials |  |  |  |  | x | Reserved |  |  |  |  |  |
| Description/ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFS ground router #3 of DART Infrastructure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address |  |  |  | NSAP |  |  |  | NET |  |  |  |  | RDI |  |  |  |  | RDCI |  |  |
| Type |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address Length (octets) |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| System |  |  |  | Class 1 IS |  |  | Class 2 IS |  |  |  |  | Class 3 IS |  |  |  |  | x | Class 4 BIS |  |  |
| Type |  |  |  | Class 5 BIS |  |  | Class 6 BIS |  |  |  |  | Class 7 BIS |  |  |  |  |  | ES |  |  |
|  | Fields | Nb Octets |  | Authorised Values |  |  |  |  |  | Hexadecimal Value |  |  |  |  |  |  |  |  |  |  |
|  | AFI | 1 |  | 47 |  |  |  |  |  | 47 |  |  |  |  |  |  |  |  |  |  |
|  | ID | 2 |  | 0027 |  |  |  |  |  | 0027 |  |  |  |  |  |  |  |  |  |  |
|  | VER | 1 |  | 01-41-81-C1 |  |  |  |  |  | 81 |  |  |  |  |  |  |  |  |  |  |
| Fields | ADM | 3 |  | n/a |  |  |  |  |  | 44 45 55 |  |  |  |  |  |  |  |  |  |  |
| Values | RDF | 1 |  | 00 |  |  |  |  |  | 00 |  |  |  |  |  |  |  |  |  |  |
|  | ARS | 3 |  | n/a |  |  |  |  |  | 46 53 31 |  |  |  |  |  |  |  |  |  |  |
|  | LOC | 2 |  | n/a |  |  |  |  |  | 00 02 |  |  |  |  |  |  |  |  |  |  |
|  | SYS | 6 |  | n/a |  |  |  |  |  | 44 46 53 47 52 33 |  |  |  |  |  |  |  |  |  |  |
|  | SEL | 1 |  | n/a |  |  |  |  |  | 01 |  |  |  |  |  |  |  |  |  |  |

| Associated Contact Person |  |  |  |
| --- | --- | --- | --- |
| Name | Firstname | Address |  |
| Schade | Thomas | Paul-Ehrlich-Straße 37-39 |  |
| City | Postcode | Region / State | Country |
| Langen | 63225 | Hessen | Germany |
| Phone Number | Fax Number | E-Mail |  |
| (+49) 6103/707-783 | (+49) 6103/707-742 | schade@se.dfs.de |  |

ATN NSAP Registration Form

| Administrative Authority |  |  |  | DFS Deutsche Flugsicherung GmbH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Country |  |  |  |  | City |  |  |  |  |  |  | Facility |  |  |  |  |
| Geographical |  |  |  | Germany |  |  |  |  | Langen |  |  |  |  |  |  | DFS Academy |  |  |  |  |
|  |  |  |  | Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location |  |  |  | Aircraft type Aircraft Type |  |  |  |  |  |  |  |  |  | Registration/Tail Number |  |  |  |  |  |  |
| Address Status |  |  |  | Operational |  |  |  |  | Trials |  |  |  |  | x | Reserved |  |  |  |  |  |
| Description/ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFS combined air-ground BIS/ground ES# 1 of DART Infrastructure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address |  |  |  | NSAP |  |  |  | NET |  |  |  |  | RDI |  |  |  |  | RDCI |  |  |
| Type |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address Length (octets) |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| System |  |  |  | Class 1 IS |  |  | Class 2 IS |  |  |  |  | Class 3 IS |  |  |  |  |  | Class 4 BIS |  |  |
| Type |  |  |  | Class 5 BIS | x |  | Class 6 BIS |  |  |  |  | Class 7 BIS |  |  |  |  |  | ES |  | x |
|  | Fields | Nb Octets |  | Authorised Values |  |  |  |  |  | Hexadecimal Value |  |  |  |  |  |  |  |  |  |  |
|  | AFI | 1 |  | 47 |  |  |  |  |  | 47 |  |  |  |  |  |  |  |  |  |  |
|  | ID | 2 |  | 0027 |  |  |  |  |  | 0027 |  |  |  |  |  |  |  |  |  |  |
|  | VER | 1 |  | 01-41-81-C1 |  |  |  |  |  | 81 |  |  |  |  |  |  |  |  |  |  |
| Fields | ADM | 3 |  | n/a |  |  |  |  |  | 44 45 55 |  |  |  |  |  |  |  |  |  |  |
| Values | RDF | 1 |  | 00 |  |  |  |  |  | 00 |  |  |  |  |  |  |  |  |  |  |
|  | ARS | 3 |  | n/a |  |  |  |  |  | 46 53 31 |  |  |  |  |  |  |  |  |  |  |
|  | LOC | 2 |  | n/a |  |  |  |  |  | 00 01 |  |  |  |  |  |  |  |  |  |  |
|  | SYS | 6 |  | n/a |  |  |  |  |  | 44 46 53 41 47 31 |  |  |  |  |  |  |  |  |  |  |
|  | SEL | 1 |  | n/a |  |  |  |  |  | 01 |  |  |  |  |  |  |  |  |  |  |

| Associated Contact Person |  |  |  |
| --- | --- | --- | --- |
| Name | Firstname | Address |  |
| Schade | Thomas | Paul-Ehrlich-Straße 37-39 |  |
| City | Postcode | Region / State | Country |
| Langen | 63225 | Hessen | Germany |
| Phone Number | Fax Number | E-Mail |  |
| (+49) 6103/707-783 | (+49) 6103/707-742 | schade@se.dfs.de |  |

ATN NSAP Registration Form

| Administrative Authority |  |  |  | DFS Deutsche Flugsicherung GmbH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Country |  |  |  |  | City |  |  |  |  |  |  | Facility |  |  |  |  |
| Geographical |  |  |  | Germany |  |  |  |  | Langen |  |  |  |  |  |  | DFS Academy |  |  |  |  |
|  |  |  |  | Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location |  |  |  | Aircraft type Aircraft Type |  |  |  |  |  |  |  |  |  | Registration/Tail Number |  |  |  |  |  |  |
|  |  |  |  | Laboratory |  |  |  |  |  |  |  |  |  | D-BFSA |  |  |  |  |  |  |
| Address Status |  |  |  | Operational |  |  |  |  | Trials |  |  |  |  |  | Reserved |  |  |  | x |  |
| Description/ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DFS combined airborne BIS/ES #1 of DART Infrastructure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address |  |  |  | NSAP |  |  |  | NET |  |  |  |  | RDI |  |  |  |  | RDCI |  |  |
| Type |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address Length (octets) |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| System |  |  |  | Class 1 IS |  |  | Class 2 IS |  |  |  |  | Class 3 IS |  |  |  |  |  | Class 4 BIS |  |  |
| Type |  |  |  | Class 5 BIS |  |  | Class 6 BIS |  |  |  | x | Class 7 BIS |  |  |  |  |  | ES |  | x |
|  | Fields | Nb Octets |  | Authorised Values |  |  |  |  |  | Hexadecimal Value |  |  |  |  |  |  |  |  |  |  |
|  | AFI | 1 |  | 47 |  |  |  |  |  | 47 |  |  |  |  |  |  |  |  |  |  |
|  | ID | 2 |  | 0027 |  |  |  |  |  | 0027 |  |  |  |  |  |  |  |  |  |  |
|  | VER | 1 |  | 01-41-81-C1 |  |  |  |  |  | 81 |  |  |  |  |  |  |  |  |  |  |
| Fields | ADM | 3 |  | n/a |  |  |  |  |  | 44 45 55 |  |  |  |  |  |  |  |  |  |  |
| Values | RDF | 1 |  | 00 |  |  |  |  |  | 00 |  |  |  |  |  |  |  |  |  |  |
|  | ARS | 3 |  | n/a |  |  |  |  |  | 3C 9A 61 |  |  |  |  |  |  |  |  |  |  |
|  | LOC | 2 |  | n/a |  |  |  |  |  | 00 01 |  |  |  |  |  |  |  |  |  |  |
|  | SYS | 6 |  | n/a |  |  |  |  |  | 44 46 53 45 41 31 |  |  |  |  |  |  |  |  |  |  |
|  | SEL | 1 |  | n/a |  |  |  |  |  | 01 |  |  |  |  |  |  |  |  |  |  |

| Associated Contact Person |  |  |  |
| --- | --- | --- | --- |
| Name | Firstname | Address |  |
| Schade | Thomas | Paul-Ehrlich-Straße 37-39 |  |
| City | Postcode | Region / State | Country |
| Langen | 63225 | Hessen | Germany |
| Phone Number | Fax Number | E-Mail |  |
| (+49) 6103/707-783 | (+49) 6103/707-742 | schade@se.dfs.de |  |