ATNP WG2/WG3

WG2 – WP 389

WG3 – WP 10-5

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL (ATNP)

Working Group 2 and Working Group 3

23 - 26 June

Langen, Germay

ATN Managed Objects

Presented by Paul Hennig

Summary

This attached paper from ATN Systems Inc. presents an analysis of the managed objected to be specified by ATNSI for their development of production reference router and end system (through upper layers) software.

Attachment:  An Analysis of ATN Systems, Inc. RRI Managed Objects, May 8, 1997.

1.	Introduction

The attached paper from ATN Systems Inc. presents an analysis of the managed objected to be specified by ATNSI for their development of production reference router and end system (through upper layers) software.

2.	Recommendation

ATNP is requested to note the material in the attachment when developing SARPs and/or Guidance Material for ATN system management provisions.

ATTACHMENT

ATN Systems, Inc.

An Analysis of ATN Systems, Inc. RRI Managed Objects

8 May 1997

This document is intended to provide an analysis of the network management functionality required for the ATN Systems, Inc., (ATNSI) Router Reference Implementation (RRI).

Prepared By:

The MITRE Corporation

1820 Dolly Madison Blvd.

McLean, VA 22102

and

ATN Systems, Inc.

8752 Center Road, Springfield, VA  22152

(703) 644-9367, fax: (703) 569-1577, email: ATNSystems@aol.com

# 1.	Introduction

This document is intended to provide an analysis of the network management functionality required for the ATN Systems, Inc., (ATNSI) Router Reference Implementation (RRI). This functionality has been specified in the ATNSI document: Exhibit F3: ATNSI Network Management Subsystem Specification, Version 4.0, [1].  Exhibit F3 focuses primarily on the specification of a set of managed objects (MO), which through their attributes and operations, provide the required management functionality. This document does not attempt to analyze those MO in the ATNSI RRI MO set which are defined in international standards. Instead, those only those MO which are newly defined for the RRI are discussed.

The ATNSI Management Information Base (MIB) comprises the set of MO which are discussed in this document. A primary objective of the MIB analysis carried out in this document is to perform a preliminary paper validation of the RRI MIB, as it is currently defined. Clearly, any true functional validation must come from operational experience. All of the management data collected through the RRI MIB must provide a means to effective monitor and control ATN Routers. It is normal practice to extend a MIB after operational experience has identified the need for additional management data. This document is intended to provide an initial, pre-implementation look at the contents of the RRI MIB.

## 1.1	Router Management Interface

Exhibit F3 contains requirements for the network management interface. This interface design and functionality are important because the network management software must coexist with the router software in an environment which must be FAA certifiable. Details can be found in the RRI Management Interface Requirements section of Exhibit F3 for the specific management interface requirements. The required certifiable architecture will be obtained through the use of a partitioned portable streams environment (PSE). A detailed description of the PSE design, the partitioning and the certification issue certification can be found in Exhibit F1: ATNSI Router Reference Implementation Specification, Version 4.0, [2].

## 1.2	ATNSI RRI MO

This section contains a list of all of the MO currently specified for the ATNSI. Table 1-1 provides a list of the RRI MO. The source column in Table 1-1 indicates the origin of the definition for each MO.

| Managed Object | Source |
| --- | --- |
| ATN System MO | ATNSI |
| Network SubSystem MO | ISO 10733 |
| NSAP MO | ISO 10733 |
| Network Entitiy MO | ISO 10733 |
| CLNP MO | ISO 10733 |
| FIB MO | ATNSI |
| ES-IS MO | ATNSI |
| SNDCF MO | ATNSI |
| IDRP MO | ISO 10747 |
| IDRP_Config MO | ISO 10747 |
| Adj_Bis MO | ISO 10747 |
| PIB MO | ATNSI |
| RIB MO | ATNSI |
| Adj_RIB MO | ATNSI |
| Loc_RIB MO | ATNSI |
| Transport Subsystem MO | ISO 10737 |
| TSAP MO | ISO 10737 |
| COTP MO | ISO 10737 |
| CLTP MO | ISO 10737 |
| Upper Layer Subsystem MO | ATNSI |
| ACSE MO | ATNSI |
| RRI Subsystem MO | ATNSI |
| ICAO_ID MO | ATNSI |
| Performance MO | ATNSI |
| Fault MO | ATNSI |
| Configuration MO | ATNSI |

Table 1-1

# 2.	Overview of Managed Object Hierarchy

Managed objects are abstractions which encapsulate the attributes and functionality of the real ATN resources which require management.

In the OSI system management standards there are three hierarchical views which are defined. These views are also referred to as trees. These include the inheritance hierarchy, the naming hierarchy and the containment hierarchy.  The inheritance hierarchy provides information relating to how each MO is derived from a more generic super class of MO. This helps to establish categories of managed resources. The naming hierarchy is the registration tree and may be thought of as a dictionary of managed objects. The containment hierarchy is similar to the inheritance hierarchy, but is more specific. The containment hierarchy shows the objects an agent contains. It represents the Management Information Base (MIB) structure. Figure 2-1 shows the RRI containment tree.

Figure 2-1

Some of the leaf nodes in the tree shown in Figure 2-1 contain subclasses. These subclasses are identified in the tables in section three. Note also that the following MO are not found in the ISO/IEC standards and have been defined for the ATN:

- RRI Subsystem and its derivatives

- SNDCF

- ES-IS

- IDRP  -  PIB

- IDRP  -  RIB

- CLNP - FIB

# 3.	Analysis of Individual Objects

This section provides lists of all of the ATNSI RRI MO, along with their attributes and a brief description of their purpose. Those MO which are defined in ISO standards are not treated in detail.

## 3.1	ACSE

The ACSE provides for the establishment of peer ATN application associations, including ATN network and systems management application associations. ACSE services include A-Associate, A-Release, A-Abort, and A-P-Abort. Figure 3-1 shows the ACSE containment relationships. Note that there is one ACSE MO per ACSE connection.

Figure 3-1

The ATN ACSE MO attributes (and contained MO) are listed in Table 3-1.

| MO/Attribute | Description |
| --- | --- |
| ACSE MO | Defined by ATNSI |
| Total_bit_count | Counter: Total ACSE bit count |
| Total_queue_count | Counter: Total ACSE queue counts |
| Total_message_count | Counter: Total ACSE Message Count |
| ACSE_connections MO | One for each ACSE connection |
| ACSE_connections_AETs | Per connection origin and destination AET |
| Total_connection_bit_count | Per connection Total bit count |
| Overhead_bit_count | Counter: Per ACSE connection bit count for overhead bits |
| Data_bit_count | Counter: Per ACSE connection bit count for data bits |
| Message_counts | Counter: Per connection message count (data only) |
| Queue_counts | Counter: Per connection queue count |

Table 3-1

## 3.2	Transport Protocol

The ATNSI RRI requires the Transport MO as defined in ISO 10737. This MO should provide support for the following:

- All Transport connections origin/destination TSAP/NSAP and associated priority

- Bit counts (both overhead and data) by Transport connection

- Aggregate for all bit counts for all connections

- PDU size and number counts (both overhead and data) by Transport connection

- Aggregate for all PDU size and number counts for all connections

- Queue counts by transport connection

- Aggregate queue counts

Figure 3-2 illustrates the Transport Subsystem Containment Hierarchy as defined in ISO 10733.

Figure 3-2

Table 3-2 lists the Transport MO as defined by 10737.

| MO/Attribute | Description |
| --- | --- |
| Transport Entity MO | Defined in ISO10733 (Container Object) |
| Transport Subsystem MO | Defined in ISO10733 (Container Object) |
| Connection Oriented TP MO | Defined in ISO10733 |
| Connection-less TP MO | Defined in ISO10733 |

Table 3-2

## 3.3	IDRP MO Requirements

The MO classes idrp_config and adjacentBIS are supported, as defined in ISO 10747.  Additional MO classes to those specified in ISO 10747 have been defined.  These are a Policy Information Base (PIB) MO class, and Routing Information Base (RIB) MO classes. Over all, the RRI IDRP MO should provide the following functionality:

- All router connections (origin/destination Data Terminal Equipment (DTE))

- Aggregate for all router connections

- IDRP traffic (inbound and outbound, bit count and packets) on every router connection

- Aggregate IDRP traffic

- Policy Information Bases (PIB)

- Routing Information Bases (RIBs), for the Adj-Rib-In, Adj-Rib-Out and Loc-RIB.

Note: The Forwarding Information Base (FIB) MO is considered to be part of CLNP.

Figure 3-3 illustrates the containment hierarchy starting at the IDRP Configuration MO.

Figure 3-3

Table 3-3 lists the IDRP MO as defined in ISO 10747, as well as the PIB MO and RIB MO, which have been defined by ATNSI.

| MO/Attribute | Description |
| --- | --- |
| IDRP Configuration MO | Define in ISO 10747 (Container Object) |
| Adjacent BIS MO | Defined in ISO 10747 |
| PIB MO | One per BIS |
| PIB ID | String identifier |
| PIB contents | Complete contents of PIB table |
| Adj-RIB MO | Two per BIS: Adj-RIB-In & Adj-RIB-Out Records |
| Adj_RIB ID | String identifier |
| Adj_RIB contents | Contents of Adjacent RIB In/Out Records |
| Loc-RIB MO | One per BIS |
| RIB ID | String identifier |
| RIB contents | Complete contents of RIB table |
| RRI-IDRP_Stats MO |  |

Table 3-3

## 3.4	CLNP MO Requirements

The RRI CLNP MO is specified as defined in ISO 10733. An additional MO shall be defined for the Forwarding Information Base (FIB). There is one FIB MO per ATN Intermediate System (IS).

In addition to the management data supplied by the CLNP MO as defined in 10733 there is a requirement for the ability to track relay transit times through the RRI for Benchmark purposes. This requirement will be met with an ATNSI defined Relay Transit Time MO.

Figure 3-4 contains an illustration of the CLNS containment hierarchy, including that portion defined in ISO 10733 as well as the ATNSI defined portion.

Figure 3-4

The ISO defined MO and attributes, along with the additional transit time attribute and FIB MO are summarized in Table 3-4.

| MO/Attribute | Description |
| --- | --- |
| RRI CLNP MO | Defined by ATNSI; Container Object |
| CLNP MO | Defined in ISO 10733 |
| Standard attributes | Defined in ISO 10733 |
| Relay Transit Times MO | Defined by ATNSI |
| Most_recent_time | Defined by ATNSI; Most recent recorded transit time |
| Max_time | Defined by ATNSI; Longest recorded transit time |
| Min_time | Defined by ATNSI; Shortest recorded transit time |
| FIB MO | Defined by ATNSI; One per IS |
| FIB_Id | String Identifier |
| FIB_Content | Complete contents of FIB table |

Table 3-4

## 3.5	ES-IS MO Requirements

The following are the functional requirements for RRI ES-IS management:

- Adjacent End System Count

- Adjacent Intermediate System Count

- Per Adjacent System Information

- System Address (NSAP)

- Subnetwork Name

- Subnetwork Point of Attachment

- ES-IS PDU and Byte Counts - Received and Transmitted Each

- ISH PDU

- ESH PDU

- RD PDU

- Totals per Adjacent System

- Historical ES-IS Information Totals

These requirements can be met by defining the RRI ES-IS MO. The containment hierarchy for the ES-IS MO is provided in Figure 3-5 below.

Figure 3-5

The attributes of this object are listed in Table 3-5.

| MO/Attributes | Description |
| --- | --- |
| RRI ES-IS MO | Defined by ATNSI; One for every ATN ES/IS supporting ISO |
| Total_ES_count | Total number of adjacent ES |
| Total_Adj_IS count | Total number of adjacent IS |
| Adjacent_System MO | One MO per adjacent system |
| NSAP | System address (NSAP) |
| Subnetwork_name | Subnetwork Name |
| Subnetwork_point_of_attachment | Subnetwork Point of Attachment |
| Num_ISH_PDU__recd | Total number of ISH PDU received |
| Num_ISH_PDU_trans | Total number of ISH PDU transmitted |
| Num_ISH_bytes_recd | Total number of ISH bytes received |
| Num_ISH_bytes_trans | Total number of ISH bytes transmitted |
| Num_ESH_PDU_recd | Total number of ESH PDU received |
| Num_ESH_PDU_trans | Total number of ESH PDU transmitted |
| Num_ESH_bytes_recd | Total number of ESH bytes received |
| Num_ESH_bytes_trans | Total number of ESH bytes transmitted |
| Num_RD_PDU_recd | Total number of RD PDU received |
| Num_RD_PDU_trans | Total number of RD PDU transmitted |
| Num_RD_bytes_recd | Total number of RD bytes received |
| Num_RD_bytes_trans | Total number of RD bytes transmitted |
| Total_PDU_received | Total number PDU received per adjacent system |
| Total_PDU_trans | Total number PDU transmitted per adjacent system |
| Total_bytes_recd | Total number bytes received per adjacent system |
| Total_bytes_trans | Total number bytes transmitted per adjacent system |
| Historical ES-IS MO | Historical ES-IS information Totals |
| Note: The attributes of this MO are TBD; It could include attributes which reflect totals of the attributes listed above for specific periods of time, e.g. Total RD PDU for past week. | Attributes reflecting historical data |

Table 3-5

## 3.6	SNDCF MO Requirements

The management data required for the management of the RRI SNDCF can be summarized as follows:

- Queue counts by subnetwork

- Aggregate queue counts

- All router connections (origin/destination Data Terminal Equipment (DTE))

- Aggregate for all router connections

- For Mobile subnetworks there is an additional requirement for data related to the following:

- Count of Join/Leave events by subnetwork

- Aggregate on join/leave events

The SNDCF MO provides access to the required data. Figure 3-6 illustrates the containment hierarchy for the SNDCF MO.

Figure 3-6

The attributes for this MO are listed in Table 3-6.

| MO/Attribute | Description |
| --- | --- |
| SNDCF MO | Defined by ATNSI; One per ATN system |
| Total_router connections | Aggregate for all router connections |
| Total_queue count | Aggregate queue counts |
| Total_join_events | Aggregate on join events |
| Total_leave_events | Aggregate on leave events |
| Subnetwork MO | One MO per subnetwork |
| Queue_count | Queue counts by subnetwork |
| Router_connections | All router connections (origin/destination Data Terminal Equipment (DTE)) |
| Number_join_events | Count of Join event for subnetwork |
| Number_leave_events | Count of Leave events for subnetwork |

Table 3-6

## 3.7	ATNSi RRI SPECIFIC MO Requirements

The ATNSI RRI requires the following additional management data:

- Processing time through the RRI based on first bit in, last bit out (benchmark/exceptions)

- ICAO 24-bit ID, hardware part number, software part number (one MO for each, and one aggregate MO for all three)

- Fault Management

- Error counts

- Retransmit counts

- Support for Ping and Traceroute initiation and reporting

- Threshholding

- Configuration Management

- Policy Downloading

- Configuration Downloading

To provide access and support for this management data the following MO have been defined specifically for the ATNSI RRI:

- RRI Subsystem MO

- ICAO_ID MO

- RRI_Fault MO

- RRI_Performance MO

- RRI_Configuration MO

The containment relationships are depicted in Figure 3-7.

Figure 3-7

These MO are listed, with their attributes and descriptions, in Table 3-7.

| MO/Attribute | Description |
| --- | --- |
| RRI Subsystem MO | Container Object |
| ICAO_ID MO | Container Object |
| 24_bit_ID | Address String: ICAO 24 bit ID |
| Hardware_part_number | Integer: Hardware number |
| Software_part_number | Integer: Software number |
| RRI_Performance MO | Container Object |
| Processing_time | Processing time through the RRI based on first bit in, last bit out (benchmark/exceptions) |
| RRI_Configuration MO | Container Object |
| Get_Policy | Operation: Download a policy table |
| Get_Configuration | Operation: Download a routing table or other configuration table |

Table 3-7

# 4.	Notes

Note 1: There are several MO which have been defined in the international standards that are not included in this document. These include the TSAP MO (shown in Figure 1-1), the NSAP MO (not shown), and the ISO 8208 MO and its derivatives. These MO may be required for the ATNSI RRI management system. In this case, the standard definitions for these MO will serve as a baseline for the corresponding RRI MO implementations.

Note 2:  The original requirement for an individual ICAO 24-bit ID MO, a hardware part number MO and a software part number MO has been modified in this document. A single ICAO ID MO is defined, with a 24-bit ID attribute, a hardware part number attribute, and a software part number attribute.

# 5.	Summary

The MO described in this document make up the ATNSI RRI MIB. Taken as a whole, the MO which have been defined in ISO standards, along with the MO defined specifically for the ATNSI RRI provide a large data set which serves as an initial implementation of an ATN Router management system. A considerable amount of performance, fault and configuration management statistics can be obtained either  directly and through derivation, using the currently defined MIB. Operational experience will provide the opportunity for extending and refining the MIB.

References

- Exhibit F3: ATNSI Network Management Subsystem Specification, Version 4.0, October, 1996.

- Exhibit F1: ATNSI Router Reference Implementation Specification, Version 4.0,  October, 1996.

- Chapter 12, Systems Management Guidance, ATN Manual, 2nd Edition, 1993.