WG2/WP410, Rev A

Title: IDRP Timers

PDR Reference:

Originator Reference: US-SV5-01

SARPs Document Reference: 5.8.3.4.14

Status: SUBMITTED

PDR Revision Date: 25/06/97

PDR Submission Date:  24/06/97

Submitting State/Organization:  USA

Submitting Author Name: Van Trees, SP

Submitting Author E-mail Address: vantreesQsed.stel.com

Submitting Author Supplemental Contact Information:

2120 Thomas View Road

Reston, VA 20191-4803

USA

SARPs Date: 14/03/97 (Phuket)

SARPs Language:  English

Summary of Defect:  The references given for IDRP timers are incorrect.  Additional recommendations on IDRP timers are proposed.

Assigned SME:  K-P Graf

Proposed SARPs amendment:

5.8.3.4.14 IDRP Timer

(partial table)

| Item | Description | ISO/IEC 10747 Ref. |
| --- | --- | --- |
| Ta | KeepAlive timer | 7.6.1.4 |
| Tr | Retransmission (tr) timer | 7.6.1.2 |
| Tmr | maxRIBIntegrityCheck timer | 7.10.2 |
| Tma | MinRouteAdvertisement timer | 7.17.3.1 |
| Trd | MinRDOriginationInterval timer | 7.17.3.2 |
| Tcw | closeWaitDelay timer | 7.6.1.5 |

Recommendation -- IDRP keepalive values should utilize initial keepalive timer values as follows:

| Router Connectivity | Airborne/Air-Ground Capability | Nominal Keepalive |
| --- | --- | --- |
| Airborne-Air/Ground | AMSS and/or HFDL | 180 minutes |
| Air/Ground-Airborne | AMSS and/or HFDL | 180 minutes |
| Airborne-Air/Ground | Mode S and/or VDL only | 30 minutes |
| Air/Ground-Airborne | Mode S and/or VDL only | 30 minutes |
| Air/Ground-Ground Ground-Ground | Ground | 5-60 seconds (per subnetwork throughput) |

Note -- Choice of Nominal Keep Alive is base on Longest Adjacency Equipage.  The Leave event is the primary means of removing connectivity.  A lost leave event in AMSS is trapped by the timer event, and routing tables are thus cleared.

Air-Ground BIS implementations shall separate BIS-BIS connections.

Note -- ISO/IEC 10747 section 11.4 in the definition of the adjacentBISPkg-P PACKAGE requires each BIS-BIS connection to operate a separate hold and keepalive timer.

SME Recommendation to CCB: <resolve, reject, forward>

CCB Decision: