Internet Communications Service
Requirements Database Tables

Changes for Version 2.0

Jean-Pierre Briand

17 April 1997

Table of Contents

1. General Provisions

1.1 ARL010: ATN Definitions

1.2 ARL011: ATN System Components

1.3 ARL012: ATN Subnetworks

1.4 ARL013: ATN Security

1.5 ARL014: ATN Priority

1.6 ARL020: ATN Routing

1.7 ARL030: ATN Addressing

1.8 ARL040: System Management Provisions

2. Transport Layer

2.1 ARL050: ATN Transport Layer

2.2 ARL051: ATN Connection Mode Transport Layer

2.3 ARL052: ATN Connectionless Mode Transport Layer

2.4 APRL101: Classes Implemented

2.5 APRL102: Specific ATN Requirements

2.6 APRL103: Initiator/Responder Capability for Protocol Classes 0-4

2.7 APRL104: Mandatory Functions for Protocol Class 4

2.8 APRL105: Mandatory Functions for Operation over Connectionless Network Service

2.9 APRL106: ISO/IEC 8073 Optional Functions

2.10 APRL107: Supported TPDUs

2.11 APRL108: Supported TPDUs

2.12 APRL109: Parameter Values for CR TPDU

2.13 APRL110: Optional Parameters for CR TPDU

2.14 APRL111: Optional Parameters for CC TPDU

2.15 APRL112: Optional Parameters for DR TPDU

2.16 APRL113: Mandatory Parameters for DT TPDU

2.17 APRL114: Optional Parameters for AK TPDU

2.18 APRL115: Use of Sequence Number Parameter in AK TPDU

2.19 APRL116: Use of Selective Acknowledgement Parameter in AK TPDU

2.20 APRL117: Optional Parameters for ER TPDU

2.21 APRL118: Class 4 TPDUs

2.22 APRL119: User Data in Transmitted TPDUs

2.23 APRL120: User Data in Received TPDUs

2.24 APRL121: Class Negotiation - Initiator

2.25 APRL122: Class Negotiation - Initiator

2.26 APRL123: Class Negotiation - Responder

2.27 APRL124: TPDU Size Negotiation

2.28 APRL125: TPDU Size Negotiation

2.29 APRL126: TPDU Size Negotiation

2.30 APRL127: Use of Extended Format

2.31 APRL128 : Expedited Data Transport Service

2.32 APRL129: Non-use of Checksum

2.33 APRL130: Use of Selective Acknowledgement

2.34 APRL131: Use of Request Acknowledgement

2.35 APRL132: Action on Detection of Protocol Error

2.36 APRL133: Action on Receipt of Invalid/Undefined Parameter in CR TPDU

2.37 APRL134: Action on Receipt of Invalid/Undefined Parameter in CR TPDU

2.38 APRL135: Action on Receipt of Invalid/Undefined Parameter in Non-CR TPDU

2.39 APRL136: Class 4 Timers & Protocol Parameters

2.40 APRL137: Class 4 Timers & Protocol Parameters

2.41 APRL138: Class 4 Timers & Protocol Parameters

2.42 APRL151: ATN Connectionless Transport Protocol

2.43 APRL152: ATN Connectionless Transport Protocol

2.44 APRL153: ATN Connectionless Transport Protocol

2.45 APRL154: ATN Connectionless Transport Protocol

2.46 APRL155: ATN Connectionless Transport Protocol

3. Network Layer

3.1 APRL201: End Systems - Support of ATN-Specific Recommendations

3.2 APRL202: End Systems - Major Capabilities

3.3 APRL203: End Systems - Supported Functions

3.4 APRL204: End Systems - Supported NPDUs

3.5 APRL205: End Systems - Supported DT Parameters

3.6 APRL206: End Systems - Supported ER Parameters

3.7 APRL207: End Systems - Inactive DT Parameters

3.8 APRL208: End Systems - Supported ERQ Parameters

3.9 APRL209: End Systems - Supported ERP Parameters

3.10 APRL210: End Systems - Timers

3.11 APRL211: End Systems -Timers

3.12 APRL201: Intermediate Systems - Support of ATN-Specific Recommendations

3.13 APRL202: Intermediate Systems - Major Capabilities

3.14 APRL251: Intermediate Systems - Supported Functions

3.15 APRL252: Intermediate Systems - Supported Security Parameters

3.16 APRL253: Intermediate Systems - Quality of Service Maintenance Function

3.17 APRL254: Intermediate Systems - Quality of Service Maintenance Function

3.18 APRL255: Intermediate Systems - Supported NPDUs

3.19 APRL256: Intermediate Systems - Supported DT Parameters

3.20 APRL257: Intermediate Systems - Supported ER Parameters

3.21 APRL258: Intermediate Systems - Supported ERQ Parameters

3.22 APRL259: Intermediate Systems - Supported ERP Parameters

3.23 APRL260: Intermediate Systems - Timer & Parameter Values

4. Subnetworks & Subnetwork Convergence

4.1 ARL070: End Systems - Subnetwork Service & Protocol

4.2 APRL261: End Systems - SNDCF for use with ISO 8802-2 Subnetworks - Functions

4.3 APRL262: End Systems - SNDCF for use with ISO 8802-2 Subnetworks - Multi-layer Dependencies

4.4 APRL263: End Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

4.5 APRL264: End Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

4.6 APRL265: End Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

4.7 APRL266: End Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

4.8 APRL267: End Systems - SNDCF for use with ISO 8208 Subnetworks - X.25 Call User Data

4.9 APRL268: End Systems - SNDCF for use with ISO 8208 Subnetworks - ISO 8208 SNDCF Timers

4.10 APRL269: End Systems - SNDCF for use with ISO 8208 Subnetworks - SNDCF Multi-layer Dependencies

4.11 APRL270: End Systems - SNDCF for use with ISO 8208 Subnetworks - SNDCF Multi-layer Dependencies

4.12 ARL070: Intermediate Systems - Subnetwork Service & Protocol

4.13 APRL261: Intermediate Systems - SNDCF for use with ISO 8802-2 Subnetworks - Functions

4.14 APRL262: Intermediate Systems - SNDCF for use with ISO 8802-2 Subnetworks - Multi-layer Dependencies

4.15 APRL263: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

4.16 APRL264: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

4.17 APRL265: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

4.18 APRL266: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

4.19 APRL267: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - X.25 Call User Data

4.20 APRL268: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - ISO 8208 SNDCF Timers

4.21 APRL269: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - SNDCF Multi-layer Dependencies

4.22 APRL270: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - SNDCF Multi-layer Dependencies

4.23 APRL501: Intermediate Systems - Major Capabilities

4.24 APRL502: Intermediate Systems - Call Setup & Clearing Procedures

4.25 APRL503: Intermediate Systems - Negotiation of Compression Algorithm

4.26 APRL504: Intermediate Systems - Local Reference Header Compression

4.27 APRL505: Intermediate Systems - Local Reference Cancellation

4.28 APRL506: Intermediate Systems - ICAO Address Compression Algorithm

4.29 APRL507: Intermediate Systems - PDU Formats - Call Request User Data

4.30 APRL508: Intermediate Systems - PDU Formats - Call Accept User Data

4.31 APRL509: Intermediate Systems - PDU Formats - Modified IDO/IEC 8473 NPDU

4.32 APRL510: Intermediate Systems - PDU Formats - Compressed Initial PDU

4.33 APRL511: Intermediate Systems - PDU Formats - Compressed Derived PDU

4.34 APRL512: Intermediate Systems - PDU Formats - Compressed Error PDU

4.35 APRL513: Intermediate Systems - PDU Formats - SNDCF Error Report PDU

4.36 APRL514: Intermediate Systems - PDU Formats - Cancellation Request

4.37 APRL515: Intermediate Systems - PDU Formats - Cancellation Accept

5. Routing Protocols

5.1 ARL062: Routing Information Exchange Protocol

5.2 APRL301: ISO/IEC 9542 - Intermediate System

5.3 APRL302: ISO/IEC 9542 - Intermediate System

5.4 APRL303: ISO/IEC 9542 - Intermediate System

5.5 APRL304: ISO/IEC 9542 - Intermediate System

5.6 APRL305: ISO/IEC 9542 - Intermediate System

5.7 APRL401: ATN Specific Protocol Requirements

5.8 APRL402: IDRP General

5.9 APRL403: IDRP Update Send Process

5.10 APRL404: Update Receive Process

5.11 APRL405: IDRP Decision Process

5.12 APRL406: IDRP Receive

5.13 APRL407: Peer Entity Authentication

5.14 APRL408: IDRP CLNS Forwarding

5.15 APRL410: IDRP Optional Transitive Attributes

5.16 APRL411: Generating Well-known Discretionary Attributes

5.17 APRL412: Propagating Well-known Discretionary Attributes

5.18 APRL413: Receiving Well-known Discretionary Attributes

5.19 APRL414: IDRP Timers

6. Routing Initiation

6.1 ARL060: Ground-Ground Interconnection

6.2 ARL061: Air-Ground Interconnection

6.3 APRL601: Air-Ground Route Initiation - General

6.4 APRL602: Airborne Router - Subnetwork Connection Responder

6.5 APRL603: Airborne Router - Subnetwork Connection Initiator

6.6 APRL604: Air-Ground Router - Subnetwork Connection Responder

6.7 APRL605: Air-Ground Router - Subnetwork Connection Initiator

6.8 APRL606: Termination Procedures

Foreword

This documents contains the details of the changes introduced into the ICS Requirements Database version 2.0. This version is aligned to the draft ICS SARPs proposed version 8.0, dated 12 March 1997.

The rest of this document is made of 2 parts:

1) Changes to ICSRDB MS-Access Tables. This section only lists the various items to be modified as a consequence of this new version.

2) Details of changes with revision marks. The changes are made on the database text representation so that they can be reviewed in context.

Changes to ICSRDB Tables

In the following tables, the Action columns indicate the Database Query type (Add, Delete, Update) required to perform the database modification. Blank fields are considered unmodified. If required, empty fields will be explicitly defined as "empty".  Since [ARL Entries].Item is a primary key to all tables, only this field is required to uniquely identify the entry to be modified.

All Tables: Primary Key Modifications

| Action | Item | New Item |
| --- | --- | --- |
| Update | iSADDQoS | iSADDQOS |
| Update | iDADDQoS | iDADDQOS |
| Update | iGUNQoS | iGUNQOS |
| Update | QoSM-s | QOSM-s |
| Update | UD2 | UD2AM |

Table: ARL Table List

No change required

Table: ARL Entries

| Action | ARL Index | Item | Description | Reference | Status | SARPs Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Update |  | SYS5 |  |  |  | 5.2.4.1.2, 5.2.4.3.3 |
| Update |  | SYS6 |  |  |  | 5.2.4.3.2 |
| Add | ARL051 | COTP17 | COTP Timer Value Ranges |  |  | 5.5.2.2.13 |
| Add | ARL051 | COTP18 | Nominal values indicated in Table 5.5-1 should be used |  |  | 5.5.2.2.13.1 |
| Add | ARL051 | COTP19 | The assignment of optimized values for timers and parameters other than the nominal values indicated in Table 5.5-1 should be based on operational experience |  |  | 5.5.2.2.13.2 |
| Add | ARL051 | COTP20 | When intended for operation exclusively over Ground/Ground subnetworks, implementations of transport protocol timer values should be optimized to ensure interoperability. |  |  | 5.5.2.2.14 |
| Update |  | ATN3 | Support of ATN Security Label? |  |  |  |
| Update |  | ATN5 | Enhanced encoding of Acknowledgment Time Parameter? |  |  |  |
| Update |  | T4F31 |  | 6.13.4.4 |  |  |
| Update |  | T4F32 |  | 6.13.4.3 |  |  |
| Update |  | SER4L | ER support on transmission of Class 4 over CLNS |  |  |  |
| Update |  | ICR1 | Bits 8 and 7 in the additional options selection parameter of a CR TPDU set to zero? |  |  |  |
| Update |  | D4ICR | User data of up to 32 octets in a CR with preferred class 4? |  |  |  |
| Update |  | D4ICC | User data of up to 32 octets in a CC? |  |  |  |
| Update |  | D4IDR | User data of up to 64 octets in a DR? |  |  |  |
| Update |  | DRCC | 32 octets of user data in a CC TPDU? |  |  |  |
| Update |  | DRDR | 64 octets of user data in a DR TPDU? |  |  |  |
| Update |  | DRCR | 32 octets of user data in a CR TPDU? |  |  |  |
| Update |  | TS1 | If maximum TPDU size is proposed in a CR TPDU then the initiator shall support all TPDU sizes from 128 octets to the maximum proposed |  |  |  |
| Update |  | PE4L |  | 6.22.2.3 | C4L: ER, DR, Discard |  |
| Update |  | RR1 | A parameter not defined in ISO/IEC 8073 shall be ignored |  |  |  |
| Update |  | RR6 |  | 13.3.3 |  |  |
| Update |  | RR7 | What action is supported on receipt of a parameter defined in ISO 8073 (other than those covered above) and having an invalid value |  |  |  |
| Update |  | U11 | A parameter not defined in ISO/IEC 8073 shall be treated as a protocol error |  |  |  |
| Update |  | U12 | A parameter which has an invalid value as defined in ISO/IEC 8073 shall be treated as a protocol error |  |  |  |
| Update |  | TA3 | IL (Local Inactivity Time) |  |  |  |
| Update |  | ATN-TA4 | ELR (Maximum Transit Delay) |  |  |  |
| Update |  | ATN-TA5 | ERL (Maximum Transit Delay) |  |  |  |
| Update |  | ATN-TA8 | IR (Remote Inactivity Time) |  |  |  |
| Update |  | TpTu |  |  | M |  |
| Update |  | eqPRR-s |  |  | ePRR-s:M |  |
| Update |  | eqPRR-r |  |  | ePRR-r:M |  |
| Update |  | epPRR-s |  |  | ePRR-s:M |  |
| Update |  | epPRR-r |  |  | ePRRr-r:M |  |
| Update |  | iSADDQOS |  |  | iQOSM:O.3 |  |
| Update |  | iDADDQOS |  |  | iQOSM:O.3 |  |
| Update |  | iGUNQOS |  |  | iQOSM:O.3 |  |
| Update |  | iSvTD |  |  | iGUNQOS:O.4 |  |
| Update |  | iCongE |  |  | iGUNQOS:O.4 |  |
| Update |  | iTDvCst |  |  | iGUNQOS:O.4 |  |
| Update |  | iREPvTD |  |  | iGUNQOS:O.4 |  |
| Update |  | iREPvCst |  |  | iGUNQOS:O.4 |  |
| Update |  | ATNIDRP1 | Does this BIS encode and use of the Security Path Attribute? |  |  | 5.8.3.2.2, 5.8.3.2.3 |
| Update |  | ATNIDRP2 | Does this BIS immediately re-advertise routes if the security information contained in the route’s security path attribute changes? |  |  | 5.8.3.2.7 |
| Update |  | ATNIDRP3 | Does this BIS support ‘policy based route aggregation’? |  |  | 5.8.3.2.6.2 |
| Update |  | ATNIDRP4 | Does this BIS support ‘policy based route information reduction’? |  |  | 5.8.3.2.6.5 |
| Update |  | ATNIDRP5 | Does this BIS support aggregation of routes with identical NLRI using ‘true route aggregation’? |  |  | 5.8.3.2.6.3 |
| Update |  | ATNIDRP6 | Does this BIS support aggregation of routes with identical NLRI using ‘route merging’? |  |  | 5.8.3.2.6.3 |
| Update |  | ATNIDRP7 | Does this BIS support aggregation of security path attribute information field? |  |  | 5.8.3.2.6.4 |
| Update |  | RTSEL | Does this BIS support the MinRouteAdvertisementInterval Timer? (except in the case specified in ATNIDRP2) |  |  |  |
| Update |  | Ta | KeepAlive timer |  |  |  |
| Update |  | subnetLeave | Processing of a per subnetwork Leave Event |  |  |  |
| Update |  | autoRoute-ar |  |  |  | 5.3.5.2.12.3 |

Additional work:

- Update Order field

Table: ARL Status

| Action | Item | ATN Status | ES | Non BIS | G-G BIS | A/G BIS | Airborne BIS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Add | COTP17 |  | M |  |  |  |  |
| Add | COTP18 |  | REC |  |  |  |  |
| Add | COTP19 |  | REC |  |  |  |  |
| Add | COTP20 |  | REC |  |  |  |  |
| Update | ATN4 |  | M |  |  |  |  |
| Update | ATN CLNP4 |  | REC | REC | REC | REC | REC |
| Update | RC4 |  | Class 4 |  |  |  |  |
| Update | RC4a |  | Class 4 |  |  |  |  |
| Update | PE4L |  | C4L: ER, DR, Discard |  |  |  |  |
| Update | eqPRR-s |  | ePRR-s:M |  |  |  |  |
| Update | eqPRR-r |  | ePRR-r:M |  |  |  |  |
| Update | epPRR-s |  | ePRR-s:M |  |  |  |  |
| Update | epPRR-r |  | ePRR-r:M |  |  |  |  |
| Update | epQOSM-s |  | eQOSM-s:M |  |  |  |  |
| Update | iQOSNOT |  |  | iQOSM:O | iQOSM:O | iQOSM:O | iQOSM:O |
| Update | iSADDQOS |  |  |  |  |  |  |
| Update | iDADDQOS |  |  |  |  |  |  |
| Update | iGUNQOS |  |  |  |  |  |  |
| Update | iSvTD |  |  | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| Update | iCongE |  |  | iGUNQOS:M | iGUNQOS:M | iGUNQOS:M | iGUNQOS:M |
| Update | iTDvCst |  |  | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| Update | iREPvTD |  |  | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| Update | iREPvCst |  |  | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| Update | QOSM-s |  |  |  |  |  |  |
| Update | ESCT-s |  |  |  |  | OX | OX |
| Update | AGGRT |  |  |  | ATNIDRP3 or ATNIDRP5:M | ATNIDRP3 or ATNIDRP5:M |  |
| Update | ISHinCC-ar |  |  |  |  |  | RespISH-ar and fsSubnet: O |
| Update | SendISH-ai |  |  |  |  |  | EventDrvn or pollReq:M |
| Update | ISHinCR-ai |  |  |  |  |  | SendISH-ar and fsSubnet: O |
| Update | ISHinCC-agr |  |  |  |  | RespISH-agr and fsSubnet: O |  |
| Update | ISHinCR-agi |  |  |  |  | SendISH-agi and fsSubnet: O |  |

# General Provisions

## ARL010: ATN Definitions

| Item | Description | SARPs Reference | ATN Status |
| --- | --- | --- | --- |
| ATND1 | The ATN Internet | 5.2.2.1 | M |
| ATND2 | ATN Routing Domains | 5.2.2.2 | M |
| ATND3 | ATSC and AINSC End Systems and Intermediate Systems (non-BISs) located within a Mobile platform should form a single Routing Domain including the airborne router (BIS) referred to in the above note, within the appropriate Administrative Domain. | 5.2.2.2.3.3 | REC |
| ATND4 | The Ground ATN Internet | 5.2.2.3 | M |
| ATND5 | The Global ATN Backbone | 5.2.2.4 | M |
| ATND6 | Within each ATN Island, those ATN RDs that are members of the Global ATN Backbone should form a single RDC, which is referred to as the ATN Island Backbone RDC. | 5.2.2.4.2.1 | REC |
| ATND7 | The Home Domain | 5.2.2.5 | M |
| ATND8 | Administrative Domains and the ATN | 5.2.2.6 | M |
| ATND9 | Default Routes | 5.2.2.7 | M |

## ARL011: ATN System Components

| Item | Description | SARPs Reference | ES Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SYS1 | ATN End Systems - General | 5.2.3 | M | - | - | - | - |
| SYS2 | ATN Routers - General | 5.2.4 | - | M | M | M | M |
| SYS3 | Support of ISO/IEC 8473 | 5.2.3.2.1.a), 5.2.4.1.2.a) | M | M | M | M | M |
| SYS4 | Support of ISO/IEC 9542 | 5.2.3.2.2, 5.2.4.1.3 | REC | O | O | M | M |
| SYS5 | Support of ISO/IEC 10747 | 5.2.4.1.2, 5.2.4.3.3 | - | - | M | M | REC |
| SYS6 | Support of non-use ISO/IEC 10747 | 5.2.4.3.2 | - | - | - | M | SYS5: O ^SYS5: M |
| SYS7 | Support of ISO/IEC 8073 | 5.2.3.3.1 | O.1 | - | - | - | - |
| SYS8 | Support of ISO/IEC 8602 | 5.2.3.3.1 | O.1 | - | - | - | - |

O.1: At least one of these options must be supported

## ARL012: ATN Subnetworks

| Item | Description | SARPs Reference | ATN Status |
| --- | --- | --- | --- |
| ATNSN1 | Byte and Code Independence | 5.2.5.1.2 | M |
| ATNSN2 | Subnetwork QoS | 5.2.5.1.3 | M |
| ATNSN3 | Subnetwork Addressing | 5.2.5.1.4 | M |
| ATNSN4 | Internal Subnetwork Routing | 5.2.5.1.5 | M |
| ATNSN5 | Minimum SNSDU Size | 5.2.5.1.6 | M |
| ATNSN6 | Invocation of Subnetwork Priority | 5.2.5.2.2 | ISMOBSN: M |
| ATNSN7 | Invocation of Subnetwork Quality of Service for Mobile Subnetworks | 5.2.5.2.3 | ISMOBSN: REC |
| ATNSN8 | Connection-Mode Subnetwork Service | 5.2.5.2.4 | ISMOBSN: M |
| ATNSN9 | Connectivity Status Changes | 5.2.5.2.5 | ISMOBSN: M |
| ATNSN10 | Segmentation/Reassembly Mechanism | 5.2.5.2.6 | ISMOBSN: REC |

ISMOBSN: True if subnetwork is an ATN Mobile Subnetwork

## ARL013: ATN Security

| Item | Description | SARPs Reference | ATN Status | ES Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATNSEC1 | ATN Security Label | 5.2.7.1 | M | M | M | M | M | M |
| ATNSEC2 | Applications Use of ATN Security Labels | 5.2.7.2 | M | M | - | - | - | - |
| ATNSEC3 | Transport Layer Security | 5.2.7.3 | M | M | - | - | - | - |
| ATNSEC4 | Network Layer Security | 5.2.7.4 | M | M | M | M | M | M |

## ARL014: ATN Priority

| Item | Description | SARPs Reference | ATN Status | ES Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATNPRI1 | Application Priority | 5.2.8.1 | M | M | - | - | - | - |
| ATNPRI2 | Transport Connection Priority | 5.2.8.2 | M | SYS7: M | - | - | - | - |
| ATNPRI3 | Connectionless Transport Service Priority | 5.2.8.3 | M | SYS8: M | - | - | - | - |
| ATNPRI4 | ATN Internet Priority | 5.2.8.4 | M | M | M | M | M | M |
| ATNPRI5 | ATN Subnetwork Priority | 5.2.8.5 | M | M | M | M | M | M |

SYS7: Support of ISO/IEC 8073

SYS8: Support of ISO/IEC 8602

## ARL020: ATN Routing

| Item | Description | SARPs Reference | ATN Status |
| --- | --- | --- | --- |
| ATNRD1 | Service Provided by an ATN Routing Domain | 5.3.2 | M |
| ATNRD2 | Forwarding CLNP NPDUs | 5.3.2.2 | M |
| ATNRD3 | Interconnection of ATN RDs | 5.3.3 | M |
| ATNRD4 | Handling Routing Information | 5.3.6 | M |
| ATNRD5 | Routing Policy Requirements for Members of an ATN Island Backbone RDC | 5.3.7.1 | M |
| ATNRD6 | Provision to Mobile RDs of aggregated routes to ATN Islands NETs and NSAPs | 5.3.7.1.4.2 | REC |
| ATNRD7 | Routing Policy Requirements for a Mobile RD | 5.3.7.2 | M |
| ATNRD8 | Routing Policy Requirements for an ATN TRD that is not a Member of the ATN Island Backbone RDC | 5.3.7.3 | M |
| ATNRD9 | Provision to Mobile RDs of aggregated routes to ATN Islands NETs and NSAPs | 5.3.7.3.4.2 | REC |
| ATNRD10 | The Routing Policy for a Fixed ATN ERD | 5.3.7.4 | M |

## ARL030: ATN Addressing

| Item | Description | SARPs Reference | ATN Status |
| --- | --- | --- | --- |
| ADDR1 | General Provisions | 5.4.1 | M |
| ADDR2 | ATN TSAP Selector | 5.4.2.2 | M |
| ADDR3 | TSAP selector values in the range 0 to 255 should be encoded using one octet, higher values should be encoded using two octets | 5.4.2.2.6 | REC |
| ADDR4 | Network Addressing Domain | 5.4.3.2 | M |
| ADDR5 | The ATN Addressing Plan | 5.4.3.4 | M |
| ADDR6 | The Reference Publication Format | 5.4.3.5 | M |
| ADDR7 | For the purposes of publication in a text format, ATN NSAP Addresses and NETs should be written as the character sequence '470027+', identifying the common prefix for all ATN NSAP Addresses, followed by the DSP expressed as a sequence of hexadecimal characters. | 5.4.3.5.1 | REC |
| ADDR8 | The Version (VER) Field | 5.4.3.8.1 | M |
| ADDR9 | The Administration (ADM) Field | 5.4.3.8.2 | M |
| ADDR10 | ADM field encoding for Fixed AINSC | 5.4.3.8.2.3.2 | REC |
| ADDR11 | The Routing Domain Format (RDF) Field | 5.4.3.8.3 | M |
| ADDR12 | The Administrative Region Selector (ARS) Field | 5.4.3.8.4 | M |
| ADDR13 | The Location (LOC) Field | 5.4.3.8.5 | M |
| ADDR14 | The System Identifier (SYS) Field | 5.4.3.8.6 | M |
| ADDR15 | The NSAP Selector (SEL) Field | 5.4.3.8.7 | M |
| ADDR16 | Predefined NSAP Address Prefixes | 5.4.3.9 | M |

## ARL040: System Management Provisions

| Item | Description | SARPs Reference | ATN Status |
| --- | --- | --- | --- |
| SYSMAN1 | ATN managed resources should be grouped into management domains in order to assign responsibility for control of the resources | 5.9.1.1 | REC |
| SYSMAN2 | States and Organizations should assign an administrative authority to establish and maintain the respective management of each of their Management Domains, and to manage the transfer of control of resources from one Management Domain to another. | 5.9.1.2 | REC |

# Transport Layer

## ARL050: ATN Transport Layer

| Item | Description | SARPs Reference | ES Status |
| --- | --- | --- | --- |
| ATNT1 | Support of ISO 8073 and ATN specific requirements | 5.5.1 | SYS7: M |
| ATNT2 | Support of ISO 8602 and ATN specific requirements | 5.5.1 | SYS8: M |
| ATNT3 | Transport Service Access Point addresses | 5.5.1.3 | M |
| ATNT4 | Exchange of Transport Selector parameters | 5.5.1.4 | M |
| ATNT5 | The transport entity should support remote Transport-Selector parameters of variable size from 0 up to 32 octets using any encoding and any value. | 5.5.1.4.2 | REC |

SYS7: Support of ISO/IEC 8073

SYS8: Support of ISO/IEC 8602

## ARL051: ATN Connection Mode Transport Layer

| Item | Description | SARPs Reference | ES Status |
| --- | --- | --- | --- |
| COTP1 | The transport layer shall not concatenate TPDUs from TCs with different transport priorities | 5.5.2.2.3 | M |
| COTP2 | The Selective Acknowledgement should be used for conservation of bandwidth by preventing retransmission of correctly received out-of-sequence TPDUs | 5.5.2.2.4 | REC |
| COTP3 | The Request of Acknowledgement should be used to reduce AK traffic | 5.5.2.2.5 | REC |
| COTP4 | The maximum TPDU size should be at least 1024 octets. | 5.5.2.2.6 | REC |
| COTP5 | The transport layer should propose a TPDU size of 1024 octets or more | 5.5.2.2.7 | REC |
| COTP6 | The transport layer should use the TPDU size parameter rather than the preferred maximum TPDU size parameter | 5.5.2.2.8 | REC |
| COTP7 | Implementations of the ATN Transport Layer should propose use of normal format in the CR TPDU | 5.5.2.2.9 | REC |
| COTP8 | The Extended format should only be proposed when explicitly necessary to meet application Quality of Service requirements | 5.5.2.2.10 | REC |
| COTP9 | The Transport Layer should accept non-use of checksum when proposed in a CR TPDU | 5.5.2.2.11 | REC |
| COTP10 | Configurable timers and variables | 5.5.2.2.12 | REC |
| COTP11 | Connection Mode transport priority | 5.5.2.3.1 | M |
| COTP12 | Connection Mode transport security | 5.5.2.3.2 | M |
| COTP13 | Encoding of Protocol Data Units | 5.5.2.4 | M |
| COTP14 | Transport Layer Congestion Avoidance | 5.5.2.5 | M |
| COTP15 | Recommended Algorithm Values | 5.5.2.5.4.1 | REC |
| COTP16 | Use of the ATN Network Service | 5.5.2.6 | M |
| COTP17 | COTP Timer Value Ranges | 5.5.2.2.13 | M |
| COTP18 | Nominal values indicated in Table 5.5-1 should be used | 5.5.2.2.13.1 | REC |
| COTP19 | The assignment of optimized values for timers and parameters other than the nominal values indicated in Table 5.5-1 should be based on operational experience | 5.5.2.2.13.2 | REC |
| COTP20 | When intended for operation exclusively over Ground/Ground subnetworks, implementations of transport protocol timer values should be optimized to ensure interoperability. | 5.5.2.2.14 | REC |

## ARL052: ATN Connectionless Mode Transport Layer

| Item | Description | SARPs Reference | ES Status |
| --- | --- | --- | --- |
| CLTP1 | CL Transport Service primitives | 5.5.3.2 | M |
| CLTP2 | Connectionless Mode Transport Quality of Service Parameters | 5.5.3.3 | M |
| CLTP3 | Encoding of Transport Protocol Data Units | 5.5.3.4 | M |
| CLTP4 | Use of Network Service by CLTP | 5.5.3.5 | M |

## APRL101: Classes Implemented

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| C0 | Class 0 | 14.2 | O.1 | O |
| C1 | Class 1 | 14.4 | C0:O | O |
| C2 | Class 2 | 14.2 | O.1 | O |
| C3 | Class 3 | 14.3 | C2:O | O |
| C4 | Class 4 operation over CONS | 14.3 | C2:O | O |
| C4L | Class 4 operation over CLNS | 14.3 | C2:O | M |

O.1: At least one of these options must be supported

## APRL102: Specific ATN Requirements

| Item | Description | SARPs Reference | ES Status |
| --- | --- | --- | --- |
| ATN1 | Support of Congestion Avoidance Procedures? | 5.5.2.5 | M |
| ATN2 | Transport to Network Priority Mapping? | 5.5.2.6.1.4.2 | M |
| ATN3 | Support of ATN Security Label? | 5.5.2.6.1.4.3 | M |
| ATN4 | Timer Settings on a per Transport Connection basis? | 5.5.2.2.12 | M |
| ATN5 | Enhanced encoding of Acknowledgment Time Parameter? | 5.5.2.4.2 | M |

## APRL103: Initiator/Responder Capability for Protocol Classes 0-4

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| IR1 | Initiating CR TPDU | 14.5 a) | O.2 | M |
| IR2 | Responding to CR TPDU | 14.5 a) | O.2 | M |

O.2: At least one of these options must be supported

## APRL104: Mandatory Functions for Protocol Class 4

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| T4F1 | TPDU transfer | 6.2 | M | M |
| T4F2 | Segmenting | 6.3 | M | M |
| T4F3 | Reassembling | 6.3 | M | M |
| T4F4 | Separation | 6.4 | M | M |
| T4F5 | Connection establishment | 6.5 | M | M |
| T4F6 | Connection refusal | 6.6 | M | M |
| T4F7 | Data TPDU numbering (normal) | 6.10 | M | M |
| T4F8 | Retention and acknowledgement of TPDUs (AK) | 6.13.4.1 | M | M |
| T4F9 | Explicit flow control | 6.16 | M | M |
| T4F10 | Checksum | 6.17 | M | M |
| T4F11 | Frozen references | 6.18 | M | M |
| T4F12 | Retransmission on time-out | 6.19 | M | M |
| T4F13 | Resequencing | 6.20 | M | M |
| T4F14 | Inactivity control | 6.21 | M | M |

## APRL105: Mandatory Functions for Operation over Connectionless Network Service

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| T4F23 | Transmission over CLNS | 6.1.2 | M | M |
| T4F24 | Normal release when operating over CLNS (explicit) | 6.7.2 | M | M |
| T4F25 | Association of TPDUs with transport connections when operating over CLNS | 6.9.2 | M | M |
| T4F26 | Expedited data transfer when operating over CLNS (Network normal) | 6.11.2 | M | M |
| T4F27 | Treatment of protocol errors when operating over CLNS | 6.22.2 | M | M |

## APRL106: ISO/IEC 8073 Optional Functions

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| T4F28 | Data TPDU numbering (extended) | 6.10 | O | O |
| T4F29 | Non-use of checksum | 6.17 | O | M |
| T4F30 | Concatenation | 6.4 | O | O |
| T4F31 | Retention and acknowledgement of TPDUs Use of selective acknowledgement | 6.13.4.4 | O | O |
| T4F32 | Retention and acknowledgement of TPDUs Use of request acknowledgement | 6.13.4.3 | O | O |

## APRL107: Supported TPDUs

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| ST1 | CR supported on transmission | 13.1 | IR1:M | M |
| ST2 | CRsupported on receipt | 13.1 | IR2:M | M |
| ST3 | CC supported on transmission | 13.1 | IR2:M | M |
| ST4 | CC supported on receipt | 13.1 | IR1:M | M |
| ST5 | DR supported on transmission | 13.1 | IR2:M | M |
| ST6 | DR supported on receipt | 13.1 | IR1:M | M |
| ST7 | DC supported on transmission | 13.1 | C4L:M | M |
| ST8 | DC supported on receipt | 13.1 | C4L:M | M |
| ST9 | DT supported on transmission | 13.1 | M | M |
| ST10 | DT supported on receipt | 13.1 | M | M |
| ST11 | ED supported on transmission | 13.1 | C4L:M | MO |
| ST12 | ED supported on receipt | 13.1 | C4L:M | MO |
| ST13 | AK supported on transmission | 13.1 | C4L:M | M |
| ST14 | AK supported on receipt | 13.1 | C4L:M | M |
| ST15 | EA supported on transmission | 13.1 | C4L:M | MO |
| ST16 | EA supported on receipt | 13.1 | C4L:M | MO |
| ST19 | ER supported on receipt | 13.1 | M | M |

C4L: Class 4 operation over CLNS

IR1: Initiating CR TPDU

IR2: Responding to CR TPDU

## APRL108: Supported TPDUs

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| SER4L | ER support on transmission of Class 4 over CLNS | 6.22.2 | O | O |

## APRL109: Parameter Values for CR TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| ICR1 | Bits 8 and 7 in the additional options selection parameter of a CR TPDU set to zero? | 13.3.4.g) | M | M |
| ICR2 | Is class 0 always offered as an alternative class? | 14.4 | O | X |

## APRL110: Optional Parameters for CR TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| I4CR7 | Called Transport-Selector | 13.3.4 a) | O | M |
| I4CR8 | Calling Transport-Selector | 13.3.4 a) | O | M |
| I4CR9 | TPDU size | 13.3.4 b) | O | O |
| I4CR10 | Version Number | 13.3.4 d) | O | O |
| I4CR11 | Protection parameters | 13.3.4 e) | O | O |
| I4CR12 | Additional option selection | 13.3.4 g) | O | M |
| I4CR13 | Throughput | 13.3.4 k) | O | O |
| I4CR14 | Residual error rate | 13.3.4 m) | O | O |
| I4CR15 | Priority | 13.3.4 n) | O | M |
| I4CR16 | Transit delay | 13.3.4 p) | O | O |
| I4CR17 | Acknowledgement time | 13.3.4 j) | O | M |
| I4CR18 | Preferred maximum TPDU size | 13.3.4 c) | O | O |
| I4CR19 | Inactivity timer | 13.3.4 r) | O | M |

## APRL111: Optional Parameters for CC TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| I4CC6 | Called Transport-Selector | 13.4.4 | O | M |
| I4CC7 | Calling Transport-Selector | 13.4.4 | O | M |
| I4CC8 | TPDU size | 13.4.4 | O | O |
| I4CC9 | Protection parameters | 13.4.4 | O | O |
| I4CC10 | Additional option selection | 13.4.4 | O | M |
| I4CC11 | Acknowledgement time | 13.4.4 | O | M |
| I4CC12 | Throughput | 13.4.4 | O | O |
| I4CC13 | Residual error rate | 13.4.4 | O | O |
| I4CC14 | Priority | 13.4.4 | O | M |
| I4CC15 | Transit delay | 13.4.4 | O | O |
| I4CC16 | Preferred maximum TPDU size | 13.4.4 | I4CR18:O | O |
| I4CC17 | Inactivity timer | 13.4.4 | O | M |

I4CR18: Preferred maximum TPDU size

## APRL112: Optional Parameters for DR TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| I4DR4 | Additional information | 13.5.4 a) | O | O |

## APRL113: Mandatory Parameters for DT TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| I4DT4 | Request of acknowledgement | 13.7.3 b) | T4F32:M | T4F32:M |

T4F32: Retention and acknowledgement of TPDUs. Use of request acknowledgement

## APRL114: Optional Parameters for AK TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| I4AK4 | Flow control confirmation | 13.9.4 c) | O | M |

## APRL115: Use of Sequence Number Parameter in AK TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| I4AK5 | Subsequence number | 13.9.4. b) | O | M |

## APRL116: Use of Selective Acknowledgement Parameter in AK TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| I4AK6 | Selective acknowledgement parameters | 13.9.4. d) | T4F31:O | T4F31:O |

T4F31: Retention and acknowledgement of TPDUs. Use of selective acknowledgement

## APRL117: Optional Parameters for ER TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| I4ER3 | Invalid TPDU | 13.12.4 a) | O | O |

## APRL118: Class 4 TPDUs

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| R4CCch | CC TPDU | 13.4.4 | M | M |
| R4DRch | DR TPDU | 13.5.4 b) | M | M |
| R4DCch | DC TPDU | 13.6.4 | M | M |
| R4DTch | DT TPDU | 13.7.4 | M | M |
| R4EDch | ED TPDU | 13.8.4 | M | M |
| R4AKch | AK TPDU | 13.9.4 a) | M | M |
| R4EAch | EA TPDU | 13.10.4 | M | M |
| R4ERch | ER TPDU | 13.12.4 b) | M | M |

## APRL119: User Data in Transmitted TPDUs

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| D4ICR | User data of up to 32 octets in a CR with preferred class 4? | 13.3.5 | M | M |
| D4ICC | User data of up to 32 octets in a CC? | 13.4.5 | M | M |
| D4IDR | User data of up to 64 octets in a DR? | 13.5.5 | M | M |

## APRL120: User Data in Received TPDUs

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| DRCC | 32 octets of user data in a CC TPDU? | 13.4.5 | IR1:M | IR1:M |
| DRDR | 64 octets of user data in a DR TPDU? | 13.5.5 | IR1:M | IR1:M |
| DRCR | 32 octets of user data in a CR TPDU? | 13.3.5 | IR2:M | IR2:M |

IR1: Initiating CR TPDU

IR2: Responding to CR TPDU

## APRL121: Class Negotiation - Initiator

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| NC | The preferred class in the CR TPDU may contain any of the classes supported by the implementation | 6.5.5 j) |  | Class 4 |

## APRL122: Class Negotiation - Initiator

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| NAC5 | Class 4 over CLNS | 6.5.5 j) | None | None |

## APRL123: Class Negotiation - Responder

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| RC4 | What classes can you respond with if CR proposes only class 4? | 6.5.4 j) Table 3 | 2,4 or connection refused depending on classes supported | Class 4 |
| RC4a | What classes can you respond with if CR proposes class 4 as preferred class and the alternative class parameter is present? | 6.5.4 j) Table 3 | 0,1,2,3,4 or connection refused depending on classes supported and coding of alternative class | Class 4 |

## APRL124: TPDU Size Negotiation

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| TS1 | If maximum TPDU size is proposed in a CR TPDU then the initiator shall support all TPDU sizes from 128 octets to the maximum proposed | 14.6 e) | I4CR9:M | I4CR9:M |
| TS2 | If the preferred maximum TPDU size parameter is used in a CR TPDU then the initiator shall support all TPDU sizes, except 0, that are multiples of 128 octets up to the preferred maximum proposed | 14.6 e) | I4CR18:M | I4CR18:M |

I4CR18: Preferred maximum TPDU size

I4CR9: TPDU size

## APRL125: TPDU Size Negotiation

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| TS3 | What is the largest value of the preferred maximum TPDU size parameter in a CR TPDU? | 14.6 e) | any multiple of 128 octets | any multiple of 128 octets |
| TS4 | What is the largest value of the preferred maximum TPDU size parameter in a CC TPDU? | 14.6 e) | any multiple of 128 octets | any multiple of 128 octets |

## APRL126: TPDU Size Negotiation

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| T4S1 | What is the largest value of the maximum TPDU size parameter in a CR TPDU with preferred class 4? | 14.6 e) | One of 128, 256, 512, 1024, 2048, 4096, 8192 | One of 128, 256, 512, 1024, 2048, 4096, 8192 |
| T4S2 | What is the largest value of the maximum TPDU size parameter which may be sent in the CC TPDU when class 4 is selected? | 14.6 e) | 128, 256, 512, 1024, 2048, 4096, 8192 | 128, 256, 512, 1024, 2048, 4096, 8192 |

## APRL127: Use of Extended Format

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| NEF3 | What formats can you propose in the CR TPDU in class 4? | 6.5.5 n) | normal, extended | normal, extended |
| NEF6 | What formats can you select in CC when extended has been proposed in CR in class 4? | 6.5.5 n) | normal, extended | normal, extended |

## APRL128 : Expedited Data Transport Service

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| TED1 | Is the expedited data indication supported in CR and CC TPDU? | 6.5.5 r) | M | MO |

## APRL129: Non-use of Checksum

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| NUC1 | What proposals can you make in the CR? | 6.5.5 p) | non-use, use | non-use, use |
| NUC2 | What proposals can you make in CC when non-use of checksum has been proposed in CR? | 6.5.5 p) | non-use, use | non-use, use |

## APRL130: Use of Selective Acknowledgement

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| USA1 | Is use of selective acknowledgement proposed in CR TPDUs ? | 6.5.5 s) | O | O |
| USA2 | Is use of selective acknowledgement selected in a CC when it has been proposed in a CR ? | 6.5.5 s) | O | O |

## APRL131: Use of Request Acknowledgement

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| ROA1 | Is use of request of acknowledgement proposed in CR TPDUs ? | 6.5.5 t) | O | O |
| ROA2 | Is use of request of acknowledgement selected in a CC when it has been proposed in a CR ? | 6.5.5 t) | O | O |

## APRL132: Action on Detection of Protocol Error

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| PE4L | Class 4 over CLNS | 6.22.2.3 | C4L: ER, DR, Discard | C4L: ER, DR, Discard |

## APRL133: Action on Receipt of Invalid/Undefined Parameter in CR TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| RR1 | A parameter not defined in ISO/IEC 8073 shall be ignored | 13.2.3 | M | M |
| RR2 | An invalid value in the alternative protocol class parameter shall be treated as a protocol error | 13.2.3 | M | M |
| RR3 | An invalid value in the class and option parameter shall be treated as a protocol error | 13.2.3 | M | M |
| RR4 | On receipt of the additional option selection parameter bits 8 to 7, and bits 6 to 1 if not meaningful for the proposed class, shall be ignored | 13.3.4 g) | M | M |
| RR6 | On receipt of the class option parameter bits 4 to 1 if not meaningful for the proposed class shall be ignored | 13.3.3 | M | M |

## APRL134: Action on Receipt of Invalid/Undefined Parameter in CR TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| RR7 | What action is supported on receipt of a parameter defined in ISO 8073 (other than those covered above) and having an invalid value | 13.2.3 | Ignore, Protocol Error | Ignore, Protocol Error |

## APRL135: Action on Receipt of Invalid/Undefined Parameter in Non-CR TPDU

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| U11 | A parameter not defined in ISO/IEC 8073 shall be treated as a protocol error | 13.2.3 | M | M |
| U12 | A parameter which has an invalid value as defined in ISO/IEC 8073 shall be treated as a protocol error | 13.2.3 | M | M |
| U13 (class 4 only) | A TPDU received with a checksum which does not satisfy the defined formula shall be discarded | 6.17.3 | M | M |

## APRL136: Class 4 Timers & Protocol Parameters

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| TA1 | T1 (Local Retransmission) | 12.2.1.1.4 | M | M |
| TA2 | N (Maximum Transmission) | 12.2.1 | M | M |
| TA3 | IL (Local Inactivity Time) | 12.2.1.1.7 | M | M |
| TA4 | W (Window Update) | 12.2.1 | M | M |
| TA5 | L (Frozen Reference Time) | 12.2.1.1.6 | M | M |

## APRL137: Class 4 Timers & Protocol Parameters

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| ATN-TA1 | R (Persistence) | 12.2.1.1.5 | O | O |
| ATN-TA2 | MLR (NSDU Lifetime) | 12.2.1.1.1 | O | O |
| ATN-TA3 | MRL (NSDU Lifetime) | 12.2.1.1.1 | O | O |
| ATN-TA4 | ELR (Maximum Transit Delay) | 12.2.1.1.2 | O | O |
| ATN-TA5 | ERL (Maximum Transit Delay) | 12.2.1.1.2 | O | O |
| ATN-TA6 | AL (Acknowledgement Time) | 12.2.1.1.3 | O | M |
| ATN-TA7 | AR (Acknowledgement Time) | 12.2.1.1.3 | O | M |
| ATN-TA8 | IR (Remote Inactivity Time) | 12.2.1.1.7 | O | M |

## APRL138: Class 4 Timers & Protocol Parameters

| Item | Description | ISO/IEC 8073 Reference | ISO/IEC 8073 Status | ES Status |
| --- | --- | --- | --- | --- |
| OT9 | Does IUT support optional timer TS2 when operating in class 4? | 6.22.2.3 | O | O |

## APRL151: ATN Connectionless Transport Protocol

| Item | Description | ISO/IEC 8602 Reference | ISO/IEC 8602 Status | ES Status |
| --- | --- | --- | --- | --- |
| NS | Network service selection | 5.3.2.2 | M | M |
| AM | Address mapping | 5.3.2.3 | M | M |

## APRL152: ATN Connectionless Transport Protocol

| Item | Description | ISO/IEC 8602 Reference | ISO/IEC 8602 Status | ES Status |
| --- | --- | --- | --- | --- |
| UD1 | Unitdata PDU supported on transmission | 6.1.3 | M | M |
| UD2AM | Unitdata PDU supported on reception | 6.1.3 | M | M |

## APRL153: ATN Connectionless Transport Protocol

| Item | Description | ISO/IEC 8602 Reference | ISO/IEC 8602 Status | ES Status |
| --- | --- | --- | --- | --- |
| TpTc | <t> TPDU UD Checksum | 6.2.4.1 | O | M |
| TpTs | <t> TPDU UD Source Transport-Selector | 6.2.4.1 | M | M |
| TpTd | <t> TPDU UD Destination Transport-Selector | 6.2.4.1 | M | M |
| TpTu | <t> TPDU UD User Data | 6.2.4.1 | M | M |

## APRL154: ATN Connectionless Transport Protocol

| Item | Description | ISO/IEC 8602 Reference | ISO/IEC 8602 Status | ES Status |
| --- | --- | --- | --- | --- |
| TpRc | <r> TPDU UD Checksum | 6.2.4.2 | M | M |
| TpRs | <r> TPDU UD Source Transport-Selector | 6.2.4.2 | M | M |
| TpRd | <r> TPDU UD Destination Transport-Selector | 6.2.4.2 | M | M |
| TpRu | <r> TPDU UD User Data | 6.2.4.2 | M | M |

## APRL155: ATN Connectionless Transport Protocol

| Item | Description | ISO/IEC 8602 Reference | ISO/IEC 8602 Status | ES Status |
| --- | --- | --- | --- | --- |
| CL | Connectionless Mode Network Service | 6.2 | M | M |

# Network Layer

## APRL201: End Systems - Support of ATN-Specific Recommendations

| Item | Description | SARPs Reference | ES Status |
| --- | --- | --- | --- |
| ATN CLNP1 | Encoding and use of the Security Parameter | 5.6.2.2 | M |
| ATN CLNP2 | Management of Network Priority | 5.6.2.3, 5.2.8.4 | M |
| ATN CLNP3 | Use of Non-Segmenting Subset | 5.6.3.1.3 | O |
| ATN CLNP4 | Echo Request Function | 5.6.3.3 | REC |
| ATN CLNP5 | Congestion Management | 5.6.2.4 | M |

## APRL202: End Systems - Major Capabilities

| Item | Description | ISO/IEC 8473 Reference | ISO/IEC 8473 Status | SARPs Reference | ES Status |
| --- | --- | --- | --- | --- | --- |
| ES | End System |  | O.1 |  | O.1 |
| IS | Intermediate System |  | O.1 |  | O.1 |
| FL-r | <r> Full protocol | 8473-1: 6 | M |  | M |
| FL-s | <s> Full protocol | 8473-1: 6 | M |  | M |
| NSS-r | <r> Non-segmenting subset | 8473-1: 5.2 | M |  | M |
| NSS-s | <s> Non segmenting subset | 8473-1: 5.2 | IS:M ^IS:O |  | IS:M ^IS:O |
| IAS-r | <r> Inactive subset | 8473-1: 5.2 | ES:O |  | ES:O |
| IAS-s | <s> Inactive subset | 8473-1: 5.2 | IAS-r:M ^IAS-r:X |  | IAS-r:M ^IAS-r:X |
| S802 | SNDCF for ISO 8802 | 8473-2: 5.4 | O.2 |  | O |
| SCLL | SNDCF for CL Link Service | 8473-4: 5.3.1 | O.2 |  | O |
| SCOL | SNDCF for CO Link Service | 8473-4: 5.3.2 | O.2 |  | O |
| SX25 | SNDCF for ISO 8208 | 8473-3: 5.4 | O.2 |  | O |
| ATN SNDCF | SNDCF for Mobile Subnetworks |  |  | 5.7 | ISMOB:M ISGRD:O ^IS:O |

O.1: At least one of these options must be supported

O.2: At least one of these options must be supported

ES: End System

IS: Intermediate System

IAS-r: <r> Inactive subset

ISMOB: If ISO 8473 is used over ground subnetworks, then true, else false

ISGRD: If ISO 8473 is used over ground subnetworks, then true, else false

## APRL203: End Systems - Supported Functions

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | SARPs Reference | ES Status |
| --- | --- | --- | --- | --- | --- |
| ePDUC | PDU Composition | 6.1 | M |  | M |
| ePDUD | PDU Decomposition | 6.2 | M |  | M |
| eHFA | Header Format Analysis | 6.3 | M |  | M |
| ePDUL-s | <s> PDU Lifetime Control | 6.4 | M |  | M |
| ePDUL-r | <r> PDU Lifetime Control | 6.4 | O |  | M |
| eRout | Route PDU | 6.5 | M |  | M |
| eForw | Forward PDU | 6.6 | M |  | M |
| eSegm | Segment PDU | 6.7 | M |  | M |
| eReas | Reassemble PDU | 6.8 | M |  | M |
| eDisc | Discard PDU | 6.9 | M |  | M |
| eErep | Error Reporting | 6.10 | M |  | M |
| eEdec-s | <s> Header Error Detection | 6.11 | M |  | M |
| eEdec-r | <r> Header Error Detection | 6.11 | M |  | M |
| eSecu-s | <s> Security | 6.13 | O | 5.6.2.2 | M |
| eSecu-r | <r> Security | 6.13 | O | 5.6.2.2 | M |
| eCRR-s | <s> Complete Route Recording | 6.15 | O |  | OX |
| eCRR-r | <r> Complete Route Recording | 6.15 | O |  | O |
| ePRR-s | <s> Partial Route Recording | 6.15 | O |  | M |
| ePRR-r | <r> Partial Route Recording | 6.15 | O |  | M |
| eCSR | Complete Source Routing | 6.14 | O |  | OX |
| ePSR | Partial Source Routing | 6.14 | O |  | OX |
| ePRI-s | <s> Priority | 6.17 | O | 5.6.3.4 | M |
| ePRI-r | <r> Priority | 6.17 | O | 5.6.3.4 | M |
| eQOSM-s | <s> QOS Maintenance | 6.16 | O |  | M |
| eQOSM-r | <r> QOS Maintenance | 6.16 | O |  | M |
| eCong-s | <s> Congestion Notification | 6.18 | eQOSM-s:M |  | eQOSM-s:M |
| eCong-r | <r> Congestion Notification | 6.18 | O |  | M |
| ePadd-s | <s> Padding | 6.12 | O |  | OX |
| ePadd-r | <r> Padding | 6.12 | M |  | M |
| eEreq | Echo request | 6.19 | O | 5.6.3.3 | O |
| eErsp | Echo response | 6.20 | O |  | M |
| eSegS | Create segments smaller than necessary | 6.8 | O |  | O |

eQOSM-s: <s> QOS Maintenance

## APRL204: End Systems - Supported NPDUs

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | ES Status |
| --- | --- | --- | --- | --- |
| eDT-t | DT (full protocol) transmit | 7.7 | M | M |
| eDT-r | DT (full protocol) receive | 7.7 | M | M |
| eDTNS-t | DT (non-segment) transmit | 7.7 | NSS-s:M | NSS-s:M |
| eDTNS-r | DT (non-segment) receive | 7.7 | M | M |
| eER-t | ER transmit | 7.9 | M | M |
| eER-r | ER receive | 7.9 | M | M |
| eIN-t | Inactive PDU transmit | 7.8 | IAS-s:M | IAS-s:M |
| eIN-r | Inactive PDU receive | 7.8 | IAS-r:M | IAS-r:M |
| eERQ-t | ERQ transmit | 7.10 | eEreq:M | eEreq:M |
| eERQ-r | ERQ receive | 7.10 | M | M |
| eERP-t | ERP transmit | 7.11 | eErsp:M | eErsp:M |
| eERP-r | ERP receive | 7.11 | M | M |

eEreq: Echo request

eErsp: Echo response

NSS-s: <s> Non segmenting subset

IAS-s: <s> Inactive subset

IAS-r: <r> Inactive subset

## APRL205: End Systems - Supported DT Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | ES Status |
| --- | --- | --- | --- | --- |
| edFxPt-s | <s> Fixed Part | 7.2 | M | M |
| edFxPt-r | <r> Fixed Part | 7.2 | M | M |
| edAddr-s | <s> Address | 7.3 | M | M |
| edAddr-r | <r> Address | 7.3 | M | M |
| edSeg-s | <s> Segmentation Part | 7.4 | M | M |
| edSeg-r | <r> Segmentation Part | 7.4 | M | M |
| edPadd-s | <s> Padding | 7.5.2 | ePadd-s:M | - |
| edPadd-r | <r> Padding | 7.5.2 | M | M |
| edSecu-s | <s> Security | 7.5.3 | eSecu-s:M | eSecu-s:M |
| edSecu-r | <r> Security | 7.5.3 | eSecu-r:M | eSecu-r:M |
| edCRR-s | <s> Complete Route Recording | 7.5.5 | eCRR-s:M | - |
| edCRR-r | <r> Complete Route Recording | 7.5.5 | eCRR-r:M | eCRR-r:M |
| edPRR-s | <s> Partial Route Recording | 7.5.5 | ePRR-s:M | ePRR-s:M |
| edPRR-r | <r> Partial Route Recording | 7.5.5 | ePRR-r:M | ePRR-r:M |
| edCSR-s | <s> Complete Source Routing | 7.5.4 | eCSR:M | - |
| edPSR-s | <s> Partial Source Routing | 7.5.4 | ePSR:M | - |
| edQOSM-s | <s> QOS Maintenance | 7.5.6 | eQOSM-s or eCong-s:M | eQOSM-s:M |
| edQOSM-r | <r> QOS Maintenance | 7.5.6 | eQOSM-r or eCong-r :M | eQOSM-r or eCong-r:M |
| edPri-s | <s> Priority | 7.5.7 | ePRI-s:M | ePRI-s:M |
| edPri-r | <r> Priority | 7.5.7 | ePRI-r:M | ePRI-r:M |
| edData-s | <s> Data | 7.6 | M | M |
| edData-r | <r> Data | 7.6 | M | M |
| edUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.21 | M | M |
| edUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

ePSR: Partial Source Routing

ePRR-r: <r> Partial Route Recording

ePRR-s: <s> Partial Route Recording

eCSR: Complete Source Routing

eCRR-r: <r> Complete Route Recording

eCRR-s: <s> Complete Route Recording

eQOSM-r: <r> QOS Maintenance

eQOSM-s: <s> QOS Maintenance

eCong-r: <r> Congestion Notification

eCong-s: <s> Congestion Notification

eSecu-r: <r> Security

eSecu-s: <s> Security

ePRI-s: <s> Priority

ePRI-r: <r> Priority

ePadd-s: <s> Padding

## APRL206: End Systems - Supported ER Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | ES Status |
| --- | --- | --- | --- | --- |
| eeFxPt-s | <s> Fixed Part | 7.2 | M | M |
| eeFxPt-r | <r> Fixed Part | 7.2 | M | M |
| eeAddr-s | <s> Address | 7.3 | M | M |
| eeAddr-r | <r> Address | 7.3 | M | M |
| eePadd-s | <s> Padding | 7.5.2 | ePadd-s:M | - |
| eePadd-r | <r> Padding | 7.5.2 | M | M |
| eeSecu-s | <s> Security | 7.5.3 | eSecu-s:M | eSecu-s:M |
| eeSecu-r | <r> Security | 7.5.3 | eSecu-r:M | eSecu-r:M |
| eeCRR-s | <s> Complete Route Recording | 7.5.5 | eCRR-s:M | - |
| eeCRR-r | <r> Complete Route Recording | 7.5.5 | eCRR-r:M | eCRR-r:M |
| eePRR-s | <s> Partial Route Recording | 7.5.5 | ePRR-s:M | ePRR-s:M |
| eePRR-r | <r> Partial Route Recording | 7.5.5 | ePRR-r:M | ePRR-r:M |
| eeCSR-s | <s> Complete Source Routing | 7.5.4 | eCSR:M | - |
| eePSR-s | <s> Partial Source Routing | 7.5.4 | ePSR:M | - |
| eeQOSM-s | <s> QOS Maintenance | 7.5.6 | eQOSM-s or eCong-s:M | eQOSM-s or eCong-s:M |
| eeQOSM-r | <r> QOS Maintenance | 7.5.6 | eQOSM-r or eCong-r:M | eQOSM-r or eCong-r:M |
| eePri-s | <s> Priority | 7.5.7 | ePri-s:M | ePri-s:M |
| eePri-r | <r> Priority | 7.5.7 | ePri-r:M | ePri-r:M |
| eeDisc-s | <s> Reason for discard | 7.9.5 | M | M |
| eeDisc-r | <r> Reason for discard | 7.9.5 | M | M |
| eeData-s | <s> Data | 7.9.6 | M | M |
| eeData-r | <r> Data | 7.9.6 | M | M |
| eeUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.21 | M | M |
| eeUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

ePSR: Partial Source Routing

ePRR-r: <r> Partial Route Recording

ePRR-s: <s> Partial Route Recording

eCSR: Complete Source Routing

eCRR-r: <r> Complete Route Recording

eCRR-s: <s> Complete Route Recording

ePRI-r: <r> Priority

ePRI-s: <s> Priority

eQOSM-r: <r> QOS Maintenance

eQOSM-s: <s> QOS Maintenance

eCong-r: <r> Congestion Notification

eCong-s: <s> Congestion Notification

eSecu-r: <r> Security

eSecu-s: <s> Security

ePadd-s: <s> Padding

## APRL207: End Systems - Inactive DT Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | ES Status |
| --- | --- | --- | --- | --- |
| eiNLPI-s | <s> Inactive Network Layer Protocol identifier | 7.8.2 | IAS-s:M | IAS-s:M |
| eiNLPI-r | <r> Inactive Network Layer Protocol Identifier | 7.8.2 | IAS-r:M | IAS-r:M |
| eiData-s | <s> Data | 7.8.3 | IAS-s:M | IAS-s:M |
| eiData-r | <r> Data | 7.8.3 | IAS-r:M | IAS-r:M |

IAS-r: <r> Inactive subset

IAS-s: <s> Inactive subset

## APRL208: End Systems - Supported ERQ Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | ES Status |
| --- | --- | --- | --- | --- |
| eqFxPt-s | <s> Fixed Part | 7.2 | M | M |
| eqFxPt-r | <r> Fixed Part | 7.2 | M | M |
| eqAddr-s | <s> Address | 7.3 | M | M |
| eqAddr-r | <r> Address | 7.3 | M | M |
| eqSeg-s | <s> Segmentation Part | 7.4 | M | M |
| eqSeg-r | <r> Segmentation Part | 7.4 | M | M |
| eqPadd-s | <s> Padding | 7.5.2 | ePadd-s:M | - |
| eqPadd-r | <r> Padding | 7.5.2 | M | M |
| eqSecu-s | <s> Security | 7.5.3 | eSecu-s:M | eSecu-s:M |
| eqSecu-r | <r> Security | 7.5.3 | eSecu-r:M | eSecu-r:M |
| eqCRR-s | <s> Complete Route Recording | 7.5.5 | eCRR-s:M | - |
| eqCRR-r | <r> Complete Route Recording | 7.5.5 | eCRR-r:M | eCRR-r:M |
| eqPRR-s | <s> Partial Route Recording | 7.5.5 | ePRR-s:M | ePRR-s:M |
| eqPRR-r | <r> Partial Route Recording | 7.5.5 | ePRR-r:M | ePRR-r:M |
| eqCSR-s | <s> Complete Source Routing | 7.5.4 | eCSR:M | - |
| eqPSR-s | <s> Partial Source Routing | 7.5.4 | ePSR:M | - |
| eqQOSM-s | <s> QOS Maintenance | 7.5.6 | eQOSM-s or eCong-s:M | eQOSM-s:M |
| eqQOSM-r | <r> QOS Maintenance | 7.5.6 | eQOSM-r or eCong-r :M | eQOSM-r or eCong-r:M |
| eqPri-s | <s> Priority | 7.5.7 | ePri-s:M | ePri-s:M |
| eqPri-r | <r> Priority | 7.5.7 | ePri-r:M | ePri-r:M |
| eqData-s | <s> Data | 7.6 | M | M |
| eqData-r | <r> Data | 7.6 | M | M |
| eqUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.21 | M | M |
| eqUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

ePSR: Partial Source Routing

ePRR-r: <r> Partial Route Recording

ePRR-s: <s> Partial Route Recording

eCSR: Complete Source Routing

eCRR-r: <r> Complete Route Recording

eCRR-s: <s> Complete Route Recording

eQOSM-r: <r> QOS Maintenance

eQOSM-s: <s> QOS Maintenance

eSecu-r: <r> Security

eSecu-s: <s> Security

eCong-r: <r> Congestion Notification

eCong-s: <s> Congestion Notification

ePRI-r: <r> Priority

ePRI-s: <s> Priority

ePadd-s: <s> Padding

## APRL209: End Systems - Supported ERP Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | ES Status |
| --- | --- | --- | --- | --- |
| epFxPt-s | <s> Fixed Part | 7.2 | M | M |
| epFxPt-r | <r> Fixed Part | 7.2 | M | M |
| epAddr-s | <s> Address | 7.3 | M | M |
| epAddr-r | <r> Address | 7.3 | M | M |
| epSeg-s | <s> Segmentation Part | 7.4 | M | M |
| epSeg-r | <r> Segmentation Part | 7.4 | M | M |
| epPadd-s | <s> Padding | 7.5.2 | ePadd-s:M | - |
| epPadd-r | <r> Padding | 7.5.2 | M | M |
| epSecu-s | <s> Security | 7.5.3 | eSecu-s:M | eSecu-s:M |
| epSecu-r | <r> Security | 7.5.3 | eSecu-r:M | eSecu-r:M |
| epCRR-s | <s> Complete Route Recording | 7.5.5 | eCRR-s:M | - |
| epCRR-r | <r> Complete Route Recording | 7.5.5 | eCRR-r:M | eCRR-r:M |
| epPRR-s | <s> Partial Route Recording | 7.5.5 | ePRR-s:M | ePRR-s:M |
| epPRR-r | <r> Partial Route Recording | 7.5.5 | ePRRr-r:M | ePRR-r:M |
| epCSR-s | <s> Complete Source Routing | 7.5.4 | eCSR:M | - |
| epPSR-s | <s> Partial Source Routing | 7.5.4 | ePSR:M | - |
| epQOSM-s | <s> QOS Maintenance | 7.5.6 | eQOSM-s or eCong-s:M | eQOSM-s:M |
| epQOSM-r | <r> QOS Maintenance | 7.5.6 | eQOSM-r or eCong-r :M | eQOSM-r or eCong-r:M |
| epPri-s | <s> Priority | 7.5.7 | ePri-s:M | ePri-s:M |
| epPri-r | <r> Priority | 7.5.7 | ePri-r:M | ePri-r:M |
| epData-s | <s> Data | 7.6 | M | M |
| epData-r | <r> Data | 7.6 | M | M |
| epUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.21 | M | M |
| epUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M |

eCSR: Complete Source Routing

eCRR-r: <r> Complete Route Recording

eCRR-s: <s> Complete Route Recording

ePSR: Partial Source Routing

ePRR-r: <r> Partial Route Recording

ePRR-s: <s> Partial Route Recording

ePRI-r: <r> Priority

ePRI-s: <s> Priority

eQOSM-r: <r> QOS Maintenance

eQOSM-s: <s> QOS Maintenance

eCong-r: <r> Congestion Notification

eCong-s: <s> Congestion Notification

eSecu-r: <r> Security

eSecu-s: <s> Security

ePadd-s: <s> Padding

## APRL210: End Systems - Timers

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | ES Status |
| --- | --- | --- | --- | --- |
| ELifReas | Is reassembly timer <= received derived PDU lifetime? | 6.8 | M | M |

## APRL211: End Systems -Timers

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | ES Status |
| --- | --- | --- | --- | --- |
| eReasTim | Reassembly Timer | 6.8 | 500ms to 127.5s | <= lifetime |

## APRL201: Intermediate Systems - Support of ATN-Specific Recommendations

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| ATN CLNP1 | Encoding and use of the Security Parameter | 5.6.2.2 | M | M | M | M |
| ATN CLNP2 | Management of Network Priority | 5.6.2.3, 5.2.8.4 | M | M | M | M |
| ATN CLNP3 | Use of Non-Segmenting Subset | 5.6.3.1.3 | O | O | O | O |
| ATN CLNP4 | Echo Request Function | 5.6.3.3 | REC | REC | REC | REC |
| ATN CLNP5 | Congestion Management | 5.6.2.4 | M | M | M | M |

## APRL202: Intermediate Systems - Major Capabilities

| Item | Description | ISO/IEC 8473 Reference | ISO/IEC 8473 Status | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ES | End System |  | O.1 |  | O.1 | O.1 | O.1 | O.1 |
| IS | Intermediate System |  | O.1 |  | O.1 | O.1 | O.1 | O.1 |
| FL-r | <r> Full protocol | 8473-1: 6 | M |  | M | M | M | M |
| FL-s | <s> Full protocol | 8473-1: 6 | M |  | M | M | M | M |
| NSS-r | <r> Non-segmenting subset | 8473-1: 5.2 | M |  | M | M | M | M |
| NSS-s | <s> Non segmenting subset | 8473-1: 5.2 | IS:M ^IS:O |  | IS:M ^IS:O | IS:M ^IS:O | IS:M ^IS:O | IS:M ^IS:O |
| IAS-r | <r> Inactive subset | 8473-1: 5.2 | ES:O |  | ES:O | ES:O | ES:O | ES:O |
| IAS-s | <s> Inactive subset | 8473-1: 5.2 | IAS-r:M ^IAS-r:X |  | IAS-r:M ^IAS-r:X | IAS-r:M ^IAS-r:X | IAS-r:M ^IAS-r:X | IAS-r:M ^IAS-r:X |
| S802 | SNDCF for ISO 8802 | 8473-2: 5.4 | O.2 |  | O | O | O | O |
| SCLL | SNDCF for CL Link Service | 8473-4: 5.3.1 | O.2 |  | O | O | O | O |
| SCOL | SNDCF for CO Link Service | 8473-4: 5.3.2 | O.2 |  | O | O | O | O |
| SX25 | SNDCF for ISO 8208 | 8473-3: 5.4 | O.2 |  | O | O | O | O |
| ATN SNDCF | SNDCF for Mobile Subnetworks |  |  | 5.7 | ISMOB:M ISGRD:O ^IS:O | ISMOB:M ISGRD:O ^IS:O | ISMOB:M ISGRD:O ^IS:O | ISMOB:M ISGRD:O ^IS:O |

O.1: At least one of these options must be supported

O.2: At least one of these options must be supported

ES: End System

IS: Intermediate System

IAS-r: <r> Inactive subset

ISMOB: If ISO 8473 is used over ground subnetworks, then true, else false

ISGRD: If ISO 8473 is used over ground subnetworks, then true, else false

## APRL251: Intermediate Systems - Supported Functions

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| iPDUC | PDU Composition | 6.1 | M |  | M | M | M | M |
| iPDUD | PDU Decomposition | 6.2 | M |  | M | M | M | M |
| iHFA | Header Format Analysis | 6.3 | M |  | M | M | M | M |
| iPDUL | <s> PDU Lifetime Control | 6.4 | M |  | M | M | M | M |
| iRout | Route PDU | 6.5 | M |  | M | M | M | M |
| iForw | Forward PDU | 6.6 | M |  | M | M | M | M |
| iSegm | Segment PDU | 6.7 | iDSNS:M |  | iDSNS:M | iDSNS:M | iDSNS:M | iDSNS:M |
| iReas | Reassemble PDU | 6.8 | O |  | O | O | O | O |
| iDisc | Discard PDU | 6.9 | M |  | M | M | M | M |
| iErep | Error Reporting | 6.10 | M |  | M | M | M | M |
| iEdec | <s> Header Error Detection | 6.11 | M |  | M | M | M | M |
| iSecu | <s>Security | 6.13 | O | 5.6.2.2 | M | M | M | M |
| iCRR | <s> Complete Route Recording | 6.15 | O |  | OX | OX | OX | OX |
| iPRR | <s> Partial Route Recording | 6.15 | O |  | M | M | M | M |
| iCSR | Complete Source Routing | 6.14 | O |  | OX | OX | OX | OX |
| iPSR | Partial Source Routing | 6.14 | O |  | OX | OX | OX | OX |
| iPri | <s> Priority | 6.17 | O | 5.6.3.4 | M | M | M | M |
| iQOSM | <s> QOS Maintenance | 6.16 | O |  | M | M | M | M |
| iCong | <s> Congestion Notification | 6.18 | O | 5.6.2.4 | M | M | M | M |
| iPadd | <s> Padding | 6.12 | M |  | M | M | M | M |
| iEreq | Echo request | 6.19 | O | 5.6.3.3 | O | O | O | O |
| iErsp | Echo response | 6.20 | O |  | M | M | M | M |
| iSegS | Create segments smaller than necessary | 6.8 | O |  | O | O | O | O |
| iDSNS | Simultaneous support of subnetworks with different SN-User data sizes | 6.7 | O |  | O | O | O | O |

iDSNS: Simultaneous support of subnetworks with different SN-User data sizes

## APRL252: Intermediate Systems - Supported Security Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| iSADSSEC | Source Address Specific Security | 7.5.3.1 | iSecu:O.5 |  | iSecu:O | iSecu:O | iSecu:O | iSecu:O |
| iDADSSEC | Destination Address Specific Security | 7.5.3.2 | iSecu:O.5 |  | iSecu:O | iSecu:O | iSecu:O | iSecu:O |
| iGUNSEC | Globally Unique Security |  | iSecu:O.5 | 5.6.2.2 | iSecu:M | iSecu:M | iSecu:M | iSecu:M |

iSecu: <s>Security

O.5: At least one of these options must be supported

## APRL253: Intermediate Systems - Quality of Service Maintenance Function

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| iQOSNAVAIL | If requested QOS not available, deliver at different QOS | 6.16 | iQOSM:M | iQOSM:M | iQOSM:M | iQOSM:M | iQOSM:M |
| iQOSNOT | Notification of failure to meet requested QOS | 6.16 | iQOSM:O | iQOSM:O | iQOSM:O | iQOSM:O | iQOSM:O |

iQOSM: <s> QOS Maintenance

## APRL254: Intermediate Systems - Quality of Service Maintenance Function

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| iSADDQOS | Source Address Specific QoS | 7.5.6.1 | iQOSM:O.3 | iQOSM:O | iQOSM:O | iQOSM:O | iQOSM:O |
| iDADDQOS | Destination Address Specific QoS | 7.5.6.2 | iQOSM:O.3 | iQOSM:O | iQOSM:O | iQOSM:O | iQOSM:O |
| iGUNQOS | Globally Unique QoS | 7.5.6.3 | iQOSM:O.3 | iQOSM:M | iQOSM:M | iQOSM:M | iQOSM:M |
| iSvTD | Sequencing versus Transit Delay | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| iCongE | Congestion Experienced | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:M | iGUNQOS:M | iGUNQOS:M | iGUNQOS:M |
| iTDvCst | Transit Delay versus Cost | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| iREPvTD | Residual Error Probability versus Transit Delay | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 |
| iREPvCst | Residual Error Probality versus Cost | 7.5.6.3 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 | iGUNQOS:O.4 |

O.3: At least one of these options must be supported

O.4: At least one of these options must be supported

iGUNQOS: Globally Unique QoS

iQOSM: <s> QOS Maintenance

## APRL255: Intermediate Systems - Supported NPDUs

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| iDT-t | DT (full protocol) transmit | 7.7 | M | M | M | M | M |
| iDT-r | DT (full protocol) receive | 7.7 | M | M | M | M | M |
| iDTNS-t | DT (non-segment) transmit | 7.7 | M | M | M | M | M |
| iDTNS-r | DT (non-segment) receive | 7.7 | M | M | M | M | M |
| iER-t | ER transmit | 7.9 | M | M | M | M | M |
| iER-r | ER receive | 7.9 | M | M | M | M | M |
| iERQ-t | ERQ transmit | 7.10 | iEreq:M | O | O | O | O |
| iERQ-r | ERQ receive | 7.10 | M | M | M | M | M |
| iERP-t | ERP transmit | 7.11 | iErsp:M | O | O | O | O |
| iERP-r | ERP receive | 7.11 | M | M | M | M | M |

iEreq: Echo request

iErsp: Echo response

## APRL256: Intermediate Systems - Supported DT Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| idFxPt-s | <s> Fixed Part | 7.2 | M | M | M | M | M |
| idFxPt-r | <r> Fixed Part | 7.2 | M | M | M | M | M |
| idAddr-s | <s> Addresses | 7.3 | M | M | M | M | M |
| idAddr-r | <r> Addresses | 7.3 | M | M | M | M | M |
| idSeg-s | <s> Segmentation Part | 7.4 | M | M | M | M | M |
| idSeg-r | <r> Segmentation Part | 7.4 | M | M | M | M | M |
| idPadd-s | <s> Padding | 7.5.2 | M | M | M | M | M |
| idPadd-r | <r> Padding | 7.5.2 | M | M | M | M | M |
| idSecu-s | <s> Security | 7.5.3 | iSecu:M | iSecu:M | iSecu:M | iSecu:M | iSecu:M |
| idSecu-r | <r> Security | 7.5.3 | iSecu:M | iSecu:M | iSecu:M | iSecu:M | iSecu:M |
| idCRR-s | <s> Complete Route Recording | 7.5.5 | iCRR:M | M | M | M | M |
| idCRR-r | <r> Complete Route Recording | 7.5.5 | iCRR:M | - | - | - | - |
| idPRR-s | <s> Partial Route Recording | 7.5.5 | M | M | M | M | M |
| idPRR-r | <r> Partial Route Recording | 7.5.5 | iPRR:M | iPRR:M | iPRR:M | iPRR:M | iPRR:M |
| idCSR-s | <s> Complete Source Routing | 7.5.4 | iCSR:M | - | - | - | - |
| idCSR-r | <r> Complete Source Routing | 7.5.4 | iCSR:M | - | - | - | - |
| idPSR-s | <s> Partial Source Routing | 7.5.4 | M | M | M | M | M |
| idPSR-r | <r> Partial Source Routing | 7.5.4 | iPSR:M | - | - | - | - |
| idQOSM-s | <s> QOS Maintenance | 7.5.6 | M | M | M | M | M |
| idQOSM-r | <r> QOS Maintenance | 7.5.6 | iQOSM or iCong:M | iQOSM or iCong:M | iQOSM or iCong:M | iQOSM or iCong:M | iQOSM or iCong:M |
| idPri-s | <s> Priority | 7.5.7 | M | M | M | M | M |
| idPri-r | <r> Priority | 7.5.7 | iPRi:M | iPri:M | iPri:M | iPri:M | iPri:M |
| idData-s | <s> Data | 7.6 | M | M | M | M | M |
| idData-r | <r> Data | 7.6 | M | M | M | M | M |
| idUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.19 | M | M | M | M | M |
| idUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.19 | M | M | M | M | M |

iCSR: Complete Source Routing

iCRR: <s> Complete Route Recording

iPSR: Partial Source Routing

iPRR: <s> Partial Route Recording

iSecu: <s>Security

iQOSM: <s> QOS Maintenance

iPri: <s> Priority

iCong: <s> Congestion Notification

## APRL257: Intermediate Systems - Supported ER Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ieFxPt-s | <s> Fixed Part | 7.2 | M | M | M | M | M |
| ieFxPt-r | <r> Fixed Part | 7.2 | M | M | M | M | M |
| ieAddr-s | <s> Address | 7.3 | M | M | M | M | M |
| ieAddr-r | <r> Address | 7.3 | M | M | M | M | M |
| ieSeg-s | <s> Seg Part | 7.4 | M | M | M | M | M |
| ieSeg-r | <r> Seg Part | 7.4 | M | M | M | M | M |
| iePadd-s | <s> Padding | 7.5.2 | M | M | M | M | M |
| iePadd-r | <r> Padding | 7.5.2 | M | M | M | M | M |
| ieSecu-s | <s> Security | 7.5.3 | iSecu:M | iSecu:M | iSecu:M | iSecu:M | iSecu:M |
| ieSecu-r | <r> Security | 7.5.3 | iSecu:M | iSecu:M | iSecu:M | iSecu:M | iSecu:M |
| ieCRR-s | <s> Complete Route Recording | 7.5.5 | M | M | M | M | M |
| ieCRR-r | <r> Complete Route Recording | 7.5.5 | iCRR:M | - | - | - | - |
| iePRR-s | <s> Partial Route Recording | 7.5.5 | M | M | M | M | M |
| iePRR-r | <r> Partial Route Recording | 7.5.5 | iPRR:M | iPRR:M | iPRR:M | iPRR:M | iPRR:M |
| ieCSR-s | <s> Complete Source Routing | 7.5.4 | iCSR:M | - | - | - | - |
| ieCSR-r | <r> Complete Source Routing | 7.5.4 | iCSR:M | - | - | - | - |
| iePSR-s | <s> Partial Source Routing | 7.5.4 | M | M | M | M | M |
| iePSR-r | <r> Partial Source Routing | 7.5.4 | iPSR:M | - | - | - | - |
| ieQOSM-s | <s> QOS Maintenance | 7.5.6 | M | M | M | M | M |
| ieQOSM-r | <r> QOS Maintenance | 7.5.6 | iQOSM or iCong:M | iQOSM or iCong:M | iQOSM or iCong:M | iQOSM or iCong:M | iQOSM or iCong:M |
| iePri-s | <s> Priority | 7.5.7 | M | M | M | M | M |
| iePri-r | <r> Priority | 7.5.7 | iPri:M | iPri:M | iPri:M | iPri:M | iPri:M |
| ieDisc-s | <s> Reason for Discard | 7.9.5 | M | M | M | M | M |
| ieDisc-r | <r> Reason for Discard | 7.9.5 | M | M | M | M | M |
| ieData-s | <s> Data | 7.6 | M | M | M | M | M |
| ieData-r | <r> Data | 7.6 | M | M | M | M | M |
| ieUnsup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded ? | 6.21 | M | M | M | M | M |
| ieUnsup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.21 | M | M | M | M | M |

iCSR: Complete Source Routing

iCRR: <s> Complete Route Recording

iPSR: Partial Source Routing

iPRR: <s> Partial Route Recording

iPri: <s> Priority

iCong: <s> Congestion Notification

iQOSM: <s> QOS Maintenance

iSecu: <s>Security

## APRL258: Intermediate Systems - Supported ERQ Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| iqFxPt-s | <s> Fixed Part | 7.2 | M | M | M | M | M |
| iqFxPt-r | <r> Fixed Part | 7.2 | M | M | M | M | M |
| iqAddr-s | <s> Addresses | 7.3 | M | M | M | M | M |
| iqAddr-r | <r> Addresses | 7.3 | M | M | M | M | M |
| iqSeg-s | <s> Segmentation Part | 7.4 | M | M | M | M | M |
| iqSeg-r | <r> Segmentation Part | 7.4 | M | M | M | M | M |
| iqPadd-s | <s> Padding | 7.5.2 | M | M | M | M | M |
| iqPadd-r | <r> Padding | 7.5.2 | M | M | M | M | M |
| iqSecu-s | <s> Security | 7.5.3 | iSecu:M | iSecu:M | iSecu:M | iSecu:M | iSecu:M |
| iqSecu-r | <r> Security | 7.5.3 | iSecu:M | iSecu:M | iSecu:M | iSecu:M | iSecu:M |
| iqCRR-s | <s> Complete Route Recording | 7.5.5 | iCRR:M | M | M | M | M |
| iqCRR-r | <r> Complete Route Recording | 7.5.5 | iCRR:M | - | - | - | - |
| iqPRR-s | <s> Partial Route Recording | 7.5.5 | M | M | M | M | M |
| iqPRR-r | <r> Partial Route Recording | 7.5.5 | iPRR:M | iPRR:M | iPRR:M | iPRR:M | iPRR:M |
| iqCSR-s | <s> Complete Source Routing | 7.5.4 | iCSR:M | - | - | - | - |
| iqCSR-r | <r> Complete Source Routing | 7.5.4 | iCSR:M | - | - | - | - |
| iqPSR-s | <s> Partial Source Routing | 7.5.4 | M | M | M | M | M |
| iqPSR-r | <r> Partial Source Routing | 7.5.4 | iPSR:M | - | - | - | - |
| iqQOSM-s | <s> QOS Maintenance | 7.5.6 | M | M | M | M | M |
| iqQOSM-r | <r> QOS Maintenance | 7.5.6 | iQOSM or iCong:M | iQOSM or ICong:M | iQOSM or ICong:M | iQOSM or ICong:M | iQOSM or ICong:M |
| iqPri-s | <s> Priority | 7.5.7 | M | M | M | M | M |
| iqPri-r | <r> Priority | 7.5.7 | iPRi:M | iPri:M | iPri:M | iPri:M | iPri:M |
| iqData-s | <s> Data | 7.6 | M | M | M | M | M |
| iqData-r | <r> Data | 7.6 | M | M | M | M | M |
| iqUnSup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.19 | M | M | M | M | M |
| iqUnSup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.19 | M | M | M | M | M |

iCSR: Complete Source Routing

iCRR: <s> Complete Route Recording

iPSR: Partial Source Routing

iPRR: <s> Partial Route Recording

iQOSM: <s> QOS Maintenance

iCong: <s> Congestion Notification

iSecu: <s>Security

iPri: <s> Priority

## APRL259: Intermediate Systems - Supported ERP Parameters

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ipFxPt-s | <s> Fixed Part | 7.2 | M | M | M | M | M |
| ipFxPt-r | <r> Fixed Part | 7.2 | M | M | M | M | M |
| ipAddr-s | <s> Addresses | 7.3 | M | M | M | M | M |
| ipAddr-r | <r> Addresses | 7.3 | M | M | M | M | M |
| ipSeg-s | <s> Segmentation Part | 7.4 | M | M | M | M | M |
| ipSeg-r | <r> Segmentation Part | 7.4 | M | M | M | M | M |
| ipPadd-s | <s> Padding | 7.5.2 | M | M | M | M | M |
| ipPadd-r | <r> Padding | 7.5.2 | M | M | M | M | M |
| ipSecu-s | <s> Security | 7.5.3 | iSecu:M | iSecu:M | iSecu:M | iSecu:M | iSecu:M |
| ipSecu-r | <r> Security | 7.5.3 | iSecu:M | iSecu:M | iSecu:M | iSecu:M | iSecu:M |
| ipCRR-s | <s> Complete Route Recording | 7.5.5 | iCRR:M | M | M | M | M |
| ipCRR-r | <r> Complete Route Recording | 7.5.5 | iCRR:M | - | - | - | - |
| ipPRR-s | <s> Partial Route Recording | 7.5.5 | M | M | M | M | M |
| ipPRR-r | <r> Partial Route Recording | 7.5.5 | iPRR:M | iPRR:M | iPRR:M | iPRR:M | iPRR:M |
| ipCSR-s | <s> Complete Source Routing | 7.5.4 | iCSR:M | - | - | - | - |
| ipCSR-r | <r> Complete Source Routing | 7.5.4 | iCSR:M | - | - | - | - |
| ipPSR-s | <s> Partial Source Routing | 7.5.4 | M | M | M | M | M |
| ipPSR-r | <r> Partial Source Routing | 7.5.4 | iPSR:M | - | - | - | - |
| ipQOSM-s | <s> QOS Maintenance | 7.5.6 | M | M | M | M | M |
| ipQOSM-r | <r> QOS Maintenance | 7.5.6 | iQOSM or iCong:M | iQOSM or ICong:M | iQOSM or ICong:M | iQOSM or ICong:M | iQOSM or ICong:M |
| ipPri-s | <s> Priority | 7.5.7 | M | M | M | M | M |
| ipPri-r | <r> Priority | 7.5.7 | iPRi:M | iPri:M | iPri:M | iPri:M | iPri:M |
| ipData-s | <s> Data | 7.6 | M | M | M | M | M |
| ipData-r | <r> Data | 7.6 | M | M | M | M | M |
| ipUnsup2 | Are received PDUs containing parameters selecting unsupported type 2 functions discarded and where appropriate an Error Report PDU generated ? | 6.19 | M | M | M | M | M |
| ipUnsup3 | Are parameters selecting unsupported Type 3 functions ignored ? | 6.19 | M | M | M | M | M |

iCSR: Complete Source Routing

iCRR: <s> Complete Route Recording

iPSR: Partial Source Routing

iPRR: <s> Partial Route Recording

iCong: <s> Congestion Notification

iQOSM: <s> QOS Maintenance

iPri: <s> Priority

iSecu: <s>Security

## APRL260: Intermediate Systems - Timer & Parameter Values

| Item | Description | ISO/IEC 8473-1 Reference | ISO/IEC 8473-1 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| iReasTim | Reassembly Timer | 6.8 | iReas:M | M | M | M | M |

iReas: Reassemble PDU

# Subnetworks & Subnetwork Convergence

## ARL070: End Systems - Subnetwork Service & Protocol

| Item | Description | SARPs Reference | ES Status |
| --- | --- | --- | --- |
| SNDCF1 | Service provided by the SNDCF | 5.7.2 | M |
| SNDCF2 | SNDCF for ISO/IEC 8802-2 Broadcast Subnetworks | 5.7.3 | C1:M |
| SNDCF3 | SNDCF for the Common ICAO Data Interchange Network (CIDIN) | 5.7.4 | C2:M |
| SNDCF4 | SNDCF for ISO/IEC 8208 General Topology Subnetworks | 5.7.5 | C3:M |
| SNDCF5 | SNDCF for ISO/IEC 8208 Mobile Subnetworks | 5.7.6 | C3:O |
| SNDCF6 | Implementations using the Mobile SNDCF as specified in 5.7.6 and the LREF Compression Procedure for Ground/Ground communications, should also use the LREF optional local reference cancellation mechanism | 5.7.5.2 | - |
| SNDCF7 | Implementations using this SNDCF for Air/Ground communications should only implement the LREF optional facility for local reference cancellation when the lifetime of the virtual circuits is of the same order as the flight time | 5.7.6.1.3 | - |

C1: Class 1

C2: Class 2

C3: Class 3

## APRL261: End Systems - SNDCF for use with ISO 8802-2 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-2 Reference | ISO/IEC 8473-2 Status | ES Status |
| --- | --- | --- | --- | --- |
| S802SNUD | Is subnetwork user data of at least 512 octets transferred transparently by the SNDCF ? | 5.2 | M | M |
| S802SNTD | Is Transit Delay determined by the SNDCF prior to the processing of User Data ? | 5.2 | M | M |

## APRL262: End Systems - SNDCF for use with ISO 8802-2 Subnetworks - Multi-layer Dependencies

| Item | Description | ISO/IEC 8473-2 Reference | ISO/IEC 8473-2 Status | ES Status |
| --- | --- | --- | --- | --- |
| S802SSg-r | <r> Maximum SN data unit size (RX) | 5.2 |  | >=512 |
| S802SSg-s | <s> Maximum SN data unit size (TX) | 5.2 |  | >=512 |

## APRL263: End Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | ES Status |
| --- | --- | --- | --- | --- |
| XSNUD | Is Subnetwork User Data of at least 512 octets transferred transparently by the SNDCF ? | 5.2 | M | M |
| XSNTD | Is Transit Delay determined by the SNDCF prior to the processing of user data ? | 5.2 | M | M |

## APRL264: End Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | ES Status |
| --- | --- | --- | --- | --- |
| XCalla | a. when no suitable call exists ? | 5.3.1 a. | O.3 | O.3 |
| XCallb | b. when queue threshold reached ? | 5.3.1 b. | O.3 | O.3 |
| XCallc | c. by systems management ? | 5.3.1 c. | O.3 | O.3 |
| XCalld | d. when queue threshold reached and timer expires ? | 5.3.4 | O.3 | O.3 |
| XCalle | e. by other local means ? | 5.3.1 | O.3 | O.3 |

O.3: At least one of these options must be supported

## APRL265: End Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | ES Status |
| --- | --- | --- | --- | --- |
| XClra | a. when idle timer expires | 5.3.2 a., 5.3.4 | O | O |
| XClrb | b. when need to re-use circuit | 5.3.2 b. | O | O |
| XClrc | c. by systems management | 5.3.2 c. | O | O |
| XClrd | d. by provider ? | 5.3.2 d. | M | M |
| XClrer | e. by other local means ? | 5.3.2 | O | O |

## APRL266: End Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | ES Status |
| --- | --- | --- | --- | --- |
| XPD | X.25 Protocol Discrimination | 5.3.3 | M | M |
| XVCC | Resolution of VC collisions | 5.3.5 | M | M |
| XMCR | Multiple VCs responding | 5.3.6 | M | M |
| XMCI | Multiple VCs initiating | 5.3.6 | O | O |
| Xpri | X.25 Priority procedure | 5.3.7 | O | M |

## APRL267: End Systems - SNDCF for use with ISO 8208 Subnetworks - X.25 Call User Data

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | ES Status |
| --- | --- | --- | --- | --- |
| PD-s | <s> Protocol Discriminator | 5.3.3 | M | M |
| PD-r | <r> Protocol Discriminator | 5.3.3 | M | M |
| LI-s | <s> Length Indication | 5.3.6 | XMCI:M | XMCI:M |
| LI-r | <r> Length Indication | 5.3.6 | M | M |
| Ver-s | <s> SNCR Version | 5.3.6 | XMCI:M | XMCI:M |
| Ver-r | <r> SNCR Version | 5.3.6 | M | M |
| SNCR-s | <s> SNCR Value | 5.3.6 | XMCI:M | XMCI:M |
| SNCR-r | <r> SNCR Value | 5.3.6 | M | M |

XMCI: Multiple VCs initiating

## APRL268: End Systems - SNDCF for use with ISO 8208 Subnetworks - ISO 8208 SNDCF Timers

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | ES Status |
| --- | --- | --- | --- | --- |
| XIDL | X25 VC Idle | 5.3.4 | XClra:O | XClra:O |
| XNVC | additional VC | 5.3.4 | O | M |

XClra: a. when idle timer expires

## APRL269: End Systems - SNDCF for use with ISO 8208 Subnetworks - SNDCF Multi-layer Dependencies

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | ES Status |
| --- | --- | --- | --- | --- |
| XSSg-r | <r> Maximum SN data unit size (Rx) | 5.2 | >=512 | >=512 |
| XSSg-s | <s> Maximum SN data unit size (Tx) | 5.2 | >=512 | >=512 |

## APRL270: End Systems - SNDCF for use with ISO 8208 Subnetworks - SNDCF Multi-layer Dependencies

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | ES Status |
| --- | --- | --- | --- | --- |
| Xvc | X.25 Virtual call service | 5.3.8 | M | M |
| Xdt | X.25 Data transfer | 5.3.8 | M | M |
| Xfc | X.25 flow control procedures | 5.3.8 | M | M |
| Xfrp | X.25 flow control + reset packets | 5.3.8 | M | M |
| Xccp | X.25 call setup and clear packets | 5.3.8 | M | M |
| Xdp | X.25 DTE and DCE data packets | 5.3.8 | M | M |
| Xrs | X.25 restart procedures | 5.3.8 | M | M |
| XDct | X.25 DCE timeouts | 5.3.8 | M | M |
| XDtT | X.25 time limits | 5.3.8 | M | M |
| Xpco | X.25 network packet coding | 5.3.8 | M | M |
| Xfcn | X.25 flow control parameter negotiation | 5.3.8 | O | O |
| Xtd | X.25 transit delay selection and negotiation | 5.3.8 | O | O |
| Xtc | X.25 throughput class negotiation | 5.3.8 | O | O |
| Xoth | Other X.25 elements | 5.3.8 | O | O |

## ARL070: Intermediate Systems - Subnetwork Service & Protocol

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| SNDCF1 | Service provided by the SNDCF | 5.7.2 | M | M | M | M |
| SNDCF2 | SNDCF for ISO/IEC 8802-2 Broadcast Subnetworks | 5.7.3 | C1:M | C1:M | C1:M | C1:M |
| SNDCF3 | SNDCF for the Common ICAO Data Interchange Network (CIDIN) | 5.7.4 | C2:M | C2:M | C2:M | - |
| SNDCF4 | SNDCF for ISO/IEC 8208 General Topology Subnetworks | 5.7.5 | C3:M | C3:M | C3:M | C3:M |
| SNDCF5 | SNDCF for ISO/IEC 8208 Mobile Subnetworks | 5.7.6 | C3:O | C3:O | M | M |
| SNDCF6 | Implementations using the Mobile SNDCF as specified in 5.7.6 and the LREF Compression Procedure for Ground/Ground communications, should also use the LREF optional local reference cancellation mechanism | 5.7.5.2 | - | SNDCF5:REC | SNDCF5:REC | - |
| SNDCF7 | Implementations using this SNDCF for Air/Ground communications should only implement the LREF optional facility for local reference cancellation when the lifetime of the virtual circuits is of the same order as the flight time | 5.7.6.1.3 | - | - | REC | REC |

C1: Class 1

C2: Class 2

C3: Class 3

## APRL261: Intermediate Systems - SNDCF for use with ISO 8802-2 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-2 Reference | ISO/IEC 8473-2 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S802SNUD | Is subnetwork user data of at least 512 octets transferred transparently by the SNDCF ? | 5.2 | M | M | M | M | M |
| S802SNTD | Is Transit Delay determined by the SNDCF prior to the processing of User Data ? | 5.2 | M | M | M | M | M |

## APRL262: Intermediate Systems - SNDCF for use with ISO 8802-2 Subnetworks - Multi-layer Dependencies

| Item | Description | ISO/IEC 8473-2 Reference | ISO/IEC 8473-2 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S802SSg-r | <r> Maximum SN data unit size (RX) | 5.2 |  | >=512 | >=512 | >=512 | >=512 |
| S802SSg-s | <s> Maximum SN data unit size (TX) | 5.2 |  | >=512 | >=512 | >=512 | >=512 |

## APRL263: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XSNUD | Is Subnetwork User Data of at least 512 octets transferred transparently by the SNDCF ? | 5.2 | M | M | M | M | M |
| XSNTD | Is Transit Delay determined by the SNDCF prior to the processing of user data ? | 5.2 | M | M | M | M | M |

## APRL264: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XCalla | a. when no suitable call exists ? | 5.3.1 a. | O.3 | O.3 | O.3 | O.3 | O.3 |
| XCallb | b. when queue threshold reached ? | 5.3.1 b. | O.3 | O.3 | O.3 | O.3 | O.3 |
| XCallc | c. by systems management ? | 5.3.1 c. | O.3 | O.3 | O.3 | O.3 | O.3 |
| XCalld | d. when queue threshold reached and timer expires ? | 5.3.4 | O.3 | O.3 | O.3 | O.3 | O.3 |
| XCalle | e. by other local means ? | 5.3.1 | O.3 | O.3 | O.3 | O.3 | O.3 |

O.3: At least one of these options must be supported

## APRL265: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XClra | a. when idle timer expires | 5.3.2 a., 5.3.4 | O | O | O | O | O |
| XClrb | b. when need to re-use circuit | 5.3.2 b. | O | O | O | O | O |
| XClrc | c. by systems management | 5.3.2 c. | O | O | O | O | O |
| XClrd | d. by provider ? | 5.3.2 d. | M | M | M | M | M |
| XClrer | e. by other local means ? | 5.3.2 | O | O | O | O | O |

## APRL266: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - Functions

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XPD | X.25 Protocol Discrimination | 5.3.3 | M | M | M | M | M |
| XVCC | Resolution of VC collisions | 5.3.5 | M | M | M | M | M |
| XMCR | Multiple VCs responding | 5.3.6 | M | M | M | M | M |
| XMCI | Multiple VCs initiating | 5.3.6 | O | O | O | O | O |
| Xpri | X.25 Priority procedure | 5.3.7 | O | M | M | M | M |

## APRL267: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - X.25 Call User Data

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PD-s | <s> Protocol Discriminator | 5.3.3 | M | M | M | M | M |
| PD-r | <r> Protocol Discriminator | 5.3.3 | M | M | M | M | M |
| LI-s | <s> Length Indication | 5.3.6 | XMCI:M | XMCI:M | XMCI:M | XMCI:M | XMCI:M |
| LI-r | <r> Length Indication | 5.3.6 | M | M | M | M | M |
| Ver-s | <s> SNCR Version | 5.3.6 | XMCI:M | XMCI:M | XMCI:M | XMCI:M | XMCI:M |
| Ver-r | <r> SNCR Version | 5.3.6 | M | M | M | M | M |
| SNCR-s | <s> SNCR Value | 5.3.6 | XMCI:M | XMCI:M | XMCI:M | XMCI:M | XMCI:M |
| SNCR-r | <r> SNCR Value | 5.3.6 | M | M | M | M | M |

XMCI: Multiple VCs initiating

## APRL268: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - ISO 8208 SNDCF Timers

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XIDL | X25 VC Idle | 5.3.4 | XClra:O | XClra:O | XClra:O | XClra:O | XClra:O |
| XNVC | additional VC | 5.3.4 | O | M | M | M | M |

XClra: a. when idle timer expires

## APRL269: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - SNDCF Multi-layer Dependencies

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XSSg-r | <r> Maximum SN data unit size (Rx) | 5.2 | >=512 | >=512 | >=512 | >=512 | >=512 |
| XSSg-s | <s> Maximum SN data unit size (Tx) | 5.2 | >=512 | >=512 | >=512 | >=512 | >=512 |

## APRL270: Intermediate Systems - SNDCF for use with ISO 8208 Subnetworks - SNDCF Multi-layer Dependencies

| Item | Description | ISO/IEC 8473-3 Reference | ISO/IEC 8473-3 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xvc | X.25 Virtual call service | 5.3.8 | M | M | M | M | M |
| Xdt | X.25 Data transfer | 5.3.8 | M | M | M | M | M |
| Xfc | X.25 flow control procedures | 5.3.8 | M | M | M | M | M |
| Xfrp | X.25 flow control + reset packets | 5.3.8 | M | M | M | M | M |
| Xccp | X.25 call setup and clear packets | 5.3.8 | M | M | M | M | M |
| Xdp | X.25 DTE and DCE data packets | 5.3.8 | M | M | M | M | M |
| Xrs | X.25 restart procedures | 5.3.8 | M | M | M | M | M |
| XDct | X.25 DCE timeouts | 5.3.8 | M | M | M | M | M |
| XDtT | X.25 time limits | 5.3.8 | M | M | M | M | M |
| Xpco | X.25 network packet coding | 5.3.8 | M | M | M | M | M |
| Xfcn | X.25 flow control parameter negotiation | 5.3.8 | O | O | O | O | O |
| Xtd | X.25 transit delay selection and negotiation | 5.3.8 | O | O | O | O | O |
| Xtc | X.25 throughput class negotiation | 5.3.8 | O | O | O | O | O |
| Xoth | Other X.25 elements | 5.3.8 | O | O | O | O | O |

## APRL501: Intermediate Systems - Major Capabilities

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| mcNego | Negotiation of Compression Algorithm | 5.7.6.2 |  |  | M | M |
| mcLocRef | Local Reference Header Compression | 5.7.6.3 |  |  | M | M |
| mcCan | Local Reference Cancellation | 5.7.6.3.6 |  |  | O | O |
| McM/I | Local Reference directory maintenance | 5.7.6.3 |  |  | Snvdl:M ^Snvdl:O | Snvdl:M ^Snvdl:O |
| mcACA | ICAO Address Compression Algorithm (ACA) | 5.7.6.4 |  |  | O | O |
| mcV42 | V.42bis Compression | 5.7.6.2 |  |  | O | O |

Snvdl: If VDL SNDCF supported then true else false

## APRL502: Intermediate Systems - Call Setup & Clearing Procedures

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| csDynam | Dynamic Call Setup | 5.7.6.2.1.1.1 |  |  | M | M |
| csSys | Call Setup by Systems Management | 5.7.6.2.1.1 |  |  | O | O |
| csDef | Use of non-standard Default packet size | 5.7.6.2.1.3 |  |  | M | M |
| csFast | Use of Fast Select | 5.7.6.2.1.4 |  |  | M | M |
| csM/I | Local Reference directory maintenance request/acceptance | 5.7.6.2.1.5.13 5.7.6.2.2.2.3 |  |  | ^csFast: - McM/I:M | ^csFast: - McM/I:M |
| csOther | Use of other optional User Facilities and CCITT-specified DTE facilities | 5.7.6.2.1.1.3 |  |  | O | O |
| csCol | Call Collision Resolution | 5.7.6.2.2.1.2 |  |  | M | M |
| csAcp | Call Acceptance Procedures | 5.7.6.2.1.6 |  |  | M | M |
| csRej | Call rejection Procedures | 5.7.6.2.1.7 |  |  | M | M |
| csOrd | Order of compression Procedures | 5.7.6.2.3.2 |  |  | M | M |
| csDiag | Use of call rejection diagnostic codes | 5.7.6.2.1.7.3 |  |  | M | M |
| csReset | Call Clearing Procedures | 5.7.6.2.4 |  |  | M | M |

McM/I: Local Reference directory maintenance

csFast: Use of Fast Select

## APRL503: Intermediate Systems - Negotiation of Compression Algorithm

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| caMaxd | Indication of the maximum of directories entries in the call user Data | 5.7.6.2.1.5.11 |  |  | mcNego:O | mcNego:O |

mcNego: Negotiation of Compression Algorithm

## APRL504: Intermediate Systems - Local Reference Header Compression

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| lrVC | Opening additional virtual circuits | 5.7.6.3.2.1.2 |  |  | M | M |
| lrDirSize | Local Directory with more than 128 entries | 5.7.6.3.1 |  |  | O | O |
| lrProt | Identification of Network Layer Protocol | 5.7.6.3.2.2 |  |  | M | M |
| lrMod | Processing of SN-UnitData Requests | 5.7.6.3.2 |  |  | M | M |
| lrEst | Establishment of new local reference | 5.7.6.3.2.4 |  |  | M | M |
| lrTransfer | Transfer of modified ISO 8473 PDU | 5.7.6.3.2.6 |  |  | M | M |
| lrInitial | Initial DT PDU Compression | 5.7.6.3.3.2 |  |  | M | M |
| lrDerived | Derived DT PDU Compression | 5.7.6.3.3.3 |  |  | M | M |
| lrError-s | Generation of Error PDU Compression | 5.7.6.3.3.3 |  |  | M | M |
| lrDiscard | Compression of discarded PDU encapsulated within Error PDU | 5.7.6.3.3.4 |  |  | lrError-s:M | lrError-s:M |
| lrCompTr | Transfer of compressed PDUs | 5.7.6.3.3.4.11 |  |  | M | M |
| lrReceived | Processing of received PDUs | 5.7.6.3.4 |  |  | M | M |
| lrUncomp-r | Processing of received uncompressed PDUs | 5.7.6.3.4.2 |  |  | M | M |
| LrReset | Purging directories entries on Reset | 5.7.6.3.7 |  |  | mcLocRef:M | mcLocRef:M |
| lrUnMod-r | Processing of received unmodified PDUs | 5.7.6.3.4.2.2 |  |  | M | M |
| lrComp-r | Processing of received compressed data PDUs | 5.7.6.3.4.3 |  |  | M | M |
| lrError-r | Processing of received compressed Error PDUs | 5.7.6.3.4.4 |  |  | M | M |
| lrSNDCFerr-s | Generation of SNDCF Error Report | 5.7.6.3.5 |  |  | M | M |
| lrSNDCFerr-r | Processing of received SNDCF Error Report | 5.7.6.3.4.5 |  |  | M | M |

mcLocRef: Local Reference Header Compression

## APRL505: Intermediate Systems - Local Reference Cancellation

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| lrcMgmt | Management of local references | 5.7.6.3.2.5 |  |  | mcCan:M | mcCan:M |
| lrcRequest-s | Generation of Cancellation Request PDU | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |
| lrcRequest-r | Processing of incoming Cancellation Request PDU | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |
| lrcReliable | Reliable transfer of Cancellation Request | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |
| lrcAccept-s | Generation of Cancellation Accept PDU | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |
| lrcAccept-r | Processing of incoming Cancellation Accept PDU | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |

mcCan: Local Reference Cancellation

## APRL506: Intermediate Systems - ICAO Address Compression Algorithm

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| acOut | Compression of outgoing PDUs | 5.7.6.4.1 |  |  | mcACA:M | mcACA:M |
| acIn | Decompression of incoming PDUs | 5.7.6.4.1 |  |  | mcACA:M | mcACA:M |
| acAddr | Address Length Determination | 5.7.6.4.2 |  |  | mcACA:M | mcACA:M |
| acComp | Compression of NSAP Addresses and address prefixes | 5.7.6.4.5 |  |  | mcACA:M | mcACA:M |
| acDecomp | Decompression of NSAP Addresses and address prefixes | 5.7.6.4.6 |  |  | mcACA:M | mcACA:M |

mcACA: ICAO Address Compression Algorithm (ACA)

## APRL507: Intermediate Systems - PDU Formats - Call Request User Data

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| crLen | Length Indicator | 5.7.6.2.1.5.3 |  |  | M | M |
| crVersion | Version Indicator | 5.7.6.2.1.5.4 |  |  | M | M |
| crSNCR | Subnetwork Connection Reference (SNCR) | 5.7.6.2.1.5.5 |  |  | M | M |
| crComp | Offerred Compression Techniques | 5.7.6.2.1.5.7 |  |  | M | M |
| crDir | Maximum Directory Size | 5.7.6.2.1.5.11 |  |  | M | M |
| crAdd-s | Additional User Data on send | 5.7.6.2.1.5.12 |  |  | O | O |
| crAdd-r | Additional User Data on receive | 5.7.6.2.1.5.12 |  |  | O | O |
| MaxDir | Maximum number of directory entries supported | 5.7.6.2.1.5.7 |  |  | >=128 | >=128 |

## APRL508: Intermediate Systems - PDU Formats - Call Accept User Data

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| caComp | Offerred Compression Techniques | 5.7.6.2.2.4.3 |  |  | mcNego:M | mcNego:M |
| caAdd-s | Additional User Data on send | 5.7.6.2.2.4.4 |  |  | mcNego:O | mcNego:O |
| caAdd-r | Additional User Data on receive | 5.7.6.2.2.4.4 |  |  | mcNego:O | mcNego:O |

mcNego: Negotiation of Compression Algorithm

## APRL509: Intermediate Systems - PDU Formats - Modified IDO/IEC 8473 NPDU

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| npLocRef-s | Local Reference Option field | 5.7.6.3.2.3 |  |  | M | M |

## APRL510: Intermediate Systems - PDU Formats - Compressed Initial PDU

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| inType | PDU Type | 5.7.6.3.3.2.2 |  |  | M | M |
| inPri | Priority | 5.7.6.3.3.2.3 |  |  | M | M |
| inLifetime | Lifetime | 5.7.6.3.3.2.4 |  |  | M | M |
| inFlags | Flag Bits | 5.7.6.3.3.2.5 to 5.7.6.3.3.2.9 |  |  | M | M |
| inLocRef | Local Reference (1 octet) | 5.7.6.3.3.2.9 |  |  | M | M |
| inLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.9 |  |  | lrDirSize:M ^lrDirsize:X | lrDirSize:M ^lrDirsize:X |
| inPDUId | PDU Identifier | 5.7.6.3.3.2.10 |  |  | M | M |
| inNSDU | User Data | Figure 5.7-4 |  |  | M | M |

lrDirSize: Local Directory with more than 128 entries

## APRL511: Intermediate Systems - PDU Formats - Compressed Derived PDU

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| drType | PDU Type | 5.7.6.3.3.3.2 |  |  | M | M |
| drPri | Priority | 5.7.6.3.3.3.3 |  |  | M | M |
| drLifetime | Lifetime | 5.7.6.3.3.3.4 |  |  | M | M |
| drFlags | Flag Bits | 5.7.6.3.3.3.5 to 5.7.6.3.3.3.8 |  |  | M | M |
| drLocRef | Local Reference (1 octet) | 5.7.6.3.3.2.8 |  |  | M | M |
| drLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.8 |  |  | lrDirSize:M ^lrDirsize:X | lrDirSize:M ^lrDirsize:X |
| drPDUId | PDU Identifier | 5.7.6.3.3.3.9 |  |  | M | M |
| drSegOff | Segment Offset | 5.7.6.3.3.3.10 |  |  | M | M |
| drTotalLen | Total Length | 5.7.6.3.3.3.11 |  |  | M | M |
| drNSDU | User Data | Figure 5.7-4 |  |  | M | M |

lrDirSize: Local Directory with more than 128 entries

## APRL512: Intermediate Systems - PDU Formats - Compressed Error PDU

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| erType | PDU Type | 5.7.6.3.3.4.2 |  |  | M | M |
| erPri | Priority | 5.7.6.3.3.4.3 |  |  | M | M |
| erLifetime | Lifetime | 5.7.6.3.3.4.4 |  |  | M | M |
| erFlags | Flag Bits | 5.7.6.3.3.4.5 to 5.7.6.3.3.4.8 |  |  | M | M |
| erLocRef | Local Reference (1 octet) | 5.7.6.3.3.2.8 |  |  | M | M |
| erLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.8 |  |  | lrDirSize:M ^lrDirsize:X | lrDirSize:M ^lrDirsize:X |
| erReason | Discard Reason | 5.7.6.3.3.4.9 |  |  | M | M |
| erNSDU | Compressed Header of discarded PDU | 5.7.6.3.3.4 |  |  | M | M |

lrDirSize: Local Directory with more than 128 entries

## APRL513: Intermediate Systems - PDU Formats - SNDCF Error Report PDU

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| sfType | PDU Type | 5.7.6.3.5 |  |  | M | M |
| sfReason | Discard Reason | 5.7.6.3.5 |  |  | M | M |
| sfLocRef | Local Reference | 5.7.6.3.5 |  |  | M | M |
| sfLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.9 |  |  | lrDirSize:M ^lrDirsize:X | lrDirSize:M ^lrDirsize:X |

lrDirSize: Local Directory with more than 128 entries

## APRL514: Intermediate Systems - PDU Formats - Cancellation Request

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| cqType | PDU Type | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |
| cqRef | Cancellation Reference | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |
| cqLocRef | Local Reference | 5.7.6.3.6 |  |  | M | M |
| cqLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.9 |  |  | lrDirSize:M ^lrDirsize:X | lrDirSize:M ^lrDirsize:X |

mcCan: Local Reference Cancellation

lrDirSize: Local Directory with more than 128 entries

## APRL515: Intermediate Systems - PDU Formats - Cancellation Accept

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| ccType | PDU Type | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |
| ccRef | Cancellation Reference | 5.7.6.3.6 |  |  | mcCan:M | mcCan:M |

mcCan: Local Reference Cancellation

# Routing Protocols

## ARL062: Routing Information Exchange Protocol

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| ATNROUT | Support of Routing Exchange protocols and ATN specific requirements | 5.8.1 |  | M | M | M |
| ESIS1 | ISO 9542 over mobile air-to-ground subnetworks | 5.8.2.1 |  | - | M | M |
| IDRP1 | Support of ATN specific features for ISO 10747 | 5.8.3.1 |  | M | M | M |
| IDRP2 | Use of the Security Path Attribute | 5.8.3.2.2 |  | M | M | M |
| IDRP3 | When an ATN Router supports data classified according to a security policy and for the purpose of implementing mandatory access controls, then the ATN Router should also support the security classification Security Tag Set defined in 5.6.2.2.6. | 5.8.3.2.2.4 |  | REC | REC | REC |
| IDRP4 | Update of Security Information | 5.8.3.2.4 |  | M | M | M |
| IDRP5 | Route Selection | 5.8.3.2.5 |  | M | M | M |
| IDRP6 | Frequency of Route Advertisement | 5.8.3.2.7 |  | M | M | M |
| IDRP7 | Interpretation of route capacity | 5.8.3.2.8 |  | M | M | M |
| IDRP8 | Network layer Reachability information | 5.8.3.2.9 |  | M | M | M |
| IDRP9 | BISPDU Authentication | 5.8.3.2.10 |  | M | M | M |
| IDRP10 | Restrictions on Route Advertisement | 5.8.3.2.11 |  | M | M | M |
| IDRP11 | RIB-Att Support | 5.8.3.2.12 |  | M | M | M |
| IDRP12 | An Air/Ground Router should aggregate all routes to destinations in Routing Domains in its own ATN Island, other than those to destinations in its own Routing Domain | 5.8.3.2.6.2.1 |  | - | REC | - |
| IDRP13 | An Air/Ground Router should aggregate all routes to destinations in ATN Islands, other than those to destinations in its own ATN Island | 5.8.3.2.6.2.2 |  | - | REC | - |
| IDRP14 | ATN Ground/Ground Routers should perform Route Aggregation and Route Information Reduction on routes to ground destinations, in line with local policy requirements for reducing the amount of routing information distributed within the ATN Ground Environment. | 5.8.3.2.6.2.3 |  | REC | - | - |
| IDRP15 | An Air/Ground Router should perform Route Information Reduction as permitted by the ATN Addressing Plan, before advertising aggregated routes to an Airborne Router. | 5.8.3.2.6.5.1 |  | - | REC | - |

## APRL301: ISO/IEC 9542 - Intermediate System

| Item | Description | ISO/IEC 9542 Reference | ISO/IEC 9542 Status | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CI | Is configuration information supported over the associated subnetwork? |  | O | 5.8.2 |  |  | M | M |
| RI | Is redirection information supported over the associated subnetwork? |  | O | 5.8.2 |  |  | OX | OX |

## APRL302: ISO/IEC 9542 - Intermediate System

| Item | Description | ISO/IEC 9542 Reference | ISO/IEC 9542 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ErrP | Protocol Error Processing | 6.13 | M |  |  | M | M |
| HCsV | PDU Header Checksum Validation | 6.12 | M |  |  | M | M |
| HCsG | PDU Header Checksum Generation | 6.12 | O |  |  | O | O |
| RpCf | Report Configuration | 6.2, 6.2.2 | CI:M |  |  | M | M |
| RcCf | Record Configuration | 6.3, 6.3.1 | CI:M |  |  | M | M |
| FlCf | Flush Old Configuration | 6.4 | CI:M |  |  | M | M |
| RqRd | Request Redirect | 6.8 | RI:M |  |  | OX | OX |
| CfNt | Configuration Notification | 6.7 | CI:O |  |  | OX | OX |
| CTGn | ESCT Generation | 6.3.2 | CI:O |  |  | OX | OX |
| AMGn | Address Mask (only) generation | 6.8 | RI:O |  |  | OX | OX |
| SMGn | Address mask and SNPA Mask generation | 6.8 | RI:O |  |  | OX | OX |

RI: Is redirection information supported over the associated subnetwork?

CI: Is configuration information supported over the associated subnetwork?

## APRL303: ISO/IEC 9542 - Intermediate System

| Item | Description | ISO/IEC 9542 Reference | ISO/IEC 9542 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESH-r | <r> End System Hello | 7.1, 7.5 | CI:M |  |  | O | O |
| ISH-<r> | <r> Intermediate System Hello | 7.1, 7.6 | CI:O |  |  | M | M |
| ISH-<s> | <s> Intermediate System Hello | 7.1, 7.6 | CI:M |  |  | M | M |
| RD-s | <s> Redirect | 7.1, 7.7 | RI:M |  |  | OX | OX |
| RD-r | <r> (ignore) Redirect | 6.9, 7.1, 7.7 | M |  |  | M | M |

RI: Is redirection information supported over the associated subnetwork?

CI: Is configuration information supported over the associated subnetwork?

## APRL304: ISO/IEC 9542 - Intermediate System

| Item | Description | ISO/IEC 9542 Reference | ISO/IEC 9542 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FxPt-s | <s> Fixed Part | 7.2.1, 7.2.7 | M |  |  | M | M |
| FxPt-r | <r> Fixed Part | 7.2.1, 7.2.7 | M |  |  | M | M |
| SA-r | <r> Source Address, one or more NSAPs | 7.3.1/2/3 | CI:M |  |  | M | M |
| NET-s | <s> Network Entity Title | 7.3.1/2/4 | M |  |  | M | M |
| NET-r | <r> Network Entity Title | 7.3.1/2/4 | ISH-r:M |  |  | ISH-r:M | ISH-r:M |
| DA-s | <s> Destination Address | 7.3.1/2/5 | RI:M |  |  | OX | OX |
| BSNPA-s | <s> Subnetwork Address | 7.3.1/2/6 | RI:M |  |  | OX | OX |
| Scty-s | <s> Security | 7.4.2 | O |  |  | O | O |
| Scty-r | <r> Security | 7.4.2 | O |  |  | O | O |
| Pty-s | <s> Priority | 7.4.3 | O |  |  | O | O |
| Pty-r | <r> Priority | 7.4.3 | O |  |  | O | O |
| QOSM-s | <s> QOS Maintenance | 7.4.4 | RI:O |  |  | OX | OX |
| AdMk-s | <s> Address Mask | 7.4.5 | RI:O |  |  | OX | OX |
| SNMk-s | <s> SNPA Mask | 7.4.6 | RI:O |  |  | OX | OX |
| ESCT-s | <s> Suggested ES Configuration Timer | 7.4.7 | CI:O |  |  | OX | OX |
| ESCT-r | <r> (ignore) Suggested ES Configuration Timer | 7.4.7 | ISH-r:M |  |  | ISH-r:M | ISH-r:M |
| OOpt-r | <r> (ignore) unsupported or unknown options | 7.4.1 | M |  |  | M | M |
| OOpt-s | <s> Other options |  | P |  |  | P | P |

CI: Is configuration information supported over the associated subnetwork?

RI: Is redirection information supported over the associated subnetwork?

ISH-<r>: <r> Intermediate System Hello

## APRL305: ISO/IEC 9542 - Intermediate System

| Item | Description | ISO/IEC 9542 Reference | ISO/IEC 9542 Status | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTv | What range of values can be set for the Holding Time Field in transmitted PDUs ? |  | M | 5.3.5.2.9 |  |  | M from: 0 seconds to: 65535 seconds with a tolerance of: 10% | M from: 0 seconds to: 65535 seconds with a tolerance of: 10% |
| CTv | If configuration information is supported, what range of values can be set for the Configuration Timer ? |  | CI:M | 5.3.5.2.5 |  |  | M from: 0 seconds to: 65535 seconds with a tolerance of: 10% | M from: 0 seconds to: 65535 seconds with a tolerance of: 10% |

APRL305:CI:APRL301 - Is configuration information supported over the associated subnetwork?

## APRL401: ATN Specific Protocol Requirements

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| ATNIDRP1 | Does this BIS encode and use of the Security Path Attribute? | 5.8.3.2.2, 5.8.3.2.3 |  | M | M | M |
| ATNIDRP2 | Does this BIS immediately re-advertise routes if the security information contained in the route’s security path attribute changes? | 5.8.3.2.7 |  | M | M | - |
| ATNIDRP3 | Does this BIS support ‘policy based route aggregation’? | 5.8.3.2.6.2 |  | O | O | - |
| ATNIDRP4 | Does this BIS support ‘policy based route information reduction’? | 5.8.3.2.6.5 |  | O | O | - |
| ATNIDRP5 | Does this BIS support aggregation of routes with identical NLRI using ‘true route aggregation’? | 5.8.3.2.6.3 |  | O.1 | O.1 | - |
| ATNIDRP6 | Does this BIS support aggregation of routes with identical NLRI using ‘route merging’? | 5.8.3.2.6.3 |  | O.1 | O.1 | - |
| ATNIDRP7 | Does this BIS support aggregation of security path attribute information field? | 5.8.3.2.6.4 |  | M | M | - |

O.1: At least one of these options must be supported

## APRL402: IDRP General

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BASIC | Are all basic BIS functions implemented? | 12.1 | M |  | M | M | M |
| MGT | Is this system capable of being managed by the specified management information? | 11 | M |  | O | O | O |
| VER | Does this BIS support Version Negotiation? | 7.8 | M |  | M | M | M |
| RTSEP | Does this BIS support the ROUTE_SEPARATOR attribute? | 7.12.1 | M |  | M | M | M |
| HOPS | Does this BIS support the RD_HOP_COUNT attribute? | 7.12.13 | M |  | M | M | M |
| PATH | Does this BIS support the RD_PATH attribute? | 7.12.3 | M |  | M | M | M |
| CAPY | Does this BIS support the Capacity Attribute? | 7.12.15 | M |  | M | M | M |
| FSM | Does this BIS manage BIS-BIS connections according to the BIS FSM description? | 7.6.1 | M |  | M | M | M |
| FCTL | Does this BIS provide flow control? | 7.7.5 | M |  | M | M | M |
| SEQNO | Does this BIS provide sequence number support? | 7.7.4 | M |  | M | M | M |
| INTG1 | Does this BIS provide Data Integrity using authentication type 1? | 7.7.1 | O.1 |  | M | M | M |
| INTG2 | Does this BIS provide Data Integrity using authentication type 2? | 7.7.2 | O.1 |  | O | O | O |
| INTG3 | Does this BIS provide Data Integrity using authentication type 3? | 7.7.3 | O.1 |  | O | O | O |
| ERROR | Does this BIS handle error handling for IDRP? | 7.20 | M |  | M | M | M |
| RIBCHK | Does this BIS operate in a 'fail-stop' manner with respect to corrupted routing information? | 7.10.2 | M |  | M | M | M |

O.1: At least one of these options must be supported

## APRL403: IDRP Update Send Process

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INT | Does the BIS provide the internal update procedures? | 7.17.1 | M |  | M | M | O |
| RTSEL | Does this BIS support the MinRouteAdvertisementInterval Timer? (except in the case specified in ATNIDRP2) | 7.17.3.1 | M |  | M | M | O |
| RTORG | Does this BIS support the MinRDOriginationInterval Timer? | 7.17.3.2 | M |  | M | M | M |
| JITTER | Does this BIS provide jitter on its timers? | 7.17.3.3 | M |  | M | M | M |

## APRL404: Update Receive Process

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INPDU | Does the BIS handle inbound BISPDUs correctly? | 7.14 | M |  | M | M | M |
| INCONS | Does this BIS detect inconsistent routing information? | 7.15.1 | M |  | M | M | M |

## APRL405: IDRP Decision Process

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TIES | Does this BIS break ties between candidate routes correctly? | 7.16.2.1 | M |  | M | M | M |
| RIBUPD | Does this BIS update the Loc-RIBs correctly? | 7.16.2 | M |  | M | M | M |
| AGGRT | Does this BIS support route aggregations? | 7.18.2.1, 7.18.2.2, 7.18.2.3 | O |  | ATNIDRP3 or ATNIDRP5:M | ATNIDRP3 or ATNIDRP5:M | - |
| LOCK | Does this BIS provide interlocks between its Decision Process and the updating of the information in its Adj-RIBs-In? | 7.16.4 | M |  | M | M | M |

ATNIDRP3: Support of 'policy based route aggregation'

ATNIDRP5: Support of aggregation of routes with identical NLRI using 'true route aggregation'

## APRL406: IDRP Receive

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RCV | Does the BIS process incoming BISPDUs and respond correctly to error conditions? | 7.14, 7.20 | M |  | M | M | M |
| OSIZE | Does this BIS accept incoming OPEN PDUs whose size in octets is between MinBISPDULength and 3000? | 6.2,7.20 | M |  | M | M | M |
| MXPDU | Does the BIS accept incoming UPDATE, IDRP ERROR and RIB REFRESH PDUs whose size in octets is between minBISPDULength and maxBISPDULength? | 6.2,7.20 | M |  | M | M | BISREF: OX ^BISREF:M |

BISREF: If RIB REFRESH PDU then true else false

## APRL407: Peer Entity Authentication

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AUTH | Does this BIS correctly authenticate the source of a BISPDU? | 7.7.2 | O |  | M | M | M |

## APRL408: IDRP CLNS Forwarding

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PSRCRT | Does the BIS correctly handle ISO/IEC 8473 NPDUs that contain a partial source route? | 8 | M |  | O | OX | O |
| DATTS | Does the BIS correctly extract the NPDU-derived Distinguishing Attributes from an ISO/IEC 8473 NPDU? | 8.2 | M |  | M | M | M |
| MATCH | Does the BIS correctly match the NPDU-derived Distinguishing Attributes with the corresponding FIB-Atts? | 8.3 | M |  | M | M | M |
| EXTF | Does the BIS correctly forward NPDUs with destinations outside its own routing domain? | 8.4 | M |  | M | M | M |
| INTF | Does the BIS correctly forward NPDUs with destinations inside its own routing domain? | 8.1 | M |  | M | M | M |

## APRL410: IDRP Optional Transitive Attributes

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MEXIT | Does this BIS support use of the MULTI-EXIT DISC attribute? | 7.12.7 | O |  | O | O | O |

## APRL411: Generating Well-known Discretionary Attributes

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EXTG | Does the BIS support generation of the EXT_INFO attribute? | 7.12.2 | O |  | O | O | O |
| NHRS | Does the BIS support generation of the NEXT_HOP attribute in support of route servers? | 7.12.4 | O |  | O | IDRPAG:OX ^IDRPAG:O | O |
| NHSN | Does the BIS support generation of the NEXT_HOP attribute to advertise SNPAs? | 7.12.4 | O |  | O | IDRPAG:OX ^IDRPAG:O | O |
| DLI | Does the BIS support generation of the DIST_LIST_INCL attribute? | 7.12.5 | O |  | O | O | O |
| DLE | Does the BIS support generation of the DIST_LIST_EXCL attribute? | 7.12.6 | O |  | O | IDRPAG:OX ^IDRPAG:O | O |
| TDLY | Does the BIS support generation of the TRANSIT DELAY attribute? | 7.12.8 | O |  | O | IDRPAG:OX ^IDRPAG:O | O |
| RERR | Does the BIS support generation of the RESIDUAL ERROR attribute? | 7.12.9 | O |  | O | IDRPAG:OX ^IDRPAG:O | O |
| EXP | Does the BIS support generation of the EXPENSE attribute? | 7.12.10 | O |  | O | IDRPAG:OX ^IDRPAG:O | O |
| LQOSG | Does the BIS support generation of the LOCALLY DEFINED QOS attribute? | 7.12.11 | O |  | OX | OX | OX |
| HREC | Does the BIS support generation of the HIERARCHICAL RECORDING attribute? | 7.12.12 | O |  | OX | OX | OX |
| SECG | Does the BIS support generation of the SECURITY attribute? | 7.12.14 | O |  | M | M | M |
| PRTY | Does the BIS support generation of the PRIORITY attribute? | 7.12.16 | O |  | O | IDRPAG:OX ^IDRPAG:O | O |

IDRPAG: If Air-Ground adjacency then true else false

## APRL412: Propagating Well-known Discretionary Attributes

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EXTGP | Does the BIS support propagation of the EXT_INFO attribute? | 7.12.2 | M |  | M | M | - |
| NHRSP | Does the BIS support propagation of the NEXT_HOP attribute in support of route servers? | 7.12.4 | O |  | O | IDRPAG:OX ^IDRPAG:O | - |
| NHSNP | Does the BIS support propagation of the NEXT_HOP attribute to advertise SNPAs? | 7.12.4 | O |  | O | IDRPAG:OX ^IDRPAG:O | - |
| DLIP | Does the BIS support propagation of the DIST_LIST_INCL attribute? | 7.12.5 | O |  | M | M | - |
| DLEP | Does the BIS support propagation of the DIST_LIST_EXCL attribute? | 7.12.6 | O |  | M | IDRPAG:OX ^IDRPAG:M | - |
| TDLYP | Does the BIS support propagation of the TRANSIT DELAY attribute? | 7.12.8 | O |  | O | IDRPAG:OX ^IDRPAG:O | - |
| RERRP | Does the BIS support propagation of the RESIDUAL ERROR attribute? | 7.12.9 | O |  | O | IDRPAG:OX ^IDRPAG:O | - |
| EXPP | Does the BIS support propagation of the EXPENSE attribute? | 7.12.10 | O |  | O | IDRPAG:OX ^IDRPAG:O | - |
| LQOSP | Does the BIS support propagation of the LOCALLY DEFINED QOS attribute? | 7.12.11 | O |  | OX | OX | - |
| HRECP | Does the BIS support propagation of the HIERARCHICAL RECORDING attribute? | 7.12.12 | O |  | OX | OX | - |
| SECP | Does the BIS support propagation of the SECURITY attribute? | 7.12.14 | O |  | M | M | - |
| PRTYP | Does the BIS support propagation of the PRIORITY attribute? | 7.12.16 | O |  | O | IDRPAG:OX ^IDRPAG:O | - |

IDRPAG: If Air-Ground adjacency then true else false

## APRL413: Receiving Well-known Discretionary Attributes

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EXTR | Does the BIS recognise upon receipt the EXT_INFO attribute? | 7.12.2 | M |  | M | M | M |
| NHRSR | Does the BIS recognise upon receipt the NEXT_HOP attribute ? | 7.12.4 | M |  | M | M | O |
| DLIR | Does the BIS recognise upon receipt the DIST_LIST_INCL attribute? | 7.12.5 | M |  | M | M | M |
| DLER | Does the BIS recognise upon receipt the DIST_LIST_EXCL attribute? | 7.12.6 | M |  | M | M | O |
| TDLYR | Does the BIS recognise upon receipt the TRANSIT DELAY attribute? | 7.12.8 | M |  | M | M | O |
| RERRR | Does the BIS recognise upon receipt the RESIDUAL ERROR attribute? | 7.12.9 | M |  | M | M | O |
| EXPR | Does the BIS recognise upon receipt the EXPENSE attribute? | 7.12.10 | M |  | M | M | O |
| LQOSR | Does the BIS recognise upon receipt the LOCALLY DEFINED QOS attribute? | 7.12.11 | M |  | O | O | O |
| HRECR | Does the BIS recognise upon receipt the HIERARCHICAL RECORDING attribute? | 7.12.12 | M |  | M | M | O |
| SECR | Does the BIS recognise upon receipt the SECURITY attribute? | 7.12.14 | M |  | M | M | M |
| PRTYR | Does the BIS recognise upon receipt the PRIORITY attribute? | 7.12.16 | M |  | M | M | O |

## APRL414: IDRP Timers

| Item | Description | ISO/IEC 10747 Reference | ISO/IEC 10747 Status | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ta | KeepAlive timer | 7.6.5 | M |  | M | M | M |
| Tr | Retransmission (tr) timer | 7.6.5 | M |  | M | M | M |
| Tmr | maxRIBIntegrityCheck timer | 7.10.2 | M |  | M | M | M |
| Tma | MinRouteAdvertisement timer | 7.17.3.1 | M |  | M | M | O |
| Trd | MinRDOriginationInterval timer | 7.17.3.2 | M |  | M | M | M |
| Tcw | closeWaitDelay timer | 7.6.4 | M |  | M | M | M |

# Routing Initiation

## ARL060: Ground-Ground Interconnection

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| ATNR1 | Ground-Ground Route Initiation | 5.3.4.2 |  | M | M |  |
| ATNR2 | Ground-Ground Routing Information Exchange | 5.3.4.3 |  | M | M |  |
| ATNR3 | Ground-Ground Route Termination | 5.3.4.4 |  | M | M |  |

## ARL061: Air-Ground Interconnection

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| ATNR4 | Air-Ground Route Initiation | 5.3.5.2 |  |  | M | M |
| ATNR5 | When in the initiator role, an ATN Router should use the ISO 8208 'Fast Select' facility, if supported by the subnetwork, and encode the first ISH PDU in the Call Request user data, according to the procedures for the Mobile SNDCF specified in Chapter 5.7. | 5.3.5.2.6.5 |  |  | REC | REC |
| ATNR6 | When in the responder role and the initiator has proposed use of the Fast Select Facility, the ATN Router should encode the first ISH PDU in the Call Accept User Data, according to the procedures for the Mobile SNDCF specified in Chapter 5.7. | 5.3.5.2.6.6 |  |  | REC | REC |
| ATNR7 | When IDRP is used to exchange routing information over an Air/Ground subnetwork, the value of the Holding Time field in the ISH PDU should be set to 65534, except when a watchdog timer is applied to the subnetwork connection (see 5.3.5.2.13) | 5.3.5.2.10.8 |  |  | REC | REC |
| ATNR8 | When IDRP is used to exchange routing information over an Air/Ground subnetwork, the Configuration Timer should be set such that no further ISH PDUs are exchanged following the Route Initiation procedure. | 5.3.5.2.10.9 |  |  | REC | REC |
| ATNR9 | A 'Leave Event' should not be generated by the Mobile SNDCF when a subnetwork connection is closed due to the expiration of the X.25 Idle timer, except if this subnetwork connection fails to be re-established. | 5.3.5.2.13.1 |  |  | REC | REC |
| ATNR10 | The watchdog timer should be configurable according to the characteristics of the subnetwork | 5.3.5.2.13.4 |  |  | REC | REC |

## APRL601: Air-Ground Route Initiation - General

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| njSubnet | Support of Subnetworks that do not provide a Join Event | 5.3.5.2 |  |  | O.1 | O.1 |
| jSubnet | Support of Subnetworks that do provide a Join Event | 5.3.5.2 |  |  | O.1 | O.1 |
| giSubnet | Support of Ground-Initiated Subnetworks | 5.3.5.2 |  |  | O.2 | O.2 |
| aiSubnet | Support of Air-Initiated Subnetworks | 5.3.5.2 |  |  | O.2 | O.2 |
| agSubnet | Support of Air or Ground-Initiated Subnetworks | 5.3.5.2 |  |  | O.2 | O.2 |
| fsSubnet | Support of Subnetworks that support Fast Select | - |  |  | O | O |
| noIDRP-a | Support of optional non-use of IDRP by Airborne BIS | 5.3.5.2.12.2 |  |  | O | O |
| noIDRP-ag | Support of optional non-use of IDRP by Air/Ground BIS | 5.3.5.2.12.1 |  |  | M | M |
| lvSubnet | Support of Subnetworks that provide a Leave Event | 5.3.5.2.13 |  |  | M | M |

O.1: At least one of these options must be supported

O.2: At least one of these options must be supported

## APRL602: Airborne Router - Subnetwork Connection Responder

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| respAR-ar | Response to incoming Call Request | 5.3.5.2.2 |  |  |  | giOragSubnet: M |
| valCR-ar | Validation of incoming Call Request | 5.3.5.2.2 |  |  |  | giOragSubnet:O |
| RespISH-ar | Generation of ISH PDU | 5.3.5.2.6 |  |  |  | giOragSubnet: M |
| ISHinCC-ar | Encoding ISH PDU in Call Accept User Data | 5.3.5.2.6 |  |  |  | RespISH-ar and fsSubnet: O |
| negNoIDRP-ar | Transmission of ISH PDU with SEL field of NET set to FEh | 5.3.5.2.6 |  |  |  | noIDRP-a:M |
| negIDRP-ar | Transmission of ISH PDU with SEL field of NET set to zero | 5.3.5.2.6 |  |  |  | ^noIDRP-a:M |
| autoRoute-ar | Inference of available routes from received NET of A/G Router | 5.3.5.2.12.3 |  |  |  | noIDRP-a:M |
| initIDRP-ar | IDRP startup procedures - Invoke activate action | 5.3.5.2.10 |  |  |  | ^noIDRP-a:M |
| supISH-ar | Suppression of multiple ISH PDUs | 5.3.5.2.10 |  |  |  | ^noIDRP-a: O |
| valNET-ar | Validation of received NET | 5.3.5.2.7 |  |  |  | ^noIDRP-a: O |

giOragSubnet: giSubnet OR agSubnet

giSubnet: Support of Ground-Initiated Subnetworks

agSubnet: Support of Air or Ground-Initiated Subnetworks

fsSubnet: Support of Subnetworks that support Fast Select

noIDRP-a: Support of optional non-use of IDRP by Airborne BIS

## APRL603: Airborne Router - Subnetwork Connection Initiator

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| polling-ai | Procedures for polling a list of subnet addresses | 5.3.5.2.3.1 |  |  |  | pollReq: M |
| backoff-ai | Backoff Procedure | 5.3.5.2.3.1.2 |  |  |  | pollReq: M |
| connect-ai | Connect on receipt of Join Event | 5.3.5.2.3.2 |  |  |  | EventDrvn: M |
| ValJoin-ai | Validation of Join Event | 5.3.5.2.3.2 |  |  |  | EventDrvn: O |
| SendISH-ai | Generation of ISH PDU | 5.3.5.2.6 |  |  |  | EventDrvn or pollReq:M |
| ISHinCR-ai | Encoding of ISH PDU in Call Request | 5.3.5.2.6 |  |  |  | SendISH-ar and fsSubnet: O |
| negNoIDRP-ai | Transmission of ISH PDU with SEL field of NET set to FEh | 5.3.5.2.8 |  |  |  | noIDRP-a:M |
| negIDRP-ai | Transmission of ISH PDU with SEL field of NET set to zero | 5.3.5.2.8 |  |  |  | ^noIDRP-a:M |
| autoRoute-ai | Inference of available routes from received NET of A/G Router | 5.3.5.2.12.2 |  |  |  | noIDRP-a:M |
| initIDRP-ai | IDRP startup procedures - listenForOpen set to true | 5.3.5.2.10 |  |  |  | ^noIDRP-a:M |
| supISH-ai | Suppression of multiple ISH PDUs | 5.3.5.2.10 |  |  |  | ^noIDRP-a: O |
| valNET-ai | Validation of received NET | 5.3.5.2.7 |  |  |  | ^noIDRP-a: O |

pollReq: aiSubnet and njSubnet

aiSubnet: Support of Air-Initiated Subnetworks

njSubnet: Support of Subnetworks that do not provide a Join Event

EventDrvn: jSubnet and (aiSubnet or agSubnet)

jSubnet: Support of Subnetworks that do provide a Join Event

agSubnet: Support of Air or Ground-Initiated Subnetworks

fsSubnet: Support of Subnetworks that support Fast Select

noIDRP-a: Support of optional non-use of IDRP by Airborne BIS

## APRL604: Air-Ground Router - Subnetwork Connection Responder

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| respAR-agr | Response to incoming Call Request | 5.3.5.2.2 |  |  | aiOragSubnet: M |  |
| valCR-agr | Validation of incoming Call Request | 5.3.5.2.2 |  |  | aiOragSubnet:O |  |
| emgncy-agr | Emergency Procedures | 5.3.5.2.2.2 |  |  | M |  |
| RespISH-agr | Generation of ISH PDU | 5.3.5.2.6 |  |  | aiOragSubnet: M |  |
| ISHinCC-agr | Encoding ISH PDU in Call Accepted User Data | 5.3.5.2.6 |  |  | RespISH-agr and fsSubnet: O |  |
| negNoIDRP-agr | Receipt of ISH PDU with SEL field of NET set to FEh | 5.3.5.2.8 |  |  | M |  |
| negIDRP-agr | Receipt of ISH PDU with SEL field of NET set to zero | 5.3.5.2.8 |  |  | M |  |
| autoRoute-agr | Inference of available routes from received NET of Airborne Router | 5.3.5.2.12.1 |  |  | M |  |
| initIDRP-agr | IDRP startup procedures - Invoke activate action | 5.3.5.2.10 |  |  | M |  |
| supISH-agr | Suppression of multiple ISH PDUs | 5.3.5.2.10 |  |  | O |  |
| valNET-agr | Validation of received NET | 5.3.5.2.7 |  |  | O |  |

aiOragSubnet: aiSubnet or agSubnet

agSubnet: Support of Air or Ground-Initiated Subnetworks

aiSubnet: Support of Air-Initiated Subnetworks

fsSubnet: Support of Subnetworks that support Fast Select

## APRL605: Air-Ground Router - Subnetwork Connection Initiator

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| connect-agi | Connect on receipt of Join Event | 5.3.5.2.4 |  |  | goOragSubnet: M |  |
| ValJoin-agi | Validation of Join Event | 5.3.5.2.4 |  |  | connect-agi: O |  |
| SendISH-agi | Generation of ISH PDU | 5.3.5.2.6 |  |  | connect-agi: M |  |
| ISHinCR-agi | Encoding of ISH PDU in Call Request | 5.3.5.2.6 |  |  | SendISH-agi and fsSubnet: O |  |
| negNoIDRP-agi | Receipt of ISH PDU with SEL field of NET set to FEh | 5.3.5.2.8 |  |  | M |  |
| negIDRP-agi | Receipt of ISH PDU with SEL field of NET set to zero | 5.3.5.2.8 |  |  | M |  |
| autoRoute-agi | Inference of available routes from received NET of Airborne Router | 5.3.5.2.12.1 |  |  | M |  |
| initIDRP-agi | IDRP startup procedures - listenForOpen set to true | 5.3.5.2.10 |  |  | M |  |
| supISH-agi | Suppression of multiple ISH PDUs | 5.3.5.2.10 |  |  | O |  |
| valNET-agi | Validation of received NET | 5.3.5.2.7 |  |  | O |  |

goOragSubnet: giSubnet or agSubnet

giSubnet: Support of Ground-Initiated Subnetworks

agSubnet: Support of Air or Ground-Initiated Subnetworks

connect-agi: Connect on receipt of Join Event

fsSubnet: Support of Subnetworks that support Fast Select

## APRL606: Termination Procedures

| Item | Description | SARPs Reference | Non-BIS Status | G-G BIS Status | A-G BIS Status | Airborne BIS Status |
| --- | --- | --- | --- | --- | --- | --- |
| lvEvent | Processing of Leave Event | 5.3.5.2.13 |  |  | M | M |
| Watchdog | Watchdog Timer | 5.3.5.2.13 |  |  | M | M |
| ConfigWD | Configurability of Watchdog for Subnetwork Characteristics | 5.3.5.2.13 |  |  | O | O |
| conLeave | Processing of a per connection Leave Event | 5.3.5.2.13 |  |  | M | M |
| subnetLeave | Processing of a per subnetwork Leave Event | 5.3.5.2.13 |  |  | M | M |