AUTOMATIC  DEPENDENT  SURVEILLANCE  PANEL

(ADSP)

DRAFT ICAO MANUAL OF

AIR TRAFFIC SERVICES (ATS)

DATA LINK APPLICATIONS

DRAFT Version  0.3

29 March 1996

TABLE OF CONTENTS

PRELIMINARIES

F1:  Document Configuration								iii

F2:  Glossary of Acronyms								iv

F3:  Explanation of Terms								ix

PART  I:  OVERVIEW OF ATS DATA LINK APPLICATIONS.

Chapter 1:  Introduction to the ICAO Manual						1-1

Appendix A: Template For A Detailed Description Of Air/Ground Data Link

Services								1A-1

Chapter 2:  The FANS Concept								2-1

Appendix A:  Human Factors							2A-1

Chapter 3:  Data Link Applications					3-1

Appendix A:  Communication Systems Performance Requirement Parameters	3A-1

Appendix B:  Transition Strategy						3B-1

Chapter 4:  Data Link Initiation Capability						4-1

Chapter 5:  Automatic Dependent Surveillance Application (ADS) Overview And High

Level Operational Requirements						5-1

Chapter 6:  Controller Pilot Data Link Communications (CPDLC) Application Overview

And High Level Operational Requirements					6-1

Chapter 7:  Digital Flight Information Services (DFIS) APPLICATION Overview And

High Level Operational Requirements						7-1

Chapter 8:  ATS Interfacility Data Communication Overview and High Level Operational 	Requirements									8-1

Chapter 9: Automatic Dependent Surveillance Broadcast (ADS-B) Application Overview

And High Level Operational Requirements					9-1

PART  II:  DATA LINK INITIATION CAPABILITY

Chapter 1:  Application Overview							1-1

Chapter 2:  General Requirements							2-1

Chapter 3:  DLIC Functional Capabilities						3-1

Appendix A:  Expected DLIC Message Traffic					3A-1

Chapter 4:  DLIC Message Description							4-1

Chapter 5:  Operational Message Sequence						5-1

Chapter 6:  DLIC Procedures								6-1

PART  III:  AUTOMATIC DEPENDENT SURVEILLANCE (ADS)

Chapter 1:  Application Overview							1-1

Appendix A:  Integration of ADS and SSR Data					1A-1

Chapter 2:  General Requirements							2-1

Chapter 3:  ADS Functional Capabilities							3-1

Appendix A:  ADS Message Exchange Rates					3A-1

Chapter 4:  ADS Message Descriptions							4-1

Appendix A:  ADS Message Data Glossary					4A-1

Appendix B:  ADS Variables Range and Resolution				4B-1

Chapter 5:  ADS Message Sequences							5-1

Chapter 6:  ADS Procedures								6-1

Chapter 7:  Exception Handling								7-1

PART  IV:  CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

Chapter 1:  Application Overview							1-1

Chapter 2:  General Requirements							2-1

Chapter 3:  CPDLC Functional Capabilities						3-1

Appendix A:  CPDLC Message Set						3A-1

Appendix B:  CPDLC Message Data Glossary					3B-1

Appendix C:  CPDLC Variables Range and Resolution				3C-1

Chapter 4:	CPDLC Procedures							4-1

Chapter 5:	Exception Handling							5-1

Chapter 6:	Departure Clearance Service Description				6-1

Chapter 7:	Transfer of Data Authority Service Description				7-1

Chapter 8:	Downstream Clearance Service Description				8-1

PART  V:  DIGITAL FLIGHT INFORMATION SERVICES (DFIS)

Chapter 1:  Application Overview							1-1

Chapter 2:  General Requirements							2-1

Chapter 3:  Functional Requirements							3-1

Chapter 4:  Procedures									4-1

Chapter 5:  Exception Handling								5-1

Chapter 6: Automatic Terminal Information Service (ATIS) Service Description		6-1

Appendix A:  ATIS Data Glossary						6A-1

Chapter 7: DFIS Meteorological Aerodrome Report (METAR) Service Description	7-1

PART  VI:  ATS Interfacility Data Communication (AIDC)

Chapter 1:  Application Overview							1-1

Appendix A:  Transition								1A-1

Chapter 2:  General Requirements							2-1

Chapter 3:  AIDC Functional Capabilities						3-1

Chapter 4:  Message Descriptions							4-1

Chapter 5:  Operational Message Sequences						5-1

PART  VII:  AUTOMATIC DEPENDENT SURVEILLANCE BROADCAST (ADS-B)

Chapter 1:  Application Overview							1-1

Chapter 2:  General Requirements							2-1

Chapter 3:  Functional Capabilities							3-1

Chapter 4:  Air Traffic Surveillance							4-1

F1.	DOCUMENT CONFIGURATION

F2.1	The following pages will constitute the Configuration Management of the draft ADSP Manual of ATS Data Link Applications.  The list of amendments on this document from one version to the next will be recorded here to provide the traceability required for configuration control audit until formal publication of the document as a Manual.

F2.2	Table A gives a summary of the revision materials which create the new version by reference to their sources.  Table B gives detail of the amendments by listing the changes and the pages, sections or paragraphs affected.

| Version Number | Revision Date | Revision Source | Remarks |
| --- | --- | --- | --- |
| 0.0 | 1 Nov. 95 | Post Montreal June/July 95 Joint WG Meeting |  |
| 0.1 | 8 DEC 95 | Montreal Nov./DEC 95 Joint WG Meeting Output |  |
| 0.2 | 1 March 96 | Pre Dakar March 96 WG /WHL Meeting |  |
| 0.3 | 29 March 96 | Post Dakar March WG /WHL Meeting |  |

Table A:  Document Status And History

| Version Number | Part/Chapter/ Appendix | Amended Pages | Remarks |
| --- | --- | --- | --- |

TABLE B:  Amendments Revision Record

F2.	GLOSSARY OF ACRONYMS/ABBREVIATIONS

| ACARS | Aircraft Communications Addressing And Reporting System |
| --- | --- |
| ACAS | Aircraft Collision Avoidance System |
| ACC | Area Control Center |
| ACK | Acknowledge |
| ADS | Automatic Dependent Surveillance |
| ADS-B | Automatic Dependent Surveillance Broadcast |
| ADSF | Automatic Dependent Surveillance Function |
| ADSP | ADS Processor |
| ADSU | ADS Unit |
| AGCS | Air-Ground Communication System |
| AIDC | ATS Interfacility Data Communications |
| AIP | Aeronautical Information Publication |
| ALRT | Alert |
| AMSS | Aeronautical Mobile-Satellite Service |
| APDLC | Aircraft Proximity Data Link Communication |
| ARM | ADS Report Message |
| ASM | Airspace Management |
| ASAS | Airborne Separation Assurance System |
| ASN.1 | Abstract Syntax Notation One |
| ATC | Air Traffic Control |
| ATCO | Air Traffic Control Officer |
| ATCU | Air Traffic Control Unit |
| ATFM | Air Traffic Flow Management |
| ATIS | Automatic Terminal Information Service |
| ATM | Air Traffic Management |
| ATN | Aeronautical Telecommunications Network |
| ATNP | Aeronautical Telecommunications Network Panel |
| ATS | Air Traffic Services |
| ATSC | Air Traffic Services Communication |
| ATSU | Air Traffic Services Unit |
| CAA | Civil Aviation Authority |
| C-ATSU | Controlling ATS Unit |
| CCR | Cancel Contract Request |
| CDA | Current Data Authority |
| CER | Cancel Emergency Mode Request |
| CMU | Communications Management Unit |
| CNS | Communication, Navigation And Surveillance |
| CNS/ATM | Communication, Navigation And Surveillance/Air Traffic Management |
| CPDLC | Controller Pilot Data Link Communication |
| CPL | Current Flight Plan |
| CTC | Cancel All Contracts And Terminate Connection Request |
| D-ATSU | Downstream ATS Unit |
| DC | Departure Clearance |
| DDA | Downstream Data Authority |
| DFIS | Digital Flight Information Service |
| DLIC | Data Link Initiation Capability |
| DSC | Downstream Clearance |
| ECR | Event Contract Request |
| EMG | Emergency |
| ETA | Estimated Time Of Arrival |
| ETD | Estimated Time Of Departure |
| FANS (Phase II) | Special Committee for the Monitoring and Co-ordination of Development and Transition Planning for the Future Air Navigation System (Phase II) |
| FASID | Facilities And Services Implementation Document |
| FDPS | Flight Data Processing System |
| FIR | Flight Information Region |
| FIS | Flight Information Service |
| FOM | Figure Of Merit |
| FPL | Filed Flight Plan |
| GG | Ground/Ground |
| GGCS | Ground/Ground Communication System |
| GNSS | Global Navigation Satellite System |
| GPWS | Ground Proximity Warning System |
| HF | High Frequency |
| HFDL | High Frequency Data Link |
| IAS | Indicated Air Speed |
| ICAO | International Civil Aviation Organization |
| ID | Identification |
| IFR | Instrument Flight Rules |
| ILS | Instrument Landing System |
| INS | Inertial Navigation System |
| IRS | Inertial Reference System |
| LACK | Logical Acknowledgment |
| LAN | Local Area Network |
| LOA | Letter Of Agreement |
| MET | Meteorology |
| METAR | Meteorological Aerodrome Report |
| Mode S | Selective Interrogation Mode of SSR |
| MOU | Memorandum Of Understanding |
| MSAW | Minimum Safe Altitude Warning |
| MSSR | Mode S SSR |
| NAK | Negative Acknowledgment |
| NAT | North Atlantic |
| NATSPG | North Atlantic Systems Planning Group |
| NCN | Non Compliance Notification |
| NDA | Next Data Authority |
| NIM | Navigational Integrity Monitoring |
| NM | Nautical Miles |
| NOTAM | Notice To Airmen |
| OCM | Oceanic Clearance Message |
| OLDI | On-Line Data Interchange |
| OR | Operational Requirement |
| OSI | Open Systems Interconnection |
| PANS-RAC | Procedures for Air Navigation Services - Rules of the Air and Air Traffic Services (Document 4444) |
| PCR | Periodic Contract Request |
| PER | Packed Encoding Rules |
| PIREP | Pilot Report |
| PSR | Primary Surveillance Radar |
| QOS | Quality Of Service |
| R-ATSU | Receiving ATS Unit |
| REC | Recall |
| RESP | Response |
| RGCSP | Review Of The General Concept Of Separation Panel |
| RNP | Required Navigation Performance |
| RTF | Radiotelephony |
| RVR | Runway Visual Range |
| R/T | Radio Transmission |
| SARPs | Standards And Recommended Practices |
| SATCOM | Satellite Communications |
| SC 170 | RTCA Special Committee 170 |
| SDPS | Surveillance Data Processing System |
| SF | Scaling Factor |
| SID | Standard Instrument Departure |
| SSR | Secondary Surveillance Radar |
| STCA | Short Term Conflict Alert |
| TAF | Terminal Area Forecast |
| T-ATSU | Transferring ATS Unit |
| TWS | Terminal Weather Service |
| URG | Urgency |
| UTC | Coordinated Universal Time |
| VDL | VHF Digital Data Link |
| VDR | VHF Data Radio |
| VHF | Very High Frequency |
| VFR | Visual Flight Rules |
| VMC | Visual Meteorological Conditions |
| VSP | Variable System Parameter |
| VOR | VHF Omnidirectional Radio Range |
| WILCO | Will Comply |
| WMO | World Meteorological Organization |

F.3	EXPLANATION OF TERMS

Aircraft Identification:   The 24-bit aircraft address specific to an individual aircraft.

Air Traffic Services Interfacility Data Communications (AIDC).  A data link application that provides the capability to exchange data between air traffic service units during the notification, coordination, and transfer of aircraft between flight information regions.

Automatic Dependent Surveillance (ADS):  A surveillance technique in which aircraft automatically provide, via a data link, data derived from on-board navigation and position-fixing systems, including aircraft identification, four-dimensional position, and additional data as appropriate.  ADS is a data link application.

Automatic Dependent Surveillance-Broadcast (ADS-B):  ADS-B is a surveillance application transmitting parameters, such as position, track and ground speed, via a broadcast mode data link, and at specified intervals, for utilization by any air and/or ground users requiring it.  ADS-B is a data link application.

ADS Agreement:  An ADS agreement is an ADS reporting plan which establishes the conditions of ADS data reporting (i.e., data required by the ground system and frequency of ADS reports which have to be agreed to prior to the provision of the ADS services).  The terms of the agreement will be exchanged between the ground system and the aircraft by means of a contract, or a series of contracts.  An ADS contract would specify under what conditions ADS reports would be initiated, and what data would be contained in the reports.  There are three types of contracts:  Periodic, Event and Demand.

Availability:  The ability of a system to perform its required function at the initiation of the intended operation.  It is quantified as the proportion of the time the system is available to the time the system is planned to be available.

Baseline Information:  Required information upon which to measure certain type of ADS events.  (i.e., altitude change event, air speed change event, ground speed change event, heading change event and track angle change event).

Continuity:  The probability of a system to perform its required function without unscheduled interruptions during the intended period of operations.

Controller Pilot Data Link Communications (CPDLC):  A data link application that provides a means of communication between controller and pilot, using data link for ATC communications.

Data Link Application:  A data link application is the implementation of data link technology to achieve specific Air Traffic Management (ATM) operational functionalities.  For example, in this context the current functionalities are DLIC, ADS, CPDLC, DFIS, AIDC, and ADS-B.

Data Link Initiation Capability:  A data link application that provides the ability to exchange addresses, names and version numbers necessary to initiate data link applications.

Data Link Service:  A data link service is a set of ATM related transactions, both system supported and manual, within a data link application, which have a clearly defined operational goal.  Each data link application service is a description of its recommended use from an operational point of view.

Digital Flight Information Services:  A data link application that allows the exchange of pertinent flight data between air and ground users.

End-to-End Transfer Delay:  The period elapsed from the time at which the originating user initiates the triggering event until the time the transmitted information has been received by the intended recipient.

Integrity:  The probability that errors will be mis-detected.  This may be when a correct message is indicated as containing one or more errors, or when a message containing one or more errors is indicated as being correct.  Note:  Integrity relates to the trust which can be placed in the correctness of the information provided.

Operational Requirement (OR):  A statement of the operational attributes of a system needed for the effective and/or efficient provision of air traffic services to users.

Reliability:  The probability that the system will deliver a particular message without one or more errors.

Note:  Definitions of other terms are provided in the Glossary of Acronyms and in the Data Glossaries for data link applications.

PART  I

INTRODUCTION  TO  ATS  DATA  LINK APPLICATIONS

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# INTRODUCTION TO THE ICAO MANUAL

## Purpose of the Document

### The purpose of this document is to describe the elements of a data link based Air Traffic Service (ATS) and its application on a world-wide basis.  The document provides guidance material for aviation authorities, airspace users and service providers in establishing a data link based service in their airspace according to regional and national plans.

### This document has been developed to:

  - explain the concept of a data link based ATC system and associated communications requirements for the digital interchange of ATS messages;

  - identify how a data link based ATS will enhance existing air traffic services;

  - provide guidance material for aviation authorities, airspace users, and service providers on:

    - system concepts and descriptions;

    - operational requirements;

    - procedures and automation capabilities; and

    - implementation and transition strategies, including particular service descriptions which guide those implementing on ways to implement portions of an application.

### The data link based system will be characterized by the use of Automatic Dependent Surveillance (ADS), Controller Pilot Data Link Communication (CPDLC) and the automatic provision of data linked flight information services from an addressable database on request by the pilot.  A Data Link Initiation Capability (DLIC) service allowing the establishment of the necessary communications link between the aircraft and the relevant ATS ground systems will be provided.  In order to support these air/ground services, an appropriate ground/ground ATS interfacility data link communications (AIDC) network will be incorporated.  Initial information on ADS Broadcast (ADS-B) is included.

### Data link applications are being developed and implemented on a regional basis.  Integration of these developments into a global implementation is envisaged within the context of the future Communication Navigation Surveillance/ Air Traffic Management (CNS/ATM) systems.  Individual States and the aeronautical industry are progressing the technical specifications for aircraft, ground equipment and other system components.  Equipment for related data link communications is also being developed.  Consequently, and in the general interest of developing harmonized and compatible systems, the contents of this guidance material should be taken into account in those developments, designs and implementations.

## Structure of the document

### The main body of this document contains the following parts:

- Introduction to ATS Data Link Applications,

- Data Link Initiation Capability (DLIC),

- Automatic Dependent Surveillance (ADS),

- Controller Pilot Data Link Communications (CPDLC),

- Digital Flight Information Service (DFIS),

- ATS Interfacility Data Communications (AIDC), and

- ADS Broadcast (ADS-B).

### The guidance material in this document for applications using Aeronautical Telecommunications Network (ATN) should be used in conjunction with the International Civil Aviation Organization (ICAO) Standards and Recommended Practices (SARPs) prepared by the ATN Panel (ATNP), and with procedures already developed for the use of ADS and other data link communications, as contained in Annexes 2 and 11 and the Procedures for Air Navigation Services   Rules of the Air and Air Traffic Services (PANS-RAC, Doc 4444).

### This guidance material is considered part of the evolutionary process for the implementation of data link related technology.  ICAO will continue its efforts in support of the timely development of all necessary material to ensure a global harmonization and standardization of future data link based ATC systems.

### Part 1 provides an introduction to data link applications being developed for use in the CNS/ATM environment, and the requirements of the overall system.  Relevant technologies are briefly described, and their inter-relationship and use in the overall Air Traffic System is outlined.  States concerned with the development of data link applications should ensure that the technical media to be provided fulfill the operational needs.

### Part 2 contains guidance material for the Data Link Initiation Capability (DLIC).  The DLIC provides the necessary information to enable data link communications between the ATC ground and air systems to be established.

### Part 3 provides guidance material and information from an operational standpoint in support of technical developments relating to ADS.  In this context, the guidance material represents a set of agreed operational principles and procedures for the efficient use of ADS in ATS.

### Part 4 explains the concept of Controller Pilot Data Link Communication (CPDLC) and its associated requirements, identifies how this will enhance air traffic services, and describes in detail the necessary message formats and their implementation.

### Part 5 indicates how flight information services will be incorporated into the data link environment, providing the ability for the pilot to receive Digital FIS (DFIS) on the flight deck on request or automatically.  An outline of the services proposed for implementation is also given.

### Part 6 relates to the concept of ATS Interfacility Data Communications (AIDC) and gives guidance on the implementation of ground to ground data link technology needed to support the air/ground ATC facilities.

### Part 7 relates to ADS Broadcast (ADS-B) and provides the initial guidance on its concept.

### Further parts may be added in later editions to reflect the introduction, development and implementation of other applications.  Appendix A to this chapter provides a template for States or organizations to submit additional applications or services to ICAO.  Existing service descriptions generally conform to this template.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

CHAPTER 1 APPENDIX A

TEMPLATE FOR A DETAILED DESCRIPTION OF AIR/GROUND DATA LINK SERVICES

1A.1	Detailed Description of Selected Services

1A.1.1	This document describes data link applications as Services.  A Service is a set of ATM related transactions, both system supported and manual, within a data link application, which have a clearly defined operational goal.  Each data link service is a description of its recommended use from an operational point of view.

1A.1.2	Services are defined in gradually increasing detail, using the following sub-sections:

Scope and Objective provides a brief, two-to-three sentence description of what the Service does from an operational perspective.

Expected Benefits, Anticipated Constraints, and Associated Human Factors

Expected benefits provides a non-exhaustive list of benefits expected from implementation of the Service uniquely or particularly applicable to the Service being described.

Anticipated constraints describes the constraints which could result from implementation of the Service uniquely or particularly applicable to the Service being described.

Human Factors provides the human factors aspects considered essential for the safe and coherent operation of the service.

Operating Method Without Data Link describes how the controller, pilot, or support systems perform the Service in today's non-data link environment.  As this heading describes controller and pilot actions, it will also cover procedures.

Operating Method With Data Link:

Normal Mode describes how the Service is normally conducted via the following sub-sections.

- Service Description describes how the controller, pilot, or support system(s) will perform the Service with data link assistance.  As this heading describes controller and pilot actions, it will also cover procedures and should identify optional features in addition to the standard features of the Service.  This heading also describes how the data link Service is carried out, stating what messages are sent by the data link partners involved, and what operational events trigger the transmittal of the messages.

- Initiation Conditions describes what conditions must be met prior to initiation of the data link service, to include association with operational events and manual action.

- Sequence Of Services states what other data link services must precede the data link service, if any.

- Additional Guidelines provide any additional features for the data link service, to include amplification of the preceding elements and any recommended enhancements that could be achieved through advanced airborne or ground equipment.

Time Sequence Diagram (TSD) is a standard method to illustrate the message flows in chronological order for the standard (nominal) execution of a Service.

An example diagram follows:

Information Exchanges provides further operational requirements for each message described in the above section.  This section is set out in a table with the following entries:

  - Message states the name of the specific message within the exchange.

  - Information required includes a plain text description of the data to be transferred in the message.  Descriptions should clearly separate and identify the mandatory and optional contents.

  - Event Trigger briefly describes the operational event that will initiate the message.

  - Source/Destination gives the operational source and destination of the message.  These can be aircraft or one of several ATSU designators, as defined in the glossary.

  - Alerting Requirements describes the need for air crew and controller alerting for the data link service..   Alerting can be one of four categories:

    - H:  High;

    - M:  Medium;

    - L:  Low;

    - N:  No alerting required.

  - Response indicates whether a response is or is not required for the message (‘Y’ and ‘N’ respectively).

An example table is as follows:

| Message | Information Required | Example operational MSG contents | Event/ Trigger | Source/ Dest. | Alert | Response |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mandatory Optional |  |  |  | Ctrlr: Pilot: |  |

Quality Of Service Requirements (QOS): Only exceptions to the global QOS requirements (when available) need be specified.

  - Communication Priority establishes the priority of messages within this Service in relation to other information flows:

    - Distress, indicating grave and imminent danger;

    - Urgent, concerning the safety of the aircraft or persons on-board or within sight;

    - Flight Safety, comprising movement and control messages and meteorological or other advice of immediate concern to an aircraft in flight or about to depart, or of immediate concern to units involved in the operational control of an aircraft in flight or about to depart;

    - Routine Surveillance or Navigation (e.g., ADS);

    - Routine operational messages, comprising aircraft operator and other messages of concern to the aircraft in flight or about to depart;

    - NOTAM-Class I distribution;

    - Meteorological messages, comprising forecasts, observations and other messages exchanged between meteorological offices;

    - Low, indicating any message with a lower priority than the above.

  - Urgency delineates the relative relationship among messages when placed in a queue for operator access.  It relates to the handling of the information by the receiving system.  It dictates the order of display, processing (including deletion, modification, and shelf-life), or other action in accordance with the sequencing of essential, routine and time-expired data.  Urgency does not influence communication processing, which is defined by communications priority; it applies to the end user processing application only.  Valid entries are:

    - Distress:  indicating grave and imminent danger;

    - Urgent:  comprising movement and control messages and meteorological or other advice of immediate concern to an aircraft in flight or about to depart, or of immediate concern to units involved in the operational control of an aircraft in flight or about to depart;

    - Normal:  comprising routine operational messages such as routine surveillance or navigation, meteorological messages, etc.;

    - Low:  indicating any message with a lesser urgency than the above.

  - Information Security provides any applicable security requirements for the messages, including:

    - Data Origin Authentication, indicating how much assurance is required that the data source is as stated.  Valid entries are:

      - Normal, indicating that the indicated originator must always be authentic;

      - Low, indicating that the non-authentic originators are acceptable in some circumstances, but must be identifiable and notified to the responsible operators.

    - Access Control, indicating the confidentiality level of the data, or the requirement for the data to be restricted to only authorized recipients.  Valid entries can be one or more of the following:

      - 'C', indicating that the data must be protected against any unauthorized access, to include copying of the data;

      - 'M', indicating that the data must be protected against any unauthorized and undetected modification of a message;

      - 'A', indicating protection against unauthorized addition of messages;

      - 'D', indicating protection against unauthorized deletion of messages.

    - Data Integrity.  Valid entries are:

      - Maximum, indicating that loss or corruption of data is unacceptable;

      - Medium, indicating that loss or corruption of data is acceptable in some circumstances, but must be identifiable and notified to the responsible operators.

      - Minimum, indicating that loss or corruption of data is acceptable and does not require notification.

    - Availability, and

    - Service Restoration Time.

Exception handling describes what should happen if the service fails, to include error reporting, exception handling, recovery needs, procedures and alternative message exchange.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# THE FANS CONCEPT

## Historical Background

### In the early 1980's, ICAO recognized the increasing limitations of the present air navigation systems and the improvements needed to take civil aviation into the 21st century.  In 1983, ICAO established the Special Committee on Future Air Navigation Systems (FANS) with the task of studying, identifying and assessing new concepts and new technology and making recommendations for the coordinated evolutionary development of air navigation over a time scale of the order of 25 years.

### FANS recognized that the limitations of the present systems were intrinsic to the systems themselves and the problems could not be overcome on a global scale except by development and implementation action of new concepts and new communications, navigation and surveillance (CNS) systems to support future enhancements to Air Traffic Management (ATM).  FANS proposed a new system concept, one which would evolve over a period of years, and which recognized that the pace of change cannot be the same everywhere on the globe.

### ICAO recognized that implementation of the new systems concepts would require global coordination and planning on an unprecedented scale, and established a follow-on committee to help ensure a coherent, cost-beneficial, global implementation of the new system concept.  Human factors and human/computer interfaces would also require careful consideration.  Appendix A to this chapter contains further information on human factors.

### The Tenth Air Navigation Conference (1991) endorsed the global concept proposed by the Special Committee for the Monitoring and Co-ordination of Development and Transition Planning for the Future Air Navigation System (FANS Phase II).  The concept, which is contained in the Global Co-ordinated Plan for Transition to the ICAO CNS/ATM Systems (Doc 9623, Report of FANS (II)/4), included a variety of satellite-based systems along with a judicious selection of ground-based systems.  An outline indication of how the various elements of the concept will be applied is given below.

## The Icao Cns/Atm System

### Communications

#### In the future CNS/ATM system, air-ground communications with aircraft will increasingly be by means of digital data link.  This will allow efficient communication paths between ground and airborne systems.  ICAO has developed a communication systems architecture that provides a range of capabilities to suit the needs of air traffic services providers and their users, from basic low-speed data to high-speed digital voice, including the Aeronautical Mobile Satellite Service (AMSS).  Various communications media, e.g., AMSS, Very High Frequency (VHF), data link and Secondary Surveillance Radar (SSR) Mode S data link, will be integrated through an ATN based on an Open Systems Interconnection (OSI) architecture.

#### Potential benefits from air-ground communications are:

- efficient linkages between ground and airborne systems,

- improved handling and transfer of data,

- reduced channel congestion,

- reduced communications errors,

- inter-operable communication media, and

- reduced workload.

### Navigation

#### Required Navigation Performance (RNP), conceived by the FANS Committee and developed by the Review of the General Concept of Separation Panel (RGCSP), defines navigation performance accuracy required for operation within a defined airspace.  The concept, in principle, allows the aircraft operator to select the type of navigation equipment to use.  It is anticipated that RNP requirements could be met by the Global Navigation Satellite System (GNSS), currently being deployed.  It is expected that GNSS will be able to provide a high integrity, highly accurate navigation service, suitable for sole means navigation, at least for enroute applications.

#### Potential benefits from GNSS are:

- improved four-dimensional navigational accuracy;

- high-integrity, high accuracy, worldwide navigation service;

- cost savings from phase-out of ground-based navigational aids; and

- improved air transport services using non-precision approaches and precision landing operations.

### Surveillance

#### The data link based ATS will use a data link to provide surveillance information for ATS.  Surveillance may be independent, i.e., using radar, or dependent, i.e., using on board derived information passed automatically to the ATC provider.  The two systems may, where necessary, be combined.

#### Potential benefits from the enhanced surveillance system are:

- enhanced flight safety;

- improved surveillance of aircraft in non-radar areas;

- possible reduction of separation minima in non-radar airspace;

- reduced delays;

- the accommodation of user-preferred flight profiles;

- increased ATC capacity; and

- more efficient and economic aircraft operations.

### Air Traffic Management (ATM)

#### The future ATM system will make maximum use of automation to reduce or eliminate constraints imposed on ATM operations by current systems, and to derive the benefits made possible by implementation of the new CNS systems.  The flexibility facilitated by the new CNS systems will allow the introduction of automation capabilities from the simplest to the most advanced, as required by the individual States, and in a globally harmonious fashion.  It is expected that the early use of ATM automation will be most visible in flow and tactical management.

#### ATM automation will make it possible to formulate real-time flow management strategies and allow for negotiation between ATS and aircraft to enhance tactical management.  Data link and voice channels, enhanced by automation aids, will be used for aircraft not capable of automated negotiation with ground systems.

#### Future ATM systems will significantly benefit the rapidly growing international air traffic operations.  The goal is to develop flexible operations by accommodating users' preferred trajectories to the optimum extent possible.  The future ATM systems will use automatic dependent surveillance, other data link applications, satellite communications, the global navigation satellite system and aviation weather system improvements to integrate ground-based automation and airborne flight management systems.

#### Potential benefits from improvements in ATM are:

- enhanced safety, reduced delays and increased airport capacity;

- more flexible ATM operations;

- enhanced surveillance capability;

- reduced congestion;

- more efficient use of airspace, including more flexibility and reduced separations;

- better accommodation of user-preferred profiles;

- enhanced weather information; and

- reduced controller workload.

### Expected Benefits of Data Link ATS

#### Significant benefits are expected to accrue from the implementation of a data link ATS.  These could include:

- Increased safety by reducing the potential for erroneous receipt of messages;

- Reduction of voice channel congestion;

- Reduction of R/T workload for both aircrew and controller;

- Increased communication availability;

- Reduction of late transfer of communications;

- Reduction of re-transmissions caused by misunderstood communications;

- Increased flexibility in handling ATC communication tasks;

- More efficient use of airspace due to more time being allocated to providing a better service to user aircraft, rather than routine communications tasks;

- Reduced controller stress/memory burden;

- Reduced controller communication time.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

CHAPTER 2 APPENDIX A

HUMAN FACTORS

2A.1	Summary

2A1.1	The success of the ICAO CNS/ATM systems concept will depend to a large degree upon its effective implementation into the operational environment.  The effectiveness of the implementation will be affected by a number of variables.  One such variable is the adoption of a systemic approach which takes into consideration facts and issues regarding all components of the CNS/ATM systems concept, rather than the technology involved exclusively.  Within this systemic approach, and in order to realize the CNS/ATM systems potential, Human Factors considerations should be included early in the design stage, before the systems and their subsystems achieve full operational status.

2A.2	Background and Justification

2A.2.1	Lapses in human performance underlie most safety breakdowns and damage-inducing events in modern, technology-based production systems, of which air transportation is a perfect example.  Measures to contain the adverse consequences of human error in aviation have traditionally followed a two-pronged approach.  They have been directed, piecemeal, either to the technology employed to achieve and improve the system's production goals, and/or to the front-line users and operators of this technology.  The contribution of improved technology to aviation safety and efficiency remains unparalleled in similar high-technology production systems.  The renewed attention dedicated to the human element in aviation over the last ten years has caused the last decade to be dubbed "the golden era of aviation Human Factors".  Nevertheless, aviation safety levels have remained fairly constant over the last twenty-five years and as such, the search for significant potential improvements continues.

2A.2.2	From the perspective of Human Factors, three reasons explain the apparent stagnation of safety levels.  The first reason can be found in what has been called an escalation of commitment: since World War Two, safety in civil aviation has been pursued through the introduction of new technology, supported by the training necessary to employ it in operational settings and the relevant regulations regarding both.  In every instance when accident investigations identified "new" safety breakdowns and/or hazards, more technology, more training and more regulations were introduced.  When "newer" safety breakdowns/hazards were further identified, more technology, further training and regulations were introduced.  And so continued the escalation of commitment of international civil aviation, with respect to technology, training and regulation.

2A2.3	Secondly,  technological solutions have on occasion been designed without full consideration of how they would properly interface with existing operational environments.  In this regard, the absence of a systemic approach to the integrated implementation of technological and Human Factors solutions has been conspicuous.  Technology and Human Factors have followed independent avenues, and little dialogue has existed among technology designers and Human Factors practitioners.  The industry has thus witnessed the emergence of fine technology which failed to deliver its promised potential because of serious flaws in its interface either with the human operator, with the demands of operational context, or with both.  The Ground Proximity Warning System (GPWS) illustrates this point:  the consequences of its piecemeal introduction are reflected in the fact that it has been necessary to change the original design seven times (latest versions of the equipment are Mk VII).  Not only is this expensive, but it breeds skepticism among users and therefore a state of affairs in which technology falls short of realizing its full safety potential.

2A.2.4	This approach, known as "technology-centred automation",  is being gradually phased-out in favour of a "human-centred automation", where technology is considered but a tool to assist humans in their monitoring and performing tasks.  Human Factors practitioners, on the other hand, have until recently been unable to convey the notion of the relevance of Human Factors knowledge to operational environments.  Understanding such relevance is essential for operational personnel and designers to include Human Factors knowledge in their professional "tool kits” and apply it in operational practice.  It appears a logical corollary that CNS/ATM systems safety and efficiency would be enhanced if its design and implementation observed a systemic approach which integrates both human capabilities and limitations, and technology.

2A.2.5	Thirdly, Human Factors knowledge has conventionally been applied in a reactive mode.  When investigations following accidents or safety breakdowns lead to the discovery of serious flaws in human performance due to inherent human limitations or are fostered by deficient human-technology interfaces, Human Factors knowledge is applied as a "band-aid” or "sticking plaster".  Action taken in this way addresses symptoms  (a discrete deficiency in human performance or in a discrete piece of equipment in a discrete operational environment) rather than causes.  This reactive application of Human Factors knowledge has traditionally fought a losing battle with the latent systemic failures which characteristically remain resistant and well-hidden in opaque high-technology systems in which, like in aviation, people must closely interact with technology to achieve the system goals.  The way to fight latent systemic failures is through the proactive application of Human Factors knowledge as part of prevention strategies to identify, assess and minimize the negative consequences of the system's potential hazards and the risks such hazards generate.

2A.3	Automation and New Technologies

2A.3.1	Automation of tasks through the introduction of technology is an attempt to increase the production of any given system, while maintaining or enhancing existing levels of safety and protection against harm and damage.  Aviation is not alone in the quest for increased production through automation, and similar endeavours have been attempted in other industries, including nuclear power generation, petrochemical, medicine and banking.  Automation of tasks through new technology should allow for increased safety through the reduction of human error and increased efficiency by enabling operational personnel to "do more with less".  Huge investments have been made in technological systems - in aviation as well as in other industries -which during their design stages appeared sound and appropriate to meet these objectives, and which when transported into the operational context, and interfaced with daily operations, did not deliver as expected.  Technology cannot then be easily changed because of claims that incidents or accidents involving it are due to design flaws which encourage the possibility of human error.  Such claims - no matter how well-founded they might be - assert their proponents, a prerogative to re-design not only the technology but also the jobs and responsibilities of operational personnel, and they involve astronomical costs which a production industry is hardly in a position to take lightly.  The implication of the above is clear: technology design should be context-conscious and human-centred from its inception.

2A.3.2	The piecemeal introduction of new technologies has on occasion been self-defeating, as the GPWS example illustrates.  Designers have automated what existing state of knowledge allowed to automate, leaving those tasks which could not be automated to be performed by the human.  Critical in this approach is the assumption that the human operator will take and restore effective control of the system in "runaway" conditions; in unexpected operational conditions which controlling computers cannot "understand" because designers did not anticipate such operational conditions could emerge.  The irony behind this approach is obvious: humans are expected to monitor the automated system and take over manually to restore the system to safety when facing operational conditions not forecast by design and for which they are neither trained nor prepared.

2A3.3	There has also been a tendency to automate what engineers believed should be automated in order to better assist operational personnel.  More often than not, such beliefs developed without the benefit of feedback from the final users of the technology, without proper understanding of the issues involved and without due consideration to potential inter-relationships between the automated system and the operational context limitations.  From a technology-centred point of view, incidents and accidents involving high technology systems appear to be mis-operations of engineered systems that are otherwise fully functional, and are therefore labeled as human error.  The typical belief is the human element is separate from the technology, and that problems reside therefore either in the human or in the technical part of the system.  This view ignores, among other things, the role of human cognition and the pressures managers and oftentimes regulators impose upon operational personnel.

2A.3.4	In practice, things are different.  The attribution of error is a judgment about human performance, applied only when a process (i.e., an operation) has a bad outcome (i.e., an accident or incident), and usually with the benefit of hindsight about the outcome.  The role of technology in fostering human error has been often overlooked.  Seldom the problem lies with the technology design in itself,  most frequently it is a result of a poor mismatch between the technology, its users and the operational context.  The limitations of human cognition vis-à-vis the use of new technology have not always been fully appreciated.  It is only in the examination of these deeper issues that it is possible to learn how to improve the integrated performance of large and complex systems, and how to incorporate these lessons pro-actively during the design of technology.

2A.3.5	If new technology is to be successfully implemented, the following must be considered during design, in addition to the technology's inherent properties:

- the operational context in which the technology will be deployed,

- the human performance-shaping potential which technology carries, since it creates the potential for new forms of error and failure,

- the fact that use of technology will be shared by interacting people,

- the organizational context with its constraints, dilemmas, trade-offs, double and multiple binds and competing goals, and

- the role of cognitive factors which may turn otherwise efficient into "clumsy" technology.

2A.3.6	Technology and human capabilities and limitations must in fact be considered as a joint human-machine cognitive system.  Designing technology without consideration of the above will yield compromise rather than optimum benefits, and may indeed become an invitation to disaster.

2A.4	Cognition and New Technologies

2A.4.1	The demands that large and complex systems place upon human performance are essentially cognitive.  The "clumsy" use of new technological possibilities in the design of computer-base devices create the potential for erroneous actions and assessments by operational personnel when combined with inherent human limitations encouraged by the pressures of real-life contexts.  Some of the questions to be considered in designing "joint cognitive systems" include:

- What are the "classic" design errors in human-computer systems, computer-based advisors, and automated systems?

- Why are they so frequently observed in operational environments?

- How do devices with "classic" design errors shape operational personnel cognition and behaviour?

- How do practitioners cope with "clumsy" technology?

- What do these factors mean in terms of human error?

2A.4.2	The most frequent example of mismatches between cognition and technology is the mode error.  Mode error requires a device where the same action or indication means different things in different contexts (i.e., modes),  and a person who loses track of the current context.  It is obvious that it is an error which can only exist at the intersection of people and technology.  If the "joint cognitive system" is duly considered, mode errors can be pro-actively anticipated and taken care of during design.

2A.4.3	At any time when new technology is introduced there is potential for safety breakdowns within the system in question.  Technological change is an intervention into an ongoing field of activity.  Developing and introducing new technology does not preserve the old ways of doing business in the ongoing field of activity, with the simple substitution of one tool for another (i.e., replacing a typewriter for a personal computer).  It represents entirely new ways of doing things, including the composition of working teams and a shift in the human role within the joint human-technology system.  Failures in automated systems produce considerably more side effects than manual systems.  Symptoms of faults may seem unrelated to the process taking place, making management and diagnosis more difficult and changing the kind of failures operational personnel would expect to see.  When facing such a panorama, it appears obvious that the long standing hobbyhorses of aviation to support change - training and regulations - are not sufficient when introducing new technology.  An integrated approach is essential.

2A.5	The Principles Of Human-Centred Automation

2A.5.1	The advantages of incorporating Human Factors considerations early in system design cannot be overstated.  As mentioned elsewhere in this appendix, the principles of human-centred automation require that the industry embrace a system approach to the design of automation systems.

2A.5.2	The human bears the ultimate responsibility for the safety of the aviation system.

2A.5.2.1	In a complex system, no matter how automated, the human has the last vote in deciding a critical issue and the human is the last line of defense in case of system breakdown.  The importance of people in a technological society is further reflected in the concept of pivotal people, which emphasizes the irreplaceability of pivotal people in stressful environments like flight operations, air traffic control, and power utility grid control.

2A.5.3	The human operator must be in command.

2A.5.3.1	For humans to assume ultimate responsibility for the safety of the system, they should be conferred with essentially unlimited authority to permit them to fulfill this ultimate responsibility.  It has been unequivocally stated that even when the automated system is in full operation, “responsibility for safe operation of an aircraft remains with the pilot-in-command,” and “responsibility for separation between controlled aircraft remains with the controller”.  If they are to retain the responsibility for safe operation or separation of aircraft, pilots and controllers must retain the authority to command and control those operations.  It is the fundamental tenet of the concept of human-centred automation that aviation systems (aircraft and ATC) automation exists to assist human operators (pilots and controllers) in carrying out their responsibilities as stated above.  If this principle is not strictly observed, and if decisions are made by automated systems instead of by human operators, complicated and unavoidable liability issues may arise.  This will obviously lead into consideration of the human operator's share of liability, which in turn will adversely affect human performance.  Thus, a question of liability becomes a Human Factors issue by default.  Human operators should never be held liable for failures or erroneous decisions unless they have full control and command of the system.  The reasons are very simple ¾ like any other machine, automation is subject to failure.  Further, digital devices fail unpredictably, and produce unpredictable manifestations of failures.  The human's responsibilities include detecting such failures, correcting their manifestations,  and continuing the operation safely until the automated systems can resume their normal functions.  Since automation cannot be made failure-proof, automation must not be designed in such a way that it can subvert the exercise of the human operator's responsibilities.

2A.5.4	To command effectively, the human operator must be involved.

2A.5.4.1	To assume the ultimate responsibility and remain in command of the situation, human operators must be involved in the operation.  They must have an active role, whether that role is to actively control the system or to manage the human or machine resources to which control has been delegated.  If humans are not actively involved, it is likely that they will be less efficient in reacting to critical system situations.  Human-centred aviation system automation must be designed and operated in such a way that it does not permit the human operator to become too remote from operational details, by requiring of that operator meaningful and relevant tasks throughout the operation.

2A.5.5	To be involved, the human must be informed.

2A.5.5.1	Without information about the conduct of the operation, involvement becomes unpredictable and decisions, if they are made, become random.  To maintain meaningful involvement, the human operator must have continuing flow of essential information concerning the state and progress of the system controlled and the automation that is controlling it.  The information must be consistent with the responsibilities of the human operator; it must include all the data necessary to support the human operator's involvement in the system.  The human operators must be prominently informed at the level required to fulfill their responsibilities.  The human operators must have enough information to be able to maintain state and situation awareness of the system.  However, care must be taken not to overload them with more information than is necessary.

2A.5.6	Functions must be automated only if there is a good reason for doing so.

2A.5.6.1	There is a growing temptation to incorporate some new technology showpiece in a design just because it can be done rather than because it is necessary.  In other words, designs may be driven by technological feasibility rather than the needs of the users who must operate and maintain the products of these designs.  Automation of functions for no other reason except that it is technologically possible may result in the user's inability to effectively employ it for the benefit of the whole system.  The question here should be “not whether a function can be automated, but whether it needs to be automated, taking into consideration the various Human Factors questions that may arise”.

2A.5.7	The human must be able to monitor the automated system.

2A.5.7.1	The ability to monitor the automated systems is necessary both to permit the human operator to remain on top of the situation, and also because automated systems are fallible.  The human can be an effective monitor only if cognitive support is provided at the control station.  Cognitive support refers to the human need for information to be ready for actions or decisions that may be required.  In automated aviation systems, one essential information element is information concerning the automation.  The human operator must be able, from information available, to determine that automation performance is, and in all likelihood will continue to be, appropriate to the desired system situation.  In most aviation systems to date, the human operator is informed only if there is a discrepancy between or among the units responsible for a particular function, or a failure of those units sufficient to disrupt or disable the performance of the function.  In those cases the operator is usually instructed to take over control of that function.  To be able to do so without delay, it is necessary that the human operator be provided with information concerning the operations to date if these are not evident from the behaviour of the system controlled.

2A.5.8	Automated systems must be predictable.

2A.5.8.1	The human operator must be able to evaluate the performance of automated systems against an internal model formed through knowledge of the normal behaviour of the systems.  Only if the systems normally behave in a predictable fashion can the human operator rapidly detect departures from normal behaviour and thus recognize failures in the automated systems.  It is important that not only the nominal behaviour, but also the range of allowable behaviour be known.  All unpredicted system behaviour must be treated as abnormal behaviour.  To recognize this behaviour, the human operator must know exactly what to expect of the automation when it is performing correctly.

2A.5.9	Automated systems must also be able to monitor the human operator.

2A.5.9.1	Humans failures may likewise be unpredictable.  Because human operators are prone to errors, it is necessary that error detection, diagnosis and correction be integral parts of any automated aviation system.  For this reason, it is necessary that human as well as machine performance be continuously monitored.  Monitoring automation capable of questioning certain classes of operator's actions that can potentially compromise safety must be designed into the system.

2A.5.10	Each element of the system must have knowledge of the others' intent.

2A.5.10.1	In highly automated operations, one way to keep the human operator actively involved is to provide information concerning the intent of the automated system.  That is, given the current decisions made or about to be made by the automated systems, what will the situation look like in the future.  Essentially, the system should not only identify a potential problem but also suggest alternative solutions and show the implications of the action taken.  Cross-monitoring can only be effective if the monitor understands what the operator of the monitored system is trying to accomplish.  To obtain the benefit of effective monitoring, the intentions of the human operator or the automated systems must be known.  The communication of intent makes it possible for all involved parties to work co-operatively to solve any problem that may arise.  For example, many air traffic control problems occur simply because pilots do not understand what the controller is trying to accomplish, and the converse is also true.  The automation of the ATC system cannot monitor human performance effectively unless it understands the operator's intent, and this is most important when the operation departs from normality.

2A.5.11	Automation must be designed to be simple to learn and operate.

2A.5.11.1	If systems are sufficiently simple automation may not be needed.  If tasks cannot be simplified, or are so time-critical that humans may not be able to perform them effectively, automation may be the solution.  Even then, simpler automation will permit simpler interfaces and better human understanding of the automated systems.  Systems automation to date has not always been designed to be operated under difficult conditions in an unfavourable environment by overworked and distracted human operators of below-average ability.  Yet these are precisely the conditions where the assistance of the automation system may be most needed.  Simplicity, clarity and intuitiveness must be among the cornerstones of automation design, for they will make it a better and effective tool.  Simple, easy to learn and use design is marked by an absence of problems in the use of a system by humans and its effects are thus invisible in the final operational system.

2A.5.11.2	The principles of human-centred automation are intended to serve as a template so that every time automation is designed and introduced it can be filtered through the template rather than justified and defended anew.

2A.6	Human Factors Issues Related To Data Link Applications

2A.6.1	Human Factors issues related to specific data link applications are described under the headings of those applications.  General Human Factors issues, to be considered with the guidance material listed in the reference section of this appendix, include:

- the level of safety targeted for the future system should be defined not only with reference to various system statistics, but also with reference to error-inducing mechanisms related to human capabilities and limitations, as well as individual cases;

- the definition of system and resource capacity should include reference to the responsibilities, capabilities and limitations of ATS personnel and flight crews, who must retain situational awareness in order to discharge their responsibilities as indicated in the principles of human-centred automation;

- the provision of large volumes of information to users should be limited to what is absolutely necessary, and should be mediated by methods that effectively package and manage such information to prevent information overload while providing pertinent information to particular operational needs;

- the responsibilities of pilots, air traffic controllers and system designers should be clearly defined prior to the implementation of new automated systems and tools;

- services and procedures will be provided to ensure the preservation of situational awareness for both data link and non-data link-equipped aircraft and ground facilities;

- when operating a data link system, there should be no increase in heads-down time that would adversely affect safe operation;

- voice communication to supplement data link system operation will be available; and

- maximum use of data link will not impose undue competition for display or control resources.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# INTRODUCTION TO DATA LINK APPLICATIONS

## Data Link Based Air Traffic Systems

### CNS/ATM systems for use in the future will be developed from existing systems and technologies in an evolutionary manner.  One of the overall objectives is to harmonize the different air traffic control systems among the regions, irrespective of the surveillance, navigation and communications systems in use.  Data link communications can support direct controller-pilot communication, the passing of automatic dependent surveillance data, the implementation of a request/reply digital flight information service to the aircraft, and exchanges between aircraft and ATC systems.  This will overcome the shortcomings of the current systems by providing for global communications, navigation and surveillance coverage from (very) low to (very) high altitudes, for digital data interchange between the air-ground systems to fully exploit the automation capabilities of both, and for the development of a fully integrated CNS end-system which will operate in a normalized manner throughout the world.

### The data link applications based system will improve the handling and transfer of information between operators, aircraft and ATS units.  The system will provide extended surveillance capabilities by using ADS and advanced ground-based data processing and display systems to the controller, thus allowing advantage to be taken of the improved navigation accuracy in four dimensions and accommodating the flight's preferred profile in all phases of flight, based on the operator's objectives.  The future data link based ATC system will also allow improved conflict detection and resolution, as well as the automated generation and transmission of conflict-free clearances and rapid adaptation to changing traffic and weather conditions.

### In oceanic areas and remote land airspaces with limited ground-based air navigation facilities, surveillance of air traffic is envisioned to be provided by ADS position reporting through satellite communications.  Surveillance of low-altitude traffic operations, including helicopters,  will be conducted in a similar manner.  In continental airspaces, surveillance of air traffic may be achieved by ADS reports integrated with ground-based radar systems.  Controller Pilot Data Link Communication (CPDLC) and the interchange of ATS messages will be carried out by satellite, SSR Mode S, VHF, high frequency (HF) or other suitable data link available.

## System Components

### There are six major components which combine to form an integrated data link based ATC system.  Implementation of data link must allow incorporation of system enhancements to be made without any disruption to operations.  The six main components of a data link based ATS are:

- pilot interface;

- aircraft (including airborne automation);

- air/ground and ground/ground data link communications;

- communication interface;

- ATC automation; and

- controller interface.

### Pilot Interface

#### The pilot interface to the data link system must be efficient and easy to operate.  Pilot-controller messages require some rapid entry mechanism.  Use of data link for pilot-controller communications will result in changes to cockpit procedures, since messages currently transmitted by voice will require system input by the pilot, and receipt of a message will require reading text.  Procedures and systems should be developed to minimize system input errors.

#### Possible impact of loss of situational awareness for flight crews needs to be considered.

### Aircraft Equipment

#### Data link applications must be supported by aircraft equipment which is able to gather the data from the pilot interface, appropriate sensors and flight management computers, format the data and direct it to the appropriate air-ground data link within the appropriate time scales.  This on-board equipment should also have the capability to receive messages originated by the controlling and other authorized ATS units.  Avionics should make maximum use of data link equipment already in place in the aircraft.

### Air/Ground and Ground/Ground Data Link Communications

#### The required air-ground data link will be ATN compatible for most applications and could be either satellite data link, VHF digital data link, Mode S data link, or any other medium which meets the operational requirements.  The ATC and aircraft systems will select the most suitable path based on time-varying considerations such as geographical location, cost, delay, throughput and link availability.  For example, in oceanic airspace, satellite data links will most likely be used, while in domestic airspace VHF or Mode S could be used.

#### The resulting communications links will appear seamless from the user's perspective  (i.e., independent of the communications systems in use).

#### Voice communication will be available to complement data link system operation.

#### To satisfy the operational requirements, the communications system will need to meet general performance standards.  These are summarized in Appendix A of this chapter.

### Ground Communication Interface

#### The air-ground data link will be connected to the ATC system through a terrestrial communications network.  The network will conform to the protocol suite defined as part of the ATN concept.  For messages from controller to pilot, the ground ATN routers must choose the most suitable data link device available and route the message to that transmitting station.

### ATC Automation

#### The ground system must be capable of supporting position reporting and communications procedures with minimal controller input, conformance monitoring, confliction avoidance, automatic transfer of control, controller alerting and many other functions concerned with safe and efficient ATS management will result from the incorporation of advanced levels of automation which will take advantage of the data link applications functionality.  CPDLC will require some level of message processing that should be included in the ATC automation component.

#### Error detection and correction, and, where appropriate, alerting mechanisms should be implemented.

#### In addition, the ATC system will allow for safe recovery from response delays, non-response, system failures, system management errors or others errors which impede operation such as unauthorized access and unauthorized transmission.  Systems will be capable of delivering messages associated with error notification and recovery within the time required for safe recovery.

#### Use of data link will not impose undue competition for display or control resources.  Systems will not preclude access to other functions or unduly conflict with higher priority functions.

### Controller Interface

#### The controller interface will contain the required tools for the composition of air-ground data link messages.  Air traffic services providers will define and develop specific controller interfaces tailored to their particular needs.  The human-machine interface will be left to the individual service provider.  The controller interface should be efficient, easy to operate and provide a rapid message input mechanism.  The interface should also provide a means to display air-ground messages.  Further guidance on the Human Factor aspects which have to be taken into consideration generally is given in Part I Chapter 2 Appendix A.

## Operational Requirements

### Certain basic operational requirements need to be fulfilled to permit the effective provision of a data link based ATS.  In this context, an operational requirement is defined as:

“A statement of the operational attributes of a system needed for the effective and/or efficient provision of air traffic services to users.”

### In deriving the operational requirements necessary for the provision of a data link based ATS, the various information needs of the pilot, aircraft, ground system and controller must be taken into account.  The performance of the various components of the system, outlined in para. 3.2.1 above, has also to be considered.  Specific operational requirements have been identified which are applicable to the global data link applications environment.

### Additional operational requirements may be developed to increase the efficiency and effectiveness of the systems as the systems evolve.  Currently defined requirements should not always be considered as being immediately needed to permit an initial level of a data link based ATS.  In the transition towards a fully developed service, limited conformance could still provide benefits to the users and to ATC providers.  This will ensure that implementation of services can proceed on an evolutionary basis.  More information on a possible transition strategy is given in Appendix B of this Chapter.

### Operational Requirements (ORs) which are generally applicable to all aspects of a data link based ATS are given in this chapter.  Where ORs are specific to particular applications, they are included in the relevant chapters of this part of the document.

### In this document an operational requirement is not a description of “how” the system need is to be met, nor should it be assumed that the solution will always be technical in nature.  It may be that the requirement will be met by appropriate procedural training or staffing actions.  However, when technical solutions are required, their definition remains the responsibility of the technical and engineering specialists who must ensure that the needs of the ATC system are met.  Close coordination between operational and technical authorities is essential if the optimum solution is to be found.

### A data link based ATS must include the capability to exchange messages between the pilot and the controller.  A direct voice communication capability should be available for at least emergency and non-routine, safety-related communications.  In order to cater for emergency situations, the system will provide for a pilot (or, exceptionally, system) initiated ADS emergency mode, which would indicate the state of emergency, and include a basic ADS report.

### The ADS Panel has identified a specific operational requirement relating to the overall implementation of the data link based ATS, outlined below.

## Generic Operational Requirements

### In any data link dialogue the end-user must be able to positively identify the other end-user.

### In any data link based ATS, provision must always be made for direct pilot-controller voice communications.

#### In particular, the pilot or controller must be capable of initiating direct controller pilot communication by voice in emergency or urgent, non-routine, safety-related situations.

#### Simple actions will be used by either the pilot or controller to initiate voice communications.  Voice communications will be of high-quality intelligibility.

#### An emergency call must always pre-empt a lower priority call.

### In addition to the specific requirements given above, several significant functional requirements have been identified concerning the overall level of sophistication required to permit effective implementation of data link in a CNS/ATM environment, as envisaged by FANS.

#### Air traffic control facilities providing a data link based ATS must be capable of receiving, storing, processing, displaying and disseminating specific flight information relating to flights equipped for and operating within environments where a data link service is provided.

#### Aircraft intending to fly within airspace where a data link service is available and wishing to take advantage of the service must be equipped with data link capabilities to permit the exchange of data link messages between the aircraft and the ATC facilities providing the service.

#### Effective human-machine interfaces must exist on the ground and in the air to permit inter-activity between the pilot, controller and ground automation.

#### Design of appropriate digital data interchange communications systems between ATC facilities is also significant to the effective implementation of a data link service.  Effective digital data interchange communication systems, techniques and procedures should be developed in parallel with the ADS-specific requirements.

#### Aircraft will be under the control of only one ATC unit, whether or not ADS is being used.  ADS reports will be made available to facilities other than the controlling ATC unit on the basis of mutual agreement and local procedures.

#### The system should be capable of facilitating automatic transfer of data link authority within data link based ATS airspace using digital data interchange.  Two cases exist:

##### The first case is where applications are transferred from the releasing unit to the accepting unit by ground/ground communications, prior to transfer of control.  This is an extension of current inter-facility procedures.

##### The second case is where the receiving unit establishes its own ADS contract prior to the boundary, with transfer of control being effected according to the inter-unit procedural agreement.

## Communications Failure

### In case of complete communications failure, procedures will be in accordance with ICAO provisions.

### In the event of an unexpected termination of a data link application, both the aircraft and the ground will be notified of the failure.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

CHAPTER 3 APPENDIX A

COMMUNICATION SYSTEMS PERFORMANCE REQUIREMENT PARAMETERS

3A.1	General Requirements

3A.1.1	In addition to the requirements specified in the application parts of this document, all data link applications require:

- the probability of non-receipt of a message will be 10-6;

- the probability that non-receipt of a message will fail to be notified to the originator will be 10-9; and

- the probability that a message will be mis-directed will be 10-7.

3A.2	Performance Requirements

3A.2.1	The figures in Tables 3A-1 and 3A-2 reflect the various levels of performance that may be selected for the purpose of providing Data Link services.  Depending on the level of service to be provided, a given State can determine what the performance needs are in a given domain by factors such as the separation minima being applied, traffic density, or traffic flow.

| Performance Levels | Mean End-to-End Transfer Delay (Seconds) | 95% End-to-End Transfer Delay (Seconds) | 99.996% End-to-End Transfer Delay (Seconds) |
| --- | --- | --- | --- |
| A | 0.5 | 0.7 | 1 |
| B | 1 | 1.5 | 2.5 |
| C | 2 | 2.5 | 3.5 |
| D | 3 | 5 | 8 |
| E | 5 | 8 | 12.5 |
| F | 10 | 15 | 22 |
| G | 12 | 20 | 31.5 |
| H | 15 | 30 | 51 |
| I | 30 | 55 | 90 |
| J | 60 | 110 | 180 |

Table 3A-1:  Transfer Delay Performance Requirements

| Application | Availability | Integrity | Reliability | Continuity |
| --- | --- | --- | --- | --- |
| DLIC | 99.9% | 10-6 | 99.9% | 99.9% |
| ADS | 99.996% | 10-7 | 99.996% | 99.996% |
| CPDLC | 99.99% | 10-7 | 99.99% | 99.99% |
| FIS | 99.9% | 10-6 | 99.9% | 99.9% |
| AIDC | 99.996% | 10-7 | 99.9% | 99.9% |
| ADS-B | 99.996% | 10-7 | 99.996% | 99.996% |

Table 3A-2:  Application Specific Performance Requirements

3A.2.2	Except in catastrophic situations, no single end-to-end outage should exceed 30 seconds (end-to-End availability may be achieved through provision of alternate communications routings where feasible).

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

CHAPTER 3 APPENDIX B

TRANSITION STRATEGY

3B.1	Transition Principles and Guidelines

3B.1.1	General principles

3B.1.1.1	The phased implementation of a data link based system will take place within an environment which places constraints and conditions on the process, namely:

- A future system can only be implemented by means of an evolutionary process.  This process has already begun;

- The impact on the pilots and air traffic controllers of human factors issues must be resolved;

- The new system must be capable of working with a wide variety of traffic densities, aircraft types, aircraft sophistication, etc.;

- The system should be protected against unauthorized access and unauthorized transmission;

- The system should be able to accommodate increased demands and future growth;

- The system should, to the extent practicable, be able to accommodate aircraft which have begun transition to these technologies; and

- Evolving systems should be able to accommodate variations in quality standards and performance characteristics.

3B.1.2	The implementation plan for a specific area will also be based on the specific requirements in that area.  The benefits to be gained from the use of data link are not the same in all areas, and implementation will be based on cost-benefit considerations and the need for overall coverage and compatibility with neighbouring areas.

3B.1.3	Not all operational requirements for a data link based ATS would need to be implemented at the same time.  However, an incremental level of capability, implemented in phases, would occur in many cases and would be in keeping with the established transition guidelines.  For example, the simplest implementation of ADS, providing position reporting only, could be used in combination with existing communications facilities.  The mere availability of a surveillance capability would provide significant safety benefits, as it would permit the detection by the controller of deviations from the cleared flight path, and thus prevent gross navigation errors.

3B.1.4	In some cases, ADS may be first introduced for specific operations in limited areas to enhance safety and efficiency, for example, for low level helicopter operations below radar coverage.  In order to be compatible with more extensive data link based ATSs and to gain experience with the standardized system, these early implementations should, as much as possible, meet the operational requirements as set out in this document.

3B.1.5	Meeting the future requirements of a full data link based ATC operational system necessitates the development of automation for ATM functions.  In order to derive maximum benefits for controllers, the design of the ATM system for a data link based service should pay close attention to the impact on the human factor aspects of the controller’s work environment, as well as the validation methods used in the development of automated functions.

3B.1.1.6	It is impossible to provide a uniform environment for all users in all States and regions in the same time frame.  However, criteria must be established and a phased implementation must be developed to maximize the benefits as quickly as possible, with the least disruption.

3B.2	Implementation Principles

3B.2.1	The global data link system should be developed in balance with other parts of the overall air navigation infrastructure.  The necessary changes should be introduced in an evolutionary fashion.  There should be little disparity in the level of service given to differently-equipped aircraft in order to provide an expeditious flow of air traffic.  System development must be harmonized to enable future technologies to be accommodated in a consistent manner throughout the globe.

3B.2.2	Guidelines for transition to the future surveillance system should be such as to encourage early equipage by users through the earliest possible accrual of the system benefits.  Although a transition period during which dual equipage, both airborne and ground, will be necessary in order to ensure the reliability and availability of the new system, the guidelines should be aimed at minimizing this period to the extent practicable.

3B.2.3	The following transition guidelines have been developed by the ICAO FANS (II) Committee and are based on the “Global Co-ordinated Plan for Transition to the ICAO CNS/ATM Systems”.

- States should begin to develop operational procedures, in accordance with ICAO SARPs and guidelines, for the implementation of ADS within airspace under their control.

This will ensure early implementation of ADS and the most efficient use of global airspace.

- Transition to data link based operations should initially begin in oceanic airspace and in continental enroute airspace with low-density traffic.

In oceanic and some continental areas, position reporting is the only available means of surveillance, and ADS could provide a significant early benefit.  In some oceanic areas where HF communication congestion occurs, a combination of CPDLC and ADS will provide relief.

- States and/or regions should coordinate to ensure that, where data link ATM is to be introduced, it is introduced approximately simultaneously in each FIR where major traffic flows occur.

This will help ensure seamless transitions through FIR boundaries, and that the benefits of data link application will be available to suitably-equipped aircraft.

- Where differing surveillance methods are employed in adjacent States or FIRs, commonality or compatibility of systems should be developed to enable a service which is transparent to the user.

This will help ensure seamless transition through State and FIR boundaries.

- During the transition period, after, say, an initial ADS position reporting is introduced, the current levels of integrity, reliability and availability of existing position reporting systems must be maintained.

This is necessary to back-up ADS and to accommodate non-ADS-equipped airspace users.

- States and/or regions should take actions within the ICAO frame work to ensure that implementation of procedural changes due to ADS and other systems results in more efficient use of airspace.

Procedural changes may include reduction of horizontal separation standards in oceanic airspace.

- During the transition to data link based operations, suitably equipped aircraft should be given precedence over non equipped aircraft for preferred routes and airspace.

For the longer term, when aircraft and ATC capabilities permit, organized track structures could be eliminated in favor of user-preferred flight trajectories with, for example, ADS surveillance and conflict probing.

- Data link ATM should be introduced in phases.

This will facilitate rapid introduction of data link capabilities.  The first phase could introduce ADS position reporting, conflict probing and flight plan conformance monitoring by ATC, and two-way satellite communications with an initial set of pre-defined message formats.  Later stages can introduce a more complete set of pre-defined message formats and further ATC automation.

- ADS equipment, standards and procedures should be developed in such a way as to permit the use of ADS as a back-up for other surveillance methods.

This is in accordance with the ICAO CNS/ATM systems for back-up of surveillance systems.

3B.2.4	During the transition period, towards the greater use of data link, the need for increased training in both the use of the data link message set and in the maintenance of the use of aviation specific English should not be overlooked.

3B.2.5	This is not a one time training need.  In order to maintain familiarity with the message set, and to retain competence in international voice communication, a suitable training program must be maintained during the transition period.

3B.3	Planning for Implementation

3B.3.1	Planning for the new overall CNS system is underway on global and a regional basis.  ICAO has developed a global plan for transition, which provides overall guidance to States and regions for their planning activities.  Several States and regions are actively progressing their planning towards implementation of the CNS/ATM systems, within the framework of the global plan, to realize the anticipated benefits from safety enhancement and capacity improvements.

3B.3.2	The transition to a data link based ATS service should ideally be accompanied by improvements in ATM and should be through structural and procedural changes that will enhance the service and provide benefits to users.  The structural changes involve airspace reorganization required to optimize the new service.  Other areas that will need to be addressed are:

- data link media,

- message formats,

- separation,

- automation,

- ATC procedures, and

- end-to-end verification and certification.

3B.4	Validation and Early Implementation

3B.4.1	The six component parts of the data link based ATS system described in Part I, Chapter 3 for the most part have been adequately demonstrated as being viable.  There is an urgent need for States and other organizations, in a position to do so, to undertake trials and implementation of pre-operational systems, as soon as practical, with a view to facilitating a timely implementation of a fully operational system.

3B.4.2	It is recognized that during transition, interim systems may require some modifications to previous practices and procedures.  Some States will develop ATS procedures which will support interim systems until such time as the users move to a fully ICAO compliant implementation.

3B.4.3	Timely implementation of operational data link based systems in specific areas would enable early benefits to be derived and provide further incentive to airspace users to equip their aircraft.  In addition, interim regional implementation has already brought benefits to users.

3B.4.4	In continental airspace, the implementation of ADS as a back-up or complement to radar can have benefits in terms of surveillance availability and reduced need for overlapping radar coverage, but will also be an essential transition step in the process of evaluating and establishing operational requirements for using ADS as a primary means of surveillance.

3B.4.5	The regional bodies most affected by the implementation of a data link based ATM system should be provided with the necessary material.  This may then be used in giving consideration to the use of ADS, including phased implementation in their forward planning.  For example, the North Atlantic Facilities and Services Implementation Document (FASID) outlines the steps to a to a full ADS environment.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# DATA LINK INITIATION CAPABILITY (DLIC) Overview and HIGH LEVEL Operational Requirements

## DLIC Overview

### The DLIC provides the necessary information to enable data link communications between ATC ground and aircraft systems.  It is an aircraft initiated application.  The DLIC encompasses the following functions:

- Logon:  data link application initiation and, if required, flight plan association,

- Update:  updating of previously coordinated initiation information,

- Contact:  instructions to perform data link initiation with another specified ground system,

- Dissemination:  local dissemination of information, and

- Ground Forwarding:  ground-ground forwarding of logon information.

### The ADS Panel has developed specific operational requirements for the establishment of data link communications between an aircraft and ground systems.  These requirements, and the method of operation, are outlined below.

## DLIC High Level Operational Requirements

### The ground system must be able to identify an aircraft's data link capabilities from the filed flight plan.

#### Data link units need advance notification of aircraft equipage in order to assign appropriate ADS contracts.  Prior to the aircraft entering ADS airspace, the relevant ATC unit's ground system database will be updated to reflect the aircraft equipage from data included in the received flight plan.

#### The pilot will include details about data link capabilities in the flight plan.

### Procedures must be in place to allow timely establishment of data link between aircraft and the ground system.

#### Before entering an airspace where the data link applications are provided by the ATC automation system, a data link connection will need to be established between the aircraft and the ground system in order to register the aircraft and allow the start of a data link dialogue when necessary.  This will be initiated from the aircraft, either automatically or by pilot intervention.

#### At a time parameter before an data link equipped aircraft enters data link airspace, the pilot or the aircraft will need to initiate the DLIC logon procedure.  The aircraft will then generate and transmit the Logon Request message which contains the aircraft-unique identifier and the data link applications it can support.  The ground system responds to the aircraft's logon request.

#### The ground system should be able to correlate the aircraft-unique identifier with the aircraft identification stored in its data base.

### During the initial establishment of a data link connection with a ground system, that ground system must be able to register the communications and surveillance capabilities of the aircraft

#### The ground system will identify the communications and surveillance capabilities of aircraft in order to establish appropriate ADS contracts.

### The ground system initially contacted by the aircraft should be able to pass the necessary aircraft address information to another ground station via ground/ground communications links.

## Detailed Implementation

### Detailed implementation and more specific operational requirements of the DLIC functionality in a data link based ATM system are given in Part II of this manual.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# AUTOMATIC DEPENDENT SURVEILLANCE APPLICATION (ADS) Overview and HIGH LEVEL Operational Requirements

## ADS Application Overview

### The implementation of ADS, through reliable data link communications and accurate aircraft navigation systems, will provide surveillance services in oceanic airspace and other areas where non-radar air traffic control services are currently provided.  The implementation of ADS will also provide benefits in en-route continental, terminal areas and on the airport surface.  The automatic transmission of the aircraft position through ADS will replace present pilot position reports.  In non-radar airspace, the effective use of ADS in air traffic services will facilitate the reduction of separation minima, enhance flight safety and better accommodate user-preferred profiles.  The ADS application and associated communications will have to be supported by advanced airborne and ground facilities and data link communications with proven end-to-end integrity, reliability and availability.

### The ADS application allows the implementation of reporting agreements, which, with the exception of an aircraft in an emergency situation, are established exclusively by the ground.  An ADS agreement is an ADS reporting plan which establishes the conditions of ADS data reporting (i.e., data required by the ATC system and the frequency of the ADS reports which have to be agreed prior to the provision of the ADS services).  The terms of an ADS agreement will allow for information to be exchanged between the ground system and the aircraft by means of a contract, or a series of contracts.  An ADS contract specifies under what conditions an ADS report would be initiated, and what data groups will be included in the reports.  There are three types of contract - ‘demand’, which provides a single report, ‘periodic’, which provides a report at a regular periodic interval determined by the ground system, and ‘event’, which provides a report when or if a specified event or events take place.

### ADS contracts necessary for the control of the aircraft will be established with each aircraft by the relevant ground system, at least for the portions of the aircraft flight over which that ground system provides ATS.  The contract may include the provision of basic ADS reports at a periodic interval defined by the ground system with, optionally, one or more additional data blocks containing specific information, which may or may not be sent with each periodic report.  The agreement may also provide for ADS reports at geographically-defined points such as waypoints and intermediate points, in addition to other specific event-driven reports.

### The aircraft must be capable of supporting contracts with at least four ATSU ground systems simultaneously.

### An ADS application can only be provided by an ATSU having appropriate automation and communication facilities.  The ADS application should be supported by direct two-way controller pilot data link and voice communication.

### The transition to the use of ADS in ATS needs careful consideration and should be based on safety, cost-benefit and feasibility studies.  The transition plan should consider the time required for acquisition of ATC and communication systems, the number of aircraft suitably equipped and the time for operators to equip, and the time frames of implementation in neighbouring FIRs.

### Implementation of ADS will overcome limitations found today in procedural ATC systems based on pilot-reported position reports.  The introduction of air-ground data links through which the ADS reports and associated messages will be transmitted, together with accurate and reliable aircraft navigation systems, presents the opportunity to improve surveillance of aircraft in those airspaces.  It offers the potential for increasing flight safety and airspace utilization by reducing ATC errors in air-ground communications and by providing ATC with accurate aircraft position information.  The exchange of ATS messages by digital data link will alleviate the overloading of ATC radio frequencies and support ATC automation, as well as the implementation of other ATS data link applications.

### The processing of automated position reports will result in improved automatic monitoring of aircraft operations.  Automatic flight plan data validation will facilitate the early detection by ATC of on-board system flight and route data insertion errors.  Conflict prediction and resolution capabilities will be supported through the use of preferred route profiles.  The display of the traffic situation as derived from ADS reports and the automated processing of ATS safety messages will significantly improve the ability of the controller to respond to pilot requests and to resolve traffic situations.

### With a combination of improved ATC automation, reliable communications and accurate navigation and surveillance, it will be possible to increase the level of tactical control and to reduce separation minima on the basis of controller intervention capability and other ATM improvements, thereby leading to possible increases in airspace capacity.

### As with current surveillance systems, the benefit of ADS for ATC purposes requires supporting complementary two-way controller pilot data and voice communication (voice for at least emergency and non-routine communication).  Where VHF coverage exists, the communication requirement is envisaged to be met by VHF voice.  In areas where high frequency (HF) communications are currently used (e.g., oceanic airspace), the provision of an ADS service during the en-route phase of flight will be supported by the routine use of CPDLC.

### The ADS Panel has developed specific operational requirements to ensure that the ADS element of the overall data link service achieves the necessary performance to allow its implementation into a data link based ATS.

## ADS High Level Operational Requirements

### The ground system will be able to identify the ADS capability of the aircraft and allocate the appropriate ADS contracts.

#### Based on the current flight plan, information obtained from the aircraft, the ADS capability of the aircraft and ATM requirements, an appropriate ADS contract will be identified by the ground system.   The necessary contract requests will be transmitted to the aircraft for acceptance.

#### At a parameter time or distance prior to the boundary, the ground system will generate and allocate an appropriate ADS contract for the aircraft, based on the current flight plan, information obtained from the aircraft, and the ATM requirements in effect.

#### The ground system will transmit the relevant ADS contracts to the aircraft.  The aircraft will confirm acceptance of the ADS contract to the ground system.

### The aircraft must be able to provide automatic position reporting in accordance with ADS contracts allocated by the ground system.

#### Aircraft with ADS capabilities will provide ADS reports in accordance with the current ADS contract.  The aircraft will generate and transmit ADS reports to the appropriate ground system in accordance with the ADS contracts in force.

#### The controller will be capable of replacing the ADS contract as required by circumstances.  The ground system will generate appropriate messages to the aircraft to initiate such modifications to existing ADS contracts.

### The aircraft must be capable of identifying any changes to position determination capability and of notifying the ground system accordingly.

#### Based on parameters established in the ADS contract, the aircraft will automatically report to the ground system when the aircraft's navigation capability (figure of merit) has changed.

### Both the aircraft and the ground system must be capable of providing an emergency mode of ADS operation to support ATC alerting procedures and to assist search and rescue operations.

#### The system should provide for a pilot-initiated emergency.  The pilot will use simple action to initiate an emergency mode.  It would also be permissible for aircraft to automatically establish the emergency mode.  The aircraft system will alert the pilot to an auto-establishment of the emergency mode.

#### The aircraft system will generate and transmit the basic ADS report at a pre-set initial reporting rate together with the state of emergency.  This preset reporting rate will be the lesser of 50 per cent of the existing periodic contract reporting period, or one minute.  A single default value of one minute may be used in initial implementations.  Aircraft identification and ground vector group will be included in every fifth report.

#### The ground system will recognize the emergency mode and alert the controller.  The ground system will be able to modify the emergency reporting rate if necessary.

#### When an emergency report mode is declared, any existing periodic contract between the ground system and that aircraft should be modified to a default emergency period contract.  While there is an emergency mode in effect, any request for a normal periodic contract should be deferred.  An emergency mode should not affect an event contract.  The periodic contract in effect when emergency mode ends should be reinstated.

#### The pilot will have the ability to cancel the emergency mode.

### The controller must be provided with the most up-to-date traffic situation available using ADS derived information.

#### In an ADS environment, the controller must be provided with the most up-to-date ADS-derived information to permit the provision of effective air traffic control.  The ground system will process the ADS position information sent by ADS-equipped aircraft.  The ground system will generate warnings (and alternative clearances, where conflict resolution algorithms are incorporated) to the controller when it identifies a potential conflict.

### The ADS application will have to allow the comparison of the four-dimensional profile stored in the aircraft system with flight data stored in the ground system to be carried out

#### Many operational errors today in non-radar airspace are due to way-point insertion errors in aircraft flight management systems.  To minimize the possibility of such blunders and to permit advanced strategic planning in a data link based ATS, the ground system will verify that the aircraft's planned four-dimensional profile is the same as the profile that ATC is expecting the aircraft to follow.

### The aircraft must permit self monitoring and automatic reporting of significant flight variances.

#### Based on parameters established in the ADS contract, the aircraft will automatically report to the appropriate ground system when flight variances relating to either speed, heading or altitude are fulfilled or exceeded, or when the aircraft's flight profile data changes.

#### The ground system will determine the flight conformance criteria applicable to the airspace and phase of flight.  The ground system will include within the ADS contract the values that trigger these reports.

#### The aircraft will recognize when one of the reporting criteria is satisfied or exceeded.  The aircraft will generate and transmit an appropriate ADS report for the specific flight variance.  The ground system will generate an alert to the controller if any parameter is exceeded.  If a variance parameter is exceeded, the report will comprise an indication of which parameter has triggered the report, the basic plus the air or ground vector block as appropriate, based on the current ADS contract.

### The ground system will have the ability to monitor the flight of the aircraft before it enters the airspace under its control.

#### As a consequence of the ADS contracts accepted by the aircraft, the aircraft will begin to send ADS reports to the appropriate ground system to initiate flight-following for planning purposes.  The ground system will use ADS information to update its database to ensure entry conditions into the airspace remain acceptable.

#### The position information of the aircraft will be made available to the controller.

### The ground system must be capable of recognizing that the aircraft has entered the airspace over which it has controlling authority.

#### In a non-radar airspace, especially when transiting from an uncontrolled airspace to an airspace where ADS applications are available, the ground system of the controlling ATC unit must recognize that the flight has entered the its airspace.  A set of data as specified by the ADS contract will then be sent by the aircraft to the ground system.

### The ground system must be able to confirm that the aircraft's projected profile coincides with that stored in the ground system.

#### Whenever the ground system receives an aircraft's projected profile information, the ground system will check and verify that it is consistent with that already held.  The ground system will generate and display an appropriate alert to the controller if any value of the specified parameters delta(lat), delta(long), delta(alt) or delta(time) are exceeded.

### The ground system must be able to verify that the aircraft is proceeding in accordance with the ATC clearance.

#### In the data link based ATS, the ground system will use the ADS position reports and other ADS message group data to provide automated flight-following and conformance-monitoring.

#### The aircraft will generate and transmit ADS data to the appropriate ground system according to the current ADS contract.  The ground system will compare the aircraft's ADS-reported position with the position predicted by the ground system.  The ground system will generate and display appropriate messages to the controller if the ADS position report does not conform, within the given parameters, to the position predicted by the ground system.

## Detailed Implementation

### Detailed implementation and more specific operational requirements of the ADS functionality in a data link based ATM system are given in Part III of this manual.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# CONTROLLER PILOT DATA LINK COMMUNICATIONS (CPDLC) APPLICATION Overview and HIGH LEVEL Operational Requirements

## CPDLC Application Overview

### One of the keys to the future air traffic management system lies with the two-way exchange of data, both between aircraft and the ATC system and between ATC systems.  Controller pilot data link communications (CPDLC) is a means of communication between controller and pilot, using data link for ATC communications.

### ICAO has developed a communications systems architecture that provides a range of capabilities to suit the needs of ATS providers and their users.  Various air/ground communication data links will be integrated through ATN based on an Open System Interconnection (OSI) architecture.  Eventually, the ATN will allow world-wide connectivity and an established quality of service which will provide optimum routing and delivery.

### During the transition towards the ICAO CNS/ATM systems, the number of data link applications which require a globally uniform approach and standardization will increase.

### The CPDLC application provides the ATS facility with data link communications services.  Sending a message by CPDLC consists of selecting the addressee, selecting and completing, if necessary, the appropriate message from a displayed menu or by other means which allow fast and efficient message selection, and executing the transmission.  The messages defined herein include clearances, expected clearances, requests, reports and related ATC information.  A "free-text" capability is also provided to exchange information not conforming to defined formats.  Receiving the message will normally take place by display and/or printing of the message.

### CPDLC will remedy a number of shortcomings of voice communication, such as voice channel congestion, misunderstanding due to bad voice quality and/or misinterpretation, and corruption of the signal due to simultaneous transmissions.

### In the future, it is expected that communications with aircraft will increasingly be by means of digital data link.  This will allow more direct and efficient linkages between ground and cockpit systems.  At the same time, extensive data exchange between ATC systems will allow efficient and timely dissemination of relevant aircraft data, and will cater for more efficient coordination and hand-over of flights between ATC units.  In turn, this will reduce controller and pilot workload and will allow an increase in capacity.

### Implementation of CPDLC will significantly change the way pilots and controllers communicate.  The effect of CPDLC on operations should be carefully studied before deciding the extent to which voice will be replaced by data link.

### Among others, the following aspects of Controller Pilot Data Link Communications are to be taken into account in considering its application and in defining procedures:

- the total time required for selecting a message, transmission of the message, and reading and interpretation of the message;

- the head-down time for the pilot and controller;

- the inability of the pilot to listen to other transmissions in the same area of operation,

- unauthorized access, and

- unauthorized transmissions.

## CPDLC High Level Operational Requirements

### A data link based ATS system must provide for the reduction of routine communication tasks which contribute to the saturation of voice frequencies.

### The ADS Panel has identified specific operational requirements relating to the capabilities of the CPDLC application.  These are outlined below:

#### The system must be capable of providing CPDLC when this application is required by the ATM system in force.

#### When required, the data link ATS will support the interchange of data link messages between the pilot and controller to support the effective provision of the data link based ATS service.

#### The pilot or controller may initiate a data link message using either the defined message set, a free-text message, or a combination of both.  The ground system will make the message available to the appropriate controller, or the aircraft system will make the message available to the pilot.

## Detailed Implementation

### Detailed implementation and more specific operational requirements of the CPDLC functionality in a data link based ATM system are given in Part IV of this manual.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# DIGITAL FLIGHT INFORMATION SERVICES (DFIS) APPLICATION Overview and HIGH LEVEL Operational Requirements

## DFIS Application Overview

### In a data link ATS system, flight-related information (e.g., weather and situational awareness) can be made available to aircraft in digital form.  This information will assist the pilot by increasing flight safety and improving situational awareness.

### Most of this information is being delivered to the aircraft via voice.  It is expected that the use of data link to transmit flight information will be implemented in an evolutionary manner.  In the future,  it is expected that DFIS will provide information that is not currently available in the aircraft.

### As the system evolves, flight information may be provided through addressed (point to point) or broadcast data link media.

### There are multiple DFIS Services that may be provided, including:

- Automatic Terminal Information Services (ATIS),

- Meteorological Aerodrome Report (METAR) Service,

- Terminal Weather Service (TWS),

- Windshear Advisory Service,

- Pilot Report Service,

- Notice to Airmen (NOTAM) Service,

- Runway Visual Range (RVR) Service,

- Terminal Area Forecast (TAF) Service,

- Precipitation Map Service, and

- Significant Meteorological Information (SIGMET) Service.

### This manual describes the first two services provided through the DFIS application:  ATIS Service and METAR Service.  These two services are both described as addressed data link services.

## DFIS High Level Operational Requirements

### When DFIS is available, the information that is transmitted must be as accurate and up-to-date as required to meet the current operation.

### Responses to requests for DFIS services must be provided in a timely manner.

## Detailed Implementation

### Detailed implementation and more specific operational requirements of the DFIS functionality in a data link based ATM system are given in Part V of this manual.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# ATS INTERFACILITY DATA COMMUNICATION Overview and HIGH LEVEL Operational Requirements

## AIDC Application Overview

### One of the keys to the future air traffic management system lies with the two-way exchange of data between aircraft and the ATC system, and between ATC systems.  Communications with aircraft will increasingly employ digital data link.  At the same time, automated data exchange between ATC systems will support timely dissemination of relevant flight data, particularly in regard to coordination and transfer of flights between ATS units.

### The AIDC application exchanges information between ATS units in support of critical ATC functions, including notification of flights approaching a Flight Information Region (FIR) boundary, coordination of boundary crossing conditions, and transfer of control.

### AIDC defines messages which are related to three phases of coordination as perceived by an ATSU.

- Notify phase, in which the aircraft trajectory and any changes may be conveyed to an ATSU from the current ATSU prior to co-ordination;

- Co-ordinate phase, in which the aircraft trajectory is co-ordinated between two or more ATSUs when the flight approaches a common boundary; and

- Transfer phase, in which communications and executive control authority is transferred from one ATSU to another.

### Other AIDC messages support ancillary ATC data exchanges between ATSUs, including the exchange of free-text messages.

## AIDC High Level Operational Requirements

### The system should be capable of facilitating automatic transfer of control and communications between ATS Units using digital data interchange.

## Detailed Implementation

### Detailed implementation and more specific operational requirements of the AIDC functionality in a data link based ATM system are given in Part VI of this manual.

PART I - OVERVIEW OF ATS DATA LINK APPLICATIONS

# AUTOMATIC DEPENDENT SURVEILLANCE BROADCAST (ADS-B) APPLICATION Overview and HIGH LEVEL Operational Requirements

## ADS-B Application Overview

### The capability for aircraft to provide aircraft position and other information to airborne and ground based systems is expected to be a part of the future ATS system.  ADS-B is a surveillance application transmitting parameters, such as position, track and ground speed, via a broadcast mode data link for utilization by any air and/or ground users requiring it.

### Each ADS-B capable emitter will periodically broadcast its position and other required data.  Any user, either airborne or ground-based, within range of this broadcast may choose to receive and process this information.  The emitter originating the broadcast need have no knowledge of what system(s) is receiving its broadcast.  Because broadcast data might be received by the ground station at a rate in excess of the requirements of the ATC system, some filtering and/or tracking may be necessary.

### ADS-B will permit enhanced airborne and ground situational awareness to provide for specific surveillance functions and cooperative pilot-controller and pilot-pilot ATM.

### ADS-B will not be limited to the traditional roles associated with ground-based radar systems.  ADS-B will provide opportunities for new functionality both onboard the aircraft and within the ground ATC automation systems.

### ADS-B will have many benefits in extending the range beyond that of secondary surveillance radar, particularly in airport surface and low altitude airspace, and in air-to-air situational awareness.

### The ADS-B application supports improved use of airspace, reduced ceiling/visibility restrictions, improved surface surveillance, and enhanced safety.  ADS-B equipage may be extended to vehicles on the airport surface movement area, uncharted obstacles not identified by a current NOTAM, and non powered airborne vehicles or obstacles.

### Depending on the implementation, ADS-B may encompass both air-ground and air-air surveillance functionality, as well as applications between and among aircraft on the ground and ground vehicles, including:

- ATC surveillance,

- Airborne situational awareness,

- Conflict detection (both airborne and ground based),

- Airborne Separation Assurance System (ASAS),

- ATC conformance monitoring, and

- ADS-B lighting control and operation.

### ADS-B services are expected to be implemented in an evolutionary manner.  This manual addresses one of these services:  ATC Surveillance.

## ADS-B High Level Operational Requirements

### An ATSU will be capable of knowing that an aircraft is ADS-B equipped.

### All aircraft operating in an ADS-B airspace will broadcast as required by the ATS provider.

### The ground system will receive, process, and display the ADS-B information.

### Procedures and/or systems must be in place to validate the ADS-B information.

## Detailed Implementation

### Detailed implementation and more specific operational requirements of the ADS-B functionality in a data link based ATM system are given in Part VII of this manual.

--  END  --