PART  V

DIGITAL  FLIGHT  INFORMATION  SERVICES

(DFIS)

PART V - DIGITAL FLIGHT INFORMATION SERVICES

# APPLICATION OVERVIEW

## Introduction

### Flight Information Services (FIS) are provided to the pilot to increase situational awareness and to enhance safety of flight.  Currently FIS is provided primarily through voice channels.  The Digital FIS (DFIS) application allows a pilot to request and receive FIS services from ground systems via data link.

### Background

#### DFIS are comprised of both airborne and ground based components.  DFIS systems enable information to be provided to aircraft via messages between aircraft avionics and ground systems.

#### It is anticipated that the DFIS application will develop in an evolutionary manner.  In the future DFIS may include services provided by both addressed and broadcast methods.  This version of the manual outlines only initial services, which are provided upon request via addressed systems.

#### In these services, the aircraft (pilot and/or avionics) requests the service by generating a request message for transmission to a DFIS ground system.

#### Each of these services should be accessed and used independently of the others and are initiated by the aircraft.

## Use of DFIS in ATS

### DFIS will be used to provide information to airborne users.  Much of this information is currently provided to pilots via radio transmissions or voice broadcasts (e.g., surface observations, ATIS).  In the future, additional information not available to pilots via voice may be provided via data link.

### Pilots (and/or aircraft avionics) can request DFIS services as desired during preflight or in-flight operations.

### There are multiple DFIS services which may be supported by DFIS systems.  These services include but are not limited to:

- Automatic Terminal Information Services (ATIS),

- Meteorological Aerodrome Report (METAR) Service,

- Terminal Weather Service (TWS),

- Windshear Advisory Service,

- Pilot Report Service,

- Notice to Airmen (NOTAM) Service,

- Runway Visual Range (RVR) Service,

- Terminal Area Forecast (TAF) Service,

- Precipitation Map Service, and

- Significant Meteorological Information (SIGMET) Service.

### This version of the manual includes requirements for two of these services:  ATIS and METAR.  These services are described in Chapters 6 and 7 respectively of this part of the document.

PART V - DIGITAL FLIGHT INFORMATION SERVICES

# GENERAL REQUIREMENTS

## Message Handling

### Message Transmission

#### The DFIS application requires:

- that messages are generated and sent in the time ordered sequence, and

- that messages are delivered in the order that they are sent.

#### The system will ensure that messages are sent to the specified recipient.

## Quality of Service

### The airborne system will have the ability to specify its required QOS based on a user preferred combination of message delay, cost, and permissible error rate.

## Time Requirements

### Wherever time is used in the DFIS application, it will be accurate to within 1 minute of UTC.

### Time stamping will be available for all messages.  The time stamp will consist of the date (YYMMDD) and time (HHMMSS).  The time stamp will be the time the message is dispatched by the originating user.

## DFIS Priority

### The priority for all DFIS messages will be “aeronautical information service messages” as determined by the ATN Internet Protocol Priority categorization.

## DFIS Performance Requirements

### Systems developed to support DFIS should be capable of meeting the communication performance appropriate for the phase of operation.

PART V - DIGITAL FLIGHT INFORMATION SERVICES

# Functional Requirements

## DFIS Provision Modes

### There are two types of DFIS delivery:  The information may be provided upon demand, or by the establishment of an update contract.  Requirements for each provision mode are described below.

### DFIS Demand Mode

#### The DFIS demand mode defines a method for the aircraft to request specific DFIS information and for the ground DFIS system to provide the aircraft with requested information.

#### Message Descriptions

##### There are six messages used by the Demand mode:

- Request message,

- Delivery message

- Processing message,

- Reject message,

- Error message, and

- Abort message.

##### The Request message provides the following information:

- the type of information requested,

- the mode of provision, and

- the parameters of the requested information.

##### The Delivery message provides the following information:

- the type of the information returned,

- the requested information formatted as specified for each service, and

- appended free text as required.

##### The Processing message indicates that the requested information is being processed.

##### The Reject message provides the following information :

- the type of the request rejected, and

- the reason for the rejection of the request

##### The Error message is sent when an error is detected and provides the following information:

- the type of the request error, and

- the reason of the error of the request.

##### The Abort message is sent when either the air or ground system wishes to abnormally terminate a DFIS link, it allows the abort initiator to indicate a reason for issuing the abort.

### DFIS Contract Mode

#### The DFIS update contract defines a method for the aircraft to request specific DFIS information and for the ground DFIS system to provide the aircraft with this information and any updates to this information.

#### Message Descriptions

##### There are seven messages that are used by the DFIS update contract.

- Request message,

- Delivery message

- Processing message,

- Reject message,

- Cancel Update Contract message,

- Error message, and

- Abort message

##### The Request message provides the following information:

- the type of DFIS information requested,

- the type of the Contract,

- the parameters of the requested information

##### In Contract mode the Delivery message is sent initially upon receipt, and subsequently whenever the DFIS information is updated and provides the following information:

- the type of the DFIS information returned,

- the requested information formatted as specified for each service, and

- appended free text as required.

##### The Processing message indicates that the requested information is being processed.

##### The Reject message provides the following information:

- the type of the DFIS request rejected,

- the DFIS information requested, if applicable and available, and

- the reason for the rejection of the DFIS request

Note:  This message can be used to indicate that the ground system does not support the DFIS contract mode.

##### The Error message is sent when an error is detected and provides the following information:

- the type of the DFIS request error, and

- the reason of the error of the DFIS request.

##### The Cancel Update Contact message is sent either by the airborne DFIS system or the ground DFIS system.  It is sent on request of the DFIS application user.

## Establishment of a DFIS Link

### The aircraft DFIS application enables the transfer of the request of DFIS information by first establishing a link with a ground DFIS application.  The aircraft specifies either demand or contract mode.

### Establishment and Operation of a Demand Mode

#### A Demand Mode Provision can work in two ways:  Information can be provided immediately or the reply can be deferred.

#### Immediate Demand Mode Message Sequence

- The airborne system sends a Request message.

- Upon receipt of a Request message, the ground system sends the appropriate Delivery message within the required response time.

#### Deferred Demand Mode Message Sequence

  - The airborne system sends a Request message.

  - Upon receipt of a Request message, the ground system detects that the requested DFIS information can be retrieved but is not yet available or cannot be sent within the required response time, then the ground system:

    - sends a Processing message, and

    - when the information becomes available, sends a Delivery message.

#### Requested Information Not Available or Non-Compliance Message Sequence

- The airborne system sends a Request message.

- Upon receipt of a Request message, the ground system detects that the requested DFIS information can not be retrieved, and the ground sends a Reject message

### Establishment and Operation of Update Contract Mode

#### Upon receipt of a valid Request Message indicating Contract Mode the ground system:

  - if the contract mode is supported

    - sends the appropriate Delivery message within the required response time and continues to send updates when available until the contract is terminated, or

    - detects that the requested DFIS information can be retrieved but is not yet available or cannot be sent within the required response time, then

- sends a Processing message, and

- when the information becomes available, sends an Delivery message and continues to send updates when available until the contract is terminated;

    - detects that the requested DFIS information cannot be retrieved and sends a Reject message

  - if the contract mode is not supported,

    - sends the appropriate Delivery message within the required response time indicating that contract mode is not supported, or

    - detects that the requested DFIS information can be retrieved but is not yet available or cannot be sent within the required response time, then

- sends a Processing message, and

- when the information becomes available, sends a Delivery message indicating that contract mode is not supported, or

    - detects that the requested DFIS information cannot be retrieved and sends a Reject message.

## DFIS Link Termination

### For the Demand Contract, the link is closed on receipt of the Delivery, Reject, or Error reply.

### For the Update Contract, the ground system sends updated DFIS information or update indications until the link is closed explicitly by either the ground or aircraft.

### Either an airborne or a ground system will have the capability to terminate the service.

PART V - DIGITAL FLIGHT INFORMATION SERvICES

# Procedures

## TBD

PART V - DIGITAL FLIGHT INFORMATION SERVICES

# EXCEPTION Handling

## Abnormal Termination of the DFIS Service

### When an error situation is detected by either an airborne or a ground system that system indicates an error.  If the error is unrecoverable the service is terminated.

### In the event of an unintended interruption of the DFIS contract mode, the contract will be terminated and the pilot will be informed.

PART V - DIGITAL FLIGHT INFORMATION SERViCES

# DESCRIPTIOn OF Automatic TErminal Information Service (ATIS) SERVICE DESCRIPTION

## Scope and Objectives

### The ATIS data link service supplements the existing availability of ATIS as a voice broadcast service, provided at aerodromes world-wide.  All types of ATIS currently in use are encompassed (i.e., arrival, departure and combined).

### The data link ATIS message will be used to provide an alternative means of transmitting the service to those equipped aircraft which request the message.

## Expected Benefits, Anticipated Constraints, and Associated Human Factors

### Benefits

- Reduced flight crew workload:  ATIS information does not need to be copied by the flight crew if the message is printed on a cockpit printer or is recallable on a data link display.  Also, the flight crew does not have to divert attention from ATS operational channels to listen to the ATIS broadcast ;

- Reduced ambiguity in the transmitted information:  data link implementation eliminates potential misinterpretation resulting from poor transmission quality; and

- Potential for increased accessibility to ATIS information:  flight crews should be able to request timely ATIS information from any aerodrome capable of providing data link information.

### Anticipated Constraints

- Reduced dialogue flexibility in case of complex meteorological conditions.

### Associated Human Factors

- Error detection and correction mechanisms will be implemented to ensure message integrity.  Technical and procedural mechanisms will be considered;

- Maximum use of data link will not impose undue competition for display or control resources.   Systems should not preclude access to other functions or conflict with higher priority functions;

- When operating the data link system, there will not be any increase in head down time that would adversely affect the safe operation of the aircraft.  Aircraft system implementation will adhere to recognized human factors practices and design criteria; and

- Voice communication will be available to supplement data link system operation.

## Operating Method in a Voice Environment

### ATIS is predominantly a voice broadcast service intended to relieve frequency congestion and air traffic controller workload by providing pertinent information to aircraft operations in the terminal area through a local broadcast, eliminating the need for a controller to transmit the information to each aircraft individually.  This is normally accomplished through the use of a voice recording of the information which is continuously broadcast over a published VHF frequency in the vicinity of the aerodrome.

Note:  Some States also provide ATIS through various forms of data link service.

### The information broadcast usually lasts less than one minute, and includes weather conditions, operating procedures, runways and approaches in use, and various other information which may affect the departure, approach and landing phases of flight.

### The recorded ATIS message is updated upon the issuance of a new weather observation, or when conditions or procedures affecting various components of the ATIS message change substantially.

### Individual ATIS messages are identified by a designator from the ICAO alphabet, alpha through zulu, on a cyclical basis.

### ATIS is received by the flight crew tuning to the published VHF frequency for a particular ATIS broadcast, and listening to the recorded message until all pertinent information is understood.  The flight crew usually copies the information.

### In many States, it is an operational requirement that flight crews obtain ATIS information prior to contacting the associated ATS facility, and confirm having obtained the current ATIS by repeating the ATIS designator to the controller.

## Operating Method in a Data Link Environment

### Service Description

#### Content of voice and data link ATIS will be identical and updated simultaneously.

#### ATIS should support the Demand and Contract modes.

#### The flight crew transmits an ATIS Request message to the appropriate ground system.  The request indicates the DFIS mode, Demand or Contract.

#### Upon receipt of a valid ATIS Request Message indicating Demand Mode the ground system:

  - sends the appropriate ATIS Delivery message within the required response time, or

  - detects that the requested ATIS information can be retrieved but is not yet available or cannot be sent within the required response time, then

    - sends a Processing message, and

    - when the information becomes available, sends an ATIS Delivery message, or

  - detects that the requested ATIS information cannot be retrieved and sends a Reject message.

#### Upon receipt of a valid ATIS Request Message indicating Contract Mode the ground system:

  - if the contract mode is supported

    - sends the appropriate ATIS Delivery message within the required response time and continues to send updates when available until the contract is terminated, or

    - detects that the requested ATIS information can be retrieved but is not yet available or cannot be sent within the required response time, then

      - sends a Processing message, and

      - when the information becomes available, sends an ATIS Delivery message and continues to send updates when available until the contract is terminated;

    - detects that the requested ATIS information cannot be retrieved and sends a Reject message

  - if the contract mode is not supported,

    - sends the appropriate ATIS Delivery message within the required response time indicating that contract mode is not supported, or

    - detects that the requested ATIS information can be retrieved but is not yet available or cannot be sent within the required response time, then

      - sends a Processing message, and

      - when the information becomes available, sends an ATIS Delivery message indicating that contract mode is not supported, or

    - detects that the requested ATIS information cannot be retrieved and sends a Reject message.

### Initiation Conditions

#### The ATIS Service may be solicited by the flight crew during any phase of operation.

### Sequence of Services

#### This service is capable of being solicited by the flight crew independently of any other service.

### Additional guidelines

#### If the aircraft does not specify contract or demand mode, the ground system will transmit a demand ATIS.

#### Contract ATIS updates will include entire contents of the new ATIS.

#### The ground system, operating in contract mode, will be capable of detecting that it cannot continue to provide updates of the requested ATIS information and inform the aircraft of this condition.

#### The ground system will be able to support multiple concurrent ATIS Contracts with the same aircraft or with several aircraft.

#### An aircraft with an ATIS contract should not be precluded from initiating an ATIS request for any aerodrome.

#### An aircraft may request ATIS information for more than one aerodrome.

#### Each ATIS request should be related to a single aerodrome.

#### If a pilot requests an arrival or departure ATIS and if the ground system only provides combined ATIS, combined ATIS will be sent with a combined indication.

#### If a pilot requests arrival and departure ATIS for an aerodrome or does not indicate which type of ATIS (arrival, departure or combined), the ground system will

- if available, provide combined ATIS with a combined indication, or

- if both arrival and departure ATIS are available, but combined ATIS is not available, provide an arrival and a departure ATIS, or

- if only arrival ATIS is available provide arrival ATIS and indicate departure ATIS is unavailable, or

- if only departure ATIS is available provide departure and indicate arrival ATIS is unavailable, or

- send a Reject message indicating ATIS is unavailable.

### Exception Handling

#### No response

##### If the pilot does not receive a valid message response within the specified time, the aircraft system will generate an alert.  This alert should not preclude further data link initiation by the pilot.

#### No ATIS delivery after receiving a processing message

##### If an ATIS processing message is received by the aircraft, and an ATIS delivery is not received within a specified time, the aircraft system will generate an alert.  This alert should not preclude further data link initiation by the pilot.

#### Discontinuation of ATIS update during a contract

##### In the event of unintended discontinuation of ATIS update during contract, the contract terminates and the pilot will be notified

### Termination Conditions

#### Use of the ATIS data link service is closed by the aircraft system:

- In Demand mode upon receipt of the ATIS message or,

- Upon receipt of the Unable message or,

- In Contract mode, upon receipt of the ATIS Contract Termination message by either the ground or aircraft.

- In Contract mode, contracts initiated while airborne should be terminated upon landing.

## ATIS Message Sequence Diagram

### Figure 6-1 illustrates an ATIS Demand service message sequence with an ATIS request and response.

Figure 6-1:  ATIS Demand Request and ATIS Report Sequence Diagram

### Figure 6-2 illustrates an ATIS Demand service message sequence with a deferred ATIS response.

Figure 6-2:  ATIS Demand Request and ATIS Deferred Report Sequence Diagram

### Figure 6-3 illustrates an ATIS Contract service message sequence with an ATIS request and response.

Figure 6-3:  ATIS Contract Request and ATIS Report Sequence Diagram

Note:  This figure illustrates airborne contact cancellation.  Other means of cancellation are described in this section.

## ATIS Messages

### Table 6-1 presents ATIS message requirements.

| Message | Information required | Type |  |  |  | Event/ Trigger |  | Source/ Destination |  | Alert |  | Response Required |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A | D | C |  |  |  |  |  |  |  |  |
| ATIS Request | Message Type Aerodrome Identification Departure and/or Arrival Indicator Contract Type |  | M M M O | M M M O | M M O O |  | Flight crew input |  | Aircraft/ Ground system |  | None required |  | Yes |
| ATIS Delivery | Aerodrome Identification Departure and/or arrival indicator Contract Type ATIS Designator Time of Observation Approach Type Arrival Runways In Use Status of Arresting System Departure Runway Significant Runway Surface Conditions Braking Action Holding Delay Transition Level Other Operational Information Surface Wind Visibility RVR Present Weather Cloud Sky and Cover Air Temperature Dew Point Temperature Altimeter Setting Significant Met Information Trend Type Landing Forecast Specific ATIS Instructions |  | M M M M O M M O M O O O O M M O O M M M M O O O | M M M M O O M M O O O O M M O O M M M M O O | M M M M O M M O M M O O O O M M O O M M M M O O O |  | Receipt of a valid ATIS Request |  | Ground system/ aircraft |  | Medium |  | No |
| ATIS termination | Aerodrome Identification Contract Termination |  | M O | M O | M O |  | Aircraft or ground system termination |  | Aircraft//ground system or ground system /aircraft |  | Pilot : Medium ground system : none required |  | No |

Table 6-1:  ATIS Message Requirements

Table Key:

A:  Arrival ATIS

D:  Departure ATIS

C:  Combined ATIS

M:  Mandatory

O:  Mandatory if applicable

## ATIS Data Glossary

### Appendix A to this chapter contains the ATIS data glossary

PART V - FLIGHT INFORMATION SERVICES

CHAPTER 6 APPENDIX A

ATIS DATA GLOSSARY

Air Temperature:  indicates the outside air temperature taken adjacent to the runway (in ° Celsius or Fahrenheit).

Altimeter Setting:  gives QNH of the aerodrome and the runway QFE when available (in hectopascal or inches of mercury).

Approach Type:  indicates the type of approach to be expected.

Arrival and/or Departure Indicator:  indicates which type of ATIS information is provided (i.e., arrival, departure, or combined).

Arrival Runways In Use:  indicates landing runways expressed as left, center or right when there is more than one.

Braking Action:  indicates the braking action in case of contaminated runway conditions.

Cloud Sky and Cover:  indicates the cloud coverage below 5000 feet expressed as scattered, broken or overcast.

Contract Termination:  indicates that the ATIS contract is terminated.

Contract Type:  Distinguishes between the types of ATIS provided (demand or contract).

Departure Runways In Use:  indicates departure runways expressed as left, center or right when there is more than one.

Dew Point:  indicates the dew point taken adjacent to the runway.

Holding Delay:  indicates an unplanned delay prior to the initiation of the next phase of flight.

Other Operational Information:  describes other pertinent operational information.

Present Weather:  Gives the weather occurring at or near the aerodrome which may affect visibility.

RVR:  indicates the Runway Visual Range (in feet or meters).

Significant Meteorological Information:  gives significant meteorological information specific to approach, take-off and climb-out areas, or NOSIG.

Significant Runway Surface Conditions:  indicates significant runway surface conditions.

Specific ATIS Instructions:  gives instruction to the pilot to acknowledge the receipt of the ATIS message upon initial contact.

Status of Arresting System:  indicates when an arresting system constitutes a potential hazard.

Surface Wind:  indicates surface wind direction (in degrees) and speed (in km per hour or knots) from the aerodrome reference point.

Time of Observation:  time at which the weather was observed expressed in hours, minutes.

Transition Level:  indicates a change from the published transition level.

Trend Type Landing Forecast:  concise statement of the expected trend of the meteorological conditions at the aerodrome.

Visibility:  indicates the horizontal distance ( in meters, km, miles or 1/16 of miles) at which an object can be seen.

PART V - Digital FLIGHT INFORMATION SERVICES

# DFIS MeteoroloGical Aerodrome Report (METAR) SERVICE DESCRIPTION

## Scope and Objective

### Pilots are required to maintain awareness of the meteorological conditions at aerodromes of interest throughout the planned flight, including departure, en route, destination and alternate locations.  The METAR data link Service provides automation assistance in requesting and delivering reports of meteorological conditions at aerodromes world-wide; specifically those reports encoded in the METAR format.

## Expected Benefits, Anticipated Constraints And Associated Human Factors

### Expected Benefits

- Reduce flight crew workload:  METAR information will not need to be copied by the flight crew if the message is printed on a cockpit printer or is able to be recalled on a data link display.  Also, the flight crew will not have to divert attention from ATS operations to request voice relay of METAR reports by air traffic controllers (or flight service specialists where available);

- Reduce air traffic controller workload:  data link implementation will reduce need for controller response to pilot voice requests for status and updates of weather conditions at specific aerodromes;

- Reduce ambiguity in the transmitted information: data link implementation will eliminate potential misinterpretation resulting from poor voice transmission quality and transcription errors during hand-copy of voice transmissions;

- Potential increased accessibility to METAR information: flight crews will be able to request METAR information from any specified site available within the accessible data base; and

- Potential reduction in congestion on voice channels:  data link implementation will reduce voice requests and responses for relay of METAR reports.

### Anticipated Constraints

- Reduced dialogue flexibility in case of complex meteorological conditions.

### Associated Human factors

- Error detection and correction mechanisms will be implemented to ensure message integrity.  Technical and procedural mechanisms will be considered;

- Maximum use of data link will not impose undue competition for display or control resources.   Systems will not preclude access to other functions or conflict with higher priority functions;

- When operating the data link system, there will not be any increase in head down time that would adversely affect the safe operation of the aircraft.  Aircraft system implementation will adhere to recognized human factors practices and design criteria; and

- Voice communication will be available to supplement data link system operation.

## Operating Method Without Data Link

### Reports of meteorological conditions at aerodromes are officially recorded and communicated each hour, on the hour, as specified in applicable ICAO/WMO publications (and supplemented by member states as necessary).  Intermediate (special) observations are also recorded and communicated when the meteorological conditions change sufficiently to effect aviation operations as defined by agreed specific meteorological threshold criteria.

### These meteorological aerodrome reports are encoded in the METAR code format and are made available to all ATSUs, and to all other aviation weather users (public and private) via local and international telecommunications networks.

### Individual METAR reports are identified by the specific aerodrome ICAO designation and the appropriate date and UTC time.

### METAR reports are received by the flight crew by requesting and receiving voice transmissions from air traffic controllers (or flight service specialists where available).  At some aerodromes, continuous VHF voice broadcasts of automated weather observations are available.  Also, most airline operators provide access to METAR reports through their flight dispatch support either through voice communications and/or ACARS data link service.

### In many States, it is an operational requirement that flight crews maintain awareness of meteorological conditions that may affect the planned flight including METAR reports for aerodromes of interest.

## Operating Method With Data Link

### Service Description

#### The METAR report contains the latest available weather observation for the specified aerodrome.

#### The flight crew transmits a METAR Request message to the appropriate ground system.

#### If the METAR Request is valid, and the METAR information is available, the ground system initiates a reply using the most current data available in the METAR data base and transmits a METAR message to the aircraft

#### If the METAR Request is valid, but the METAR information is not available, the ground system transmits a service not available response to the aircraft.

### Initiation Conditions

#### The METAR Service may be solicited by the flight crew during any phase of operation.

### Sequence of Services

#### This service is capable of being solicited by the flight crew independently of any other service.

### Additional guidelines

#### An aircraft may request METAR information for more than one airport.

#### Each METAR request should be related to a single airport.

### Error Handling

#### No response

##### If the pilot does not receive a valid message response within the required time, the aircraft system will generate an alert.  This alert should not preclude further data link initiation by the pilot.

#### No METAR Delivery After Receiving a Processing Message

##### If after receipt of a Processing message, a METAR Reply message response is not received within the required time, the aircraft system shall generate an alert.  This alert should not preclude further data link initiation by the pilot.

### Termination Conditions

#### Use of the METAR data link Service is closed by the aircraft system:

- upon receipt of the METAR Reply message or,

- upon receipt of the service not available message.

## Time Sequence Diagram

### Figure 7-1 illustrates a METAR service message sequence with an METAR request and response.

Figure 7-1:  METAR Request and METAR Report Sequence Diagram

## Information Exchanges

### The following table contains the required METAR data`

| Message | Information required | Event/ Trigger | Source/ Destination. | Alert | Response Required |
| --- | --- | --- | --- | --- | --- |
| METAR Request | Mandatory : Message Type Airport Identification | Flight crew input | Aircraft/ Ground system | None Required | Yes |
| METAR Reply | Mandatory Airport Identification Date & Time (UTC) METAR Report Type METAR Coded Weather Elements Mandatory if applicable Free text remarks modifying or expanding on METAR coded weather elements. | Receipt of a valid METAR Request | Ground system/ Aircraft | Medium | No |
| METAR termination | Airport Identification DFIS Uplink Error Message | Aircraft or Ground System | Aircraft /Ground System Ground System /Aircraft | Pilot : medium Ground system: none: required | No |

Table 7-1:  METAR Service Information Exchange

Note:  Specific content will vary depending on weather conditions and the METAR Report Type.  For more detailed information on METAR contents, refer to WMO Publication No. 306, Manual on Codes, Volume I.

--  END --