PART  III

AUTOMATIC  DEPENDENT  SURVEILLANCE

(ADS)

PART III AUTOMATIC DEPENDENT SURVEILLANCE

# APPLICATION OVERVIEW

## Introduction

### This part of the manual contains guidance material for the Automatic Dependent Surveillance (ADS) section.  ADS is a surveillance technique for use by air traffic services in which aircraft automatically provide, via a data link, data derived from on-board position-fixing and navigation systems.  ADS will allow controllers to obtain position data and other information from ADS equipped aircraft in a timely manner in accordance with their requirements, and will allow the aircraft to be tracked in non-radar airspace.

### The primary objective of the ADS application is to provide automated aircraft position data for ATC.  The ADS application may also be useful in air traffic flow management (ATFM) and airspace management (ASM).

### ATM benefits from the use of the ADS application may include separation minima reduction, and more efficient use of airspace.

### Although the application of ADS does not specifically encompass ATC communications, automation or procedures, all of these elements must be tailored to support the ADS application and to make meaningful use of the data.  Thus, it is critical to consider the ATC automation and communications systems as the foundation upon which an ADS-based ATC system is built.  The implementation of ADS into air traffic systems will be an evolving process.  There will be a gradual transition from procedurally-oriented strategic air traffic control towards a more tactical control environment.

### The ADS application and associated communications will have to be supported by advanced airborne and ground facilities and data link communications with proven end-to-end integrity, reliability and availability.

### ADS is one of the applications supported by the ATN.

### Figure 1-1 depicts a general overview of several components of an ADS system.

Figure 1-1 ADS System Concept

## Use of ADS in ATS

### The implementation of ADS, through reliable data link communications and accurate aircraft navigation systems, will provide surveillance services in oceanic airspace and other areas where non-radar air traffic control services are currently provided.  The implementation of ADS will also provide benefits in en-route continental, terminal areas and on the airport surface.  The automatic transmission of the aircraft position through ADS will replace present pilot position reports.  The content and frequency of reporting will be determined by the controlling ATC unit.  In non-radar airspace, the effective use of ADS in air traffic services will facilitate the reduction of separation minima, enhance flight safety and better accommodate user-preferred profiles.

### Use of ADS Outside of Radar Coverage

#### In oceanic and other areas which are beyond the coverage of land-based radar, ADS reports will be used by ATS to improve position determination, resulting in improvements in safety, efficient utilization of airspace and improved controller efficiency.  This is expected to increase airspace capacity and allow more economical routing and spacing of aircraft.

#### The introduction of ADS in non-radar airspace will better enable controllers to identify potential losses of separation or non-conformance with the flight plan and to take the appropriate action.

### ADS Transition Airspace

#### In transition airspace where other means of surveillance become available, provisions are required which integrate ADS and other surveillance information.  Further information is provided in Part III Chapter 1 Appendix A.

### Within Radar Coverage

#### ADS will be beneficial in areas where it may serve as a supplement to or for back-up for radar, thereby reducing the need for primary surveillance radar.  Furtherinformation is provided in Part III Chapter 1 Appendix A.

## Functional Description

### ADS information can assist ATC in performing the following functions:

- Position monitoring.  The automation system processes incoming ADS messages to verify their validity and match them to the appropriate flight plan.

- Conformance monitoring.  The ADS reported position is compared to the expected aircraft position, which is based on the current flight plan.  Longitudinal variations  which exceed a pre-defined tolerance limit will be used to adjust expected arrival times at subsequent fixes.  Horizontal and vertical deviations which exceed a pre-defined tolerance limit will permit an out-of-conformance alert to be issued to the controller.

- Conflict detection,  The ADS data can be used by the ground system automation to identify violation of separation minima.

- Conflict prediction.  The ADS future position data can be used by the automation system to identify potential violations of separation minima.

- Conflict resolution.  ADS reports may be used by the automation system to develop possible solutions to potential conflicts when they are detected.

- Clearance validation.  Data contained in ADS reports are compared with the current clearance and discrepancies are identified.

- Tracking.  The tracking function is intended to extrapolate the current position of the aircraft based on ADS reports.

- Wind estimation.  ADS reports containing wind data may be used to update wind estimations and hence expected arrival times at way-points.

- Flight management.  ADS reports may assist automation in generating optimum conflict-free clearances to support fuel-saving techniques such as cruise climbs.

### ADS Agreements

#### The ATC unit controlling the aircraft should establish the composition and requirements for the transmission of ADS reports through an ADS agreement with the aircraft.  This ADS agreement will be fulfilled by one or more contracts.

#### Where possible an ADS agreement should be established between an aircraft and the ground system prior to the entry into ADS airspace.  An ADS agreement may also remain in effect for a period of time after an aircraft has exited ADS airspace.

#### Termination of an ADS agreement may be achieved automatically by the ground system.

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

CHAPTER 1 APPENDIX A

INTEGRATION OF ADS AND SSR DATA

1A.1	Application Overview

1A.1.1	Introduction

1A.1.1.1	The safe operation of aircraft at close proximity requires an increase in the availability of very accurate positional data in order to apply separation closer to the minima and increase the airspace capacity.  This material concerning the ADS/SSR integration offers guidance in order to achieve a single estimation of aircraft position, by processing both ADS and radar data ( data fusion).  Consequently, an enhancement of tracking algorithms is necessary in order to take advantage of all available surveillance sources as well as processing new parameters related to aircraft motion.

1A.1.1.2	The primary objective of the ADS/SSR integration technique is to take advantage of ADS concept implementation and surveillance, within areas covered by radar surveillance as well as transition areas between radar and ADS only coverage.  Complete radar coverage in ADS/SSR airspace is not required, although outer horizontal limits should normally be coincident.  In addition an ADS transition buffer is advisable.  In areas where duplicate radar coverage is currently mandatory the integration of ADS might lead to a mitigation of that requirement, as well as for the provision of single radar coverage in area where the installation of radar system is not feasible or economically justifiable.

1A.1.1.3	The use of the ADS/SSR integration in areas already having multiple radar coverage will provide the system with the capability of making as uniform as possible track quality within radar covered airspace, thus overcoming residual radar shortcomings.  ADS/SSR integration will result in the augmentation of surveillance performance in existing radar environments, as well as beyond radar coverage.  The ADS/SSR integration will result also in a more reliable data availability for conflict detection and conformance monitoring function thus reducing the probability of false alarms of this function.  This will be essentially due to kinematic data measured on-board and availability of aircraft intent.

1A1.1.4	Since the ADS technique relies upon the capability of an ADS facility to set up a contract with the aircraft to send reports with appropriate content and periodicity, the contract management function will play a key role in defining the most appropriate periodicity and content to optimize the ADS/SSR integration.  The strategy to define the best contract for this function should take into account constraints on airspace and traffic scenario, as well as aircraft flight plan and communication infrastructure performances.

1A.2	Scope and Benefits

1A.2.1	The ADS/SSR data integration can provide the following improvements to the  surveillance function:

  - automatic acquisition of certain airborne data containing parameters such as true track, speed, etc., which will improve the ground tracking of aircraft.

  - availability of surveillance data also when the radar limitations occur.  These limitations are:

    - mechanical rotation of the radar antenna, and

    - garbling, fruit and splitting

  - coding of the altitude data in 40 feet increments and the availability of the vertical rate, as provided by Ground Vector or Air Vector, which will improve the ability of ATC to monitor and make high quality predictions of aircraft trajectories in the vertical plane, thus improving the Short-Term Conflict Alert (STCA) function to significantly reduce the number of false alarms;

  - automatic acquisition of aircraft call signs by ATC system, thus overcoming current problems connected with SSR code-call sign correlation and with radar identification and transfer procedures;

  - acquisition of surveillance data, when satellite data link is used to support the ADS function, also when radar shortcomings such as line of sight propagation limitations (e.g., shadowing by orography, earth curvature, low level flight) become apparent;

  - minimization of the number of SSRs required to supply mono-radar coverage, since ADS fills in the small areas not covered by them (“gap filler”);

  - increase of the level of availability using the ADS as one more level of redundancy;

  - availability of a means for a cross-check of ADS/Navigation data or radar integrity (NIM, Navigational Integrity Monitoring);

  - possibility of adapting the degree of surveillance redundancy for each aircraft according to instantaneous ATC needs, thus providing redundancy in a very cost effective manner.

1A.2.2	In general the improvements in a) to i ) above are applicable to integration between ADS and Mode A/C and Monopulse SSR.  In addition the improvements in e) to i) are applicable also to integrated systems using Monopulse Mode-S SSR.

1A.3	Performance Considerations

1A.3	When considering ADS/SSR integration, the following should be taken into account:

- performance requirements for ADS, including availability and integrity,

- accuracy of both Radar and ADS position reports,

- use of ADS data for example, as part of a data fusion and not just as back-up,

- trajectory prediction requirements,

- the development of a common surveillance processing system, where both the ADS and Radar tracks may be amalgamated to generate a single system track, and

- the synchronization of both radar and ADS update rates.

1A.4	Additional Operational Considerations

1A4.1	In addition to position information, the ADS/SSR integration process could benefit from aircraft periodically reporting further information such as:

- Ground Vector, containing track, ground speed and vertical rate; and

- Event ADS reports including lateral deviation, altitude, speed and FOM change.

1A.4.2	The use of this other data could substantially reduce the need for ADS periodic position reports.

1A.5	ADS Contract Considerations

1A.5.1	The following criteria may have to be taken into account when defining an optimal ADS contract strategy:

- flight plans and related airspace information,

- radar coverage maps,

- communication network capabilities,

- aircraft capabilities,

- accuracy requirements, and

- tracking needs

1A.5.2	It may be necessary to define a contract assignment for each phase of operations namely:

- En-route navigation,

- Terminal Area operations, and

- Ground movements.

1A.6	En-Route Operation

1A.6.1	For this phase of operation the main ADS contract could be a periodic contract (basic or basic and Ground Vector or Air Vector, depending on aircraft capability) with low reporting rate, since when the aircraft are flying straight and level, ground processing systems are able to achieve accurate position estimates also in this case.

1A.6.2	Use of ADS periodic reports containing basic information and Ground Vector when available, may allow a reduction in the data rate and an improvement in tracking.

1A.6.3	In addition, event contracts with the aircraft could allow the detection of the start of a manoeuvre.  Such event contracts could indicate a change in one or more of the following kinematic parameters:

- Lateral deviation,

- Altitude,

- Vertical rate,

- Ground speed

- FOM, and

- Heading

1A.6.4	Extensive use of event reporting could minimize the number of required periodic reports.

1A.7	Terminal Area Operations

1A.7.1	For this phase of operation the main ADS contract is a periodic contract (basic or basic and additional data, depending on aircraft capability) with high reporting rate, since this may be necessary to satisfy the required tracking accuracy.

1A.7.2	Use of ADS periodic reports with additional data (Ground Vector) when available, may help to minimize the frequency of required data.

1A.7.3	It is unlikely that in this phase of operations, event reports would be necessary, since the high periodic rate would already provide a good track reconstruction capability.

1A.8	Ground Movements

1A.8.1	When the aircraft are moving on the airport surface the ADS contract could be a periodic contract with very high reporting rate, to insure that the required tracking accuracy is met.

1A.9	Transition

1A.9.1	During transition phases and while ADS separations equivalent to radar separations are not yet achievable, ADS information could be employed as an assistance and back-up to radar control service.  If it is used as back-up appropriate large separations may need to be taken into account.

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

# GENERAL REQUIREMENTS

## Performance Requirements

### Systems developed to support ADS will be capable of meeting the communication performance appropriate for the phase of operation.

## Message Handling

### In addition to the general performance requirements in Part 1, the ADS application requires:

- that messages are generated and sent in the time ordered sequence, and

- that messages are delivered in the order that they are sent.

### When ADS messages are queued by the ground system they will be handled in the following order:

- emergency mode ADS reports,

- event or demand ADS reports, and then

- periodic ADS reports.

### If more than one message is queued in a, b, or c in 2.2.2 above it will be handled in the order received.

## Quality of Service

### The ground system must have the ability to specify the required QOS based on a user preferred combination of message delay, cost, and permissible error rate.

## Time Requirements

### Wherever time is used in the ADS application, it will be accurate to within 1 second of UTC.

### Time stamping will be available for all messages.  The time stamp will consist of the date (YYMMDD) and time (HHMMSS).

## ADS Priority

### The priority for ADS will be “normal priority flight safety messages” as determined by the ATN Internet Protocol Priority categorization.

## ADS Operational Timers

### In order to meet the more stringent of the performance requirements in Part I, the aircraft system should be capable of responding to a request for information within 0.5 second.

### If the aircraft cannot respond with a reply message containing the requested information within 0.5 second, it sends a positive acknowledgment of receipt of the request, and must send the information within 30 seconds.

## Source of ADS Data

### ADS navigational data must be supplied by the on-board navigational equipment actually navigating the aircraft

### Information contained within an ADS report should be no less recent than two seconds or ten percent of the periodic contract rate, if applicable, whichever is the shorter.

## ADS Report Availability

### ADS reports will be made available to facilities other than the controlling ATC unit on the basis of mutual agreement.

## ADS Contract Requirements

### The avionics will be capable of supporting contracts with at least four ATS ground systems simultaneously.

### The avionics will be capable of supporting one demand, one event and one periodic contract with each ground system simultaneously.

### The avionics must be capable of supporting a 60 seconds periodic contract reporting rate.

### If a ground system requests a contract with an aircraft, and the aircraft cannot support any additional contracts the aircraft will reply with the ICAO facility designators of the ground systems with which it currently has contracts.

### Procedures will be established to ensure that only appropriate ATC ground systems initiate ADS contracts with a given aircraft.

### In the event of an unexpected termination of the ADS application, both the avionics and the ground system will be notified of the failure.  The resumption of the ADS application is incumbent on the ground system.

### An existing contract will remain in place until any new contract of the same type is accepted by both the avionics and the ground system.

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

# ADS FUNCTIONAL CAPABILITIES

## Background

### The ADS application is designed to give automatic reports from an aircraft to a ground system.  The aircraft provides the information to the ground system in four ways:

- on demand,

- when triggered by an event,

- on a periodic basis, and

- in an emergency.

### The system will be capable of distinguishing each of the four ways listed above.

## Operating Method

### The ADS application comprises the following functions:

- establishment and operation of a Demand Contract,

- establishment and operation of an Event Contract,

- establishment and operation of a Periodic Contract,

- cancellation of all contract(s),

- establishment and operation of Emergency Mode,

- modification of the Emergency Mode, and

- cancellation of the Emergency Mode.

## Establishment and Operation of a Demand Contract

### The Demand Contract provides the capability for a ground system to request a single ADS report from an aircraft and specify which optional ADS data is required (if any) in addition to the basic ADS report.

### Any number of demand contracts may be sequentially established with an aircraft.

### If the avionics can comply with the demand contract request it sends the requested report.

### If there are errors in the contract request, or if the avionics cannot comply with the request, it sends a negative acknowledgment indicating the reason for rejection.

### If the avionics can partially comply with the contract request, it sends a message which includes;

- a non-compliance notification indicating those parts of the contract it cannot comply with,

- the basic ADS report, and

- the information requested which can be supplied.

## Establishment and Operation of an Event Contract

### The event contract allows the ground system to request the avionics to send ADS reports when the specified events occur, principally for the purpose of conformance monitoring by ATC.

### The event contract states the event types that are to trigger reports and also any required threshold values delimiting the event types.

### An ADS event report consists of the basic ADS information in every ADS event report, and if required by the event that triggered the sending of the report, other required ADS information may also be included.

### Only one event contract may exist between a ground system and an aircraft at any one time, but this may contain multiple event types.

### Each time an event contract is established it replaces any event contract already in place.

### If the avionics can comply with the event contract request it sends a ADS report with basic information, if required by the event type, any additional required baseline information, and a positive acknowledgment.  Should the contracted event occur, the required ADS report(s) is sent.

### If there are errors in the event contract request, or if the avionics cannot comply with the request, it sends a negative acknowledgment to the ground system indicating the reason for its inability to accept the contract.

### If the avionics can partially comply with the request, it sends a non-compliance notification indicating those parts of the contract with which it cannot comply.  Event reports are subsequently sent only for those events that the aircraft can comply with.

### Should an event for lateral deviation change, altitude range deviation, or vertical rate change occur, a report is sent once every minute while the limit(s) specified in the contract are exceeded.  For these type of events, once the event has occurred, reports cease each time the event parameters are within the specified thresholds and resume at one minute intervals each time the event parameters exceed the specified thresholds.  For all other events, a single report is sent every time the event occurs.

### If more than one of the events described below occurs at the same time, the avionics sends separate ADS event reports for each event.

### Event Types:

#### The following event types have been defined for ADS:

- Vertical rate change,

- Way-point change,

- Lateral deviation change,

- Altitude change,

- Altitude range deviation,

- Airspeed change,

- Ground speed change,

- Heading change,

- Extended projected profile change,

- FOM (Figure of Merit) field change, and

- Track angle change.

#### Vertical Rate Change

##### The vertical rate change event can be triggered in two ways.  For positive vertical rate threshold the event is triggered when the aircraft’s rate of climb is greater than the vertical rate threshold i.e., its rate of climb is greater than planned.  For negative vertical rate threshold the event is triggered when the aircraft’s rate of descent is greater than the vertical rate threshold i.e., its rate of descent is greater than expected.

##### The ADS vertical rate event report is sent once every minute whenever the aircraft’s rate of climb/descent exceeds the value of the vertical rate change threshold.

##### The avionics will cease sending ADS reports when the aircraft’s rate of climb/decent is less than or equal to the value of vertical rate change threshold.

##### An ADS report sent as a result of the occurrence of a vertical change event will contain the basic ADS information and ground vector information.

##### Figure 3-1 illustrates a vertical rate change event.

Figure 3-1:  Illustration of Vertical Rate Event

#### Way-Point Change

##### The way-point change event is triggered by a change in the next way-point.  This change is normally due to routine way point sequencing.  However, it will also be triggered by a change in a way point which is not part of the ATC clearance but is entered by the pilot for operational reasons.

##### The ADS report resulting from a way-point change event is sent once each time the event occurs.

##### An ADS report sent as a result of the occurrence of a way-point change event contains the basic ADS information and the projected profile information.

##### Figure 3-2 illustrates the way-point change event.

Figure 3-2:  Illustration of Way-Point Change Event

#### Lateral Deviation Change

##### The lateral deviation change event is triggered when the absolute value of the lateral distance between the aircraft’s actual position and the aircraft’s expected position on the active flight plan becomes greater than the lateral deviation threshold.

##### The ADS lateral deviation change report is sent once every minute when the aircraft’s lateral deviation is greater than the value of the lateral deviation threshold.

##### The avionics will cease sending ADS reports when the lateral deviation of the aircraft is less than or equal to the value of lateral deviation change threshold.

##### An ADS report sent as a result of the occurrence of a lateral deviation change event contains basic ADS information and ground vector information.

##### Figure 3-3 illustrates the lateral deviation change event.

Figure 3-3:  Illustration of Lateral Deviation Change Event

#### Altitude Change

##### The altitude change event report is triggered when the aircraft’s altitude differs negatively or positively from its value in the previous ADS report, by an amount exceeding the altitude change threshold specified in the event contract request.  If there has been no previous report, a basic ADS report is sent.

##### The ADS report resulting from an altitude change event is sent once each time the event occurs.

##### An ADS report sent as a result of the occurrence of an altitude change event contains basic ADS information and ground vector information.

##### Figure 3-4 illustrates an altitude change event.

Figure 3-4:  Illustration of Altitude Change Event

#### Altitude Range Deviation

##### The altitude range deviation is triggered when the aircraft’s altitude is higher than the altitude ceiling or lower than the altitude floor.

##### The ADS altitude range deviation event report is sent once every minute when the aircraft’s altitude is greater than the value of the altitude ceiling or less than the value of the altitude floor.

##### The avionics will cease sending ADS reports when its altitude is less than or equal to the value of altitude ceiling and greater than or equal to the value of the altitude floor.

##### An ADS report sent as a result of the occurrence of an altitude range deviation event report contains basic ADS information and ground vector information.

##### Figure 3-5 illustrates an altitude range deviation event.

Figure 3-5:  Illustration of An Altitude Range Deviation Event

#### Airspeed Change

##### The airspeed change event is triggered when the aircraft’s airspeed differs negatively or positively from its value at the time of the previous ADS report containing an air vector, by an amount exceeding the airspeed change threshold specified in the event contract request.  If there has been no such previous report, a report is sent.

##### The ADS report resulting from an airspeed change event is sent once each time the event occurs.

##### An ADS report sent as a result of the occurrence of a airspeed change event contains basic ADS information and air vector information.

#### Ground Speed Change

##### The ground speed change event is triggered when the aircraft’s ground speed differs negatively or positively from its value at the time of the previous ADS report containing a ground vector, by an amount exceeding the ground speed threshold specified in the event contract request.  If there has been no such previous report, a report is sent.

##### The ADS report resulting from a ground speed change event is sent is sent once each time the event occurs.

##### An ADS report sent as a result of the occurrence of a ground speed change event contains basic ADS information and ground vector information.

#### Heading Change

##### The heading change event is triggered when the aircraft’s heading differs negatively or positively from its value at the time of the previous ADS report containing an air vector, by an amount exceeding the heading change threshold specified in the event contract request.  If there has been no such previous report, a report is sent.

##### The ADS report resulting from a heading change event is sent once each time the event occurs.

##### An ADS report sent as a result of the occurrence of a heading change event contains basic ADS information and air vector information.

##### Figure 3-6 illustrates the heading change event.

Figure 3-6:  Illustration of Heading Change Event

#### Extended Projected Profile Change

##### The extended projected profile change event report is triggered by a change to any of the set of future way points that define the active route of flight.  The number of way points covered in the contract is either defined by a specified time interval or a by selected number from the time of the request.

##### The ADS report resulting from an extended projected profile change event is sent once each time the event occurs.

##### An ADS report sent as a result of the occurrence of a extended projected change event contains basic ADS information and extended projected profile information with the waypoints covered by either the specified time interval or within the specified number of future waypoints.

#### FOM (Figure of Merit) Field Change

##### The FOM (Figure of Merit) field change event is triggered by change in the navigational accuracy, navigational system redundancy or in the Airborne Collision Avoidance System (ACAS) availability.

##### The ADS report resulting from a FOM field change event is sent once each time the event occurs.

##### An ADS report sent as a result of the occurrence of a FOM field change event contains only basic ADS information.

#### Track Angle Change

##### The track angle change event is triggered when the aircraft’s track angle differs negatively or positively from its value at the time of the previous ADS report containing a ground vector, by an amount exceeding the track angle change threshold specified in the event contract request.  If there has been no such previous such report, a report is sent.

##### The ADS report resulting from a track angle change event is sent once each time the event occurs.

##### An ADS report sent as a result of the occurrence of a track angle change event contains basic ADS information and ground vector information.

##### Figure 3-7 illustrates the track angle change event.

Figure 3-7:  Illustration of Track Angle Change Event

## Establishment and Operation of a Periodic Contract

### The Periodic Contract provides the capability for a ground system to request periodic reports from an aircraft.  The ground specifies which optional ADS data is required (if any) in addition to the basic ADS data.  It also specifies the rate at which the basic ADS information is required and a modulus (multiple of the basic reporting rate) on the basic rate for each (if any) optional data required.

### Only one periodic contract may exist between a given ground system and a given aircraft at any one time.

### Each time a periodic contract is established, it replaces any periodic contract already in place.

### If the avionics can comply with the periodic contract request it sends the requested ADS reports.

### If there are errors in the periodic contract request, or if the avionics cannot comply with the periodic contract request, it sends a negative acknowledgment to the ground system indicating the reason for its inability to accept the contract.

### If the avionics can partially comply with the request, it sends a non-compliance notification indicating which parts of the periodic contract cannot be complied with.  Periodic reports are subsequently sent containing only the requested information that the avionics can supply.

### If the avionics cannot meet the requested report rate it will send periodic reports at 1 minute intervals.

## Cancellation of Contract(s) Operation

### Cancellation of Contracts allows the ground system to cancel a contract or all contracts that is (are) in operation.  The ground system specifies what contracts will be canceled.  The avionics acknowledges the cancellation and ceases sending the ADS reports for the canceled contract(s).

## Establishment and Operation of Emergency Mode

### This function allows the avionics to initiate emergency mode, either on instruction from the pilot or automatically.  Emergency mode is entered between the aircraft and all ground systems that currently have periodic or event contracts established with that aircraft.

### Any existing periodic contract is suspended during operation of the emergency mode.  Neither an event nor a demand contract is affected.  Initially the emergency reporting rate is the lesser of one minute or half of any existing periodic contract rate.

### The position, time and FOM are sent with each ADS emergency mode report, and the aircraft identification and ground vector sent with every fifth message.

## Modifying an Emergency Contract

### This capability allows the ground system to send an emergency contract modification message to the avionics.  The avionics modifies the reporting rate of the emergency contract, and then sends the emergency reports at the new interval.  This only affects the emergency mode reports to the ground system making the request.

## Cancellation of Emergency Mode

### This function allows the pilot to cancel the emergency mode, or the ground system to cancel the emergency mode indication.

### When the pilot cancels emergency mode , the avionics sends a cancel emergency mode message to each ground system in emergency mode.  If there was a periodic contract in place before the emergency was declared, it is reinstated.

### When the ground system cancels the emergency mode indication, the avionics continues to send ADS reports to the ground system as in emergency mode, but the reports are no longer designated as emergency reports by the ground system.

## Summary Table of ADS Functions

### Table 3-1 summarizes ADS functionality described above.

| Message | Purpose | Triggering Conditions | Source/ Destination |
| --- | --- | --- | --- |
| Demand Contract Request | Obtain single ADS report on demand, specifying what data are to be reported | Controller/FDPS request | Ground/Air |
| Periodic contract request | Request establishment of routine ADS reporting contract; specifying what data are to be reported and at what rate | Airspace proximity, changing airspace conditions | Ground/Air |
| Event contract request | Request establishment of event ADS contract; specifying certain flight conditions under which relevant data will be reported | Airspace proximity, changing airspace conditions | Ground/Air |
| Non-compliance notification | Indicates which data cannot be complied with for a given contract | Contract establishment | Air/Ground |
| ADS report | Provide ADS data according to contract request | Contract conditions for initiating a report are met | Air/Ground |
| Cancel contract request | Request cancellation of a specific contract | Air traffic conditions no longer require certain reporting | Ground/Air |
| Cancel all contracts | Request cancellation of all contracts | Air traffic conditions no longer require any ADS reports from the avionics | Ground/Air |
| Cancel emergency mode | Indicates cancellation of previously declared emergency state | Pilot canceled emergency mode | Air/Ground |
| Negative acknowledgment | Indicates that an error has been detected or that the avionics cannot comply with any part of the contract, indicating reason | Contract establishment, cancellation | Air/Ground |
| Acknowledgment | Indicates that avionics can comply with contract, however the avionics is unable to send the initial report within 0.5 seconds | Contract establishment, cancellation, Cancel emergency mode indication | Air/Ground |

Table 3-1.  ADS Functionality Summary

## Appendices to Chapter 3

### Appendix A to Part III Chapter 3 provides guidance on expected ADS message traffic in specific airspace domains.

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

CHAPTER 3 APPENDIX A

ADS MESSAGE EXCHANGE RATES

3A.1	Table 3.A-1 details the possible message exchange rate in the environments specified.  The rates shown are the expected averages, per flight.

|  | Oceanic-continental en route low density | Oceanic high-density | Continental high-density | Terminal area high-density | Aerodrome (includes approach, taxi and departure) |
| --- | --- | --- | --- | --- | --- |
| Demand Contract | 1 to 3 per FIR/Sector | 1 to 2 per FIR/Sector | 1 to 2 per FIR/Sector | 1 to 2 per FIR/Sector | 3 - 6 |
| Periodic contract request | 1 to 3 per FIR/Sector | 1 to 2 per FIR/Sector | 1 to 2 per FIR/Sector | 1 to 2 per FIR/Sector | 3 |
| Event contract request | 1 - 3 per FIR/Sector | 1 - 2 per FIR/Sector | 1 per FIR/Sector | 1 per FIR/Sector | 2 |
| Cancel contract request | 2 per FIR | 2 per FIR | 2 per FIR | 2 per FIR | 2 |
| ADS periodic report (with basic ADS) | 1 every 15 to 30 min | 1 every 5 to 15 min. | 1 every 10 s. to 5 min. | 1 every 3 to 10 s. | 1 every 0.5 to 5 s. |
| Air and/or ground vector in ADS periodic report | 1 - 3 per FIR/Sector | 1 every 4th report | 1 every 4th report | 1 every 4th report | 1 every 2nd report |
| Meteorological information in ADS periodic report | 1 per waypoint, or 1 per hour, | 1 per waypoint, or 1 per hour, | 1 per waypoint, or 1 per hour, | Negligible | Negligible |
| ADS event report with projected profile | 1 per waypoint | 1 per waypoint | 1 per waypoint | 1 per waypoint | 1 per waypoint |
| ADS demand report with extended projected profile | 1 per FIR | 1 per FIR | 1 per FIR | 1 per FIR | 1 |
| Other ADS Messages | Under exceptional conditions | Under exceptional conditions | Under exceptional conditions | Under exceptional conditions | Under exceptional conditions |
| Instantaneous number of aircraft to be supported per ATSU | 300 | 750 | 1250 | 450 | 250 |

Table 3A-1.  Exchange Rates Expected for ADS Messages

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

# ADS MESSAGE DESCRIPTIONS

## Message Descriptions

### Basic ADS Information

#### Every ADS report contains the following information:

- the 3-D position of the aircraft (latitude, longitude, and altitude);

- the time; and

- an indication of the accuracy of the position data information (figure of merit).

### Optional ADS Information

#### In addition to the basic information included in each ADS report, an ADS report may contain any (or all) of the following information:

- aircraft identification,

- ground vector,

- air vector,

- projected profile,

- meteorological information,

- short term intent,

- intermediate intent, and

- extended projected profile.

#### The aircraft identification is contained in field 7 of the ICAO flight plan.

#### The ADS ground vector is composed of the following information:

- Track,

- Ground speed, and

- Rate of climb or descent.

#### The ADS air vector is composed of the following information:

- Heading,

- Mach or IAS, and

- Rate of climb or descent.

#### The ADS projected profile is composed of the following information:

- Next way-point;

- Estimated altitude at next way-point;

- Estimated time at next way-point;

- (Next + 1) way-point;

- Estimated altitude at (next + 1) way-point; and

- Estimated time at (next + 1) way-point.

#### The ADS meteorological information is composed of the following:

- Wind direction,

- Wind speed,

- Temperature, and

- Turbulence.

#### The ADS short-term intent is composed of the following information:

- Latitude at projected intent point,

- Longitude at projected intent point,

- Altitude at projected intent point, and

- Time of projection.

##### If an altitude, track or speed change is predicted to occur between the aircraft's current position and the projected intent point (indicated above), additional information to the short term intent data would be provided as intermediate intent (repeated as necessary) as follows:

- Distance from current point to change point,

- Track from current point to change point,

- Altitude at change point, and

- Predicted time to change point.

#### The ADS Extended projected profile is composed of the following information:

- Next way-point

- Estimated altitude at next way-point

- Estimated time at next way-point

- (Next + 1) way-point,

- Estimated altitude at (next + 1) way-point,

- Estimated time at (next + 1) way-point,

- (Next + 2) way-point,

- Estimated altitude at (next + 2) way-point,

- Estimated time at (next + 2) way-point...

- ...[repeated for up to (next + 128) way-points]

### Positive Acknowledgment

#### A positive acknowledgment indicates acceptance of a requested contract and contains no further information.

### Negative Acknowledgment

#### A negative acknowledgment indicates rejection of the requested contract and may contain information for the rejection.

### Non-Compliance Notification

#### A non-compliance notification contains an indication what part of a requested contract cannot be complied with.

### Demand Contract Message

#### The demand contract message indicates the contract type and indicates which of the optional ADS information is to be included in the ADS report.

### Demand ADS Response Message

#### A demand ADS report contains the basic ADS data and optional ADS data as required.

### Event Contract Message

#### The event contract message indicates the contract type, contains an indication of the events to be reported on, together with thresholds (as required) for each event specified.

### ADS Event Response Messages

#### The event contract response message contains an identification of the event type and the required ADS data for the particular event.

### Periodic Contract Message

#### The periodic contract message indicates the contract type, the required report interval, an indication of which of the optional ADS information is to be included in the periodic reports, and the modulus from the basic interval for each optional field to be included.

### Periodic ADS Response Message

#### A periodic ADS report contains the basic ADS data and optional ADS data as required.

### Cancel Contract Message

#### The cancel contract message contains an indication of the contract (i.e., periodic, or event) to be canceled.

#### A cancel contract message without a contract type parameter indicates that all ADS contracts with the ground system are to be canceled.

### Emergency Mode Messages

#### The position, time and FOM are sent with each ADS emergency mode report.  In addition to the above, the aircraft identification and ground vector are sent with every fifth message.

### Modify Emergency Mode Message

#### The emergency mode modification message contains only a new reporting rate.

### Cancel Emergency Mode Message

#### The cancel emergency mode message indicates that a pilot has canceled the emergency mode.

### ADS Message Glossary and Message Variable Range and Resolution

#### An ADS data glossary is provided in Appendix A to this chapter.

#### The range and resolution for variables used in ADS messages is presented in Appendix B to this chapter.

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

CHAPTER 4 APPENDIX A

ADS MESSAGE DATA GLOSSARY

4A.1	ADS Message Data Glossary

4A.1.1	The following data are used as the ADS message variables, or component of the variables, and are shown here in alphabetical order:

ADS Event Report:  ADS information consisting of a sequence of Event Type and ADS Report.

ADS Emergency Report:  ADS information consisting of the following sequence:

- Position,

- Time Stamp,

- FOM,

- Aircraft Identification (optional), and

- Ground Vector (optional).

ADS Report:  ADS information consisting of the following sequence:

- Position,

- Time Stamp,

- FOM,

- Aircraft Identification (optional),

- Projected Profile (optional),

- Ground Vector (optional),

- Air Vector (optional),

- Meteorological Information (optional),

- Short Term Intent (optional), and

- Extended Projected Profile (optional).

Aircraft Identification:  Field 7 of the ICAO flight plan.

Air Speed:  Provides airspeed as a choice of the following:  Mach, IAS, or Mach and IAS.

Air Speed Change:  Provides the threshold of change for either Mach speed or indicated air speed that requires that an avionics generate an ADS report when the current aircraft speed differs more than the specified threshold from the air speed in the last ADS report.

Air Vector:  Provides the air vector as a sequence of Heading, Air Speed, and Vertical Rate.

Altitude:  Specifies altitude.

Altitude Ceiling:  The altitude above which an Altitude Deviation Event is triggered.  Provided as an Altitude.

Altitude Change:  Provides the threshold of change for altitude that requires the avionics to generate an ADS report when the current altitude differs by more than the specified threshold from the altitude in the last ADS report.

Altitude Floor:  The altitude below which an Altitude Deviation Event is triggered.  Provided as an Altitude.

Altitude Range Change:  Threshold of change permissible between altitudes in consecutive ADS reports.

Cancel Contract:  Allow the ground to cancel event and/or periodic contracts in effect.

Contract Type:  Indicates which type of ADS contract is specified:  demand, event, or periodic.

Demand Contract:  Indicates that an avionics is to generate an ADS report containing the indicated data upon receipt of the contract.  The data that can be indicated includes:  aircraft identification, projected profile, ground vector, air vector, meteorological information, short term intent, and extended projected profile.

Distance: Distance.

ETA: Estimated time of arrival at a waypoint.

Event Contract:  Indicates Event Types and the threshold for the specified event types.

Event Type:  An indication of what type of ADS event is specified,

- Vertical rate change,

- Way-point change,

- Lateral deviation change,

- Altitude change,

- Altitude range deviation,

- Airspeed change,

- Ground speed change,

- Heading change,

- Extended projected profile change,

- FOM (Figure of Merit) field change, and

- Track angle change.

Extended Projected Profile:  Provides a sequence (1-128) of waypoint position data and ETA at the specified waypoint.

Extended Projected Profile Change:  Indicates that an ADS report be generated when there is a change in the extended projected profile.

Extended Projected Profile Modulus:  Sequence of Modulus and Extended Projected Profile Request.

Extended Projected Profile Request:  a choice indicating whether the extended projected profile information is to be provided on a time or waypoint interval, and the interval of the specified choice.

Following Way Point:  Indicates the waypoint after the next way point.

FOM:  Indicates the figure of merit of the current ADS data.  The information consists of the Position Accuracy  and indications 1) whether or not multiple navigational units are operating, and 2) whether or not ACAS is available.

FOM Field Change:  Indicates that an ADS report be generated when any FOM field changes.

Ground Speed:  Provides ground speed.

Ground Speed Change:  Provides the threshold of change for ground speed that requires the avionics to generate an ADS report when the current aircraft ground speed has differed by more than the specified threshold from the last ADS report.

Ground Vector:  Provides the ground vector of an aircraft provided as a sequence of Track, Ground Speed, and Vertical Rate.

Heading:  Provides aircraft heading in degrees.

Heading Change:  Provides the threshold of change for heading in degrees that requires the avionics to generate an ADS report when the current heading has differed by more than the specified threshold from the last ADS report.

IAS:  Indicated air speed.

ICAO Facility Designator:   The 8 letter code which uniquely defines an ICAO ATSU facility.

Intermediate Intent:  Set of points between current position and the time indicated in the Short Term Intent.  Consists of a sequence of the following:  Distance, Track, Altitude and Projected Time.

Lateral Deviation Change:  Provides the threshold of change for lateral value that requires the avionics to generate an ADS report when the current lateral deviation exceeds the specified threshold.

Latitude:  Latitude in degrees, minutes, and seconds.

Longitude:  Longitude in degrees, minutes, and seconds.

Mach:  Air speed given as a Mach number.

Mach and IAS:  Air speed provided as both Mach and Indicated Air Speed.

Meteorological Information:  A sequence of Wind Direction, Wind Speed, Temperature and Turbulence.

Modulus:  Provides a multiplier on the basic ADS report interval.

Next Time:  Time at next waypoint.

Next Way Point: specifies the next waypoint in the avionics.

Non-Compliance Notification: used to indicate partial compliance to a contract.

Periodic Contract:  Provides the requirements for the generation of ADS reports.  The periodic contract provides the reporting interval, and the modulus for when and what optional data is included in an ADS periodic report.

Position:  Provides aircraft position information using a sequence of Latitude, Longitude, and Altitude.

Position Accuracy:  An indication of the navigational accuracy.

Projected Profile:  A sequence of Next Way Point, Next Time, and Following Waypoint.

Projected Time:  Predicted time at a particular point.

Reporting Interval:  Provides the required ADS reporting interval.

Report Type:  Indicates which type of ADS report is provided:  demand, event or periodic.

Request Type:  A choice indicating which type of ADS request is being uplinked.  The choices are as indicated below:

- cancel event contract,

- cancel periodic contract,

- demand contract,

- event contract,

- cancel emergency,

- modify emergency,

- periodic contract, or

- cancel all contracts.

Short Term Intent:  A sequence of Position, ETA, and Intermediate Intent (optional) data structures.

Temperature:  Temperature in degrees Celsius.

Track:  Provides track angle in degrees.

Track Angle Change:  Provides the threshold of change for track angle in degrees which triggers avionics to generate an ADS report when the current track angle differs by more than the specified threshold from the track angle in the last ADS report.

Turbulence:  Indicates severity of turbulence on a scale of 0-15.

Vertical Rate:  Rate of climb/descent (climb positive, descent negative).

Vertical Rate Change: The threshold of change for vertical rate that requires the avionics to generate an ADS report when the current vertical rate differs by more than the specified threshold from the vertical rate in the last ADS report.

Way Point Change:  Change in the next waypoint information.

Wind Direction:  Wind direction in degrees.

Wind Speed:  Wind speed.

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

CHAPTER 4 APPENDIX B

ADS VARIABLES RANGE AND RESOLUTION

4B.1	ADS Variables Range and Resolution

4B.1.1	Table 4B-1 provides the required range and resolution for the message variables used in the ADS application.

| Category | Variables/Parameters | Unit | Range | Resolution |
| --- | --- | --- | --- | --- |
| Aircraft identification |  | IA5 | 2-8 characters | N/A |
| Altitude |  | Altitude Units | -10 to +10 000 | 1 |
| Altitude Range Change |  | Altitude Units | 1 - 110 | 1 |
| Airspeed | Mach | Mach number | .5 - 4.0 | 0.001 |
|  | IAS | Speed-unit | -110 - + 4200 | 1 |
| Date | Year | Year of Century | 0 - 99 | 1 |
|  | Month | Month of Year | 1 - 12 | 1 |
|  | Day | Day of Month | 1 - 31 | 1 |
| Distance |  | Distance Units | 1-2000 | 1 |
| FOM |  | Integer | 0-7 | 1 |
| Ground Speed |  | Speed-unit | -110 - + 4200 | 1 |
| Ground Speed Change |  | Speed-unit | 0-500 | 1 |
| Heading |  | Degrees | 0.1 - 360 | 0.1 |
| Heading Change |  | Degrees | 1 - 359 | 1 |
| ICAO Facility Designator |  | Character string | 8 | N/A |
| Lateral Deviation Change |  | Distance Units | 1-200 | 1 |
| Latitude | Latitude Degrees | Degrees | ± 90 | 1 |
|  | Latitude Minutes | Minutes | 0 - 59 | 1 |
|  | Latitude Seconds | Seconds | 0 - 59.9 | 0.1 |
| Longitude | Longitude Degrees | Degrees | ± 180 | 1 |
|  | Longitude Minutes | Minutes | 0 - 59 | 1 |
|  | Longitude Seconds | Seconds | 0 - 59.9 | 0.1 |
| Modulus |  | Integer | 1-100 | 1 |
| Reporting Interval |  | Second | 1-59 | 1 |
|  |  | Minutes | 1-120 | 1 |
| Temperature | Temperature Celsius | Degrees Celsius | - 100 to + 100 | 1 |
| Time | Time Hours | Hours of Day | 0 - 23 | 1 |
|  | Time Minutes | Minutes of Hour | 0 - 59 | 1 |
|  | Time Seconds | Seconds of Minute | 0 - 59 | 1 |
| Track | Track Angle | Degrees | 0.1 - 360 | 0.1 |
| Track Angle Change |  | Degrees | 1 - 359 | 1 |
| Turbulence | Relative measure | Bit string | 0 - 15 | N/A |
| Vertical Rate |  | Altitude Units/ Minute | -3310 to +3310 | 1 |
| Wind | Wind direction | Degrees | 1 - 360 | 1 |
|  | Wind Speed | Speed Units | 0 - 600 | 1 |

Table 4B-1:  ADS Variables Range and Resolution

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

# ADS MESSAGE SEQUENCES

Note  These sequence diagrams illustrate the expected message sequence for each ADS function, and do not illustrate exception handling.

## ADS Demand Contract

### The following sequence of messages, shown in Figure 5-1, occurs when the ADS Demand Contract is sent and the avionics can comply with the request.

Figure 5-1: Demand Contract Request with ADS Demand Report

### The following sequence of messages, shown in Figure 5-2, occurs when the ADS Demand Contract is sent and the avionics cannot comply with the request.

Figure 5-2: Demand Contract Request with Negative Acknowledgment

### The following sequence of messages, shown in Figure 5-3, occurs when the ADS Demand Contract is sent and the avionics cannot comply fully with the request.

Figure 5-3: Demand Contract Request with Non-Compliance Notification

## ADS Event Contract

### The following sequence of messages, shown in Figure 5-4, occurs when an ADS Event Contract is sent and the avionics can comply with the request.

Figure 5-4: Event Contract Request, Aircraft can Comply

### The following sequence of messages, shown in Figure 5-5, occurs when the ADS Event Contract is sent and the avionics cannot comply with the request.

Figure 5-5: Event Contract Request with Negative Acknowledgment

### The following sequence of messages, shown in Figure 5-6 occurs when the ADS Event Contract is sent and the avionics cannot comply fully with the request.

Figure 5-6: Event Contract Request with Non-Compliance Notification

## ADS Periodic Contract

### The following sequence of messages, shown in Figure 5-7, occurs when an ADS Periodic Contract is sent and the avionics can comply with the request.

Figure 5-7:  Periodic Contract Request, Aircraft can Comply

### The following sequence of messages, shown in Figure 5-8, occurs when the ADS Periodic Contract is sent and the avionics cannot comply with the request.

Figure 5-8: Periodic Contract Request with Negative Acknowledgment

### The following sequence of messages, shown in Figure 5-9 occurs when the ADS Periodic Contract is sent and the avionics cannot comply fully with the request.

Figure 5-9: Periodic Contract Request with Non-Compliance Notification

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

# ADS PROCEDURES

## Introduction

### As stated earlier in the document, operational requirements do not necessarily need a technical solution, but may be satisfied by the institution of suitable local or inter-facility procedures.

## Procedures for Effective use of ADS.

### Advance information on the data link capabilities of participating aircraft needs to be known to the appropriate ground facilities.  While this is envisaged as being contained in the flight plan, procedures must be in place to enable this information to be exchanged between units in areas where other methods of indicating aircraft intent are used.

### In line with current practice, the transferring ground system will advise the receiving ground system of the capabilities and intent of the aircraft wishing to enter the receiving ground system airspace to permit proper entry of the information into the receiving ground system’s automation.

### While at least four ADS contracts may be simultaneously in force, appropriate local procedures will need to be in place to ensure that non-current contracts are dropped in sufficient time to allow the receiving ground system to set up the controlling ADS contract.  Such procedures will also take care of the case where aircraft are crossing from airspace where ADS service is provided into non-ADS airspace, to ensure closure of all ADS contracts, and thus efficient use of resources.

### The probability exists that errors may be input into the aircraft navigation system prior to departure.  Since ADS is by definition dependent on the on-board navigation system, procedures will be required to ensure pre-departure conformance checking in order to correct these errors.

### As ADS will be implemented regionally to different levels of capability, with a mixed-equipage aircraft fleet, procedures will be necessary between adjacent ATS facilities to ensure efficient levels of service to all aircraft users.

### It is anticipated that specific ATC procedures will be developed as experience is gained with the system, and as appropriate separation minima are developed for global use.

### ATS providers should ensure that the number of separation standards applied in a given airspace are kept to a minimum.

### In a mixed environment, the source of surveillance data should be readily apparent to the controller.

### In a mixed environment, procedures must be in place to ensure that all sources of the display refresh rate will be synchronous regardless of the source of surveillance information data.

PART III- AUTOMATIC DEPENDENT SURVEILLANCE

# EXCEPTION HANDLING

## Handling of Messages Received Out of  Sequence

### The sequencing of messages between an airborne system and a ground system is dependent on the type of contract established.

### If the ground system receives messages of the same contract type out of sequence, as determined by the time stamping of the messages, the ground system will terminate that contract and notify both the controller and the airborne system.

## Non Receipt of Messages

### Non receipt of requested demand and periodic contracts reports will be a matter for local implementation.

### Non receipt of requested baseline information as part of an event contract will be a matter for local implementation.

Note:  Non receipt of event contract reports may be undetectable.

## Invalid Data and Logical Errors

### Ground systems will be capable of detecting logical errors and invalid data.  In these circumstances the controller will be notified.

-- END --