PART  II

DATA  LINK  INITIATION  CAPABILITY

(DLIC)

PART II - DATA LINK INITIATION CAPABILITY

# APPLICATION OVERVIEW

## Introduction

### Purpose

#### This section contains guidance material for the Data Link Initiation Capability (DLIC).  The DLIC process supports addressing requirements for Air Traffic Services Communication (ATSC) such as Automatic Dependent Surveillance (ADS), Controller Pilot Data Link Communications (CPDLC), and Digital Flight Information Services (DFIS).

### Background

#### The DLIC provides the necessary information to enable data link communications between ATC ground and aircraft systems.  The DLIC encompasses the following functions:

- Logon:  data link application initiation and, if required, flight plan association,

- Update:  updating of previously coordinated initiation information,

- Contact:  instructions to perform data link initiation with another specified ground system,

- Dissemination:  local dissemination of information, and

- Ground Forwarding:  ground-ground forwarding of logon information.

PART II - DATA LINK INITIATION CAPABILITY

# GENERAL REQUIREMENTS

## DLIC Priority

### The priority for DLIC will be “flight regularity communications”, as determined by the ATN Internet Protocol Priority categorization.

## DLIC Performance Requirements

### Response Timers

#### Upon receipt of a Logon Request from an aircraft the ground system will generate a Logon Response within 0.5 seconds.

#### Upon receipt of a Contact Request from a ground system the aircraft will generate a Logon Request within 0.5 seconds to the indicated ground system.

#### Upon receipt of a Logon Response from a ground system, during the operation of a Contact Function, the aircraft will generate a Contact Response within 0.5 seconds to the initiating ground system.

## Time Requirements

### Wherever time is indicated in the DLIC, it will be accurate to within 1 second of UTC.

### Time stamping is required for all messages.  The time stamp will consist of the date (YYMMDD) and time (HHMMSS).

## Address Management

### Procedures need to be in place to allow dissemination of DLIC FIR addresses to aircraft.  It is recommended that each ground facility supporting air-ground data link applications have DLIC addresses published in State AIPs.  This would allow look-up tables to be imbedded in the flight deck avionics implementation, reducing pilot input.

Note:  A given FIR may have multiple DLIC addresses; and more than one FIR may share the same DLIC address.

PART II - DATA LINK INITIATION CAPABILITY

# DLIC FUNCTIONAL CAPABILITIES

## The DLIC Logon Function

### Operational Requirements

#### The DLIC logon function must provide a method for an airborne system to initiate data link service with a ground system.  The DLIC logon function must also provide the ground system with the capability to terminate a DLIC link.

#### The airborne system provides information on each application (e.g., ADS, CPDLC, and DFIS) for which it requires a data link service.  Where ground initiated data link application services are requested, additional information may be provided to allow unambiguous association of the aircraft with flight plan information stored on the ground.  Where service for exclusively air initiated applications is requested, the airborne system provides only the application information (i.e., name, and version).

#### The ground system responds indicating whether or not the data link initiation was successful, and indicates whether to terminate or maintain a DLIC link.  If successful, the ground system response includes information on each data link application it can support.  Unsuccessful logon will be handled by local procedures.

#### Up to a maximum of 256 applications must be supported.

#### Each time a logon is accomplished between a given aircraft and a ground system the latest exchanged information replaces any previous information for the indicated application.

#### When initiating the DLIC logon function the airborne system must provide its DLIC name, address, and version number as part of the Logon Request message.

#### The DLIC logon function will provide the capability for the ground system to accept any logon request.  If the ground system does not have a flight plan for the aircraft requesting a logon, or the flight plan information from the aircraft does not correlate with the ground flight plan information appropriate to that aircraft, the ground system may not be capable of supplying information for a requested application.

#### The DLIC logon function will provide the capability to indicate a reason for rejecting a DLIC logon request.

### Message Descriptions

#### The Logon Request message provides:

- the aircraft’s DLIC application name, address, and version number,

- the application name, version number and address, for each application that can be ground-initiated for which the air craft requires service,

- the application name and version number for each application which can be air-initiated, and

- flight plan information when required.

#### The Logon Response message provides an indication of success or failure of the logon and if failure, may provide a reason for the failure.  For each desired air-initiated application the ground provides the application name, version number, and address.

## The DLIC Update Function

### Operational Requirements

#### The DLIC update function must provide a method for the ground system to provide updated ground addressing information to an aircraft system for applications previously coordinated in the logon function.

#### Up to a maximum of 256 applications can be updated.

#### Each time an update function is accomplished between a given ground system and an aircraft, only the updated information is altered; other previously coordinated data remains valid.

#### Message Descriptions

##### An Update message can provide updated ground information for up to 256 applications.  For each updated application the ground provides the application’s name, version number and address.

## The DLIC Contact Function

### Operational Requirements

#### The DLIC contact function must provide a method for the ground system to request the aircraft system to initiate the logon function with another designated ground system.  It is expected that the contact function will only be used when ground connectivity is not available between respective ground system applications.  This function presumes that the logon function has been accomplished with the ground system initiating the contact function.  After completing the requested logon process, or if unable to do so, the airborne system provides the ground system that issued the contact request a contact status that indicates the success or lack of success of the requested contact.

### Message Descriptions

#### The Contact Request message provides the ground system address that the initiating ground system is requesting the aircraft to logon with.

#### The Contact Response message provides the information indicating whether the requested contact was successful.

## DLIC Dissemination Function

### Operational Requirements

#### The DLIC dissemination function is a means of making application information available to other applications in the air or ground systems.  It must make available the application name, address, and version number for each application exchanged in the logon, update or ground forwarding functions to other applications in the aircraft or on the ground respectively.

### Message Descriptions

#### There are no messages exchanged for this function.

## DLIC Ground Forwarding

### Operational Requirements

#### Where ground-ground connectivity is available between ground systems, the DLIC ground forwarding function must provide a method for a ground system that has accomplished data link initiation to pass the airborne information to other data link ground system(s).  Where this capability is implemented, an aircraft only has to perform an initial logon, and each subsequent ground system can use the DLIC update function to accomplish data link initiation.

#### The identical Logon Request message received by the ground system (from either an aircraft or another ground system), is used to ground forward data link initiation information between ground systems.

### Message Descriptions

#### The Ground Forwarding message contains the airframe identification to which the information pertains and the same application and flight plan information as was contained in the initial Logon Request message.

## Guidance on Expected DLIC Message Traffic

### Appendix A to Part II Chapter 3 provides guidance on expected DLIC message traffic in specific airspace domains.

PART II- DATA LINK INITIATION CAPABILITY

CHAPTER 3 APPENDIX A

EXPECTED DLIC MESSAGE TRAFFIC

3A.1	Expected DLIC Message Traffic

3A1.1	Table 3A-1 details the anticipated message exchange rate for DLIC in the environments specified.  The rates shown are the expected averages, per flight.

| Parameter | Oceanic-Continental enroute low density | Oceanic high-density | Continental high-density | Terminal area high-density | Aerodrome (Including: Approach/ Departure/ Taxi) |
| --- | --- | --- | --- | --- | --- |
| DLIC message exchange per aircraft | 2 per ATSU | 2 per ATSU | 2 per ATSU | 2 per ATSU | 2 per flight |
| Instantaneous number of aircraft to be supported per ATSU | 300 | 750 | 1250 | 450 | 250 |

Table 3A-1.  Exchange Rates Expected for DLIC Messages

PART II - DATA LINK INITIATION CAPABILITY

# DLIC MESSAGE DESCRIPTIONS

## DLIC Messages

### Logon Request Message

#### The DLIC Logon Request message contains the following information:

- airframe identification,

- air applications

- aircraft identification (as required)

- ICAO facility designation (as required),

- Departure airport (as required),

- Destination airport (as required), and

- ETD (as required).

### Logon Response Message

#### The DLIC Logon Response message contains the following information:

- response,

- failure reason (optional), and

- ground applications (optional).

### Update Message

#### The DLIC Update message contains the following information:

- ground applications.

### Contact Request Message

#### The DLIC Contact Request message contains the following information:

- ICAO facility designation, and

- DLIC address.

### Contact Response Message

- The DLIC Contact Response message contains only a response.

### Ground Forwarding Message

#### The DLIC Forward message contains the following information:

- airframe identification,

- air applications

- aircraft identifcation (if provided in Logon Request),

- ICAO facility designation (if provided in Logon Request),

- Departure airport (if provided in Logon Request),

- Destination airport (if provided in Logon Request), and

- ETD (if provided in Logon Request if provided in Logon Request).

## Message Content Glossary

Air Applications:  An indication of 1-256 airborne data link applications.  Consists of Application Name, Version Number, and, when required for ground initiated applications, Application Address.

Aircraft Identification:  Field 7 of the ICAO flight plan.

Airframe Identification:  24 bit ICAO identification.

Airport:  An IA5 string of 4 characters indicating an airport’s identification.

Application Address:  An application’s unique technical communications address.

Application Name:  An IA5 string of 3 characters indicating an application name (e.g., ADS, CPC, FIS, or CMA.)

Departure Airport:  Name of departure airport.

Destination Airport:  Name of destination airport.

DLIC Address:  Application Address for DLIC.

ETD:  Estimated time of departure.

Ground Applications:  An indication of 1-256 ground data link applications.  Consists of Application Name, Version Number, and Application Address data.

HHMMSS:  Hour, minutes, seconds.

ICAO Facility Designation:  An IA5 string of 8 characters indicating the ICAO facility designation for a ground system.

Response:  An indication of whether the requested action was successful.  The values are as specified:

- success,

- logon not successful, or

- contact not successful.

Response Reason:  Reason for logon failure.

Time:  Time in hours and minutes.

Time Hours: Time as hours of day.

Time Minutes:  Time as minutes of an hour.

Version Number:  Version number of the specified application.

YYMMDD:  Date in year, month, day

## Data Structure Range and Resolution

### Table 4-1 provides the range and resolution of the data structures used in DLIC.

| Data Structure | Type | Range | Resolution |
| --- | --- | --- | --- |
| Aircraft Identification | IA5 | 2-8 | N/A |
| Airframe Identifiction | Bit String | 24 bits | N/A |
| Airport | IA5 | 4 characters | N/A |
| Application Name | IA5 | 3 characters | N/A |
| HHMMSS | Integer | HH - 00-23 MM - 00-59 SS - 00-59 | 1 1 1 |
| ICAO Facility Designation | IA5 | 8 characters | N/A |
| Time Hours | Integer | 0-23 | 1 hour |
| Time Minutes | Integer | 0-59 | 1 minute |
| Version Number | Integer | 0-255 | 1 |
| YYMMDD | Integer | YY - 0-99 MM - 1-12 DD - 1-31 | 1 1 1 |

Table 4-1:  DLIC Data Range and Resolution

PART II - DATA LINK INITIATION CAPABILITY

# OPERATIONAL MESSAGE SEQUENCE

Note: these sequence diagrams illustrate the expected message sequence for each DLIC function, and do not illustrate exception handling.

## Sequence Rules

### DLIC Logon Function

#### The following sequence of messages, shown in Figure 5-1, occurs when the DLIC logon function is initiated.

Figure 5-1:  Sequence Diagram for DLIC Logon Function

### DLIC Update Function

#### The following sequence of messages, shown in Figure 5-2, occurs when the DLIC update function is initiated.

Figure 5-2:  Sequence Diagram for DLIC Update Function

### DLIC Contact Function

#### The following sequence of messages, shown in Figure 5-3, occurs when the DLIC contact function is initiated.

Figure 5-3:  Sequence Diagram for DLIC contact function

#### DLIC Ground Forwarding Function

##### The following sequence of messages, shown in Figure 5-4, occurs when the DLIC ground forwarding function is initiated.

Figure 5-4:  Sequence Diagram for DLIC Ground Forwarding Function

Note:  This diagram includes ground forwarding of addresses and subsequent update to an aircraft.

PART II - DATA LINK INITIATION CAPABILITY

# DLIC PROCEDURES

## Procedures

### The procedures described here are the minimum required for initiation of data link services.

### Sufficient procedures will be required for the airborne and ground system users to initiate the capability to provide and receive services via data link applications.

### These procedures may be performed by automation, or manually by the aircrew or controller depending upon the implementation.

### The system will provide the pilot, controller, or automation with the capability to access initiation data for the purposes of conducting data link services.  The information must be unique in regard to a specific aircraft to ATSU.

## Sequence of Events

### An aircraft  will inform a ground system of its requirement to participate in data link services by providing the information as outlined in 6.3 below.

### A ground system will in turn provide that aircraft with the necessary information to establish a link with the desired application(s), (e.g., ADS, CPDLC, DFIS, etc.).

### The DLIC application will make the initiation information available to other applications as required.

### In the event that the initiation process fails, the airborne system/pilot will be informed.

## Required Information

### States will provide the information regarding application names, addresses, and version numbers for those applications supported in airspaces for which they have jurisdiction.  This information should be published in Aeronautical Information Publications (AIPs).

## DLIC Link Procedural Requirements

Note:  An operational requirement for maintaining a DLIC link has been identified for small FIRs.

-- END --