PART  IV

CONTROLLER  PILOT  DATA  LINK  COMMUNICATION

(CPDLC)

PART IV- CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

# APPLICATION OVERVIEW

## Introduction

### Controller pilot data link communication (CPDLC) is a means of communication between controller and pilot, using data link for ATC communication.

### The CPDLC application provides air-ground data communication for ATC service.  This includes a set of clearance/information/request message elements which correspond to voice phraseology employed by Air Traffic Control procedures.  The controller is provided with the capability to issue altitude assignments, crossing constraints, lateral deviations, route changes and clearances, speed assignments, radio frequency assignments, and various requests for information.  The pilot is provided with the capability to respond to messages, to request clearances and information, to report information, and to declare/rescind an emergency.  A "free text" capability is also provided to exchange information not conforming to defined formats.  An auxiliary capability is provided to allow a ground system to use data link to forward a CPDLC message to another ground system.

### Controllers and pilots will use CPDLC in conjunction with the existing voice communication.  It is expected to be used for routine or frequent types of transactions.  Although initial implementation is intended to conform to existing procedures, it is anticipated that future evolution of the system and procedures will result in the greater automation of functions for both aircraft and ground systems.

### The introduction of CPDLC does not affect the principle that there is only one controlling authority for a given aircraft at a given time.

### Sending a message by CPDLC consists of selecting the recipient, selecting the appropriate message from a displayed menu or by other means which allow fast and efficient message selection, and executing the transmission.  The received message may be displayed and/or printed.

### CPDLC may be used to remedy a number of shortcomings of voice communication, such as voice channel congestion, misunderstanding due to poor voice quality and/or misinterpretation, and corruption of the signal due to simultaneous transmissions.

### Implementation of CPDLC will significantly change the way pilots and controllers communicate.  The effect of CPDLC on operations should be carefully studied before deciding the extent to which voice will be replaced by data link.

### The following aspects of CPDLC should be taken into account in considering its application and in defining procedures:

- the total time required for selecting a message, transmission of the message, and reading and interpretation of the message;

- the head-down time for the pilot and controller; and

- the inability of the pilot to monitor other data link transmissions to and from other aircraft in the same area of operation.

## CPDLC Definitions

Current Data Authority:  The ground system which is technically permitted to conduct a CPDLC dialogue with an aircraft.

Downstream Data Authority: The ground system which is technically permitted to conduct a DSC dialogue with an aircraft.

Downstream Clearance:  A downstream clearance is a conditional clearance issued to an aircraft by an ATSU that is not the current control authority of that aircraft.  The clearance may only be activated by the authority in whose airspace the DSC applies.

Next Data Authority:  The ground system so designated by the Current Data Authority.

## Use of CPDLC In ATS

### CPDLC is expected to be used for routine operations in areas where the use of voice communication is considered not efficient or unnecessary, thereby reducing voice channel use and possibly reducing the number of required voice channels.

### Voice communication will continue to be required.  Voice is particularly suited where a rapid exchange, short transactions communication style is required.

### CPDLC messages are classified according to uplink and downlink categories.  Each message has associated urgency, alerting and response attributes.

### The CPDLC application has three primary functions:

- the exchange of controller/pilot messages with the current data authority,

- the transfer of data authority involving current and next data authority, and

- downstream clearance delivery with a downstream data authority.

### CPDLC Links

#### To accomplish the CPDLC application three CPDLC links are defined:

- CDA link; the CPDLC link with the Current Data Authority,

- NDA link; the CPDLC link with the Next Data Authority, and

- DDA link; the CPDLC link with a Downstream Data Authority.

PART IV - CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

# General Requirements

## Message Handling

### Message Transmission

#### The CPDLC application requires:

- that messages are generated and sent in a time ordered sequence, and

- that messages are delivered in the order that they are sent.

#### The system will ensure that messages are sent to the specified recipient.

#### When a ground system receives a message requesting an unsupported function or service, the ground system will respond indicating that the requested service is unsupported.

#### The system will be capable of supporting up to 64 unfinished message exchanges between one ground system and each of the aircraft with which it is linked.

## Quality of Service

### The ground system will have the ability to specify its required QOS based on a user preferred combination of message delay, cost, and permissible error rate.

## Time Requirements

### Wherever time is used in the CPDLC application, it will be accurate to within 1 second of UTC.

### Time stamping will be available for all messages.  The time stamp will consist of the date (YYMMDD) and time (HHMMSS).  The time stamp will be the time the message is dispatched by the originating user.

## CPDLC Priority

### The priority for all CPDLC messages will be “normal priority flight safety messages” as determined by the ATN Internet Protocol Priority categorization.

PART IV- CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

# CPDLC Functional Capabilities

## Data Link Requirements for CPDLC

### Controller Pilot Message Exchange With The Current Data Authority (CDA)

#### This function allows the establishment of a CDA link between an aircraft and the CDA for the exchange of CPDLC messages.  This function provides messages for the following:

- general information exchange;

- clearance:

  - delivery,

  - request, and

  - response;

- altitude/identity surveillance;

- monitoring of current/planned position;

- advisories:

  - request and

  - delivery;

- system management functions; and

- emergency situations.

### Transfer of Data Authority

#### The Transfer of Data Authority function provides the capability for the CDA to designate another ground system as the Next Data Authority (NDA).  Once a ground system becomes the NDA, a NDA link can be established.  This capability is intended to prevent a loss of communication that would occur if the NDA were prevented from actually establishing any CPDLC link with an aircraft until it became the CDA.

### Down Stream Clearance Message Exchange

#### The Down Stream Clearance (DSC) function provides the capability for the aircraft to establish a Downstream Data Authority (DDA) link with an air traffic service unit which is not the current data authority for the purpose of receiving a down stream clearance.

#### Although the entire CPDLC message set will be available for the DSC function, it is recommended that regional agreements be established to govern the use of the CPDLC message elements for the DSC function.  It is expected that the primary use of the DSC function will be for clearance delivery messages.  Other uses of the DSC function may be identified.

#### There will be procedures that prevent the pilot from executing a clearance received on a DDA link until the aircraft enters the airspace of that DDA.  If the information a pilot receives on the DDA link requires action while still in the airspace of the CDA, the clearance for such action must be obtained from the CDA.

## Composition of a CPDLC Message

### A CPDLC message is composed of a message header, and from one to five message elements.

### The message header for air/ground message exchange is composed of a message identification number, a message reference number if required, a time stamp, and an indication if a logical acknowledgment is required (optional).

### A message element consists of a message element identifier, data as indicated by the specified message element, and associated message element attributes.

### Free text messages may contain the IA5 character set, consisting only the following characters:  (0...9)  (A..Z)  (,)  (.)  (/)  (-)  (+)  (()  ())  and the space character.

### Message Identification Numbers

#### Message identification numbers provided by a CPDLC ground system for messages to/from an aircraft have no relationship to the message identification numbers provided by the same ground system for another aircraft.

#### Similarly, message identification numbers provided by an aircraft on a given CPDLC link for messages to/from a ground system have no relationship to the message identification numbers provided by the same aircraft with another ground system.

#### The message identification number provided by the ground user will be different from any other message identification number currently in use with that particular aircraft.

#### The aircraft identification number provided by the avionics will be different from any other message identification number currently in use with that particular ground system.

#### A message identification number will be deemed currently in use until:

- if the message does not require a response, the message is sent, or

- if the message requires a response, the closure response is received.

#### When a CDA or DDA link is established, all message identification numbers will be considered available.

#### Message identification numbers should be provided sequentially.

### Message Reference Numbers

#### All response messages will contain a message reference number.

#### The message reference number will be identical to the message identification number of the received message to which it refers.

### Message Attributes

#### Message attributes dictate certain message handling requirements for the CPDLC-user receiving a message.  The CPDLC messages have Urgency, Alert, and Response attributes.

#### Urgency

##### The Urgency (URG) attribute delineates the queuing requirements for received messages that are displayed to the end-user.  Urgency types are presented in Table 3-1.

| Type | Description | Precedence |
| --- | --- | --- |
| D | Distress | 1 |
| U | Urgent | 2 |
| N | Normal | 3 |
| L | Low | 4 |

Table 3-1:  Urgency Attribute (Uplink and Downlink)

#### Alert

##### The alert (ALRT) attribute delineates the type of alerting required upon message receipt.  Alert types are presented in Table 3-2.

| Type | Description | Precedence |
| --- | --- | --- |
| H | High | 1 |
| M | Medium | 2 |
| L | Low | 3 |
| N | No alerting required | 4 |

Table 3-2:  Alert Attribute (Uplink and Downlink)

#### Response

##### The response (RESP) attribute mandates response requirements for a given message element.  Response types are presented in Table 3-3 for uplink messages and Table 3-4 for downlink messages.

| Type | Response Required | Valid Responses | Precedence |
| --- | --- | --- | --- |
| W/U | Yes | WILCO, UNABLE, STANDBY permitted, LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 1 |
| A/N | Yes | AFFIRM, NEGATIVE, STANDBY permitted, LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 2 |
| R | Yes | ROGER, UNABLE, STANDBY permitted LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 3 |
| Y | Yes | Any CPDLC downlink message, LOGICAL ACKNOWLEDGMENT (only if required) | 4 |
| N | No, unless logical acknowledgment required | LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary, only when logical acknowledgment is required) | 5 |

Table 3-3:  Response Attribute (Up-Link)

| Type | Response Required | Valid Responses | Precedence |
| --- | --- | --- | --- |
| Y | Yes | Any CPDLC uplink message, LOGICAL ACKNOWLEDGMENT (only if required), | 1 |
| N | No, unless logical acknowledgment required | LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary, only when logical acknowledgment is required) | 2 |

Table 3-4:  Response Attribute (Down-Link)

#### Attribute Association

##### For each message element, Urgency, Alert, and Response attribute types are associated with it as specified in Appendix A to this chapter.

##### When a message contains a single message element, the message attributes are the message element attributes.

##### When a message contains multiple message elements, the highest precedence message element attribute type associated with any element in the message becomes the message attribute type for the entire message.  Message element attribute table entries are listed in order of precedence (i.e., a precedence value of 1 is highest followed by 2, etc.)  For example, this means that a message containing multiple message elements, where at least one element has a W/U attribute, the whole message then has a W/U attribute.

### Response Messages

#### A message containing the ERROR message element will always be permitted as a response message.

#### Any message that is considered a response message (i.e., it contains a message reference number) will have message urgency and alert attributes not less than the message to which it refers.

#### If the CPDLC-user sends a message containing the ERROR message element instead of the expected response message, the ERROR message will contain the received message identification number as the message reference number.  This ERROR message will be a closure response message.

### Logical Acknowledgment Messages

#### The logical acknowledgment provides confirmation from a receiving system to the message originator, that the message has been successfully received and is acceptable for display to the responsible person, if this is required.  The logical acknowledgment in no way replaces any required operational response.

#### A ground system will determine if the use of the logical acknowledgment (either air or ground) is permitted/required within its airspace.

#### The logical acknowledgment message element must not be combined with any other message element in a message.

#### A logical acknowledgment response message, if required, will be sent prior to sending any other related response message(s), other than an ERROR message, if necessary.

### Message Differentiation

#### A CPDLC message intended for transmission on a DDA link must be clearly distinguishable from a CPDLC message intended for transmission on a CDA link.

## CPDLC Message Receipt Requirements

### CDA/DDA Link Messages

#### When a CPDLC-user queues received messages, from a CDA and DDA in the same queue the messages from a CDA will be placed ahead of message from a DDA regardless of message urgency.

#### A CPDLC message received on a DDA link must be clearly distinguishable from a CPDLC message received on a CDA link.

### Logical Acknowledgment Prohibited

#### Upon receipt of the CPDLC message USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED the aircraft will be prohibited from requiring a logical acknowledgment for any message exchanged with that ground system for the duration of a CDA or DDA link.

### Urgency Requirements

#### When a CPDLC-user queues received messages, messages with the highest Urgency type will be placed at the beginning of the queue.

#### When a CPDLC-user queues received messages, messages with the same Urgency type, will be queued in order of receipt.

### Alerting Requirements

#### CPDLC will provide three distinct alerts determined by the received message alert attribute.

### Message Response Requirements

#### A CPDLC-user will only respond to a received message in its entirety.  This means for example, that if 3 message elements are concatenated in a single message, any response given applies to the whole message, and not to any individual message element.

#### If a message is received that requires a response the CPDLC-user will:

- be allowed to send any permitted response messages, and

- send one and only one closure response message.

#### For a given message, once the CPDLC-user has sent the closure response message, no other response messages will be sent.

#### Table 3-5 contains the closure response messages permitted for each message response category.

| Permitted Response | Closure Response |
| --- | --- |
| W/U | A response message containing at least a WILCO, UNABLE, or ERROR message element |
| A/N | A response message containing at least an AFFIRM, NEGATIVE, or ERROR message element |
| R | A response message containing at least a ROGER, UNABLE or ERROR message element |
| Y | The first response message sent from the aircraft that does not contain a STANDBY or, (only when required), a LOGICAL ACKNOWLEDGMENT The first response message sent by the ground that does not contain a STANDBY, (only when required), a LOGICAL ACKNOWLEDGMENT or REQUEST DEFERRED message element from the ground system. |
| N | The only permitted response will be a closure response message that contains a LOGICAL ACKNOWLEDGMENT when this has been required or an ERROR message element when a logical acknowledgment is required.. |

Table 3-5:  Permitted Closure Responses by Response Category

##### The CPDLC application must provide the message initiator the capability to provide closure for a CPDLC message, independent of CPDLC closure message receipt.

## Establishment of the CDA or NDA Link

### Either the airborne system or the ground system can request CPDLC.  Acceptance by either the airborne system or the ground system of a request for CPDLC establishes a CDA or NDA link.

### Upon acceptance of a CDA or NDA link, the CPDLC application will have the capability to inform both the controller and pilot of this link establishment.

### An aircraft will be permitted to request CPDLC with any ground system, if the aircraft has no existing CDA or NDA link.  If the ground system accepts the CPDLC request, that ground system will become the CDA.

### Only if an aircraft has received a message from the Current Data Authority designating a Next Data Authority, will the aircraft be permitted to request CPDLC with the specified ground system.

#### In general, ground acceptance of an airborne request for CPDLC is determined by local procedures.

#### However, if a ground system receives a request for CPDLC from an aircraft, for which it currently has a CDA or NDA link, it will:

- accept the request, and

- cancel the first NDA or CDA link.

Note:  The aircraft could realize that a CDA or NDA link has been lost, and request CPDLC before the ground is aware of the loss of the CDA or NDA link.  By allowing the ground to accept a “second” CDA or NDA link from the aircraft, the potential for loss of communication is minimized.

### If the ground requests CPDLC with an aircraft and the aircraft does not have a CDA or NDA link, then the aircraft will accept the CPDLC request and consider the ground system as the Current Data Authority.

### If the ground requests CPDLC with an aircraft and the aircraft already has a CDA link, the aircraft will accept the CPDLC request if

- the request is from the ground system that is the CDA, or

- the request is from the Next Data Authority.

### If the aircraft accepts a “second” CDA or NDA link, the “first” CDA or NDA link with that ground system will be terminated.

Note:  The ground could realize that a CDA or NDA link has been lost, and request CPDLC before the aircraft is aware of the loss of the CDA or NDA link.  By allowing the aircraft to accept a “second” CD or NDA link from the ground, the potential for loss of communication is minimized..

#### The aircraft will reject a request for CPDLC from any other ground system, and will indicate to the requesting ground system what ground system is the Current Data Authority.

### The aircraft will disregard CPDLC messages over the NDA link and indicate to the originator that it is not the Current Data Authority.

### Only the Current Data Authority can designate a ground system as the Next Data Authority.

### The Current Data Authority can designate only one ground system as the Next Data Authority at a time (i.e., only one per CPDLC message).

### An airborne system will only consider a ground system as the Next Data Authority if it has received such an indication from the Current Data Authority.

### Any indication from the Current Data Authority designating a Next Data Authority will replace any previously received Next Data Authority designation for another ground system.

### If the ground system rejects a request for CPDLC it will provide a reason for the rejection using a CPDLC message.

## Establishment of the DDA Link

### Only the airborne system can request DSC.  Acceptance by the ground system of a request for DSC establishes a DDA link.

### Upon acceptance of a DDA link the CPDLC application will have the capability to inform both the controller and pilot of this link establishment.

### If an aircraft has no DDA link, that aircraft will be permitted to request DSC with any ground system that is not its CDA.  The ground system may only accept a request for DSC if it has a filed flight plan for the requesting aircraft.  If the ground system accepts the DSC request, that ground system will become the Downstream Data Authority.

### Generally, ground acceptance of an airborne request for DSC even when the ground has a filed flight plan for that aircraft is determined by local procedures.

#### However, if a ground system receives a request for DSC from an aircraft, for which it currently has a DDA link, it will:

- accept the request, and

- cancel the first DDA link.

Note:  The aircraft could realize that a DDA link has been lost, and request DSC before the ground is aware of the loss of the DDA link.  By allowing the ground to accept a “second” DDA link from the aircraft, the potential for loss of communication is minimized.

### If the ground system rejects a request for DSC it will provide a reason for the rejection using a CPDLC message.

## Link Termination and Transfer

### Once normal link termination is initiated, only CPDLC closure response messages may be exchanged over the CDA or DDA link being terminated.

### Once termination is initiated, the system will have the capability of informing the pilot or controller of this action.

### Once normal link termination is initiated, the pilot and controller will be informed of any message for which closure is outstanding.

### If a CDA or NDA link is terminated for any reason, any DDA link will not be affected.

### Normally, CPDLC service termination with the Current Data Authority is initiated by the ground system to end service or transfer service to the next ATS facility.

### The ground system will not perform a normal termination of the CDA or DDA link while there are any CPDLC messages for which closure is outstanding.

### Any NDA link will be terminated by the aircraft if it receives a subsequent designation of Next Data Authority.  When terminating a NDA link in this situation the aircraft will indicate to the ground system being terminated that it is no longer the Next Data Authority.

### When the CDA link is terminated normally, the ground system currently designated as the Next Data Authority will become the Current Data Authority.

### If the CDA link is terminated for any reason other than under instruction from the Current Data Authority, any designation of a ground system as a Next Data Authority will be deleted, and any NDA link in place will be terminated.

### Only an aircraft can normally terminate a DDA link.

### DDA normal link termination will be automatically initiated if a DDA becomes a CDA, and the pilot will be informed of this action.

## Message Presentation

### The presentation of messages is a local implementation.

### The CPDLC message element description is presented in Appendix A to this chapter.  Appendix B to this chapter contains a data glossary.  Appendix C to this chapter provides data range and resolution.

## Message Errors

### When an error is detected in a received message, a response message indicating an error and providing the reason for the error will be sent if the message permits a response message.  Error reasons are provided as part of the Data Glossary in Appendix B to this chapter.

## Service Descriptions

### Service descriptions for some functions using the CPDLC message set are included in this part of the manual.  Chapter 6 contains a description of the Departure Clearance.  Chapter 7 contains a description of the Transfer of Data Authority.  Chapter 8 contains a description of the Downstream Clearance.

PART IV- CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

CHAPTER 3 APPENDIX A

CPDLC MESSAGE SET

3A.1	Introduction

3A.1.1	In the interest of maintaining continuity message elements have not been deleted as proposed changes have been accepted, instead message elements have been added as refined requirements have been identified.

3A.1.2	This Appendix defines the allowed message elements from which CPDLC messages can be composed.

3A.1.3	Where possible, the text for the message element has been sourced from section three (Phraseologies) of the document PANS/RAC 4444.

3A.1.4	The column headed Message Element is a suggested text for presentation to the pilot or controller.

3A.2	Up-Link Messages

A.2.1	Uplink messages for CPDLC are presented in this section.

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 0 | Indicates that ATS cannot comply with the request. | UNABLE | N | M | N |
| 1 | Indicates that ATS has received the request and will respond shortly. | STANDBY | N | L | N |
| 2 | Indicates that ATS has received the request but it has been deferred until later. | REQUEST DEFERRED | N | L | N |
| 3 | Indicates that ATS has received and understood the request. | ROGER | N | L | N |
| 4 | Yes. | AFFIRM | N | L | N |
| 5 | No. | NEGATIVE | N | L | N |
| 235 | Notification of receipt of unlawful interference message. | ROGER 7500 | U | H | N |
| 211 | Indicates that the ATS has received the request and has passed it to the Next Control Authority. | REQUEST FORWARDED | N | L | N |
| 218 | Indicates to the pilot that the request has already been received on the ground. | REQUEST ALREADY RECEIVED | L | N | N |

Table A-1:  Responses/Acknowledgments (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 6 | Notification that an altitude change instruction should be expected. | EXPECT [altitude] | L | L | R |
| 7 | Notification that an instruction should be expected for the aircraft to commence climb at the specified time. | EXPECT CLIMB AT [time] | L | L | R |
| 8 | Notification that an instruction should be expected for the aircraft to commence climb at the specified position. | EXPECT CLIMB AT [position] | L | L | R |
| 9 | Notification that an instruction should be expected for the aircraft to commence descent at the specified time. | EXPECT DESCENT AT [time] | L | L | R |
| 10 | Notification that an instruction should be expected for the aircraft to commence descent at the specified position. | EXPECT DESCENT AT [position] | L | L | R |
| 11 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time. | EXPECT CRUISE CLIMB AT [time] | L | L | R |
| 12 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position. | EXPECT CRUISE CLIMB AT [position] | L | L | R |
| 13 | Notification that an instruction should be expected for the aircraft to commence climb at the specified time to the specified altitude. | AT [time] EXPECT CLIMB TO [altitude] | L | L | R |
| 14 | Notification that an instruction should be expected for the aircraft to commence climb at the specified position to the specified altitude. | AT [position] EXPECT CLIMB TO [altitude] | L | L | R |
| 15 | Notification that an instruction should be expected for the aircraft to commence descent at the specified time to the specified altitude. | AT [time] EXPECT DESCENT TO [altitude] | L | L | R |
| 16 | Notification that an instruction should be expected for the aircraft to commence descent at the specified position to the specified altitude. | AT [position] EXPECT DESCENT TO [altitude] | L | L | R |
| 17 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time to the specified altitude. | AT [time] EXPECT CRUISE CLIMB TO [altitude] | L | L | R |
| 18 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position to the specified altitude. | AT [position] EXPECT CRUISE CLIMB TO [altitude] | L | L | R |
| 19 | Instruction to maintain the specified altitude. | MAINTAIN [altitude] | N | M | W/U |
| 20 | Combined instruction that a climb to a specified altitude is to commence and the altitude is to be maintained when reached. | CLIMB TO AND MAINTAIN [altitude] | N | M | W/U |
| 21 | Combined instruction that at the specified time, a climb to the specified altitude is to commence and once reached the specified altitude is to be maintained. | AT [time] CLIMB TO AND MAINTAIN [altitude] | N | M | W/U |
| 22 | Combined instruction that at the specified position, a climb to the specified altitude is to commence and once reached the specified altitude is to be maintained. | AT [position] CLIMB TO AND MAINTAIN [altitude] | N | M | W/U |
| 185 | Combined instruction that after passing the specified position, a climb to the specified altitude is to commence and once reached the specified altitude is to be maintained. | AFTER PASSING [position] CLIMB TO AND MAINTAIN [altitude] | N | M | W/U |
| 23 | Combined instruction that a descent to a specified altitude is to commence and the altitude is to be maintained when reached. | DESCEND TO AND MAINTAIN [altitude] | N | M | W/U |
| 24 | Combined instruction that at a specified time a descent to a specified altitude is to commence and once reached the specified altitude is to be maintained. | AT [time] DESCEND TO AND MAINTAIN [altitude] | N | M | W/U |
| 25 | Combined instruction that at the specified position a descent to the specified altitude is to commence and when the specified altitude is reached it is to be maintained. | AT [position] DESCEND TO AND MAINTAIN [altitude] | N | M | W/U |
| 186 | Combined instruction that after passing the specified position, a descent to the specified altitude is to commence and once reached the specified altitude is to be maintained. | AFTER PASSING [position] DESCEND TO AND MAINTAIN [altitude] | N | M | W/U |
| 26 | Combined instruction that a climb is to commence at a rate such that the specified altitude is reached at or before the specified time. | CLIMB TO REACH [altitude] BY [time] | N | M | W/U |
| 27 | Combined instruction that a climb is to commence at a rate such that the specified altitude is reached at or before the specified position. | CLIMB TO REACH [altitude] BY [position] | N | M | W/U |
| 28 | Combined instruction that a descent is to commence at a rate such that the specified altitude is reached at or before the specified time | DESCEND TO REACH [altitude] BY [time] | N | M | W/U |
| 29 | Combined instruction that a descent is to commence at a rate such that the specified altitude is reached at or before the specified position. | DESCEND TO REACH [altitude] BY [position] | N | M | W/U |
| 192 | Combined instruction that a change of altitude is to continue, but at a rate such that the specified altitude is reached at or before the specified time. | REACH [altitude] BY [time] | N | M | W/U |
| 209 | Combined instruction that a change of altitude is to continue, but at a rate such that the specified altitude is reached at or before the specified position. | REACH [altitude] BY [position] | N | M | W/U |
| 30 | An altitude within the defined block altitude specified is to be maintained. | MAINTAIN BLOCK [altitude] TO [altitude] | N | M | W/U |
| 31 | Combined instruction that a climb to an altitude within the block altitude defined is to commence. | CLIMB TO AND MAINTAIN BLOCK [altitude] TO [altitude] | N | M | W/U |
| 32 | Combined instruction that a descent to an altitude within the block altitude defined is to commence. | DESCEND TO AND MAINTAIN BLOCK [altitude] TO [altitude] | N | M | W/U |
| 34 | A cruise climb is to commence and continue until the specified altitude is reached. | CRUISE CLIMB TO [altitude] | N | M | W/U |
| 35 | A cruise climb can commence once above the specified altitude. | CRUISE CLIMB ABOVE [altitude] | N | M | W/U |
| 219 | Instruction to stop the climb below the previously assigned altitude. | STOP CLIMB AT [altitude] | U | M | W/U |
| 220 | Instruction to stop the descent above the previously assigned altitude. | STOP DESCENT AT [altitude] | U | M | W/U |
| 36 | The climb to the specified altitude should be made at the aircraft’s best rate. | EXPEDITE CLIMB TO [altitude] | U | M | W/U |
| 37 | The descent to the specified altitude should be made at the aircraft’s best rate. | EXPEDITE DESCENT TO [altitude] | U | M | W/U |
| 38 | Urgent instruction to immediately climb to the specified altitude. | IMMEDIATELY CLIMB TO [altitude] | D | H | W/U |
| 39 | Urgent instruction to immediately descend to the specified altitude. | IMMEDIATELY DESCEND TO [altitude] | D | H | W/U |
| 40 | Urgent instruction to immediately stop a climb once the specified altitude is reached. | IMMEDIATELY STOP CLIMB AT [altitude] | D | H | W/U |
| 41 | Urgent instruction to immediately stop a descent once the specified altitude is reached. | IMMEDIATELY STOP DESCENT AT [altitude] | D | H | W/U |
| 171 | Instruction to climb at not less than the specified rate. | CLIMB AT [vertical rate] MINIMUM | N | M | W/U |
| 172 | Instruction to climb at not above the specified rate. | CLIMB AT [vertical rate] MAXIMUM | N | M | W/U |
| 173 | Instruction to descend at not less than the specified rate. | DESCEND AT [vertical rate] MINIMUM | N | M | W/U |
| 174 | Instruction to descend at not above the specified rate. | DESCEND AT [vertical rate] MAXIMUM | N | M | W/U |
| 236 | Authorization for the pilot to conduct flight at any altitude from the minimum IFR altitude up to and including the altitude specified. | CRUISE [altitude] | N | M | W/U |

Table A-2:  Vertical Clearances (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 42 | Notification that an altitude change instruction should be expected which will require the specified position to be crossed at the specified altitude. | EXPECT TO CROSS [position] AT [altitude] | L | L | R |
| 43 | Notification that an altitude change instruction should be expected which will require the specified position to be crossed at or above the specified altitude. | EXPECT TO CROSS [position] AT OR ABOVE [altitude] | L | L | R |
| 44 | Notification that an altitude change instruction should be expected which will require the specified position to be crossed at or below the specified altitude. | EXPECT TO CROSS [position] AT OR BELOW [altitude] | L | L | R |
| 45 | Notification that an altitude change instruction should be expected which will require the specified position to be crossed at the specified altitude which is to be maintained subsequently. | EXPECT TO CROSS [position] AT AND MAINTAIN [altitude] | L | L | R |
| 46 | The specified position should be crossed at the specified altitude. This may require the aircraft to modify its climb or descent profile. | CROSS [position] AT [altitude] | N | M | W/U |
| 47 | The specified position is to be crossed at or above the specified altitude. | CROSS [position] AT OR ABOVE [altitude] | N | M | W/U |
| 48 | The specified position is to be crossed at or below the specified altitude. | CROSS [position] AT OR BELOW [altitude] | N | M | W/U |
| 49 | Combined instruction that the specified position is to be crossed at the specified altitude and that altitude is to be maintained when reached. | CROSS [position] AT AND MAINTAIN [altitude] | N | M | W/U |
| 50 | The specified position is to be crossed at an altitude between the specified altitudes. | CROSS [position] BETWEEN [altitude] AND [altitude] | N | M | W/U |
| 51 | The specified position is to be crossed at the specified time. | CROSS [position] AT [time] | N | M | W/U |
| 52 | The specified position is to be crossed at or before the specified time. | CROSS [position] AT OR BEFORE [time] | N | M | W/U |
| 53 | The specified position is to be crossed at or after the specified time. | CROSS [position] AT OR AFTER [time] | N | M | W/U |
| 54 | The specified position is to be crossed at a time between the specified times. | CROSS [position] BETWEEN [time] AND [time] | N | M | W/U |
| 55 | The specified position is to be crossed at the specified speed and the specified speed is to be maintained until further advised. | CROSS [position] AT [speed] | N | M | W/U |
| 56 | The specified position is to be crossed at a speed equal to or less than the specified speed and the specified speed or less is to be maintained until further advised. | CROSS [position] AT OR LESS THAN [speed] | N | M | W/U |
| 57 | The specified position is to be crossed at a speed equal to or greater than the specified speed and the specified speed or greater is to be maintained until further advised. | CROSS [position] AT OR GREATER THAN [speed] | N | M | W/U |
| 58 | The specified position is to be crossed at the specified time and the specified altitude. | CROSS [position] AT [time] AT [altitude] | N | M | W/U |
| 59 | The specified position is to be crossed at or before the specified time and at the specified altitude. | CROSS [position] AT OR BEFORE [time] AT [altitude] | N | M | W/U |
| 60 | The specified position is to be crossed at or after the specified time and at the specified altitude. | CROSS [position] AT OR AFTER [time] AT [altitude] | N | M | W/U |
| 61 | Combined instruction that the specified position is to be crossed at the specified altitude and speed and the altitude and speed are to be maintained. | CROSS [position] AT AND MAINTAIN [altitude] AT [speed] | N | M | W/U |
| 62 | Combined instruction that at the specified time the specified position is to be crossed at the specified altitude and the altitude is to be maintained. | AT [time] CROSS [position] AT AND MAINTAIN [altitude] | N | M | W/U |
| 63 | Combined instruction that at the specified time the specified position is to be crossed at the specified altitude and speed and the altitude and speed are to be maintained. | AT [time] CROSS [position] AT AND MAINTAIN [altitude] AT [speed] | N | M | W/U |

Table A-3:  Crossing Constraints (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 64 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction. | OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 65 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified position. | AT [position] OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 66 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified time. | AT [time] OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 67 | The cleared flight route is to be rejoined. | PROCEED BACK ON ROUTE | N | M | W/U |
| 68 | The cleared flight route is to be rejoined at or before the specified position. | REJOIN ROUTE BY [position] | N | M | W/U |
| 69 | The cleared flight route is to be rejoined at or before the specified time. | REJOIN ROUTE BY [time] | N | M | W/U |
| 70 | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified position. | EXPECT BACK ON ROUTE BY [position] | L | L | R |
| 71 | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified time. | EXPECT BACK ON ROUTE BY [time] | L | L | R |
| 72 | Instruction to resume own navigation following a period of tracking or heading clearances. May be used in conjunction with an instruction on how or where to rejoin the cleared route. | RESUME OWN NAVIGATION | N | M | W/U |

Table A-4:  Lateral Offsets (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 73 | Notification to the aircraft of the instructions to be followed from departure until the specified clearance limit. | [departure clearance] | N | M | W/U |
| 74 | Instruction to proceed directly from its present position to the specified position. | PROCEED DIRECT TO [position] | N | M | W/U |
| 75 | Instruction to proceed, when able, directly to the specified position. | WHEN ABLE PROCEED DIRECT TO [position] | N | M | W/U |
| 76 | Instruction to proceed, at the specified time, directly to the specified position. | AT [time] PROCEED DIRECT TO [position] | N | M | W/U |
| 77 | Instruction to proceed, at the specified position, directly to the next specified position. | AT [position] PROCEED DIRECT TO [position] | N | M | W/U |
| 78 | Instruction to proceed, upon reaching the specified altitude, directly to the specified position. | AT [altitude] PROCEED DIRECT TO [position] | N | M | W/U |
| 79 | Instruction to proceed to the specified position via the specified route. | CLEARED TO [position] VIA [route clearance] | N | M | W/U |
| 80 | Instruction to proceed via the specified route. | CLEARED [route clearance] | N | M | W/U |
| 81 | Instruction to proceed in accordance with the specified procedure. | CLEARED [procedure name] | N | M | W/U |
| 33 | Instruction to proceed to the intended destination which has no instrument approach procedure authorized or available, and is in uncontrolled airspace. | CLEARED OUT OF CONTROLLED AIRSPACE | N | M | W/U |
| 82 | Approval to deviate up to the specified distance from the cleared route in the specified direction. | CLEARED TO DEVIATE UP TO [distance offset] [direction] OF ROUTE | N | M | W/U |
| 83 | Instruction to proceed from the specified position via the specified route. | AT [position] CLEARED [route clearance] | N | M | W/U |
| 84 | Instruction to proceed from the specified position via the specified procedure. | AT [position] CLEARED [procedure name] | N | M | W/U |
| 85 | Notification that a clearance to fly on the specified route may be issued. | EXPECT [route clearance] | L | L | R |

| 86 | Notification that a clearance to fly on the specified route from the specified position may be issued. | AT [position] EXPECT [route clearance] | L | L | R |
| --- | --- | --- | --- | --- | --- |
| 87 | Notification that a clearance to fly directly to the specified position may be issued. | EXPECT DIRECT TO [position] | L | L | R |
| 88 | Notification that a clearance to fly directly from the first specified position to the next specified position may be issued. | AT [position] EXPECT DIRECT TO [position] | L | L | R |
| 89 | Notification that a clearance to fly directly to the specified position commencing at the specified time may be issued. | AT [time] EXPECT DIRECT TO [position] | L | L | R |
| 90 | Notification that a clearance to fly directly to the specified position commencing when the specified altitude is reached may be issued. | AT [altitude] EXPECT DIRECT TO [position] | L | L | R |
| 91 | Instruction to enter a holding pattern with the specified characteristics at the specified position and altitude. | HOLD AT [position] MAINTAIN [altitude] INBOUND TRACK [degrees] [direction] TURNS [leg type] | N | M | W/U |
| 92 | Instruction to enter a holding pattern with the published characteristics at the specified position and altitude. | HOLD AT [position] AS PUBLISHED MAINTAIN [altitude] | N | M | W/U |
| 93 | Notification that an onwards clearance may be issued at the specified time. | EXPECT FURTHER CLEARANCE AT [time] | L | L | R |
| 94 | Instruction to turn left or right as specified onto the specified heading. | TURN [direction] HEADING [degrees] | N | M | W/U |
| 95 | Instruction to turn left or right as specified onto the specified track. | TURN [direction] GROUND TRACK [degrees] | N | M | W/U |
| 215 | Instruction to turn a specified number of degrees left or right. | TURN [degrees][direction] | N | M | W/U |
| 190 | Instruction to fly on the specified heading. | FLY HEADING [degrees] | N | M | W/U |
| 96 | Instruction to continue to fly on the current heading. | CONTINUE PRESENT HEADING | N | M | W/U |
| 97 | Instruction to fly on the specified heading from the specified position. | AT [position] FLY HEADING [degrees] | N | M | W/U |
| 221 | Instruction to stop turn at the specified heading prior to reaching the previously assigned heading. | STOP TURN HEADING [degrees] | U | M | W/U |

| 98 | Instruction to turn immediately left or right as specified onto the specified heading. | IMMEDIATELY TURN [direction] HEADING [degrees] | D | H | W/U |
| --- | --- | --- | --- | --- | --- |
| 99 | Notification that a clearance may be issued for the aircraft to fly the specified procedure. | EXPECT [procedure name] | L | L | R |

Table A-5:  Route Modifications (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 100 | Notification that a speed instruction may be issued to be effective at the specified time. | AT [time] EXPECT [speed] | L | L | R |
| 101 | Notification that a speed instruction may be issued to be effective at the specified position. | AT [position] EXPECT [speed] | L | L | R |
| 102 | Notification that a speed instruction may be issued to be effective at the specified altitude. | AT [altitude] EXPECT [speed] | L | L | R |
| 103 | Notification that a speed range instruction may be issued to be effective at the specified time. | AT [time] EXPECT [speed] TO [speed] | L | L | R |
| 104 | Notification that a speed range instruction may be issued to be effective at the specified position. | AT [position] EXPECT [speed] TO [speed] | L | L | R |
| 105 | Notification that a speed range instruction may be issued to be effective at the specified altitude. | AT [altitude] EXPECT [speed] TO [speed] | L | L | R |
| 106 | The specified speed is to be maintained. | MAINTAIN [speed] | N | M | W/U |
| 188 | After passing the specified position the specified speed is to be maintained. | AFTER PASSING [position] MAINTAIN [speed] | N | M | W/U |
| 107 | The present speed is to be maintained. | MAINTAIN PRESENT SPEED | N | M | W/U |
| 108 | The specified speed or a greater speed is to be maintained. | MAINTAIN [speed] OR GREATER | N | M | W/U |
| 109 | The specified speed or a lesser speed is to be maintained. | MAINTAIN [speed] OR LESS | N | M | W/U |
| 110 | A speed with the specified range is to be maintained. | MAINTAIN [speed] TO [speed] | N | M | W/U |
| 111 | The present speed is to be increased to the specified speed and maintained until further advised. | INCREASE SPEED TO [speed] | N | M | W/U |
| 112 | The present speed is to be increased to the specified speed or greater, and maintained at or above the specified speed until further advised. | INCREASE SPEED TO [speed] OR GREATER | N | M | W/U |
| 113 | The present speed is to be reduced to the specified speed and maintained until further advised. | REDUCE SPEED TO [speed] | N | M | W/U |
| 114 | The present speed is to be reduced to the specified speed or less and maintained at or below the specified speed until further advised. | REDUCE SPEED TO [speed] OR LESS | N | M | W/U |
| 115 | The specified speed is not to be exceeded. | DO NOT EXCEED [speed] | N | M | W/U |
| 116 | Notification that the aircraft need no longer comply with the previously issued speed restriction. | RESUME NORMAL SPEED | N | M | W/U |
| 189 | The present speed is to be changed to the specified speed. | ADJUST SPEED TO [speed] | N | M | W/U |
| 222 | Notification that the aircraft may keep its preferred speed without restriction. | NO SPEED RESTRICTION | L | L | R |
| 223 | Instruction to reduce present speed to the minimum safe approach speed | REDUCE TO MINIMUM APPROACH SPEED | N | M | W/U |

Table A-6:  Speed Changes (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 117 | The ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | CONTACT [icaounitname] [frequency] | N | M | W/U |
| 118 | At the specified position the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | AT [position] CONTACT [icaounitname] [frequency] | N | M | W/U |
| 119 | At the specified time the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | AT [time] CONTACT [icaounitname] [frequency] | N | M | W/U |
| 120 | The ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | MONITOR [icaounitname] [frequency] | N | M | W/U |
| 121 | At the specified position the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | AT [position] MONITOR [icaounitname] [frequency] | N | M | W/U |
| 122 | At the specified time the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | AT [time] MONITOR [icaounitname] [frequency] | N | M | W/U |
| 123 | The specified beacon code (SSR code) is to be selected. | SQUAWK [beacon code] | N | M | W/U |
| 124 | The SSR transponder responses are to be disabled. | STOP SQUAWK | N | M | W/U |
| 125 | The SSR transponder responses should include altitude information. | SQUAWK MODE CHARLIE | N | M | W/U |
| 126 | The SSR transponder responses should no longer include altitude information. | STOP SQUAWK MODE CHARLIE | N | M | W/U |
| 179 | The ‘ident’ function on the SSR transponder is to be actuated. | SQUAWK IDENT | N | M | W/U |

Table A-7:  Contact/Monitor/Surveillance Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 127 | Instruction to report when the aircraft is back on the cleared route. | REPORT BACK ON ROUTE | N | L | R |
| 128 | Instruction to report when the aircraft has left the specified altitude. | REPORT LEAVING [altitude] | N | L | R |
| 129 | Instruction to report when the aircraft is in altitude flight at the specified altitude. | REPORT WHEN LEVEL AT [altitude] | N | L | R |
| 175 | Instruction to report when the aircraft has reached the specified altitude. | REPORT REACHING [altitude] | N | L | R |
| 180 | Instruction to report when the aircraft is within the specified altitude range. | REPORT REACHING BLOCK [altitude] TO [altitude] | N | L | R |
| 130 | Instruction to report when the aircraft has passed the specified position. | REPORT PASSING [position] | N | L | R |
| 181 | Instruction to report the present distance to or from the specified position. | REPORT DISTANCE [to/from] [position] | N | M | Y |
| 184 | Instruction to report at the specified time the distance to or from the specified position. | AT TIME [time] REPORT DISTANCE [to/from] [position] | N | L | Y |
| 228 | Instruction to report the estimated time of arrival at the specified position. | REPORT ETA [position] | L | L | Y |
| 131 | Instruction to report the amount of fuel remaining and the number of persons on board. | REPORT REMAINING FUEL AND PERSONS ON BOARD | U | M | Y |
| 132 | Instruction to report the present position. | REPORT POSITION | N | M | Y |
| 133 | Instruction to report the present altitude. | REPORT PRESENT ALTITUDE | N | M | Y |
| 134 | Instruction to report the requested speed. | REPORT [speed qualifier] [speed type] SPEED | N | M | Y |
| 135 | Instruction to confirm and acknowledge the currently assigned altitude. | CONFIRM ASSIGNED ALTITUDE | N | L | Y |
| 136 | Instruction to confirm and acknowledge the currently assigned speed. | CONFIRM ASSIGNED SPEED | N | L | Y |
| 137 | Instruction to confirm and acknowledge the currently assigned route. | CONFIRM ASSIGNED ROUTE | N | L | Y |
| 138 | Instruction to confirm the previously reported time over the last reported waypoint. | CONFIRM TIME OVER REPORTED WAYPOINT | N | L | Y |
| 139 | Instruction to confirm the identity of the previously reported waypoint. | CONFIRM REPORTED WAYPOINT | N | L | Y |
| 140 | Instruction to confirm the identity of the next waypoint. | CONFIRM NEXT WAYPOINT | N | L | Y |
| 141 | Instruction to confirm the previously reported estimated time at the next waypoint. | CONFIRM NEXT WAYPOINT ETA | N | L | Y |
| 142 | Instruction to confirm the identity of the next but one waypoint. | CONFIRM ENSUING WAYPOINT | N | L | Y |
| 143 | The request was not understood. It should be clarified and resubmitted. | CONFIRM REQUEST | N | L | Y |
| 144 | Instruction to report the selected beacon code. | CONFIRM SQUAWK | N | L | Y |
| 145 | Instruction to report the present heading. | REPORT HEADING | N | M | Y |
| 146 | Instruction to report the present ground track. | REPORT GROUND TRACK | N | M | Y |
| 182 | Instruction to report the identification code of the last ATIS received. | CONFIRM ATIS CODE | N | L | Y |
| 147 | Instruction to make a position report. | REQUEST POSITION REPORT | N | M | Y |
| 216 | Instruction to file a flight plan. | REQUEST FLIGHT PLAN | N | M | Y |
| 217 | Instruction to report that the aircraft has landed. | REPORT ARRIVAL | N | M | Y |
| 229 | Instruction to report the preferred alternate aerodrome for landing. | REPORT ALTERNATE AERODROME | L | L | Y |
| 231 | Instruction to indicate the pilot’s preferred altitude | STATE PREFERRED ALTITUDE | L | L | Y |
| 232 | Instruction to indicate the pilot’s preferred time and/ or position to commence descent to the aerodrome of intended arrival. | STATE TOP OF DESCENT | L | L | Y |

Table A-8:  Report/Confirmation Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 148 | Request for the earliest time at which the specified altitude can be accepted. | WHEN CAN YOU ACCEPT [altitude] | N | L | Y |
| 149 | Instruction to report whether or not the specified altitude can be accepted at the specified position. | CAN YOU ACCEPT [altitude] AT [position] | N | L | A/N |
| 150 | Instruction to report whether or not the specified altitude can be accepted at the specified time. | CAN YOU ACCEPT [altitude] AT [time] | N | L | A/N |
| 151 | Instruction to report the earliest time when the specified altitude can be accepted. | WHEN CAN YOU ACCEPT [speed] | N | L | Y |
| 152 | Instruction to report the earliest time when the specified offset track can be accepted. | WHEN CAN YOU ACCEPT [distance offset] [direction] OFFSET | N | L | Y |

Table A-9:  Negotiation Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 153 | ATS advisory that the altimeter setting should be the specified setting. | ALTIMETER [altimeter] | N | L | R |
| 213 | ATS advisory that the specified altimeter setting relates to the specified facility. | [icao facility designation] ALTIMETER [altimeter] | N | L | R |
| 154 | ATS advisory that the radar service is terminated. | RADAR SERVICE TERMINATED | N | L | R |
| 191 | ATS advisory that the aircraft is entering airspace in which no air traffic services are provided and all existing air traffic services are terminated. | ALL ATS TERMINATED | N | M | R |
| 155 | ATS advisory that radar contact has been established at the specified position. | RADAR CONTACT [position] | N | M | R |
| 156 | ATS advisory that radar contact has been lost. | RADAR CONTACT LOST | N | M | R |
| 210 | ATS advisory that the aircraft has been identified on radar at the specified position. | IDENTIFIED [position] | N | M | R |
| 193 | Indication that radar identification has been lost. | IDENTIFICATION LOST | N | M | R |
| 157 | A continuous transmission is detected on the specified frequency. Check the microphone button. | CHECK STUCK MICROPHONE [frequency] | U | M | N |
| 158 | ATS advisory that the ATIS information identified by the specified code is the current ATIS information. | ATIS [atis code] | N | L | R |
| 212 | ATS advisory that the specified ATIS information at the specified airport is current. | [icao facility designation] ATIS [atis code] CURRENT | N | L | R |
| 214 | ATS advisory that indicates the RVR value for the specified runway. | RUNWAY [runway] VISUAL RANGE [rvr] | N | M | R |
| 224 | ATS advisory that no delay is expected. | NO DELAY EXPECTED | N | L | R |
| 225 | ATS advisory that the expected delay has not been determined. | DELAY NOT DETERMINED | N | L | R |
| 226 | ATS advisory that the aircraft may expect to be cleared to commence its approach procedure at the specified time. | EXPECTED APPROACH TIME [time] | N | L | R |

Table A-10:  Air Traffic Advisories (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 159 | A system generated message that the ground system has detected an error. | ERROR [error information] | U | M | N |
| 160 | Notification to the avionics that the next data authority is the specified ATSU. | NEXT DATA AUTHORITY [icao facility designation] | L | N | N |
| 161 | Notification to the avionics that the data link connection with the current data authority is being terminated. | END SERVICE | L | N | N |
| 162 | Notification that the ground system does not support this message. | SERVICE UNAVAILABLE | L | L | N |
| 234 | Notification that the ground system does not have a flight plan for that aircraft. | FLIGHT PLAN NOT HELD | L | L | N |
| 163 | Notification to the pilot of an ATSU identifier. | [icao facility designation] | L | N | N |
| 227 | Confirmation to the aircraft system that the ground system has received the message to which the logical acknowledgment refers and found it acceptable for display to the responsible person. | LOGICAL ACKNOWLEDGMENT | N | M | N |
| 233 | Notification to the pilot that messages sent requiring a logical acknowledgment will not be accepted by this ground system. | USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED | N | M | N |

Table A-11:  System Management Messages (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 164 | The associated instruction may be complied with at any future time. | WHEN READY | L | N | N |
| 230 | The associated instruction is to be complied with immediately. | IMMEDIATELY | D | H | N |
| 165 | Used to link two messages, indicating the proper order of execution of clearances/ instructions. | THEN | L | N | N |
| 166 | The associated instruction is issued due to traffic considerations. | DUE TO [traffic type] TRAFFIC | L | N | N |
| 167 | The associated instruction is issued due to airspace restrictions. | DUE TO AIRSPACE RESTRICTION | L | N | N |
| 168 | The indicated communication should be ignored. | DISREGARD | U | M | R |
| 176 | Notification that the operator is responsible for maintaining separation from other traffic and is also responsible for maintaining Visual Meteorological Conditions. | MAINTAIN OWN SEPARATION AND VMC | N | M | W/U |
| 177 | Used in conjunction with a clearance/instruction to indicate that the operator may execute when prepared to do so. | AT PILOTS DISCRETION | L | L | N |
| 169 |  | [free text] | N | L | R |
| 170 |  | [free text] | D | H | R |
| 194 |  | [free text] | N | L | Y |
| 178 |  | [free text] | N | L | N |
| 195 |  | [free text] | L | L | R |
| 196 |  | [free text] | N | M | W/U |
| 197 |  | [free text] | U | M | W/U |
| 198 |  | [free text] | D | H | W/U |
| 199 |  | [free text] | N | M | W/U |
| 200 |  | [free text] | L | L | R |
| 201 |  | [free text] | N | M | W/U |
| 202 |  | [free text] | D | H | W/U |
| 203 |  | [free text] | N | M | R |
| 204 |  | [free text] | N | M | Y |
| 183 |  | [free text] | N | M | N |
| 205 |  | [free text] | N | M | A/N |
| 206 |  | [free text] | L | N | Y |
| 187 |  | [free text] | L | N | N |
| 207 |  | [free text] | L | L | Y |
| 208 |  | [free text] | L | L | N |

Table A-12:  Additional Messages (uplink)

A-3	Downlink Messages

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 0 | The instruction will be complied with. | WILCO | N | M | N |
| 1 | The instruction cannot be complied with. | UNABLE | N | M | N |
| 2 | Wait for a reply. | STANDBY | N | M | N |
| 3 | Message received and understood. | ROGER | N | M | N |
| 4 | Yes. | AFFIRM | N | M | N |
| 5 | No. | NEGATIVE | N | M | N |

Table A-13:  Responses (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 6 | Request to fly at the specified altitude. | REQUEST [altitude] | N | L | Y |
| 7 | Request to fly at an altitude within the specified altitude interval. | REQUEST BLOCK [altitude] TO [altitude] | N | L | Y |
| 8 | Request to cruise climb to the specified altitude. | REQUEST CRUISE CLIMB TO [altitude] | N | L | Y |
| 9 | Request to climb to the specified altitude. | REQUEST CLIMB TO [altitude] | N | L | Y |
| 10 | Request to descend to the specified altitude. | REQUEST DESCENT TO [altitude] | N | L | Y |
| 11 | Request that at the specified position a climb to the specified altitude be approved. | AT [position] REQUEST CLIMB TO [altitude] | N | L | Y |
| 12 | Request that at the specified position a descent to the specified altitude be approved. | AT [position] REQUEST DESCENT TO [altitude] | N | L | Y |
| 13 | Request that at the specified time a climb to the specified altitude be approved. | AT [time] REQUEST CLIMB TO [altitude] | N | L | Y |
| 14 | Request that at the specified time a descent to the specified altitude be approved. | AT [time] REQUEST DESCENT TO [altitude] | N | L | Y |
| 69 | Request that a descent be approved on a see-and-avoid basis. | REQUEST VMC DESCENT | N | L | Y |

Table A-14:  Vertical Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 15 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved. | REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |
| 16 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified position. | AT [position] REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |
| 17 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified time. | AT [time] REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |

Table A-15:  Lateral Off-Set Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 18 | Request to fly at the specified speed. | REQUEST [speed] | N | L | Y |
| 19 | Request to fly within the specified speed range. | REQUEST [speed] TO [speed] | N | L | Y |

Table A-16:  Speed Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 20 | Request for voice contact. | REQUEST VOICE CONTACT | N | L | Y |
| 21 | Request for voice contact on the specified frequency. | REQUEST VOICE CONTACT [frequency] | N | L | Y |

Table A-17:  Voice Contact Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 22 | Request to track from the present position direct to the specified position. | REQUEST DIRECT TO [position] | N | L | Y |
| 23 | Request for the specified procedure clearance. | REQUEST [procedure name] | N | L | Y |
| 24 | Request for a route clearance. | REQUEST CLEARANCE [route clearance] | N | L | Y |
| 25 | Request for a clearance. | REQUEST [clearance type] CLEARANCE | N | L | Y |
| 26 | Request for a weather deviation to the specified position via the specified route. | REQUEST WEATHER DEVIATION TO [position] VIA [route clearance] | N | M | Y |
| 27 | Request for a weather deviation up to the specified distance off track in the specified direction. | REQUEST WEATHER DEVIATION UP TO [distance offset] [direction] OF ROUTE | N | M | Y |
| 70 | Request a clearance to adopt the specified heading. | REQUEST HEADING [degrees] | N | L | Y |
| 71 | Request a clearance to adopt the specified ground track. | REQUEST GROUND TRACK [degrees] | N | L | Y |

Table A-18:  Route Modification Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 28 | Notification of leaving the specified altitude. | LEAVING [altitude] | N | L | N |
| 29 | Notification of climbing to the specified altitude. | CLIMBING TO [altitude] | N | L | N |
| 30 | Notification of descending to the specified altitude. | DESCENDING TO [altitude] | N | L | N |
| 31 | Notification of passing the specified position. | PASSING [position] | N | L | N |
| 78 | At the specified time, the aircraft’s position was as specified. | AT [time] [distance] [to/from] [position] | N | L | N |
| 32 | Notification of the present altitude. | PRESENT ALTITUDE[altitude] | N | L | N |
| 33 | Notification of the present position. | PRESENT POSITION [position] | N | L | N |
| 34 | Notification of the present speed. | PRESENT SPEED [speed] | N | L | N |
| 35 | Notification of the present heading in degrees. | PRESENT HEADING [degrees] | N | L | N |
| 36 | Notification of the present ground track in degrees. | PRESENT GROUND TRACK [degrees] | N | L | N |
| 37 | Notification that the aircraft is maintaining the specified altitude. | ALTITUDE [altitude] | N | L | N |
| 72 | Notification that the aircraft has reached the specified altitude. | REACHING [altitude] | N | L | N |
| 76 | Notification that the aircraft has reached an altitude within the specified altitude range. | REACHING BLOCK [altitude] TO [altitude] | N | L | N |
| 38 | Read-back of the assigned altitude. | ASSIGNED ALTITUDE [altitude] | N | M | N |
| 77 | Read-back of the assigned altitude range. | ASSIGNED BLOCK [altitude] TO [altitude] | N | M | N |
| 39 | Read-back of the assigned speed. | ASSIGNED SPEED [speed] | N | M | N |
| 40 | Read-back of the assigned route. | ASSIGNED ROUTE [route clearance] | N | M | N |
| 41 | The aircraft has regained the cleared route. | BACK ON ROUTE | N | M | N |
| 42 | The next waypoint is the specified position. | NEXT WAYPOINT [position] | N | L | N |
| 43 | The ETA at the next waypoint is as specified. | NEXT WAYPOINT ETA [time] | N | L | N |
| 44 | The next but one waypoint is the specified position. | ENSUING WAYPOINT [position] | N | L | N |
| 45 | Clarification of previously reported waypoint passage. | REPORTED WAYPOINT [position] | N | L | N |
| 46 | Clarification of time over previously reported waypoint. | REPORTED WAYPOINT [time] | N | L | N |
| 47 | The specified beacon code has been selected and the SSR transponder is operational. | SQUAWKING [beacon code] | N | L | N |
| 48 | Position report. | POSITION REPORT [position report] | N | M | N |
| 79 | The code of the latest ATIS received is as specified. | ATIS [atis code] | N | L | N |
| 89 | The specified ICAO unit is being monitored on the specified frequency. | MONITORING [icaounitname] [frequency] | U | M | N |
| 102 | Used to report that an aircraft has landed. | LANDING REPORT | N | N | N |
| 104 | Notification of estimated time of arrival at the specified position. | ETA [position][time] | L | L | N |
| 105 | Notification of the alternative aerodrome for landing. | ALTERNATE AERODROME [airport] | L | L | N |
| 106 | Notification of the preferred altitude. | PREFERRED ALTITUDE [altitude] | L | L | N |
| 109 | Notification of the preferred time to commence descent for approach. | TOP OF DESCENT [time] | L | L | N |
| 110 | Notification of the preferred position to commence descent for approach. | TOP OF DESCENT [position] | L | L | N |
| 111 | Notification of the preferred time and position to commence descent for approach. | TOP OF DESCENT [time] position] | L | L | N |

Table A-19:  Reports (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 49 | Request for the earliest time at which a clearance to the specified speed can be expected. | WHEN CAN WE EXPECT [speed] | L | L | Y |
| 50 | Request for the earliest time at which a clearance to a speed within the specified range can be expected. | WHEN CAN WE EXPECT [speed] TO [speed] | L | L | Y |
| 51 | Request for the earliest time at which a clearance to regain the planned route can be expected. | WHEN CAN WE EXPECT BACK ON ROUTE | L | L | Y |
| 52 | Request for the earliest time at which a clearance to descend can be expected. | WHEN CAN WE EXPECT LOWER ALTITUDE | L | L | Y |
| 53 | Request for the earliest time at which a clearance to climb can be expected. | WHEN CAN WE EXPECT HIGHER ALTITUDE | L | L | Y |
| 54 | Request for the earliest time at which a clearance to cruise climb to the specified altitude can be expected. | WHEN CAN WE EXPECT CRUISE CLIMB TO [altitude] | L | L | Y |
| 87 | Request for the earliest time at which a clearance to climb to the specified altitude can be expected. | WHEN CAN WE EXPECT CLIMB TO [altitude] | L | L | Y |
| 88 | Request for the earliest time at which a clearance to descend to the specified altitude can be expected. | WHEN CAN WE EXPECT DESCENT TO [altitude] | L | L | Y |

Table A-20:  Negotiation Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 55 | Urgency prefix. | PAN PAN PAN | U | H | N |
| 56 | Distress prefix. | MAYDAY MAYDAY MAYDAY | D | H | N |
| 112 | Indicates specifically that the aircraft is being subjected to unlawful interference. | SQUAWKING 7500 | U | H | N |
| 57 | Notification of fuel remaining and number of persons on board. | [remaining fuel] OF FUEL REMAINING AND [souls on board] SOULS ON BOARD | U | H | N |
| 58 | Notification that the pilot wishes to cancel the emergency condition. | CANCEL EMERGENCY | U | M | N |
| 59 | Notification that the aircraft is diverting to the specified position via the specified route. | DIVERTING TO [position] VIA [route clearance] | U | H | N |
| 60 | Notification that the aircraft is deviating the specified distance in the specified direction off the cleared route and maintaining a parallel track. | OFFSETTING [distance offset] [direction] OF ROUTE | U | H | N |
| 61 | Notification that the aircraft is descending to the specified altitude. | DESCENDING TO [altitude] | U | H | N |
| 80 | Notification that the aircraft is deviating from the cleared route by the specified distance in the specified direction. | DEVIATING [distance offset] [direction] OFF ROUTE | U | H | N |

Table A-21:  Emergency Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 62 | A system generated message that the avionics has detected an error. | ERROR [error information] | U | L | N |
| 63 | A system generated denial to any CPDLC message sent from a ground facility that is not the Current Data Authority. | NOT CURRENT DATA AUTHORITY | L | L | N |
| 99 | A system generated message to inform a ground facility that it is now the Current Data Authority. | CURRENT DATA AUTHORITY | L | L | N |
| 64 | Notification to the ground system that the specified ATSU is the current data authority. | [icao facility designation] | L | L | N |
| 107 | A system generated message sent to a ground system that tries to connect to an aircraft when a current data authority has not designated the ground system as the NDA. | NOT AUTHORIZED NEXT DATA AUTHORITY | L | L | N |
| 73 | A system generated message indicating the software version number. | [version number] | L | L | N |
| 100 | Confirmation to the aircraft system that the ground system has received the message to which the logical acknowledgment refers and found it acceptable for display to the responsible person. | LOGICAL ACKNOWLEDGMENT | N | M | N |

Table A-22:  System Management Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 65 | Used to explain reasons for aircraft operator’s message. | DUE TO WEATHER | L | L | N |
| 66 | Used to explain reasons for aircraft operator’s message. | DUE TO AIRCRAFT PERFORMANCE | L | L | N |
| 74 | States a desire by the aircraft operator to provide his/her own separation under see and avoid conditions. | REQUEST TO MAINTAIN OWN SEPARATION AND VMC | L | L | Y |
| 75 | Used in conjunction with another message to indicate that the operator wishes to execute request when the air crew is prepared to do so. | AT PILOTS DISCRETION | L | L | N |
| 101 | Allows the aircraft operator to indicate a desire for termination of CPDLC service with the current data authority. | REQUEST END OF SERVICE | L | L | Y |
| 103 | Allows the aircraft operator to indicate that he has canceled IFR flight plan. | CANCELING IFR | N | L | Y |
| 108 | Notification that de-icing action has been completed. | DE-ICING COMPLETE | L | L | N |
| 67 |  | [free text] | N | L | N |
| 68 |  | [free text] | D | H | Y |
| 90 |  | [free text] | N | M | N |
| 91 |  | [free text] | N | L | Y |
| 92 |  | [free text] | L | L | Y |
| 93 |  | [free text] | U | H | N |
| 94 |  | [free text] | D | H | N |
| 95 |  | [free text] | U | M | N |
| 96 |  | [free text] | U | L | N |
| 97 |  | [free text] | L | L | N |
| 98 |  | [free text] | N | N | N |

Table A-23:  Additional Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 81 | We can accept the specified altitude at the specified time. | WE CAN ACCEPT [altitude] AT [time] | L | L | N |
| 82 | We cannot accept the specified altitude. | WE CANNOT ACCEPT [altitude] | L | L | N |
| 83 | We can accept the specified speed at the specified time. | WE CAN ACCEPT [speed] AT [time] | L | L | N |
| 84 | We cannot accept the specified speed. | WE CANNOT ACCEPT [speed] | L | L | N |
| 85 | We can accept a parallel track offset the specified distance in the specified direction at the specified time. | WE CAN ACCEPT [direction] [distance offset] at [time] | L | L | N |
| 86 | We cannot accept a parallel track offset the specified distance in the specified direction. | WE CANNOT ACCEPT [direction] [distance offset] | L | L | N |

Table A-24:  Negotiation Responses (downlink)

1

PART IV-CONTROLLLER PILOT DATA LINK COMMUNICATION

CHAPTER 3 APPENDIX B

CPDLC MESSAGE DATA GLOSSARY

B.1	CPDLC Data Glossary

B.1.1	The following data are used as the CPDLC message variables, or component of the variables, and are shown here in alphabetical order:

Aircraft Equipment Code:  Component variable used to indicate the type of communication, navigation and approach aid equipment on an aircraft as defined in the Procedures for Air Navigation Services - Rules of the Air and Air Traffic Services (PANS-RAC, ICAO Doc 4444).  Aircraft equipment code consists of the following sequence of information:

- an indication of whether or not COM NAV Approach Equipment is available,

- optionally a sequence containing 1-24 COM NAV Equipment Status, and

- an indication of what SSR Equipment is available.

Aircraft Identification:  Field 7 of the ICAO flight plan.

Aircraft Type:  An IA5 string consisting of 2-5 characters used to identify the aircraft type as defined in Aircraft Type Designators (ICAO Doc 8643).  This data structure also includes 1 character used to specify the wake turbulence category.

Airport:  Four characters that specifies the ICAO four-letter identifier for the airport.

Airway Identifier:  Specifies the particular airway to be used within the route of the aircraft.  (IA5 string of 2-5 characters.)

Airway Intercept:  Specifies the airway which will be intercepted and followed in the aircraft route of flight.

Altimeter:  Indicates the aircraft altimeter setting in metric or English measurement units.

Altitude:  Specifies the altitude in one of the following ways:

- Altitude in feet or meters,

- Altitude GNSS in feet or meters, or

- Flight Level in feet or meters.

Altitude Current:  Specifies the current aircraft altitude.

Altitude GNSS:  Specifies the altitude above GNSS zero altitude reference, obtained from GPS/GNSS position data, in either feet or meters.

Altitude Tolerance:  Choice to indicate at, at or above, or at or below, concerning the related altitude value.

Approved Departure Time:  Departure time issued by ATC or ATFM.

ATIS Code:  Specifies the alphanumeric value for the current version of the automatic terminal information service (ATIS) in effect at a given location.

ATW Along Track Waypoint:  Sequence of information used to compute additional way-points to an aircraft's route of flight.  The following data composes the ATW Along Track Waypoint:

- Position,

- ATW Distance,

- Speed (optional), and

- ATW Altitude Sequence (optional).

ATW Altitude:  Contains ATW Altitude Tolerance and Altitude.

ATW Altitude Sequence:  Sequence of 1 or 2 ATW Altitudes.

ATW Altitude Tolerance:  Indicates the vertical tolerance factor for altitude clearances.  Used in altitude clearances to indicate the acceptable vertical clearance of an aircraft relative to a particular altitude.  Indicates:

- at,

- at or above, or

- at or below.

ATW Distance:  Used to specify the distance along a route of flight at which point to add the fix.  Composed of ATW Distance Tolerance and Distance.

ATW Distance Tolerance:  Indicates whether a distance can be plus or minus.

Beacon Code:  Specifies the Mode A value (beacon code) for the aircraft.

Clearance Expiry Time:  Time after which a given clearance is no longer valid.

Clearance Limit:  The point to which an aircraft is granted an air traffic control clearance.  Specified as a Position.

Clearance Type:  Specifies a particular type of clearance.  Where specified, the following clearance types are permitted:

- approach,

- departure,

- further,

- start-up,

- pushback

- taxi,

- take-off,

- landing,

- oceanic,

- en-route, or

- downstream.

COM NAV Approach Equipment Available:  Indicates the presence or absence of communication, navigation and approach aid equipment.

COM NAV Equipment Status:  Indicates which communication, navigation and approach aid equipment is available for use as defined in the Procedures for Air Navigation Services - Rules of the Air and Air Traffic Services (PANS-RAC, ICAO Doc 4444).

Date:  Gives the date in YYMMDD format using Year, Month, and Day data.

Date Time Track Generated:  Date and time of the creation of a track.

Date Time Group:  Provides date and time as YYMMDD and HHMMSS.

Date Time Track Start:  Date and time of the track activation.

Date Time Track Stop:  Date and time of the track termination.

Day:  Day of the month.

Degree Increment:  Specifies the number of degrees (of latitude or longitude) separating reporting points.

Degree Minutes:  Provides minutes of a latitude or longitude degree.

Degree Seconds:  Provides seconds of a latitude or longitude degree.

Degrees:  Indicates the degree value in degrees magnetic or degrees true.

Degree Value:  Degrees of a circle.

Departure Airport:  Flight plan departure airport.

Departure Clearance:  Sequence of data structures necessary to provide a departure clearance.  The sequence of data structures that compose a departure clearance data structure are:

- Aircraft Identification,

- Clearance Limit,

- Flight Information,

- Further Instructions (optional).

Departure Expected Clearance Time: Indicates time when a clearance is expected.  Associated with flow management program in effect.

Departure Frequency:  Provides departure frequency as a Frequency and ICAO Unit Name.

Departure Minimum Interval:  Specifies the minimum interval of time to depart behind the preceding aircraft.

Departure Runway:  Runway of departure.

Departure Time:  Sequence of Approved Departure Time (optional), Departure Time Controlled (optional), Departure Expected Clearance Time (optional) data, Departure Minimum Interval (optional).

Departure Time Controlled:  Specifies the time the aircraft is allowed to depart within a time window.

Destination Airport:  Flight plan destination airport.

Direction:  Indicates the horizontal direction specified in terms of the current direction relative to the aircraft or in terms of the cardinal points of the compass.  Values are as indicated:

- left,

- right

- either side,

- north,

- south,

- east,

- west,

- north east,

- north west,

- south east, or

- south west.

Distance:  Provides the distance in metric or English units.

Distance Offset Direction:  Sequence of Distance Offset and Direction data.

Distance Offset:  Specifies the offset distance from the aircraft's route in metric or English units.

Distance To Next Point:  Indicates the distance to the next way-point in metric or English units.

Error Information:  Indicates the error conditions as follows:

- unrecognized message reference number,

- end service with pending messages,

- logical acknowledgment not accepted, or

- more than one next data authority element.

EFC Time:  Specifies the time when a further clearance is expected.

Fix Name:  Specifies the ICAO identifier for a given fix.

Fix Next:  Indicates the next point in the aircraft's route as a Position.

Fix Next Plus One:  Specifies the point after the next point in the aircraft's route as a Position.

Flight Information:  Information for a route of flight.  Specified as:

- Route of Flight, or

- Levels of Flight, or

- Route of Flight and Levels of Flight.

Flight Level:  As defined in PANS/RAC (Doc 4444).

Flight Plan Segment:  Indicates the type of information used to define a particular point in the aircraft route of flight.

Free Text:  Used to convey unstructured information.

Frequency:  Specifies the frequency and an indicator of the RF spectrum used for the given frequency.  The types of frequency that can be provided include:

- Frequency HF,

- Frequency VHF,

- Frequency UHF, or

- Frequency Sat Channel,

Frequency Sat Channel:  Specifies the appropriate address for use with a satellite voice system.

Further Instructions:  Provides additional information in a departure clearance as follows:

- Beacon Code (optional),

- Departure Frequency (optional),

- Clearance Expiry Time (optional),

- Departure Airport (optional),

- Destination Airport (optional),

- Departure Time (optional),

- Departure Runway (optional)

- Revision Number (optional), or

- ATIS Code (optional).

Hold At Waypoint:  Sequence of data structures used to define the holding procedure to be used at a particular point.  The Hold At Waypoint consists of an sequence of the following:

- Position,

- Hold At Waypoint Speed Low (optional),

- ATW Altitude (optional),

- Hold At Waypoint Speed High (optional),

- Direction (optional),

- Degrees (optional),

- EFC Time (optional), and

- Legtype (optional).

Hold At Waypoint Speed Low:  Specifies a holding speed.  When used with Hold At Waypoint Speed High, specifies the lower value for a holding speed range.

Hold At Waypoint Speed High:  Specifies the upper value for a holding speed range.

Hold Clearance:  Provides a holding clearance to the aircraft.  The Hold Clearance is provided using:

- Position,

- Altitude,

- Degrees,

- Direction, and

- Legtype (optional).

Hours:  Specifies the hour in 24-hour notation.

ICAO Facility Designation:  Specifies the ICAO eight-letter location indicator for the facility.

ICAO Facility Identification:  Provides an ICAO facility identification as either an ICAO Facility Designation or an ICAO Facility Name.

ICAO Facility Function:  Indicates the ATC function performed by the ICAO facility as follows:

- center,

- approach,

- tower,

- final,

- ground control,

- clearance delivery,

- departure, or

- control.

ICAO Facility Name:  Specifies the ICAO facility name.

ICAO Unit Name:  Sequence of ICAO Facility Identification and ICAO Facility Function.

ICAO Unit Name Frequency:  Sequence of ICAO Unit Name and Frequency.

ICAO Unit Name Of Departure Frequency:  Specifies the ICAO facility name for the departure frequency.

Intercept Course From:  The Intercept Course From is used to specify a fix and a bearing from that fix needed to intercept a route using Intercept Course From Selection and Degrees.

Intercept Course From Selection.  Used to specify the point from which the intercept course originates and an indication of which type of fix is specified.  Provided as one of the following:

- Published Identifier,

- Latitude Longitude,

- Place Bearing Place Bearing, or

- Place Bearing Distance.

Latitude:  Provides latitude as Latitude Degrees, Degree Minutes (optional), and Degree Seconds (optional).

Latitude Degrees:  Degrees of latitude.

Latitude Direction:  Indicates whether north or south latitude is specified.

Latitude Longitude:  Sequence of Latitude and Longitude.

Latitude Reporting Points:  Indicates the latitude on which to base incremental reporting points.  Provided as of Latitude Direction and Latitude Degrees.

Latlon Reporting Points:  Provides either Latitude Reporting Points or Longitude Reporting Points.

Leg Distance:  Indicates the aircraft leg in metric or English distance units.

Leg Time:  Specifies aircraft leg in terms of minutes.

Leg Type:  Provides either Leg Distance or Leg Time.

Levels of Flight:  Specified as a choice of:

- Altitude, or

- Procedure Name, or

- Altitude and Procedure Name.

Longitude:  Provides longitude as Longitude Degrees, Degree Minutes (optional), and Degree Seconds (optional).

Longitude Degrees:  Degrees of longitude.

Longitude Direction:  Indicates whether east or west longitude is specified.

Longitude Reporting Points:  Indicates the longitude on which to base incremental reporting points.  Provided as Longitude Direction and Longitude Degrees.

Month:  Month of the year.

Navaid:  Specifies a particular navigation aid.

Persons On Board:  Specifies the number of persons on the aircraft.

Place Bearing:  Sequence of Fix Name, Latitude Longitude (optional), and Degrees.

Place Bearing Distance:  Used to indicate a location based on the degrees and distance from a known point.  Provided using Place Bearing and Distance data.

Place Bearing Place Bearing:  Used to define a point as the intersection formed by two bearings from two known points.  Provided as two Place Bearing.

Point Altitude:  Specifies altitude related details concerning a given point.  Provided using Flight Level and ATW Altitude Tolerance.

Point Altitude Block:  Provides an altitude range using two Flight Level.

Position:  Information used to specify a location.  Position can be specified as:

- Fix Name,

- Navaid,

- Airport,

- Latitude Longitude, or

- Place Bearing Distance.

Position Current:  Specifies the current location of the aircraft as a Position.

Position Report: Uses the following data necessary to provide an aircraft position report as follows:

- Position Current,

- Time At Position Current,

- Altitude,

- Fix Next (optional),

- Time ETA At Fix Next (optional),

- Fix Next Plus One (optional),

- Time ETA destination (optional),

- Remaining Fuel (optional),

- Temperature (optional),

- Winds (optional),,

- Turbulence (optional),

- Icing (optional),

- Speed (optional),

- Speed Ground (optional),

- Vertical Change (optional),

- Track Angle (optional),

- True Heading (optional),

- Distance (optional),

- Supplementary Information (optional),

- Reported Waypoint Position (optional),

- Reported Waypoint Time (optional), and

- Reported Waypoint Altitude (optional).

Procedure:  Specifies the name of the procedure.

Procedure Approach:  Specifies a procedure as an approach procedure.

Procedure Arrival:  Specifies a procedure as an arrival procedure.

Procedure Departure:  Specifies a procedure as a departure procedure.

Procedure Name:  Used to uniquely identify the standard arrival, approach or departure procedure using the following:

- Procedure Type,

- Procedure, and

- Procedure Transition (optional).

Procedure Type:  Specifies the type of procedure as arrival, approach, or departure.

Procedure Transition:  Specifies the name of the procedure transition.

Published Identifier:  Used to provide the location of the specified fix.  Provided using Fix Name and Latitude Longitude.

Remaining Fuel:  Specifies the amount of fuel remaining on the aircraft using Time data.

Reported Waypoint Position:  The position of the waypoint for which the report is being made.

Reported Waypoint Time:  The time of the waypoint for which the report is being made.

Reported Waypoint Altitude:  The altitude of the waypoint for which the report is being made.

Reporting Points:  Used to indicate reporting points along a route of flight based on a specific Latitude and/or Longitude increment expressed in degrees.

Revision Number:  Specifies the revision number of the departure clearance.  Used to differentiate different revisions of the departure clearance for a given aircraft flight.

Route Clearance:  Data necessary to provide a route clearance.  Provided using the following data:

- Aircraft Identification (optional),

- Departure Airport (optional),

- Destination Airport (optional),

- Gate (optional),

- Runway Departure (optional),

- Procedure Departure (optional),

- Runway Arrival (optional),

- Procedure Approach (optional),

- Procedure Arrival (optional),

- Airway Intercept (optional),

- Route Information Sequence (optional), and

- Route Information Additional (optional).

Route Information:  Indicate the method used to define the aircraft route of flight.  The actual aircraft route of flight will probably consist of multiple Route Information sequences as follows:

- Published Identifier (optional)

- Latitude Longitude (optional),

- Place Bearing Place Bearing (optional),

- Place Bearing Distance (optional),

- Airway Identifier (optional), and

- Track Detail (optional).

Route Information Additional:  Additional data used to further specify a route clearance.  Provided using the following:

- ATW Along Track Waypoint Sequence (optional),

- Reporting Points (optional),

- Intercept Course From Sequence (optional),

- Hold At Waypoint Sequence (optional),

- Waypoint Speed Altitude Sequence (optional), and

- RTA Required Time Arrival Sequence (optional).

Route Of Flight:  Specifies route of flight using Route Information

RTA Required Time Arrival:  Sequence used to associate an estimated time of arrival with a specific point along a route of flight.  The RTA Required Time Arrival consists of:

- Position,

- RTA Time, and

- RTA Tolerance (optional).

RTA Time:  Used to specify the required time of arrival for an aircraft at a specific point.

RTA Tolerance:  Specifies the possible tolerance expressed in minutes in the RTA time.

Runway:  Specifies a runway using Runway Direction and Runway Configuration.

Runway Arrival:  Specifies the arrival runway.

Runway Configuration:  Used to specifically identify one runway in a group of parallel runways.  Can be specified as left, right, or center.

Runway Departure:  Specifies the departure runway.

Runway Direction:  Specifies the direction of the runway.

RVR:  Runway Visual Range as distance.

Speed:  Provides the aircraft speed as one of the following:

- Speed Indicated,

- Speed True,

- Speed Ground, or

- Speed Mach.

Speed Ground:  Ground speed expressed in either English or metric units.

Speed Indicated:  Indicated aircraft speed expressed in either English or metric units.

Speed Mach:  Aircraft speed specified as a Mach value.

Speed Qualifier:  Qualifies the given speed type.  Values include minimum approach or cruise.

Speed True:  Aircraft true speed expressed in either English or metric units.

Speed Type:  Indicates what type of speed is to be provided:

- Indicated,

- True,

- Ground,

- Mach, or

- Not specified.

SSR Equipment Available:  Indicates which surveillance equipment is available for use as defined in the Procedures for Air Navigation Services - Rules of the Air and Air Traffic Services (PANS-RAC, ICAO Doc 4444).

Temperature:  Temperature specified in centigrade or Fahrenheit.

Time:  Sequence of Hours and Time Minutes.

Time At Position Current:  Specifies the time that the current location of the aircraft was indicated.

Time ETA Destination:  Specifies the time an aircraft is expecting to land at the destination airport.

Time ETA At Fix Next:  Specifies the time an aircraft is expecting to cross the next point in the route.

Time HHMMSS:  Provides time as HHMMSS.

Time Minutes:  Specifies time in minutes of an hour.

Time Seconds:  Specifies time in seconds of a minute.

To From:  Specifies to or from.

Time Tolerance:  Provides a time tolerance as:  at, at or before, or at or after.

To From Position:  Used to indicate a “to” or “from” relative to a specified position.

Track Angle:  Specifies the aircraft ground track in degrees.

Track Detail:  Associates a sequence of fixes with a particular track name.  Specified using Track Name and Latitude Longitude.

Track Name:  Specifies the name of an identified group of points which make up a section of a route.

Traffic Type:  Indicates what type of traffic is present.  Permitted types:

- opposite direction,

- same direction,

- converging, or

- crossing.

True Heading:  Specifies the aircraft true heading in degrees.

True Track Angle:  Specifies true track angle to the next way-point using degrees.

Turbulence:  Specifies the severity of turbulence.  Can be one of the following:  “light”, “moderate”, or “severe”.

Vertical Change:  Sequence of Vertical Direction and Vertical Rate.

Vertical Direction:  Specifies whether the rate of vertical change is in the upward or downward direction.

Vertical Rate:  Specifies the vertical rate of change in English or metric units.

Waypoint Speed Altitude:  Used to associate altitudes and speeds with particular points in a route clearance.  It is composed using the following:

- Position,

- Speed (optional), and

- ATW Altitude Sequence (optional).

Winds:  Provides wind using Wind Direction and Wind Speed.

Wind Direction:  Specifies the direction of the wind using Degree Value.

Wind Speed:  Provides wind speed in metric or English measurement units.

Year:  Provides year as last two digits of a year.

PART IV-CONTROLLLER PILOT DATA LINK COMMUNICATION

CHAPTER 3 APPENDIX C

CPDLC VARIABLES RANGE AND RESOLUTION

3C.1	CPDLC Variables Range and Resolution

3C.1.1 Table 3C-1 provides the required range and resolution for the message variables used in the CPDLC application.

| VARIABLES | PARAMETERS | Unit | Range/Size | Resolution |
| --- | --- | --- | --- | --- |
| Aircraft Identification |  | IA5 Character String | 2-8 characters | N/A |
| Aircraft Type |  | IA5 Character String | 2-5 characters | N/A |
| Airport |  | IA5 Character String | 4 characters | N/A |
| Airway Identifier |  | IA5 Character String | 1-5 characters | N/A |
| Altimeter | Altimeter English | Inches Hg | 22.00 - 32.00 | 0.01 |
|  | Altimeter Metric | Hecto Pascal | 750.0 - 1250.0 | 0.1 |
| Altitude | Flight Level English | 1 level (100 ft) | 30 - 700 | 1 |
|  | Flight Level Metric | 1 level (10 m) | 100 - 2 500 | 1 |
|  | Altitude English | Feet | -600 - +70 000 | 10 |
|  | Altitude Metric | Metres | -30 to +25 000 | 1 |
| ATIS Code |  | IA5 Character | 1 character | N/A |
| Beacon Code |  | Integer | 4 octal digits | N/A |
| Bearing |  | Degrees | 1 - 360 | 1 |
| Date | Year | Year of Century | 0 - 99 | 1 |
|  | Month | Month Of Year | 1 - 12 | 1 |
|  | Day | Day Of Month | 1 - 31 | 1 |
| Degrees | Degrees Magnetic | Degrees | 1 - 360 | 1 |
|  | Degrees True | Degrees | 1 - 360 | 1 |
| Departure Minimum Interval |  | Minutes | 0.1 - 15.0 | 0.1 |
| Distance | Distance nm | Nautical Miles | 0 - 1000 | 0.1 |
|  | Distance km | Kilometres | 0 - 2000 | 0.25 |
| Distance Offset | Distance Offset Metric | Kilometers | 1 - 500 | 1 |
|  | Distance Offset NM | Nautical Miles | 1 - 250 | 1 |
| Distance to Next Point | Distance To Next Waypoint English | Nautical Miles | 1 - 1000 | 0.1 |
|  | Distance To Next Waypoint Metric | Kilometres | 1 - 2000 | 0.1 |
| Distance Offset | Distance Offset nm | Nautical Miles | 1 - 250 | 1 |
|  | Distance Offset km | Kilometres | 1 - 500 | 1 |
| Fix Name |  | IA5 Character String | 1-5 characters | N/A |
| Free Text |  | IA5 Character String | 1 - 200 characters | N/A |
| Frequency | Frequency HF | KiloHz | 2 850 - 28 000 | 1 |
|  | Frequency VHF | MegaHz | 117.000 - 138.000 | 0.00833 |
|  | Frequency UHF | MegaHz | 225.000 - 399.975 | 0.025 |
|  | Satellite Channel | Digit string | 12 digits | N/A |
| ICAO Facility | ICAO Facility Designation | IA5 Character String | 8 characters | N/A |
|  | ICAO Facility Name | IA5 Character String | 3 - 18 characters | N/A |
|  | Navaid | IA5 Character String | 1 - 4 characters | N/A |
| Latitude | Longitude Degrees | Degrees | 0 - 90 | 0.001 |
|  | Degree Minutes | Minutes | 0 - 59.99 | 0.01 |
|  | Degree Seconds | Seconds | 0 - 59 | 1 |
| Longitude | Longitude Degrees | Degrees | 0 - 180 | 0.001 |
|  | Degree Minutes | Minutes | 0 - 59.99 | 0.01 |
|  | Longitude Seconds | Seconds | 0 - 59 | 1 |
| Leg Distance | Leg Distance English | Nautical Miles | 0-50 | 1 |
|  | Leg Distance Metric | Kilometres | 0-100 | 1 |
| Leg Time |  | Minutes | 0-10 | 1 |
| Persons On Board |  | Integer | 1 - 1000 | 1 |
| Procedure Transition |  | IA5 Character String | 1 - 5 characters | N/A |
| Revision Number |  | Integer | 1 - 16 | 1 |
| RTA Tolerance |  | Minutes | 0.1-15.0 | 0.1 |
| Runway Direction |  | Integer | 1 - 36 | 1 |
| RVR | RVR Feet | Feet | 0-6100 | 1 |
|  | RVR Meters | Meters | 0-5000 | 1 |
| Speed | Ground Speed English | Knots | - 50 to + 2000 | 1 |
|  | Ground Speed Metric | Kilometres/Hour | - 100 to + 4000 | 1 |
|  | Mach | Mach Number | 0.5 - 4.0 | 0.001 |
|  | Indicated English | Knots | 0-400 | 1 |
|  | Indicated Metric | Kilometers/Hour | 0-800 | 1 |
|  | Speed True Metric | Kilometers/Hour | 0-4000 | 1 |
|  | Speed True English | Knots | 0-2000 | 1 |
| Temperature | Temperature Celsius | Degrees Celsius | - 100 to + 100 | 1 |
| Time | Time Hours | Hours of Day | 0 - 23 | 1 |
|  | Time Minutes | Minutes of Hour | 0 - 59 | 1 |
|  | Time Seconds | Seconds of Minute | 0 - 59 | 1 |
| Track Name |  | IA5 Character String | 1-6 characters | N/A |
| Version Number | Version number | Integer | 0 - 15 | N/A |
| Vertical Rate | Vertical Rate English | Feet/Minute | ± 30 000 | 10 |
|  | Vertical Rate Metric | Metres/Minute | ± 10 000 | 10 |
| Wind Speed | Wind Speed English | Knots | 0 - 250 | 1 |
|  | Wind Speed Metric | Kilometres/Hour | 0 - 500 | 1 |

Table 3C-1:  CPDLC Variables Range and Resolution

PART IV- CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

# CPDLC Procedures

## General procedures

### CPDLC will use standard pilot/controller messages, as defined in Appendix A to Chapter 3 of this part, with free text messages used when required.

### When a required response is not successfully delivered, the message initiator is responsible for querying the state of the response via an appropriate medium.

### Except when the clearance specifies otherwise, execution of a clearance received via data link may begin upon pilot initiation of the action which sends the acceptance message.

### Messages are to be reviewed and responded to in a timely manner upon receipt.  If messages are queued, they are to be displayed sequentially in the order of receipt, with the exception that messages with a higher urgency will be displayed first.

### When a controller or pilot communicates via voice, the response should be via voice.

### If a data link message which requires a closure response is subsequently negotiated via voice, an appropriate data link closure response for that message will still be initiated.

Note:  Even though a voice response may have been provided, a data link response is necessary to ensure proper synchronization of ground and aircraft systems.

### Pilot CPDLC message alerts may be suppressed during take-off, approach, and landing.  If a message requiring an alert is received during the suppression period and is still pending at the end of the period, the alert will be re-initiated when the suppression period is over.

### Procedures should accommodate mixed data link and voice capability.

### Aircraft operating procedures for CPDLC should be consistent and independent of the flight phase or ATS facility.

PART IV- CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

# EXCEPTION Handling

## Error Conditions

### Messages Received Out of Order

#### The recipient will be informed when a message is received out of order.

### Duplicate Message Identification Numbers

#### If a CPDLC message is received containing an identification number identical to that of an identification number currently in use, the CPDLC link will be terminated and both users will be informed indicating duplicate message identifiers.

#### Duplicate message identification numbers are only detectable by the recipient system.  Detection will result in the second message being unavailable for display, as a CPDLC link abnormal termination will be initiated by the recipient system.  Both end users will be informed of the reason for the abnormal termination, specifically Duplicate Message Identification Numbers.

#### The originator system has no knowledge of which messages have resulted in Duplicate Message Identification Numbers in the recipient system, but the originator system must be aware of all messages within a particular link for which closures are pending.  In the event of a CPDLC link abnormal termination, these messages must be available for display to the originating end user.  Because the originating end user may not be aware of which of these messages have been received by the recipient, procedures must be in place stating that the originating end user will coordinate with the recipient end user as to the status and resolution of these messages.

Note:  The generation of Duplicate Message Identification Numbers is a catastrophic system failure.

### Invalid Reference Number

#### If the CPDLC-user receives a message containing a message reference number which is not identical to any message identification number currently in use, the CPDLC-user responds indicating the error.

### Unauthorized Use of Logical Acknowledgment

#### When the ground system receives a message requiring a logical acknowledgment where the use of logical acknowledgment has been prohibited, the ground system will:

- send a message containing an ERROR message element with the [errorinformation] parameter set to the value [logicalAcknowledgmentNot Accepted].

- discard the message requiring a logical acknowledgment.

PART IV- CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

# Departure Clearance Service Description

## Scope and Objective

### A flight due to depart from an airfield must first obtain departure information and clearance from the Controlling ATS Unit (ATSU).  The Departure Clearance (DC) Service provides automated assistance for requesting and delivering clearances, with the objective of reducing pilot and controller workload and diminishing clearance delivery delays.

## Expected Benefits, Anticipated Constraints, and Associated Human Factors

### Benefits

- reduction of the potential for communication errors between a pilot and controller;

- reduction of channel load;

- reduction of ground delays;

- reduction in pilot workload (e.g., reduced hand-copy of clearances and information, reduced radiotelephony monitoring and use);

- reduction in controller workload (e.g., reduced radiotelephony and monitoring);

- automatic validation of flight plan in the ATSU; and

- automatic preparation of departure clearance and information elements for controller validation.

### Constraints

- reduction of voice induced situational awareness for pilots; and

- reduced dialogue flexibility in case of non-routine communication.

### Human factors

#### Regardless of the level of automation of the system in use, it must be ensured that controllers and pilots have the opportunity to review, validate and acknowledge any clearances being delivered or received.

## Operating Method Without Data Link

### Where local procedures or flight category require, flights intending to depart from an airport must first obtain a departure clearance (DC) from the ATSU.  This process can only be accomplished if the flight operator has filed a flight plan with the appropriate ATM authority.  The DC may contain information relative to the take off phase of flight (e.g., take-off runway and SID, SSR code, departure slot, next contact frequency).

### The departure clearance procedure normally consists of the following chronological steps:

- The pilot calls and requests a departure clearance using R/T, generally prior to start-up.

- The controller acknowledges the request and formulates the clearance based on available flight plan data and in accordance with the allocated slot time of departure, if any.  The clearance is merged with existing flight plan data, either at the controller’s workstation or within local flight data processing systems.

- The controller delivers the clearance to the pilot using R/T.

- The pilot acknowledges the clearance via a full readback on R/T.

Note:  The pilot may negotiate a revised clearance after stage c) above.

## Operating Method With Data Link

### The departure clearance service will be available after data link communication initiation until the time the aircraft commences movement under its own power.  If local procedures permit, the departure clearance service should also be available from the time the aircraft commences movement under its own power until the aircraft is cleared to enter the flight’s take-off runway.  The use will be restricted to revisions of granted clearances normally initiated by the ATSU.

### The DC is processed directly between ATSU and aircraft.  Due to the use of digital communication and information representation, further automation is possible; some of these possibilities are included as supplementary information in the DC Service Description:

- When the flight is within a parameter time of its estimated time of departure, the pilot transmits a DC Request to the ATSU using data communication.

- If the DC Request is valid and a corresponding flight plan is available in the ATSU, the ATSU transfers a response to the aircraft accepting the request.

- A DC is composed and/or verified by the controller based on flight plan related data.  The ATSU transmits the DC to the aircraft.  In the event of a revised clearance, the DC Clearance will contain the Version Number data field containing the clearance version number applicable to this aircraft.

- The pilot will verify the operational contents of the DC, and if the pilot accepts and can comply with the operational contents, he will transmit a “WILCO” indication.  If the pilot is unable to accept the operational contents, he will transmit an "UNABLE” indication.

- A controller may send another clearance (reclearance) at any time before take-off (e.g., in the case of pilot re-negotiation or due to a change of runway) via this operating method beginning with step 3.  An appropriate pilot alerting mechanism must be in place.

Note:  The pilot may re-negotiate the clearance following the transmission of a pilot acknowledgment, by R/T if required.

### Initiation Conditions

#### The DC service is invoked on pilot input initiating the DC request, a suitable time prior to departure.

### Sequence of Services

#### This service will be capable of being invoked independently of other services.

Note:  There is a dependency on the data link initiation service having been completed, for address provision.

### Additional Guidelines

#### The service should not be available to the aircraft after take-off.

#### Since the delivery of the DC will be subject to local procedures, the time from the request for a DC to its delivery to the aircraft is a local variable.

#### Free text can be appended to either the DC request or the clearance as required.

#### Both the aircraft and the ATSU will have the capability to map a response with a request.

#### Ground and airborne systems will be updated with the operational contents of any R/T transactions.

### Error Handling

#### If after a DC request is sent, instead of the expected response, an error message is received.  If the error is correctable, the DC request may be corrected and re-sent.  In other cases, the pilot will initiate R/T.

Note:  In the event of any doubt or ambiguity, the negotiation will be carried out by R/T.

### Closure Conditions

#### Closure conditions, if required, should be the subject of local implementation.

### Time Sequence Diagram

### Figure 6-1 shows the normal sequence of messages in the DC.

Figure 6-1:  Time Sequence Diagram for Departure Clearance

## Information Exchanges

### Table 6-1 shows the information exchanged to effect a departure clearance.

| Message | Information required | Event/ Trigger | Source/ Destination. | Alert | Response Required |
| --- | --- | --- | --- | --- | --- |
| 25: REQUEST [clearance type] CLEARANCE | Indication that the clearance type is [departure], optional | Pilot input. | Aircraft/ ATSU | Low | Yes |
| 73: [departure clearance] | Aircraft Identification, Clearance Limit, Flight Information, Route of Flight, or Published Identifier, or Latitude Longitude, or Place Bearing Place Bearing, or Place Bearing Distance, or Airway Identifier, or Track Detail. Levels of Flight, or Altitude, or Procedure Name, or Altitude and Procedure Name. Route of Flight and Levels of Flight. Published Identifier, or Latitude Longitude, or Place Bearing Place Bearing, or Place Bearing Distance, or Airway Identifier, or Track Detail, and Altitude, or Procedure Name, or Altitude and Procedure Name. Further Instructions (optional). Beacon Code (optional), Departure Frequency (optional), Clearance Expiry Time (optional), Departure Airport (optional), Destination Airport (optional), Departure Time (optional), Departure Runway (optional), Revision Number (optional), ATIS Code (optional). | Receipt of a request for a clearance ( in appropriate domain) or of a request for a departure clearance. | ATSU/ aircraft | Medium | W/U |

Table 6-1:  Departure Clearance Service Information Exchange

PART IV- CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

# Transfer Of DATA Authority SERVICE DESCRIPTION

## Transfer of Data Authority

### Requirements For Transfer of Data Authority

#### CPDLC airborne and ground systems, and supporting procedures, will ensure that Transfer of Data Authority using the CPDLC application messages can be carried out in the following circumstances:

- independent of the transferring and receiving ATSUs ground/ground data communication capability;

- when both the transferring and receiving ATSUs are equipped for air/ground data link; and

- when only the transferring ATSU is equipped for air/ground data link.

#### ATSUs may elect to use ground/ground data exchanges in support of the transfer of data authority, subject to bilateral agreements, local procedures, local infrastructure.  Such ground/ground exchanges are not required for successful completion of the transfer.

#### Ground/ground data communication in support of ATC transfer of data authority, if applied, will be in accordance with the ICAO Manual for ATS Data Link Applications, ATS Inter-facility Data Communication (AIDC) section, and any applicable regional supplementary material.

### Transfer of Data Authority and Voice Communication in Conjunction

#### Transfer of the CPDLC link, can be carried out in conjunction with the transfer of voice ATC communication.  This process involves the transfer of all controller/pilot communication, between the CDA and the NDA, both the voice channel and the CPDLC link.

Note:  ATC voice channel changes which do not involve a change in CPDLC data authority (e.g., sector/sector transfers in the same ATSU), are not covered in this section.  Such transfers are handled as standard CPDLC exchanges, using the appropriate messages.

#### Sequence Diagram for Transfer of Data and Voice Concurrently

- If the CDA notifies the aircraft its NDA the establishment of a NDA link is permitted.

- A NDA link is established.

- The CDA instructs the aircraft to monitor or contact the NDA on the appropriate voice channel using the CONTACT and/or MONITOR messages

- The aircrew acknowledges the instruction to transfer voice and activates the new voice channel.

- The CDA link is terminated normally.

- The NDA now becomes the CDA and CPDLC messages can be exchanged.

#### Figures 7-1a and 7-1b illustrates transfer of data authority without supporting ground/ground connectivity.

Note: No Context Management or Log-On/Off functionality reflected in any of the following drawings.

T-ATSU and R-ATSU Equipped For Air/Ground Data Link

Figure 7-1b:  Transfer of Data Authority and Communication Without Ground/Ground Connectivity:

T-ATSU Initiated

T-ATSU not equipped or DSC connection when R-ATSU equipped for air/ground dateline -

Figure 7-1b:  Transfer of Data Authority and Communication Without Ground/Ground Connectivity:  Aircraft initiated

#### Figures 7-2a and 7-2b illustrates transfer of data authority with supporting ground/ground connectivity.

* Indicates an optional message

Figure 7-2a:  Transfer of Data Authority and Communication With Ground/Ground Connectivity : Automatic CPDLC Link Transfer (i.e., without contact/monitor instruction and associated pilot acknowledgment)

* Indicates an optional message

Figure 7-2b:  Transfer of Data Authority and Communication With  AIDC Ground/Ground Connectivity :

Manual CPDLC Link Transfer (i.e., contact/monitor instruction with pilot WILCO)

### Transfer of Data Authority Without Transfer of Voice Communication

#### Transfer of data authority can also occur which do not involve a change of the ATC voice channel changes in CPDLC data authority.

PART IV- CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC)

# DownstrEAM Clearance SERVICE DESCRIPTION

## Scope and Objective

### Aircrew, in specific instances, need to obtain clearances or information from ATSUs which may be responsible for control of the aircraft in the future, but are not yet in control of the aircraft.  Such ‘downstream’ clearances and information are often provided through ground/ground coordination, but are also obtained via direct contact with the ‘Downstream’ ATSU (D-ATSU) in certain circumstances (e.g., when ground/ground communication are unavailable or inefficient, due to the size of the airspace, due to the complexity of the route structure, or due to meteorological conditions).

### Unless otherwise coordinated, downstream clearances or information have no effect on the aircraft’s profile within the current and any later controlling ATSU (C-ATSU) airspaces, prior to actual transfer of control to the D-ATSU.  If established, direct contact with the C-ATSU is maintained by the aircrew, and is unaffected by communication with the D-ATSU.  The need for the C-ATSU to be made aware of the information obtained from the D-ATSU will be governed by local procedures.

### The Downstream Clearance (DSC) Service provides assistance for requesting and obtaining D-ATSU clearances or information, using air/ground data link.

### The DSC service can only be initiated by the aircrew.

### The DSC is a service within the CPDLC application.  Unless specifically indicated, DSC uses all generic CPDLC functionality, including message handling as well as operational and performance requirements.

## Expected Benefits, Anticipated Constraints, and Associated Human Factors

### Expected Benefits

#### The expected benefits of DSC include:

- reduction of voice channel load (voice frequency congestion);

- reduction of R/T workload for both aircrew and controller;

- added aircrew flexibility for requesting and receiving clearances or information;

- better, more flexible pacing of communication tasks for aircrew;

- improved support for aircrew flight planning and task management;

- provision of an enhanced planning capability for the D-ATSU.

### Anticipated Constraints

#### Operational responsibility

##### Procedures and system support must be put in place to permit aircrew to have controlled data link access to an ATSU other than the C-ATSU.  This capability will be based on the following two levels of ATSU operational access to controller/aircrew data link exchanges:

- Current ATSU (C-ATSU):  As the ATSU currently responsible for control of the aircraft, authority to exchange all CPDLC messages with that aircraft;

- Downstream ATSU (D-ATSU):  As an ATSU which is expected to be responsible for control of the aircraft in the future, but which is not yet responsible for its control, authority to exchange a limited sub-set of data link messages with that aircraft, none of which could be construed by either the aircrew or the avionics as affecting the immediate control of the aircraft.

- The C-ATSU and the D-ATSU must not be the same Data Authority.

#### Operational Procedures and System Support

##### D-ATSU operational access to controller/aircrew exchanges will be strictly controlled through operational procedures.  The following operational principles will be observed in relation to D-ATSU operational access to controller/aircrew data link exchanges:

  - There can be only one Downstream clearance link at any one time.

  - To maintain aircrew situational awareness and communication access security, avionics will provide aircrew with the ability to:

    - Maintain control and awareness over C-ATSU access, including transfer of that access from one ATSU to another.  According to procedures, this transfer will be conducted in accordance with C-ATSU directives.

    - Maintain control and awareness over D-ATSU access.  D-ATSU controller/aircrew data link access will only be established via aircrew request, and will concern exchanges specifically initiated by the aircrew (i.e., only the D-ATSU authorized clearances or information requested by the aircrew will be accepted).

    - Clearly and unambiguously differentiate between messages from the C-ATSU and messages from a D-ATSU.

  - Any data link message sent via a DSC link must be readily identifiable as such on the display of both the sender and the recipient of the message.

  - D-ATSU controller/aircrew message use should be controlled through ICAO regional operational procedures and published in AIPs for each case

  - There will be procedures that prevent the aircrew from executing a clearance received via a Downstream clearance link until the aircraft enters the airspace of the control authority from which the Downstream clearance was received.  If the information an airborne air user receives via a Downstream clearance link requires action while still in the airspace of the current control authority, the clearance for such action must be obtained from that current control authority.

  - The ground system must have the ability to reject any request for DSC.

### Associated Human Factors

#### See section Operational Procedures and System Support above.

#### Message elements available for this service should be restricted by the end user systems in accordance with the Service Description.

## Operating Method Without Data Link

### In the current operational ATC environment, aircrew contact a D-ATSU, while maintaining communication contact with the C-ATSU, in order to obtain a clearance or information concerning the aircraft’s future flight profile.  Such contacts are always initiated by the aircrew, and can be conducted either via a second voice channel or, where suitable facilities are available, via air/ground data link (i.e., Oceanic Clearance Message (OCM)).

| Step | Operating Method Without Data Link | Operating Method With Data Link |
| --- | --- | --- |
| 1 | The aircrew contacts the D-ATSU to request the clearance or information related to D-ATSU airspace, advising any applicable preferences. | The aircrew contacts the D-ATSU via data link to request the clearance or information related to D-ATSU airspace, advising any applicable preferences. |
| 2 | The D-ATSU provides the aircrew with the requested clearance or information, according to the method used for the request. | The D-ATSU provides the aircrew with the requested clearance or information, via data link. |
| 3 | The aircrew acknowledges the clearance or information according to the method used for the clearance or information. | The aircrew acknowledges the clearance or information via data link. |

Table 8-1:  Operating Method With and Without Data Link

## Operating Method With Data Link

### Normal Mode

#### Service Description

##### DSC will be available in all flight phases.  Use of the Service will be restricted to aircrew initiated requests, and the directly associated responses.

##### The DSC operating method conforms to the existing operating method.  The normal sequence of events is:

  - The aircrew triggers a DSC request to the D-ATSU.

  - If a Logical Acknowledgment (LACK) is required in accordance with local procedures, the D-ATSU transmits a LACK to the aircraft.

  - Based on flight related data and data received in the DSC Request, the D-ATSU composes the requested DSC or information and transmits it to the aircraft.

  - If a LACK is required by the D-ATSU in accordance with local procedures, the aircraft transmits a LACK to the D-ATSU.

  - The aircrew verifies the D-ATSU Response containing the proposed DSC and:

    - If they can accept and comply with the operational contents, without any change being required to its current clearance, the aircrew transmits a WILCO;

    - If the proposed DSC affects the current cleared trajectory, the aircrew transmits a STANDBY.  The aircrew will inform the C-ATSU requesting the necessary coordination.  When this is confirmed the aircrew transmits a WILCO to the D-ATSU.

    - If the aircrew is unable to accept the operational contents, they will transmit an UNABLE.

### Initiation Conditions

#### The DSC Service is invoked exclusively on aircrew initiation of the request to the D-ATSU.

#### The DSC will be completed prior to the aircraft entering the D-ATSU airspace

### Termination conditions

#### The DSC link is normally terminated by the aircrew after completion of the final response to a receipt of a DSC.

#### DSC link termination procedures will be initiated if the D-ATSU becomes the C-ATSU.  In these circumstances, only closure and termination messages can be sent over the DSC link.

### Sequence of Services

#### The DSC service can operate independently of any other service.

### Additional Guidelines

#### The DSC Service requires flight plan information at the D-ATSU.

#### Table 8-2 indicates the message elements from the CPDLC message set (Part IV, Chapter 3, Appendix A) which must be prohibited from inclusion in isolation in any DSC related data link dialogue.  They may be concatenated with appropriate DSC message elements with caution.

| Message Type | Table Number | Message Element Number |
| --- | --- | --- |
| Uplink | A2: Vertical Clearances (Uplink) | 19, 20, 23, 26, 27, 28, 29, 30, 192, 209, 31, 32, 34, 36, 37, 38, 39, 40, 41 |
|  | A4: Lateral Offsets (Uplink) | 67, 68, 69, 72 |
|  | A5: Route Modifications (Uplink) | 74, 75, 33, 82, 94, 95, 215, 190, 96, 97, 221, 98 |
|  | A6: Speed Changes (Uplink) | 106, 107, 108, 109, 110, 111, 112, 113, 114, 115, 116, 189, 223, 222 |
|  | A7: Contact/Monitor Surveillance Requests (Uplink) | 123, 124, 125, 126, 179 |
|  | A11: System Management Messages (Uplink) | 154, 155, 156, 210, 193, 160, 161, 191 |
|  | A12: Additional Messages (Uplink) | 164, 230, 176, 177 |
| Downlink | A14: Vertical Requests (Downlink) | 6, 7, 8, 9, 10, 69 |
|  | A15: Lateral Offset Requests (Downlink) | 15 |
|  | A18: Route Modification Requests (Downlink) | 26, 27, 70, 71 |
|  | A22: System Management Messages (Downlink) | 63,99,107 |
|  | A23: Additional Messages | 74, 103 |

Table 8-2:  Prohibited Message Elements in Isolation for DSC

## Time Sequence Diagram

### Figure 8-1 shows the normal sequence of messages in the DSC.

Figure 8-1: Time Sequence Diagram For Downstream Clearance Request

## Information Exchanges

### Applicable CPDLC Message Elements

#### Although the entire CPDLC message set will be available for DSC, it is recommended that regional agreements be established to govern the use of the CPDLC message elements for DSC.  It is expected that the primary use will be for the message elements required to effect requests and deliveries of oceanic clearances.  Other uses of DSC may be identified.

-- END --