PART VII

PART VII - AUTOMATIC DEPENDENT SURVEILLANCE-BROADCAST (ADS-B)

AUTOMATIC  DEPENDENT  SURVEILLANCE BROADCAST  (ADS-B)

# APPLICATION Overview

## Introduction

### ADS-B is a surveillance application which allows the transmission of parameters, such as position, ground track and ground speed, via a broadcast mode data link for use by any air and/or ground users requiring it.  This capability will permit enhanced airborne and ground situational awareness to provide for specific surveillance functions and cooperative pilot-controller and pilot-pilot ATM.

### The ADS-B application is not limited to the traditional roles associated with ground-based radar systems.  ADS-B will provide opportunities for new functionality both onboard the aircraft and within the ground ATC automation systems.  Depending on the implementation, ADS-B may encompass both air-ground and air-air surveillance functionality, as well as applications between and among aircraft on the ground and ground vehicles.   ADS-B will have many benefits in extending the range beyond that of secondary surveillance radar, particularly in airport surface and low altitude airspace, and in air-to-air situational awareness.

### Each ADS-B capable aircraft will periodically broadcast its position and other required data provided by the onboard navigation system.  Any user, either airborne or ground-based, within range of this broadcast may choose to receive and process this information.  The aircraft originating the broadcast need have no knowledge of what system(s) is receiving its broadcast.  Because broadcast data might be received by the ground station at a rate in excess of the requirements of the ATC system, some filtering and/or tracking may be necessary.

### The ADS-B application supports improved use of airspace, reduced ceiling/visibility restrictions, improved surface surveillance, and enhanced safety.  ADS-B equipage may be extended to vehicles on the airport surface movement area, uncharted obstacles not identified by a current NOTAM, and non-powered airborne vehicles or obstacles.

### For the purpose of this document, the following definitions have been adopted:

- ADS-B emitter is an aircraft, whether on the ground or airborne, a ground vehicle, or an uncharted obstacle, which is equipped with ADS-B transmitter, and continually broadcasts its identification, position, and other defined parameters via a data link

- ADS-B receiver is a ground-based user, such as an ATSU, which receives and process ADS-B messages or an aircraft or vehicle either in the air or on the ground which receives and processes ADS-B data.

Note:  ADS-B receivers may also be ADS-B emitters.

- Air-ground operation is a transmission from an ADS-B emitter used by a ground receiver for the purpose of surveillance and monitoring.

- Air to Air operation is a transmission from an ADS-B emitter used by another ADS-B air receiver.

### The requirements and performance characteristics for ADS-B information may differ between airborne aircraft and aircraft on the airport surface.  It should be noted that there are other broadcast data link messages that may be exchanged that are outside the scope of ADS-B.  Because of its broadcast nature, it is not possible to ensure the security of ADS-B messages.

## Scope

### ADS-B consists of several services, including those designed for both air-ground and air-air use.  This version of the manual only addresses ATC Surveillance.  Other potential services using ADS-B derived data are being investigated including:

- Airborne situational awareness,

- Conflict detection (both airborne and ground based),

- Airborne Separation Assurance System (ASAS),

- ATC conformance monitoring, and

- ADS-B lighting control and operation.

### It is anticipated that other services will be added in future.

### Many other forms of broadcast data may become available, including flight information services (e.g., NOTAMS and weather information).  These services are inherently different from ADS-B in that they require sources of data external to the aircraft or broadcasting unit, broadcast information other than encompassed in ADS-B, and independently defined performance requirements.  These types of broadcast services are outside the scope of this document.

PART VII - AUTOMATIC DEPENDENT SURVEILLANCE-BROADCAST (ADS-B)

# General Requirements of the ADS-B system

## Performance Requirements

### Systems developed to support ADS-B will be capable of meeting the communications performance appropriate for the service provided.

## Message Handling

### The ADS-B application requires:

- message delivery at a rate appropriate to the service,

- message generation and transmission in a time ordered sequence, and

- message delivery in the order sent.

## Time Requirements

### Time used in the ADS-B service shall be accurate to within 1 second of UTC.

### Time stamping shall be available for all messages.  The time stamp shall consist of date (YYMMDD) and time (HHMMSS).

## Priority

### When messages are forwarded for surveillance purposes, the priority will be “normal priority flight safety messages” as determined by the ATN Internet Protocol Priority categorization.

## Quality Of Service (QOS)

### Where messages are forwarded for surveillance purposes, ATS providers shall specify the required QOS.

PART VII - AUTOMATIC DEPENDENT SURVEILLANCE-BROADCAST (ADS-B)

# Functional Capabilities

## Broadcast Requirements

### Each ADS-B emitter will periodically broadcast its position and other required data.  Any receiver within range of the broadcast may receive and process the information.  The emitter originating the broadcast need have no knowledge of what system(s) is receiving its broadcast.

## Message Elements

### The following message elements shall comprise the minimum set of information to be transmitted by an ADS-B emitter:

- Airframe Identifier,

- Latitude,

- Longitude,

- Altitude,

- Time,

- Figure of Merit (FOM),

- Aircraft Identification, and

- Emitter Category.

### System design should allow for excess capacity as future services may require additional elements.  Potential message elements may include:

- Ground Vector:  containing Ground Track, Ground Speed and Vertical Rate,

- Air Vector:  containing Heading, IAS or Mach, and Vertical Rate,

- Short Term Intent:  containing Next Waypoint and Target Altitude,

- Rate of Turn, and

- Aircraft Type.

## ADS-B Data Glossary

### Appendix A contains the ADS-B Data Glossary

## ADS-B Variables Ranges and Resolutions

### Appendix B contains the ADS-B messages variables ranges and resolutions

PART VII - AUTOMATIC DEPENDENT SURVEILLANCE-BROADCAST

CHAPTER 3 APPENDIX A

ADS-B MESSAGE DATA GLOSSARY

3A.1	ADS-B Data Glossary

3A.1.1	The following data are used as the ADS-B message element variables, and are shown here in alphabetical order.

Airframe Identifier:  Refers to the unique 24-bit address specific to an airframe, vehicle or other emitter.

Aircraft Identification:  Field 7 of the ICAO flight plan.

Aircraft Type:  Refers to the particular classification of the aircraft, as defined by ICAO.

Air Speed:  Provides air speed as a choice of the following:  Mach, IAS, or Mach and IAS.

Air Vector:  Provides the air vector as a sequence of:  Heading, Air Speed, and Vertical Rate.

Altitude:  specifies altitude.

Distance:  specifies distance.

Emitter Category:  Refers to the characteristics of the originating ADS-B unit.  It should be listed as one of the following:

- Small (<12,500 lbs or 5,682 kg)

- Medium (12,500 to 75,000 lbs or 5,682 to 34,091 kg)

- Large (75,000 to 190,000 lbs or 34,091 to 83,364 kg)

- Extra Large (190,000 to 300,000 lbs or 83,364 to 136,364 kg)

- Heavy (>300,000 lbs or 136364 kg)

- High Performance (larger than 5G acceleration capability)

- Rotocraft

- Glider/Sailplane

- Lighter-than-air

- Parachutist/skydiver

- Surface Vehicle

- Fixed ground or tethered obstruction

- Unmanned Aerial Vehicle

- Space/Transatmospheric vehicle

- Ultralight/Handglider/Paraglider

Note:  16-24 reserved for future work.

ETA:  Estimated time of arrival at a waypoint.

Figure of Merit:  Indicates the figure of merit of the current ADS data.  The information consists of the Position Accuracy and indications 1) whether or not multiple navigational units are operating, and 2) whether or not ACAS is available.

Ground Speed:  Provides ground speed.

Ground Vector:  Provides the ground vector of an aircraft provided as a sequence of    Track, Ground Speed and Vertical Rate.

IAS:  Indicated air speed.

Intermediate Intent:  Set of points between current position and the time indicated in the Short Term Intent.  Consists of a sequence of the following:  Distance, Track, Altitude and Projected Time.

Latitude:  Latitude in degrees, minutes, and seconds.

Longitude:  Longitude in degrees, minutes, and seconds.

Mach:  Air speed given as a Mach number.

Mach and IAS:  Air speed provided as both Mach and indicated air speed.

Next Waypoint:  Specifies the next waypoint in the avionics.

Position Accuracy:  An indication of the navigational accuracy.

Projected Time:  Predicted time at a particular point.

Short Term Intent:  A sequence of:  Latitude, Longitude, ETA, and Intermediate Intent (optional) data structures.

Time Stamp:  Date and time that an ADS-B message is generated.

Track:  Provides track angle in degrees.

Turn Rate:  Refers to the aircraft rate of turn.

Vertical Rate:  Rate of climb/descent (climb positive, descent negative)

PART VII- AUTOMATIC DEPENDENT SURVEILLANCE - BROADCAST

CHAPTER 3 APPENDIX B

ADS-B VARIABLES RANGE AND RESOLUTION

3B.1	ADS-B Variables Range and Resolution

3B.1.1 Table 3B-provides the required range and resolution for the message variables used in the ADS-B application.

| Category | Variables/Parameters | Unit | Range | Resolution |
| --- | --- | --- | --- | --- |
| Aircraft Identification |  | IA5 string | 2-8 | N/A |
| Airframe Identifier |  | Bit string | 24 | N/A |
| Altitude | Altitude Units | alt-unit | -10 to +10 000 | 1 |
| Airspeed | Mach | Mach number | .5 - 4.0 | 0.001 |
|  | IAS | Speed-unit | -110 to + 4200 | 1 |
| Date | Year | year of century | 0-99 | 1 |
|  | Month | month of year | 1 - 12 | 1 |
|  | Day | day of month | 1 - 31 | 1 |
| Emitter Category |  | Integer | 1-24 | 1 |
| FOM |  | Integer | 0-7 | 1 |
| Ground Speed |  | Speed-unit | -110 to + 4200 | 1 |
| Heading |  | Degrees | 0.1 - 360 | 0.1 |
| Latitude | Latitude degrees | degrees | ± 90 | 1 |
|  | Latitude minutes | minutes | 0 - 59 | 1 |
|  | Latitude seconds | seconds | 0 - 59.9 | 0.1 |
| Longitude | Longitude degrees | degrees | ± 180 | 1 |
|  | Longitude minutes | minutes | 0 - 59 | 1 |
|  | Longitude seconds | seconds | 0 - 59.9 | 0.1 |
| Time | Time hours | hours of day | 0 - 2300 | 1 |
|  | Time minutes | minutes | 0 - 59 | 1 |
|  | Time seconds | seconds | 0 - 59 | 1 |
| Track | Track Angle | Degrees | 0.1 - 360 | 0.1 |
| Turn Rate |  | Degrees/minute | 0 - 360 | 5 |
| Vertical Rate |  |  |  |  |
|  |  | Altitude Units/Minute | ±3310 - |  |

Table 3B-1:  ADS-B Variables Range and Resolution

PART VII - AUTOMATIC DEPENDENT SURVEILLANCE-BROADCAST (ADS-B)

# Air Traffic Surveillance

## Scope and Objective

### In the future, enhanced navigation and communication capabilities will permit the use of aircraft derived position information to be transmitted to the ground.  These data will be used as surveillance data to supplement ground-based ATC Surveillance, and may also be used as a sole means of surveillance data, particularly in areas in which there is no radar coverage.

### This chapter describes the use and requirements of ADS-B for ATC Surveillance in en route, terminal and airport environments.

## Expected Benefits, Anticipated Constraints and Associated Human Factors

### Expected Benefits

#### Once the ground infrastructure is deployed, the increased accuracy, the update rate and additional parameters available with ADS-B should result in the following benefits:

- Improved services in airspace not having radar coverage;

- Improved airspace utilization;

- Improved conflict prediction and detection;

- Improved airport surface movement, guidance, and control;

- Improved runway incursion prevention; and

- Improved automated conformance monitoring.

#### Some benefits can be realized even before full aircraft equipage.

### Anticipated Constraints

#### In order to provide surveillance based exclusively on ADS-B information, all aircraft operating in the airspace must be ADS-B equipped.

#### ATS providers will require an independent means of validation of the ADS-B position information.

## Operating Environment Without ADS-B

### The surveillance environment comprises primary/secondary radar, with differing update rates, which support the provision of specified separation minima.

### The nature and expense of these radar limit the area of coverage where these surveillance services can be provided.  As a result, large separation minima, and procedural methods of air traffic control are used where no surveillance coverage is available.

## Operating Environment With ADS-B

### ADS-B will enhance ATC surveillance in the following ways:

- In a mixed ADS-B/radar surveillance environment, ADS-B data will complement or supplement radar data.

- ADS-B will extend surveillance services into non-radar airspace, such as low-altitude airspace, remote airspace and coastal waters.

### At such time that an airspace is determined to be fully populated with aircraft equipped with ADS-B, ATS providers may evaluate the necessity of replacing or maintaining other ground-based surveillance equipment.

### As ADS-B is implemented regionally to different initial levels of capability with mixed aircraft equipage, ATS providers must ensure efficient levels of service to all airspace users.

### General Operational Requirements

#### To provide a basis for the design of ADS-B systems for ATC surveillance, the following general operational requirements have been determined.

- An ATSU will be capable of knowing that an aircraft is ADS-B equipped.

- All aircraft operating in an ADS-B airspace will broadcast as required by the ATS provider.

- The ground system will receive, process and display the ADS-B information.

- Procedures and/or systems must be in place to validate the ADS-B information.

#### In addition the parameter requirements in Part I of this manual, a summary of ATC specific performance requirements using ADS-B are presented in table 4-1.

| Parameter | Operational Domain |  |  |
| --- | --- | --- | --- |
|  | En Route | Terminal | Airport Surface/ Vicinity |
| Max. Update Period | 10 seconds | 5 seconds | 1 second* |
| Probability of update within period | 98% | 98% | 98% |
| Position Accuracy | 350 meters | 150 meters | 3 meters |
| Instantaneous number of aircraft to be supported per ATSU | 1250 | 750 in a 60 NM radius | 100 in motion and 150 stationary |
| Message Latency | 2 seconds | 1 second | 1 second |

Table 4-1:  ATC Specific Surveillance Requirements Using ADS-B

Note 1:  A less frequent update rate may be permissible for stationary emitters.

### ATC Procedures

#### ATC Procedures for the use of ADS-B will be dependent on the phase of flight, and the communications infrastructure of the ATS provider.  Procedures for the use of ADS (Part III, Chapter 6) and the procedures for the use of Controller-Pilot Data Link Communications (Part IV, Chapter 6) may be applicable, along with procedures given in the Procedures for Air Navigation/Rules of the Air and Air Traffic Control (Doc 4444) and Regional Supplementary Procedures (Doc 7030).  It is anticipated that specific ATC procedures will be developed as experience is gained with the systems and as appropriate separation minima are developed for global use.

#### The probability exists that errors may be input into the aircraft navigation system prior to departure.  Since ADS-B is by definition dependent on the on-board navigation system, procedures will be required to ensure pre-departure conformance checking in order to correct these errors.

#### ATS providers should ensure that the number of separation standards applied in a given airspace are kept to a minimum.

#### In a mixed environment, the source of surveillance data should be readily apparent to the controller.

#### In a mixed environment, procedures must be in place to ensure that all sources of the display refresh rate will be synchronous regardless of the source of surveillance information data.

### Exception Handling

#### The ADS-B application will be capable of providing a warning to pilot and controller whenever the navigation accuracy is degraded below that required to operate in the airspace as this will affect the application of separation.

#### Back-up procedures should be developed for ADS-B complete and partial system failure.

-- END --