# AIDC-USER REQUIREMENTS

- ¾ This section identifies the requirements imposed upon the AIDC-User by the AIDC-AE.

## Inter-Operability

### To achieve inter-operability for the implementation of AIDC between ATS units, or ATS regions on a global basis, the following requirements shall be satisfied:

- a common AIDC message set and associated services is agreed upon by the ATS units involved;

- agreement, by the ATS units involved, as to what flight related conditions dictate the invocation of the AIDC services; and

- agreement as to the timing associated with the use of the AIDC services.

## Message Handling

### Mesage Priorities

#### In cases where message queueing is implemented by the AIDC-User, an application specific priority scheme shall be implemented.

##### Under this scheme the AIDC messages received shall be assigned one of the following priorities:

- Normal;

- Urgent; or

- Distress.

##### General freetext messages shall be assigned a priority of “Normal”.

##### Emergency freetext messages shall be assigned the priority of “Distress”.

##### Surveillance data transfer messages shall be assigned the priority of “Urgent”.

##### All other AIDC messages shall be assigned the priority “Normal”.

##### The AIDC-User shall process messages with a priority of Distress first, followed by messages with a priority of Urgent and then messages with a priority of Normal.

## Operational Timers

### The AIDC-User shall manage a number of timers associated with the sending of AIDC messages.  These timers are descibed below:

- a Confirmation Timer, used to detect the failure of the AIDC-User to receive the user confirmation for a previously sent message;

- a Response Timer, used to detect the failure of the AIDC-User to receive the appropriate message in response to a previously sent coordination or transfer message;

- a Monitor Timer, used to detect the failure of the AIDC-User to receive another expected AIDC message; and

- a Standby Timer, used to extend the time before a response to a coordination message is expected.

## AIDC-AE Specific Requirements

### Management of AIDC-AE Instantiations

#### As each instantiation of the AIDC-AE is on a flight by flight basis, the AIDC-User shall manage these instantiations by some local means.

### The User-confirmation Service

#### Upon the receipt of an AIDC message, the AIDC-User shall validate the semantics of the message and use the User-confirmation service to indicate to the peer AIDC-User whether the message has been accepted or rejected.

#### In order for the AIDC-AE to correlate user service primitives with the User-confirmation primitives, the AIDC-User shall generate and manage a set of unique identifiers.

#### The identifiers received by an AIDC-User in indication service primitives, shall be used as the Referenced Number parameter of the User-confirmation request primitive.

### Coordination Messages

- ¾ The Coordinate-start service is used for the initial flight coordination and the updating of the coordination conditions for a flight.  The passing of either the CoordinateInitial or CoordinateUpdate message when using the service is under user control and procedures.

#### If for some reason the D-ATSU AIDC-User cannot accept the type of message received, it shall signal that condition with a negative User-confirmation

## Error Processing Requirements

### In the event of information input by the user being incompatible with that able to be processed by the system, the user shall be notified.

### Associated with this primitive is a time interval that shall define the time allowed between the sending of an AIDC primitive and the receipt of the User-confirmation primitive.