# THE AIDC-AE ABSTRACT SERVICE

## Introduction

- ¾ This section defines the abstract service interface used by an AIDC-User to access the AIDC-AE services.

## Standard Parameters

- The following service primitive parameters are defined here rather than repeated for each service definition.

### Called ICAO Facility Designator

#### The Called ICAO Facility Designator parameter shall be provided by the AIDC-User.

#### The Called ICAO Facility Designator parameter shall be the called ATS system’s eight letter ICAO facility designator.

### The Called ICAO Facility Designator parameter shall conform to the ASN.1 abstract syntax of ICAOFacilityDesignator.

### Calling ICAO Facility Designator

#### The Calling ICAO Facility Designator parameter shall be provided by the AIDC-AE.

#### The Calling ICAO Facility Designator parameter shall be the calling ATS system’s eight letter ICAO facility designator.

### The Calling ICAO Facility Designator parameter shall conform to the ASN.1 abstract syntax of ICAOFacilityDesignator.

### Message Number

#### The Message Number parameter shall be provided by the AIDC-User except when invoking the User-abort or User-confirmation request primitives.

#### The Message Number parameter value shall consist of a unique identifier for reference in the User-confirmation.

### The Message Number parameter shall conform to the ASN.1 abstract syntax of MessageNumber.

## AIDC User Service

### AIDC-User Services

### The AIDC service shall exhibit external behaviour consistent with having implemented an AIDC-AE with the following abstract services, making them available to the AIDC-User:

- User-confirmation service as defined in section 3.4.

- Notify service as defined in section 3.5.1

- Coordinate-start service as defined in section 3.6.4

- Coordinate-end service as defined in section 3.6.5

- Coordinate-negotiate service as defined in section 3.6.6

- Coordinate-standby service as defined in section 3.6.7

- Transfer-initiate service as defined in section 3.7.3

- Transfer-request service as defined in section 3.7.4

- Transfer-conditions-proposal service as defined in section 3.7.4

- Transfer-conditions-accept service as defined in section 3.7.6

- Transfer-control service as defined in section 3.7.7

- Transfer-communication service as defined in section 3.7.8

- Transfer-communication-assume service as defined in section 3.7.9

- Info-transfer service as defined in section 3.8.1

- End service as defined in section 3.9.1

- User-abort service as defined in section 3.9.2

- Provider-abort service as defined in section 3.9.3

### Service Primitive Sequencing

#### The AIDC-User Service shall consist of three regimes, one asynchronous service, and a set of termination services.

#### The three regimes shall occur in a strict sequence from first to last:

- Notifying regime,

- Coordinating regime, and

- Transferring regime.

#### The Coordinating regime shall be allowed to be re-entered at the end of the Transferring regime.

#### Recommendation.¾ The Notifying regime should consist of the Notify service.

#### Recommendation.¾ The Coordinating regime should consist of the:

- Coordinate-start service,

- Coordinate-negotiate service,

- Coordinate-standby service, and

- Coordinate-end service.

#### Recommendations.¾ The Transferring regime should consist of the:

- Transfer-initiate service,

- Transfer-conditions-proposal service,

- Transfer-conditions-accept service,

- Transfer-request service,

- Transfer-control service,

- Transfer-communication service, and

- Transfer-communication-assume service.

#### The sequencing of service primitives is shown in Table A-1 of Appendices A to these SARPs.

## User Confirmation Service

### Upon the receipt of any primitive indication containing an Information parameter (e.g. Notify Information, Coordinate Start Information), the AIDC-User shall validate the value of this parameter and respond with a User-confirmation request primitive.

### The User-confirmation service shall be an unconfirmed service.

### Table 3-1 specifies the parameters that shall be passed when the primitives of the User-confirmation service are invoked.

Table 3-1: User-confirmation Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Result | M | M(=) |
| Reason | U | C(=) |
| Referenced Number | M | M(=) |

### Result

#### The Result parameter shall be provided by the AIDC-User.

#### The Result parameter shall conform to the ASN.1 abstract syntax of Result.

### Reason

#### Recommendation.¾ The Reason parameter should be provided by the AIDC-User when the Response parameter has the abstract value of “rejected”.

#### The Reason parameter shall conform to the ASN.1 abstract syntax ApplicationErrorData.

### Referenced Number

#### The Referenced Number parameter shall be provided by the AIDC-User.

#### The Referenced Number parameter shall contain the Message Number of the message being confirmed.

#### The Referenced Number parameter shall conform to the ASN.1 abstract syntax of ReferencedNumber.

## Notifying Regime

### Notify Service

- ¾ The purpose of the Notify service is to allow a C-ATSU to update the information a D-ATSU maintains on a flight that is expected to enter its area of interest at some future time.

### Service Primitives.

#### The Notify service shall be an unconfirmed service.

#### Table 3-2 specifies the parameters that shall be passed when the primitives of the Notify service are invoked.

Table 3-2: Notify Service Primitives Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Called ICAO Facility Designator | M |  |
| Calling ICAO Facility Designator |  | M |
| Notification Information | M | M(=) |
| Message Number | M | M(=) |

#### Notification Information

##### The Notification Information parameter shall be provided by the AIDC-User when invoking the Notify request primitive.

##### The Notification Information parameter shall conform to the ASN.1 abstract syntax Notify.

## Coordinating Regime

### The Coordinating regime shall begin with invocation of a Coordinate-start primitive.

### The Coordinating regime shall end with the invocation of a Coordinate-end primitive.

### Upon entering the Coordinating regime, further use of the Notify service shall be prohibited.

### Coordinate-start Service

- ¾ The purpose of the Coordinate-start service is to allow an ATSU1 to begin the coordination of, or update, the conditions of transfer of a flight with an ATSU2.

- ¾ If rejected, any currently agreed coordination conditions remain in affect.

#### Service Primitives.

##### The Coordinate-start service shall be an unconfirmed service.

##### Table 3-3 specifies the parameters that shall be passed when the primitives of the Coordinate-start service are invoked.

Table 3-3: Coordinate-start Service primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Called ICAO Facility Designator | M |  |
| Calling ICAO Facility Designator |  | M |
| Coordinate Start Information | M | M(=) |
| Message Number | M | M(=) |

##### Coordinate Start Information

###### The Coordinate Start Information parameter shall be provided by the AIDC-User.

###### The Coordinate Start Information parameter shall conform to the ASN.1 abstract syntax of CoordinateStart.

### Coordinate-end Service

- ¾ The purpose of the Coordinate-end service is to allow an ATSU1 to notify an ATSU2 that the ATSU1 either accepts or rejects the coordination conditions proposed.

- ¾ The successful completion of this service ends the coordinating regime for a flight.

#### Service Primitives.

##### The Coordinate-end service shall be an unconfirmed service.

##### The Coordinate-end service primitives shall only be invoked while in the Coordinating regime.

##### Table 3-4 specifies the parameters that shall be passed when the primitives of the Coordinate-end service are invoked.

Table 3-4: Coordinate-end Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Coordinate End Information | M | M(=) |
| Result | M |  |
| Message Number | M | M(=) |

##### Coordinate End Information

###### The Coordinate End Information parameter shall be provided by the AIDC-User.

###### The Coordinate End Information parameter shall conform to the ASN.1 abstract syntax of CoordinateEnd.

##### Result

###### The Result parameter shall be provided by the AIDC-User.

#### The Result parameter shall conform to the ASN.1 abstract syntax of Result.

### Coordinate-negotiate Service

- ¾ The purpose of the Coordinate-negotiate service is to allow an ATSU1 to negotiate modifications to a flight's existing coordination conditions with an ATSU2.

- ¾ If rejected, any currently agreed coordination conditions remain in affect.

#### Service Primitives

#### The Coordinate-negotiate service shall be an unconfirmed service.

##### The Coordinate-negotiate service primitives shall only be invoked while in the Coordinating regime.

##### Table 3-5 specifies the parameters that shall be passed when the primitives of the Coordinate-negotiate service are invoked.

Table 3-5: Coordinate-negotiate Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Coordinate Negotiate Information | M | M(=) |
| Message Number | M | M(=) |

##### Coordinate Negotiate Information

###### The Coordinate Negotiate Information parameter shall be provided by the AIDC-User.

###### The Coordinate Negotiate Information parameter shall conform to the ASN.1 abstract syntax CoordinateNegotiate.

### Coordinate-standby Service

- ¾ The purpose of the Coordinate-standby service is to allow an ATSU1 to notify an ATSU2 that the coordination dialogue between the ATSUs is being temporarily suspended.

- ¾ Each invocation of this service effectively extends, by a defined amount, the time before a user response to a coordinate service indication is given.

#### Service Primitives

##### The Coordinate-standby service shall be an unconfirmed service.

##### The Coordinate-standby service primitives shall only be invoked while in the Coordinating regime.

##### Table 3-6 specifies the parameters that shall be passed when the primitives of the Coordinate-standby service are invoked.

Table 3-6: Coordinate-standby Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Coordinate Standby Information | M | M(=) |
| Message Number | M | M(=) |

##### Coordinate Standby Information

###### The Coordinate Standby Information parameter shall be provided by the AIDC-User.

###### The Coordinate Standby Information parameter shall conform to the ASN.1 abstract syntax CoordinateStandby.

## Transferring regime

### The Transferring regime shall be entered after the completion of the Coordinating regime.

### The Transferring regime shall begin with the invocation of any of the following:

- the Transfer-initiate service;

- the Transfer-request service; or

- the Transfer-control service.

### Transfer-initiate Service

- ¾ The purpose of the Transfer-initiate service is to allow a C-ATSU to initiate the transfer phase for a flight by passing executive control information to an R-ATSU.

#### Service Primitives.

##### The Transfer-initiate service shall be an unconfirmed service.

##### Table 3-7 specifies the parameters that shall be passed when the primitives of the Transfer-initiate service are invoked.

Table 3-7: Transfer-initiate Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Transfer Initiate Information | M | M(=) |
| Message Number | M | M(=) |

##### Transfer Initiate Information

###### The Transfer Initiate Information parameter shall be provided by the AIDC-User.

###### The Transfer Initiate Information parameter shall conform to the ASN.1 abstract syntax TransferInitiate.

### Transfer-request Service

- ¾ The purpose of the Transfer-request service is to allow an R-ATSU to request control & communications authority for a flight from a C-ATSU.

#### Service Primitives.

##### The Transfer-request service shall be an unconfirmed service.

##### Table 3-8 specifies the parameters that shall be passed when the primitives of the Transfer-request service are invoked.

Table 3-8: Transfer-request Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Transfer Request Information | M | M(=) |
| Message Number | M | M(=) |

##### Transfer Request Information

###### The Transfer Request Information parameter shall be provided by the AIDC-User.

###### The Transfer Request Information parameter shall conform to the ASN.1 abstract syntax TransferRequest.

### Transfer-conditions-proposal Service

- ¾ The purpose of the Transfer-conditions-proposal service is to allow a C-ATSU to propose the conditions under which executive control authority for a flight can be given to an R-ATSU.

#### Service Primitives.

##### The Transfer-conditions-proposal service shall be an unconfirmed service.

##### The Transfer-conditions-proposal service primitives shall only be invoked after the successful completion of the Transfer-initiate service.

##### Table 3-9 specifies the parameters that shall be passed when the primitives of the Transfer-conditions-proposal service are invoked.

Table 3-9: Transfer-conditions-proposal Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Transfer Conditions Proposal Information | M | M(=) |
| Message Number | M | M(=) |

##### Transfer Conditions Proposal Information

###### The Transfer Conditions Proposal Information parameter is provided by the AIDC-User.

###### The Transfer Conditions Proposal Information parameter shall conform to the ASN.1 abstract syntax TransferConditionsProposal.

### Transfer-conditions-accept Service

- ¾ The purpose of the Transfer-conditions-accept service is to allow an R-ATSU to indicate that it is willing to accept executive control conditions proposed for a flight by the C-ATSU.

- ¾ This service, when used, is only used in response to the Transfer-conditions-proposal service.

#### Service Primitives.

##### The Transfer-conditions-accept service shall be an unconfirmed service.

##### The Transfer-conditions-accept service primitives shall only be invoked after the successful completion of the Transfer-conditions-proposal service.

##### Table 3-10 specifies the parameters that shall be passed when the primitives of the Transfer-conditions-accept service are invoked.

Table 3-10: Transfer-conditions-accept Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Transfer Conditions Accept Information | M | M(=) |
| Message Number | M | M(=) |

##### Transfer Conditions Accept Information

###### The Transfer Conditions Accept Information parameter shall be provided by the AIDC-User.

###### The Transfer Conditions Accept Information parameter shall conform to the ASN.1 abstract syntax TransferConditionsAccept.

### Transfer-control Service

- ¾ The purpose of the Transfer-control service is to allow a C-ATSU to indicate that it wants to relinquish control authority for a flight to an R-ATSU.

#### Service Primitives.

##### The Transfer-control service shall be a confirmed service.

##### Table 3-11 specifies the parameters that shall be passed when the primitives of the Transfer-control service are invoked.

Table 3-11: Transfer-control Service Primitive Parameters

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| Transfer Control Information | M | M(=) | M | M(=) |
| Result |  |  | M | M(=) |
| Message Number | M | M(=) | M | M(=) |

##### Transfer Control Information

###### The Transfer Control Information parameter shall be provided by the C-ATSU AIDC-User when invoking the Transfer-control request primitive.

###### The Transfer Control Information parameter shall conform to the ASN.1 abstract syntax TransferControl, when the C-ATSU AIDC-User invokes the Transfer-control request primitive.

###### The Transfer Control Information parameter shall be provided by the R-ATSU AIDC-User when invoking the Transfer-control response service primitive.

##### Result

###### The Result parameter shall be provided by the C-ATSU AIDC-User.

###### The Result parameter shall conform to the ASN.1 abstract syntax Result.

###### When the Result parameter value is set to “accept”, the Transfer Control Information parameter shall conform to the ASN.1 abstract syntax TransferControlAssume.

###### When the Result parameter value is set to “reject”, the Transfer Control Information parameter shall conform to the ASN.1 abstract syntax TransferControlReject.

### Transfer-communication Service

- ¾ The purpose of the Transfer-communication service is to allow a C-ATSU to indicate that it, is relinquishing communication authority for a flight to an R-ATSU.

#### Service Primitives.

##### The Transfer-communication service shall be an unconfirmed service.

##### The Transfer-communication service primitives shall only be invoked after the successful completion of the Transfer-initiate service.

##### Table 3-12 specifies the parameters that shall be passed when the primitives of the Transfer-communication service are invoked.

Table 3-12: Transfer-communication Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Transfer Communication Information | M | M(=) |
| Message Number | M | M(=) |

##### Transfer Communication Information

###### The Transfer Communication Information parameter shall be provided by the AIDC-User.

###### The Transfer Communication Information parameter shall conform to the ASN.1 abstract syntax TransferCommunication.

### Transfer-communication-assume Service

- ¾ The purpose of the Transfer-communication-assume service is to allow an R-ATSU to indicate to a C-ATSU that communication with a flight has been established.

#### Service Primitives.

##### The Transfer-communication-assume service shall be an unconfirmed service.

##### The Transfer-communication-assume service primitives shall only be invoked after the successful completion of the Transfer-initiate service.

##### Table 3-13 specifies the parameters that shall be passed when the primitives of the Transfer-communication-assume service are invoked.

Table 3-13: Transfer-communication-assume Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Transfer Communication Assume Information | M | M(=) |
| Message Number | M | M(=) |

##### Transfer Communication Information

###### The Transfer Communication Assume Information parameter shall be provided by the AIDC-User.

###### The Transfer Communication Assume Information parameter shall conform to the ASN.1 abstract syntax TransferCommunicationAssume.

## Asynchronous Services

### Info-transfer Service

- ¾ The Info-transfer service permits an ATSU1 to transmit general executive data, surveillance data, general free text data, or emergency free text data, or to point-out a flight to an ATSU2.

#### Service Primitives.

#### It shall be possible to invoke the primitives of the Info-transfer service at any time after invoking the primitives of the Notify or Coordinate-start service.

##### The Info-transfer service shall be an unconfirmed service.

##### Table 3-14 specifies the parameters that shall be passed when the primitives of the Info-transfer service are invoked.

Table 3-14: Info-transfer Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Information | M | M(=) |
| Message Number | M | M(=) |

##### Information

###### The Information parameter shall be provided by the AIDC-User.

###### The Information parameter shall conform to the ASN.1 abstract syntax of any of the following:

- GeneralExecutiveData,

- GeneralPoint,

- SurveillanceData,

- GeneralFreeText, or

- EmergencyFreeText.

## Termination Services

### End Service

- ¾ The purpose of the End service is to allow an ATSU1 to indicate to an ATSU2 that it is in the process of:

- cancelling the notification for a flight and terminating the AIDC service; or

- cancelling the coordination for a flight and terminating the AIDC service; or

- terminating the AIDC service.

#### Service Primitives.

##### The End service shall be an unconfirmed service.

##### Table 3-15 specifies the parameters that shall be passed when the primitives of the End service are invoked.

Table 3-15: End Service primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Cancel Information | C | C(=) |
| Message Number | M | M(=) |

##### Cancel Information

###### The Cancel Information parameter shall be provided by the AIDC-User when cancelling the notification or coordination for a flight.

###### The Cancel Information parameter shall conform to the ASN.1 abstract syntax of Cancel.

### User-abort Service

- ¾ The purpose of the User-abort service is to allow an AIDC-User to immediately terminate the AIDC service.

- ¾ The User-abort service may be used for both operational and technical reasons.

#### Service Primitives.

##### The User-abort service shall be an unconfirmed service.

##### It shall be possible to invoke the primitives of the User-abort service at any time.

##### The User-abort service primitives shall have no parameters.

### Provider-abort Service

- ¾ The purpose of the Provider-abort service is to provide the capability for the AIDC-service provider to inform the AIDC-User that it can no longer provide the AIDC service.

#### Service Primitives.

###### The Provider-abort service shall be an unconfirmed service.

###### The primitives of the Provider-abort service shall be invoked by the AIDC-AE service-provider.

###### Table 3-16 specifies the parameter that shall be passed when the primitive of the Provider -abort service is invoked.

Table 3-16: Provider-abort Service primitive Parameters

| Parameter Name | Ind |
| --- | --- |
| Provider Abort Reason | M |

###### Provider Abort Reason

The Provider Abort Reason parameter shall be provided by the AIDC-AE service provider.

The Provider Abort Reason parameter shall conform to the ASN.1 abstract syntax ProviderAbortReason.