# GENERAL REQUIREMENTS

## Performance Requirements

- ¾ Systems developed to support AIDC functionality will be capable of meeting the Required Communication Performance (RCP) appropriate for the phase of operation as specified in [6].

## Time Accuracy Requirements

### Absolute times which are sent as parameters over the ATN service provider shall be as accurate as the required resolution of the time parameter.

### Absolute times shall be UTC.

### Dates shall be expressed as UTC (i.e. the day increments at 2359:z).

### Relative times shall be accurate to ± 0.1 second.

### Where time stamps are used they shall consist of year, month, day and hour, minute, second.

## Security Requirements

- ¾ There are no internationally approved operational requirements for security currently identified.

- ¾ Implementation of any security checks on the communication interface is a matter of local implementation.

## Backwards Compatibility Requirements

- ¾ This specification describes the version 1 of the CNS/ATM-1 Package AIDC application. Best efforts will be made to ensure that subsequent versions of this protocol will be backwards compatible.

### For CNS/ATM-1 the AIDC-AE version number shall be set to 1 (one).