ISO/IEC JTC1/SC21  N-10393

Date: 22 May, 1996

Project JTC1.21.53.00.00.00

ISO/IEC

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION
INTERNATIONAL ELECTROTECHNICAL COMMISSION
ISO/IEC JTC 1/SC 21

Secretariat: ANSI (U.S.)

Title:	Information technology - Open Systems Interconnection - Structure of Management Information: Managed Objects for Supporting Upper Layers

Source:	ISO/IEC JTC 1/SC 21/WG 7 Meeting, Kansas City, USA, May 1996

Status:	This document has been recommended to progress to CD for National Body Ballot

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work.

In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75% of the national bodies casting a vote.

Managed Objects for Supporting Upper Layers

# 1.	Scope

This Recommendation | International Standard defines generic upper-layer managed objects.

This Recommendation | International Standard

· establishes a model for common supporting upper layer objects.

·provides generic and formal definitions for common supporting upper layer information (managed objects)

This Recommendation | International Standard does not

·define new management functions

·specify a framework or methodology for conformance tests

In the context of this Recommendation | International Standard, the term Supporting Upper Layers is used to refer to ACSE, Presentation, and Session layers.

# 2.	Normative references

The following ITU-T Recommendations and International Standards contain provision which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and International Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent editions of the Recommendations and Standards indicated below. Members of IEC and ISO maintain registers of current valid International Standards. The ITU-T Secretariat maintains a list of the currently valid ITU-T Recommendations.

## 2.1	Identical ITU-T Recommendations | International Standards

Ñ	CCITT Recommendation  X.701 (1992) | ISO/IEC 10040 : 1992, Information technology - Open Systems Interconnection - Systems management overview.

Ñ	CCITT Recommendation X.720 (1992) | ISO/IEC 10165-1 : 1993, Information technology - Open Systems Interconnection -  Structure of management information -  Management information model.

Ñ	CCITT Recommendation X.721 (1992) | ISO/IEC 10165-2 : 1993, Information technology - Open Systems Interconnection -  Structure of management information -  Definition of management information.

Ñ	CCITT Recommendation X.722 (1992) | ISO/IEC 10165-4 : 1992, Information technology - Open Systems Interconnection -  Structure of management information - Guidelines for the definition of managed objects.

Ñ	CCITT Recommendation X.723 (1993) | ISO/IEC 10165-5 : 1993, Information technology - Open Systems Interconnection - Structure of management information - Generic management information.

Ñ	CCITT Recommendation X.726 | ISO/IEC 10165-8,. , Information technology - Open Systems Interconnection - Structure of management information - Managed Objects for Upper Layers.

## 2.2	Paired ITU-T Recommendations | International Standards equivalent in technical content

Ñ	CCITT Recommendation X.200 (1989), Reference model of Open Systems Interconnection for CCITT applications.

ISO 7498 : 1984, Information processing systems - Open Systems Interconnection - Basic Reference Model..

Ñ	CCITT Recommendation X.208 (1989), Specification of abstract syntax  notation one (ASN.1).

ISO/IEC 8824 : 1990, Information technology - Open Systems Interconnection - Specification of Abstract Syntax Notation One (ASN.1).

Ñ	CCITT Recommendation X.290 (1994), Conformance Testing Methodology and Framework - Part 1: General Aspects.

ISO 9646-1 : 1994, Information processing systems - Open Systems Interconnection - Conformance Testing Methodology and Framework - Part 1: General Aspects.

Ñ	CCITT Recommendation X.700, Management Framework Definition for Open Systems Interconnection (OSI) for CCITT Applications, ITU, Geneva.

ISO 7498-4 : 1989, Information processing systems - Open Systems Interconnection -  Basic Reference Model - Part 4: Management Framework.

Ñ	CCITT Recommendation X.710 (1991), Common Management Information Service Definition for CCITT Applications.

ISO/IEC 9595 : 1991, Information technology - Open Systems Interconnection -  Common management information service definition.

Ñ	CCITT Recommendation X.711 (1991), Common management information protocol specification for CCITT applications.

ISO/IEC 9596-1 : 1991, Information technology - Open Systems Interconnection - Common management information protocol specification.

# 3	Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply.

3.1	Basic reference model definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.200 | ISO 7498-1

a)	open system;

b)	systems management.

3.2	Management framework definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.700 | ISO/IEC 7498-4

a)	managed object;

b)	systems management application-entity.

3.3	CMIS definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.710 | ISO/IEC 9595

a)	attribute;

b)	Common Management Information Service Element;

c)	Common Management Information Service.

3.4	Remote Operations definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.219 | ISO/IEC 13712-1.

a)	invoker;

b)	performer.

3.5	Systems management overview definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.701 | ISO/IEC 10040

a)	agent;

b)	agent role;

c)	generic definitions;

d)	managed object class;

e)	managed (open) system;

f)	manager;

g)	manager role;

h)	MIS-User;

i)	notification;

j)	notification type;

k)	systems management application protocol;

l)	systems management functional unit.

3.6	Management information model definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.720 | ISO/IEC 10165-1

a)	attribute type;

b)	containment hierarchy.

3.7	Guidelines for the definition of managed objects definitions

This Recommendation | International Standard makes use of the following terms defined in CCITT Rec. X.722 | ISO/IEC 10165-4.

template

# 4	Symbols and abbreviations

ACSE	Association Control Service Element

AE		Application Entity

AEI		Application Entity Invocation

AP		Application Process

API		Application Process Invocation

ASE		Application Service Element

ASO	Application Service Object

CL		Connectionless

CO		Connection Oriented

DN		Distinguished Name

FU		Functional Unit

PM		Protocol Machine

RDN	Relative Distinguished Name

# 5	Conventions

This Recommendation | International Standard makes use of the template notation defined in Rec. 722 | ISO/IEC 10165-4 as the notation for defining managed object classes and the abstract syntax notation defined in Rec. X.208 |  ISO/IEC 8824 as the means of defining ASN.1 data types associated with the use of template notation.

# 6	Overview

This Recommendation | International standard defines Managed Objects that represent OSI ManagementÕs view of those elements of an Open System which support the OSI Session, Presentation and Application Layer services subject to OSI management operations.

The defined objects are of four different kinds:

•	Communcation Entities  representing active functional elements which perform communication processing functions within a system.

•	Protocol Machines performing communications functions within an entity.

•	Service AccessPoints at which services are provided by an entity to the user entity.

•	Connections and Associations established between entities for the transfer of service or protocol data.

The first three kinds of objects are of a static nature, and the last kind is of a more dynamic nature.

6.1 	Upper Layer Model

The basic building blocks of the upper layers which typically exist beyond a single association are:

• 	One or more Application Service Elements (ASE) or Application Service Objects (ASOs), bundled into an Application Entity (AE). (The ASE ACSE is always included in the AE.);

•	The Presentation Layer entity;

•	The Session Layer entity.

For modeling purposes, we may think of these building blocks as being present even when the implementation has merged their functions in a monolithic manner.

The capabilities of an upper layer entity, UL-entity (i.e., AE, Presentation Layer entity, Session Layer entity), are made visible as attributes of its contained upper layer Protocol Machine Managed objects. For simplicity, this contribution only addresses Upper layer Connection Oriented Protocol Machines (UL-COPM).

The available functional units (i.e., those which are implemented by an UL-COPM) is an attribute of each of the UL-COPM managed objects.

Additionally, the AE managed object class has an attribute to list the Application contexts which the AE makes available. Also, the presentation COPM managed object has an attribute indicating the supported Abstract syntaxes and transfer Syntaxes.

The UL-entity managed objects are defined as a subclass of the communicationsEntity managed object class defined in Recommendation X.723 | ISO/IEC 10165-5.

The UL-COPM managed objects are defined as subclasses of the connectionOrientedProtocolMachine managed object class defined in Recommendation X.723 | ISO/IEC 10165-5.

Only management of ACSE is standardized in this recommendation | international standard. Other ASEs, like CMISE, ROSE, SMASE etc. might be specified in other standards.

The relationship between the Upper Layer object at each layer follows the structure as defined in the lower layers. This structure of objects in the lower layers, as defined by ITU-T Rec.  X.283  | ISO/IEC 10733, Elements of Management Information Related to OSI Net Layer Standards and ITU-T X.284 | ISO/IEC 10737,  Elements of Management Information Related to OSI Transport Layer Standards, is as illustrated in Figure 1.

Figure 1 - Generic OSI Layer Containment Hierarchy

N-Layer subsystems are subclasses of the managed object class Rec. X.723 | ISO/IEC 10165-6 : subsystem. This managed object class is used as a common containment point for managed objects in a system that relate to the operation on the N-layer.

N-layer entities are subclasses of the managed object class Rec. X.723 | ISO/IEC 10165-5 : communicationsEntity. This managed object class is used to represent the active functional elements which perform communications processing functions within a system, e.g., Network Entity, Transport Entity, etc.. The localSapNames attribute contains a set of distinguished names of layer (N-1) SAPs or ports at which services are provided to the entity.

N-layer protocol machines are subclasses of  either Rec. X.723 | ISO/IEC 10165-5 : clProtocolMachine or Rec. X.723 | ISO/IEC 10165-5 : coProtocolMachine. These represent the protocol machine performing communications (connectionless-mode or connection-mode, respectively) within an entity. The information represented may include the information across all invocations initiated by this protocol machine. A protocol machine includes  the mechanism for both the establishment and data-transfer phases of the communications.

N-layer invocations are subclasses of Rec. X.723 | ISO/IEC 10165-5 : singlePeerConnection and represent an association or a connection established between N-layer entities for the transfer of service or protocol data (using an N-layer protocol of the N-layer protocol machine). The underlyingconnectionNames attribute contains the distinguished names of the managed objects that represent the underlying connections or physical media used by the connection (i.e., supporting service invocation). Invocation objects are instantiated contained in their respective protocol machine Mos.

This model applies to all the layers in the OSI model. At the application layer, however, the protocol machine is actually composed of different protocol modules or components. These components represent the different establishment (and termination) and data transfer phases of communication. Generally, ACSE will be the module that provides the establishment phase with one or more ASEs (or ASOs) composing the data transfer phase. Although the ASEs are often referred to as protocol machines themselves, they actually represent only a component (e.g., CMISE PM, SMASE PM, ACSE PM). While it is true that each ASE has a state machine, it is only the combination of ASEs and ACSE that constitutes a protocol machine in the same sense as the lower layers, i.e., containing both the establishment and data transfer phases. Therefore, in the Application Layer Model, it is necessary to show the “structure” of the "constructed" PM and its associated invocations.

In the Application Layer Model, containment is used to show the relationship between a PM and its components (i.e., its "structure"). Similarly, containment is used to show the relationship between the invocation of an application layer PM and its components (i.e., the invocations of the PM’s components). The containment structures of an application layer PM and that of its invocation should be isomorphic. See Figure 3: ALS Managed Objects. However, in the application layer model, because the PM is a composition, it is not modelled as a separate object from its entity, the Application Entity. Rather, only the component PMs are represented as managed objects. Therefore, in the application layer, the application layer entity and its PM are modelled as one object, called the Application Entity (AE). This object then contains the objects that represent the modules or components of the PM. However, the invocation of this application Layer PM is modelled as an object in order to represent the structure of the component invocations as a containment hierarchy.

In the lower layers as seen above, the PM invocations are contained in their respective PMs.  Unlike the lower layers, however, the Application Layer models the components of the protocol machine and its invocations. Therefore, in the application layer model, the component invocations are contained in the application layer invocation object and represent the structure of the invocation. Each component invocation has a pointer to its corresponding PM component. The containing Application Layer invocation object is then contained in the AL PM (which is actually modelled with the AL entity).

For ALS, the concept of an ASO and its recursive structure also needs to be modelled. See Figure 3. With respect to ALS, an AE is a specialized ASO Entity and is the outermost ASO Entity that includes an Establishment ASE (e.g., ACSE) and that refers to a PSAP. An AE can contain the following: ASO Entities which include ASCE (ASOEs); ASOs which do not contain ACSE; and ASEs. As defined in the ACSE ALS, an ASO is two or more ASEs combined with a Control Function (CF). An ASO which contains both an Establishment ASE (e.g., ACSE) and a data transfer ASE is called an ASO Entity.

## 6.2	Managed Object Hierarchy

### 6.2.1	Session Layer Managed Object Classes

The following managed object classes are defined for the session layer:

a)	Session Entity

b)	Session Connection-oriented Protocol Machine

c)	S (Session) SAP

d)	Session Connection

e)	Session Subsystem

The session layer containment hierarchy is illustrated in Figure 2.

Figure 2 - Session Layer Containment Hierarchy

### 6.2.2	Presentation Layer Managed Object Classes

The following managed object classes are defined for the presentation layer:

f)	Presentation Entity

g)	Presentation Connection-oriented Protocol Machine

h)	P (Presentation) SAP

i)	Presentation Connection

j)	Presentation Subsytem

The presentation layer containment hierarchy is illustrated in Figure 3.

Figure 3 - Presentation Layer Containment Hierarchy

### 6.2.3	Application Layer Managed Object Classes

The following managed objects are defined for the application layer:

k)	Application Entity

l)	Application Entity Invocation

m)	ACSE

n)	ACSE Connectionoriented Protocol Machine

o)	ACSE Association

p)	Application Subsystem

q)	AP (Application Process) Invocation

r)	ASO Entity

s)	ASO

t)	ASO Invocation

A generic application layer containment hierarchy is illustrated in Figure 4.

Figure 4 - Generic Application Layer Containment Hierarchy

## 6.3	Inheritance

The inheritance hierarcy for the managed object classes defined for the session, presentation and application layers is illustrated below in Figure 5

Figure 5 - Inheritance Hierarchy of Upper Layer Objects

## 6.3	Relationships

### 6.3.1 Layer N-1 Services

Each entity has a relation to the SAP MO of its underlying layer.

### 6.3.2 Connections

There is a relationship between a connection/association and its underlying connection managed objects. At the presentation and session layers, and for entities at the application layer (i.e., ASO Entities, Application Entity), there is a relationship between a connection/association and its underlying connection managed object. The underlyingConnectionNames attribute pointer is used to represent this relationship. For example, the presentation connection is related to the session connection via the underlyingConnectionNames attribute pointer. An Application Entity Invocation points to its Application Entity using the same attribute pointer.

### 6.3.3 Supporting Services

At the application layer, there is a relationship between an ASO/ASE connection/association and its underlying connection managed object. This relationship is represented via containment.  A containing object is the supporting service for its contained objects.

### 6.3.4 API-AEI

There is a containment relationship between an Application Process Invocation (superior) and the Application Entity Invocations (subordinate) that perform its communication functions.

## 6.4	Session and presentation layer invocations

Figure 6 illustrates the various relationships among managed objects at the session and presentation layers. These relationships are represented either via containment (illustrated as straight lines) or via pointer attributes (illustrated as arrowed lines).

Figure 6 - Presentation and Session Layer Relationships

## 6.5	Application entity and application entity invocations

An example of a containment hierarchy for a typical application layer using ACSE and other ASEs appears in Figure 7. This figure also identifies the other relationships between object instances represented by pointer attributes (illustrated as arrowed lines). Figure 8 shows an example for a generic application layer using ASO Entities, ASOs and ASEs.

Figure 5- Application Layer Containment Hierarchy with ACSE

The solid lines represent containment relationships between managed object instances.  Containment relationships are used to determine the Distinguished names for managed object instances (i.e., the contained managed object instance is named within the scope of the containing managed object instance).  The arrowed lines represent a relationship attributes (i.e., pointer from one managed object instance to another).

An Application process is part of an open system.  A managed open system (represented by a managed object such as system managed object defined in Recommendation X.721 | ISO/IEC 10165-2 or managedElement managed object defined in Recommendation M.3100) may contain zero or more Application processes in its application subsystem. An Application process may also be contained in the system object directly. In this case, the Application Entity is contained in the Application Subsystem rather than the Application Process.

The name of the application entity is its application entity title, and the name of the application process is its application process title.  It is possible,  with the proper name bindings and choice of naming attributes, to have the name of the  managed objects representing an Application entity and its Application process to be the same as the Distinguished Names of their  form 1 Application entity title and Application process title, respectively.

To enable  naming harmony between the AE-title and the name of the AE managed object, the naming attribute (Relative Distinguished Name / RDN) of the Application entity managed object instance must be the form1 Application entity qualifier.

Figure 8 - Generic Application Layer Containment Hierarchy

Note 1 - Each component of the uppermost ASO Invocation points to its corresponding component under the ASO Entity.

Note 2 - An ASO Entity must contain an Establishment ASE (may be ACSE or non-ACSE).

Note 3 - The ASO Invocation structure must be isomorphic to its corresponding ASO Entity structure

Note 4 - Entities that are shaded are outside the scope of this Application Layer.

Note 5 - Solid lines indicate containment relationships, arrowed lines indicate attribute pointer relationships.

# 7	Generic definitions

## 7.1	Imported definitions

## 7.1.1	Imported managed object classes

The following managed object classes and their associated attributes are imported from Rec. X.723 | ISO/IEC 10165-5 and Rec. X.721 |lISO/IEC 10165-2:

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":applicationProcess

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":applicationProcessId

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":applicationProcessTitle

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":supportEntityNames

- "Rec. X.721 | ISO/IEC 10165-2 : 1992":operationalState

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntity

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntityId

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":localSapNames

- "Rec. X.721 | ISO/IEC 10165-2 : 1992":operationalState

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":coProtocolMachine

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":coProtocolMachineId

- "Rec. X.721 | ISO/IEC 10165-2 : 1992":operationalState

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":Sap1

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":sapId

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":sap1Address

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":userEntityNames

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":Single peer connection

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":connectionId

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":underlyingConnectionNames

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":supportedConnectionNames (conditional)

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":subsystem

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":subsystemId

## 7.1.2	Imported name bindings

The following name bindings  are imported from Rec. X.723 | ISO/IEC 10165-5:

- applicationProcess-system

- communicationsEntity-subsystem

- coProtocolMachine-communicationsEntity

- sap1-communicationsEntity

- singlePeerConnection- coProtocolMachine

- subsystem-system

The communicationsEntity-subsystem namebinding is used for naming instances of sessionEntity with respect to sessionSubsystem and for naming instances of presentationEntity to presentationSubsystem.

The coProtocolMachine-communicationsEntity namebinding is used for naming instances of sessionCopm with respect to sessionEntity and for naming instances of presentationCopm with respect to presentationEntity.

The sap1-communicationEntity namebinding is used for naming instances of sSap to sessionEntity and for naming instances of pSap to presentationEntity.

The singlePeerConnection-coProtocolMachine namebinding is used for naming instances of sessionConnection to sessionCopm and for naming instances of presentationConnection to presentationCopm.

## 7.2	Session layer managed object classes

### 7.2.1 	Session Subsystem

This subclass of GMI:subsystem holds reference information about Session subsystem.

It specializes by adding only behaviour.

### 7.2.2 	Session Entity

This subclass of GMI:communicationsEntity holds reference information about Session entity.

It specializes by adding only behaviour.

### 7.2.3	Session COPM

This subclass of GMI:coProtocolMachine holds reference information pertaining to the capabilities of a Session protocol machine.

It specializes by adding the following attributes:

- Session protocol versions supported (bitstring);

- Session functional units supported (bitstring);

- optional support requirements (i.e., segmenting).

### 7.2.4 	SSAP

This subclass of GMI:sap1 has the S-selector and the binding between the Presentation Entity and the sap.

It specializes by adding only behaviour.

### 7.2.5 	Session Connection

This subclass of GMI:singlePeerConnection holds reference information pertaining to the single session connection for the association.  The underlying connections attribute is used to point to the transport connection used.

It specializes by adding the following attributes:

- Session protocol version in use;

- Session functional units negotiated in use;

- it may have an attribute for the negotiated maximum TSDU size if segmenting is in use.

NOTE - Subclasses of this object class may be defined to include statistics on PPDU transfers which take place over the connection.

## 7.3	Presentation layer managed object classes

### 7.3.1	Presentation Subsystem

This subclass of GMI:subsytem holds reference information about Presentation Subsystem.

It specializes by adding only behaviour.

### 7.3.2	Presentation Entity

This subclass of GMI:communicationsEntity holds reference information about Presentation Entity

It specializes by adding only behaviour.

### 7.3.3	Presentation COPM

This subclass of GMI:coProtocolMachine holds reference information pertaining to the capabilities of a Presentation protocol machine.

It specializes by adding the following attributes:

- Presentation functional units supported (bitstring);

- Abstract syntaxes supported (set of OBJECT IDENTIFIER);

- Transfer syntaxes supported (set of OBJECT IDENTIFIER).

### 7.3.4	PSAP

This subclass of GMI:sap1 has the P-selector and the binding between the AE and the sap.

It specializes by adding only behaviour.

### 7.3.5	Presentation Connection

This subclass of GMI:singlePeerConnection holds reference information pertaining to the single presentation connection for the association.  The underlying connections attribute is used to point to the session connection endpoint used.

It specializes by adding the following attributes:

- presentation functional units negotiated in use

- presentation contexts in use

NOTE - Subclasses of this object class may be defined to include statistics on PPDU transfers which take place over the connection.

## 7.4	Application layer managed object classes

### 7.4.1	Application subsystem

This subclass of GMI:subsystem holds reference information about an Application subsystem.

It specializes by adding only behaviour.

### 7.4.2	ASO

This subclass of DMI:top holds reference information about an ASO.

It specializes by adding the following attributes:

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntityId;

- asoTitle;

- asoQualifier;

- "Rec X.721 | ISO/IEC 10165-2 : 1992":operationalState.

### 7.4.3	ASO entity

This subclass of aso and GMI:communicationsEntity holds reference information about an ASO entity.

It specializes by adding only behaviour.

### 7.4.4	Application entity

This subclass of asoEntity holds reference information about an Application entity and its associated ACSE protocol machine.

It specializes by adding behaviour which restricts the values of its asoEntity attributes.

### 7.4.5	ACSE

This subclass of ASO holds reference information about ACSE application service element.

It specializes by adding the following attributes:

- ACSE functional units supported as initiator (bitstring);

- ACSE functional units supported as responder (bitstring).

EDITOR'S NOTE - Should the ACSE object class indicate which ACSE editions are being supported?

### 7.4.6	ASO Invocation

This subclass of DMI:top holds reference information about ASO, application service object.

It specializes by adding the following attributes:

- "Rec. X.723 | ISO/IEC 10165-5 : 1995":connectionId;

- invocationOfPointer.

### 7.4.7	Application entity invocation

This subclass of asoInvocation holds reference information about an application entity invocation and adds the following attributes:

It specializes by adding only behaviour.

### 7.4.8	ACSE association

This subclass of asoInvocation holds reference information about an ACSE association and adds the following attributes:

It specializes by adding the following attributes:

- calling  AE title;

- ACSE functional units negotiated in use;

- Application context in use;

- parent ASO invocation (relationship pointer to parent ASO invocation..  It could be the AEI);

- it may also have an attribute for the called AE Title, if the AE Title is known.

### 7.4.9	AP invocation

This subclass of DMI:top holds reference information about an application process (AP) invocation.

It specializes by adding the following attributes:

- apInvocationId;

- invocationOfPointer, to identify the specific AP it is an invocation of.

## 7.5	Name bindings

The following name bindings are defined in this document for support of the model:

- apInvocation-applicationSubsystem;

- applicationProcess-applicationSubsystem;

- asoInvocation-apInvocation;

- asoInvocation-asoInvocation;

- asoEntity-applicationProcess;

- aso-asoEntity;

- aso-aso.

Annex A

Monolithic Upper Layer

(This annex forms an integral part of this Recomendation | International Standard.)

Given that a separate layer managed object design is defined, the characteristics of the separate managed objects can be included in defining a merged or monolithic upper layer objects (e.g., a monolithic upper layer entity MO, a monolithic upper layer connection protocol machine MO, and a monolithic upper layer connection MO, as shown in Figure 5).  Monolithic upper layer implementations are not adequately modelled by separate managed objects for each protocol layer. The monolithic upper layer object classes define esentially merge the functionality of the seperate session layer, presentation layer and application entity object classes into one object class for an entity, a connection protocol machine and a connection. Note that the behaviour at the application layer for the application entity's constituent ASO Entities, ASOs, and ASEs does not change.

Figure 9 - Monolithic Upper Layer Managed Object Classes and their Relationships

Note 1 - Transport entities are shaded because they are outside the scope of this Recommendation | International Standard.

Note 2 - Solid lines indicate containment relationships, arrowed lines indicate attribute pointer relationships.

The management information in the monolithic managed object design is essentially the same as the management information in the separable upper layer managed object design previously defined.

The information being kept is the same for the monolithic managed object case as for the separate managed object case,  except for the lack of need for the one-to-one relationship pointers.

The monolithic upper layer entity managed objects are appropriate when the implementation has combined the upper layer protocols into a single entity.

For the monolithic UL entity managed object class, multiple inheritance is used from the application entity, presentation entitiy, session entity, presentation COPM, and session COPM managed object classes. The PSAP and SSAP do not need to be represented because null selectors are appropriate in the monolithic case. Specific implementations which use non-null selectors could subclass. The local SAP names attribute is used to point to the TSAP managed object.

Similarly, for the monolithic UL Connection managed object class, multiple inheritance is used from the Application Entity Invocation, Presentation Connection, and Session Connection managed object classes.

It would not be easy to merge the ASO/ASE specific association information objects, due to the one-to-many containment relationship.  Having a different monolithic ASO/ASE-association endpoint subclass defined for each ASO/ASE-type is not practical due to the combinatorial complexity involved.

There is benefit in retaining separate managed object instances to maintain each ASO/ASE invocation's view of the association.

### A.1	Monolithic Upper Layer Subsystem

This subclass of applicationSubsystem, presentationSubsystem, and sessionSubsystem holds reference information about an application subsystem which has monolithic connection of all three upper layers.

It specializes by adding only behaviour.

### A.2	Monolithic Upper Layer Entity

This subclass of applicationEntity, presentationEntity, sessionEntity, presentationCopm and sesionCopm holds reference information about an application entity which has monolithic implementation of all three upper layers.

It specializes by adding the following attributes:

- presentation selector value;

- session selector value.

### A.3	Monolithic Upper Layer Connection

This subclass of applicationEntityInvocation, presentationConnection, and sessionConnection holds reference information about an application connection which has monolithic connection of all three upper layers.

It specializes by adding only behaviour.

Annex B

Management Information Definitions

(This annex forms an integral part of this Recomendation | International Standard.)

--<GDMO.Document "Draft Rec. X.726 | ISO/IEC CD 10165-8 : 1997"

-- {joint-iso-ccitt ms(9) smi(3) part8(8)>--

--<GDMO.Version 1.3 "Draft Rec. X.726 | ISO/IEC CD 10165-8 : 1997" >--

aCSE MANAGED OBJECT CLASS

DERIVED FROM      aso;

CHARACTERIZED BY

aCSEPackage PACKAGE

BEHAVIOUR

aCSEBehaviour BEHAVIOUR

DEFINED AS

!  This subclass of aso is a specialized application

service object holds reference information about an ACSE

application service element. It specializes by addin the

following attributes:

-acseFUinitiator, the ACSE functioal units supported as

initiator (bitstring);

- acseFUresponder, the ACSE functional units supported as

responder (bitstring).

An application service element does not qualify as an

asoEntity or as a "Rec. X.723 | ISO/IEC 10165--5 :

1995":communicationsEntity because it does not have both

the association phase and the transfer phase of

communication. ACSE provides only the association phase of

communication.

!;;

ATTRIBUTES

acseFUinitiator GET,

acseFUresponder GET;;;

REGISTERED AS {UL-AttributeModule.acseMOCId};

acseAssociation MANAGED OBJECT CLASS

DERIVED FROM      asoInvocation;

CHARACTERIZED BY

acseAssociationPackage PACKAGE

BEHAVIOUR

acseAssociationBehaviour BEHAVIOUR

DEFINED AS

!This subclass of asoInvocation holds reference information

for an ACSE association. It specializes by adding the

following attributes:

- calling AE title,

- ACSE functional units negotiated in use,

- Application context in use,

- parent ASO invocation (relationship pointer to parent ASO

invocation, which could be the AEI).

It may also have an attribute for the called AE title, if

known.!;;;;

CONDITIONAL PACKAGES

calledAEtitlePackage PACKAGE

ATTRIBUTES

calledAEtitle GET;

REGISTERED AS {UL-AttributeModule.calledAEtitlePId};

PRESENT IF  !Present only if the called AE Title is known. !;

REGISTERED AS {UL-AttributeModule.acseAssociationMOCId};

apInvocation MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.721 | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

apInvocationPackage PACKAGE

BEHAVIOUR

apInvocationBehaviour BEHAVIOUR

DEFINED AS

! This subclass of "Rec. X.721 | ISO/IEC 10165-2 :

1992":top holds reference nformation about an Application

Process Invocation. It specializes by adding the following

attributes:

- apInvocationId;

- invocationOfPointer.

The apInvocationId attribute serves as the naming attribute

for instances of this class. The invocationOfPointer

attribute identifies the specific Application Process

(using the distinguished name, DN, of an instance of the

"Rec. X.723 | ISO/IEC 10165-5 : 1995":applicationProcess

managed object class) of which the instance of this class

is an invocation. !;;

ATTRIBUTES

apInvocationId GET,

invocationOfPointer GET;;;

REGISTERED AS {UL-AttributeModule.apInvocationMOCId};

applicationEntity MANAGED OBJECT CLASS

DERIVED FROM      asoEntity;

CHARACTERIZED BY

applicationEntityPackage PACKAGE

BEHAVIOUR

applicationEntityBehaviour BEHAVIOUR

DEFINED AS

!This subclass of asoEntity holds reference information

about an application entity. For an instance of the

applicationEntity object class, the asoQualifier attribute

contains the value of the local application entity (AE)

qualifier. The asoTitle attribute contains the value of the

associated application process (AP) title. The

communicationsEntityId attribute holds the instance's

Application Entity (AE) Id. The localSapNames attribute

identifies a pSAP object instance.

An applicationEntity object is always the outermost ASO

Entity in an AP and serves as the supporting service for

all of its contained ASOs (including ASO Entities and

ASEs). !;;;;

REGISTERED AS {UL-AttributeModule.applicationEntityMOCId};

applicationEntityInvocation MANAGED OBJECT CLASS

DERIVED FROM

"Rec. X.723 | ISO/IEC 10165-5 : 1995":singlePeerConnection,

asoInvocation;

CHARACTERIZED BY

applicationEntityInvocationPackage PACKAGE

BEHAVIOUR

aEInvocationBehaviour BEHAVIOUR

DEFINED AS

! This subclass of"Rec. X.723 | ISO/IEC 10165-5 :

1995":singlePeerConnection and asoInvocation represents an

invocation of an application entity.  It adds no additional

attributes.

The inherited "Rec. X.723 | ISO/IEC 10165-5 :

1995":connectionId attribute serves as the naming attribute

for instances of this class, and represents the application

entity (AE) invocation id. The inherited attribute "Rec.

X.723 | ISO/IEC 10165-5 : 1995":underlyingConnectionNames

identifies the supporting services or connections, which

for instances of this class is a presentation address (the

distinguished name of an object instance representing a

presentation invocation). The inherited invocationOfPointer

attribute identifies the application entity of which an

instance of this class is an invocation. !;;;;

REGISTERED AS {UL-AttributeModule.applicationEntityInvocationMOCId};

applicationSubsystem MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":subsystem;

CHARACTERIZED BY

applicationSubsystemPackage PACKAGE

BEHAVIOUR

applicationSubsystemBehviour BEHAVIOUR

DEFINED AS

! This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995"::subsystem is used as a common containment point for

managed objects in a system that relate to the application

layer (i.e., application entity Mos). It can use the "Rec.

X.723 | ISO/IEC 10165-5 : 1995":subsystem-system Name

Binding. !;;;;

REGISTERED AS {UL-AttributeModule.applicationSubsystemMOCId};

aso MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.721 | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

asoPackage PACKAGE

BEHAVIOUR

asoBehaviour BEHAVIOUR

DEFINED AS

! This subclass of "Rec. X.721 | ISO/IEC 10165-2 :

1992":top holds reference nformation about an ASO, which is

a collection of two or more ASOs (including specialized

ASOs, e.g., ASEs) together with a Control Function that

moderates the interactions of the component ASEs and ASOs

with the service provided and the supporting service. An

ASO contains one or more ASEs/ASOs that provide for data

transfer phase of communication. An ASO may also contain an

ASE that provides for the establishment phase of

communication. An ASO association can only be established

to an ASO that provides for both the establishment and data

transfer phases.

This managed object class is non-instantiable. Subclasses

of this class provide information about specific ASOs

including the informaion about the Controlling Function.

The information about the Control Function may only be

specified via the behaviour definitions of subclasses of

this class, but can be made visible via attributes of the

subclass if needed.

It specializes by adding the following attributes:

- "Rec. X.723 | ISO/IEC 10165-5 :

1995":communicationsEntityId;

- asoTitle;

- asoQualifier;

- "Rec. X.721 | ISO/IEC 10165-2 : 1992":operationalState.

The attribute "Rec. X.723 | ISO/IEC 10165-5 :

1995":communicationsEntityId serves as the naming attribute

for instances of this class. The ASO Name is formed by

concatenating the asoTitle and the asoQualifier. The

operationalState is used to indicate whether instances of

this class are enabled or disabled, the syntax and

semantics of which are defined in Rec. X.721 | ISO/IEC

10165-2 and Rec. X.731 | ISO/IEC 10164-2. !;;

ATTRIBUTES

"Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntityId,

asoTitle,

asoQualifier,

"Rec. X.721 | ISO/IEC 10165-2 : 1992":operationalState;;;

REGISTERED AS {UL-AttributeModule.asoMOCId};

asoEntity MANAGED OBJECT CLASS

DERIVED FROM

"Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntity,

aso;

CHARACTERIZED BY

asoEntityPackage PACKAGE

BEHAVIOUR

asoEntityBehaviour BEHAVIOUR

DEFINED AS

! This subclass of aso and "Rec. X.723 | ISO/IEC 10165-5 :

1995":communicationsEntity holds reference information

about an ASO entity, which is an ASO that contains ACSE for

the establishment phase of communication. Thus, it

represents an ASO with which communications may be

established.

An instance of an asoEntity serves as the supporting

service for its contained objects, which can include ASOs

(i.e., subclasses of the ASO managed object class: this

includes ASEs, e.g., ACSE, CMISE, SMASE , and also other

ASO Entities).

It specializes by adding the following attribute:

- applContextNameSupport.

The applContextNameSupport attribute identifies the

specific application contexts that are supported by an

instance of this class. These are identified using the

given application contexts' object identifier values. !;;

ATTRIBUTES

applContextNameSupport GET;;;

REGISTERED AS {UL-AttributeModule.asoEntityMOCId};

asoInvocation MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.721 | ISO/IEC 10165-2 : 1992":top;

CHARACTERIZED BY

asoInvocationPackage PACKAGE

BEHAVIOUR

asoInvocationBehaviour BEHAVIOUR

DEFINED AS

! This subclass of "Rec. X.721 | ISO/IEC 10165-2 :

1992":top holds reference nformation about an ASO

Invocation. This managed object class is non-instantiable.

Subclasses of this class provide information about

invocations of specific ASOs.

An instance of a subclass of ASO Invocation serves as the

supporting service for its * contained *  object instances,

which are in turn also subclasses of ASO Invocation (e.g.,

AE Invocation, ACSE Invocation, CMISE Invocation, SMASE

Invocation).

It specializes by adding the following attributes:

- connectionId;

- invocationOfPointer.

The connectionId attribute serves as the naming attribute

for instances of this class. The invocationOfPointer

attribute identifies a specific ASO of which the instance

of this class is an invocation. !;;

ATTRIBUTES

"Rec. X.723 | ISO/IEC 10165-5 : 1995":connectionId GET,

invocationOfPointer GET;;;

REGISTERED AS {UL-AttributeModule.asoInvocationMOCId};

monoULConnection MANAGED OBJECT CLASS

DERIVED FROM

applicationEntityInvocation,

presentationConnection,

sessionConnection;

CHARACTERIZED BY

monoULConnectionPackage PACKAGE

BEHAVIOUR

monoULConnectionBehaviour BEHAVIOUR

DEFINED AS

! The monULConnection managed object class is a subclass

ofthree object classes:  applicationEntityInvocation,

presentationConnection and sessionConnection. It holds

reference information about an application connection which

has monolithic connection of all three upper layers. It

adds no additional attributes. !;;;;

REGISTERED AS {UL-AttributeModule.monoULConnectionMOCId};

monoULEntity MANAGED OBJECT CLASS

DERIVED FROM

applicationEntity,

presentationEntity,

sessionEntity,

presentationCopm,

sessionCopm;

CHARACTERIZED BY

monoULEntityP1 PACKAGE

BEHAVIOUR

monoULEntityBehaviour BEHAVIOUR

DEFINED AS

! The monoULEntity managed object class is a subclass of

applicationEntity, presentationEntity, sessionEntity,

presentationCopm and sessionCopm object classes. It holds

reference information about an application entity which has

monolithic implementation of all three upper layers.

The communicationsEntityId attribute is multiply inherited

from all three "Entity" superclasses and serves as the

naming attribute for instances of this class. It should

have the exact value as the coProtocolMachineId attribute,

multiply inherited from "Copm" superclasses.

The operationalState attribute has the semantics and syntax

defined in "Rec. X.731 | ISO/IEC 10164-2" State Management

Function. The value of the localSAPName attribute

(inherited) should contain the address of the SAP that the

virtual sessionEntity points to. The value of the

asoQualifier attribute (inherited from aso) is the local AE

Qualifier. The value of the asoTitle attribute (inherited

from aso) is the AP Title. The value of the application

context names supported (inherited From applicationEntity)

contains the values supported by the virtual application

layer.

Since this is a subclass of presentationCopm and

sessionCopm, it also holds reference information pertaining

to the capabilities of a protocol machine which has

monolithic implementation of all three upper layers.

The following attributes are inherited from

presentationCopm:

- Presentation functional units supported (bitstring);

- Abstract syntaxes supported (set of OBJECT IDENTIFIER);

- Transfer syntaxes supported (set of OBJECT IDENTIFIER).

The following attributes are inherited from sessionCopm::

- Session functional units supported (bitstring);

- Optional session support requirements (i.e., segmenting);

- Session protocol versions supported.

This object class specializes by adding the following

attributes:

- presentation selector value;

- session selector value.

These two attributes are needed because in the monolithic

implementation the funtionality of the PSAP and SSAP object

classes are "absorbed" into this object class along with

their containers (presentationEntity and sessionEntity). !;;

ATTRIBUTES

presSelectorValue GET,

sessionSelectorValue GET;;;

REGISTERED AS {UL-AttributeModule.monoULEntityMOCId };

monolithicUpperLayerSubsytem MANAGED OBJECT CLASS

DERIVED FROM

sessionSubsystem,

presentationSubsystem,

applicationSubsystem;

CHARACTERIZED BY

monolithicUpperLayerSubsystemPackage PACKAGE

BEHAVIOUR

monolithicUpperLayerSubsystemBehaviour BEHAVIOUR

DEFINED AS

! This subclass of sessionSubsystem, presentationSubsystem,

applicationSubsystem is used as a common containment point

for managed objects in a system that relate to the a

monolithic upper layer. It can use the "Rec. X.723 |

ISO/IEC 10165-5 : 1995":subsystem-system Name Binding. !;;;;

REGISTERED AS {UL-AttributeModule.monolithicUpperLayerSubsystemMOCId};

pSap MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":sap1;

CHARACTERIZED BY

pSapP1 PACKAGE

BEHAVIOUR

pSapB1 BEHAVIOUR

DEFINED AS

!This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995":sap1 has the P-selector and the binding between the

AE and the sap.!;;;;

REGISTERED AS {UL-AttributeModule.pSapMOCId};

presentationConnection MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":singlePeerConnection;

CHARACTERIZED BY

presConnEndpointP1 PACKAGE

BEHAVIOUR

presConnnectionBehaviour BEHAVIOUR

DEFINED AS

!This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995":singlePeerConnection holds reference information

pertaining to the single presentation connection for the

association. The underlying connections attribute is used

to point to the session connection endpoint used.

It specializes by adding the following attributes:

- presentation functional units negotiated in use;

- presentation contexts in use.  !;;

ATTRIBUTES

presFUinUse GET,

presContextInUse GET;;;

REGISTERED AS {UL-AttributeModule.presentationConnectionMOCId};

presentationCopm MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":coProtocolMachine;

CHARACTERIZED BY

presentationCopmP1 PACKAGE

BEHAVIOUR

presentationCopmBehaviour BEHAVIOUR

DEFINED AS

!This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995":coProtocolMachine holds reference information

pertaining to the capabilities of a Presentation protocol

machine.

It specializes by adding the following attributes:

- Presentation functional units supported (bitstring);

- Abstract syntaxes supported (set of OBJECT IDENTIFIER);

- Transfer syntaxes supported (set of OBJECT IDENTIFIER).

!;;

ATTRIBUTES

presFUsupport GET,

abstrSyntaxSupport GET,

transfSyntaxSupport GET;;;

REGISTERED AS {UL-AttributeModule.presentationCopmMOCId};

presentationEntity MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntity;

CHARACTERIZED BY

presentationEntityP1 PACKAGE

BEHAVIOUR

presentationEntityB1 BEHAVIOUR

DEFINED AS

! This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995":communicationsEntity holds reference information

about  a Presentation Entity. It specializes by adding only

behaviour.!;;;;

REGISTERED AS {UL-AttributeModule.presentationEntityMOCId};

presentationSubsystem MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":subsystem;

CHARACTERIZED BY

presentationSubsystemPackage PACKAGE

BEHAVIOUR

presentationSubsystemBehaviour BEHAVIOUR

DEFINED AS

! This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995"::subsystem is used as a common containment point for

managed objects in a system that relate to the presentation

layer (i.e., presentation entity MOs). It can use the"Rec.

X.723 | ISO/IEC 10165-5 : 1995": subsystem-system Name

Binding. !;;;;

REGISTERED AS {UL-AttributeModule.presentationSubsystemMOCId};

sSap MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":sap1;

CHARACTERIZED BY

sSapP1 PACKAGE

BEHAVIOUR

sSapB1 BEHAVIOUR

DEFINED AS

!This managed object is a subclass of "Rec. X.723 | ISO/IEC

10165-5 : 1995":sap1 and has the S- selector and binding

between PE and SAP.!;;;;

REGISTERED AS {UL-AttributeModule.sSapMOCId};

sessionConnection MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":singlePeerConnection;

CHARACTERIZED BY

sessionConnEndpointP1 PACKAGE

BEHAVIOUR

sessionConnEndpointBehaviour BEHAVIOUR

DEFINED AS

!This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995":singlePeerConnection holds reference information

pertaining to the single session connection for the

association. The underlying connections attribute is used

to point to the transport connection used.

It specializes by adding the following attributes:

- Session protocol version in use;

- Session functional units negotiated in use;

- it may have an attribute for the negotiated maximum TSDU

size if segmenting is in use.   !;;

ATTRIBUTES

sessProtInUse GET,

sessFUinUse GET;;;

CONDITIONAL PACKAGES

maxTSDUsizePackage PACKAGE

ATTRIBUTES

maxTSDUsize GET;

REGISTERED AS {UL-AttributeModule.maxTSDUsizePId};

PRESENT IF  "Present only if segmenting is in use.";

REGISTERED AS {UL-AttributeModule.sessionConnectionMOCId};

sessionCopm MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":coProtocolMachine;

CHARACTERIZED BY

sessionCopmP1 PACKAGE

BEHAVIOUR

sessionCopmBehaviour BEHAVIOUR

DEFINED AS

!This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995":coProtocolMachine holds reference information

pertaining to the capabilities of a Session protocol

machine.

It specializes by adding the following attributes:

- Session protocol versions supported (bitstring);

- Session functional units supported (bitstring);

- optional support requirements (i.e., segmenting).  !;;

ATTRIBUTES

sessProtVerSupport GET,

sessFUsupport GET,

sessOptSupport GET;;;

REGISTERED AS {UL-AttributeModule.sessionCopmMOCId};

sessionEntity MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntity;

CHARACTERIZED BY

sessionEntityP1 PACKAGE

BEHAVIOUR

sessionEntityB1 BEHAVIOUR

DEFINED AS

"This managed object holds reference information about the

session entity and is a subclass of

GMI:communicationsEntity.";;;;

REGISTERED AS {UL-AttributeModule.sessionEntityMOCId};

sessionSubsystem MANAGED OBJECT CLASS

DERIVED FROM      "Rec. X.723 | ISO/IEC 10165-5 : 1995":subsystem;

CHARACTERIZED BY

sessionSubsystemPackage PACKAGE

BEHAVIOUR

sessionSubsystemBehaviour BEHAVIOUR

DEFINED AS

! This subclass of "Rec. X.723 | ISO/IEC 10165-5 :

1995":subsystem is used as a common containment point for

managed objects in a system that relate to the session

layer (i.e., session entity MOs). It can use the "Rec.

X.723 | ISO/IEC 10165-5 : 1995":subsystem-system Name

Binding. !;;;;

REGISTERED AS {UL-AttributeModule.sessionSubsystemMOCId};

abstrSyntaxSupport ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.AbstrSyntaxSupport;

REGISTERED AS {UL-AttributeModule.abstrSyntaxSupportAId};

acseFUinUse ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.AcseFUs;

REGISTERED AS {UL-AttributeModule.acseFUinUseAId};

acseFUinitiator ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.AcseFUs;

REGISTERED AS {UL-AttributeModule.acseFUinitiatorAId};

acseFUresponder ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.AcseFUs;

REGISTERED AS {UL-AttributeModule.acseFUresponderAId};

aeInvocationId ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.AeInvocationId;

REGISTERED AS {UL-AttributeModule.aeInvocationIdAId};

apInvocationId ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.ApInvocationId;

REGISTERED AS {UL-AttributeModule.ulAttributeIDAId};

apInvocationPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.ApInvocation;

REGISTERED AS {UL-AttributeModule.apInvocationIdAId};

applContextInUse ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.ApplContextInUse;

REGISTERED AS {UL-AttributeModule.applContextInUseAId};

applContextNameSupport ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.ApplContextNameSupport;

REGISTERED AS {UL-AttributeModule.applContextNameSupportAId};

asoQualifier ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.ASOQualifier;

REGISTERED AS {UL-AttributeModule.asoQualifierAId};

asoTitle ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.ASOTitle;

REGISTERED AS {UL-AttributeModule.asoTitleAId};

calledAEtitle ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.CallAEtitle;

REGISTERED AS {UL-AttributeModule.calledAEtitleAId};

callingAEtitle ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.CallAEtitle;

REGISTERED AS {UL-AttributeModule.callingAEtitleAId};

invocationOfPointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.ObjectInstance;

REGISTERED AS {UL-AttributeModule.invocationOfPointerAId};

maxTSDUsize ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.MaxTSDUsize;

REGISTERED AS {UL-AttributeModule.maxTSDUsizeAId};

parentASOinvoc ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.ParentASOinvoc;

REGISTERED AS {UL-AttributeModule.parentASOinvocAId};

presContextInUse ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.PresContextInUse;

REGISTERED AS {UL-AttributeModule.presContextInUseAId};

presFUinUse ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.PresFUs;

REGISTERED AS {UL-AttributeModule.presFUinUseAId};

presFUsupport ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.PresFUs;

REGISTERED AS {UL-AttributeModule.presFUsupportAId};

presSelectorValue ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.PresSelectorValue;

REGISTERED AS {UL-AttributeModule.presSelectorValueAId};

sessFUinUse ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.SessFUs;

REGISTERED AS {UL-AttributeModule.sessFUinUseAId};

sessFUsupport ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.SessFUs;

REGISTERED AS {UL-AttributeModule.sessFUsupportAId};

sessOptSupport ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.SessOptSupport;

REGISTERED AS {UL-AttributeModule.sessOptSupportAId};

sessProtInUse ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.SessProtVer;

REGISTERED AS {UL-AttributeModule.sessProtInUseAId};

sessProtVerSupport ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.SessProtVer;

REGISTERED AS {UL-AttributeModule.sessProtVerSupportAId};

sessionSelectorValue ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.SessionSelectorValue;

REGISTERED AS {UL-AttributeModule.sessionSelectorValueAId};

supportingServices ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.SupportingServices;

REGISTERED AS {UL-AttributeModule.supportingServicesAId};

transfSyntaxSupport ATTRIBUTE

WITH ATTRIBUTE SYNTAX   UL-AttributeModule.TransfSyntaxSupport;

REGISTERED AS {UL-AttributeModule.transfSyntaxSupportAId};

ap-applicationSubsystem NAME BINDING

SUBORDINATE OBJECT CLASS "Rec. X.723 | ISO/IEC 10165-5 : 1995":applicationProcess AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS applicationSubsystem AND SUBCLASSES;

WITH ATTRIBUTE "Rec. X.723 | ISO/IEC 10165-5 : 1995":applicationProcessId;

REGISTERED AS {UL-AttributeModule.ap-applicationSubsystemNBId};

apInvocation-applicationSubsystem NAME BINDING

SUBORDINATE OBJECT CLASS apInvocation AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS applicationSubsystem AND SUBCLASSES;

WITH ATTRIBUTE apInvocationId;

REGISTERED AS {UL-AttributeModule.apInvocation-applicationSubsystemNBId};

aso-aso NAME BINDING

SUBORDINATE OBJECT CLASS aso AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS aso AND SUBCLASSES;

WITH ATTRIBUTE "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntityId;

REGISTERED AS {UL-AttributeModule.aso-asoNBId};

aso-asoEntity NAME BINDING

SUBORDINATE OBJECT CLASS aso AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS asoEntity AND SUBCLASSES;

WITH ATTRIBUTE "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntityId;

REGISTERED AS {UL-AttributeModule.aso-asoEntityNBId};

asoEntity-applicationProcess NAME BINDING

SUBORDINATE OBJECT CLASS asoEntity AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS "Rec. X.723 | ISO/IEC 10165-5 : 1995":applicationProcess AND SUBCLASSES;

WITH ATTRIBUTE "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntityId;

REGISTERED AS {UL-AttributeModule.asoEntity-applicationProcessNBId};

asoInvocation-apInvocation NAME BINDING

SUBORDINATE OBJECT CLASS asoInvocation AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS apInvocation AND SUBCLASSES;

WITH ATTRIBUTE "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntityId;

REGISTERED AS {UL-AttributeModule.asoInvocation-apInvocationNBId};

asoInvocation-asoInvocation NAME BINDING

SUBORDINATE OBJECT CLASS asoInvocation AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS asoInvocation AND SUBCLASSES;

WITH ATTRIBUTE "Rec. X.723 | ISO/IEC 10165-5 : 1995":communicationsEntityId;

REGISTERED AS {UL-AttributeModule.asoInvocation-asoInvocationNBId};

UL-AttributeModule {joint-iso-ccitt ms(9) smi(3) part8(8) asn1Module(2) 0 }

--<ASN1.Version 1990,1994 UL-AttributeModule

--{joint-iso-ccitt ms(9) smi(3) part8(8) asn1Module(2) 0 }>--

DEFINITIONS ::=

BEGIN

--EXPORTS everything

IMPORTS

RelativeDistinguishedName

FROM InformationFramework {joint-iso-ccitt ds(5) modules(1) informationFramework(1) }

AP-title, Application-context-name, AE-invocation-identifier, AP-invocation-identifier, AE-qualifier, AE-title, ACSE-requirements

FROM ACSE-1 {joint-iso-ccitt association-control(2) abstract-syntax(1) apdus(0) version(1)}

Presentation-requirements,  Presentation-selector, Context-list, Abstract-syntax-name,

Transfer-syntax-name

FROM ISO8823-PRESENTATION

ObjectInstance

FROM CMIP-1 {joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3) };

--value references--

ulMObjectClass OBJECT IDENTIFIER ::= {joint-iso-ccitt ms(9) smi(3) part8(8) managedObjectClass(3) }

ulMoPackage OBJECT IDENTIFIER ::= {joint-iso-ccitt ms(9) smi(3) part8(8) package(4) }

ulAttributeID OBJECT IDENTIFIER ::= {joint-iso-ccitt ms(9) smi(3) part8(8) attribute(7) }

ulNameBinding OBJECT IDENTIFIER ::= {joint-iso-ccitt ms(9) smi(3) part8(8) nameBinding(6) }

acseMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 1}

acseAssociationMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 2}

apInvocationMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 3}

applicationEntityMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 4}

applicationEntityInvocationMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 5}

applicationSubsystemMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 6}

asoMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 7}

asoEntityMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 8}

asoInvocationMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 9}

monoULConnectionMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 10}

monoULEntityMOCId  OBJECT IDENTIFIER ::= {ulMObjectClass 11}

monolithicUpperLayerSubsystemMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 12}

pSapMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 13}

presentationConnectionMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 14}

presentationCopmMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 15}

presentationEntityMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 16}

presentationSubsystemMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 17}

sSapMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 18}

sessionConnectionMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 19}

sessionCopmMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 20}

sessionEntityMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 21}

sessionSubsystemMOCId OBJECT IDENTIFIER ::= {ulMObjectClass 22}

calledAEtitlePId OBJECT IDENTIFIER ::= {ulMoPackage 1}

maxTSDUsizePId OBJECT IDENTIFIER ::= {ulMoPackage 2}

abstrSyntaxSupportAId OBJECT IDENTIFIER ::= {ulAttributeID 1}

acseFUinUseAId OBJECT IDENTIFIER ::= {ulAttributeID 2}

acseFUinitiatorAId OBJECT IDENTIFIER ::= {ulAttributeID 3}

acseFUresponderAId OBJECT IDENTIFIER ::= {ulAttributeID 4}

aeInvocationIdAId OBJECT IDENTIFIER ::= {ulAttributeID 5}

ulAttributeIDAId OBJECT IDENTIFIER ::= {ulAttributeID 6}

apInvocationIdAId OBJECT IDENTIFIER ::= {ulAttributeID 7}

applContextInUseAId OBJECT IDENTIFIER ::= {ulAttributeID 8}

applContextNameSupportAId OBJECT IDENTIFIER ::= {ulAttributeID 9}

asoQualifierAId OBJECT IDENTIFIER ::= {ulAttributeID 10}

asoTitleAId OBJECT IDENTIFIER ::= {ulAttributeID 11}

calledAEtitleAId OBJECT IDENTIFIER ::= {ulAttributeID 12}

callingAEtitleAId OBJECT IDENTIFIER ::= {ulAttributeID 13}

invocationOfPointerAId OBJECT IDENTIFIER ::= {ulAttributeID 14}

maxTSDUsizeAId OBJECT IDENTIFIER ::= {ulAttributeID 15}

parentASOinvocAId OBJECT IDENTIFIER ::= {ulAttributeID 16}

presContextInUseAId OBJECT IDENTIFIER ::= {ulAttributeID 17}

presFUinUseAId OBJECT IDENTIFIER ::= {ulAttributeID 18}

presFUsupportAId OBJECT IDENTIFIER ::= {ulAttributeID 19}

presSelectorValueAId OBJECT IDENTIFIER ::= {ulAttributeID 20}

sessFUinUseAId OBJECT IDENTIFIER ::= {ulAttributeID 21}

sessFUsupportAId OBJECT IDENTIFIER ::= {ulAttributeID 22}

sessOptSupportAId OBJECT IDENTIFIER ::= {ulAttributeID 23}

sessProtInUseAId OBJECT IDENTIFIER ::= {ulAttributeID 24}

sessProtVerSupportAId OBJECT IDENTIFIER ::= {ulAttributeID 25}

sessionSelectorValueAId OBJECT IDENTIFIER ::= {ulAttributeID 26}

supportingServicesAId OBJECT IDENTIFIER ::= {ulAttributeID 27}

transfSyntaxSupportAId OBJECT IDENTIFIER ::= {ulAttributeID 28}

ap-applicationSubsystemNBId OBJECT IDENTIFIER ::= {ulNameBinding 1}

apInvocation-applicationSubsystemNBId OBJECT IDENTIFIER ::= {ulNameBinding 2}

aso-asoNBId OBJECT IDENTIFIER ::= {ulNameBinding 3}

aso-asoEntityNBId OBJECT IDENTIFIER ::= {ulNameBinding 4}

asoEntity-applicationProcessNBId OBJECT IDENTIFIER ::= {ulNameBinding 5}

asoInvocation-apInvocationNBId OBJECT IDENTIFIER ::= {ulNameBinding 6}

asoInvocation-asoInvocationNBId OBJECT IDENTIFIER ::= {ulNameBinding 7}

--Type references--

AbstrSyntaxSupport ::= SET OF Abstract-syntax-name

AcseFUs ::= ACSE-requirements  -- from ammendment 1 to acse protocol

AeInvocationId   ::=  AE-invocation-identifier

ApInvocationId  ::=  AP-invocation-identifier

ApInvocation ::= AP-invocation-identifier

ApplContextInUse   ::=  Application-context-name

ApplContextNameSupport ::=  SET OF Application-context-name

AsoInvocId  ::=  INTEGER  -- This should be compatible syntax with AeInvocationId

ASOId ::= GraphicString

ASOInvocationId ::= GraphicString

ASOTitle ::= SEQUENCE {

ap-title	AP-title,

aso-name	SEQUENCE OF ASOQualifier}

ASOQualifier ::= RelativeDistinguishedName     --of ASO--

CallAEtitle  ::=   AE-title

LocalAEqualifier ::=  AE-qualifier

MaxTSDUsize  ::=   INTEGER -- four octets maximum length

ParentASOinvoc  ::=   ObjectInstance

PresContextInUse ::=  Context-list -- only one transfer syntax used for each abstract syntax

PresFUs   ::= Presentation-requirements

PresSelectorValue  ::=  Presentation-selector

SessFUs ::=  BIT STRING {	half-duplex 		(0),

duplex 			(1),

expedited-data		(2),

minor-synchronize 	(3),

major-synchronize 	(4),

resynchronize		(5),

activity-management 	(6),

negotiated-release 	(7),

capability-data		(8),

exceptions		(9),

typed-data		(10) }

SessionSelectorValue ::=  OCTET STRING -- 16 octets max

SessOptSupport  ::= BIT STRING { segmenting (0) }

SessProtVer ::= BIT STRING { 	version1 		(0),

version2 		(1) }

SupportingServices ::= SET OF ObjectInstance

TransfSyntaxSupport ::= SET OF Transfer-syntax-name

END	-- of UL-AttributeModule

--<GDMO.EndDocument "Draft Rec. X.726 | ISO/IEC CD 10165-8 : 1997"

-- {joint-iso-ccitt ms(9) smi(3) part8(8)>--

Annex C

ICS Proforma

(This annex forms an integral part of this Recomendation | International Standard.)

<To Be Provided>

EDITOR'S NOTE - National Body Comment is invicted.

Annex D

List of Open Issues

(This annex does not form an integral part of this Recomendation | International Standard.)

1	The definitions do not specify attributes for statistical counters associated with upper layer connections. Some National Bodies (e.g., AFNOR, contribution WG8 N109) expressed concern that statistics should be included in Association endpoints and upper layer connection objects. Should statistical counters be included in the standard definitions, or should they be constrained to being added to subclasses of the definitions in this document ?

STATUS: Closed. Counters are included in subclasses.

Editors Note: It would propably be a good idea to define counters and communication alarms in conditional packages, so we can get a standardized interface, but not put severe demands on implementations.

2	Should the definitions allow different managed object designs for the separable upper layer implementations (clause 7.4) and monolithic upper layer implementations (clause 7.5) ?

One position held strongly by at least one NB is that two alternative models for the upper layers will only cause unnecessary complexity in implementations and interworking problems. Consequently, a single, monolithic model should be adopted.

Another position, also held strongly by at least another NB, was that both (monolithic and separate layers) models should be supported. Each of these MOs represent aspects of a process. A monolithic implementation may have one process, but other implementations are not monolithic and must be represented with multiple processes. The failure modes of different implementations vary. These generic definitions must therefore anticipate refinements to implementation specific MOs. Consequently, the standard/recommendation should not impose one implementation/model.

Only a single naming attribute should be used so that the monolithic managed object class can multiply inherit from the separate managed object classes or so that implementations of the monolithic managed object class can be allomorphic to the separate layer classes.

NB comments are requested.

STATUS: Open

3	Should the definitions use additional Systems Management function capabilities, such as the Alarm Reporting Notifications, Security Alarm Reporting Notifications, and Security Audit Trail Notifications ?

If other notifications are needed, NB are requested to provide input. For the security package, the following text is suggested:

Security Conditional Package (for ACSE COPM MO)

Security Additions

ISO/IEC 10164-7, Security Alarm Reporting Function, specifies generic notifications to be used when security errors are detected. ISO/IEC 10164-8, Security Audit Trail Function, specifies generic notifications to be used when non-alarm security events occur. Both types of notifications can be sent to a manager (which may be logged by the manager) and/or logged in a local log according to values of Event Forwarding Discriminators and Log Discriminators in the system in which the security related event occurs.

The Association Security Addition defines a conditional package that, in turn, defines the circumstances in which these security related notifications are appropriate.

If the ACSE Peer Entity Authentication mechanism is used, a security notification characterized by Event Type: Security Service or Mechanism Violation and Security Alarm Causes: Authentication Failure (from ISO/IEC 10164-7) may be emitted when the mechanism does not permit establishment of the association.

The types of events for which Service Report Notifications are to be emitted, and the related Service Report Causes are: establishment of association (request for service), failure to establish association (denial of service), association abort (service failure), and termination of association (response from service).

Note: Whether the relationship between ACSE events and the Service Report Causes noted above are the most appropriate, and the relationship between denial of service and the proposed security alarm is for further study.

STATUS: Closed. this is included in subclasses.

4	Should the names for the upper layer subclasses of GMI: singlePeerConnection be harmonized ? The current document uses "ACSE invocation", "presentation connection", and "session connection". In all these cases, the managed objects model one endpoint's view of an upper layer connection.

STATUS: Open

Editors Note: ACSE Association has been proposed by the editor.

5	Should ROSE MOs be included? (NB comments are requested.)

STATUS: Closed. ROSE MOs are not necessary.

6	Should this be a separate standard, an amendment to GMI ISO 10165-5, or else ? NB comments are requested.

STATUS: Closed. Sepearate standard.

7	Should the relationship between entities/protocol machines and their respective invocations be modelled by containment or pointer?

Preliminary resolution: containment, for consistency with lower layers and GMI.

8	How should the naming of the application entity invocation be handled? The paragraph on OSI naming and addressing does not appear to be relevant to the management requirements of this object.
Preliminary resolution: The AEI is contained by an AE. Its distinguished attribute, with syntax Integer, is the AEI identifier.

9	Is the relationship between PSAPs and AE really many to many? If so, in which directions does the relationship need to be traceable by management?

10	What is an ASO? Are ASOs relevant from a management point of view?
Preliminary resolution: ASOs are not relevant to management.

11	What is the difference between ASE and ASO? 
Preliminary resolution: only the names appear different, and, if an alternatve containment is used, there is no structural or functional difference from a management perspective.

Editors Note: ASEs are contained in ASOs in ALS. ASOs are structural ÒcontainmentÓ concept. ASEs represent communication functionality.

12	Should the application layer maintain a certain parallelism of structure with respect to the other layers, e.g. would an ASE be contained by a subsystem?

Preliminary resolution: yes, this makes for more consistency and a simpler representation which is still capable of representing both the depth and breadth of recursion.

Editors Note: The ASE could be defined as a subclass of Communications Entity, to enable the NB subsystem-ASE.

Also, a NB AE-AS could be defined, so that ASEs could be contained within its AE.

13	What is a CF and do these need to be visible to management?
Preliminary resolution: No.

14	From a management perspective, it appears to make sense to collapse entities(ASEs) and Protocol Machines. Do we have a multi-protocol ASE, or other good reason to not collapse them?

Editors Note: Yes, there are multi-PM ASEs. For example connectionless and connectionoriented ACSE.

15	Multiple Aes may make use of the same Protocol Machine, and multiple Protocol Machines may be used by a single AE. Therefore the relationship between them should not be modelled by containment.

16	The AP contains both the APIs, as well as the AEs. AEs may contain AEIs. ASEs either contain(or are the same object as) their PM. PMs may contain PM invocations(connections). the relationships between the various invocations are represented by one-way(set valued?) pointers going down the stack. The relationships between AEs and ASEs and ASEs and PSAPs are represented by multi valued one-way pointers going down the stack.

17 	How are existing management standards in the application layer structured?

18 	Should we define subsystem Mos?

19	Should we define connectionless Pms?

20 	Is the AEI MO necessary?

21	Where is API defined?

22. 	Should application entity be subclassed from "GMI"coProtocolMachine in addition to "GMI"communicationsEntity and aso? Although this doesn't add much in the way of attributes, it does add more in terms of behaviour and consistency of the model.

ShS