AIDC SARPs Requirements Decomposition

FAA AOP-600

8/31/98

| Paragraph | Requirement |  | Usage | Implementation |  | Test |
| --- | --- | --- | --- | --- | --- | --- |
| 3.2.2.2.1 | The AIDC application shall utilize the Upper Layer Architecture as defined in Sub-Volume IV. |  | mandatory |  |  |  |
| 3.2.2.2.2 | The AIDC application shall use the Naming, Addressing, Presentation Context Identification and Registration from 4.3.2 |  | mandatory |  |  |  |
| 3.2.2.2.2 | The AIDC application shall use the APRL for Session from 4.4 |  | mandatory |  |  |  |
| 3.2.2.2.2 | The AIDC application shall use the APRL for Presentation from 4.5 |  | mandatory |  |  |  |
| 3.2.2.2.2 | The AIDC application shall use the APRL for ACSE as specified in 4.6. |  | mandatory |  |  |  |
| 3.2.3.1.1.1 | The Called ICAO Facility Designation parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.1.1.2 | The Called ICAO Facility Designation parameter shall be the called ATS system’s ICAO four-letter location indicator or the ICAO eight-letter combined location indicator, three letters designator and an additional letter. |  | mandatory |  |  |  |
| 3.2.3.1.2.1 | The Calling ICAO Facility Designation parameter shall be provided by the AIDC-AE. |  | mandatory |  |  |  |
| 3.2.3.1.2.2 | The Calling ICAO Facility Designation parameter shall be the calling ATS system’s ICAO four-letter location indicator or the ICAO eight-letter combined location indicator, three letters designator and an additional letter. |  | mandatory |  |  |  |
| 3.2.3.1.3.1 | The Message Number parameter shall be provided by the AIDC-User except when invoking the User-abort request primitive. |  | mandatory |  |  |  |
| 3.2.3.1.3.1 | The Message Number parameter shall be provided by the AIDC-User except when invoking the User-confirmation request primitive. |  | mandatory |  |  |  |
| 3.2.3.1.3.2 | The Message Number parameter value shall consist of a unique identifier for reference in the User-confirmation. |  | mandatory |  |  |  |
| 3.2.3.1.3.3 | The Message Number parameter shall conform to the ASN.1 abstract syntax of MessageNumber. |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with User-confirmation service as defined in 3.2.3.4. |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Notify service as defined in 3.2.3.5.1 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Coordinate-start service as defined in 3.2.3.6.4 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Coordinate-end service as defined in 3.2.3.6.5 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Coordinate-negotiate service as defined in 3.2.3.6.6 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Coordinate-standby service as defined in 3.2.3.6.7 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Transfer-initiate service as defined in 3.2.3.7.3 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Transfer-request service as defined in 3.2.3.7.4 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Transfer-conditions-proposal service as defined in 3.2.3.7.5 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Transfer-conditions-accept service as defined in 3.2.3.7.6 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Transfer-control service as defined in 3.2.3.7.7 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Transfer-communication service as defined in 3.2.3.7.8 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Transfer-communication-assume service as defined in 3.2.3.7.9 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Info-transfer service as defined in 3.2.3.8.1 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with End service as defined in 3.2.3.9.1 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with User-abort service as defined in 3.2.3.9.2 |  | mandatory |  |  |  |
| 3.2.3.2.1 | The AIDC service shall exhibit external behavior consistent with an AIDC-AE with Provider-abort service as defined in 3.2.3.9.3 |  | mandatory |  |  |  |
| 3.2.3.3.1 | The AIDC-User Service shall consist of a Notifying regime |  | mandatory |  |  |  |
| 3.2.3.3.1 | The AIDC-User Service shall consist of a Coordinating regime |  | mandatory |  |  |  |
| 3.2.3.3.1 | The AIDC-User Service shall consist of a Transferring regime. |  | mandatory |  |  |  |
| 3.2.3.3.2 | The AIDC-User Service shall consist of three regimes which occur in the sequence: Notifying regime, Coordinating regime, Transferring regime. |  | mandatory |  |  |  |
| 3.2.3.3.3 | The Coordinating regime shall be allowed to be re-entered at the end of the Transferring regime. |  | mandatory |  |  |  |
| 3.2.3.3.4 | The Notifying regime should consist of the Notify service. |  | recommendation |  |  |  |
| 3.2.3.3.5 | The Coordinating regime should consist of the Coordinate-start service |  | recommendation |  |  |  |
| 3.2.3.3.5 | The Coordinating regime should consist of the Coordinate-standby service |  | recommendation |  |  |  |
| 3.2.3.3.5 | The Coordinating regime should consist of the Coordinate-standby service |  | recommendation |  |  |  |
| 3.2.3.3.5 | The Coordinating regime should consist of the Coordinate-end service |  | recommendation |  |  |  |
| 3.2.3.3.6 | The Transferring regime should consist of the Transfer-initiate service |  | recommendation |  |  |  |
| 3.2.3.3.6 | The Transferring regime should consist of the Transfer-conditions-proposal service |  | recommendation |  |  |  |
| 3.2.3.3.6 | The Transferring regime should consist of the Transfer-conditions-accept service |  | recommendation |  |  |  |
| 3.2.3.3.6 | The Transferring regime should consist of the Transfer-control service |  | recommendation |  |  |  |
| 3.2.3.3.6 | The Transferring regime should consist of the Transfer-request service |  | recommendation |  |  |  |
| 3.2.3.3.6 | The Transferring regime should consist of the Transfer-communication service |  | recommendation |  |  |  |
| 3.2.3.3.6 | The Transferring regime should consist of the Transfer-communication service |  | recommendation |  |  |  |
| 3.2.3.3.6 | The Transferring regime should consist of the Transfer-communication-assume service. |  | recommendation |  |  |  |
| 3.2.3.4.1 | Upon the receipt of any primitive indication containing an Information parameter (e.g. Notification Information, Coordinate Start Information), the AIDC-User shall validate the value of this parameter and respond with a User-confirmation request primitive. |  | mandatory |  |  |  |
| 3.2.3.4.2 | The User-confirmation service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.4.3 | When the user-confirmation service request is invoked, passing the Result parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.4.3 | When the user-confirmation service indication is invoked, passing the Result parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.4.3 | When the user-confirmation service request is invoked, passing the Reason parameter is a user option. |  | user option |  |  |  |
| 3.2.3.4.3 | When the user-confirmation service indication is invoked, passing the Reason parameter is conditional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.3.4.3 | When the user-confirmation service request is invoked, passing the Referenced Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.4.3 | When the user-confirmation service indication is invoked, passing the Referenced Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.4.4.1 | The Result parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.4.4.2 | The Result parameter shall conform to the ASN.1 abstract syntax of Result. |  | mandatory |  |  |  |
| 3.2.3.4.5.1 | The Reason parameter should be provided by the AIDC-User when the Response parameter has the abstract value of "rejected". |  | recommendation |  |  |  |
| 3.2.3.4.5.2 | The Reason parameter shall conform to the ASN.1 abstract syntax ApplicationErrorData. |  | mandatory |  |  |  |
| 3.2.3.4.6.1 | The Referenced Number parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.4.6.2 | The Referenced Number parameter shall contain the Message Number of the message being confirmed. |  | mandatory |  |  |  |
| 3.2.3.4.6.3 | T he Referenced Number parameter shall conform to the ASN.1 abstract syntax of MessageNumber. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.1 | The Notify service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.2 | When the Notify service request is invoked, passing the Called ICAO Facility Designation parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.2 | When the Notify service indication is invoked, passing the Called ICAO Facility Designation parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.5.1.1.2 | When the Notify service request is invoked, passing the Calling ICAO Facility Designation parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.5.1.1.2 | When the Notify service indication is invoked, passing the Calling ICAO Facility Designation parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.2 | When the Notify service request is invoked, passing the Notification Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.2 | When the Notify service indication is invoked, passing the Notification Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.2 | When the Notify service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.2 | When the Notify service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.3.1 | The Notification Information parameter shall be provided by the AIDC-User when invoking the Notify request primitive. |  | mandatory |  |  |  |
| 3.2.3.5.1.1.3.2 | The Notification Information parameter shall conform to the ASN.1 abstract syntax Notify. |  | mandatory |  |  |  |
| 3.2.3.6.1 | The Coordinating regime shall begin with the invocation of a Coordinate-start primitive. |  | mandatory |  |  |  |
| 3.2.3.6.2 | The Coordinating regime shall end with the invocation of a Coordinate-end primitive. |  | mandatory |  |  |  |
| 3.2.3.6.3 | Upon entering the Coordinating regime, further use of the Notify service shall be prohibited. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.1 | The Coordinate-Start service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.2 | When the Coordinate-Start service request is invoked, passing the Called ICAO Facility Designation parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.2 | When the Coordinate-Start service indication is invoked, passing the Called ICAO Facility Designation parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.6.4.1.2 | When the Coordinate-Start service request is invoked, passing the Calling ICAO Facility Designation parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.6.4.1.2 | When the Coordinate-Start service indication is invoked, passing the Calling ICAO Facility Designation parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.2 | When the Coordinate-Start service request is invoked, passing the Coordinate Start Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.2 | When the Coordinate-Start service indication is invoked, passing the Coordinate Start Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.2 | When the Coordinate-Start service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.2 | When the Coordinate-Start service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.3.1 | The Coordinate Start Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.6.4.1.3.2 | The Coordinate Start Information parameter shall conform to the ASN.1 abstract syntax of CoordinateInitial or CoordinateUpdate. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.1 | The Coordinate-End service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.2 | The Coordinate-End service primitives shall only be invoked while in the Coordinating regime. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.3 | When the Coordinate-End service request is invoked, passing the Coordinate End Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.3 | When the Coordinate-End service indication is invoked, passing the Coordinate End Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.3 | When the Coordinate-End service request is invoked, passing the result parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.3 | When the Coordinate-End service indication is invoked, passing the result parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.6.5.1.3 | When the Coordinate-End service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.3 | When the Coordinate-End service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.4.1 | The Coordinate End Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.4.2 | The Coordinate End Information parameter shall conform to the ASN.1 abstract syntax of CoordinateAccept or CoordinateReject. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.5.1 | The Result parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.6.5.1.5.2 | The Result parameter shall conform to the ASN.1 abstract syntax of Result. |  | mandatory |  |  |  |
| 3.2.3.6.6.1.1 | The Coordinate-Negotiate service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.6.6.1.2 | The Coordinate-Negotiate service primitives shall only be invoked while in the Coordinating regime. |  | mandatory |  |  |  |
| 3.2.3.6.6.1.3 | When the Coordinate-Negotiate service request is invoked, passing the Coordinate Negotiate Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.6.1.3 | When the Coordinate-Negotiate service indication is invoked, passing the Coordinate Negotiate Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.6.6.1.3 | When the Coordinate-Negotiate service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.6.1.3 | When the Coordinate-Negotiate service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.6.6.1.4.1 | The Coordinate Negotiate Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.6.6.1.4.2 | The Coordinate Negotiate Information parameter shall conform to the ASN.1 abstract syntax CoordinateNegotiate. |  | mandatory |  |  |  |
| 3.2.3.6.7.1.1 | The Coordinate-Standby service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.6.7.1.2 | The Coordinate-Standby service primitives shall only be invoked while in the Coordinating regime. |  | mandatory |  |  |  |
| 3.2.3.6.7.1.3 | When the Coordinate-Standby service request is invoked, passing the Coordinate Standby Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.7.1.3 | When the Coordinate-Standby service indication is invoked, passing the Coordinate Standby Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.6.7.1.3 | When the Coordinate-Standby service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.6.7.1.3 | When the Coordinate-Standby service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.6.7.1.4.1 | The Coordinate Standby Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.6.7.1.4.2 | The Coordinate Standby Information parameter shall conform to the ASN.1 abstract syntax CoordinateStandby. |  | mandatory |  |  |  |
| 3.2.3.7.1 | The Transferring regime shall be entered after the completion of the Coordinating regime. |  | mandatory |  |  |  |
| 3.2.3.7.2 | The Transferring regime shall begin with the invocation of any of the following: the Transfer-initiate service. |  | mandatory |  |  |  |
| 3.2.3.7.2 | The Transferring regime shall begin with the invocation of any of the following: the Transfer-request service. |  | mandatory |  |  |  |
| 3.2.3.7.2 | The Transferring regime shall begin with the invocation of any of the following: the Transfer-control service. |  | mandatory |  |  |  |
| 3.2.3.7.3.1.1 | The Transfer-initiate service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.7.3.1.2 | When the Transfer-Initiate service request is invoked, passing the Transfer Initiate Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.3.1.2 | When the Transfer-Initiate service indication is invoked, passing the Transfer Initiate Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.3.1.2 | When the Transfer-Initiate service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.3.1.2 | When the Transfer-Initiate service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.3.1.3.1 | The Transfer Initiate Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.7.3.1.3.2 | The Transfer Initiate Information parameter shall conform to the ASN.1 abstract syntax TransferInitiate. |  | mandatory |  |  |  |
| 3.2.3.7.4.1.1 | The Transfer-request service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.7.4.1.2 | When the Transfer-Request service request is invoked, passing the Transfer Request Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.4.1.2 | When the Transfer-Request service indication is invoked, passing the Transfer Request Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.4.1.2 | When the Transfer-Request service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.4.1.2 | When the Transfer-Request service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.4.1.3.1 | The Transfer Request Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.7.4.1.3.2 | The Transfer Request Information parameter shall conform to the ASN.1 abstract syntax TransferRequest. |  | mandatory |  |  |  |
| 3.2.3.7.5.1.1 | The Transfer-conditions-proposal service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.7.5.1.2 | The Transfer-conditions-proposal service primitives shall only be invoked after the successful completion of the Transfer-initiate service. |  | mandatory |  |  |  |
| 3.2.3.7.5.1.3 | When the Transfer-Conditions-Proposal service request is invoked, passing the Transfer Conditions Proposal Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.5.1.3 | When the Transfer-Conditions-Proposal service indication is invoked, passing the Transfer Conditions Proposal Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.5.1.3 | When the Transfer-Conditions-Proposal service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.5.1.3 | When the Transfer-Conditions-Proposal service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.5.1.4.1 | The Transfer Conditions Proposal Information parameter is provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.7.5.1.4.2 | The Transfer Conditions Proposal Information parameter shall conform to the ASN.1 abstract syntax TransferConditionsProposal. |  | mandatory |  |  |  |
| 3.2.3.7.6.1.1 | The Transfer-conditions-accept service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.7.6.1.2 | The Transfer-conditions-accept service primitives shall only be invoked after the successful completion of the Transfer-conditions-proposal service. |  | mandatory |  |  |  |
| 3.2.3.7.6.1.3 | When the Transfer-Conditions-Accept service request is invoked, passing the Transfer Conditions Accept Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.6.1.3 | When the Transfer-Conditions-Accept service indication is invoked, passing the Transfer Conditions Accept Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.6.1.3 | When the Transfer-Conditions-Accept service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.6.1.3 | When the Transfer-Conditions-Accept service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.6.1.4.1 | The Transfer Conditions Accept Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.7.6.1.4.2 | The Transfer Conditions Accept Information parameter shall conform to the ASN.1 abstract syntax TransferConditionsAccept. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.1 | The Transfer-control service shall be a confirmed service. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service request is invoked, passing the Transfer Control Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service indication is invoked, passing the Transfer Control Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service response is invoked, passing the Transfer Control Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service confirmation is invoked, passing the Transfer Control Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service request is invoked, passing the Result parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service indication is invoked, passing the Result parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service response is invoked, passing the Result parameter is not applicable. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service confirmation is invoked, passing the Result parameter is mandatory |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service request is invoked, passing the Message Number parameter is mandatory with the same data as the request |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service response is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.2 | When the Transfer-Control service confirmation is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.3.1 | The Transfer Control Information parameter shall be provided by the C-ATSU AIDC-User when invoking the Transfer-control request primitive. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.3.2 | The Transfer Control Information parameter shall conform to the ASN.1 abstract syntax TransferControl, when the C-ATSU AIDC-User invokes the Transfer-control request primitive. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.3.3 | The Transfer Control Information parameter shall be provided by the R-ATSU AIDC-User when invoking the Transfer-control response service primitive. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.3.4 | The Transfer Control Information parameter shall conform to the ASN.1 abstract syntax TransferControlData, when the R-ATSU AIDC-User invokes the Transfer-control response primitive. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.4.1 | The Result parameter shall be provided by the C-ATSU AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.4.2 | The Result parameter shall conform to the ASN.1 abstract syntax Result. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.4.3 | When the Result parameter value is set to "accept", the Transfer Control Information parameter shall conform to the ASN.1 abstract syntax TransferControlAssume. |  | mandatory |  |  |  |
| 3.2.3.7.7.1.4.4 | When the Result parameter value is set to "reject", the Transfer Control Information parameter shall conform to the ASN.1 abstract syntax TransferControlReject. |  | mandatory |  |  |  |
| 3.2.3.7.8.1.1 | The Transfer-communication service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.7.8.1.2 | The Transfer-communication service primitives shall only be invoked after the successful completion of the Transfer-initiate service. |  | mandatory |  |  |  |
| 3.2.3.7.8.1.3 | When the Transfer-Communication service request is invoked, passing the Transfer Communication Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.8.1.3 | When the Transfer-Communication service indication is invoked, passing the Transfer Communication Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.8.1.3 | When the Transfer-Communication service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.8.1.3 | When the Transfer-Communication service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.8.1.4.1 | The Transfer Communication Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.7.8.1.4.2 | The Transfer Communication Information parameter shall conform to the ASN.1 abstract syntax TransferCommunication. |  | mandatory |  |  |  |
| 3.2.3.7.9.1.1 | The Transfer-communication-assume service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.7.9.1.2 | The Transfer-communication-assume service primitives shall only be invoked after the successful completion of the Transfer-initiate service. |  | mandatory |  |  |  |
| 3.2.3.7.9.1.3 | When the Transfer-Communication Assume service request is invoked, passing the Transfer Communication Assume Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.9.1.3 | When the Transfer-Communication Assume service indication is invoked, passing the Transfer Communication Assume Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.9.1.3 | When the Transfer-Communication Assume service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.7.9.1.3 | When the Transfer-Communication Assume service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.7.9.1.4.1 | The Transfer Communication Assume Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.7.9.1.4.2 | The Transfer Communication Assume Information parameter shall conform to the ASN.1 abstract syntax TransferCommunicationAssume. |  | mandatory |  |  |  |
| 3.2.3.8.1.1.1 | It shall be possible for any ATSU to invoke the primitives of the Info-transfer service at any time after the initial invocation of primitives of the Notify service. |  | mandatory |  |  |  |
| 3.2.3.8.1.1.1 | It shall be possible for any ATSU to invoke the primitives of the Info-transfer service at any time after the initial invocation of the Coordinate-start service primitives when the Notify service is not used. |  | mandatory |  |  |  |
| 3.2.3.8.1.1.2 | It shall be possible for the C-ATSU only, to invoke the primitives of the Info-transfer service before invoking any other service primitives. |  | mandatory |  |  |  |
| 3.2.3.8.1.2 | The Info-transfer service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.8.1.3 | When the Info-Transfer service request is invoked, passing the Called ICAO Facility Designation parameter is conditional dependent upon 3.2.3.8.1.4.1. |  | conditional |  |  |  |
| 3.2.3.8.1.3 | When the Info-Transfer service indication is invoked, passing the Called ICAO Facility Designation parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.8.1.3 | When the Info-Transfer service indication is invoked, passing the Calling ICAO Facility Designation parameter is not applicable. |  | n/a |  |  |  |
| 3.2.3.8.1.3 | When the Info-Transfer service request is invoked, passing the Calling ICAO Facility Designation parameter is conditional dependent upon 3.2.3.8.1.5.1. |  | conditional |  |  |  |
| 3.2.3.8.1.3 | When the Info-Transfer service request is invoked, passing the Information parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.8.1.3 | When the Info-Transfer service indication is invoked, passing the Information parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.8.1.3 | When the Info-Transfer service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.8.1.3 | When the Info-Transfer service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.8.1.4.1 | The Called ICAO Facility Designation parameter shall be supplied, as specified in 3.2.3.1.1, only when the Info-transfer request primitive is invoked and noregime has yet been established by the AIDC-user. |  | conditional |  |  |  |
| 3.2.3.8.1.5.1 | The CallingICAOFacilityDesignation parameter shall be supplied, as specified in 3.2.3.1.2, only when the Info-transfer indication primitive corresponds to a request primitive in which the Called ICAO Facility Designation was supplied. |  | conditional |  |  |  |
| 3.2.3.8.1.6.1 | The Information parameter shall be provided by the AIDC-User. |  | mandatory |  |  |  |
| 3.2.3.8.1.6.2 | The Information parameter shall conform to the ASN.1 abstract syntax of any of the following: GeneralExecutiveData. |  | mandatory |  |  |  |
| 3.2.3.8.1.6.2 | The Information parameter shall conform to the ASN.1 abstract syntax of any of the following: GeneralPoint. |  | mandatory |  |  |  |
| 3.2.3.8.1.6.2 | The Information parameter shall conform to the ASN.1 abstract syntax of any of the following: SurveillanceData. |  | mandatory |  |  |  |
| 3.2.3.8.1.6.2 | The Information parameter shall conform to the ASN.1 abstract syntax of any of the following: GeneralFreeText. |  | mandatory |  |  |  |
| 3.2.3.8.1.6.2 | The Information parameter shall conform to the ASN.1 abstract syntax of any of the following: EmergencyFreeText. |  | mandatory |  |  |  |
| 3.2.3.9.1.1.1 | The End service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.9.1.1.2 | When the End service request is invoked, passing the Cancel Information parameter is conditional dependent upon 3.2.3.9.1.1.3.1. |  | conditional |  |  |  |
| 3.2.3.9.1.1.2 | When the End service request is invoked, passing the Cancel Information parameter is conditional with the same data as the request. |  | conditional |  |  |  |
| 3.2.3.9.1.1.2 | When the End service request is invoked, passing the Message Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.9.1.1.2 | When the End service indication is invoked, passing the Message Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.3.9.1.1.3.1 | The Cancel Information parameter shall be provided by the AIDC-User when canceling the notification for a flight. |  | mandatory |  |  |  |
| 3.2.3.9.1.1.3.1 | The Cancel Information parameter shall be provided by the AIDC-User when canceling the coordination for a flight. |  | mandatory |  |  |  |
| 3.2.3.9.1.1.3.2 | The Cancel Information parameter shall conform to the ASN.1 abstract syntax of Cancel. |  | mandatory |  |  |  |
| 3.2.3.9.2.1.1 | The User-abort service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.9.2.1.2 | It shall be possible to invoke the primitives of the User-abort service at any time. |  | mandatory |  |  |  |
| 3.2.3.9.2.1.3 | The User-abort service primitives shall have no parameters. |  | mandatory |  |  |  |
| 3.2.3.9.3.1.1 | The Provider-abort service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.3.9.3.1.2 | The primitives of the Provider-abort service shall be invoked by the AIDC-AE service-provider. |  | mandatory |  |  |  |
| 3.2.3.9.3.1.3 | When the Provider-Abort service indicator is invoked, passing the Provider Abort Reason parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.3.9.3.1.3 | When the Provider-Abort service indicator is invoked, passing the Result Source parameter is conditional dependent upon 3.2.3.9.3.1.5.1. |  | conditional |  |  |  |
| 3.2.3.9.3.1.4.1 | The Provider Abort Reason parameter shall be provided by the AIDC-AE service provider. |  | mandatory |  |  |  |
| 3.2.3.9.3.1.4.2 | The Provider Abort Reason parameter shall conform to the ASN.1 abstract syntax ProviderAbortReason. |  | mandatory |  |  |  |
| 3.2.3.9.3.1.5.1 | The Result Source parameter shall optionally be provided by the AIDC-AE service provider, only when the Provider Abort Reason parameter has one of the abstract values: "rejectedpermanent". |  | optional |  |  |  |
| 3.2.3.9.3.1.5.1 | The Result Source parameter shall optionally be provided by the AIDC-AE service provider, only when the Provider Abort Reason parameter has one of the abstract values: "rejectedtransient". |  | optional |  |  |  |
| 3.2.3.9.3.1.5.2 | The Result Source parameter shall conform to the abstract syntax of the A-ASSOCIATE Result Source parameter, as defined in ISO/IEC 8649. |  | mandatory |  |  |  |
| 3.2.4.1.1.1 | The User Data parameter, if provided, shall contain data provided by the user of the AIDC-ASE service. |  | conditional |  |  |  |
| 3.2.4.1.2.1 | The Msg Number parameter shall be provided by the AIDC-ASE user except when invoking the AIDC-usr-abort primitive. |  | mandatory |  |  |  |
| 3.2.4.1.2.1 | The Msg Number parameter shall be provided by the AIDC-ASE user except when invoking the AIDC-ucf request primitive. |  | mandatory |  |  |  |
| 3.2.4.1.2.2 | The Msg Number parameter shall consist of a unique identifier for reference in the AIDC-ucf primitives. |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-ucf service as defined in 3.2.4.2.2 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-nfy service as defined in 3.2.4.2.3 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-crd-start service as defined in 3.2.4.2.4 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-crd-end service as defined in 3.2.4.2.5 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-crd-ngtt service as defined in 3.2.4.2.6. |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-crd-stndby service as defined in 3.2.4.2.7 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-tfr-init service as defined in 3.2.4.2.8 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-tfr-rqst service as defined in 3.2.4.2.9 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-tfr-prpsl service as defined in 3.2.4.2.10 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-tfr-accept service as defined in 3.2.4.2.11 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-tfr-cntrl service as defined in 3.2.4.2.12 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-tfr-comm service as defined in 3.2.4.2.13 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-tfr-comm-assm service as defined in 3.2.4.2.14 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-inf-tfr service as defined in 3.2.4.2.15. |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-end service as defined in 3.2.4.2.16 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-usr-abrt service as defined in 3.2.4.2.17 |  | mandatory |  |  |  |
| 3.2.4.2.1.1 | An implementation of the AIDC-AE shall exhibit behavior consistent with having implemented an AIDC-ASE with the following abstract services: AIDC-pvd-abrt service as defined in 3.2.4.2.18. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.1 | The AIDC-ucf service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.2 | When the AIDC-ucf service request is invoked, passing the Result parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.2 | When the AIDC-ucf service indication is invoked, passing the Result parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.2 | When the AIDC-ucf service request is invoked, passing the Reason parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.2.1.2 | When the AIDC-ucf service indication is invoked, passing the Reason parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.2.1.2 | When the AIDC-ucf service request is invoked, passing the Reference ID parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.2 | When the AIDC-ucf service indication is invoked, passing the Reference ID parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.3.1 | The Result parameter shall be provided by the AIDC-ASE user. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.3.2 | The Result parameter shall indicate the acceptance of the service primitive. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.3.2 | The Result parameter shall indicate the rejection of the service primitive. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.4.1 | The Reason parameter shall be provided by the AIDC-ASE user. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.5.1 | The Reference ID parameter shall be provided by the AIDC-ASE user. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.5.2 | The Reference ID parameter shall contain the Msg Number of the service that is being accepted. |  | mandatory |  |  |  |
| 3.2.4.2.2.1.5.2 | The Reference ID parameter shall contain the Msg Number of the service that is being rejected. |  | mandatory |  |  |  |
| 3.2.4.2.3.1.1 | The AIDC-nfy service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.3.1.2 | When the AIDC-nfy service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.3.1.2 | When the AIDC-nfy service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.3.1.2 | When the AIDC-nfy service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.3.1.2 | When the AIDC-nfy service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.4.1.1 | The AIDC-crd-start service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.4.1.2 | When the AIDC-crd-start service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.4.1.2 | When the AIDC-crd-start service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.4.1.2 | When the AIDC-crd-start service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.4.1.2 | When the AIDC-crd-start service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.5.1.2 | When the AIDC-crd-start service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.5.1.2 | When the AIDC-crd-start service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.5.1.2 | When the AIDC-crd-start service request is invoked, passing the Result parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.5.1.2 | When the AIDC-crd-start service indication is invoked, passing the Result parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.5.1.2 | When the AIDC-crd-start service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.5.1.2 | When the AIDC-crd-start service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.5.1.3.1 | The Result parameter shall be provided by the AIDC-ASE user. |  | mandatory |  |  |  |
| 3.2.4.2.5.1.3.2 | The Result parameter shall be used to indicate acceptance of the ending of the Coordinating Regime. |  | mandatory |  |  |  |
| 3.2.4.2.5.1.3.2 | The Result parameter shall be used to indicate rejection of the ending of the Coordinating Regime. |  | mandatory |  |  |  |
| 3.2.4.2.6.1.1 | The AIDC-crd-ngtt service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.6.1.2 | When the AIDC-crd-ngtt service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.6.1.2 | When the AIDC-crd-ngtt service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.6.1.2 | When the AIDC-crd-ngtt service request is invoked, passing the Msg Number parameter is not applicable. |  | n/a |  |  |  |
| 3.2.4.2.6.1.2 | When the AIDC-crd-ngtt service indication is invoked, passing the Msg Number parameter is not applicable. |  | n/a |  |  |  |
| 3.2.4.2.7.1.1 | The AIDC-crd-stndby service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.7.1.2 | When the AIDC-crd-stndby service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.7.1.2 | When the AIDC-crd-stndby service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.7.1.2 | When the AIDC-crd-stndby service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.7.1.2 | When the AIDC-crd-stndby service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.8.1.1 | The AIDC-tfr-init service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.8.1.2 | When the AIDC-tfr-init service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.8.1.2 | When the AIDC-tfr-init service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.8.1.2 | When the AIDC-tfr-init service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.8.1.2 | When the AIDC-tfr-init service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.9.1.1 | The AIDC-tfr-rqst service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.9.1.2 | When the AIDC-tfr-rqst service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.9.1.2 | When the AIDC-tfr-rqst service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.9.1.2 | When the AIDC-tfr-rqst service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.9.1.2 | When the AIDC-tfr-rqst service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.10.1.1 | The AIDC-tfr-prpsl service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.10.1.2 | When the AIDC-tfr-prpsl service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.10.1.2 | When the AIDC-tfr-prpsl service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.10.1.2 | When the AIDC-tfr-prpsl service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.10.1.2 | When the AIDC-tfr-prpsl service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.11.1.1 | The AIDC-tfr-accept service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.11.1.2 | When the AIDC-tfr-prpsl service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.11.1.2 | When the AIDC-tfr-prpsl service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.11.1.2 | When the AIDC-tfr-prpsl service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.11.1.2 | When the AIDC-tfr-prpsl service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.12.1.1 | The AIDC-tfr-cntrl service shall be a confirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service response is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service confirmation is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service request is invoked, passing the Result parameter is not applicable. |  | n/a |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service indication is invoked, passing the Result parameter is not applicable. |  | n/a |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service response is invoked, passing the Result parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service confirmation is invoked, passing the Result parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service response is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.12.1.2 | When the AIDC-tfr-cntrl service confirmation is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.12.1.3.1 | The Result parameter, shall be provided by the AIDC-ASE user. |  | mandatory |  |  |  |
| 3.2.4.2.13.1.1 | The AIDC-tfr-comm service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.13.1.2 | When the AIDC-tfr-comm service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.13.1.2 | When the AIDC-tfr-comm service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.13.1.2 | When the AIDC-tfr-comm service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.13.1.2 | When the AIDC-tfr-comm service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.14.1.1 | The AIDC-tfr-comm-assm service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.14.1.2 | When the AIDC-tfr-comm-assm service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.14.1.2 | When the AIDC-tfr-comm-assm service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.14.1.2 | When the AIDC-tfr-comm-assm service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.14.1.2 | When the AIDC-tfr-comm-assm service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.15.1.1 | The AIDC-inf-tfr service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.15.1.2 | When the AIDC-inf-tfr service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.15.1.2 | When the AIDC-inf-tfr service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.15.1.2 | When the AIDC-inf-tfr service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.15.1.2 | When the AIDC-inf-tfr service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.16.1.1 | The AIDC-end service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.16.1.2 | When the AIDC-end service request is invoked, passing the User Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.2.16.1.2 | When the AIDC-end service indication is invoked, passing the User Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.2.16.1.2 | When the AIDC-end service request is invoked, passing the Msg Number parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.16.1.2 | When the AIDC-end service indication is invoked, passing the Msg Number parameter is mandatory with the same data as the request. |  | mandatory |  |  |  |
| 3.2.4.2.17.1.1 | The AIDC-usr-abrt service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.17.1.2 | The AIDC-usr-abrt service primitives shall have no parameters. |  | mandatory |  |  |  |
| 3.2.4.2.18.1 | The AIDC-pvd-abrt service shall be an AIDC-ASE service-provider initiated service. |  | mandatory |  |  |  |
| 3.2.4.2.18.2.1 | The AIDC-pvd-abrt service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.2.18.2.2 | When the AIDC- pvd-abrt service indication is invoked, passing the Abort Reason parameter is mandatory. |  | mandatory |  |  |  |
| 3.2.4.2.18.2.3.1 | The Abort Reason parameter shall be provided by the AIDC-ASE. |  | mandatory |  |  |  |
| 3.2.4.2.18.2.3.2 | The Abort Reason parameter shall be used to identify the reason for the abort. |  | mandatory |  |  |  |
| 3.2.4.3.1.1 | An implementation of the AIDC-AE shall exhibit the behaviour consistent with having implemented an AIDC-ASE supported by the following abstract services: AIDC-DATA service as defined in 3.2.4.3.2 |  | mandatory |  |  |  |
| 3.2.4.3.1.1 | An implementation of the AIDC-AE shall exhibit the behaviour consistent with having implemented an AIDC-ASE supported by the following abstract services: AIDC-ABORT service as defined in 3.2.4.3.3 |  | mandatory |  |  |  |
| 3.2.4.3.1.1 | An implementation of the AIDC-AE shall exhibit the behaviour consistent with having implemented an AIDC-ASE supported by the following abstract services: AIDC-P-ABORT service as defined in 3.2.4.3.4 |  | mandatory |  |  |  |
| 3.2.4.3.2.1.1 | The AIDC-DATA service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.3.2.1.2 | When the AIDC-DATA service request is invoked, passing the AIDC Data parameter is a user option. |  | optional |  |  |  |
| 3.2.4.3.2.1.2 | When the AIDC-DATA service indication is invoked, passing the AIDC Data parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.3.2.1.3.1 | The AIDC Data parameter, if any, shall be provided by the AIDC-ASE. |  | mandatory |  |  |  |
| 3.2.4.3.2.1.3.2 | The AIDC Data parameter shall conform to the ASN.1 syntax AIDC-APDU. |  | mandatory |  |  |  |
| 3.2.4.3.3.1.1 | The AIDC-ABORT service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.3.3.1.2 | When the AIDC- ABORT service request is invoked, passing the AIDC Abort parameter is a user option. |  | optional |  |  |  |
| 3.2.4.3.3.1.2 | When the AIDC- ABORT service indication is invoked, passing the AIDC Abort parameter is optional with the same data as the request if used. |  | conditional |  |  |  |
| 3.2.4.3.3.2.1 | The Abort Reason parameter, if any, shall be provided by the AIDC-ASE. |  | mandatory |  |  |  |
| 3.2.4.3.3.2.2 | The Abort Reason parameter shall conform to the ASN.1 syntax of ProviderAbortReason. |  | mandatory |  |  |  |
| 3.2.4.3.4.1.1 | The AIDC-P-ABORT service shall be an unconfirmed service. |  | mandatory |  |  |  |
| 3.2.4.3.4.1.2 | The AIDC-P-ABORT service primitives shall have no parameters. |  | mandatory |  |  |  |
| 3.2.5.1 | An implementation of the AIDC-AE should behave as if there existed a Control Function as defined in 3.2.5. |  | Recommendation |  |  |  |
| 3.2.5.1.1 | With the permissible exception of abort service primitives, the AIDC-AE shall process service primitives in the order in which they are received: this ensures that the AE will, with the exception of aborts, guarantee message sequencing. |  | mandatory (based on use of CF as defined in 3.2.5.1) |  |  |  |
| 3.2.5.2.1 | The AIDC-AE CF shall be in one of the following states at a given time: Null (STA0) - This is the state of the CF when there is no association in existence. |  | mandatory (CF) |  |  |  |
| 3.2.5.2.1 | The AIDC-AE CF shall be in one of the following states at a given time: Association Pending (STA1) - The CF enters this state when the AIDC-User has invoked a Notify request primitive. |  | mandatory (CF) |  |  |  |
| 3.2.5.2.1 | The AIDC-AE CF shall be in one of the following states at a given time: Association Pending (STA1) - The CF enters this state when the AIDC-User has invoked a Coordinate-start request primitive. |  | mandatory (CF) |  |  |  |
| 3.2.5.2.1 | The AIDC-AE CF shall be in one of the following states at a given time: Association Pending (STA1) - The CF enters this state when an indication has been received that the peer has made a request to establish an association. |  | mandatory (CF) |  |  |  |
| 3.2.5.2.1 | The AIDC-AE CF shall be in one of the following states at a given time: Data Transfer (STA2) - The CF enters this state once the establishment phase is complete. An association has successfully been established and the communicating partners are free to send and receive data. |  | mandatory (CF) |  |  |  |
| 3.2.5.2.1 | The AIDC-AE CF shall be in one of the following states at a given time: Release Pending (STA3) - The CF enters this state when the AIDC-User has requested the termination of the AIDC service. |  | mandatory (CF) |  |  |  |
| 3.2.5.2.1 | The AIDC-AE CF shall be in one of the following states at a given time: Release Pending (STA3) - The CF enters this state when an indication has been received that the peer has made a request to terminate the association. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.1 | The invocation of an Info-transfer request primitive shall implicitly cause the establishment of an association, if one does not exist, between two peer AIDC-AEs. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.1 | The invocation of a Notify request primitive shall implicitly cause the establishment of an association, if one does not exist, between two peer AIDC-AEs. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.1 | The invocation of a Coordinate-start request primitive shall implicitly cause the establishment of an association, if one does not exist, between two peer AIDC-AEs. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.2 | The association establishment and release between peer AIDC-AEs shall be performed by invoking the primitives of ACSE. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of an Info-transfer request primitive, the CF shall: construct the Calling AP Title from the local ICAO Facility Designation according to the specification in 4.3.2. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of a Notify request primitive, the CF shall: construct the Calling AP Title from the local ICAO Facility Designation according to the specification in 4.3.2. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of a Coordinate-start request primitive, the CF shall: construct the Calling AP Title from the local ICAO Facility Designation according to the specification in 4.3.2. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of an Info-transfer request primitive, the CF shall: invoke an A-ASSOCIATE Request primitive with the parameters specified in Table 3.2.5-1. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of a Notify request primitive, the CF shall: invoke an A-ASSOCIATE Request primitive with the parameters specified in Table 3.2.5-1. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of a Coordinate-start request primitive, the CF shall: invoke an A-ASSOCIATE Request primitive with the parameters specified in Table 3.2.5-1. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of an Info-transfer request primitive, the CF shall: enter the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of a Notify request primitive, the CF shall: enter the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | Upon the receipt of a Coordinate-start request primitive, the CF shall: enter the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | A-ASSOCIATE Request Primitive Parameters |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.1.3 | A-ASSOCIATE Request Parameter | Value |  |  |  |  |
|  | Mode | Not used (default value) | not used |  |  |  |
|  | Application Context Name | {iso (1) identified-organization (3) icao (27) atn-ac (3) 1 | mandatory (CF) |  |  |  |
|  | Application Context Name List | Not used | not used |  |  |  |
|  | Calling AP Title | {iso (1) identified-organization (3) icao (27) atn-end-system-ground (2) <end-system-id> (n) operational (0) | mandatory (CF) |  |  |  |
|  | Calling AE Qualifier | idc (6) | mandatory (CF) |  |  |  |
|  | Calling AP Invocation-identifier | Not used | not used |  |  |  |
|  | Calling AE Invocation-identifier | Not used | not used |  |  |  |
|  | Called AP Title | Not used | not used |  |  |  |
|  | Called AE Qualifier | Not used | not used |  |  |  |
|  | Called AP Invocation-identifier | Not used | not used |  |  |  |
|  | Called AE Invocation-identifier | Not used | not used |  |  |  |
|  | ACSE Requirements | Not used | not used |  |  |  |
|  | Authentication-mechanism Name | Not used | not used |  |  |  |
|  | Authentication-value | Not used | not used |  |  |  |
|  | User Information | Not used | not used |  |  |  |
|  | Calling Presentation Address | Local Implementation | optional |  |  |  |
|  | Called Presentation Address | Local Implementation | optional |  |  |  |
|  | Presentation Context Definition List | Not used | not used |  |  |  |
|  | Default Presentation Context Name | Not used | not used |  |  |  |
|  | Quality of Service | See 3.2.8.2 | mandatory (CF) |  |  |  |
|  | Presentation Requirements | Not used (default value) | not used |  |  |  |
|  | Session Requirements | No Orderly Release (NOR), Duplex | mandatory (CF) |  |  |  |
|  | Initial Synchronization Point Serial No | Not used | not used |  |  |  |
|  | Initial Assignment of Tokens | Not used | not used |  |  |  |
|  | Session-connection Identifier | Not used | not used |  |  |  |
| 3.2.5.3.1.2.1.1 | It shall be valid to invoke the User-confirmation request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.2.1.2 | If the CF is in the RELEASE PENDING state, and the User-confirmation request primitive is invoked, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.2.1.2 | If the CF is in the NULL state, and the User-confirmation request primitive is invoked, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.2.1.2 | If the CF is in the ASSOCIATION PENDING state, and the User-confirmation request primitive is invoked, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.2.2.1 | The CF shall invoke an AIDC-ucf request primitive with: the User-confirmation Result parameter as the AIDC-ucf Result parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.2.2.1 | The CF shall invoke an AIDC-ucf request primitive with: the User-confirmation Referenced Number parameter as the AIDC-ucf Reference ID parameter |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.2.2.1 | The CF shall invoke an AIDC-ucf request primitive with: the User-confirmation Reason parameter as the AIDC-ucf Reason parameter, if present. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.3.1.1 | It shall be valid to invoke the Notify request primitive when the CF is in the NULL state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.3.1.1 | It shall be valid to invoke the Notify request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.3.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.3.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.3.2.1 | If in the NULL state the CF shall: store the value of the Notify request primitive parameters. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.3.2.1 | If in the NULL state the CF shall: set the Boolean variable nfy-assc. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.3.2.1 | If in the NULL state the CF shall: perform the implicit association function as specified in 3.2.5.3.1.1. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.3.2.2 | If in the DATA TRANSFER state, the CF shall invoke an AIDC-nfy request primitive with: the Notify request primitive Notification Information parameter as the AIDC-nfy request primitive User Data parameter |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.3.2.2 | If in the DATA TRANSFER state, the CF shall invoke an AIDC-nfy request primitive with: the Notify request primitive Message Number parameter as the AIDC-nfy request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.4.1.1 | It shall be valid to invoke the Coordinate-start request primitive when the CF is in the NULL state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.4.1.1 | It shall be valid to invoke the Coordinate-start request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.4.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.4.1.2 | If the CF is in the RELEASE PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.4.2.1 | If in the NULL state the CF shall: store the value of the Coordinate-start request primitive parameters. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.4.2.1 | If in the NULL state the CF shall: set the Boolean variable crd-assc. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.4.2.1 | If in the NULL state the CF shall: perform the implicit association function as specified in 3.2.5.3.1.1. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.4.2.2 | If in the DATA TRANSFER state, the CF shall invoke an AIDC-crd-start request primitive with: the Coordinate-start request primitive Coordination Start Information parameter as the AIDC-crd-start request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.4.2.2 | If in the DATA TRANSFER state, the CF shall invoke an AIDC-crd-start request primitive with: the Coordinate-start request primitive Message Number parameter as the AIDC-crd-start request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.5.1.1 | It shall be valid to invoke the Coordinate-end request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.5.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.5.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.5.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.5.2.1 | Upon the receipt of a Coordinate-end request primitive, the CF shall invoke an AIDC-crd-end request primitive with: the Coordinate-end request primitive Coordinate End Information parameter as the AIDC-crd-end request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.5.2.1 | Upon the receipt of a Coordinate-end request primitive, the CF shall invoke an AIDC-crd-end request primitive with: the Coordinate-end request primitive Result parameter as the AIDC-crd-end request primitive Result parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.5.2.1 | Upon the receipt of a Coordinate-end request primitive, the CF shall invoke an AIDC-crd-end request primitive with: the Coordinate-end request primitive Message Number parameter as the AIDC-crd-end request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.6.1.1 | It shall be valid to invoke the Coordinate-negotiate request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.6.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.6.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.6.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.6.2.1 | Upon the receipt of a Coordinate-negotiate request primitive, the CF shall invoke an AIDC-crd-ngtt request primitive with: the Coordinate-negotiate request primitive Coordinate Negotiate Information parameter as the AIDC-crd-ngtt request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.6.2.1 | Upon the receipt of a Coordinate-negotiate request primitive, the CF shall invoke an AIDC-crd-ngtt request primitive with: the Coordinate-negotiate request primitive Message Number parameter as the AIDC-crd-ngtt request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.7.1.1 | It shall be valid to invoke the Coordinate-standby request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.7.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.7.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.7.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.7.2.1 | Upon the receipt of a Coordinate-standby request primitive, the CF shall invoke an AIDC-crd-stndby request primitive with: the Coordinate-standby request primitive Coordinate Standby Information parameter as the AIDC-crd-stndby request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.7.2.1 | Upon the receipt of a Coordinate-standby request primitive, the CF shall invoke an AIDC-crd-stndby request primitive with: the Coordinate-standby request primitive Message Number parameter as the AIDC-crd-stndby request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.8.1.1 | It shall be valid to invoke the Transfer-initiate request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.8.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.8.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.8.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.8.2.1 | Upon the receipt of a Transfer-initiate request primitive, the CF shall invoke an AIDC-tfr-init request primitive with: the Transfer-initiate request primitive Transfer Initiate Information parameter as the AIDC-tfr-init request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.8.2.1 | Upon the receipt of a Transfer-initiate request primitive, the CF shall invoke an AIDC-tfr-init request primitive with: the Transfer-initiate request primitive Message Number parameter as the AIDC-tfr-init request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.9.1.1 | It shall be valid to invoke the Transfer-request request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.9.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.9.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.9.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.9.2.1 | Upon the receipt of a Transfer-request request primitive, the CF shall invoke an AIDC-tfr-rqst request primitive with: the Transfer-request request primitive Transfer Request Information parameter as the AIDC-tfr-rqst request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.9.2.1 | Upon the receipt of a Transfer-request request primitive, the CF shall invoke an AIDC-tfr-rqst request primitive with: the Transfer-request request primitive Message Number parameter as the AIDC-tfr-rqst request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.10.1.1 | It shall be valid to invoke the Transfer-conditions-proposal request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.10.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.10.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.10.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.10.2.1 | Upon the receipt of a Transfer-conditions-proposal request primitive, the CF shall invoke an AIDC-tfr-prpsl request primitive with: the Transfer-conditions-proposal request primitive Transfer Conditions Proposal Information parameter as the AIDC-tfr-prpsl request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.10.2.1 | Upon the receipt of a Transfer-conditions-proposal request primitive, the CF shall invoke an AIDC-tfr-prpsl request primitive with: the Transfer-conditions-proposal request primitive Message Number parameter as the AIDC-tfr-prpsl request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.11.1.1 | It shall be valid to invoke the Transfer-conditions-accept request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.11.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.11.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.11.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.11.2.1 | Upon the receipt of a Transfer-conditions-accept request primitive, the CF shall invoke an AIDC-tfr-accept request primitive with: the Transfer-conditions-accept request primitive Transfer Conditions Accept Information parameter as the AIDC-tfr-accept request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.11.2.1 | Upon the receipt of a Transfer-conditions-accept request primitive, the CF shall invoke an AIDC-tfr-accept request primitive with: the Transfer-conditions-accept request primitive Message Number parameter as the AIDC-tfr-accept request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.12.1.1 | It shall be valid to invoke the Transfer-control request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.12.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.12.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.12.1.2 | If the CF is in RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.12.2.1 | Upon the receipt of a Transfer-control request primitive, the CF shall invoke an AIDC-tfr-cntrl request primitive with: the Transfer-control request primitive Transfer Control Information parameter as the AIDC-tfr-cntrl request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.12.2.1 | Upon the receipt of a Transfer-control request primitive, the CF shall invoke an AIDC-tfr-cntrl request primitive with: the Transfer-control request primitive Transfer Control Information parameter as the AIDC-tfr-cntrl request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.13.1.1 | It shall be valid to invoke the Transfer-control response primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.13.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.13.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.13.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.13.2.1 | Upon the receipt of a Transfer-control response primitive, the CF shall invoke an AIDC-tfr-cntrl response primitive with: the Transfer-control response primitive Transfer Control Information parameter as the AIDC-tfr-cntrl response primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.13.2.1 | Upon the receipt of a Transfer-control response primitive, the CF shall invoke an AIDC-tfr-cntrl response primitive with: the Transfer-control response primitive Result parameter as the AIDC-tfr-cntrl response primitive Result parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.13.2.1 | Upon the receipt of a Transfer-control response primitive, the CF shall invoke an AIDC-tfr-cntrl response primitive with: the Transfer-control response primitive Message Number parameter as the AIDC-tfr-cntrl response primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.14.1.1 | It shall be valid to invoke the Transfer-communication request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.14.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.14.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.14.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.14.2.1 | Upon the receipt of a Transfer-communication request primitive, the CF shall invoke an AIDC-tfr-comm request primitive with: the Transfer-communication request primitive Transfer Communication Information parameter as the AIDC-tfr-comm request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.14.2.1 | Upon the receipt of a Transfer-communication request primitive, the CF shall invoke an AIDC-tfr-comm request primitive with: the Transfer-communication request primitive Message Number parameter as the AIDC-tfr-comm request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.15.1.1 | It shall be valid to invoke the Transfer-communication-assume request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.15.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.15.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.15.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.15.2.1 | Upon the receipt of a Transfer-communication-assume request primitive, the CF shall invoke an AIDC-tfr-comm-assm request primitive with: the Transfer-communication request primitive Transfer Communication Assume Information parameter as the AIDC-tfr-comm-assm request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.15.2.1 | Upon the receipt of a Transfer-communication-assume request primitive, the CF shall invoke an AIDC-tfr-comm-assm request primitive with the Transfer-communication request primitive Message Number parameter as the AIDC-tfr-comm-assm request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.16.1.1 | It shall be valid to invoke the Info-transfer request primitive when the CF is in the NULL state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.16.1.1 | It shall be valid to invoke the Info-transfer request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.16.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.16.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.16.2.1 | If in the NULL state the CF shall: store the value of the Info-transfer request primitive parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.16.2.1 | If in the NULL state the CF shall: set the Boolean variable inf-assc. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.16.2.1 | If in the NULL state the CF shall: perform the implicit association function as specified in 5.3.1.1 |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.16.2.2 | If in the DATA TRANSFER state, the CF shall invoke an AIDC-inf-tfr request primitive with: the Info-transfer request primitive Information parameter as the AIDC-inf-tfr request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.16.2.2 | If in the DATA TRANSFER state, the CF shall invoke an AIDC-inf-tfr request primitive with: the Info-transfer request primitive Message Number parameter as the AIDC-inf-tfr request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.17.1.1 | It shall be valid to invoke the End request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.17.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.17.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.17.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.17.2.1 | Upon the receipt of a End request primitive, the CF shall invoke an AIDC-end request primitive with: the End request primitive Cancel Information parameter, if present, as the AIDC-end request primitive User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.17.2.1 | Upon the receipt of a End request primitive, the CF shall invoke an AIDC-end request primitive with: the End request primitive Message Number parameter as the AIDC-end request primitive Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.1.1 | It shall be valid to invoke the User-abort request primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.1.1 | It shall be valid to invoke the User-abort request primitive when the CF is in the the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.1.1 | It shall be valid to invoke the User-abort request primitive when the CF is in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.1.18.2.1 | Upon the receipt of a User-abort request primitive in the DATA TRANSFER state, the CF shall: invoke an AIDC-usr-abrt request primitive. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.2.1 | Upon the receipt of a User-abort request primitive in the RELEASE PENDING state, the CF shall: invoke an AIDC-usr-abrt request primitive. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.2.1 | Upon the receipt of a User-abort request primitive in the DATA TRANSFER state, the CF shall: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.2.1 | Upon the receipt of a User-abort request primitive in the DATA TRANSFER state, the CF shall: remain in, the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.2.1 | Upon the receipt of a User-abort request primitive in the RELEASE PENDING state, the CF shall: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.2.1 | Upon the receipt of a User-abort request primitive in the RELEASE PENDING state, the CF shall: remain in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.2.2 | Upon the receipt of a User-abort request primitive in the ASSOCIATION PENDING state, the CF shall: invoke an A-ABORT request primitive. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.1.18.2.2 | Upon the receipt of a User-abort request primitive in the ASSOCIATION PENDING state, the CF shall: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.1.1.1 | It shall be valid to invoke the AIDC-ucf indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.1.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.1.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.1.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.1.2.1 | Upon the receipt of a AIDC-ucf indication primitive, the CF shall invoke a User-confirmation indication primitive with: the AIDC-ucf indication primitive Result parameter as the User-confirmation indication primitive Result parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.1.2.1 | Upon the receipt of a AIDC-ucf indication primitive, the CF shall invoke a User-confirmation indication primitive with: if present, the AIDC-ucf indication primitive Reason parameter as the User-confirmation primitive Reason parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.1.2.1 | Upon the receipt of a AIDC-ucf indication primitive, the CF shall invoke a User-confirmation indication primitive with: the AIDC-ucf indication Reference ID parameter as the User-confirmation indication Referenced Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.2.1.1 | It shall be valid to invoke the AIDC-nfy indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.2.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.2.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.2.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.2.2.1 | Upon the receipt of a AIDC-nfy indication primitive, the CF shall invoke the Notify indication primitive with: the stored Calling ICAO Facility Designation as the Notify indication primitive Calling ICAO Facility Designation parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.2.2.1 | Upon the receipt of a AIDC-nfy indication primitive, the CF shall invoke the Notify indication primitive with: the AIDC-nfy indication primitive User Data parameter as the Notify indication primitive Notification Information parameterr. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.2.2.1 | Upon the receipt of a AIDC-nfy indication primitive, the CF shall invoke the Notify indication primitive with: the AIDC-nfy indication primitive Msg Number parameter as the Notify indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.3.1.1 | It shall be valid to invoke the AIDC-crd-start indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.3.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.3.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.3.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.3.2.1 | Upon the receipt of a AIDC-crd-start indication primitive, the CF shall invoke the Coordinate-start indication primitive with: the stored Calling ICAO Facility Designation as the Coordinate-start indication primitive Calling ICAO Facility Designation parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.3.2.1 | Upon the receipt of a AIDC-crd-start indication primitive, the CF shall invoke the Coordinate-start indication primitive with: the AIDC-crd-start indication primitive User Data parameter as the Coordinate-start indication primitive Coordinate Start Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.3.2.1 | Upon the receipt of a AIDC-crd-start indication primitive, the CF shall invoke the Coordinate-start indication primitive with: the AIDC-crd-start indication primitive Msg Number parameter as the Coordinate-start indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.4.1.1 | It shall be valid to invoke the AIDC-crd-end indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.4.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.4.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.4.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.4.2.1 | Upon the receipt of a AIDC-crd-end indication primitive the CF shall invoke a Coordinate-end indication primitive with: the AIDC-crd-end indication primitive User Data parameter as the Coordinate-end indication primitive Coordinate End Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.4.2.1 | Upon the receipt of a AIDC-crd-end indication primitive the CF shall invoke a Coordinate-end indication primitive with: the AIDC-crd-end indication primitive Msg Number parameter as the Coordinate-end indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.5.1.1 | It shall be valid to invoke the AIDC-crd-ngtt indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.5.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.5.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.5.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.5.2.1 | Upon the receipt of a AIDC-crd-ngtt indication primitive the CF shall invoke a Coordinate-negotiate indication primitive with: the AIDC-crd-ngtt indication primitive User Data parameter as the Coordinate-negotiate indication primitive Coordinate Negotiate Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.5.2.1 | Upon the receipt of a AIDC-crd-ngtt indication primitive the CF shall invoke a Coordinate-negotiate indication primitive with: the AIDC-crd-ngtt indication primitive Msg Number parameter as the Coordinate-negotiate indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.6.1.1 | It shall be valid to invoke the AIDC-crd-stndby indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.6.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.6.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.6.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.6.2.1 | Upon the receipt of a AIDC-crd-stndby indication primitive the CF shall invoke a Coordinate-standby indication primitive with: the AIDC-crd-stndby indication primitive User Data parameter as the Coordinate-standby indication primitive Coordinate Standby Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.6.2.1 | Upon the receipt of a AIDC-crd-stndby indication primitive the CF shall invoke a Coordinate-standby indication primitive with: the AIDC-crd-stndby indication primitive Msg Number parameter as the Coordinate-standby indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.7.1.1 | It shall be valid to invoke the AIDC-tfr-init indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.7.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.7.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.7.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.7.2.1 | Upon the receipt of a AIDC-tfr-init indication primitive the CF shall invoke a Transfer-initiate indication primitive with: the AIDC-tfr-init indication primitive User Data parameter as the Transfer-initiate indication primitive Transfer Initiate Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.7.2.1 | Upon the receipt of a AIDC-tfr-init indication primitive the CF shall invoke a Transfer-initiate indication primitive with: the AIDC-tfr-init indication primitive Msg Number parameter as the Transfer-initiate indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.8.1.1 | It shall be valid to invoke the AIDC-tfr-rqst indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.8.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.8.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.8.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.8.2.1 | Upon the receipt of a AIDC-tfr-rqst indication primitive the CF shall invoke a Transfer-request indication primitive with: the AIDC-tfr-rqst indication primitive User Data parameter as the Transfer-request indication primitive Transfer Request Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.8.2.1 | Upon the receipt of a AIDC-tfr-rqst indication primitive the CF shall invoke a Transfer-request indication primitive with: the AIDC-tfr-rqst indication primitive Msg Number parameter as the Transfer-request indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.9.1.1 | It shall be valid to invoke the AIDC-tfr-prpsl indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.9.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.9.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.9.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.9.2.1 | Upon the receipt of a AIDC-tfr-prpsl indication primitive the CF shall invoke a Transfer-conditions-proposal indication primitive with: the AIDC-tfr-prpsl indication primitive User Data parameter as the Transfer-conditions-proposal indication primitive Transfer Conditions Proposal Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.9.2.1 | Upon the receipt of a AIDC-tfr-prpsl indication primitive the CF shall invoke a Transfer-conditions-proposal indication primitive with: the AIDC-tfr-prpsl indication primitive Msg Number parameter as the Transfer-conditions-proposal indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.10.1.1 | It shall be valid to invoke the AIDC-tfr-accept indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.10.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.10.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.10.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.10.2.1 | Upon the receipt of a AIDC-tfr-accept indication primitive the CF shall invoke a Transfer-conditions-accept indication primitive with: the AIDC-tfr-accept indication primitive User Data parameter as the Transfer-conditions-accept indication primitive Transfer Conditions Accept Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.10.2.1 | Upon the receipt of a AIDC-tfr-accept indication primitive the CF shall invoke a Transfer-conditions-accept indication primitive with: the AIDC-tfr-accept indication primitive Msg Number parameter as the Transfer-conditions-accept indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.11.1.1 | It shall be valid to invoke the AIDC-tfr-cntrl indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.11.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.11.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.11.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.11.2.1 | Upon the receipt of a AIDC-tfr-cntrl indication primitive the CF shall invoke a Transfer-control indication primitive with: the AIDC-tfr-cntrl indication primitive User Data parameter as the Transfer-control indication primitive Transfer Control Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.11.2.1 | Upon the receipt of a AIDC-tfr-cntrl indication primitive the CF shall invoke a Transfer-control indication primitive with: the AIDC-tfr-cntrl indication primitive Msg Number parameter as the Transfer-control indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.12.1.1 | It shall be valid to invoke the AIDC-tfr-cntrl confirmation primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.12.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.12.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.12.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.12.2.1 | Upon the receipt of a AIDC-tfr-cntrl confirmation primitive the CF shall invoke a Transfer-control confirmation primitive with: the AIDC-tfr-cntrl confirmation primitive User Data parameter as the Transfer-control confirmation primitive Transfer Control Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.12.2.1 | Upon the receipt of a AIDC-tfr-cntrl confirmation primitive the CF shall invoke a Transfer-control confirmation primitive with: the AIDC-tfr-cntrl confirmation primitive Result parameter as the Transfer-control confirmation primitive Result parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.12.2.1 | Upon the receipt of a AIDC-tfr-cntrl confirmation primitive the CF shall invoke a Transfer-control confirmation primitive with: the AIDC-tfr-cntrl confirmation primitive Msg Number parameter as the Transfer-control confirmation primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.13.1.1 | It shall be valid to invoke the AIDC-tfr-comm indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.13.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.13.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.13.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.13.2.1 | Upon the receipt of a AIDC-tfr-comm indication primitive the CF shall invoke a Transfer-communication indication primitive with: the AIDC-tfr-comm indication primitive User Data parameter as the Transfer-communication indication primitive Transfer Communication Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.13.2.1 | Upon the receipt of a AIDC-tfr-comm indication primitive the CF shall invoke a Transfer-communication indication primitive with: the AIDC-tfr-comm indication primitive Msg Number parameter as the Transfer-communication indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.14.1.1 | It shall be valid to invoke the AIDC-tfr-comm-assm indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.14.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.14.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.14.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.14.2.1 | Upon the receipt of a AIDC-tfr-comm-assm indication primitive the CF shall invoke a Transfer-communication-assume indication primitive with: the AIDC-tfr-comm-assm indication primitive User Data parameter as the Transfer-communication-assume indication primitive Transfer Communication Assume Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.14.2.1 | Upon the receipt of a AIDC-tfr-comm-assm indication primitive the CF shall invoke a Transfer-communication-assume indication primitive with: the AIDC-tfr-comm-assm indication primitive Msg Number parameter as the Transfer-communication-assume indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.15.1.1 | It shall be valid to invoke the AIDC-inf-tfr indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.15.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.15.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.15.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.15.2.1 | Upon the receipt of a AIDC-inf-tfr indication primitive the CF shall invoke a Info-transfer indication primitive with: the AIDC-inf-tfr indication primitive User Data parameter as the Info-transfer indication primitive Information parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.15.2.1 | Upon the receipt of a AIDC-inf-tfr indication primitive the CF shall invoke a Info-transfer indication primitive with: the AIDC-inf-tfr indication primitive Msg Number parameter as the Info-transfer indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.16.1.1 | It shall be valid to invoke the AIDC-end indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.16.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.16.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.16.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.16.2.1 | Upon the receipt of a AIDC-end indication primitive the CF shall invoke a End indication primitive with: the AIDC-end indication primitive User Data parameter, if present, as the End indication primitive Cancel Information parameter;. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.16.2.1 | Upon the receipt of a AIDC-end indication primitive the CF shall invoke a End indication primitive with: the AIDC-end indication primitive Msg Number parameter as the End indication primitive Message Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.17.1.1 | It shall be valid to invoke the AIDC-usr-abrt indication primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.17.1.1 | It shall be valid to invoke the AIDC-usr-abrt indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.17.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.17.2.1 | Upon the receipt of a AIDC-usr-abrt indication primitive the CF shall invoke a User-abort indication primitive. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.18.1.1 | It shall be valid to invoke the AIDC-pvd-abrt indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.18.1.1 | It shall be valid to invoke the AIDC-pvd-abrt indication primitive when the CF is in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.18.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.18.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.2.18.2.1 | Upon the receipt of a AIDC-pvd-abrt indication primitive the CF shall: if the CF is in the DATA TRANSFER state: invoke a Provider-abort indication primitive with the Provider-abort Provider Abort Reason parameter having the value delivered in the AIDC-pvd-abrt Abort Reason parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.18.2.1 | Upon the receipt of a AIDC-pvd-abrt indication primitive the CF shall: if the CF is in the DATA TRANSFER state: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.2.18.2.1 | Upon the receipt of a AIDC-pvd-abrt indication primitive the CF shall: if the CF is in the RELEASE PENDING state invoke a Provider-abort indication primitive with the Provider-abort Provider Abort Reason parameter having the abstract value delivered in the AIDC-pvd-abrt Abort Reason parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.1.1.1 | It shall be valid to invoke the AIDC-DATA request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.1.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.3.1.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.3.1.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.3.1.2.1 | Upon the receipt of a AIDC-DATA request primitive the CF shall: encode the AIDC-DATA Request User Data parameter using the definition of presentation-user-data in 4.3.2.6. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.1.2.1 | Upon the receipt of a AIDC-DATA request primitive the CF shall: invoke a P-DATA request primitive with the resulting encoding from a) above, as the User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.2.1.1 | It shall be valid to invoke the AIDC-ABORT request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.2.1.1 | It shall be valid to invoke the AIDC-ABORT request primitive when the CF is in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.2.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.3.2.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.3.2.2.1 | Upon the receipt of a AIDC-ABORT request primitive the CF shall: if the CF is in the DATA TRANSFER state: invoke an A-ABORT request primitive with parameters as follows: if no AIDC-ABORT Abort Reason parameter is present, the A-ABORT AbortSource parameter set to abstract value "acse-service-user" and the A-ABORT Diagnostic value set ro the abstract value "no-reason-given". |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.2.2.1 | Upon the receipt of a AIDC-ABORT request primitive the CF shall: if the CF is in the DATA TRANSFER state: invoke an A-ABORT request primitive with parameters as follows: if the AIDC-ABORT Abort Reason parameter the abstract value "protocolerror", the A-ABORT AbortSource prameter set to abstract value "acse-service-user" and the A-ABORT Diagnostic value set to the abstract value "protocol-error". |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.2.2.1 | Upon the receipt of a AIDC-ABORT request primitive the CF shall: if the CF is in the DATA TRANSFER state: invoke an A-ABORT request primitive with parameters as follows: if the AIDC-ABORT Abort Reason parameter has the abstract value "timerexpired", the A-ABORT AbortSource prameter set to abstract value "acse-service-user" and the A-ABORT Diagnostic value set to the abstract value "protocol-error". |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.2.2.1 | Upon the receipt of a AIDC-ABORT request primitive the CF shall: if the CF is in the DATA TRANSFER state: invoke an A-ABORT request primitive with parameters as follows:otherwise, the A-ABORT AbortSource parameter set to the abstract value "acse-service-user" and no A-ABORT Diagnostic value. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.2.2.1 | Upon the receipt of a AIDC-ABORT request primitive the CF shall: if the CF is in the DATA TRANSFER state: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.3.2.2.1 | Upon the receipt of a AIDC-ABORT request primitive the CF shall: if the CF is in the RELEASE PENDING state, invoke an A-ABORT request primitive with no parameters. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.1.1 | It shall be valid to invoke the A-ASSOCIATE indication primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.4.1.1.2 | If the CF is in the DATA TRANSFER state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.4.1.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.4.1.2.1 | Upon the receipt of an A-ASSOCIATE indication primitive, the CF shall: extract and store the encoded Calling ICAO Facility Designation from Calling AP Title parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Upon the receipt of an A-ASSOCIATE indication primitive, the CF shall: invoke an A-ASSOCIATE response primitive with the parameters as sfollows: |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Parameter | Value |  |  |  |  |
| 3.2.5.3.4.1.2.1 | Mode | Not used (default value) | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Application Context Name | {iso (1) identified-organization (3) icao (27) atn-ac (3) 1 | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Application Context Name List | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Responding AP Title | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Responding AE Qualifier | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Responding AP Invocation-identifier | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Responding AE Invocation-identifier | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | ACSE Requirements | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Authentication-mechanism Name | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Authentication-value | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | User Information | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Result | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Diagnostic | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Responding Presentation Address | Local Implementation | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Presentation Context Definition Result List | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Default Presentation Context Result | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Quality of Service | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Presentation Requirements | Not used (default value) | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Session Requirements | No Orderly Release (NOR), Duplex | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Initial Synchronization Point Serial No | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Initial Assignment of Tokens | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.1.2.1 | Session-connection Identifier | Not used | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.1.1 | It shall be valid to invoke the A-ASSOCIATE confirmation primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.4.2.1.2 | If the CF is in the DATA TRANSFER state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.4.2.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the Result parameter has the abstract value "accepted", then: if the nfy-assc variable is set:- invoke an AIDC-nfy request primitive with: the stored Notification Information as the User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the Result parameter has the abstract value "accepted", then: if the nfy-assc variable is set:- invoke an AIDC-nfy request primitive with: the stored Message Number parameter as the Msg Number parameter |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the Result parameter has the abstract value "accepted", then: if the nfy-assc variable is set:- invoke an AIDC-nfy request primitive with: the DATA TRANSFER state |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the Result parameter has the abstract value "accepted", then: if the crd-assc variable is set:- invoke an AIDC-crd-start request primitive with: the stored Coordinate Start Information as the User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the Result parameter has the abstract value "accepted", then: if the crd-assc variable is set:- invoke an AIDC-crd-start request primitive with: the stored Message Number parameter as the Msg Number parameter |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the Result parameter has the abstract value "accepted", then: if the crd-assc variable is set:- invoke an AIDC-crd-start request primitive with: the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the inf-assc variable is set:- invoke an AIDC-inf-tfr request primitive with: the stored Info-transfer Information as the User Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the inf-assc variable is set:- invoke an AIDC-inf-tfr request primitive with: the stored Message Number parameter as the Msg Number parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the inf-assc variable is set:- invoke an AIDC-crd-start request primitive with: the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.2.2.1 | Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall: if the Result parameter has the abstract value "rejected (permanent)", then: invoke a Provider-abort indication primitive with the A-ASSOCIATE confirmation primitive Result Source parameter as the Provider-abort indication primitive Provider Abort Reason parameter set to the abstract value "rejectedpermanent" or "rejectedtransient" corresponding to the value of the A-ASSOCIATE indication Result parameter and enter the NULL state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.1.1 | It shall be valid to invoke the A-ABORT indication primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.1.1 | It shall be valid to invoke the A-ABORT indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.1.1 | It shall be valid to invoke the A-ABORT indication primitive when the CF is in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the ASSOCIATION PENDING state: invoke an AIDC-ABORT indication primitive as follows: in each case, ignore any delivered A-ABORT UserInformation parameter value. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the DATA TRANSFER state: invoke an AIDC-ABORT indication primitive as follows: in each case, ignore any delivered A-ABORT UserInformation parameter value. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the ASSOCIATION PENDING state: invoke an AIDC-ABORT indication primitive as follows: if the A-ABORT AbortSource parameter has the abstract value "acse-service-user" and the A-ABORT Diagnostic parameter has the abstract value "no-reason-given", omit the AIDC-ABORT AbortReason parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the DATA TRANSFER state: invoke an AIDC-ABORT indication primitive as follows: if the A-ABORT AbortSource parameter has the abstract value "acse-service-user" and the A-ABORT Diagnostic parameter has the abstract value "no-reason-given", omit the AIDC-ABORT AbortReason parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the ASSOCIATION PENDING state: invoke an AIDC-ABORT indication primitive as follows: if the A-ABORT Diagnostic parameter has the abstract value "protocol-error", set the AIDC-ABORT AbortReason parameter to the abstract value "protocolerror". |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the DATA TRANSFER state: invoke an AIDC-ABORT indication primitive as follows: if the A-ABORT Diagnostic parameter has the abstract value "protocol-error", set the AIDC-ABORT AbortReason parameter to the abstract value "protocolerror". |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the ASSOCIATION PENDING state: invoke an AIDC-ABORT indication primitive as follows: otherwise, set the AIDC-ABORT Abort Reason parameter to the abstract value "undefinederror". |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the DATA TRANSFER state: invoke an AIDC-ABORT indication primitive as follows: otherwise, set the AIDC-ABORT Abort Reason parameter to the abstract value "undefinederror". |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the ASSOCIATION PENDING state: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the DATA TRANSFER state: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.3.2.1 | Upon the receipt of an A-ABORT indication primitive, the CF shall: if the CF is in the RELEASE PENDING state, enter the NULL state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.1.1 | It shall be valid to invoke the A-P-ABORT indication primitive when the CF is in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.1.1 | It shall be valid to invoke the A-P-ABORT indication primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.1.1 | It shall be valid to invoke the A-P-ABORT indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.4.4.2.1 | Upon the receipt of an A-P-ABORT indication primitive, the CF shall: if in the ASSOCIATION PENDING state: invoke a Provider-abort indication with abstract value "providererror" as the value of the Provider-abort indication primitive Provider Abort Reason parameter, and discard any ProviderReason parameter in the A-P-ABORT indication. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.2.1 | Upon the receipt of an A-P-ABORT indication primitive, the CF shall: if in the ASSOCIATION PENDING state: enter the NULL state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.2.1 | Upon the receipt of an A-P-ABORT indication primitive, the CF shall: if in the DATA TRANSFER state: invoke an AIDC-P-ABORT indication primitive, and discard any Provider Reason parameter in the A-P-ABORT indication. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.2.1 | Upon the receipt of an A-P-ABORT indication primitive, the CF shall: if in the RELEASE PENDING state: invoke an AIDC-P-ABORT indication primitive, and discard any Provider Reason parameter in the A-P-ABORT indication. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.2.1 | Upon the receipt of an A-P-ABORT indication primitive, the CF shall: if in the DATA TRANSFER state: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.4.4.2.1 | Upon the receipt of an A-P-ABORT indication primitive, the CF shall: if in the RELEASE PENDING state: enter the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.1.1.1 | It shall be valid to invoke the P-CONNECT request primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.1.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.5.1.1.2 | If the CF is in the DATA TRANSFER state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.5.1.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.5.1.2.1 | Upon the receipt of an P-CONNECT request primitive, the CF shall invoke the equivalent Presentation service primitive of the ATN service provider. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.2.1.1 | It shall be valid to invoke the P-CONNECT response primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.2.1.2 | If the CF is in the DATA TRANSFER state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.5.2.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.5.2.2.1 | Upon the receipt of an P-CONNECT response primitive, the CF shall: transparently invoke the equivalent presentation service primitive. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.2.2.1 | Upon the receipt of an P-CONNECT response primitive, the CF shall: enter the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.3.1.1 | It shall be valid to invoke the P-U-ABORT request primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.3.1.1 | It shall be valid to invoke the P-U-ABORT request primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.3.1.1 | It shall be valid to invoke the P-U-ABORT request primitive when the CF is in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.3.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.5.3.2.1 | Upon the receipt of an P-U-ABORT request primitive, the CF shall: if the P-U-Abort request user data parameter is present, and the CF is in the DATA TRANSFER state: encode the presentation user data as indicated in 4.3.2.6 with the P-U-ABORT user data parameter (an ACSE ABRT APDU) as the presentation data value and presentation context identifier value corresponding to "acse-apdu". |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.3.2.1 | Upon the receipt of an P-U-ABORT request primitive, the CF shall: if the P-U-Abort request user data parameter is present, and the CF is in the DATA TRANSFER state: invoke a P-DATA Request primitive with the resulting encoding as the value of the UserData parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.3.2.1 | Upon the receipt of an P-U-ABORT request primitive, the CF shall: if the P-U-Abort request user data parameter is not present: invoke a P-U-ABORT request primitive without any parameters. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.3.2.1 | Upon the receipt of an P-U-ABORT request primitive, the CF shall: if the CF is not in the DATA TRANSFER state: invoke a P-U-ABORT request primitive without any parameters. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.5.3.2.1 | Upon the receipt of an P-U-ABORT request primitive, the CF shall: enter the NULL state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.1.1.1 | It shall be valid to invoke the P-CONNECT indication primitive when the CF is in the NULL state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.1.1.2 | If the CF is in the ASSOCIATION PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.1.1.2 | If the CF is in the DATA TRANSFER state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.1.1.2 | If the CF is in the the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.1.2.1 | Upon the receipt of an P-CONNECT indication primitive, the CF shall: enter the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.1.2.1 | Upon the receipt of an P-CONNECT indication primitive, the CF shall: invoke the equivalent Presentation service primitive at the lower ACSE service boundary. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.2.1.1 | It shall be valid to invoke the P-CONNECT confirmation primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.2.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.2.1.2 | If the CF is in the DATA TRANSFER state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.2.1.2 | If the CF is in the RELEASE PENDING state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.2.2.1 | Upon the receipt of an P-CONNECT confirmation primitive, the CF shall invoke the equivalent Presentation service primitive at the lower ACSE service boundary. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.3.1.1 | It shall be valid to invoke the P-U-ABORT indication primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.3.1.1 | It shall be valid to invoke the P-U-ABORT indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.3.1.1 | It shall be valid to invoke the P-U-ABORT indication primitive when the CF is in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.3.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.3.2.1 | Upon the receipt of an P-U-ABORT indication primitive, the CF shall invoke the equivalent Presentation service primitive at the lower ACSE service boundary. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.4.1.1 | It shall be valid to invoke the P-P-ABORT indication primitive when the CF is in the ASSOCIATION PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.4.1.1 | It shall be valid to invoke the P-P-ABORT indication primitive when the CF is in the the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.4.1.1 | It shall be valid to invoke the P-P-ABORT indication primitive when the CF is in the RELEASE PENDING state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.4.1.2 | If the CF is in the NULL state, then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.4.2.1 | Upon the receipt of an P-P-ABORT indication primitive, the CF shall invoke the corresponding Presentation service primitive at the lower ACSE service boundary. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.5.1.1 | It shall be valid to invoke the P-DATA indication primitive when the CF is in the DATA TRANSFER state. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.5.1.2 | If the CF is in the NULL state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.5.1.2 | If the CF is in the ASSOCIATION PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.5.1.2 | If the CF is in the RELEASE PENDING state then appropriate error recovery action should be taken. |  | recommendation |  |  |  |
| 3.2.5.3.6.5.2.1 | Upon the receipt of an P-DATA indication primitive, the CF shall: decode the presentation-user-data as indicated in 4.3.2.6 to determine the destination ASE of the APDU. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.5.2.1 | Upon the receipt of an P-DATA indication primitive, the CF shall: extract the Presentation data value. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.5.2.1 | Upon the receipt of an P-DATA indication primitive, the CF shall: if the presentation context identifier has the abstract value "user-ase", invoke an AIDC-DATA indication primitive with the extracted AIDC-APDU as the AIDC-DATA AIDC Data parameter. |  | mandatory (CF) |  |  |  |
| 3.2.5.3.6.5.2.1 | Upon the receipt of an P-DATA indication primitive, the CF shall: if the presentation context identifier has the abstract value "acse-ase" and the ACSE-APDU has the syntax ABRT-apdu, invoke a P-U-Abort indication primitive with the UserData parameter containing the received APDU. |  | mandatory (CF) |  |  |  |
| 3.2.5.4.1 | The AIDC-AE shall behave as if it has a CF which functions in accordance with the following state table which shows the state transitions and actions performed by the AIDC-CF in response to service primitives submitted to the AIDC-CF. The source of the service primitive invocation is shown in column one of the Table and the service primitive invocations are shown in the second column. Blank cells shall indicate error conditions. The error handling shall result in the association being aborted, if one exists. |  | mandatory (CF) |  |  |  |
| 3.2.5.4.4.2 | The AIDC-User should be notified when an association is aborted. |  | recommendation |  |  |  |

Table 3.2.5-3 AIDC CF State Table

| Source  | State  | STA0 | STA1 | STA2 | STA3 |
| --- | --- | --- | --- | --- | --- |
|  | Event  | NULL | ASSOCIATION PENDING | DATA TRANSFER | RELEASE PENDING |
| From AIDC-User | Notify Req | Store Notification Information A-ASSOCIATE Req STA1 |  | AIDC-nfy Req  STA2 |  |
|  | Coordinate-start Req | Store Confirmation Start Information set crd-assc A-ASSOCIATE Req STA 1 |  | AIDC-crd-start Req  STA2 |  |
|  | Info-transfer Req | Store Info-transfer informatlon set inf-assc A-ASSOCIATE Req  STA1 |  | AIDC-inf-tfr Req  STA2 |  |
|  | End Req |  |  | set end = Message Number AIDC-end Req  STA2 |  |
|  | User-abort Req |  | A-ABORT Req  STA3 | AlDC-usr-abrt Req  STA3 |  STA0 |
|  | All other Req |  |  | equivqlent AIDC-ASE Req  STA2 |  |
| From AIDC-ASE (upper) | AIDC-ucf Ind+ |  |  | if end = Reference ID User confirmation Ind+ A-ABORT Req  STA3 if end = NULL User confirmation Ind+  STA2 |  |
|  | AlDC-ucf Ind- |  |  | if end = Reference ID User confirmation Ind- set end=NULL if end = NULL User-confirmation Ind-  STA2 |  |
|  | AIDC-usr-abrt Ind |  | User-abort Ind  STA0 | User-abort Ind  STA0 | User-abort Ind  STA0 |
|  | AIDC-pvd-abrt Ind |  | Provider-abort Ind  STA0 | Provider-abort Ind  STA0 | Provider-abort Ind  STA0 |
|  | All other Ind |  |  | equivqlent AIDC-User service invocation  STA2 |  |
| From AIDC-ASE (lower) | AIDC-DATA Req |  |  | P-DATA Req (APDU)  STA2 |  |
|  | AIDC-ABORT Req |  | A-ABORT Rcq  STA3 | A-ABORT Req  STA3 |  |
|  | A ASSOCIATE lnd |  | A-ASSOCIATE Rsp+  STA1 |  |  |
|  | A-ASSOCIATE Cnf+ |  | if nfy-assc AIDC-nfy Req or if crd-assc AIDC-crd-start Req or if inf-assc AIDC-inf-tfr Req  STA2 |  |  |
| From ACSE (upper) | A-ASSOCIATE Cnf- |  | Provider-abort Ind  STA0 |  |  |
|  | A-ABORT Ind |  | Provider-abort Ind  STA0 | AIDC-ABORT Ind  STA3 | AIDC-ABORT Ind  STA3 |
|  | A-P-ABORT Ind |  | Provider-abort Ind  STA0 | AIDC-P-ABORT Ind  STA3 | AIDC-P-ABORT Ind  STA3 |
|  | P-CONNECT Req |  | P-CONNECT Req  STA1 |  |  |
|  | P-CONNECT Rsp+ |  | P-CONNECT Rsp+  STA2 |  |  |
| From ACSE (lower) | P-CONNECT Rsp- |  | P-CONNECT Rsp-  STA0 |  |  |
|  | P-U-ABORT Req |  | P-U-ABORT Req  STA0 | P-U-ABORT Req  STA0 | P-U-ABORT Req  STA0 |
|  | P-CONNECT Ind | P-CONNECT Ind  STA1 |  |  |  |
|  | P-CONNECT Cnf+/- |  | P-CONNECT Cnf  STA2 |  |  |
| From ATN Service Provider | P-DATA Ind |  |  | P-DATA Ind  STA2 |  |
|  | P-U-ABORT Ind |  | P-U-ABORT Ind  STA1 | P-U-ABORT Ind  STA2 | P-U-ABORT Ind  STA3 |
|  | P-P-ABORT Ind |  | P-P-ABORT Ind  STA1 | P-P-ABORT Ind  STA2 | P-P-ABORT Ind  STA3 |

| Paragraph | Requirement | Usage | Implementation | Test |
| --- | --- | --- | --- | --- |
| 3.2.6.1.4.1 | When the predicate P1 is set true, then the AIDC-tfr-accept service shall be used in response to the AIDC-tfr-prpsl service. | mandatory |  |  |
| 3.2.6.1.4.2 | When the predicate P1 is set false, the invocation of the AIDC-tfr-accept service shall be prohibited. | mandatory |  |  |
| 3.2.6.1.5.1.1 | It shall be valid to invoke the AIDC-nfy request primitive when the AIDC-ASE protocol machine is in the IDLE state, or the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the IDLE state: create an AIDC-nfy-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the NOTIFY state: create an AIDC-nfy-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the IDLE state: invoke an AIDC-DATA request primitive with the AIDC-nfy-apdu as the AIDC Data parameter | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the NOTIFY state: invoke an AIDC-DATA request primitive with the AIDC-nfy-apdu as the AIDC Data parameter | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the IDLE state: if running, stop timer t 2IN. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the NOTIFY state: if running, stop timer t 2IN. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: if in the NOTIFY state, then stop the timer t 2NC . | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the IDLE state: start the timer t C. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the NOTIFY state: start the timer t C. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the IDLE state or the NOTIFY state: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If in the IDLE state or the NOTIFY state: set the variable vs 1 = notify. | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If the AIDC-ASE protocol machine is any other state: invoke an AIDC-pvd-abrt indication primitive with the AbortReason parameter set to the abstract value: "protocolerror". | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If the AIDC-ASE protocol machine is any other state: invoke an AIDC-ABORT request primitive with the AbortReason set to abstract value "protocolerror". | mandatory |  |  |
| 3.2.6.1.5.2.1 | Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall: If the AIDC-ASE protocol machine is any other state: enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.6.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-nfy-apdu when the AIDC-ASE protocol machine is in the IDLE state. | mandatory |  |  |
| 3.2.6.1.6.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-nfy-apdu when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.6.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-nfy-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-nfy-apdu. | mandatory |  |  |
| 3.2.6.1.6.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-nfy-apdu the AIDC-ASE shall: invoke an AIDC-nfy indication primitive with the extracted User Data and Msg Number parameters as the AIDC-nfy indication primitive User Data parameter, and the AIDC-nfy indication Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.6.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-nfy-apdu the AIDC-ASE shall: if running stop the timer t 1IN . | mandatory |  |  |
| 3.2.6.1.6.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-nfy-apdu the AIDC-ASE shall: if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.6.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-nfy-apdu the AIDC-ASE shall: if in the NOTIFY state, then stop the timer t 1NC . | mandatory |  |  |
| 3.2.6.1.6.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-nfy-apdu the AIDC-ASE shall: set the variable vr 1 = notify. | mandatory |  |  |
| 3.2.6.1.6.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-nfy-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.7.1.1 | It shall be valid to invoke the AIDC-crd-start request primitive when the AIDC-ASE protocol machine is in the IDLE state. | mandatory |  |  |
| 3.2.6.1.7.1.1 | It shall be valid to invoke the AIDC-crd-start request primitive when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.7.1.1 | It shall be valid to invoke the AIDC-crd-start request primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.7.1.1 | It shall be valid to invoke the AIDC-crd-start request primitive when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: create an AIDC-crd-start-apdu based on the User Data parameter, and the Msg Number parameter | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: invoke an AIDC-DATA request primitive with the AIDC-crd-start-apdu as the AIDC Data parameter | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: if running, stop timer t 2IN . | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: if in the NOTIFY state, stop timer T 1CT . | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: if in the COORDINATED state, then: if running, stop timer t 1CT . | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: if in the COORDINATED state, then: if running, stop timer t 2CT . | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: if in the TRANSFERRED state, then stop the timer t TE . | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: set the variable vs 1 = back. | mandatory |  |  |
| 3.2.6.1.7.2.1 | Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.8.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-start-apdu when the AIDC-ASE protocol machine is in the IDLE state. | mandatory |  |  |
| 3.2.6.1.8.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-start-apdu when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.8.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-start-apdu when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.8.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-start-apdu when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-crd-start-apdu. | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: invoke an AIDC-crd-start indication primitive with the extracted User Data and the Msg Number parameters as the AIDC-crd-start indication Msg Number parameter and the AIDC-crd-start indication primitive User Data parameter. | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: if running, stop timer t 1IN . | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: if in the NOTIFY state, then stop the timer t 1NC . | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: if in the COORDINATED state, then: if running, stop timer t 2CT . | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: if in the COORDINATED state, then: if running, stop the timer t 1CT . | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: if in the TRANSFERRED state, then stop the timer t TE . | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: set the variable vr 1 =back. | mandatory |  |  |
| 3.2.6.1.8.2.1 | Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-apdu the AIDC ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.9.1.1 | It shall be valid to invoke the AIDC-crd-end request primitive when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.9.1.1 | It shall be valid to invoke the AIDC-crd-end request primitive when the AIDC-ASE protocol machine is in the RE-NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.9.1.1 | It shall be valid to invoke the AIDC-crd-end request primitive when the AIDC-ASE protocol machine is in the BACKWARD COORDINATING state. | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: create an AIDC-crd-end-apdu based on the User Data parameter, the Result parameter and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-crd-end-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: if in the NEGOTIATING state, then set the variable vs 1 = coord-end. | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: if in the NEGOTIATING state, then if the AIDC-crd-end Result parameter has the value accept then set the variable vse=accept. | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: if in the NEGOTIATING state, then if the AIDC-crd-end Result parameter has the value reject then set the variable vse=reject. | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: if in the RE-NEGOTIATING state, set the variable vs 1 = coord-end. | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: if in the BACKWARD COORDINATING state, set the variable vs 1 = back-end. | mandatory |  |  |
| 3.2.6.1.9.2.1 | Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.10.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-end-apdu when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.10.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-end-apdu when the AIDC-ASE protocol machine is in the RE-NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.10.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-end-apdu when the AIDC-ASE protocol machine is in the BACKWARD COORDINATING state. | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: extract the User Data parameter, the Result parameter and the Msg Number parameter from the AIDC-crd-end-apdu. | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: invoke an AIDC-crd-end indication primitive with the extracted User Data, Result and Msg Number parameters as the AIDC-crd-end indication User Data parameter, the AIDC-crd-end indication Result parameter and the AIDC-crd-end indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: if running, stop the timer t 1R ; d) if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: if running, stop the timer t S . | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: if in the NEGOTIATING state, then: set the variable vr 1 = coord-end. | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: if in the NEGOTIATING state, then: if the AIDC-crd-end Result parameter has the value accept then set the variable v re = accept. | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: if in the RE-NEGOTIATING state, set the variable v s1 = coord-end. | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: if in the BACKWARD COORDINATING state, set the variable vr 1 = back-end. | mandatory |  |  |
| 3.2.6.1.10.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-end-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.11.1.1 | It shall be valid to invoke the AIDC-crd-ngtt request primitive when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.11.1.1 | It shall be valid to invoke the AIDC-crd-ngtt request primitive when the AIDC-ASE protocol machine is in the RE-NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.11.1.1 | It shall be valid to invoke the AIDC-crd-ngtt request primitive when the AIDC-ASE protocol machine is in the BACKWARD-COORDINATING state. | mandatory |  |  |
| 3.2.6.1.11.2.1 | Upon the receipt of an AIDC-crd-ngtt request primitive the AIDC-ASE shall: create an AIDC-crd-ngtt-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.11.2.1 | Upon the receipt of an AIDC-crd-ngtt request primitive the AIDC-ASE shall: invoke an AIDC-DATA request primitive with the AIDC-crd-ngtt-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.11.2.1 | Upon the receipt of an AIDC-crd-ngtt request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.11.2.1 | Upon the receipt of an AIDC-crd-ngtt request primitive the AIDC-ASE shall: set the variable vs 1 = coord-negot. | mandatory |  |  |
| 3.2.6.1.11.2.1 | Upon the receipt of an AIDC-crd-ngtt request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.12.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.12.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu when the AIDC-ASE protocol machine is in the RE-NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.12.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu when the AIDC-ASE protocol machine is in the BACKWARD-COORDINATING state. | mandatory |  |  |
| 3.2.6.1.12.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-crd-ngtt-apdu. | mandatory |  |  |
| 3.2.6.1.12.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu the AIDC-ASE shall: invoke an AIDC-crd-ngtt indication primitive with the extracted User Data and Msg Number parameters as the AIDC-crd-ngtt indication User Data parameter, and the AIDC-crd-ngtt indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.12.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu the AIDC-ASE shall: if running, stop the timer t 1R . | mandatory |  |  |
| 3.2.6.1.12.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu the AIDC-ASE shall: if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.12.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu the AIDC-ASE shall: if running, stop the timer t S . | mandatory |  |  |
| 3.2.6.1.12.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu the AIDC-ASE shall: set the variable vr 1 = coord-negot. | mandatory |  |  |
| 3.2.6.1.12.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-ngtt-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.13.1.1 | It shall be valid to invoke the AIDC-crd-stndby request primitive when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.13.1.1 | It shall be valid to invoke the AIDC-crd-stndby request primitive when the AIDC-ASE protocol machine is in the RE-NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.13.1.1 | It shall be valid to invoke the AIDC-crd-stndby request primitive when the AIDC-ASE protocol machine is in the BACKWARD-COORDINATING state. | mandatory |  |  |
| 3.2.6.1.13.2.1 | Upon the receipt of an AIDC-crd-stndby request primitive the AIDC-ASE shall: create an AIDC-crd-stndby-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.13.2.1 | Upon the receipt of an AIDC-crd-stndby request primitive the AIDC-ASE shall: invoke an AIDC-DATA request primitive with the AIDC-crd-stndby-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.13.2.1 | Upon the receipt of an AIDC-crd-stndby request primitive the AIDC-ASE shall: start the timer t C. | mandatory |  |  |
| 3.2.6.1.13.2.1 | Upon the receipt of an AIDC-crd-stndby request primitive the AIDC-ASE shall: set the variable vs 1 = coord-stndby. | mandatory |  |  |
| 3.2.6.1.13.2.1 | Upon the receipt of an AIDC-crd-stndby request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.14.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.14.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu when the AIDC-ASE protocol machine is in RE-NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.14.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu when the AIDC-ASE protocol machine is in the BACKWARD-COORDINATING state. | mandatory |  |  |
| 3.2.6.1.14.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-crd-stndby-apdu. | mandatory |  |  |
| 3.2.6.1.14.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu the AIDC-ASE shall: invoke an AIDC-crd-stndby indication primitive with the extracted User Data and Msg Number parameters as the AIDC-crd-stndby indication User Data parameter, and the AIDC-crd-stndby indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.14.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu the AIDC-ASE shall: if running, stop the timer t 1R . | mandatory |  |  |
| 3.2.6.1.14.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu the AIDC-ASE shall: if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.14.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu the AIDC-ASE shall: set the variable vs 1 = coord-stndby. | mandatory |  |  |
| 3.2.6.1.14.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-stndby-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.15.1.1 | It shall be valid to invoke the AIDC-tfr-init request primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.15.2.1 | Upon the receipt of an AIDC-tfr-init request primitive the AIDC-ASE shall: create an AIDC-tfr-init-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.15.2.1 | Upon the receipt of an AIDC-tfr-init request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-tfr-init-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.15.2.1 | Upon the receipt of an AIDC-tfr-init request primitive the AIDC-ASE shall: if running, stop the timer t 1CT . | mandatory |  |  |
| 3.2.6.1.15.2.1 | Upon the receipt of an AIDC-tfr-init request primitive the AIDC-ASE shall: if running, stop the timer t 2CT . | mandatory |  |  |
| 3.2.6.1.15.2.1 | Upon the receipt of an AIDC-tfr-init request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.15.2.1 | Upon the receipt of an AIDC-tfr-init request primitive the AIDC-ASE shall: set the variable vs 1 = trns-init. | mandatory |  |  |
| 3.2.6.1.15.2.1 | Upon the receipt of an AIDC-tfr-init request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.16.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-init-apdu when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.16.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-init-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-tfr-init-apdu. | mandatory |  |  |
| 3.2.6.1.16.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-init-apdu the AIDC-ASE shall: invoke an AIDC-tfr-init indication primitive with the extracted User Data and Msg Number parameters as the AIDC-tfr-init indication User Data parameter, and the AIDC-tfr-init indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.16.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-init-apdu the AIDC-ASE shall: if running, stop the timer t 1CT . | mandatory |  |  |
| 3.2.6.1.16.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-init-apdu the AIDC-ASE shall: if running, stop the timer t 2CT . | mandatory |  |  |
| 3.2.6.1.16.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-init-apdu the AIDC-ASE shall: set the variable vr 1 = trns-init. | mandatory |  |  |
| 3.2.6.1.16.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-init-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.17.1.1 | It shall be valid to invoke the AIDC-tfr-rqst request primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.17.2.1 | Upon the receipt of an AIDC-tfr-rqst request primitive the AIDC-ASE shall: create an AIDC-tfr-rqst-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.17.2.1 | Upon the receipt of an AIDC-tfr-rqst request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-tfr-rqst-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.17.2.1 | Upon the receipt of an AIDC-tfr-rqst request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.17.2.1 | Upon the receipt of an AIDC-tfr-rqst request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.18.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-rqst-apdu when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.18.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-rqst-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-tfr-rqst-apdu. | mandatory |  |  |
| 3.2.6.1.18.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-rqst-apdu the AIDC-ASE shall: invoke an AIDC-tfr-rqst indication primitive with the extracted User Data parameter and Msg Number parameters as the AIDC-tfr-rqst indication User Data parameter, and the AIDC-tfr-rqst. | mandatory |  |  |
| 3.2.6.1.18.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-rqst-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.19.1.1 | It shall be valid to invoke the AIDC-tfr-prpsl request primitive when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.19.2.1 | Upon the receipt of an AIDC-tfr-prpsl request primitive the AIDC-ASE shall: create an AIDC-tfr-prpsl-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.19.2.1 | Upon the receipt of an AIDC-tfr-prpsl request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-tfr-prpsl-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.19.2.1 | Upon the receipt of an AIDC-tfr-prpsl request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.19.2.1 | Upon the receipt of an AIDC-tfr-prpsl request primitive the AIDC-ASE shall: if P1, then set the variable vs 1 = trns-prpsl. | mandatory |  |  |
| 3.2.6.1.19.2.1 | Upon the receipt of an AIDC-tfr-prpsl request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.20.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-prpsl-apdu when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.20.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-prpsl-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-tfr-prpsl-apdu. | mandatory |  |  |
| 3.2.6.1.20.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-prpsl-apdu the AIDC-ASE shall: invoke an AIDC-tfr-prpsl indication primitive with the extracted User Data and Msg Number parameters as the AIDC-tfr-prpsl indication User Data parameter, and the AIDC-tfr-prpsl indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.20.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-prpsl-apdu the AIDC-ASE shall: if P1, then set the variable vr 1 = trns-prpsl. | mandatory |  |  |
| 3.2.6.1.20.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-prpsl-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.21.1.1 | It shall be valid to invoke the AIDC-tfr-accept request primitive if the predicate P1 is true and when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.21.2.1 | Upon the receipt of an AIDC-tfr-accept request primitive the AIDC-ASE shall: create an AIDC-tfr-accept-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.21.2.1 | Upon the receipt of an AIDC-tfr-accept request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-tfr-accept-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.21.2.1 | Upon the receipt of an AIDC-tfr-accept request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.21.2.1 | Upon the receipt of an AIDC-tfr-accept request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.22.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-accept-apdu if the predicate P1 is true and when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.22.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-accept-apdu the AIDC-ASE shall: if running stop timer t 1R . | mandatory |  |  |
| 3.2.6.1.22.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-accept-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-tfr-accept-apdu;. | mandatory |  |  |
| 3.2.6.1.22.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-accept-apdu the AIDC-ASE shall: invoke an AIDC-tfr-accept indication primitive with the extracted User Data and Msg Number parameters as the AIDC-tfr-accept indication User Data parameter, and the AIDC-tfr-accept indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.22.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-accept-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.23.1.1 | It shall be valid to invoke the AIDC-tfr-comm request primitive when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.23.2.1 | Upon the receipt of an AIDC-tfr-comm request primitive the AIDC-ASE shall: create an AIDC-tfr-comm-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.23.2.1 | Upon the receipt of an AIDC-tfr-comm request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-tfr-comm-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.23.2.1 | Upon the receipt of an AIDC-tfr-comm request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.23.2.1 | Upon the receipt of an AIDC-tfr-comm request primitive the AIDC-ASE shall: set the variable vs 1 = trns-comm. | mandatory |  |  |
| 3.2.6.1.23.2.1 | Upon the receipt of an AIDC-tfr-comm request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.24.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-comm-apdu when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.24.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-tfr-comm-apdu. | mandatory |  |  |
| 3.2.6.1.24.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-apdu the AIDC-ASE shall: invoke an AIDC-tfr-comm indication primitive with the extracted User Data and Msg Number parameters as the AIDC-tfr-comm indication User Data parameter, and the AIDC-tfr-comm indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.24.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-apdu the AIDC-ASE shall: if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.24.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-apdu the AIDC-ASE shall: if running, stop the timer t 3R . | mandatory |  |  |
| 3.2.6.1.24.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-apdu the AIDC-ASE shall: set the variable vr 1 = trns-comm. | mandatory |  |  |
| 3.2.6.1.24.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.25.1.1 | It shall be valid to invoke the AIDC-tfr-comm-assm request primitive when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.25.1.1 | It shall be valid to invoke the AIDC-tfr-comm-assm request primitive when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.25.2.1 | 3.2.6.1.25.2.1 Upon the receipt of an AIDC-tfr-comm-assm request primitive the AIDC-ASE shall: create an AIDC-tfr-comm-assm-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.25.2.1 | 3.2.6.1.25.2.1 Upon the receipt of an AIDC-tfr-comm-assm request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-tfr-comm-assm-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.25.2.1 | 3.2.6.1.25.2.1 Upon the receipt of an AIDC-tfr-comm-assm request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.25.2.1 | 3.2.6.1.25.2.1 Upon the receipt of an AIDC-tfr-comm-assm request primitive the AIDC-ASE shall: if running, stop the timer t 3R . | mandatory |  |  |
| 3.2.6.1.25.2.1 | 3.2.6.1.25.2.1 Upon the receipt of an AIDC-tfr-comm-assm request primitive the AIDC-ASE shall: set the variable vs 1 = trns-assm. | mandatory |  |  |
| 3.2.6.1.25.2.1 | 3.2.6.1.25.2.1 Upon the receipt of an AIDC-tfr-comm-assm request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.26.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-comm-assm-apdu when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.26.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-comm-assm-apdu when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.26.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-assm-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-tfr-comm-assm-apdu. | mandatory |  |  |
| 3.2.6.1.26.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-assm-apdu the AIDC-ASE shall: a) extract the parameter from the AIDC-tfr-comm-assm-apdu; b) invoke an AIDC-tfr-comm-assm indication primitive with the extracted User Data and Msg Number parameters as the AIDC-tfr-comm-assm indication User Data parameter, and the AIDC-tfr-comm-assm indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.26.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-assm-apdu the AIDC-ASE shall: if running, stop the timer t 1R . | mandatory |  |  |
| 3.2.6.1.26.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-assm-apdu the AIDC-ASE shall: if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.26.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-assm-apdu the AIDC-ASE shall: set the variable vr 1 = trns-assm. | mandatory |  |  |
| 3.2.6.1.26.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-comm-assm-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.27.1.1 | It shall be valid to invoke the AIDC-tfr-cntrl request primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.27.2.1 | Upon the receipt of an AIDC-tfr-cntrl request primitive the AIDC-ASE shall: create an AIDC-tfr-cntrl-Req-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.27.2.1 | Upon the receipt of an AIDC-tfr-cntrl request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-tfr-cntrl-Req-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.27.2.1 | Upon the receipt of an AIDC-tfr-cntrl request primitive the AIDC-ASE shall: if running, stop the timer t 1CT . | mandatory |  |  |
| 3.2.6.1.27.2.1 | Upon the receipt of an AIDC-tfr-cntrl request primitive the AIDC-ASE shall:. if running, stop the timer t 2CT . | mandatory |  |  |
| 3.2.6.1.27.2.1 | Upon the receipt of an AIDC-tfr-cntrl request primitive the AIDC-ASE shall:. start the timer t C . | mandatory |  |  |
| 3.2.6.1.27.2.1 | Upon the receipt of an AIDC-tfr-cntrl request primitive the AIDC-ASE shall: set the variable vs 1 = trn-start. | mandatory |  |  |
| 3.2.6.1.27.2.1 | Upon the receipt of an AIDC-tfr-cntrl request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.28.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-apdu when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.28.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-tfr-cntrl-Req-apdu. | mandatory |  |  |
| 3.2.6.1.28.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-apdu the AIDC-ASE shall: invoke an AIDC-tfr-cntrl indication primitive with the extracted User Data and Msg Number parameters as the AIDC-tfr-cntrl indication User Data parameter, and the AIDC-tfr-cntrl indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.28.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-apdu the AIDC-ASE shall: if running, stop the timer t 1CT . | mandatory |  |  |
| 3.2.6.1.28.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-apdu the AIDC-ASE shall: if running, stop the timer t 2CT. | mandatory |  |  |
| 3.2.6.1.28.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-apdu the AIDC-ASE shall: if running, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.28.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-apdu the AIDC-ASE shall: set the variable vr 1 = trn-start . | mandatory |  |  |
| 3.2.6.1.28.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.29.1.1 | It shall be valid to invoke the AIDC-tfr-cntrl response primitive when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.29.2.1 | Upon the receipt of an AIDC-tfr-cntrl response primitive the AIDC-ASE shall: create an AIDC-tfr-cntrl-Rsp-apdu based on the User Data parameter, the Result parameter and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.29.2.1 | Upon the receipt of an AIDC-tfr-cntrl response primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-tfr-cntrl-Rsp-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.29.2.1 | Upon the receipt of an AIDC-tfr-cntrl response primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.29.2.1 | Upon the receipt of an AIDC-tfr-cntrl response primitive the AIDC-ASE shall: if the Result parameter has the abstract value "accepted"; set vs 1 = trns-accept. | mandatory |  |  |
| 3.2.6.1.29.2.1 | Upon the receipt of an AIDC-tfr-cntrl response primitive the AIDC-ASE shall: if the Result parameter has the abstract value "rejected"; set vs 1 = trns-reject. | mandatory |  |  |
| 3.2.6.1.29.2.1 | Upon the receipt of an AIDC-tfr-cntrl response primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.30.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Rsp-apdu when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.30.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Rsp-apdu the AIDC-ASE shall: extract the User Data parameter, the Result parameter, and the Msg Number parameter from the AIDC-tfr-cntrl-Rsp-apdu. | mandatory |  |  |
| 3.2.6.1.30.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Rsp-apdu the AIDC-ASE shall: invoke an AIDC-tfr-cntrl confirmation primitive with the extracted User Data, Result and Msg Number parameters as the AIDC-tfr-cntrl confirmation User Data parameter, and the AIDC-tfr-cntrl indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.30.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Rsp-apdu the AIDC-ASE shall: if running, stop the timer t 1R . | mandatory |  |  |
| 3.2.6.1.30.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Rsp-apdu the AIDC-ASE shall: if running, stop the timer t 2R. | mandatory |  |  |
| 3.2.6.1.30.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Rsp-apdu the AIDC-ASE shall: if the Result parameter has the abstract value "accepted"; set vr 1 = trns-accept. | mandatory |  |  |
| 3.2.6.1.30.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Rsp-apdu the AIDC-ASE shall: if the Result parameter has the abstract value "rejected"; set vr 1 = trns-reject. | mandatory |  |  |
| 3.2.6.1.30.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Rsp-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.31.1.1 | It shall be valid to invoke the AIDC-inf-tfr request primitive when the AIDC-ASE protocol machine is in the IDLE state. | mandatory |  |  |
| 3.2.6.1.31.1.1 | It shall be valid to invoke the AIDC-inf-tfr request primitive when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.31.1.1 | It shall be valid to invoke the AIDC-inf-tfr request primitive when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.31.1.1 | It shall be valid to invoke the AIDC-inf-tfr request primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.31.1.1 | It shall be valid to invoke the AIDC-inf-tfr request primitive when the AIDC-ASE protocol machine is in the IDLE state, or in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.31.1.1 | It shall be valid to invoke the AIDC-inf-tfr request primitive when the AIDC-ASE protocol machine is in the IDLE state, or in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.31.1.1 | It shall be valid to invoke the AIDC-inf-tfr request primitive when the AIDC-ASE protocol machine is in the IDLE state, or in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.31.2.1 | Upon the receipt of an AIDC-inf-tfr request primitive the AIDC-ASE shall: create an AIDC-info-tfr-apdu based on the User Data parameter, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.31.2.1 | Upon the receipt of an AIDC-inf-tfr request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-inf-tfr-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.31.2.1 | Upon the receipt of an AIDC-inf-tfr request primitive the AIDC-ASE shall: if running, stop the timer t 2IN . | mandatory |  |  |
| 3.2.6.1.31.2.1 | Upon the receipt of an AIDC-inf-tfr request primitive the AIDC-ASE shall: start the timer t C . | mandatory |  |  |
| 3.2.6.1.31.2.1 | Upon the receipt of an AIDC-inf-tfr request primitive the AIDC-ASE shall: set the variable vs 1 = info-trans. | mandatory |  |  |
| 3.2.6.1.31.2.1 | Upon the receipt of an AIDC-inf-tfr request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number | mandatory |  |  |
| 3.2.6.1.32.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu when the AIDC-ASE protocol machine is in the IDLE state. | mandatory |  |  |
| 3.2.6.1.32.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.32.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.32.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.32.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.32.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.32.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.32.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu the AIDC-ASE shall: extract the User Data parameter, and the Msg Number parameter from the AIDC-inf-tfr-apdu. | mandatory |  |  |
| 3.2.6.1.32.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu the AIDC-ASE shall: invoke an AIDC-inf-tfr indication primitive with the extracted User Data and Msg Number parameters as the AIDC-inf-tfr indication User Data parameter, and the AIDC-inf-tfr indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.32.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu the AIDC-ASE shall: if running, stop the timer t 1IN . | mandatory |  |  |
| 3.2.6.1.32.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu the AIDC-ASE shall: set the variable vr 1 = info-trans. | mandatory |  |  |
| 3.2.6.1.32.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-inf-tfr-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.33.1.1 | It shall be valid to invoke the AIDC-ucf request primitive when the AIDC-ASE protocol machine is in the IDLE state. | mandatory |  |  |
| 3.2.6.1.33.1.1 | It shall be valid to invoke the AIDC-ucf request primitive when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.33.1.1 | It shall be valid to invoke the AIDC-ucf request primitive when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.33.1.1 | It shall be valid to invoke the AIDC-ucf request primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.33.1.1 | It shall be valid to invoke the AIDC-ucf request primitive when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.33.1.1 | It shall be valid to invoke the AIDC-ucf request primitive when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.33.1.1 | It shall be valid to invoke the AIDC-ucf request primitive when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.33.1.1 | It shall be valid to invoke the AIDC-ucf request primitive when the AIDC-ASE protocol machine is in the BACKWARD-COORDINATED state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: create an AIDC-ucf-apdu based on the Result parameter, the Reason parameter if present, and the Reference ID parameter. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: invoke an AIDC-DATA request primitive with the AIDC-ucf-apdu as the User Data parameter. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the IDLE state: if the variable vr 1 = notify, then start the timer t 1NC and enter the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then if in the IDLE state: if the variable vr 1 = coord-start, then enter the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the IDLE state: if the variable v r1 = info-trans, then start the timer t 1NC. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the or the NOTIFY state: if the variable vr 1 = notify, then; start the timer t 1NC ; and enter the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if the variable vr 1 = coord-start, then enter the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if the variable vr 1 = end, then enter the IDLE state | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the NEGOTIATING state: if the variable vr 1 = coord-end, then: if variable vre = accept, then: start the timer t 1CT ; set the variable v re = NULL; and enter the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the NEGOTIATING state: if the variable vr 1 = coord-end, then: if variable vre = reject, then: start the timer t 1NC ; set the variable vre = NULL; and enter the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the NEGOTIATING state: if the variable vr 1 = coord-stndby, then start the timer t S . | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the NEGOTIATING state: if the variable vr 1 = end, then enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the COORDINATED state: if the variable vr1 = coord-start, then: enter RE-NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the COORDINATED state: if the variable vr 1 = trns-init, then: start the timer t 3R ; and enter PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the COORDINATED state: if the variable vr 1 = trns-start, then enter the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the COORDINATED state: if the variable vr 1 = end, then: stop all timers; and enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the RE-NEGOTIATING state: if the variable vr 1 = coord-end, then: start the timer t 1CT ; and enter the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the RE-NEGOTIATING state: if the variable vr 1 = coord-stnby, then start the timer ts. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the RE-NEGOTIATING state: if the variable vr 1 = end, then enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the PRE-TRANSFER state: if the variable vr 1 = trns-comm, then enter the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the PRE-TRANSFER state: if the variable vr 1 = trns-assm, then: start the timer t TE ; and enter the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the TRANSFERRING state: if the variable vr 1 = trns-assm, then: start the timer t TE ; and enter the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the TRANSFERRING state: if the variable vr 1 = trns-accept, then: start the timer t TE ; and enter the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the TRANSFERRING state: if the variable vr 1 = trns-reject, then: start the timer t 1CT ; and enter the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the TRANSFERRED state: if the variable vr 1 = back, then enter the BACKWARD COORDINATING state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the TRANSFERRED state: if the variable vr 1 = end, then enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "accepted" then: if in the BACKWARD-COORDINATING state: if the variable vr 1 = back-end, then enter the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the IDLE state, then: if vr 1 = notify start t 1NC . | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the IDLE state, then: if vr 1 = coord-start timer t 1NC . | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the IDLE state, then: if vr 1 = info-trans start timer t 1IN . | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the IDLE state, then: set the variable vr 1 = NULL, vr 2 = NULL and vre = NULL. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the IDLE state, then: remain in the current state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the NOTIFY state, then: start timer t 1NC . | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the NOTIFY state, then: set the variable vr 1 = NULL, vr 2 = NULL, and vre=NULL. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the NOTIFY state, then: remain in the same state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the COORDINATED state then: start t 1CT . | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the COORDINATED state then: set the variables vr 1 = NULL, vr 2 = NULL, and vre=NULL. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in the COORDINATED state then: remain in the current state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in any other state, then: if the variable vr 1!=NULL, then start the timer t 2R . | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in any other state, then: set the variable vr 1 = NULL, vr 2 = NULL, and vre=NULL. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if vr 2 = Reference ID, then: if the AIDC-ucf request primitive Result parameter has the abstract value "rejected", then: if in any other state, then: remain in the current state. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if the variable vr 2 != Reference ID then: invoke an AIDC-pvd-abrt indication. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if the variable vr 2 != Reference ID then: invoke an AIDC-ABORT request; | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if the variable vr 2 != Reference ID then: stop all timers. | mandatory |  |  |
| 3.2.6.1.33.2.1 | Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall: if the variable vr 2 != Reference ID then: enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.34.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-ucf-apdu when the AIDC-ASE protocol machine is in the IDLE state. | mandatory |  |  |
| 3.2.6.1.34.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-ucf-apdu when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.34.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-ucf-apdu when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.34.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-ucf-apdu when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.34.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-ucf-apdu when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.34.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-ucf-apdu when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.34.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-ucf-apdu when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: stop timer t c . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: extract the Result parameter, the Reason parameter if present, and the Reference ID parameter from the AIDC-ucf-apdu. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the IDLE state, or the NOTIFY state: if the variable vs 1 = notify, then: start the timer t 2NC ; and enter the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the IDLE state, or the NOTIFY state: if the variable vs 1 = coord-start, then: start the timer t 1R ; and enter the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the IDLE state, or the NOTIFY state: if the variable vs 1 = info-trans, then start the timer t 2IN . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the IDLE state, or the NOTIFY state: if the variable vs 1 = end, then enter IDLE state; | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the NEGOTIATING state: if the variable vs 1 = coord-negot, then start the timer t 1R . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the NEGOTIATING state: if the variable vs 1 = coord-end, then: if variable vse = accept, then: start the timer t 2CT ; and set the variable vse = NULL; and enter the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the NEGOTIATING state: if the variable vs 1 = coord-end, then: if variable vse = reject, then: start the timer t 2NC ; set the variable vse = NULL; and enter the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the NEGOTIATING state: if the variable vs 1 = end, then enter IDLE state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the COORDINATED state:if the variable vs 1 = coord-start, then: start the timer t 1R ; and enter the RE-NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the COORDINATED state: if the variable vs 1 = trns-init, then enter the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the COORDINATED state: if the variable vs 1 = trns-start, then enter the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the COORDINATED state: if the variable vs 1 = end, then: stop all timers; start timer t 2NC ; and enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the RE-NEGOTIATING state: if the variable vs1 = coord-negot, then start the timer t 1R . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the RE-NEGOTIATING state: if the variable vs1 = coord-end, then: start the timer t 2CT ; enter the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the RE-NEGOTIATING state: if the variable vs 1 = end, then enter IDLE state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the PRE-TRANSFER state: if the variable vs1 = trns-prpsl, then: start timer t 1R ; and remain in the current state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the PRE-TRANSFER state: if the variable vs 1 = trns-comm, then enter the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the PRE-TRANSFER state: if the variable vs 1 = trns-assm, then enter the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the TRANSFERRING state: if the variable vs 1 = trns-assm, then enter the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the TRANSFERRING state: if the variable vs 1 = trns-accept, then enter the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the TRANSFERRING state:if the variable vs 1 = trns-reject, then: start the timer t 2CT ; and enter the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the TRANSFERRED state: if the variable vs 1 = back, then enter the state BACKWARD COORDINATING | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the TRANSFERRED state: if the variable vs 1 = end, then enter IDLE state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value "accepted": if in the BACKWARD-COORDINATING state: if the variable vs 1 = back-end, then enter the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the IDLE state then: if vr 1 = notify start timer t 2NC . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the IDLE state then: if vr 1 = coord-start start timer t 2NC . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the IDLE state then: if vr 1 = info-trans start timer t 2IN . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the IDLE state then: set the variable vs 1 = NULL, vs 2 = NULL and vse=NULL. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the IDLE state then: remain in the current state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the NOTIFY state then: start timer t 2NC . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the NOTIFY state then: set the variable vs 1 = NULL; vs 2 = NULL, and vse=NULL. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the NOTIFY state then: remain in the current state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the COORDINATED state then: start timer t 2CT . | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the COORDINATED state then: set the variable vs 1 = NULL, vs 2 = NULL, and vse=NULL. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in the COORDINATED state then: remain in the current state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in any other state then: stop all timers. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in any other state then: set the variable vs 1 = NULL, vs 2 = NULL, and vse=NULL. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 = Reference ID, invoke an AIDC-ucf indication primitive with the extracted parameters in a) above as the AIDC-ucf indication primitive Result parameter, the AIDC-ucf indication primitive Reason parameter, and the AIDC-ucf indication primitive Reference ID parameter; if the Response parameter of the AIDC-ucf request primitive has the abstract value: rejected, then: if in any other state then: remain in the current state. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2Reference ID, then: invoke an AIDC-pvd-abrt indication. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 Reference ID, then: invoke an AIDC-ABORT request. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 Reference ID, then: stop all timers. | mandatory |  |  |
| 3.2.6.1.34.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-apdu the AIDC-ASE shall: if the variable vs 2 Reference ID, then: enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.35.1.1 | It shall be valid to invoke the AIDC-end request primitive when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.35.1.1 | It shall be valid to invoke the AIDC-end request primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.35.1.1 | It shall be valid to invoke the AIDC-end request primitive when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.35.2.1 | Upon the receipt of an AIDC-end request primitive the AIDC-ASE shall:create an AIDC-end-apdu based on the User Data parameter, if present, and the Msg Number parameter. | mandatory |  |  |
| 3.2.6.1.35.2.1 | Upon the receipt of an AIDC-end request primitive the AIDC-ASE shall: invoke a AIDC-DATA request primitive with the AIDC-end-apdu as the AIDC Data parameter. | mandatory |  |  |
| 3.2.6.1.35.2.1 | Upon the receipt of an AIDC-end request primitive the AIDC-ASE shall: if in the NOTIFY state, stop the timer t 2NC . | mandatory |  |  |
| 3.2.6.1.35.2.1 | Upon the receipt of an AIDC-end request primitive the AIDC-ASE shall: if in the COORDINATED state, then: if running, stop the timer t 1CT . | mandatory |  |  |
| 3.2.6.1.35.2.1 | Upon the receipt of an AIDC-end request primitive the AIDC-ASE shall: if in the COORDINATED state, then: if running, stop the timer t 2CT . | mandatory |  |  |
| 3.2.6.1.35.2.1 | Upon the receipt of an AIDC-end request primitive the AIDC-ASE shall: start the timer t C. | mandatory |  |  |
| 3.2.6.1.35.2.1 | Upon the receipt of an AIDC-end request primitive the AIDC-ASE shall: set the variable vs 1 = end . | mandatory |  |  |
| 3.2.6.1.35.2.1 | Upon the receipt of an AIDC-end request primitive the AIDC-ASE shall: set the variable vs 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.36.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-end-apdu when the AIDC-ASE protocol machine is in the IDLE state. | mandatory |  |  |
| 3.2.6.1.36.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-end-apdu when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.36.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-end-apdu when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.36.1.1 | It shall be valid to invoke the AIDC-DATA indication primitive with an AIDC-end-apdu when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: extract the User Data parameter, if present, and the Msg Number parameter from the AIDC-end-apdu. | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: invoke an AIDC-end indication primitive with the extracted User Data and Msg Number parameters as the AIDC-end indication User Data parameter, and the AIDC-end indication primitive Msg Number parameter respectively. | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: if in the IDLE state, stop the timer t 2R . | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: if in the NOTIFY state, stop the timer t 1NC . | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: if in the COORDINATED state: if running, stop the timer t 1CT. | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: if in the COORDINATED state: if running, stop the timer t 2CT. | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: if in the TRANSFERRED state, stop the timer t TE . | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: set the variable vr 1 = end. | mandatory |  |  |
| 3.2.6.1.36.2.1 | Upon the receipt of an AIDC-DATA indication primitive with an AIDC-end-apdu the AIDC-ASE shall: set the variable vr 2 = Msg Number. | mandatory |  |  |
| 3.2.6.1.37.1.1 | It shall be valid to invoke the AIDC-usr-abrt request primitive when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.37.1.1 | It shall be valid to invoke the AIDC-usr-abrt request primitive when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.37.1.1 | It shall be valid to invoke the AIDC-usr-abrt request primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.37.1.1 | It shall be valid to invoke the AIDC-usr-abrt request primitive when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.37.1.1 | It shall be valid to invoke the AIDC-usr-abrt request primitive when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.37.1.1 | It shall be valid to invoke the AIDC-usr-abrt request primitive when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.37.1.1 | It shall be valid to invoke the AIDC-usr-abrt request primitive when the AIDC-ASE protocol machine is in the BACKWARD-COORDINATING state. | mandatory |  |  |
| 3.2.6.1.37.2.1 | Upon the receipt of an AIDC-usr-abrt request primitive the AIDC-ASE shall: invoke an AIDC-ABORT request primitive. | mandatory |  |  |
| 3.2.6.1.37.2.1 | Upon the receipt of an AIDC-usr-abrt request primitive the AIDC-ASE shall: stop all timers. | mandatory |  |  |
| 3.2.6.1.37.2.1 | Upon the receipt of an AIDC-usr-abrt request primitive the AIDC-ASE shall: enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.38.1.1 | It shall be valid to invoke the AIDC-ABORT indication primitive when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.38.1.1 | It shall be valid to invoke the AIDC-ABORT indication primitive when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.38.1.1 | It shall be valid to invoke the AIDC-ABORT indication primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.38.1.1 | It shall be valid to invoke the AIDC-ABORT indication primitive when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.38.1.1 | It shall be valid to invoke the AIDC-ABORT indication primitive when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.38.1.1 | It shall be valid to invoke the AIDC-ABORT indication primitive when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.38.1.1 | It shall be valid to invoke the AIDC-ABORT indication primitive when the AIDC-ASE protocol machine is in the BACKWARD-COORDINATING state. | mandatory |  |  |
| 3.2.6.1.38.2.1 | Upon the receipt of a AIDC-ABORT indication primitive the AIDC-ASE shall: invoke an AIDC-usr-abrt indication primitive. | mandatory |  |  |
| 3.2.6.1.38.2.1 | Upon the receipt of a AIDC-ABORT indication primitive the AIDC-ASE shall: stop all timers. | mandatory |  |  |
| 3.2.6.1.38.2.1 | Upon the receipt of a AIDC-ABORT indication primitive the AIDC-ASE shall: enter the IDLE state. | mandatory |  |  |
| 3.2.6.1.39.1.1 | It shall be valid to invoke the AIDC-P-ABORT indication primitive when the AIDC-ASE protocol machine is in the NOTIFY state. | mandatory |  |  |
| 3.2.6.1.39.1.1 | It shall be valid to invoke the AIDC-P-ABORT indication primitive when the AIDC-ASE protocol machine is in the NEGOTIATING state. | mandatory |  |  |
| 3.2.6.1.39.1.1 | It shall be valid to invoke the AIDC-P-ABORT indication primitive when the AIDC-ASE protocol machine is in the COORDINATED state. | mandatory |  |  |
| 3.2.6.1.39.1.1 | It shall be valid to invoke the AIDC-P-ABORT indication primitive when the AIDC-ASE protocol machine is in the PRE-TRANSFER state. | mandatory |  |  |
| 3.2.6.1.39.1.1 | It shall be valid to invoke the AIDC-P-ABORT indication primitive when the AIDC-ASE protocol machine is in the TRANSFERRING state. | mandatory |  |  |
| 3.2.6.1.39.1.1 | It shall be valid to invoke the AIDC-P-ABORT indication primitive when the AIDC-ASE protocol machine is in the TRANSFERRED state. | mandatory |  |  |
| 3.2.6.1.39.1.1 | It shall be valid to invoke the AIDC-P-ABORT indication primitive when the AIDC-ASE protocol machine is in the BACKWARD-COORDINATING state. | mandatory |  |  |
| 3.2.6.1.39.2.1 | Upon the receipt of a AIDC-P-ABORT indication primitive the AIDC-ASE shall: invoke an AIDC-pvd-abrt indication primitive with the AbortReason parameter set to the abstract value: communications service failure. | mandatory |  |  |
| 3.2.6.1.39.2.1 | Upon the receipt of a AIDC-P-ABORT indication primitive the AIDC-ASE shall: stop all timers. | mandatory |  |  |
| 3.2.6.1.39.2.1 | Upon the receipt of a AIDC-P-ABORT indication primitive the AIDC-ASE shall: enter the IDLE state. | mandatory |  |  |
| 3.2.6.2.1.1 | If a timer expires the AIDC-ASE shall: interrupt any current activity. | mandatory |  |  |
| 3.2.6.2.1.1 | If a timer expires the AIDC-ASE shall: invoke AIDC-ABORT request primitive. | mandatory |  |  |
| 3.2.6.2.1.1 | If a timer expires the AIDC-ASE shall: invoke an AIDC-pvd-abrt indication primitive with the AIDC-pvd-abrt AbortReason parameter set to the abstract value: timer expired. | mandatory |  |  |
| 3.2.6.2.2.1 | If an AIDC-ASE has an irrecoverable system error, the AIDC-ASE shall: interrupt any current activity. | mandatory |  |  |
| 3.2.6.2.2.1 | If an AIDC-ASE has an irrecoverable system error, the AIDC-ASE shall: invoke AIDC-ABORT request primitive. | mandatory |  |  |
| 3.2.6.2.2.1 | If an AIDC-ASE has an irrecoverable system error, the AIDC-ASE shall: invoke an AIDC-pvd-abrt indication with the AIDC-pvd-abrt AbortReason parameter set to the abstract value: undefined-error. | mandatory |  |  |
| 3.2.6.2.3.1 | If a APDU received is determined to be invalid, the AIDC-ASE shall: invoke an AIDC-ABORT request primitive. | mandatory |  |  |
| 3.2.6.2.3.1 | If a APDU received is determined to be invalid, the AIDC-ASE shall: invoke an AIDC-pvd-abrt indication primitive with the AbortReason parameter set to the abstract value: invalid APDU. | mandatory |  |  |
| 3.2.6.2.3.1 | If a APDU received is determined to be invalid, the AIDC-ASE shall: enter the IDLE state. | mandatory |  |  |
| 3.2.6.3.1 | An AIDC-ASE shall measure the time between the initial event and the corresponding final event for each of the timers in Table 3.2.6-1. | mandatory |  |  |
| 3.2.6.3.1.1 | An implementation of the AIDC-ASE shall provide a means for configuring each of the timer values in Table 3.2.6.1. | mandatory |  |  |
| 3.2.6.3.2 | If the maximum time is exceeded before the final event has occurred, an AIDC-ASE shall take appropriate action as defined in 3.2.6.2.1. | mandatory |  |  |
| 3.2.6.3.2.1 | The actions defined in 3.2.6.2.1 should be taken when the maximum time as indicated in Table 3.2.6-1 has expired. | recommendation |  |  |

Table 3.2.6-1

| Purpose | Timer | Timer Value | Timer Start Event | Timer Stop Event (Pimitive invocation) |
| --- | --- | --- | --- | --- |
| User Confirmation for all AIDC services except Abort services | tc | 2 min | AIDC-DATA Req with AIDC-ASE-apdu | AIDC-DATA Ind with AIDC-ucf-apdu |
| Monitors time for transition between an initial invocation of the Info-transfer service primitive in the IDLE state and a subsequent Regime | t1lN | see 3.2.6.3.1.1 | +ve AIDC-ucf Ind in response to AIDC-inf-tfr Req | AIDC-nfy Ind AIDC-crd-start Ind |
| Monitors timer tor transition between an initial invocation of the Info-transfer serv1ce primitive in the IDLE state and a subsequent Reigme | t2IN | see 3.2.6.3.1.1 | +ve AlDC-ucf-lND in response to AlDC-inf-tfr-Req | AIDC-nfy Req AIDC-crd-start Req |
| Monitors time tor transition between Notifying Regime and Coordinating Regime. | t1NC | see 3.2.6.3.1.1 TBD | +ve AIDC-ucf Req in response to AIDC-nfy Ind | AIDC-nfy Ind AIDC-crd-start Ind AIDC-end Ind |
| Monitors time for transition between Notifying Regime and Coordinating Regime Complementry timer for t1NC | t2NC | see 3.2.6.3.1.1 | +ve AIDC-ucf Ind in response to AIDC-nfy Ind | AIDC-nfy Req AIDC-crd-start Req AIDC-end Req |
| Response monitoring timer used with the following services: AIDC-crd-start AIDC-crd-ngtt AIDC- tfr-prpsl (if P1 true) AIDC-tfr-comm AIDC-tfr-cntrl | t1R | see 3.2.6.3.1.1 | +ve AIDC-ucf Ind in response to: AIDC-crd-start Req AIDC-crd-ngtt Req AIDC- tfr-prpsl Req (if P1 true) AIDC-tfr-comm Req AIDC-tfr-cntrl Req | AIDC-crd-ngtt Ind AIDC-crd-stndby Ind AIDC-crd-end Ind AIDC-tfr-accept Ind AIDC-tfr-comm-assm Ind AIDC-tfr-cntrl Cnf |
| Response monitoring timer after -ve User Confirmation for all services except: AIDC-tfr-rqst AIDC-tfr-prpsl AIDC-tfr-accept AIDC-info-transfer AIDC-usr-abrt AIDC-pvd-abrt | t2R | see 3.2.6.3.1.1 | -ve AlDC-ucf Req | AIDC-ASE service Ind primitive except for the following services: AIDC-tfr-rqst AIDC-tfr-prpsl AIDC-tfr-accept AIDC-int-tfr AIDC-usr-abrt AIDC-pvd-abrt |
| Standby timer associated with AIDC-crd-stndby service | tS | see 3.2.6.3.1.1 | +ve AlDC-ucf Req in response to AIDC-crd-stndby Ind | AIDC-crd-ngtt Ind AIDC-crd-end Ind |
| Monitors time for transition between Coordinating Regime and Transferring Regime | t1CT | see 3.2.6.3.1.1 | +ve AlDC-ucf Req in response to AlDC-crd-end Ind | AIDC-tfr-init Ind or Req AIDC-tfr-cntrl Ind or Req AIDC-end Ind or Req AIDC-crd-start Ind or Req |
| Monitors time for transition between Coordinating Regime and Transferring Regime. Complementry timer for t1CT | t2CT | see 3.2.6.3.1.1 | +ve AlDC-ucf Ind in response to AlDC-crd-end Req | AIDC-tfr-init Req or Ind AIDC-tfr-cntrl Req or Ind AIDC-end Req or Ind AIDC-crd-start Ind or Req |
| Response monitoring timer used with the AIDC-tfr-init service | t3R | see 3.2.6.3.1.1 | +ve AlDC-ucf Req in response to AlDC-tfr-init Ind | AIDC-tfr-comm Ind AIDC-tfr-comm-assm-Req |
| End timer to monitor time after completion of Transferring Regime to the invocation of the AlDC-end service or commencement of backward coordination | tTE | see 3.2.6.3.1.1 | +ve AlDC-ucf Req in response to AIDC-tfr-comm-assm Ind or AIDC-tfr-cntrl Cnf | AIDC-crd-start Ind or Req AIDC-end Ind |

| 3.2.6.4.1 | The AIDC-ASE shall behave in accordance with the following state table, which show diagrammatically the state transitions and actions performed by the AIDC-ASE in response to incoming events. | mandatory |  |  |
| --- | --- | --- | --- | --- |
| 3.2.6.4.1 | Each cell in the state table shows optionally, one or more variables, denoted "vrN, or vsN", where N is an integer. The variables are defined as required and take on a value within the state table. |  |  |  |
| 3.2.6.4.1 | Each cell in the state table shows the new state that the AIDC-ASE shall enter after the action has been performed |  |  |  |
| 3.2.6.4.1 | Each cell in the state table shows the action, if any, which the AIDC-ASE shall perform. |  |  |  |

State Table

| State Event b |  | IDLE |  |  | NOTIFY |  |  | NEGOTIATING |  |  | COORDINATED |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIDC-nfy REQ |  | vs1 = notify vs2 = Msg Number AIDC-DATA REQ with AIDC-nfy-apdu start tC ==> IDLE |  |  | vs1 = notify vs2 = Msg Number AIDC-DATA REQ with AIDC-nfy-apdu stop t2NC ==> NOTIFY |  |  |  |  |  |  |  |
| rcv AIDC-nfy-apdu •• |  | vs1 = notify vs2 = Msg Number AIDC-nfy IND stop t2R ==> IDLE |  |  | vs1 = notify vs2 = Msg Number AIDC-nfy IND stop t2R ==> NOTIFY |  |  |  |  |  |  |  |
| AlDC-crd-start-apdu |  | vs1 = coord-start vs2 = Msg Number start tC AIDC-DATA REQ with AIDC-crd-start-apdu ==> IDLE |  |  | vs1 = coord-start vs2 = Msg Number stop t2NC start tC AIDC-DATA REQ with AIDC-crd-start-apdu ==> NOTIFY |  |  |  |  |  |  |  |
| rcv AlDC-crd-start-apdu |  | vs1 = coord-start vr2 = Msg Number AIDC-crd-start-IND stop t2R ==> IDLE |  |  | vs1 = coord-start vr2 = Msg Number AIDC-crd-start IND stop t2R stop t1NC ==> NOTIFY |  |  |  |  |  |  |  |
| AlDC-crd-end REQ |  |  |  |  | + vs1=coord-end v2=Msg Number start tC AlDC-crd-end IND with AIDC-crd-end-apdu => NEGOTIATING -vs1=negot vs2=Msg Number AIDC-DATA REQ with AlDC-crd- end-apdu => NEGOTIATING |  |  |  |  |  |  |  |
| rcv AlDC-crd-end-apdu |  |  |  |  | + vs1=coord-end v2=Msg Number AlDC-crd-end IND stop t1R stop t2R => NEGOTIATING -vs1=coord-end vs2=Msg Number AIDC-crd-end IND stop t1R stop t2R stop tS => NEGOTIATING |  |  |  |  |  |  |  |
| AlDC-crd-ngtt REQ |  |  |  |  | AIDC-DATA with AlDC-crd-stndby start tC =>NEGOTIATING |  |  |  |  |  |  |  |
| rcv AlDC-crd-ngtt-apdu |  |  |  |  | AIDC-crd-ngtt IND stop t1R stop t2R stop tS ==> NEGOTIATING |  |  |  |  |  |  |  |
| AlDC-crd-stndby REQ |  |  |  |  |  |  |  | AIDC-DATA with AlDC-crd-stndby-apdu start tC ==> NEGOTIATING |  |  |  |  |
| rcv AlDC-crd-stndby-apdu |  |  |  |  |  |  |  | AlDC-crd-stndby IND stop t1R stop t2R ==> NEGOTIATING |  |  |  |  |
| AIDC tfr init REQ |  |  |  |  |  |  |  |  |  |  | vs1= trns-init vs2 = Msg Number AIDC-DATA REQ with AlDC-tfr-init-apdu stop t2CT stop t2CT start tC ==>COORDINATED |  |
| rcv AlDC-tfr-init -apdu |  |  |  |  |  |  |  |  |  |  | vr1 = trns-init vr2= Msg Number AlDC-tfr-init IND stop t1CT stop t2CT ==> COORDINATED |  |
| AlDC-tfr-prpsl REQ |  |  |  |  |  |  |  |  |  |  | if P1, vr1 = trns-prpsl vr2= Msg Number AIDC-DATA REQ with AIDC-tfr-prpsl-apdu ==> COORDINATED |  |
| rcv AlDC-tfr-prpsl-apdu |  |  |  |  |  |  |  |  |  |  | if Pl, vr1 = trns-prpsl vr2= Msg Number ==> COORDINATED |  |
| AlDC-tfr-cntrl REQ |  |  |  |  |  |  |  |  |  |  | vs1 =trns-start vs2=Msg Number AIDC-DATA REQ with AlDC-tfr-cntrl-req-apdu stop t1CT stop t2CT start tC ==> COORDINATED |  |
| rcv AlDC-tfr-cntrl-req-apdu |  |  |  |  |  |  |  |  |  |  | vr1 = trns-pending vr2 =Msg Number AlDC-tfr-cntrl IND stop t1CT stop t2CT start t2R ==>COORDINATED |  |
| AlDC-info-trnsfr REQ |  | vs1 = trans vs2 = Msg Number AIDC-DATA REQ with AIDC-inf-tfr apdu start tC ==> IDLE |  |  | vs1 = trans vs2 = Msg Number AIDC-DATA REQ with AIDC-inf-tfr apdu start tC ==>NOTIFY |  |  | vs1 = trans vs2 = Msg Number AIDC-DATA REQ with AIDC-inf-tfr apdu start tC ==>NEGOTIATING |  |  | vs1 = trans vs2 = Msg Number AIDC-DATA REQ with AIDC-inf-tfr apdu start tC ==>COORDINATED |  |
| rcv AlDC-info-trnsfr apdu |  | vr1 = info-trans vr2 =Msg Number AlDC-inf-ttr IND ==>IDLE |  |  | vr1 = info-trans vr2 =Msg Number AlDC-inf-ttr IND ==> NOTIFY |  |  | vr1 = info-trans vr2 =Msg Number AlDC-inf-ttr IND ==>NEGOTIATING |  |  | vr1 = info-trans vr2 =Msg Number AlDC-inf-ttr IND ==>COORDINATED |  |
| AlDC-end REQ |  |  |  |  | vr1 = end vr2 = Msg Number AIDC-DATA REQ with AlDC-end apdu stop t2NC start tC ==> NOTIFY |  |  |  |  |  | vr1 = end vr2 = Msg Number AIDC-DATA REQ with AlDC-end apdu stop t1TC stop t2TC start tC ==>COORDINATED |  |
| rcv AlDC-end -apdu |  |  |  |  | vr1 = end vr2 = Msg Number AIDC-end IND stop t1NC ==> NOTIFY |  |  |  |  |  | vr1 = end vr2 = Msg Number AIDC-end IND stop t1NC stop t2NC ==>COORDINATED |  |
| AlDC-usr-abrt REQ |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |
| AIDC-ABORT ind |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |
| AIDC ucf REQ |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c4 ==> NOTIFY if c1 & c6 ==>NEGOTIATING if c1 & c32 { if c4 start t1NC if c6 start t1NC if c32 start t1NC ==>IDLE } vr1 = NULL vr2 = NULL} if !c2 { AlDC-pvd-abrt IND ATDC-ABORT REQ stop all timers ==>IDLE} |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c4 { start t1NC ==> NOTIFY if c1 & c6 ==>NEGOTIATING if c1 & c32 { ==> NOTIFY lf c1 & c30 { stop all timers ==>IDLE } if !c1 { if vr1 <> NULL { start t2R ==> NOTIFY if vr1 = NULL if vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE} |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if cl & c12 { start t1CT ==>COORDINATED } if c1 & c32 { ==>NEGOTIATING if !c1 { if vr1 <> NULL { start t2R ==>NEGOTIATING } if vr1 = NULL if vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE} |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if cl & c14 { start t3R ==>PRE-TRANSFER } if c1 & c16 ==>TRANSFERRING lf c1 & c30 { stop all timers ==>IDLE } lf c1 & c32 { ==>COORDINATED } if !c1 { if vr1 <> NULL { start t2R ==>COORDINATED } if vr1 = NULL if vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE} |  |
| rev ATDC-ucf REQ pdu |  | if c3 { AlDC-ucf IND if c1 &c5 ==> NOTIFY if cl & c7 { start t1R ==>NEGOTIATING if c1 & c33 { start t2IN ==>IDLE } vs1=NULL vs2=NULL } if !c3 { AlDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c3 { AlDC-ucf IND if c1 &c5 { start t2NC ==> NOTIFY } if cl & c7 { start t1R ==>NEGOTIATING if c1 & c3 1 { stop all timers ==>IDLE } if c1 &c33 { ==> NOTIFY vs1=NULL vs2=NULL } if !c3 { AlDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c3 { AlDC-ucf IND if c1 &c13 ==>COORDINATED if c1 & c9 { start t1R ==>NEGOTIATING } if !c1 { ==>NEGOTIATING vs1=NULL vs2=NULL } if !c3 { AlDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c3 { AlDC-ucf IND if c1 &c7 ==> NEGOTIATING if c1 & c15 { ==>PRE-TRANSFER if c1 & c17 { ==>TRANSFERRING if c1 & c31 { stop all timers ==>IDLE } if c1 &c33 { ==>COORDINATED } if !c1 { ==>COORDINATED } vs1=NULL vs2=NULL } if !c3 { AlDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |
| AlDC-crd-start REQ |  |  |  |  |  |  |  | vs2 = back vs2 = Msg Number AIDC-DATA REQ with AIDC-crd-start-apdu start tC stop tTE ==>TRANSFERRED |  |  |  |  |
| rcv AlDC-crd-start-apdu |  |  |  |  |  |  |  | vr1 = back vr2 = Msg Number AlDC-crd-start IND stop t2R stop tTE ==>TRANSFERRED |  |  |  |  |
| AlDC-crd-cnd REQ |  |  |  |  |  |  |  |  |  |  | vs1 = back-end vs2 = Msg Number AIDC-DATA REQ with AlDC-crd-end-apdu start tC ==>BACKWARD COORDINATING |  |
| rcv AIDC-crd-end-apdu |  |  |  |  |  |  |  |  |  |  | vr1 = back-end vr2 = Msg Number AlDC-crd-end IND stop t1R stop t2R stop tS |  |
| AlDC-crd-ngtt REQ |  |  |  |  |  |  |  |  |  |  | vs2 = Msg Number AIDC-DATA REQ with AlDC-crd-end-apdu start tC ==>BACKWARD COORDINATING |  |
| rcv AlDC-crd-ngtt-apdu |  |  |  |  |  |  |  |  |  |  | vs2 = Msg Number AlDC-crd-end IND stop t1R stop t2R stop tS ==>BACKWARD COORDINATING |  |
| AlDC-crd-stndby REQ |  |  |  |  |  |  |  |  |  |  | s2 = Msg Number AIDC-DATA REQ with AlDC-crd-end-apdu start tC ==>BACKWARD COORDINATING |  |
| rcv AlDC-crd-stnby-apdu |  |  |  |  |  |  |  |  |  |  | vs2 = Msg Number stop t1R stop t2R ==>BACKWARD COORDINATING |  |
| AlDC-tfr-rqst REQ |  | vs2 = Msg Number AIDC-DATA REQ with AIDC-tfr-rqst-apdu start tC ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| rcv AlDC-tfr-rqst-apdu |  | vs2 = Msg Number AIDC tfr-rqst IND start tC ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| AlDC-tfr-prpsl REQ |  | if P1, vs = trns-prpsl vs2 = Msg Number AIDC-DATA REQ with AlDC-tfr-prpsl-apdu start tC ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| rcv AlDC-tfrprpsl-apdu |  | f P1, vs = trns-prpsl vs2 = Msg Number AIDC-tfr-prpsl IND ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| AlDC-tfr-acccpt REQ |  | vs2 = Msg Number AIDC-DATA REQ with AlDC-tfr-accept-apdu start tC ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| rcv AlDC-tfr-accept-apdu |  | vr2 = Msg Number ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| AlDC-tfr-comm REQ |  | vr1 =trns-comm vr2 = Msg Number AIDC-DATA REQ with AlDC-tfr-comm-apdu start tC ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| rcv AIDC-tfr-comm-apdu |  | vr1 =trns-comm vr2 = Msg Number AIDC-tfr-comm IND stop t2R stop t3R ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| AIDC-tfr-comm-assum REQ |  | vr1 = tnsf-assum vr2 = Msg Number AIDC-DATA REQ with AlDC-tfr-comm-assume-apdu start tC stop t3R ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| rcv AlDC-tfr-comm-assum-apdu |  | vr1 = tnsf-assum vr2 = Msg Number AlDC-tfr-comm-assume IND stop t1R stop t2R ==>PRE-TRANSFER |  |  |  |  |  |  |  |  |  |  |
| AlDC-tfr-cntrl RSP |  |  |  |  | +vr1 = tnsf-accpt vr2 = Msg Number AIDC-DATA REQ with AlDC-tfr-cntrl-Rsp-apdu start tC ==>TRANSFERRING - vrl = tnsf-reject vs2 = Msg Number AIDC-DATA REQ with AlDC-tfr-cntrl-Rsp-apdu start tC ==>TRANSFERRING |  |  |  |  |  |  |  |
| rcv AlDC-tfr-cntl-rsp-apdu |  |  |  |  | +vs1 = tnsf-accpt vs2 = Msg Number AlDC-tfr-cntrl CNF stop t1R stop t2R ==>TRANSFERRING - vrl = tnsf-reject vs2 = Msg Number AlDC-tfr-cntrl CNF stop t1R stop t2R ==>TRANSFERRING |  |  |  |  |  |  |  |
| AlDC-info-trnsfr REQ |  | vs1 = info-trans vs2 = Msg Number AIDC-DATA REQ with AlDC-info-trnsfr-apdu start tC ==>PRE-TRANSFER |  |  | vs1 = info-trans vs2 = Msg Number AIDC-DATA REQ with AlDC-info-trnsfr-apdu start tC ==>TRANSFERRING |  |  | vs1 = info-trans vs2 = Msg Number AIDC-DATA REQ with AlDC-info-trnsfr-apdu start tC ==>TRANSFERRED |  |  | vs1 = info-trans vs2 = Msg Number AIDC-DATA REQ with AlDC-info-trnsfr-apdu start tC ==>BACKWARD-COORDINATING |  |
| rcv AlDC-info-trnsfr REQ pdu |  | vs1 = info-trans vs2 = Msg Number AlDC-info-trnsfr IND ==>PRE-TRANSFER |  |  | vs1 = info-trans vs2 = Msg Number AlDC-info-trnsfr IND ==>TRANSFERRING |  |  | vs1 = info-trans vs2 = Msg Number AlDC-info-trnsfr IND ==>TRANSFERRED |  |  | vs1 = info-trans vs2 = Msg Number AlDC-info-trnsfr IND ==>BACKWARD-COORDINATING |  |
| AIDC-end REQ |  |  |  |  |  |  |  | vs1 = end vs2 = Msg Number AIDC-DATA REQ with AIDC-end-apdu stop tTE ==>TRANSFERRED |  |  |  |  |
| rcv AlDC-end REQ pdu |  |  |  |  |  |  |  | vr1 = end vr2 = Msg Number AIDC-end IND stop tTE ==>TRANSFERRED |  |  |  |  |
| AlDC-usr-abrt-REQ |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |
| AIDC-ABORT ind |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |  | stop all timers ==>IDLE |  |
| AIDC-ucf REQ |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c22 ==>TRANSFERRING if c1 & c24 { start tTE ==>TRANSFERRED } if c1 & c32 { ==>PRE-TRANSFER } if !c1 { if vr1 <> NULL start t2R ==>PRE-TRANSFER } |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c18 { start tTE ==>TRANSFERRED } if c1 & c24 { start tTE ==>TRANSFERRED } if c1 & c20 { start t1CT ==>COORDINATED } if c1 & c32 { ==>TRANSFERRING } if !c1 { if vrl <> NULL start t2R ==>TRANSFERRING } vr1 = NULL vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c26 ==>BACKWARD COORDINATING if c1 & c30 { stop all timers ==>IDLE if c1 & c32 { ==>TRANSFERRED } vr1 = NULL vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c28 ==>TRANSFERRED if c1 & c32 { ==>BACKWARD COORDINATING if !c1 { if vrl <> NULL start t2R ==>TRANSFERRED } vr1 = NULL vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |
| rcv AIDC-ucf-REQ pdu |  | if c3 { AlDC-ucf IND if c1 & c23 ==>TRANSFERRING if c1 & c25 ==>TRANSFERRED if c1 & c33 { ==>PRE-TRANSFER } if !c1 ==>PRE-TRANSFER vs1 = NULL vs2 = NULL } if !c3 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c3 { AlDC-ucf IND if c1 & c19 { start tTE ==>TRANSFERRED } if c1 & c21 { start tTE ==>TRANSFERRED } if c1 & c25 ==>TRANSFERRED if c1 & c33 { ==>TRANSFERRING if !c1 ==>TRANSFERRING vs1 = NULL vs2 = NULL } if !c3 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c3 { AlDC-ucf IND if c1 & c27 ==>BACKWARD COORDINATING if c1 & c33 { ==>TRANSFERRING } if !c1 ==>TRANSFERRED vs1 = NULL vs2 = NULL } if !c3 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c3 { AlDC-ucf IND if c1 & c29 ==>TRANSFERRED } if c1 & c33 { ==>BACKWARD COORDINATING if !c1 ==>BACKWARD COORDINATING vs1 = NULL vs2 = NULL } if !c3 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |
| AIDC-ucf REQ | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c22 ==>TRANSFERRING if c1 & c24 { start tTE ==>TRANSFERRED } if c1 & c32 { ==>PRE-TRANSFER } if !c1 { if vr1 <> NULL start t2R ==>PRE-TRANSFER } |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c18 { start tTE ==>TRANSFERRED } if c1 & c24 { start tTE ==>TRANSFERRED } if c1 & c20 { start t1CT ==>COORDINATED } if c1 & c32 { ==>TRANSFERRING } if !c1 { if vrl <> NULL start t2R ==>TRANSFERRING } vr1 = NULL vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c26 ==>BACKWARD COORDINATING if c1 & c30 { stop all timers ==>IDLE if c1 & c32 { ==>TRANSFERRED } vr1 = NULL vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c2 { AIDC-DATA REQ with AlDC-ucf-apdu if c1 & c28 ==>TRANSFERRED if c1 & c32 { ==>BACKWARD COORDINATING if !c1 { if vrl <> NULL start t2R ==>TRANSFERRED } vr1 = NULL vr2 = NULL } if !c2 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  |
| rcv AIDC-ucf-REQ pdu |  |  | if c3 { AlDC-ucf IND if c1 & c23 ==>TRANSFERRING if c1 & c25 ==>TRANSFERRED if c1 & c33 { ==>PRE-TRANSFER } if !c1 ==>PRE-TRANSFER vs1 = NULL vs2 = NULL } if !c3 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c3 { AlDC-ucf IND if c1 & c19 { start tTE ==>TRANSFERRED } if c1 & c21 { start tTE ==>TRANSFERRED } if c1 & c25 ==>TRANSFERRED if c1 & c33 { ==>TRANSFERRING if !c1 ==>TRANSFERRING vs1 = NULL vs2 = NULL } if !c3 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |  | if c3 { AlDC-ucf IND if c1 & c27 ==>BACKWARD COORDINATING if c1 & c33 { ==>TRANSFERRING } if !c1 ==>TRANSFERRED vs1 = NULL vs2 = NULL } if !c3 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  | if c3 { AlDC-ucf IND if c1 & c29 ==>TRANSFERRED } if c1 & c33 { ==>BACKWARD COORDINATING if !c1 ==>BACKWARD COORDINATING vs1 = NULL vs2 = NULL } if !c3 { AIDC-pvd-abrt IND AIDC-ABORT REQ stop all timers ==>IDLE } |  |

| Paragraph | Requirement | Usage | Implementation | Test |
| --- | --- | --- | --- | --- |
| 3.2.7.1.1 | Each AIDC-APDU shall conform to the abstract syntax as defined below: | mandatory |  |  |

AIDC-APDU ::= CHOICE

{

| aidc-ucf-apdu | [0] | AIDC-ucf-apdu, | mandatory |  |
| --- | --- | --- | --- | --- |
| aidc-nfy-apdu | [1] | AIDC-nfy-apdu, | mandatory |  |
| aidc-crd-start-apdu | [2] | AIDC-crd-start-apdu, | mandatory |  |
| aidc-crd-end-apdu | [3] | AIDC-crd-end-apdu, | mandatory |  |
| aidc-crd-ngtt-apdu | [4] | AIDC-crd-ngtt-apdu, | mandatory |  |
| aidc-crd-stndby-apdu | [5] | AIDC-crd-stndby-apdu, | mandatory |  |
| aidc-tfr-init-apdu | [6] | AIDC-tfr-init-apdu, | mandatory |  |
| aidc-tfr-rqst-apdu | [7] | AIDC-tfr-rqst-apdu, | mandatory |  |
| aidc-tfr-prpsl-apdu | [8] | AIDC-tfr-prpsl-apdu, | mandatory |  |
| aidc-tfr-accept-apdu | [9] | AIDC-tfr-accept-apdu, | mandatory |  |
| aidc-tfr-cntrl-req-apdu | [10] | AIDC-tfr-cntrl-Req-apdu, | mandatory |  |
| aidc-tfr-cntrl-rsp-apdu | [11] | AIDC-tfr-cntrl-Rsp-apdu, | mandatory |  |
| aidc-tfr-comm-apdu | [12] | AIDC-tfr-comm-apdu, | mandatory |  |
| aidc-tfr-comm-assm-apdu | [13] | AIDC-tfr-comm-assm-apdu, | mandatory |  |
| aidc-inf-tfr-apdu | [14] | AIDC-inf-tfr-apdu, | mandatory |  |
| aidc-end-apdu | [15] | AIDC-end-apdu, | mandatory |  |

...

-AIDC-

ucf-apdu ::= SEQUENCE

{

| result | [0] | Result, | mandatory |  |
| --- | --- | --- | --- | --- |
| reason | [1] | ApplicationErrorData | OPTIONAL, |  |
| referenceid | [2] | MessageNumber | mandatory |  |

}

AIDC-nfy-apdu ::= SEQUENCE

{

| calledICAOFacilityDesignation | [0] | ICAOFacilityDesignation, | mandatory |  |
| --- | --- | --- | --- | --- |
| callingICAOFacilityDesignation | [1] | ICAOFacilityDesignation | OPTIONAL, |  |
| notify | [2] | Notify, | mandatory |  |
| msgnumber | [3] | MessageNumber | mandatory |  |

}

AIDC-crd-start-apdu ::= SEQUENCE

{

| calledICAOFacilityDesignation | [0] | ICAOFacilityDesignation, | mandatory |  |
| --- | --- | --- | --- | --- |
| callingICAOFacilityDesignation | [1] | ICAOFacilityDesignation | OPTIONAL, |  |
| startdata | [2] | Startdata, | mandatory |  |
| msgnumber | [3] | MessageNumber | mandatory |  |

}

Startdata ::= CHOICE

{

| coordinateinitial | [0] | CoordinateInitial, | mandatory |  |
| --- | --- | --- | --- | --- |
| coordinateupdate | [1] | CoordinateUpdate | mandatory |  |

}

AIDC-crd-end-apdu ::= SEQUENCE

{

| enddata | [0] | Enddata, | mandatory |  |
| --- | --- | --- | --- | --- |
| result | [1] | Result, | mandatory |  |
| msgnumber | [2] | MessageNumber | mandatory |  |

Enddata ::= CHOICE

{

| coordinateaccept | [0] | CoordinateAccept, | mandatory |  |
| --- | --- | --- | --- | --- |
| coordinatereject | [1] | CoordinateReject | mandatory |  |

}

AIDC-crd-ngtt-apdu ::= SEQUENCE

{

| coordinatenegotiate | [0] | CoordinateNegotiate, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

AIDC-crd-stndby-apdu ::= SEQUENCE

{

| coordinatestandby | [0] | CoordinateStandby, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

AIDC-tfr-init-apdu ::= SEQUENCE

{

| transferinitiate | [0] | TransferInitiate, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

AIDC-tfr-rqst-apdu ::= SEQUENCE

{

| transferrequest | [0] | TransferRequest, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

AIDC-tfr-prpsl-apdu ::= SEQUENCE

{

| transferconditionsproposal | [0] | TransferConditionsProposal | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

AIDC-tfr-accept-apdu ::= SEQUENCE

{

| transferconditionsaccept | [0] | TransferConditionsAccept, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

AIDC-tfr-cntrl-Req-apdu ::= SEQUENCE

{

| transfercontrol | [0] | TransferControl, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

AIDC-tfr-cntrl-Rsp-apdu ::= SEQUENCE

{

| transfercontroldata | [0] | TransferControlData, | mandatory |  |
| --- | --- | --- | --- | --- |
| result | [1] | Result, | mandatory |  |
| msgnumber | [2] | MessageNumber | mandatory |  |

}

TransferControlData ::= CHOICE

{

| transfercontrolassume | [0] | TransferControlAssume, | mandatory |  |
| --- | --- | --- | --- | --- |
| transfercontrolreject | [1] | TransferControlReject | mandatory |  |

}

AIDC-tfr-comm-apdu ::= SEQUENCE

{

| transfercomm | [0] | TransferComm, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

AIDC-tfr-comm-assm-apdu ::= SEQUENCE

{

| transfercommassume | [0] | TransferCommAssume, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

AIDC-inf-tfr-apdu ::= SEQUENCE

{

| infodata | [0] | InfoData, | mandatory |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

InfoData ::= CHOICE

{

| generalexecutivedata | [0] | GeneralExecutiveData, | mandatory |  |
| --- | --- | --- | --- | --- |
| generalpoint | [1] | GeneralPoint, | mandatory |  |
| surveillancegeneral | [2] | SurveillanceGeneral, | mandatory |  |
| generalfreetext | [3] | GeneralFreeText, | mandatory |  |
| emergencyfreetext | [4] | EmergencyFreeText, | mandatory |  |

...

}

AIDC-end-apdu ::= SEQUENCE

{

| cancel | [0] | Cancel | OPTIONAL, |  |
| --- | --- | --- | --- | --- |
| msgnumber | [1] | MessageNumber | mandatory |  |

}

MessageType ::= ENUMERATED

{

| aidc-ucf-apdu | (0), | mandatory |  |
| --- | --- | --- | --- |
| aidc-nfy-apdu | (1), | mandatory |  |
| aidc-crd-start-apdu | (2), | mandatory |  |
| aidc-crd-end-apdu | (3), | mandatory |  |
| aidc-crd-ngtt-apdu | (4), | mandatory |  |
| aidc-crd-stndby-apdu | (5), | mandatory |  |
| aidc-tfr-init-apdu | (6), | mandatory |  |
| aidc-tfr-rqst-apdu | (7), | mandatory |  |
| aidc-tfr-prpsl-apdu | (8), | mandatory |  |
| aidc-tfr-accept-apdu | (9), | mandatory |  |
| aidc-tfr-cntrl-req-apdu | (10), | mandatory |  |
| aidc-tfr-cntrl-rsp-apdu | (11), | mandatory |  |
| aidc-tfr-comm-apdu | (12), | mandatory |  |
| aidc-tfr-comm-assm-apdu | (13), | mandatory |  |
| aidc-inf-tfr-apdu | (14), | mandatory |  |
| aidc-end-apdu | (15), | mandatory |  |

...

Notify ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| flightRuleFlightType | [3] | FlightRuleFlightType | OPTIONAL, |  |
| beaconCode | [4] | BeaconCode | OPTIONAL, |  |
| aircraftNumberType | [5] | AircraftNumberType, | mandatory |  |
| cnsEquipment | [6] | CNSEquipment | OPTIONAL, |  |
| boundaryEstimate | [7] | BoundaryEstimate, | mandatory |  |
| route | [8] | Route | OPTIONAL, |  |
| otherInfo | [9] | OtherInformation | OPTIONAL, |  |
| timestamp | [10] | YMDHMS, | mandatory |  |

...

}

CoordinateInitial ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| flightRuleFlightType | [3] | FlightRuleFlightType | OPTIONAL, |  |
| beaconCode | [4] | BeaconCode | OPTIONAL, |  |
| aircraftNumberType | [5] | AircraftNumberType, | mandatory |  |
| cnsEquipment | [6] | CNSEquipment | OPTIONAL, |  |
| boundaryEstimate | [7] | BoundaryEstimate, | mandatory |  |
| route | [8] | Route | OPTIONAL, |  |
| otherInfo | [9] | OtherInformation | OPTIONAL, |  |
| timestamp | [10] | YMDHMS, | mandatory |  |

...

CoordinateUpdate ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| beaconCode | [3] | BeaconCode | OPTIONAL, |  |
| boundaryEstimate | [4] | BoundaryEstimate, | mandatory |  |
| route | [5] | Route | OPTIONAL, |  |
| timestamp | [6] | YMDHMS, | mandatory |  |

...

}

CoordinateNegotiate ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| boundaryEstimate | [3] | BoundaryEstimate, | mandatory |  |
| route | [4] | Route | OPTIONAL, |  |
| timestamp | [5] | YMDHMS, | mandatory |  |

...

}

CoordinateStandby ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| timestamp | [3] | YMDHMS, | mandatory |  |

...

}

CoordinateAccept ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| frequency | [3] | Frequency | OPTIONAL, |  |
| timestamp | [4] | YMDHMS, | mandatory |  |

...

CoordinateReject ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| timestamp | [3] | YMDHMS, | mandatory |  |

...

}

TransferInitiate ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| executiveData | [3] | ExecutiveData | OPTIONAL, |  |
| trackData | [4] | TrackData | OPTIONAL, |  |
| timestamp | [5] | YMDHMS, | mandatory |  |

...

}

TransferConditionsProposal ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| executiveData | [3] | ExecutiveData | OPTIONAL, |  |
| timestamp | [4] | YMDHMS, | mandatory |  |

...

}

TransferConditionsAccept ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| frequency | [3] | Frequency | OPTIONAL, |  |
| timestamp | [4] | YMDHMS, | mandatory |  |

...

TransferRequest ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| frequency | [3] | Frequency | OPTIONAL, |  |
| timestamp | [4] | YMDHMS, | mandatory |  |

...

}

TransferControl ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| executiveData | [3] | ExecutiveData | OPTIONAL, |  |
| timestamp | [4] | YMDHMS, | mandatory |  |

...

}

TransferControlAssume ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| timestamp | [3] | YMDHMS, | mandatory |  |

...

}

TransferControlReject ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| timestamp | [3] | YMDHMS, | mandatory |  |

...

TransferComm ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| executiveData | [3] | ExecutiveData | OPTIONAL, |  |
| releaseIndicator | [4] | ReleaseIndicator | OPTIONAL, |  |
| timestamp | [5] | YMDHMS, | mandatory |  |

...

}

TransferCommAssume ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| timestamp | [3] | YMDHMS, | mandatory |  |

...

}

SurveillanceGeneral ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime | OPTIONAL, |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| trackData | [3] | TrackData, | mandatory |  |
| timestamp | [4] | YMDHMS, | mandatory |  |

...

GeneralPoint ::= SEQUENCE

{

| functionalAddress | [0] | FunctionalAddress | OPTIONAL, |  |
| --- | --- | --- | --- | --- |
| flightID | [1] | FlightID, | mandatory |  |
| departure | [2] | DepartureAirportTime | OPTIONAL, |  |
| destination | [3] | DestinationAirport | OPTIONAL, |  |
| flightRuleFlightType | [4] | FlightRuleFlightType, | mandatory |  |
| beaconCode | [5] | BeaconCode | OPTIONAL, |  |
| aircraftNumberType | [6] | AircraftNumberType, | mandatory |  |
| cnsEquipment | [7] | CNSEquipment, | mandatory |  |
| boundaryEstimate | [8] | BoundaryEstimate | OPTIONAL, |  |
| route | [9] | Route | OPTIONAL, |  |
| otherInfo | [10] | OtherInformation | OPTIONAL, |  |
| timestamp | [11] | YMDHMS, | mandatory |  |

...

}

GeneralExecutiveData ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| frequency | [1] | Frequency, | mandatory |  |
| executivedata | [2] | ExecutiveData, | mandatory |  |
| timestamp | [4] | YMDHMS, | mandatory |  |

...

}

EmergencyFreeText ::= SEQUENCE

{

| functionalAddress | [0] | FunctionalAddress | OPTIONAL, |  |
| --- | --- | --- | --- | --- |
| flightID | [1] | FlightID, | mandatory |  |
| freeText | [2] | FreeText, | mandatory |  |
| timestamp | [3] | YMDHMS, | mandatory |  |

...

}

GeneralFreeText ::= SEQUENCE

{

| functionalAddress | [0] | FunctionalAddress | OPTIONAL, |  |
| --- | --- | --- | --- | --- |
| flightID | [1] | FlightID, | mandatory |  |
| freeText | [2] | FreeText, | mandatory |  |
| timestamp | [3] | YMDHMS, | mandatory |  |

...

Cancel ::= SEQUENCE

{

| flightID | [0] | FlightID, | mandatory |  |
| --- | --- | --- | --- | --- |
| departure | [1] | DepartureAirportTime, | mandatory |  |
| destination | [2] | DestinationAirport | OPTIONAL, |  |
| boundaryEstimate | [3] | BoundaryEstimate | OPTIONAL, |  |
| otherinfo | [4] | OtherInformation | OPTIONAL, |  |
| timestamp | [5] | YMDHMS, | mandatory |  |

...

ProviderAbortReason ::= ENUMERATED

{

| protocolerror | (0), | mandatory |  |
| --- | --- | --- | --- |
| timerexpired | (1), | mandatory |  |
| undefinederror | (2), | mandatory |  |
| providererror | (3), | mandatory |  |
| rejectedpermanent | (4), | mandatory |  |
| rejectedtransient | (5), | mandatory |  |

...

}

AircraftNumberType ::= SEQUENCE

{

| numberOfAircraft | [0] | NumberOfAircraft | OPTIONAL, |  |
| --- | --- | --- | --- | --- |
| aircraftType | [1] | AircraftType, | mandatory |  |
| wakeTurbulenceCategory | [2] | WakeTurbulenceCategory | OPTIONAL |  |

}

AircraftIdentification ::= IA5String (SIZE(2..7))

AircraftType ::= IA5String (SIZE(2..4))

AircraftAddress ::= BIT STRING (SIZE(24))

Airport ::= IA5String (SIZE(4))

DepartureAirportTime ::= SEQUENCE

{

| airport | [0] | Airport, | mandatory |  |
| --- | --- | --- | --- | --- |
| time | [1] | Time | OPTIONAL |  |

}

ATSRouteDesignator ::= IA5String (SIZE(2..7))

Level ::= CHOICE

{

| levelFeet | [0] | LevelFeet, | mandatory |  |
| --- | --- | --- | --- | --- |
| levelMetre | [1] | LevelMetre, | mandatory |  |
| levelFlightLevel | [2] | LevelFlightLevel, | mandatory |  |
| levelFlightLevelMetric | [3] | LevelFlightLevelMetric | mandatory |  |

}

LevelFeet ::= INTEGER (-60..7000)

-- unit = Feet, Range (-600..70000), resolution = 10

LevelMetre ::= INTEGER (-30..25000)

-- unit =metre, Range (-30..25000), resolution = 1

LevelFlightLevel ::= INTEGER (30..700)

-- unit = Level (100 feet), Range (30..700), resolution = 1

LevelFlightLevelMetric ::= INTEGER (100..2500)

-- unit = Level (10 metres), Range (100..2500), resolution = 1

ApplicationErrorData ::= SEQUENCE

{

| messageType | [0] | MessageType, | mandatory |  |
| --- | --- | --- | --- | --- |
| componentType | [1] | ComponentType, | mandatory |  |
| errorCode | [2] | ErrorCode, | mandatory |  |
| errorData | [3] | ErrorData | OPTIONAL |  |

}

ATWLevel ::= SEQUENCE

{

| atw | [0] | ATWLevelTolerance, | mandatory |  |
| --- | --- | --- | --- | --- |
| level | [1] | Level | mandatory |  |

ATWLevelTolerance ::= ENUMERATED

{

| at | (0), | mandatory |  |
| --- | --- | --- | --- |
| atorabove | (1), | mandatory |  |
| atorbelow | (2) | mandatory |  |

}

BeaconCode ::= SEQUENCE SIZE (4) OF BeaconCodeOctalDigit

BeaconCodeOctalDigit ::= INTEGER (0..7)

BoundaryEstimate ::= SEQUENCE

{

| boundaryFix | [0] | Position, | mandatory |  |
| --- | --- | --- | --- | --- |
| crossingTime | [1] | Time, | mandatory |  |
| crossingLevel | [2] | Level, | mandatory |  |
| atwLevel | [3] | ATWLevel | OPTIONAL |  |

}

CNSEquipment ::= SEQUENCE

{

| comNavEquipmentStatus | [0] | SEQUENCE SIZE (0..24) OF ComNavEquipmentStatus | OPTIONAL |  |
| --- | --- | --- | --- | --- |
| ssrEquipmentAvailable | [1] | SSREquipmentAvailable, | mandatory |  |
| adsAvailable | [2] | BOOLEAN, | mandatory |  |
| acasAvailable | [3] | BOOLEAN, | mandatory |  |
| dataLink | [4] | SEQUENCE SIZE (0..4) OFDataLink | mandatory |  |

}

ComNavEquipmentStatus ::= ENUMERATED

{

| aloranA | (0), | mandatory |  |
| --- | --- | --- | --- |
| cloranC | (1), | mandatory |  |
| ddme | (2), | mandatory |  |
| edecca | (3), | mandatory |  |
| fadf | (4), | mandatory |  |
| ggnss | (5), | mandatory |  |
| hhfRtf | (6), | mandatory |  |
| iinertialNavigation | (7), | mandatory |  |
| lils | (8), | mandatory |  |
| momega | (9), | mandatory |  |
| ovor | (10), | mandatory |  |
| pdoppler | (11), | mandatory |  |
| rrnavRouteEquipment | (12), | mandatory |  |
| ttacan | (13), | mandatory |  |
| uuhfRTF | (14), | mandatory |  |
| vvhfRTF | (15), | mandatory |  |

...

DataLink ::= ENUMERATED

{

| hf | (0), | mandatory |  |
| --- | --- | --- | --- |
| modeS | (1), | mandatory |  |
| satcom | (2), | mandatory |  |
| vhf | (3) | mandatory |  |

}

Date ::= SEQUENCE

{

| year | [0] | Year, | mandatory |  |
| --- | --- | --- | --- | --- |
| month | [1] | Month, | mandatory |  |
| day | [2] | Day | mandatory |  |

}

Day ::= INTEGER (1..31)

-- unit = Day, Range (1..31), resolution = 1

Degrees ::= CHOICE

{

| degreesMagnetic | [0] | DegreesMagnetic, | mandatory |  |
| --- | --- | --- | --- | --- |
| degreesTrue | [1] | DegreesTrue | mandatory |  |

}

DegreesMagnetic ::= INTEGER (1..360)

-- unit = degree, Range (1..360), resolution = 1

DegreeMinutes ::= INTEGER (0..5999)

-- unit = Minute, Range (0..59.99), resolution = 0.01

DegreeSeconds ::= INTEGER (0..59)

-- unit = Second, Range (0..59), resolution = 1

DegreesTrue ::= INTEGER (1..360)

-- unit = degree, Range (1..360), resolution = 1

DestinationAirport ::= Airport

DirectRouting ::= SEQUENCE

{

| fix2 | [0] | Position | OPTIONAL, |  |
| --- | --- | --- | --- | --- |
| fix1 | [1] | Position | mandatory |  |

}

Distance ::= CHOICE

{

| distanceNM | [0] | DistanceNM, | mandatory |  |
| --- | --- | --- | --- | --- |
| distancekm | [1] | Distancekm | mandatory |  |

}

Distancekm ::= INTEGER (0..2000)

-- unit = kilometre, Range (0..2000), resolution = 1

DistanceNM ::= INTEGER (0..1000)

-- unit = Nautical Mile, Range (0..1000), resolution = 1

ExecutiveData ::= SEQUENCE

{

| speed | [0] | Speed | OPTIONAL, |  |
| --- | --- | --- | --- | --- |
| level | [1] | Level | OPTIONAL, |  |
| heading | [2] | DegreesMagnetic | OPTIONAL, |  |
| vertRate | [3] | VerticalChange | OPTIONAL, |  |
| directRouting | [4] | DirectRouting | OPTIONAL |  |

}

FixName ::= IA5String (SIZE(1..5))

FlightID ::= SEQUENCE

{

| aircraftIdentification | [0] | AircraftIdentification, | mandatory |  |
| --- | --- | --- | --- | --- |
| selcal | [1] | Selcal | OPTIONAL, |  |
| registration | [2] | Registration | OPTIONAL, |  |
| airframeID | [3] | AircraftAddress | OPTIONAL |  |

FlightRule ::= ENUMERATED

{

| ifr | (0), | mandatory |  |
| --- | --- | --- | --- |
| vfr | (1), | mandatory |  |
| ifrfirst | (2), | mandatory |  |
| vfrfirst | (3) | mandatory |  |

}

FlightRuleFlightType ::= SEQUENCE

{

| flightRule | [0] | FlightRule, | mandatory |  |
| --- | --- | --- | --- | --- |
| flightType | [1] | FlightType | mandatory |  |

}

FlightType ::= ENUMERATED

{

| scheduledAirTransport | (0), | mandatory |  |
| --- | --- | --- | --- |
| nonScheduledAirTransport | (1), | mandatory |  |
| generalAviation | (2), | mandatory |  |
| military | (3), | mandatory |  |
| otherFlights | (4) | mandatory |  |

}

FreeText ::= IA5String (SIZE(1..256))

Frequency ::= CHOICE

{

| frequencyHF | [0] | FrequencyHF, | mandatory |  |
| --- | --- | --- | --- | --- |
| frequencyVHFChannel | [1] | FrequencyVHFChannel, | mandatory |  |
| frequencyUHF | [2] | FrequencyUHF, | mandatory |  |
| frequencySatChannel | [3] | FrequencySatChannel | mandatory |  |

}

FrequencyHF ::= INTEGER (2850..28000)

-- unit = Kilohertz, Range (2850..28000), resolution = 1

FrequencyVHFChannel ::= INTEGER (23600..27398)

-- unit: VHF Channel, Range: 118.000..136.990), resolution = 0.005

FrequencyUHF ::= INTEGER (9000..15999)

-- unit = Megahertz, Range (225.000..399.975), resolution = 0.025

FrequencySatChannel ::= NumericString (SIZE(12))

-- FrequencySatChannel corresponds to a 12-digit telephone number

FunctionalAddress ::= IA5String (SIZE(1..18))

ICAOFacilityDesignation ::= IA5String (SIZE(4..8))

Latitude ::= SEQUENCE

{

| latitudeDegrees | [0] | LatitudeDegrees, | mandatory |  |
| --- | --- | --- | --- | --- |
| latitudeMinutes | [1] | DegreeMinutes | OPTIONAL, |  |
| latitudeSeconds | [2] | DegreeSeconds | OPTIONAL, |  |
| latitudeDirection | [3] | LatitudeDirection | mandatory |  |

}

LatitudeDegrees ::= INTEGER (0..90000)

-- unit = Degree, Range (0..90), resolution = 0.001

LatitudeDirection ::= ENUMERATED

{

| north | (0), | mandatory |  |
| --- | --- | --- | --- |
| south | (1) | mandatory |  |

}

LatitudeLongitude ::= SEQUENCE

{

| latitude | [0] | Latitude, | mandatory |  |
| --- | --- | --- | --- | --- |
| longitude | [1] | Longitude | mandatory |  |

}

Longitude ::= SEQUENCE

{

| longitudeDegrees | [0] | LongitudeDegrees, | mandatory |  |
| --- | --- | --- | --- | --- |
| longitudeMinutes | [1] | DegreeMinutes | OPTIONAL, |  |
| longitudeSeconds | [2] | DegreeSeconds | OPTIONAL, |  |
| longitudeDirection | [3] | LongitudeDirection | mandatory |  |

}

LongitudeDegrees ::= INTEGER (0..180000)

-- unit = Degree, Range (0..180), resolution = 0.001

LongitudeDirection ::= ENUMERATED

{

| east | (0), | mandatory |  |
| --- | --- | --- | --- |
| west | (1) | mandatory |  |

MessageNumber ::= INTEGER (0..999999)

Month ::= INTEGER (1..12)

-- unit = Month, Range (1..12), resolution = 1

Navaid ::= IA5String (SIZE(1..4))

NumberOfAircraft ::= INTEGER (1..2)

OtherInformation ::= FreeText

PlaceBearing ::= SEQUENCE

{

| fixName | [0] | FixName, | mandatory |  |
| --- | --- | --- | --- | --- |
| latitudeLongitude | [1] | LatitudeLongitude | OPTIONAL, |  |
| degrees | [2] | Degrees | mandatory |  |

}

PlaceBearingDistance ::= SEQUENCE

{

| placeBearing | [0] | PlaceBearing, | mandatory |  |
| --- | --- | --- | --- | --- |
| distance | [1] | Distance | mandatory |  |

}

PlaceBearingPlaceBearing ::= SEQUENCE SIZE (2) OF PlaceBearing

Position ::= CHOICE

{

| fixName | [0] | FixName, | mandatory |  |
| --- | --- | --- | --- | --- |
| navaid | [1] | Navaid, | mandatory |  |
| airport | [2] | Airport, | mandatory |  |
| latitudeLongitude | [3] | LatitudeLongitude, | mandatory |  |
| placeBearingDistance | [4] | PlaceBearingDistance | mandatory |  |

}

PublishedIdentifier ::= SEQUENCE

{

| fixName | [0] | FixName, | mandatory |  |
| --- | --- | --- | --- | --- |
| latitudeLongitude | [1] | LatitudeLongitude | mandatory |  |

}

Registration ::= IA5String (SIZE(7))

ReleaseIndicator ::= ENUMERATED

{

| climb | (0), | mandatory |  |
| --- | --- | --- | --- |
| descent | (1), | mandatory |  |
| turns | (2), | mandatory |  |
| allActions | (3) | mandatory |  |

}

Result ::= ENUMERATED

{

| accepted | (0), | mandatory |  |
| --- | --- | --- | --- |
| rejected | (1), | mandatory |  |

...

}

Route ::= SEQUENCE

{

SEQUENCE SIZE (1..128) OF RouteInformation,

| position | [0] | Position, | mandatory |  |
| --- | --- | --- | --- | --- |
| time | [1] | Time, | mandatory |  |
| level | [2] | Level, | mandatory |  |
| speedGround | [3] | SpeedGround, | mandatory |  |
| trueTrackAngle | [4] | TrueTrackAngle | mandatory |  |

}

RouteInformation ::= CHOICE

{

| publishedIdentifier | [0] | PublishedIdentifier, | mandatory |  |
| --- | --- | --- | --- | --- |
| latitudeLongitude | [1] | LatitudeLongitude, | mandatory |  |
| placeBearingPlaceBearing | [2] | PlaceBearingPlaceBearing, | mandatory |  |
| placeBearingDistance | [3] | PlaceBearingDistance, | mandatory |  |
| aTSRouteDesignator | [4] | ATSRouteDesignator, | mandatory |  |
| trackDetail | [5] | TrackDetail | mandatory |  |

}

Selcal ::= IA5String (SIZE(4))

Speed ::= CHOICE

{

| speedGround | [0] | SpeedGround, | mandatory |  |
| --- | --- | --- | --- | --- |
| speedGroundMetric | [1] | SpeedGroundMetric, | mandatory |  |
| speedMach | [2] | SpeedMach, | mandatory |  |
| speedIndicated | [3] | SpeedIndicated, | mandatory |  |
| speedIndicatedMetric | [4] | SpeedIndicatedMetric, | mandatory |  |
| speedTrue | [5] | SpeedTrue, | mandatory |  |
| speedTrueMetric | [6] | SpeedTrueMetric | mandatory |  |

SpeedGround ::= INTEGER (-50..2000)

-- unit = Knots, Range (-50..2000), resolution = 1

SpeedGroundMetric ::= INTEGER (-100..4000)

-- unit = kilometre/hour, Range (-100..4000), resolution = 1

SpeedIndicated ::= INTEGER (0..400)

-- unit = Knots, Range (0..400), resolution = 1

SpeedIndicatedMetric ::= INTEGER (0..800)

-- unit =kilometre/hour, Range (0..800), resolution = 1

SpeedMach ::= INTEGER (500..4000)

-- unit = Mach, Range (0.5..4.0), resolution = 0.001

SpeedTrue ::= INTEGER (0..2000)

-- unit = Knots, Range (0..2000), resolution = 1

SpeedTrueMetric ::= INTEGER (0..4000)

-- unit =kilometre/hour, Range (0..4000), resolution = 1

SSREquipmentAvailable ::= ENUMERATED

{

| nnil | (0), | mandatory |  |
| --- | --- | --- | --- |
| atransponderModeA | (1), | mandatory |  |
| ctransponderModeAandC | (2), | mandatory |  |
| xatransponderModeS | (3), | mandatory |  |
| ptransponderModeSPA | (4), | mandatory |  |
| itransponderModeSID | (5), | mandatory |  |
| satransponderModeSPAID | (6), | mandatory |  |

...

}

-- PA: Pressure Level; ID: Aircraft Identification

Time ::= SEQUENCE

{

| hours | [0] | TimeHours, | mandatory |  |
| --- | --- | --- | --- | --- |
| minutes | [1] | TimeMinutes | mandatory |  |

}

Timehhmmss ::= SEQUENCE

hoursminute Time,

seconds TimeSeconds

}

TimeHours ::= INTEGER (0..23)

-- unit = Hour, Range (0..23), resolution = 1

TimeMinutes ::= INTEGER (0..59)

-- unit = Minute, Range (0..59), resolution = 1

TimeSeconds ::= INTEGER(0..59)

-- unit= Second, Range (0..59), resolution = 1

TrackData ::= SEQUENCE

{

| position | [0] | Position, | mandatory |  |
| --- | --- | --- | --- | --- |
| time | [1] | Time, | mandatory |  |
| level | [2] | Level, | mandatory |  |
| speedGround | [3] | SpeedGround, | mandatory |  |
| trueTrackAngle | [4] | TrueTrackAngle | mandatory |  |

}

TrackDetail ::= SEQUENCE

{

| trackName | [0] | TrackName, | mandatory |  |
| --- | --- | --- | --- | --- |
| latitudeLongitude | [1] | LatitudeLongitude | mandatory |  |

}

TrackName ::= IA5String (SIZE(1..6))

TrueTrackAngle ::= Degrees

VerticalChange ::= SEQUENCE

{

| direction | [0] | VerticalDirection, | mandatory |  |
| --- | --- | --- | --- | --- |
| rate | [1] | VerticalRate | mandatory |  |

}

VerticalDirection ::= ENUMERATED

{

| up | (0), | mandatory |  |
| --- | --- | --- | --- |
| down | (1) | mandatory |  |

VerticalRate ::= CHOICE

{

| verticalRateEnglish | [0] | VerticalRateEnglish, | mandatory |  |
| --- | --- | --- | --- | --- |
| verticalRateMetric | [1] | VerticalRateMetric | mandatory |  |

}

VerticalRateEnglish ::= INTEGER (0..3000)

-- unit = Feet/Minute, Range (0..30000), resolution = 10

VerticalRateMetric ::= INTEGER (0..1000)

-- unit =metre/Minute, Range (0..1000), resolution = 1

WakeTurbulenceCategory ::= ENUMERATED

{

| high | (0), | mandatory |  |
| --- | --- | --- | --- |
| medium | (1), | mandatory |  |
| low | (2) | mandatory |  |

}

Year ::= INTEGER (1996..2095)

-- unit = Year, Range (1996..2095), resolution = 1

YMDHMS ::= SEQUENCE

{

| date | [0] | Date, | mandatory |  |
| --- | --- | --- | --- | --- |
| timehhmmss | [1] | Timehhmmss | mandatory |  |

-ComponentType

::= ENUMERATED

{

| ctUnknown | (0), | mandatory |  |
| --- | --- | --- | --- |
| ctNotApplicable | (1), | mandatory |  |
| ctAircraftNumberType | (2), | mandatory |  |
| ctBeaconCode | (3), | mandatory |  |
| ctBoundaryEstimate | (4), | mandatory |  |
| ctCNSEquipment | (5), | mandatory |  |
| ctDepartureAirportTime | (6), | mandatory |  |
| ctDestinationAirport | (7), | mandatory |  |
| ctExecutiveData | (8), | mandatory |  |
| ctFlightID | (9), | mandatory |  |
| ctFlightRuleFlightType | (10), | mandatory |  |
| ctFreeText | (11), | mandatory |  |
| ctFrequency | (12), | mandatory |  |
| ctFunctionalAddress | (13), | mandatory |  |
| ctReleaseIndicator | (14), | mandatory |  |
| ctRoute | (15), | mandatory |  |
| ctTrackData | (16), | mandatory |  |
| ctUnrecognised | (255), | mandatory |  |

...

}

ErrorCode ::= ENUMERATED

{

| invalidNumberOfAircraft | (0), | mandatory |  |
| --- | --- | --- | --- |
| invalidAircraftType | (1), | mandatory |  |
| invalidWakeTurbulenceCategory | (2), | mandatory |  |
| invalidBeaconCodeOctalDigit | (3), | mandatory |  |
| invalidFixName | (4), | mandatory |  |
| invalidNavaid | (5), | mandatory |  |
| invalidAirport | (6), | mandatory |  |
| invalidLatitude | (7), | mandatory |  |
| invalidLongitude | (8), | mandatory |  |
| invalidTime | (9), | mandatory |  |
| invalidLevelFeet | (10), | mandatory |  |
| invalidLevelMeters | (11), | mandatory |  |
| invalidLevelFlightLevel | (12), | mandatory |  |
| invalidLevelFlightLevelMetric | (13), | mandatory |  |
| invalidATWLevelTolerance | (14), | mandatory |  |
| invalidComNavEquipmentStatus | (15), | mandatory |  |
| invalidSSREquipmentAvailable | (16), | mandatory |  |
| invalidDataLink | (17), | mandatory |  |
| invalidSpeedGround | (18), | mandatory |  |
| invalidSpeedGroundMetric | (19), | mandatory |  |
| invalidSpeedMach | (20), | mandatory |  |
| invalidSpeedIndicated | (21), | mandatory |  |
| invalidSpeedIndicatedMetric | (22), | mandatory |  |
| invalidSpeedTrue | (23), | mandatory |  |
| invalidSpeedTrueMetric | (24), | mandatory |  |
| invalidVerticalDirection | (25), | mandatory |  |
| invalidVerticalRateEnglish | (26), | mandatory |  |
| invalidVerticalRateMetric | (27), | mandatory |  |
| invalidAircraftIdentification | (28), | mandatory |  |
| invalidSelcal | (29), | mandatory |  |
| invalidRegistration | (30), | mandatory |  |
| invalidAirframeID | (31), | mandatory |  |
| invalidFlightRule | (32), | mandatory |  |
| invalidFlightType | (33), | mandatory |  |
| invalidFrequencyHF | (34), | mandatory |  |
| invalidFrequencyVHFChannel | (35), | mandatory |  |
| invalidFrequencyUHF | (36), | mandatory |  |
| invalidFrequencySatChannel | (37), | mandatory |  |
| invalidFunctionalAddress | (38), | mandatory |  |
| invalidReleaseIndicator | (39), | mandatory |  |
| invalidDistanceKm | (40), | mandatory |  |
| invalidDistanceNM | (41), | mandatory |  |
| invalidATSRouteDesignator | (42), | mandatory |  |
| invalidTrackName | (43), | mandatory |  |
| invalidmsgnumber | (250), | mandatory |  |
| invalidreferenceid | (251), | mandatory |  |
| invalidcallingICAOFacilityDesignation | (252), | mandatory |  |
| invalidcalledICAOFacilityDesignation | (253), | mandatory |  |
| invalidtimestamp | (254), | mandatory |  |
| unknown | (255), | mandatory |  |

...

}

ErrorData ::= BIT STRING (SIZE(1..256))

END

| Paragraph | Requirement | Usage | Implementation | Test |
| --- | --- | --- | --- | --- |
| 3.2.8.1.1 | The AIDC application shall use PER as defined in ISO/IEC 8825-2, using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in 3.2.7, or a functionally equivalent means which provides the same result. | mandatory |  |  |
| 3.2.8.2.1.2 | For the AIDC application no routing class is specified, thus the value corresponding to "ATSC: No Traffic Type Policy Preference" shall be conveyed. | mandatory |  |  |
| 3.2.8.2.2.1 | The AIDC application service priority shall have the value corresponding to "normal priority flight safety messages". | mandatory |  |  |
| 3.2.8.2.2.2 | Priority shall map to the session connection priority component of the A-ASSOCIATE request primitive Quality of Service parameter (see Table 3.2.5-1), using the values for Transport Layer Priority specified in Table 1.2 (see .3.8). | mandatory |  |  |
| 3.2.8.2.3.1 | The required AIDC application service RER shall have the value corresponding to "low". | mandatory |  |  |
| 3.2.8.2.3.2 | Thus the residual error rate component of the A-ASSOCIATE request primitive Quality of Service parameter (see Table 3.2.5-1) shall be set to zero. | mandatory |  |  |
| 3.2.9.1.1 | To achieve inter-operability for the implementation of AIDC between ATS units, or ATS regions on a global basis, the following requirements shall be satisfied: a common AIDC message set and associated services is agreed upon by the ATS units involved. | mandatory |  |  |
| 3.2.9.1.1 | To achieve inter-operability for the implementation of AIDC between ATS units, or ATS regions on a global basis, the following requirement shall be satisfied: agreement, by the ATS units involved, as to what flight related conditions dictate the invocation of the AIDC services. | mandatory |  |  |
| 3.2.9.1.1 | To achieve inter-operability for the implementation of AIDC between ATS units, or ATS regions on a global basis, the following requirement shall be satisfied: agreement as to the timing associated with the use of the AIDC services. | mandatory |  |  |
| 3.2.9.1.1 | To achieve inter-operability for the implementation of AIDC between ATS units, or ATS regions on a global basis, the following requirement shall be satisfied: agreement as to the predicate P1 as defined in 3.2.6.1.4. | mandatory |  |  |
| 3.2.9.2.1.1 | In cases where message queueing is implemented by the AIDC-User, an application specific priority scheme shall be implemented. | mandatory |  |  |
| 3.2.9.2.1.1.1 | Under this scheme the AIDC messages received shall be assigned one of the following priorities: Normal, Urgent or Distress. | mandatory |  |  |
| 3.2.9.2.1.1.2 | General freetext messages shall be assigned the priority of "Normal". | mandatory |  |  |
| 3.2.9.2.1.1.2 | Emergency freetext messages shall be assigned the priority of "Distress". | mandatory |  |  |
| 3.2.9.2.1.1.2 | Surveillance data transfer messages shall be assigned the priority of "Urgent". | mandatory |  |  |
| 3.2.9.2.1.1.2 | All other AIDC messages shall be assigned the priority "Normal". | mandatory |  |  |
| 3.2.9.2.1.1.3 | The AIDC-User shall process messages with a priority of Distress first, followed by messages with a priority of Urgent and then messages with a priority of Normal. | mandatory |  |  |
| 3.2.9.3.1 | The AIDC-User shall manage a number of timers associated with the sending of AIDC messages. These timers are: a Confirmation Timer, used to detect the failure of the AIDC-User to receive the user confirmation for a previously sent message. | mandatory |  |  |
| 3.2.9.3.1 | The AIDC-User shall manage a number of timers associated with the sending of AIDC messages. These timers are: a Response Timer, used to detect the failure of the AIDC-User to receive the appropriate message in response to a previously sent coordination or transfer message. | mandatory |  |  |
| 3.2.9.3.1 | The AIDC-User shall manage a number of timers associated with the sending of AIDC messages. These timers are: a Monitor Timer, used to detect the failure of the AIDC-User to receive another expected AIDC message. | mandatory |  |  |
| 3.2.9.3.1 | The AIDC-User shall manage a number of timers associated with the sending of AIDC messages. These timers are: a Standby Timer, used to extend the time before a response to a coordination message is expected. | mandatory |  |  |
| 3.2.9.4.1.1 | As each instantiation of the AIDC-AE is on a flight by flight basis, the AIDC-User shall manage these instantiations by some local means. | mandatory |  |  |
| 3.2.9.4.2.1 | Upon the receipt of an AIDC message, the AIDC-User shall validate the semantics of the message and use the User-confirmation service to indicate to the peer AIDC-User whether the message has been accepted or rejected. | mandatory |  |  |
| 3.2.9.4.2.2 | In order for the AIDC-AE to correlate user service primitives with the User-confirmation primitives, the AIDC-User shall generate and manage a set of unique identifiers. | mandatory |  |  |
| 3.2.9.4.2.3 | The identifiers received by an AIDC-User in indication service primitives, shall be used as the Referenced Number parameter of the User-confirmation request primitive. | mandatory |  |  |
| 3.2.9.4.3.1 | If for some reason the D-ATSU AIDC-User cannot accept the type of message received, it shall signal that condition with a negative User-confirmation | mandatory |  |  |
| 3.2.9.5.1 | In the event of information input by the user being incompatible with that able to be processed by the system, the user shall be notified. | mandatory |  |  |