|  | ATNP/WG3/WP 15-13 8 January 1999 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Honolulu, USA,   19 - 22 January 1999

Specification for Generic ATN Communication Service

Prepared by: WG3/SG3 (Upper Layer Architecture)

Presented by: Tony Kerr

SUMMARY

This paper provides an updated Specification and draft Guidance Material for a Generic ATN Communication Service (GACS).  This follows the acceptance of the general concept of Simple ATN Messaging by ATNP/WG3 in Rio in March 1998, comments received since the detailed specification was presented to WG3 in Utrecht in July 1998, and a detailed review of the protocol parameters following the WG3 meeting in Bordeaux in October 1998.  WG3/SG3 has considered how the provisions in the first part of this specification can be best incorporated into the ULCS Technical Provisions in a future Amendment to ICAO Doc. 9705.

The Working Group is invited to approve these proposals as candidate extensions to Sub-Volume 4 for ATNP/3.

# 1	Introduction

At the March 1998 ATNP WP meetings in Rio, the need for a simple generic end-to-end ATN communication service was recognised, and the Simple ATN Messaging (SAM) concept was broadly approved, with some reservations.  WG3/SG3 was actioned to analyse the arguments and report back to WG3 in Utrecht, in July 1998.

An initial draft specification of the Simple ATN Messaging (SAM) service and protocol was produced by Eurocontrol, and formed the basis of discussion at the WG3/SG3 meeting in April 1998 in Bracknell, UK.  SG3 concluded that GACS (as it is now called) is a worthwhile technical addition for the next package of ATN standards to be approved by ATNP/3, and that it in no way affects the stability of the existing applications.

Offline comments in Rio indicated that the name “SAM” might cause confusion, so the name was changed to Generic ATN Communication Service, or “GACS”.  There were a number of reasons for the re-naming: 'Simple' could wrongly be taken to imply that the other apps are over-complex, and 'Messaging' implies a clash with ATSMHS and to better position GACS as a communications service interface in the same sense as ULCS and ICS.

A number of detailed technical comments have been received on the draft GACS specification presented at Utrecht.  STNA of France is gratefully acknowledged as a source of detailed comments.  These comments were resolved at the WG3/SG3 meeting in Toulouse in September 1998, and the specification was updated accordingly and presented to the WG3 meeting In Bordeaux in October 1998.

As a result of comments made in Bordeaux, and subsequent thorough review, the technical provisions have been updated and the presentation of parameters in the protocol specification have been reformatted for clarity.

WG3/SG3 at its meeting in Toulouse in January 1999 considered how the provisions in the first part of this specification can be best incorporated into the ULCS Technical Provisions in a future Amendment to ICAO Doc. 9705.

Eurocontrol is currently in the process of validating the GACS technical provisions, as described in an accompanying Working Paper.

The attached document is an update of the GACS specification, which takes account of all comments received since the previous WG3 meeting.

# 2	References

ATNP/WG3/WP12-19 - The Case for a Simple ATN Messaging Service (SAM)

ATNP/WG3/WP14-10 - Specification for Generic ATN Communication Service (V0.E)

# 3	Recommendations

The Working Group is invited to consider the attached updated specification of the GACS service and protocol, previously approved for addition to the Sub-Volume 4 (ATN Upper Layers) technical provisions for approval by ATNP/3.