ATNP WG3/WP 15- 28

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Honolulu, USA

January 18 - 22, 1998

Update on AEEC Activities

Prepared by:  G. Saccone

Summary This paper summarizes discussions concerning ATN AEEC documents and activities.

# AEEC Activities on the ATN

## The Airlines Electronics Engineering Committee (AEEC) is currently developing the specifications for the implementation of the ATN.  There will be two documents that cover ATN implementations:  AEEC specifications 637A Data Link Internetworking Services and 638A Data Link Applications.  637A will cover the implementation of the internet SARPs (this is what Paul Hennig presented earlier in Bordeaux as “ATN-1”), while 638A will cover upper layers and applications.  These documents will not be a copy of the SARPs (and/or Guidance Material); rather they will explain the operation of the SARPs, referencing the SARPs directly where applicable, and include critical requirements that go beyond the SARPs.  For 638A, and specifically CM, the requirements that go beyond the SARPs include how the initial CM address is input, storage requirements of application information on an aircraft, aircrew interactions, and others.

Chapter 1 is an introduction

Chapter 2 is the “ATN Compatible System” section, which describes the ULA ( the ACF description in ARINC 622, Data Link Applications over ACARS)

Chapter 3 contains the ATN applicationsCu.  This chapter is broken into a number of sections, mostlyrom CM guidance.  One of the areas new sections relates to “Global CM Requirements”. rements such as the number of addresses whichupported.

Appendix BToology choices

Appendix COpions for initial CM address input

Appendix DCM air and ground ASE state diagrams

Appendix ECM time-sequence diagrams and example scenarios

Appendix FEnoding examples, uidance and dimensioning information

Appendix GDiferences between AFN and CM

Appendix HFuure considerations (backwards compatibility, version numbers, etc).

## Again, 638A  not directly copy the SARPs for requirements purposes.  The portions taken from the SARPs only help in understanding thech as the primitive and parameter tables from chapter 3 and the time sequence diagrams of chapter 5).

# Issues

## some issues concerning the purp 637A and 638A documents.  Although the draft presented at the AEEC drafting ber 1998 was a second iteration, tht of review comments at the meeting.  This was mainly due to the fact the AEEC was deal with many other pressing issues.owever, 637A and 638A were given high priority for completion.

## Probably the most significant issues are the roles of and relationships between documents produced by the ATN Panel, AEEC and RTCA.  The ATN Panel has, of course, produced the SARPs.  The AEEC produces documents relating to avionics implementation: Tse documents ed, to the level of pinouts for bacp  The RTCA is producing MOPs, which document minimal systemcapabilities.rdination between these efforts so that incompatibilities and inconsistencies are not introduced by different groups, and that the same work is not being done in different places.  Also, the groups will need help with the interpretation of the SARPs, again in order to avoid operational and technical inconsistencies.

## The document completion timeline is also a issue.  There may need to be placed on the AEEC and RTCA work in order to adhere to ATN introduction and trials schedules.  Not meeting these schedules cfound impact on the future of ATN.

## questioned whether or not theS and CPDLC sections would need to be included in 638A before implementations could proceed.  The cu stance on this subject is that the combination of the ULAong with the CPDLC nd ADS from the ATN SARPs, shon thenear term.  However, concerns were voiced that if CPDLC and ADS were not to be includede problems with introducing the ATN as were experienced with introducing FANS-1/A.

# Completion Times

## Currently, the planned completion for 638A is September 99. nly the ULA and CM will be covered.  (Athough there are placeholders for ADS, CPDLC, FIS and other applications, these will not be included in the first draft.)Completion is dependent on Industry agreeing the solutions for all of the implementation details that are critical to the successful fielding of CM.  Although there has not been a significant level of discussion within AEEC on these implementation details to date, some of the Galveston meeting participants agreed to bring position papers to the next meeting in March.

Conclusion

## The Working Group is invited to comment on the issues raised in this paper, and participate if possible in solving the issues raised.