- Attachment – Draft Specification of the CIDIN/AMHS Gateway (Version 0.1)

Editor´s note 1.- In the following, draft material is provided for both SARPs and Guidance Material. The inserted Notes (written in italics) serve two purposes:

- Notes for insertion into the SARPs; or

- Provision of text for the Guidance Material.

The final use of these Notes should be decided during a detailed technical review.

Editor´s note 2.- The inserted Editor´s notes (also written in italics) have only a temporary meaning and shall provide background information for the technical review concerning:

- Explanation of a taken solution or approach; or

- Information on special characteristics of the CIDIN; or

- Identification of still existing problems.

Editors note 3.- The structure of the material and used phrases are adopted as far as possible from the SARPs specifying the AFTN/AMHS Gateway (ICAO DOC 9705 – AN/956, Sub-volume III, Section 3.1) and associated Guidance Material (CAMAL Part III, Section 6).

Editor´s note 4.- For references to ICAO DOC 9705 – AN/956, Sub-volume III, Section 3.1 just paragraph numbers are used.

Editor´s note 5.- The below used paragraph numbers relate to an inclusion of the material in the current version of ICAO DOC 9705 – AN/956, Sub-volume III, Section 3.1 (Basic ATS Message Service). A re-numbering of the material may be necessary for the inclusion in the future SARPs for the Extended ATS Message Service.

3.1.2.4 CIDIN/AMHS Gateway Specification

Editors note 1.- Reference is made to the following CIDIN documentation:

[1]	Annex 10, Volume III, Amendment (71), Part I – Digital Data Communication

Systems – Section 8.6.5 CIDIN

[2]	EUR CIDIN Manual, First Edition, ICAO EUR DOC 005 (1996)

[3]	Report on the first AFSG meeting (AFSG/1), Appendix C, ICAO office Paris, (April 1998)

[4]	Report of the EUR AFS/96 Meeting, Appendix C, ICAO office Paris, (April 1996).

Editor´s note 2.- The basic specifications of the CIDIN are laid down in Annex 10, Vol. III,[1]. Significant functional enhancements as specified by working groups in the EUR region are published in the EUR CIDIN Manual [2]. The functional enhancements include, among others, a modified addressing concept: CIDIN addresses shall now identify the access points of applications entities in CIDIN centres or stations and not longer centres and stations in their entirety. In addition, the semantics of CIDIN addresses are specified.

Editor´s note 3.- [3] is a provisional update of the SARPs contained in the Appendix B of [2]. The update includes the alignment of paragraph numbering to Annex 10, Volume III. (The paragraph numbers of the Appendix B of [2] still refer to the former Annex 10, Volume I). [4] specifies the OPMET data transmission on CIDIN. This material is a prerequisite for specification of the OPMET related MTCU.

Editor´s note 4.- Considering this non-consolidated situation of the CIDIN documentation, in the draft CIDIN/AMHS Gateway specification for references the following approach has been taken: In the first instance reference is made to Annex 10, Vol. III.[1]. Where SARPs updates exist reference is made to the EUR CIDIN Manual [2], Appendix B, even though the provisional SARPs update [3]is meant. This has been done in the expectation that [3] will form the new Appendix B of [2] in short time. In addition, the discrepancies in the paragraph numbering between Annex 10, Vol. III [1], and the SARPs contained in the CIDIN Manual [2] are eliminated.

Editor´s note 5.- In the course of the drafting group activities the final alignment with the CIDIN base documentation should be done (under the assumption that the revised CIDIN Manual is then available).

Editor´s note 6.- The amendment of the ATSMHS SARPs with the CIDIN/AMHS Gateway needs supplementary additions and modifications in the general AMHS descriptions. These are seen as subject of separate draft material.

- 3.1.2.4.1	General

3.1.2.4.1.1	A CIDIN/AMHS Gateway shall provide for an interworking between the CIDIN and the ATN such that communication with ATS Message Servers and other CIDIN/AMHS Gateways is possible.

Note 1. – Figure 3.1.2.4.1-1 illustrates the position of the CIDIN/AMHS Gateway in a mixed CIDIN/AMHS communication environment.

Note 2. – For the AFTN application of the CIDIN (i.e. the CIDIN is used for transport of AFTN-formatted messages) connectivity with the ATN can be also achieved by means of the AFTN/AMHS Gateway with an appropriate adaptation of its AFTN component to the CIDIN transport service (implementation matter). However, the direct mapping between CIDIN messages and AMHS information objects as performed by the CIDIN/AMHS Gateway provides a better efficiency and does not assume the conservation of AFTN communication components. The AFTN/AMHS Gateway provides no alternative for non-AFTN applications of the CIDIN (e.g. CIDIN is used for transport of bit-oriented information).

Note 3.– The gateway-to-gateway communication is supported by an one-to-one mapping process between AMHS information objects and CIDIN messages. However, required connectivity between CIDIN areas separated by AMHS areas may be reached in a more efficient manner by performance of the CIDIN protocols across an existing X.25 network which is supporting in parallel the AMHS at the ATN subnetwork level. That means, the X.25 network resources are shared between the CIDIN and ATN/AMHS communications.

3.1.2.4.1.2	A CIDIN/AMHS gateway shall consist of the following logical components:

- CIDIN Component

- ATN Component

- Message Transfer and Control Units where each is dedicated to a particular CIDIN application; and

- Control Position.

Note 1.- The above division into logical components and as depicted in Figure 3.1.2.4.1-2 is a convenient way of specifying the gateway functions. There is no requirement for a CIDIN/AMHS Gateway to be implemented according to this structure.

Note 2.- For each CIDIN application a specific Message Transfer and Control Unit is specified. This modular approach allows tailoring with respect to those CIDIN applications which are actually in use. Furthermore, additional types of Message Transfer and Control Units may be specified on a regional basis.

Note 3.- In the following, provisions are given for CIDIN applications of the types AFTN, OPMET and CIDIN operator service message. CIDIN operator service messages are only relevant for gateway-to-gateway operation and/or integration of the gateway in the CIDIN network management. (The exchange of operator service messages within the CIDIN forms a nucleus of an envisaged network management functionality at the application layer.)

Note 4.- The provisions for the CIDIN/AMHS Gateway relate to a stand-alone facility where the CIDIN component is connected with one CIDIN centre which provides access to the remaining CIDIN. However, from an implementation viewpoint, it is likely that in many occasions an CIDIN/AMHS Gateway will be co-located with an CIDIN centre. Such a co-location may be also logical, which means that the CIDIN/AMHS Gateway and the CIDIN centre do not communicate through a CIDIN link, but rather using local interfaces. In such a case, the functions specified for the CIDIN component are not required. It is then sufficient that:

a)	the co-located Message Transfer and Control Units, ATN component and CIDIN centre together fulfil the required functions;

b)	the CIDIN centre provides the Message Transfer and Control Units  of the CIDIN/AMHS Gateway with an interface identical to that specified in the SARPs.

3.1.2.4.1.3	A CIDIN/AMHS Gateway shall be able to perform actions upon receipt of any category of AMHS information object by its ATN component.

3.1.2.4.1.4	A CIDIN/AMHS Gateway shall be able to perform actions upon receipt of any type of CIDIN message by its CIDIN component.

- 3.1.2.4.2	CIDIN/AMHS gateway components

          - CIDIN component

            - The CIDIN component shall handle the interface to the CIDIN and provide an

interface to the Message Transfer and Control Unit(s), implementing:

- all the applicable requirements of Annex 10, Volume III, 8.6.5,

and the functional enhancements provided with the EUR CIDIN Manual in a manner so as to be indistinguishable from an operational CIDIN station by the CIDIN centre to which the gateway is connected; and

- additional requirements which are necessary due to the CIDIN component

pertaining to a CIDIN/AMHS Gateway.

Note 1.- A CIDIN station performs only the CIDIN entry and exit functions, i.e. appears only as source and sink of CIDIN messages and does not support the CIDIN relay function (in contrast to a CIDIN centre).

Note 2.- Analogous to the AFTN/AMHS Gateway the CIDIN/AMHS Gateway is physically connected to only one CIDIN centre (which performs the CIDIN relay function for the gateway if required). That means, no routing table has to be maintained in the CIDIN packet layer of the gateway.

Note 3.- An additional requirement to be met by the CIDIN component is the transfer of CIDIN messages for a set of CIDIN exit addresses to an individual application entity or Message Transfer and Control Unit, respectively.

3.1.2.4.2.1.2	The CIDIN component shall incorporate a CIDIN procedure handler providing for all CIDIN functions prescribed for the interface to the CIDIN.

Note.- According to the clause 3.1.2.4.2.1.2 the CIDIN component provides the communication functions as specified in Annex 10, Volume III, 8.6.5, for the data link, X.25 packet, CIDIN packet and CIDIN transport protocol layer.

3.1.2.4.2.1.3	When received by the CIDIN component, CIDIN network management messages as generally identified in Annex 10, Volume III, Table 8-7, for the CIDIN transport layer (message type MT=1) shall be handled by the CIDIN component of the gateway in one of three mutually exclusive manners, depending on the category of the network management message:

- transfer to the appropriate Message Transfer and Control Unit to be processed as specified in 3.1.2.4.3.2 if the management message is an acknowledgement message (network management field (NMF) value = 3), as specified in Annex 10, Volume III, 8.6.6.4.2.5.1;

- transfer to the appropriate Message Transfer and Control Unit to be processed

as specified in 3.1.2.4.3.2 if the management message is an MCF error message (NMF value = 8), as specified in Annex 10, Volume III, 8.6.6.4.2.7;

- transfer to the appropriate Message Transfer and Control Unit to be processing as specified in 3.1.2.4.3.3 if the management message is an

enquiry response message (NMF value = 4), as specified in Annex 10, Volume III, 8.6.6.4.2.5.2.

Note.- The transfer of received enquiry response messages to the Message Transfer and Control Unit provides information on the re-establishment of the connectivity to exit addresses. This enables the Message Transfer and Control Unit to control the message acceptance/rejection at the interface to the ATN component in dependence on the current connectivity status in the CIDIN (cf. ATSMHS SARPs, 3.1.2.3.2.2.3).

            - The CIDIN component shall pass all CIDIN information messages as generally

identified in Annex 10, Volume III, Table 8-7, (message type MT=0) received from the CIDIN to the appropriate Message Transfer and Control Unit for processing according to 3.1.2.4.4.1 (AFTN application), 3.1.2.4.5.1 (OPMET application) or 3.1.2.4.6.1 (network management message), respectively.

Note.- In the current CIDIN SARPs, the network management messages to be handled in an application entity (MT=0) are limited to CIDIN operator service messages supporting co-ordination between (human) CIDIN operators at CIDIN centres or stations.

3.1.2.4.2.1.5	The processing by the CIDIN component shall ensure that all CIDIN information messages (MT=0) and network management messages (MT=1) received from the CIDIN and passed to a Message Transfer and Control Unit for further processing by the CIDIN/AMHS Gateway are constructed in accordance with the provisions of 3.1.2.4.2.6.

3.1.2.4.2.1.6	Operator notifications required in Annex 10, Vol. III or the EUR CIDIN Manual, respectively, shall be passed to the Control Position of the Gateway.

Note 1.- The occurrence of the following events shall be reported to the Control Position:
1) Loss of connectivity in the CIDIN packet layer (Annex 10, Volume III, 8.6.6.3.2.7);
2) Loss of connectivity in the CIDIN transport layer (EUR CIDIN Manual, 8.6.6.4.2.5.2.3); and
3) Re-establishment of connectivity in the CIDIN transport layer (EUR CIDIN Manual, 8.6.6.4.2.5.2.6).

Note 2.- In the CIDIN SARPs no long-term message retention requirements are placed on the

CIDIN protocol layers. In practice, long-term retention (traffic logging) is performed in the application entities. The CIDIN/AMHS Gateway follows this concept by placing the traffic logging on the Message Transfer and Control Units. The required information have to be provided by the CIDIN component (interface requirement). – Message retention for re-transmission if the network acknowledgement fails is an integral part of the CIDIN transport layer and needs here no further specification.

3.1.2.4.2.1.7	The CIDIN component shall pass information messages (MT=0) received from the CIDIN to that Message Transfer and Control Unit which supports the Message Code and Format (MCF) value assigned to the message.

Note 1.- The identification of an application entity (Message Transfer and Control Unit) by means of the MCF value is a special approach for the CIDIN/AMHS Gateway. According to the CIDIN specification, each application entity is identified by a pair of entry and exit addresses. However, adoption of this concept for the gateway would require in the CIDIN transport layer an administration of a (big) set of addresses for each Message Transfer and Control Unit (MTCU) as the entry and exit addresses also identify the AMHS recipient from the CIDIN point of view. In comparison to that, the MCF approach needs only a slight adaptation in the CIDIN transport layer of the gateway.

Note 2.- For AFTN formatted messages conveyed across the CIDIN the entry and exit addresses does only identify CIDIN entry and exit point and has no recipient relevance. Furthermore, recipients are identified by destination addresses (cf. 3.1.2.4.3.2.1). Thus, the AFTN-MTCU in a gateway is identified by a single pair of entry and exit addresses. This special case may be considered if an CIDIN/ATN Gateway implementation shall only include the AFTN-MTCU.

3.1.2.4.2.1.8	When the CIDIN component receives an information message with an MCF value which is not supported by any Message Transfer and Control Unit, it shall send an MCF error message to the entry centre or station in compliance with Annex 10, Volume III, 8.6.6.4.2.7.

3.1.2.4.2.1.9	Upon acceptance of an information message for a given exit address by the responsible Message Transfer and Control Unit, the CIDIN Component shall be sent back to the entry centre a network acknowledgement message for that accepted exit address in compliance with the provisions of Annex 10, Volume III, 8.6.6.4.2.5.1.3.

Note.- According to the provision 3.1.2.4.2.1.9 a CIDIN network acknowledgement message reports only the successful conveyance of the CIDIN information message between the CIDIN entry centre (station) and the gateway. There is no indication on successful message deliveries to the addressed recipients.

Editor´s note.- The conditions on that an MTCU shall accept a CIDIN information message from the CIDIN Component are specified in 3.1.2.4.3.2.8.

3.1.2.4.2.1.10	If, for any reason, the Message Transfer and Control Unit is unable to accept a CIDIN information message for one or more exit addresses, then the CIDIN component shall not return a network acknowledgement message to the entry centre or station for those exit addresses.

Note.- Such a condition may be caused in the Message Transfer and Control Unit by the inability to pass an AMHS information object to the ATN Component or the absence of address mapping information for all or a subset of CIDIN exit addresses.

3.1.2.4.2.1.11	If the Message Transfer and Control Unit re-assumes the acceptance of information message for an exit address, the CIDIN Component shall acknowledge the enquiry message for the given exit address in compliance with the provisions of Annex 10, Volume III, 8.6.6.4.2.5.2.5.

3.1.2.4.2.1.12	The CIDIN component shall ensure that all information messages constructed by the Message Transfer and Control Unit for transmission over the CIDIN are handled in accordance with the CIDIN procedures, in application of 3.1.2.4.2.1.3 above.

Note.- Figure 3.1.2.4.2-1 identifies the various categories of CIDIN messages which are accepted from or sent to the MTCUs, respectively.

          - ATN component

Editor´s note. – This chapter is mainly an adaptation of the SARPs 3.1.2.3.2.2 (ATN component of the AFTN/AMHS Gateway) to the CIDIN/AMHS Gateway. However, additional functionality is required for the determination of the appropriate MTCU.

3.1.2.4.2.2.1	The ATN Component shall allow the CIDIN/AMHS Gateway to function as an end system on the ATN.

3.1.2.4.2.2.2	The ATN component shall handle the interface to the AMHS, and provide an interface to the Message Transfer and Control Units as specified in 3.1.2.4.2.4, implementing a MTA complying with the profile specification included in 3.1.2.2.2.1 so as to be externally indistinguishable from the ATS Message Server by the ATS Message Server(s) or other CIDIN/AMHS Gateway(s) to which it is connected.

3.1.2.4.2.2.3	The ATN component shall pass messages, reports and probes received from the AMHS to that Message Transfer and Control Unit which supports the required CIDIN application.

Editor´s note. – The determination of the “supporting” MTCU will be accomplished on the basis of the recipient address(es). The MTA functionality which is forming part of the ATN Component includes the required routing capability. Mis-routed information objects (i.e. incompatible CIDIN application)will be detected in the MTCUs by means of the ATN Directory. Needs further refinement. (See Honolulu WG3/SG1 meeting report, section 4.2.4.)

3.1.2.4.2.2.4	If, for any reason, the required Message Transfer and Control Unit is unable to accept messages or probes passed by the ATN Component, then the ATN Component shall behave as follows:

- attempt to reroute the message or probe as specified in ISO/IEC 10021-4, 14.3.4.4;

- if no alternative route is available in the MTA-routing table or all such routes cannot be successfully used, reject the message for all message recipients, whose responsibility element in the per-recipient-indicators has the abstract-value “responsible” in the received message, with the non-delivery-reason-code and non-delivery-diagnostic-code elements of the non-delivery report taking the abstract-values specified in the base standards (ISO/IEC 10021-4, 14.3.4.4, item 1).

Note. – Such a condition may be caused by the inability of the required Message and Transfer Control Unit to pass CIDIN messages to the CIDIN Component.

3.1.2.4.2.2.5	If the AMHS Management Domain operating an CIDIN/AMHS Gateway desires to implement Message Handling System optional functional groups in addition to the specification of 3.1.2.4.2.2.2 above, this shall be performed in the ATN Component.

Note. – This applies in particular to the Redirection Functional Group. If implemented, redirection may be performed by the ATN Component, caused by a failure situation as envisaged in 3.1.2.4.2.2.4 above for example.

3.1.2.3.2.2.6	The ATN Component shall ensure that all information objects constructed by the Message Transfer and Control Units for transfer in the AMHS are handled in accordance with the procedures specified in the base standards for a relaying MTA implementing the profile specified in 3.1.2.2.2.1, in application of 3.1.2.4.2.2.2 above.

3.1.2.3.2.2.7	The ATN Component shall implement a traffic logging function identical to that of the MTA included in an ATS Message Server as specified in 3.1.2.2.2.2.

3.1.2.3.2.2.8	The ATN Component shall ensure that all AMHS information objects passed to the Message Transfer and Control Unit comply with the base standards.

          - Message Transfer and Control Units

Editor´s note. – This chapter is mainly an adaptation of the SARPs 3.1.2.3.2.3 (MTCU component of the AFTN/AMHS Gateway) to the CIDIN/AMHS Gateway. However, additional attention has to be given to the multiple MTCU aspect.

3.1.2.4.2.3.1	Each particular Message Transfer and Control Unit available in the CIDIN/AMHS Gateway shall provide a bi-directional conversion facility between the CIDIN component and the ATN component for one CIDIN application, consisting of:

- a set of general functions as specified in 3.1.2.4.3; and

- one of the CIDIN/AMHS conversion functions as respectively specified in 3.1.2.4.4 for the AFTN application, in 3.1.2.4.5 for the OPMET application and in 3.1.2.4.6 for CIDIN operator messages.

Note. – CIDIN operator messages (received from the CIDIN) may be destined for the CIDIN/AMHS Gateway itself. The MTCU has to pass such messages to the Control Position.

3.1.2.4.2.3.2	The Message and Transfer and Control Units shall pass all the AMHS information objects which it constructs in application of 3.1.2.4.2.4 to the ATN Component of the gateway, for further conveyance in the AMHS.

3.1.2.4.2.3.3	For the generation of AMHS messages and reports, and for the processing of received AMHS messages, probes and reports, the Message Transfer and Control Units shall have the capability to interpret the semantics and to perform actions related to the ISO/IEC 10021 Elements of Service which are part of the basic requirements of the MT service as specified in ISO/IEC ISP 12062-3.

3.1.2.4.2.3.4	The Message Transfer and Control Units in a CIDIN/AMHS Gateway shall pass all the CIDIN messages which they construct in application of 3.1.2.4.4, 3.1.2.4.5 or 3.1.2.4.6, respectively, to the CIDIN Component of the CIDIN/AMHS Gateway, for further conveyance in the CIDIN.

3.1.2.4.2.3.5	The Message Transfer and Control Units shall ensure that all the AMHS information objects which they construct comply with section 7 (for IPMs) and section 8 (for RNs) of ISO/IEC 10021-7, complemented with the additional requirements included in Table 3.1.2-1 if the AFTN application is supported, and with the section 12.2.1.1 of ISI/IEC 10021-4 (for messages) and section 12.2.1.3 of ISO/IEC 10021-4 (for reports).

3.1.2.4.2.3.6	The Message Transfer and Control Units shall ensure that the CIDIN user data fields they construct comply with the EUR CIDIN Manual 3.2.6 (AFTN messages), x.y.z (t.b.d.) (OPMET messages) or 6.5.1 (CIDIN operator messages), respectively.

          - Interface between the ATN component and the Message Transfer and Control Units

Editor´s note. – The interface is identical with that of the AFTN/AMHS Gateway. Therefore, the text from chapter 3.1.2.3.2.4 is only adapted to the multiple MTCU aspect.

3.1.2.4.2.4.1	The ATN Component shall exchange information objects with the Message Transfer and Control Units via its MTA transfer-port as specified in ISO/IEC 10021-4, section 12.2.

3.1.2.4.2.4.2	The ATN Component shall invoke the Message-transfer, Report-transfer and Probe-transfer abstract operations, respectively, to pass AMHS messages, reports and probes to the Message Transfer and Control Units.

3.1.2.3.2.4.3	The Message Transfer and Control Units shall invoke the Message-transfer and Report-transfer abstract operations, respectively, to pass AMHS messages and reports to the ATN Component.

          - Interface between the CIDIN component and the Message Transfer and Control Units

Editor´s note 1. - The CIDIN specifications address the interactions with users only in a rudimentary manner: Message submission by an application entity in the entry centre is specified by the service parameters accepted/expected for the CIDIN transport header generation (EUR CIDIN Manual, Appendix B, 8.6.6.4.2.2.) and the application entity shall be notified of unsuccessful transmissions (EUR CIDIN Manual, Appendix B, 8.6.6.4.2.5.2.3.). No other details are specified for the CIDIN user interface. For the CIDIN/AMHS Gateway the interface between the MTCUs and the CIDIN Component is specified by means of service primitives using the existing description in the EUR CIDIN Manual as a basis.

Editor´s note 2. – The following interface specification follows the proposal made in WP/185 (Rio meeting, March 1998). However, the implementation is not mandatory for the gateway specification (see Note 1 to 3.1.2.4.1.2).

3.1.2.4.2.5.1	For the interface between the CIDIN Component and the and Message Transfer Control Units the following provisions shall apply:

- information objects shall be transferred according to the table of CIDIN priorities specified in EUR CIDIN Manual, Appendix B, 8.6.6.3.1.3.1, and

- use of service primitives as specified in 3.1.2.4.2.5.2 to .4, respectively.

3.1.2.4.2.5.2	The Message Transfer and Control Units shall request the transport of user data as CIDIN message with the service primitive T-Message.Request with using the service parameters listed in 3.1.2.4.2.5.5.

#### Editor´s note.- The provision of the MTCUs with local operation results upon a message request is seen as an implementation matter.

3.1.2.4.2.5.3	The CIDIN component shall deliver CIDIN user data to the Message Transfer and Control Units with the service primitive T-Message.Indication with using the service parameters listed in 3.1.2.4.2.5.5.

3.1.2.4.2.5.4	The CIDIN Component shall provide the Message Transfer and Control Unit with acknowledgement information on a previously requested transport of user information with the service primitive T-Message.Confirmation with using the service parameters listed in 3.1.2.4.2.5.5.

3.1.2.4.2.5.5	In application of the service primitives the parameters listed in Table 3.1.2.4.2.5-1 shall be used.

##### Table 3.1.2.4.2.5-1  T-MESSAGE Parameters

| Parameter name | Req | Ind | Cnf |
| --- | --- | --- | --- |
| Exit address (Ax) | M | M(=)3) | M(=)4) |
| Destination address (Ad) | C | C(=)4) |  |
| Entry address (Ae) | 1) |  |  |
| Message priority (MP) | M | M(=) |  |
| Message code/format (MCF) | M | M(=) |  |
| Network acknowledgement (NA) | C |  |  |
| User data | M | M(=) |  |
| ACK |  |  | C |
| MCF_error |  |  | C |
| Message Identification Number (MIN) | 2) | M(=) | M(=) |

Note 1. - The entry address Ae may be known to the CIDIN component from a previous binding process or a local configuration data.

Note 2. – The MTCUs may be get knowledge of the allocated MIN by provision of a local result upon a message request.

Note 3. – The exit address forms an element of the list of exit addresses given in the message request.

Note 4. – The parameter provides the set of destination addresses which was allocated to the provided exit address.

Note 5. – For the specification of the service parameters refer to EUR CIDIN Manual (1996)

Editor´s notes. – The reference will be refined when the CIDIN documentation is stabilised. In the Guidance material additional information to the meaning of the individual service parameters may be given.

3.1.2.4.2.6	CIDIN/AMHS Gateway Control Position

Editor´s note. – The following provisions has been adopted with small changes from the SARPs 3.1.2.3.2.6. The material may be completed when the specification of the MTCUs will be concluded.

3.1.2.3.2.6.1	The CIDIN/AMHS Gateway Control Position shall be used as place where errors and other events which occurred in the CIDIN/AMHS Gateway are reported for appropriate actions.

Note 1. – The reports relate to events/errors detected/occurred in the CIDIN Component as defined in the Annex 10, Volume III and EUR CIDIN Manual, Appendix B (cf. 3.1.2.4.2.1.6) and to errors occurred in the MTCUs.

Note 2. – From a practical point of view also the CIDIN operator messages destined to the gateway may be presented to the operator via the Control Position.

3.1.2.3.2.6.2	The appropriate action to be undertaken on reporting of an error to an CIDIN/AMHS Gateway control position shall be either:

- a matter of policy which is local to the AMHS Management Domain operating the CIDIN/AMHS Gateway; or

- subject to multilateral agreements.

Editor´s note. – The Annex 10 material as well as the EUR CIDIN Manual provides no advice on appropriate operator actions upon reported errors.

        - General functions

Editor´s note.- This section is related to general provisions and principles which applies to all types of MTCUs.

          - Traffic logging

Editor´s note. – This chapter is mainly an adaptation of the SARPs 3.1.2.3.3.1 (Traffic logging) to the CIDIN/AMHS Gateway. However, additional logging information is required as no logging requirements are placed on the CIDIN component (cf. 3.1.2.4.2.1.6, Note 2). In addition, distinctions have to be made between AFTN and non-AFTN applications.

3.1.2.4.3.1.1	The Message Transfer and Control Units shall perform long-term logging, as specified in 3.1.2.4.3.1.2 to 3.1.2.4.3.1.5, for a period of at least thirty days, of information related to the following exchanges of information objects with the ATN Component and with the CIDIN Component:

- AMHS message transfer out (to the ATN Component);

- AMHS report transfer out (to the ATN Component);

- AMHS message transfer in (from the ATN Component);

- AMHS report transfer in (from the ATN Component);

- CIDIN information message conveyance out (to the CIDIN Component); and

- CIDIN information message conveyance in (from the CIDIN Component).

3.1.2.4.3.1.2	For the long-term logging of information related to an AMHS Message Transfer In and CIDIN Information Message Conveyance Out flow, the following parameters, relating to the messages, shall be logged by the Message Transfer and Control Unit:

- input message-identifier;

- IPM-identifier, if any;

- common-fields and either receipt-fields or non-receipt-fields of IPN (Inter-Personal Notification), if any;

- action taken thereon (NDR, conversion, NA, MCF error etc tbd;

- event date/time;

- message type indicator (MT) and message code and format (MCF) component, if any;

- message identification number (MIN), as returned by the CIDIN Component;

- entry address, if any;

- exit address(es) with associated destination address(es), if any;

- origin line of AFTN message or AFTN service message, if any; and

- text part of the AFTN message.

Note.- The destination address(es) in parameter i) and the parameters j) and k) applies to the AFTN application only (MCF = 2).

Editor´s note 1.- The logging of the text part (parameter j) corresponds to the long-term retention requirement placed on the AFTN Component (see 3.1.2.3.2.1.10). This logging enables the gateway to respond to AFTN service messages requesting requesting repetition by the originator. The functionality corresponds to that specified for the AFTN/AMHS Gateway.

Editor´s note 2.- User data of other categories of CIDIN information messages as AFTN will be not logged for two reasons: 1) The volume of data is much higher than for AFTN messages and 2) there is no service message for requesting repetition by the originator.

3.1.2.4.3.1.3	For the long term logging of information related to a CIDIN Information Message Conveyance In and AMHS Message Transfer Out flow, the following parameters, relating to the messages, shall be logged by the Message Transfer and Control Units:

- message type indicator (MT) and message code and format (MCF) component as provided by the CIDIN Component;

- message identification number (MIN), as provided by the CIDIN Component;

- entry address, as provided by the CIDIN Component;

- exit address(es) with associated destination address(es), as provided by the CIDIN Component, if any;

- Origin line of AFTN message (or AFTN acknowledgement message), if any;

- action taken thereon (convert as IPM, convert as RN, NA, MCF error, AFTN service message indicating an unknown addressee indicator generation, reject with rejection cause);

- event date/time

- MTS-identifier, if any; and

- IPM-identifier, if any.

Note.- The destination address(es) in parameter d) and the parameter e) applies to the AFTN application only (MCF = 2).

3.1.2.4.3.1.4	For the long-term logging of information related to an AMHS Message Report In and/or CIDIN Information Message containing an AFTN service message indicating an unknown addressee indicator conveyance Out, the following parameters, relating to the report and/or CIDIN message, shall be logged by the Message Transfer and Control Units:

- report-identifier (if report in);

- subject-identifier (if report in);

- action taken thereon if report in (discard, convert into AFTN service message, forwarding to Control Position)

- event date/time

- message type indicator (MT) and message code and format (MCF) component (if CIDIN message out);

- message identification number (MIN) (if CIDIN message out);

- entry address (if CIDIN message out);

- exit address with associated destination address (if CIDIN message out);

- Origin line of converted AFTN service message (if CIDIN message out); and

- Origin line of subject AFTN message which was encapsulated in the received CIDIN information message (if CIDIN message out and no report in).

Editor´s note.- The invocation of a CIDIN network management message of the type “MCF error” will be logged as “action taken” in connection with the subject message (cf. 3.1.2.4.3.1.3). - There is no CIDIN information object available for mapping of a received AMHS message report. The only reasonable action is forwarding to the Control Position.

3.1.2.4.3.1.5	For the long-term logging of information related to an CIDIN Information Message containing an AFTN service message indicating an unknown addressee indicator Conveyance In and/or to an AMHS Message Report Out, the following parameters, relating to the CIDIN message and /or report, shall be logged by the Message Transfer and Control Units:

- message type indicator (MT) and message code and format (MCF) component (if CIDIN message in);

- message identification number (MIN), (if CIDIN message in);

- entry address, (if CIDIN message in);

- exit address with associated destination address (if CIDIN message in);

- Origin line of AFTN service message (if CIDIN message in);

- Origin line of subject AFTN message (if CIDIN message in);

- action taken thereon if CIDIN message in (convert into NDR, tbd.);

- event date/time

- report-identifier (if report out); and

- subject-identifier (if report out).

          - Handling of CIDIN service confirmations in the Message Transfer and Control Units

Editor´s note 1. – According to 3.1.2.4.2.1.3 a) and b) CIDIN network acknowledgement messages and MCF error messages are passed to the responsible MTCU for further processing. The respective provisions are stated below (3.1.2.4.3.2.1 to .7). In addition, the error situation caused by a missing CIDIN network acknowledgement message (lost connectivity) is handled.

Editor´s note 2.- According to 3.1.2.4.2.1.9/10 the CIDIN Component shall only return a CIDIN network acknowledgement message (for an exit address) to the entry centre upon acceptance of the received CIDIN information message for an exit address by the responsible MTCU. The respective provision below (3.1.2.4.3.2.8) specifies the conditions on that an MTCU shall accept an CIDIN information message from the CIDIN Component for a given exit address. There is a need to distinguish between AFTN and non-AFTN applications.

Editor´s note 3.- A CIDIN network acknowledgement message is only delivered if the CIDIN user (application) requests a confirmed transport service (EUR CIDIN Manual, Appendix B, 8.6.6.4.2.5).

Editor´s note 4.- The CIDIN applications of the types AFTN, OPMET and CIDIN operator messages make use of the confirmed transport service (EUR CIDIN Manual 3.1.6, 3.2.5.3, Report of the EUR AFS/96 Meeting, Appendix C). Thus, for the current specification of the CIDIN/AMHS gateway only the use of the confirmed transport service has to be taken into account.

Editor´s note 5.- MCF messages are generated for both the confirmed and unconfirmed CIDIN transport service (Annex 10, Volume III, 8.6.6.4.2.7)

3.1.2.4.3.2.1	The reception of a CIDIN network acknowledgement message passed from the CIDIN component shall resulting in an entry in the long-term log record of the related CIDIN information message (3.1.2.4.3.1.2) which indicates successful CIDIN transmission for the given exit address.

Note.- The reception of a CIDIN network acknowledgement message does not effect the generation of an AMHS delivery report. A required delivery report has been already generated by the responsible MTCU upon successful passing of the CIDIN information message to the CIDIN component (para reference for the generation of a required DR in the MTCUs tbd).

Editor´s note.- The view reflected in the above Note is adopted from the AMHS/AFTN Gateway behaviour according to 3.1.2.3.5.1.7, i.e. de-coupling of the AMHS report generation from the message forwarding in the target messaging system (AFTN).

3.1.2.4.3.2.2	If the confirmation returned by the CIDIN component indicates loss of connectivity for a given exit address the responsible Message Transfer and Control Unit shall perform the following actions:

- logging of the error situation (3.1.2.4.3.1.2) and reporting to the control position; and either

- passing a copy of the concerned CIDIN information message to the CIDIN component with use of a secondary exit address, if any, according to the EUR CIDIN Manual 3.3.3.4; or

- termination of the transmission to the non-reachable exit address for the concerned CIDIN information message and reporting unsuccessful message delivery to the AMHS as follows, depending on the abstract-values of the originator-report-request and of the originating-MTA-report-request elements in the per-recipient-indicators in the corresponding per-recipient-fields of the subject AMHS message:

- generation of a non-delivery report as specified in 3.1.2.4.3.2.3, for the concerned set of recipients which meet the following condition, if any:

i)	the abstract value of the originator-report-request differs from “report”; and

- the abstract-value of the originating_MTA-report-request differs from “report” and from “audited report”; or

- generation of an IPM as specified in 3.1.2.4.3.2.4, for all other concerned recipients.

Note.- The sub-clause c) aims at avoiding the generation of a non-delivery-report after the generation of a delivery report by the MTCU for the same subject AMHS message (cf. Note to 3.1.2.4.3.2.1).

3.1.2.4.3.2.3	A non-delivery-report related to the non-reachable MF-Addresses which have not caused the generation of an IPM as the result of 3.1.2.4.3.2 c) shall be generated in compliance with:

- the specification of 3.1.2.4.3.4 using the elements of the subject AMHS message; and

- the following specification of abstract-values:

- “unable-to-transfer” for the non-delivery-reason-code; and

2)	“tbd” for the non-delivery-diagnostic-code.

3.1.2.4.3.2.4	An IPM related to the non-reachable MF-Addresses which have not caused the generation of a non-delivery report as result of 3.1.2.4.3.2 c) shall be generated in compliance with:

- the specification of 3.1.2.4.3.5 using the elements of the subject AMHS message; and

- “unable-to-transfer due to lost of CIDIN connectivity” for the IPM body.

3.1.2.4.3.2.5	Upon the reception of a MCF error message passed from the CIDIN component, the Message Transfer and Control Unit shall perform the following actions:

- logging of the error situation (3.1.2.4.3.1.2) and reporting to the control position; and

- termination of transmission to the referred exit address for the concerned CIDIN information message and reporting incompatible application/address combination to the AMHS as follows, depending on the abstract-values of the originator-report-request and of the originating-MTA-report-request elements in the per-recipient-indicators in the corresponding per-recipient-fields of the subject AMHS message:

- generation of a non-delivery report as specified in 3.1.2.4.3.2.6, for the set of concerned recipients which meet the following condition, if any:

i)	the abstract value of the originator-report-request differs from “report”; and

- the abstract-value of the originating MTA-report-request differs from “report” and from “audited report”; or

- generation of an IPM as specified in 3.1.2.4.3.2.7, for all other recipients.

Note.- The sub-clause b) aims at avoiding the generation of a non-delivery-report after the generation of a delivery report by the MTCU for the same subject AMHS message.

3.1.2.4.3.2.6	A non-delivery-report related to the set of MF-Addresses which have not caused the generation of an IPM as the result of 3.1.2.4.3.5 b) shall be generated in compliance with:

- the specification of 3.1.2.4.3.4 using the elements of the subject AMHS message;

and

b)	the following specification of abstract-values:

- “unable-to-transfer” for the non-delivery-reason-code; and

2)	“tbd” for the non-delivery-diagnostic-code.

3.1.2.4.3.2.7	An IPM related to the incompatible MF-Addresses which have not caused the generation of a non-delivery report as result of 3.1.2.4.3.5 b) shall be generated in compliance with:

- the specification of 3.1.2.4.3.5 using the elements of the subject AMHS message;

and

- “unable-to-transfer due to incompatible CIDIN application/address combination” for the IPM body.

3.1.2.4.3.2.8	Upon reception by a Message Transfer and Control Unit of an CIDIN information message passed by the CIDIN Component, the Message Transfer and Control Unit shall indicate “message acceptance” to the CIDIN Component in one of the following manner, depending on the type of the supported CIDIN application:

a)	for the AFTN application “message acceptance” is indicated immediately after message reception for all associated exit addresses; or

b)	for non-AFTN applications “message acceptance” is indicated for each exit address after their successful conversion in an AMHS recipient address.

Editor´s note 1.- For the AFTN application the destination addresses (and not the exit addresses) have to be converted in AMHS recipient addresses (cf. 3.1.2.4.3.6). In case of an unsuccessful address conversion the AFTN origin station will be notified by an AFTN service message of the type “unknown address indicator”. This AFTN service message is leaving the gateway encapsulated in a CIDIN information message. The provisions for generation of CIDIN information messages with encapsulated AFTN service messages are given in 3.1.2.4.4.3.

Editor´s note 2.- The kind of notification of the AFTN origin station on an unsuccessful conversion in AMHS recipient addresses is in accordance with the procedure applied to the AFTN/AMHS Gateway (3.1.2.3.4.2.1.4.2).

Note.- The Figures 3.1.2.4.3.2-1 and -2 provides an overview on the handling of CIDIN network acknowledgement and MCF error messages by the CIDIN/AMHS Gateway.

          - Monitoring of the CIDIN connectivity in the Message Transfer and Control Units

Editor´s note1.- If there is a CIDIN user request for a confirmed transport service for a given information message and if no network acknowledgement message has been received within the defined time period TMR (response time-out), the entry centre (station) will retransmit the respective message. If a network acknowledgement message is still not obtained, the entry centre (station) shall send an enquiry message  to that exit address. On receipt of an enquiry message, the exit centre (station) shall acknowledge the enquiry message by transmitting an enquiry response message back to the entry centre (station). (EUR CIDIN Manual, Appendix B, 8.6.6.4.2.5.2)

Editor´s note 2.- The MTCUs shall monitor the actual CIDIN connectivity to exit addresses to avoid passing of information messages to the CIDIN component which can not be delivered.

Editor´s note 3.- An MTCU can recognise on lost connectivity to an exit address by the provision of a negative confirmation from the CIDIN component (3.1.2.4.3.2.2). In addition, the MTCUs are notified by the CIDIN component on the re-establishment of connectivity to an exit address by providing received enquiry response messages (3.1.2.4.2.1.3 c).

3.1.2.4.3.3.1	Each Message Transfer and Control Unit shall maintain the actual CIDIN connectivity for their known exit address space.

3.1.2.4.3.3.2	When a Message Transfer and Control Unit is notified on the missing CIDIN network acknowledgement message from an exit address used in an information message passed to the CIDIN component, the Message Transfer and Control Unit shall

- set the exit address in the status “non-reachable”; and

- terminate the transfer of information messages to the CIDIN component for that exit address in compliance with 3.1.2.4.3.2.2 c).

3.1.2.4.3.3.3	When a Message Transfer and Control Unit receives an AMHS message for recipients which can only reached via a “non-reachable” exit address, the Message Transfer and Control Unit shall perform the following actions:

- rejection of the message for the concerned message recipients; and

- generation of a non-delivery-report related to those recipients in compliance with:

- the specification of 3.1.2.4.3.4 using the elements of the subject AMHS message; and

- the following specification of abstract-values:

- “unable-to-transfer” for the non-delivery-reason-code; and

- “tbd” for the non-delivery-diagnostic-code.

Note.- A message rejection according the above clause assumes that no (further) secondary exit address is available.

3.1.2.4.3.3.4	Upon reception by the Message Transfer and Control Unit of a CIDIN ENQ message passed by the CIDIN component, the Message Transfer and Control Unit shall:

- re-set the reported exit address in the status “reachable”; and

- re-assume transmission of CIDIN information messages to that exit address.

Editor´s note.- An analysis of the conditions for initiation of an ENQ response is required. See suppression of CIDIN NA in Fig. 3.1.2.4.3.2-2 what causes incoming ENQ messages from that entry centre.

          - Generation of AMHS reports

Editor´s note.- This section provides the common provisions for the generation of AMHS reports caused by various events. As far as possible reference will be made to material provided with the corresponding section 3.1.2.3.5.6 of the AFTN/AMHS Gateway specification. – To be completed.

          - Generation of AMHS IPMs for provision of service information

Editor´s note.- This section provides the common provisions for delivery of service information to AMHS users by means of IPMs where the use of AMHS reports is not applicable. This approach is adopted from the AFTN/AMHS Gateway. – To be completed.

          - Address conversion

            - General provisions

Note 1.- CIDIN entry and exit addresses (Ae, Ax) identify individual access points to the CIDIN where messages are submitted for conveyance by the CIDIN (entry points) or to which messages are sent across the CIDIN (exit points). For a given message flow that CIDIN centre at which a message is submitted is called entry centre, message delivery is performed in the exit centre. (EUR CIDIN Manual: 3.2.5.2 and Appendix B 8.6.6.3.1.3.2, 8.6.6.4.1.4.5)

Note 2.- In addition, the CIDIN supports so-called destination addresses (Ads) which identify recipients in a network which is using the CIDIN only a intermediate transport facility. A CIDIN exit point may be used for a set of destination addresses. In practice, a given CIDIN message may bear a number of exit addresses (multi-address capability of the CIDIN) where in turn each exit address is associated with a set of destination addresses. For the transport of AFTN messages across the CIDIN the concept of destination addresses is applied to the AFTN destination addresses. (EUR CIDIN Manual: 3.2.6 and Appendix B 8.6.6.3.1.3.3-5)

Note 3.- The determination of the exit address(es) for a set of destination addresses contained in a given application message (AFTN message) is task of the application entity residing in the entry centre. (EUR CIDIN Manual 3.2.6.2)

Note 4.- From the above CIDIN addressing provisions the following can be postulated for the address conversion in the CIDIN/AMHS Gateway:

- For non-AFTN applications (MTCUs) address translations have to be performed between CIDIN entry/exit addresses and AMHS originator/recipient addresses.

- For the AFTN application (MTCU), address translations have to be performed between AFTN originator/destination addresses and AMHS originator/recipient addresses as only the AFTN addresses have an end-to-end relevance. The CIDIN exit address via which a given destination address is reached has to be determined according to the CIDIN conventions.

Note 5.- The Figures 3.1.2.4.3.6-1 and -2 depicts the principle approach for conversion of AMHS addresses in CIDIN addresses and vice versa.

Note 6.- The length of CIDIN entry and exit addresses shall not exceed 8 letters. An addressing scheme is specified where the four leading letters are formed by the ICAO location indicator of the concerned entry or exit centre, respectively. (EUR CIDIN Manual: 3.3.4.1 and Appendix B 8.6.6.3.1.3.2, 8.6.6.4.1.4.5)

3.1.2.4.3.4.1.1	For interoperability with the AFTN application of the CIDIN the mutual address conversions shall be performed between AFTN addresses (i.e. addressee indicators, originator indicators) and MF addresses( i.e. recipient-name, originator).

3.1.2.4.3.4.1.2	For interoperability with non-AFTN applications of the CIDIN the mutual address conversions shall be performed between CIDIN addresses (i.e. exit addresses, entry addresses) and MF addresses (i.e. recipient-name, originator).

            - Address look-up tables

Editor´s note.- The provisions should comply with the principles applied to address conversions supported by the ATN Directory information as envisaged for the Extended ATS Message Service. (See Honolulu WG3/SG1 meeting report, sections 4.2.3 and 4.2.4.)

Editor´s note. – The following headlines have been taken from WP/226 (WG3/SG1 Bordeaux meeting). Their insertion here shall indicate the remaining work to be done.

          - Action upon reception of AMHS Probe

###### Common provisions for all types of MTCUs: Initiation of DR or NDR

        - Conversions related to the AFTN application of the CIDIN

Editor´s -note 1.- CIDIN messages associated with the AFTN application include AFTN (user) messages and AFTN service messages with end-to-end significance within the AFTN.

Editor´s note 2.- For consistency with the AFTN/AMHS Gateway functionality in an interconnected AFTN/CIDIN/AMHS environment, the conversions related to the AFTN application have to follow the principles specified in SARPs 3.1.2.3.4 and .5 (AFTN/AMHS conversions).

          - CIDIN/AFTN to AMHS conversion

            - Control function

Editor´s note.- Adaptation of SARPs 3.1.2.3.4.1. The message categories to be considered are AFTN message, AFTN acknowledgement message, AFTN service message related to unknown addressee indicators and AFTN service message requesting repetition by the originator. (The latter message category is in the AFTN/AMHS Gateway handled by the AFTN component.) The various message categories are presented by the service parameters of the T-MESSAGE.Indication).

            - Conversion of CIDIN/AFTN messages

Conversion into IPM in accordance with SARPs 3.1.2.3.4.2

            - Conversion of CIDIN/AFTN acknowledgement messages

Conversion into IPN(RN) in accordance with SARPs 3.1.2.3.4.3

            - Conversion of CIDIN/AFTN service messages related to unknown addressee indicators

Conversion into NDR in accordance with SARPs 3.1.2.3.4.4

            - Actions for CIDIN/AFTN service messages requesting repetition by the originator

Provision according to SARPs 3.1.2.3.2.1.12

          - AMHS to CIDIN/AFTN conversion

The conversion follows the principles of SARPS 3.1.2.3.5, however the generated AFTN messages have to be mapped onto the service parameters of the T-MESSAGE.Request.

            - Control function

Adaptation of SARPs 3.1.2.3.5. Identification of IPM, IPN(RN), NDR and Probe.

            - AMHS IPM conversion

Conversion into CIDIN/AFTN message in accordance with SARPs 3.1.2.3.5.2.

            - AMHS RN conversion

Conversion into CIDIN/AFTN acknowledgement message in accordance with SARPs 3.1.2.3.5.3.

            - AMHS Non-delivery report conversion

Conversion into CIDIN/AFTN service message related to unknown addressee indicators in accordance with SARPs 3.1.2.3.5.4.

            - Actions upon reception of AMHS Probe

CIDIN/AFTN specific actions in accordance with SARPs 3.1.2.3.5.5.

          - Generation of CIDIN information messages with encapsulated AFTN service information

Editor´s note.- The MTCU supporting the AFTN application generates under certain circumstances AFTN service messages of the type “acknowledgement” or “unknown addressee indicator” destined for the AFTN origin station (see Figure 3.1.2.4.2-1). This service messages leaves the gateway encapsulated in a CIDIN information message. The AFTN application in the exit centre will de-capsulate the AFTN service information for further forwarding to their final destinations.

        - Conversions related to the OPMET application of the CIDIN

        - Conversions related to CIDIN Operator Service Message interchange

- END -