# WP 6/25 : Pilot Preferences Downlink

| Service No. | Name | Abb. | Cat | References | Version No. |
| --- | --- | --- | --- | --- | --- |
| O-13 | Pilot Preferences Downlink | PPD |  |  | 0.91 29/3/99 |

The PPD service described in this document at the time of its publication is not covered by the ICAO SARPS.

# Table of Contents

1.	Scope and Objective	2

2.	Expected Benefits, Anticipated Constraints, and Associated Human Factors	2

2.1	Expected Benefits	2

2.2	Anticipated Constraints	2

2.3	Associated Human Factors	2

3.	Service Operational Context	2

4.	Overview of Operating Methods with and without Data Link	3

4.1	Information Exchange Diagram	3

5.	General Service Requirements	4

6.	Detailed Service Operating Method with Data Link	5

6.1	Service Availability	5

6.2	Service Description	5

6.2.1	Normal Mode	5

6.2.2	Sequence of Services	6

6.2.3	Additional Guidelines	6

6.2.4	Abnormal Mode	6

7.	Time Sequence Diagrams	6

8.	Quality Of Service Requirements	7

9.	Information Exchanges	7

10.	Message Sequence Matrix	8

# 1.	Scope and Objective

The objective of the PPD service is to enhance ATC planning through the provision of Aircrew preferred parameters to the Controller.

The PPD service allows Aircrew to provide the Controller with information not available in the filed flight plan (e.g.  maximum flight level) as well as requests for modification of some flight plan elements (e.g.  requested flight level).

This information forms an important addition to Controller decision making, enabling a better understanding of Aircrew needs and improving the Controller’s assessment of the traffic situation.  Therefore the control process is enhanced by becoming more responsive to Aircrew needs and improving its efficiency.

The PPD service is foreseen to be initially used in European continental airspace.

# 2.	Expected Benefits, Anticipated Constraints, and Associated Human Factors

## 2.1	Expected Benefits

- Direct provision of up-to-date Aircrew preferences to the Controller;

- Reduction of both Controller and Aircrew workload;

- Reduction of voice channel occupancy;

- Extension of the domain of common reference for Aircrew and Controller;

- Improvement of capacity and efficiency by allowing for more anticipation in planning of traffic through early awareness of Aircrew preferences.

## 2.2	Anticipated Constraints

- None.

## 2.3	Associated Human Factors

- Requires Aircrew inputs.

# 3.	Service Operational Context

Aircrew have preferences on the way the flight is to be conducted for various operational reasons.  In order to elaborate pertinent control strategies, Controllers need to be aware of these preferences.

The Filed Flight Plan contains some of these preferences, but Controllers very often need to obtain from the Aircrew some preferences not contained in the filed flight plan such as ‘maximum flight level’ or ‘top of descent’.  Moreover, the information contained in the filed flight plan available in the ATSU may not correspond to the Aircrew preferences.  For instance, the requested flight level filed in a repetitive flight plan may not be the one preferred by the Aircrew for a specific flight due to constraints such as aircraft performance or changing weather conditions.

Most of the Aircrew preferences information is frequently unavailable to the Controllers until the aircraft is on their frequency.  Its availability before the aircraft reaches the sector would enable the Controllers to improve planning and become more responsive to Aircrew needs.

The PPD service automates the provision to the Controllers of these Aircrew preferences and allows for the provision of those parameters to the Controllers even before the aircraft reaches their sector.

Additionally, the PPD service will reduce Aircrew workload, since the Aircrew will state their preferences only once and they will be automatically notified at the appropriate time to all the relevant Controllers.  This will relieve the Aircrew from the need to repeat the request each time they are transferred to a new Controller.

# 4.	Overview of Operating Methods with and without Data Link

| Step | Operating Method without Data Link | Operating Method with Data Link |
| --- | --- | --- |
| 1 | Controllers obtain the Aircrew preferences through voice communications. Often the Aircrew must re-issue the same request to several ATSUs. | Once input by the Aircrew, the parameters are automatically downlinked without Controller or Aircrew involvement. |
| 2 |  | The ground system automatically makes the downlinked parameters available for display to the Controller. |
| 3 |  | No Controller response to the transmitted data is necessary in the PPD service. |

## 4.1	Information Exchange Diagram

# 5.	General Service Requirements

| Number | Requirement |
| --- | --- |
| 1 | Aircrew shall be able to specify values for the PPD parameters listed in sub-section 6 below. If the Aircrew does not specify a value for one or more of the PPD parameters, they shall remain unspecified (i.e. no value defined for those parameters). The Aircrew shall be able to change or to delete (i.e. leave unspecified) any PPD parameter value at any time. |
| 2 | When the FMS holds a computed value for one of the PPD parameters to be downlinked, this value shall be presented to the Aircrew as a default that may be confirmed or modified if so required prior to its transmission. The Aircrew may also decide not to accept the FMS default value and leave the parameter unspecified. |
| 3 | The Aircrew specified PPD parameters shall be available to ATSUs currently or expected to be responsible for the flight. ATSUs may receive PPD data via air-ground or ground-ground communications. Note: This PPD service description does not require any specific transmission method (i.e. PPD parameters could be downlinked in one or several messages, on event or periodically, using air-ground addressed, air-ground broadcast or ground-ground communications or using any combination of all these options. This service description does not prescribe any technical implementation. Messages for the transmission of PPD parameters and their associated attributes are expected to be developed by the relevant ICAO Panel.). |
| 5 | The most recent PPD parameter values shall be made available to the Controller at all times. |
| 6 | The Controller's HMI should indicate aircraft for which PPD data is available. |
| 7 | The avionics shall not modify the Aircrew specified PPD parameters without prior Aircrew confirmation. |
| 8 | An initial set of PPD parameters is identified in sub-section 6 below. Data link systems should be capable to incorporate additional parameters in the PPD service when required. |
| 9 | The Aircrew shall be able to specify PPD parameter values for each flight phase separately. PPD parameter values are only valid within the phase for which they are specified. |
| 10 | The Aircrew shall be able to know whether the C-ATSU Controllers have the current PPD parameters available to them or not. This notification shall not require any inputs from any Controller. |
| 11 | The controller should be able to choose the parameters to be displayed. |

Note: ODIAC will have to review statement 10 and decide if it’ll remain in the present form, or will be reformulated.

# 6.	Detailed Service Operating Method with Data Link

## 6.1	Service Availability

The PPD service shall be available in all phases of flight (from pre-flight to landing).

## 6.2	Service Description

### 6.2.1	Normal Mode

At their convenience, the Aircrew may input their preferences and constraints.  The parameters for which the Aircrew shall be able to specify their preferences are listed below.

The Aircrew shall not be required to input values for all the PPD parameters.  They shall be able to input values for the parameters for which they have preferences and leave the others without specified value.  No default values shall be automatically input by the system for the parameters without specified value.  The Controller shall only be informed of the parameters for which a value has been provided  by Aircrew.

When the FMS holds a computed value for one of the PPD parameters to be downlinked, this value shall be presented to the Aircrew as a proposition that may be confirmed or modified if so required prior to its transmission.  The Aircrew may also decide not to accept the FMS default value and leave the parameter without value.

- Two types of parameters are supported by the PPD service:

Preferences: 	These are passive Aircrew requests

Limits: 	These are operating constraints on the way the aircraft can be flown that the Aircrew wish to provide to the Controllers.

- The preferences and limits supported by the PPD service shall be:

| Parameter | Type |  | Units |  | Number of Values |  | End of Validity |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Preferred Operating Climb Airspeed | Preference |  | knots/ Mach |  | 1 |  | Top of Climb |  |
| Preferred Cruise Flight Level | Preference |  | FL |  | 1 |  | Top of Descent |  |
| Maximum Flight Level | Limit |  | FL |  | 1 |  | Top of Descent |  |
| Preferred Operating Cruise Airspeed | Preference |  | knots/ Mach |  | 1 |  | Top of Descent |  |
| Top of Descent Time | Preference |  | hms |  | 1 |  | Top of Descent |  |
| Preferred Operating Descent Airspeed | Preference |  | knots/ Mach |  | 1 |  | At ATSU’s instruction. |  |
| Current Configuration Operational Speed Range | Limit |  | knots/ Mach |  | 2 |  | Landing |  |
| Minimum Clean Manoeuvring Speed | Limit |  | knots |  | 1 |  | Landing |  |
| Preferred Arrival Holding Speed |  | Preference |  | knots |  | 1 |  | Landing |
| Intermediate Approach Speed | Preference |  | knots |  | 1 |  | Landing |  |
| Minimum Uncorrected Approach Speed | Limit |  | knots |  | 1 |  | Landing |  |
| Preferred Landing Runway | Preference |  | Designator |  | 1 |  | Landing |  |

The PPD preferences specified by the Aircrew shall be available at the concerned ATSU(s) from the moment of the establishment of data link communications with the aircraft, except when received via ground ground communications, and updated upon Aircrew’s entry of new values.

A PPD preference shall be valid from the moment it is input by the Aircrew until the associated End of Validity event takes place.  PPD preferences shall not be displayed to the Controller after their associated End of Validity event has taken place.

### 6.2.2	Sequence of Services

- The PPD service does not require any preceding Service.

### 6.2.3	Additional Guidelines

The avionics should provide assistance to the Aircrew in order to ensure that the PPD values are valid (e.g.  automatic warnings when PPD parameters are no longer valid).

### 6.2.4	Abnormal Mode

| Abnormal Event | Response |
| --- | --- |
| Reception of a PPD message which can not be correlated with the aircraft it originates from. | The PPD message shall be discarded. |

# 7.	Time Sequence Diagrams

Not applicable.

# 8.	Quality Of Service Requirements

| Message Category: | Not applicable. The PPD service does not require specific messages to downlink the parameters, but the maximum time elapsed between the Aircrew input of a parameter (or an updated value for a parameter) and its delivery to the Controller working position shall be less than 60 seconds 99.996% of the time. |
| --- | --- |
| Communications Priority: | 4 |
| Information Urgency:. | Normal |
| Information Security: |  |
| Data Origin Authentication: | Normal |
| Access Control: | M, A |
| Data Integrity: | Maximum |

# 9.	Information Exchanges

- Messages for the transmission of PPD parameters and their associated attributes are expected to be developed by the relevant ICAO Panel.

| Parameter | Presentation Units |  | Presentation Resolution |
| --- | --- | --- | --- |
| Preferred Operating Climb Airspeed | knots Mach |  | 5 knots 0.005 Mach |
| Preferred Cruise Flight Level | FL |  | 1 |
| Maximum Flight Level | FL |  | 1 |
| Preferred Operating Cruise Airspeed | knots Mach |  | 5 knots 0.005 Mach |
| Top of Descent Time | hms |  | 1 second |
| Preferred Operating Descent Airspeed | knots Mach |  | 5 knots 0.005 Mach |
| Current Configuration Operational Speed Range | knots Mach |  | 5 knots 0.005 Mach |
| Minimum Clean Manoeuvring Speed | knots |  | 5 knots |
| Preferred Arrival Holding Speed |  | knots | 5 knots |
| Intermediate Approach Speed | knots |  | 5 knots |
| Minimum Uncorrected Approach Speed | knots |  | 5 knots |
| Preferred Landing Runway | Designator |  | N/A |

# 10.	Message Sequence Matrix

Not applicable.