# Attachment O : Common Trajectory Coordination (COTRAC)

| Service No. | Name | Abb. | Cat | References | Version No. |
| --- | --- | --- | --- | --- | --- |
| O- | Common Trajectory Coordination (COTRAC) | COTRAC |  | “ICAO Manual of Air Traffic Services (ATS) Data Link Applications” | 0.5 22/01/99 |

# Table of Contents

1.	Scope and Objective	1

2.	Expected Benefits, Anticipated Constraints, and Associated Human Factors	1

2.1	Expected Benefits	1

2.2	Anticipated Constraints	2

2.3	Associated Human Factors	2

3.	Service Operational Context	2

4.	Information Exchange Diagram	3

5.	Overview of operating methods with and without data link	3

6.	Detailed service operating method with data link	4

6.1	Functional levels (TBD)	5

# Scope and Objective

The purpose of COTRAC is to establish and agree trajectory contracts between aircrew and controllers in real time using graphical interfaces, air and ground data communications and automation systems, in particular the FMS, by means of a structured negotiation method in order to significantly enhance ATM capacity and flexibility.

The coordination of trajectories can be performed more effectively by involving Airline operations directly or through Collaborative Decision Making and maximizing ground/ground data communications to reduce costs.

Ground automation tools such as conflict detection, arrival managers, monitoring aids, can be enhanced through the availability of accurately predictable trajectories.

COTRAC will also contribute to the optimal use of the Flexible Use of Airspace (FUA) and Route Navigation (R-NAV) concepts.

Airborne situational awareness is improved through the display of flight path intent of aircraft affecting own trajectory, e.g. in case of parallel approaches.

Note:	It is not the purpose of COTRAC to determine how a trajectory can be used to enhance automation tools.

# Expected Benefits, Anticipated Constraints, and Associated Human Factors

## Expected Benefits

- Ensures that 4D route clearances are complete until destination.

- Optimised flight efficiency due to the reduction of the uncertainty about the future trajectory.

- ;

- Provision to all potential users of accurately predictable 4D trajectories;

- Decreased pilot and controller workload through Automation and usage of graphical interfaces, resulting in an increased productivity and a major capacity enhancement.

- Improvement of quality of ground ATC tools such as trajectory prediction, conflict detection, monitoring aids,  arrival and departure managers, through availability of aircraft derived trajectories;

- Improved flight planning for controllers;

- Potential for improved airborne situational awareness;

- Possibility for controllers to perform “What if” scenarios;

- Common air and ground reference and conformance for aircraft trajectories.

## Anticipated Constraints

- Complex end to end communications, data processing and interface systems required for optimal use;

- Graphical interfaces are a prerequisite;

- Common data base references must be available between ground and airborne systems;

- Production and formal approval of new ICAO and industry Standards endorsing the operational use of such a system.

- Certification of airborne avionics and related software

## Associated Human Factors

- Humans must be able to control and validate the use of the system at all times;

- Extensive operational validation will be required prior to operational use;

- Care must be taken not to negatively interfere with Aircrew primary flight tasks;

- Significant deviation from Current Controller/Pilot communications Standards.

# Service Operational Context

- Definitions

- TRAJECTORY

- A trajectory is a representation of the path of an aircraft, describing the horizontal and vertical profile over time. (Definition from the Trajectory prediction group).

- The representation may also allow identification of points on the route such as sector boundaries and penetration of special use airspace. Typical ATM systems may represent trajectories as a sequence of annotated 4-D points, although this is not the only representation.

Trajectory is the curve described by an aircraft under action of its engine(s); the trajectory consist of time, point in space and level.

- PREDICTED TRAJECTORY

- A predicted trajectory is a representation of the expected trajectory of an aircraft, based on a set of input data and assumptions.

- The predicted trajectory includes, inter alia, position, level, and time data, and uncertainty data which identifies the probability of deviation from the prediction.

- TRAJECTORY CONTRACT

A trajectory contract is a representation of the path that will be followed by an aircraft as a result of an agreement between the current controller and the aircrew. The contract may contain one or more of the following elements: time, position, level, attribute and shall always contain an acknowledgement from the aircrew.

A flight trajectory constitutes the essential common data which requires to be known to the aircrew, ATS and Airline operations, there is only one trajectory for a given flight; establishing a mechanism enabling knowledge and usage of aircraft trajectories by the different interested parties throughout a flight will enable to provide all potential users with accurately predictable trajectories.

The ATC negotiation process consist mainly in requiring an aircraft to modify its flight profile using controller/aircrew communications; the controller should first know the current aircraft profile, which often requires a question to the aircrew e.g. report heading, report speed, report vertical rate,. Etc. The controller then determines, on the basis of traffic situation, separation to be effected or any other operational reason, what profile the aircraft will have to follow. The controller then instructs the aircrew to effect the required profile change, the aircrew acknowledges and effects the maneuver accordingly. At a later stage the aircrew may be required to resume own navigation, in case of a tactical instruction.

The process entails extensive communications to the extent that the majority of controller/aircrew communications relate to trajectory.

An other limiting factor is that controller and ground systems do not know the actual aircraft trajectory intent, other than via the filed and current flight plan or via extrapolation techniques.

Currently, the aircraft’s trajectory is known only by the aircrew, using or not Flight Management Systems; the FMS enables in particular flight planning and navigation computation. The aircraft trajectory from departure gate to destination runway can be computed in 4D points including company routes, SIDs, STARs, Airways, pilot defined waypoints and points at which a maneuver is to be performed.

The ADS and CPDLC can provide basic forms of COTRAC.

# Information Exchange Diagram

# Overview of operating methods with and without data link

| Step | Operating Method without Data Link | Operating Method with Data Link |
| --- | --- | --- |
| 1 | The controller mentally determines the future path of a flight using surveillance tools and flight data. | The controller interrogates the aircraft systems (e.g. via the ADS) to obtain the FMS based Flight path intent. |
| 2 |  | The aircraft system downlinks the requested Flight path intent data. |
|  | The controller determines the future trajectory to be followed by the aircraft on the basis of traffic situation, airspace configuration, prevailing weather conditions and the flight plan. | The controller uses a graphical interface to alter the flight path such that the currently expected operational objectives are met. |
|  | The controller requests information to the aircrew on actual or predicted trajectory items, e.g. speed, heading, ETAs... | The controller observes the FMS flight path intent and/or air vector data, and may request pilot intentions. |
| 3 | The controller instructs the aircrew to alter its current trajectory. | The controller uplinks the required trajectory to the aircraft adding any attribute as necessary |
| 4 | The aircrew acknowledges the instruction and proceeds in accordance with the clearance. | The aircrew acknowledges the received trajectory and execute it by loading the FMS accordingly. |
| 5 | The controller monitors that the clearance is executed in accordance with its terms. | The recalculated trajectory is downlinked to update ground based systems and for air and ground conformance. |

# Detailed service operating method with data link

Aircraft FMS trajectory can be defined using the following parameters (non-exhaustive):

- Co-ordinates of a point or position relative to another point on the trajectory;

- Times at the point to the nearest second.

- Levels at the point to the nearest 100ft.

- The published ATS route from which a point is derived (if applicable).

- the maneuver to be undertaken at a point and associated parameters.

A maneuver is characterized as follows by:

- The point at which the aircraft is modifying its current trajectory, trajectory change point (TCP)

- A turn,

- Change of speed,

- Change of altitude.

It is a prerequisite that common reference data is used in air and ground end systems, COTRAC  will be based on coordinating the basic elements of data to be exchanged using universal standards. The basic data elements are point, level and time, to which an attribute will be added to indicate the status of the coordination.

Trajectory Points shall be expressed in WGS 84 LAT/LON coordinates;

Time shall be UTC;

Level shall be as in current ICAO documentation.

In order to achieve an optimum coordination a structured negotiation method shall be applied.

The application of structured negotiation and common reference supported by air/ground data communications, appropriate HMI and procedures will enable COTRAC.

The completion of a COTRAC dialogue constitutes an ATC contract, a clearance.

The structured negotiation shall consist in the exchange of trajectory items supplemented by an attribute indicating the status of the coordination and agreement between the aircrew and the controller.

- OPERATIONAL ATTRIBUTES:

| ATTRIBUTE | UPLINK | DOWNLINK |
| --- | --- | --- |
| PROPOSED [XYZ, UTC, FL] | * A COTRAC contract is proposed to the Aircrew as stated | N/A |
| REQUESTED | N/A | * A COTRAC contract is requested by the Aircrew as stated |
| CLEARED | A COTRAC contract is issued and is expected to be executed | N/A |
| ACCEPTED | The requested COTRAC is accepted and becomes a cleared COTRAC | * The Cleared or proposed COTRAC to which this reply refers is accepted by the aircrew as stated and will be complied with. |
| ACKNOWLEDGED | The uplinked COTRAC message has been acknowledged by the airborne system, is valid and will be presented to the Pilot | * The accepted, Cleared or proposed COTRAC to which this reply refers is acknowledged by the ground system as stated and is valid for use by ground system and controller |
| BEING EXAMINED (STANDBY ?) | The requested COTRAC is under examination by the controller* | The proposed COTRAC is under examination by the Aircrew* |
| REVISED | A previously agreed COTRAC is revised as stated | N/A |
| REJECTED | A requested COTRAC is rejected by ATC | An uplink COTRAC message is rejected by the airborne system |
| CONFIRMED | tbd | tbd |
| UNABLE | The requested COTRAC is not available | The aircrew is unable to accept the proposed COTRAC* |

## Functional levels (TBD)

Basic COTRAC: within one ATSU’s airspace

Advanced COTRAC: within 2 or more ATSU’s airspaces

Ground COTRAC (gate to gate COTRAC ?)

Downstream COTRAC