ATNP WG3-17 WP – 39

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Gran Canaria, Spain

September 27 - October 1, 1999

Modification to CM for Rejected Logon Definition

Prepared by:  G. Saccone

Summary This paper presents a solution for an identified industry for adding a clarification requirement in Package 1 CM.

# Introduction

This paper discusses a solution to an operational ambiguity that has been identified by implementers of CM, involving what is considered a “rejected” CM logon.  Since different implementers may take different approaches to this aspect of CM, it would seem reasonable to make an explicit definition of a rejected logon.  This modification would be to the Package 1 CM SARPs.

# Discussion

In the ATS Manual of Data Link Operations, an operational requirement of CM is the ability of the ground CM to “reject” a logon, and give an indication back to the aircraft that the logon has been rejected.  A rejected logon has been interpreted by the ATN Panel, and WG3/SG2, to mean that no application information is returned to the aircraft.  Therefore, if a CM-ground-user wants to reject a logon, the CM-ground-user would simply not supply any application information in the logon response.  The CM-air-user would then interpret this as a rejection, either for operational reasons (e.g. optional information that is required for a particular airspace is missing) or technical reasons (e.g. there is a problem with the ground system and the service is not available).

However, as implementations are built, there is confusion as to what constitutes a rejected logon.  Since the interpretation of a rejected logon is an implementation detail, it is not explicitly defined in the SARPs.  It is mentioned in the guidance, but the guidance has not been widely read for a variety of reasons.  And of course, guidance is just that—guidance; it does not have “shall” status.  Accordingly, implementers may make different interpretations on rejected logon definitions, and this may lead to interoperability problems.  For example, one implementation may choose to interpret a rejected logon as is currently defined in the guidance (responding to a CM-logon request with no data) while another implementation may decide to invoke a CM-user-abort service if a CM-logon is to be rejected.

While operational procedures should cover these cases, operations will become more complex, as this aspect of CM becomes local, not global.

# Concept

In order to unambiguously define what a rejected logon means, and to possibly avoid unnecessary use of aborts, it is proposed to add a new user requirement in 2.1.7 of CM, to read as follows:

2.1.7.x.x  If a CM-ground-user wishes to reject a logon for any reason, the CM-ground-user shall put no information in the CMLogonResponse APDU.

Note. —  This may be done for either operational reasons (e.g. optional information that is required for a particular airspace is missing or information for the requested facility is not available) or technical reasons (e.g. there is a problem with the ground system and the service is not available).

This change will have no impact on SARPs interoperability, since this a change in user requirements only and not the "bits on the wire."  Therefore, there are no backwards compatibility problems with this proposed solution.

# Conclusion

This paper gives a low-impact solution to explicitly define what constitutes a “rejected logon”.  The group is invited to comment on this approach, and its applicability for inclusion into Package 1 CM.