Draft Validation Report for FIS(METAR) ATN Application

SUMMARY

This is the draft ATNP/3 Validation Report for the major enhancement which has been made to the ATN Flight Information Services (FIS) ATN Application by ATNP/WG3/SG2, i.e. the support of the METAR service by the FIS Application.

This report presents the results of the validation and implementation programmes that have been undertaken by various States and Organisations, which apply to the FIS Application Version 2.  It summarises the results and analyses them against a set of high-level validation objectives (VOs).

It is concluded that the enhanced technical provisions will be sufficiently validated for inclusion in ICAO Doc. 9705 Edition 3.

Table of Contents

1. 	Introduction	1

1.1. 	Scope	1

1.2. 	Background	1

2. 	High Level Validation Objectives	1

2.1. 	Validation Objectives (VO)	1

2.2. 	Grouping of Requirements	1

3. 	Validation Means	1

4. 	Functional Validation Achieved by States and Organisations	1

5. 	Summary of Activities Supporting Validation	1

5.1. 	ATNP/WG3/SG2	1

5.2. 	NATS Review	1

5.3. 	CENA CHARME project	1

5.3.1	General	1

5.3.2	FIS(METAR) Prototyping and Validation	1

6. 	Defect Report Summary	1

7. 	Results and Analysis	1

7.1. 	SVO 1	1

7.2. 	SVO 2	1

7.3. 	FVO 1	1

7.4. 	FVO 2	1

7.5. 	FVO 3	1

7.6. 	FVO 4	1

7.7. 	FVO 5	1

7.8. 	FVO 6	1

7.9. 	FVO 7	1

7.10. 	TVO 1	1

7.11. 	TVO 2	1

7.12. 	TVO 3	1

7.13. 	TVO 4	1

7.14. 	TVO 5	1

7.15. 	TVO 6	1

7.16. 	TVO 7	1

7.17. 	TVO 8	1

7.18. 	TVO 9	1

7.19. 	TVO 10	1

7.20. 	TVO 11	1

8. 	Conclusions	1

# 1	Introduction

## 2	Scope

Since the publication of the first edition of the Manual of Technical Provisions for the Aeronautical Telecommunication Network (ATN) (ICAO Doc. 9705/AN-956), a number of enhancements to Sub-Volume 2 of that document, the Air-Ground ATN Applications, have been progressed within ATNP/WG3.  The effect of the enhancements in question is to add new functionality, and hence new technical provisions, which need to be validated before they can be published.

This is the draft ATNP/3 Validation Report for the major enhancement which has been made to the ATN Flight Information Services (FIS) ATN Application by ATNP/WG3/SG2, i.e. the support of the METAR service by the FIS Application.  This report presents the results of the validation and implementation programmes that have been undertaken by various States and Organisations, which apply to the METAR enhancement.  It summarises the results and analyses them against a set of high-level validation objectives (VOs).

The METAR enhancement have been designed for backwards compatibility and interoperability with the first edition of Doc 9705, and this compatibility also needs to be validated.

## 3	Background

The first edition of ICAO Document 9705/AN-956 was published in November 1998. This document contains in section 2.4 the specification of Version 1 of the FIS Application, i.e. the version supporting the Automatic Terminal Information Service (ATIS) only. Since that time a number of Proposed Defect Reports (PDR) have been raised against this version and have been have resolved through the ATNP Configuration Control Board (CCB). This will result in the publication by ICAO in November 1999 of amendment 1 of Doc. 9705. Amendment 1 still relates to version 1 of the FIS Application.

On the basis of Doc 9705 Ed. 1 Amendment 1, the specification of Version 2 of the FIS Application has been developed by WG3/SG2 and validated through paper review activity and software implementation. The first mature draft of this specification was presented to WG3 in Spain in October 1999 as well as the draft of the validation report (this document).

The change history is summarised below:

- Table 1-1: Change History

| ICAO Version Number | CCB Version Number | Date | FIS Protocol Version | Comment |
| --- | --- | --- | --- | --- |
| - | Version 1.1 | March 97 | 1 | Phuket version |
| - | Version 2.2 | Dec 97 | 1 | Montreal version |
| Doc 9705 Edition 1 | Version 2.3 | Nov 98 | 1 |  |
| Doc 9705 Edition 2 | Version 3.0 | Nov 99 | 1 | Output Naples |
| - | Edition 2.0p | Oct 99 | 1 and 2 | Input Spain |
| Doc 9705 Edition 3 | ? | ATNP/3 | 1 and 2 |  |

# 4	High Level Validation Objectives

## 5	Validation Objectives (VO)

- At the lowest level of validation, every technical provision clause (“shall” and “should” statement) is validated for correctness, consistency, lack of ambiguity and lack of duplication. This is typically done as an integral stage of implementation.  This report concentrates instead on high-level validation objectives.  Each validation objective is categorised as:

- System Level Validation Objective (SVO), relating to the system level requirements which are based on operational requirements within the ICAO Draft Manual of ATS Data link Applications, or elsewhere.

- Functional Validation Objective (FVO), relating to the functional characteristics described in the Technical Provisions.

- Technical Validation Objective (TVO), relating to the technical details in the Technical Provisions.

- The following table lists the high-level validation objectives adopted for the air-ground ATN applications functional enhancements.

- Table 2-1: Validation Objectives

|  | - VO | - Description |
| --- | --- | --- |
|  | - SVO 1 | - To determine which System Level Requirements are satisfied by the functional descriptions in combination with the user requirements and recommended practices. |
|  | - SVO 2 | - To determine if the ATN specifications are mutually consistent and that backwards compatibility is achieved. |
|  | - FVO 1 | - To determine if the functional descriptions are compatible with the technical requirements. |
|  | - FVO 2 | - To determine if the user requirements and recommended practices are compatible with the technical requirements. |
|  | - FVO 3 | - To determine if the technical provisions are complete. |
|  | - FVO 4 | - To determine if the technical provisions are unambiguous. |
|  | - FVO 5 | - To determine if the technical provisions are consistent. |
|  | - FVO 6 | - To determine if there are redundant technical provisions, i.e. requirements which would have no effect if removed. Note: This VO should be interpreted to mean that there are no requirements that are not necessary for the defined functionality, or to achieve migration to future functionality. It is not meant to eliminate possible duplicated statements of requirement that are known to exist. |
|  | - FVO 7 | - To determine if provision has been made to ensure that the technical provisions are implementation independent. |
|  | - TVO 1 | - To determine if the protocol description supports the stated end to end services. |
|  | - TVO 2 | - To determine if the protocol description has any unacceptable behaviour |
|  | - TVO 3 | - To determine if the abstract service interface parameters are mapped appropriately to PDU fields and/or communication service interface parameters, and vice versa. |
|  | - TVO 4 | - To determine if protocol errors in the peer application entity are correctly handled. |
|  | - TVO5 | - To determine if the SARPs are consistent with the Upper Layer architecture to the extent that this is a requirement, e.g. use of the Dialogue Service, application of the control function. |
|  | - TVO 6 | - To determine if the APDUs are correctly specified. |
|  | - TVO 7 | - To determine if provision for QOS management has been addressed. |
|  | - TVO 8 | - To determine if provision for future migration has been addressed. |
|  | - TVO 9 | - To determine if efficiency requirements have been addressed, e.g. minimising size of data transfer, appropriate maintenance of dialogue. |
|  | - TVO 10 | - To determine that the functionality described in the technical provisions is implementable. |
|  | - TVO 11 | - To determine that independent implementations built in accordance with the technical provisions will be able to interoperate. |

## 6	Grouping of Requirements

- For the validation of version 2 of the FIS Application, the following functional groups of requirements have been identified:

- requirements describing the METAR service and supporting protocol in Version 2: all the new requirements need to be validated separately. It should be noted that 95% of the FIS service and protocol is common to ATIS and METAR and have already been validated for Package-1. The validation exercise will concentrate on the requirements specific to METAR.

- requirements describing the ATIS service and supporting protocol in Version 2: all these requirements have already been validated for version 1. The validation of these requirements will consist in checking the non-regression of the functionality in version 2.

- requirements allowing the simultaneous operation of METAR and ATIS in Version 2. The support of more than one FIS service is a new feature in version 2 which need to be attentively validated.

- requirements guarantying the interoperability between Versions 1 and 2. Some requirements have been added in version 1 and 2 to allow a minimum level of interoperability between systems implementing different versions of the FIS application.

# 7	Validation Means

The following generic means of validation have been identified, and are used in Table 4.1.

- Two or more independently developed interoperating implementations validated by two or more states/organisations.

- Two or more independently developing interoperating implementations validated by one state/organisation.

- One implementation validated by more than one state/organisation.

- One implementation validated by one state/organisation.

- Partial implementation validated by one or more state/organisation.

- Simulation, analysis using tools e.g. ASN.1 compiler, modelling tools.

- Analysis and inspection.

# 8	Functional Validation Achieved by States and Organisations

The following table summarises the validation activities that have completed to date. The letters in the table correspond to the validation means given in section 3.

Note.—  In the present draft, the matrix is incomplete.  It will continue to be updated as the validation programmes listed in section 5 progress.

Table 4-1: Validation Activities Summary

| Group | ATNP/WG3/SG2 | CENA CHARME | NATS |
| --- | --- | --- | --- |
| Use of METAR with FIS/V2 – establishment of demand contract | g) | d) | g) |
| Use of ATIS with FIS/V2 - Non regression | g) | d) |  |
| Simultaneous use of ATIS and METAR with FIS/V2 | g) | d) |  |
| Interoperability FIS/V1 – FIS/V2 | g) | d) |  |

The validation programme has employed a number of validation methods including inspection and desk checking, the specification of an API based on the abstract service interface, as well as the development of a prototype.

# 9	Summary of Activities Supporting Validation

## 10	ATNP/WG3/SG2

Inspection and analysis of the FIS Application Version 2 SARPs has been performed by ATNP/WG3/SG2. This has involved close reading of the text with the specific aim of checking to make certain that there are no defects in the SARPs.

## 11	NATS Review

The U.K. NATS MET department reviewed the initial draft of the ASN.1 specifications. This review identified inconsistencies between the technical description (ASN.1) and the operational description of the METAR fields provided by ADSP in tabular forms. Co-ordination with the ADSP on these issues allows to progress de definition of the METAR parameters to a mature and stable state.

## 12	CENA CHARME project

### 5.3.1	General

The objectives of CHARME are to provide the French DGAC with:

- an ATN platform for data-link experiments on "Package-1" applications,

- a base for the prototyping of future air/ground data-link ATN applications,

- an infrastructure for the validation of some of the ATN "Package-2" features, with a priority on security services, naming and addressing extension, system management related to security, and key management mechanisms by CM ASE.

The CHARME developments consist of commercial off the shelf (COTS) products and CENA-originated components.

- The COTS components are the CO Session and Presentation entities, an ASN.1 compiler and associated PER runtime, and a development environment for communications entities. This environment provides testing and integration facilities, and proved to enable the porting of CHARME to various hardware platforms and operating systems.

- The CENA developments include the CL Session, Presentation and CO/CL Application entities, OSI-specified ASEs (CO/CL ACSE/Ed 2, ROSE, CMISE) and ICAO-specified ASEs (ADS, ARF, CM, CPDLC and FIS).

The CHARME Upper Layers have successfully been integrated on SUN and DEC ALPHA systems with the ProATN Lower Layers and with the DGAC-proprietary implementation of the ATN Lower Layers. CHARME will be used for providing the ATN Application service in the new ATC data link system. This system is developed by the DGAC to interface data link equipped aircraft with operational Controller Working Positions and Flight Plan Processing Systems.

This integration resulted in:

- a "Package-1" CO full-ATN stack including all versions of the ASEs (CM V1 and V2, ADS, ARF, CPDLC, FIS V1 and V2),

- a "Package-2" CO and CL full-ATN stack (with the exception of GACS).

The Package-2 stack includes ROSE and CMISE for system management, as well as the Security ASO for upper-layers security.

CENA provided the first implementation of the FIS Application Version 1 and will be a key contributor in the validation of Version 2.

### FIS(METAR) Prototyping and Validation

The CHARME project developed the first prototype of the FIS(METAR) application and conducted validation exercises following three phases as follos:

- the ASN.1 description specified in the draft SARPs was compiled with an COTS ASN.1 compiler. This permitted to identify several errors in the description of the METAR messages.

- based on the Version 1 FIS Application developed in a previous phase of the project, the software of version 2 of the application was produced, leading to the identification of several errors in the protocol specification. Interoperability between air and ground version 2 compliant systems was tested.

- interoperability between version 1 and version 2 compliant systems was finally tested.

# 13	Defect Report Summary

The table below gives a summary of the defect reports raised during the validation programme.

| - Ref. | - Source | - Section | - Description of change |
| --- | --- | --- | --- |
|  | - NATS | - ASN.1 | Operational review of the METAR fields raised some issues: - typos, - specification of the "no significant weather" type |
|  | - CENA | - ASN.1 | Compilation of the ASN.1 file identifies errors: - typos, - IMPORTS list incomplete, - FreeText type not defined |
|  | - CENA | - Protocol Specification | Upon receipt of a FISAccept, FISReject and FISReport APDUs for an Update Contract, the FIS-air-ASE must first check that the report type (ATIS, METAR…) matches the report type for which the contract was requested. Otherwise the dialogue must be aborted. |

# 14	Results and Analysis

## 15	SVO 1

To determine which System Level Requirements are satisfied by the functional descriptions in combination with the user requirements and recommended practices.

All the system level requirements relevant to the ATIS service were satisfied in FIS Version 1. They are de facto satisfied in FIS Version 2.

As determined by inspection, all the system level requirements relevant to METAR service are satisfied by the revision of Sub-Volume 2. (g)

This validation objective may be considered as being achieved.

## 16	SVO 2

To determine if the ATN specifications are mutually consistent and that backwards compatibility is achieved.

Version 2 is an extension to Version 1, meaning that the existing specifications of version 1 have been modified only in very few well-known areas. This approach guarantees the consistence and the backwards compatibility between version 1 and version 2 specifications of the FIS Application. (g)

Version 2 does not modify the relationships of the FIS Application with the other ATN Applications. The consistency with the other ATN specifications is therefore maintained.

Version 2 is not consistent with the ULCS enhancements.

This validation objective may be considered as being achieved.

## 17	FVO 1

To determine if the functional descriptions are compatible with the technical requirements.

During Package-1 validation, the technical requirements were examined to ensure they provide the intended functionality. Adding the METAR service did not change in nature the FIS functionality since METAR is using the existing FIS Demand contract. As a consequence, in Version 2, the functional descriptions remain compatible with the technical requirements. (g)

Upon completion of the Version 2 specification inspection and analysis process by several parties, no incompatibility has been reported, nor any defect report has been generated in this area. (g)

This validation objective may be considered as being achieved.

## 18	FVO 2

To determine if the user requirements and recommended practices are compatible with the technical requirements.

All the user requirements relevant to ATIS service were compatible with the technical requirements in FIS Version 1. They are de facto compatible in FIS Version 2.

All of the user requirements and recommendations in section 2.4.7 (User Requirements) related to the METAR service have been examined and have been determined to be compatible with the technical requirements. (g)

This validation objective may be considered as being achieved.

## 19	FVO 3

To determine if the technical provisions are complete.

All statements in the sections added for the METAR service on protocol have been analysed, syntax-checked and implemented in prototype form, and care was taken to not to make any assumptions where there were no “shall” statements. (d, g)

The defects reported during the specification review and the prototyping activities have been analysed and taken into account where appropriate. (d, g)

This validation objective may be considered as being achieved.

## 20	FVO 4

To determine if the technical provisions are unambiguous.

Specification of the API ensured that the ASI parts of the SARPs are specified unambiguously. (d, g)

This validation objective may be considered as being achieved.

## 21	FVO 5

To determine if the technical provisions are consistent.

Specification of the API ensured that the various ASI primitives and their parameters are specified in a consistent way. (g)

The FIS Version 2 prototype was built in a manner consistent with the specification. (d)

This validation objective may be considered as being achieved.

## 22	FVO 6

To determine if there are redundant technical provisions, i.e. requirements which would have no effect if removed.

Note:  This VO should be interpreted to mean that there are no requirements that are not necessary for the defined functionality, or to achieve migration to future functionality.  It is not meant to eliminate possible duplicated statements of requirement that are known to exist.

The tabulated requirements indicate that all stated requirements related to the METAR service are necessary. (g)

This validation objective may be considered as being achieved.

## 23	FVO 7

To determine if provision has been made to ensure that the technical provisions are implementation independent.

Development of FIS Version 1 Applications on different system, using different ASN.1 compiler and providing different APIs, have shown that the SARPs for FIS(ATIS) were implementation-independent. No implementation-related constraint has been defined for METAR. Taking the same specification approach for FIS(METAR) should guarantee the same result.

This validation objective may be considered as being achieved.

## 24	TVO 1

To determine if the protocol description supports the stated end to end services.

The end-to-end service provided to FIS-users is similar for both ATIS and METAR. This validation objective achieved for FIS Version 1 is therefore implicitly achieved for FIS Version 2. (g)

## 25	TVO 2

To determine if the protocol description has any unacceptable behaviour

The FIS Version 2 protocol was implemented completely. No unacceptable behaviour was detected. (d)

This validation objective may be considered as being achieved.

## 26	TVO 3

To determine if the abstract service interface parameters are mapped appropriately to PDU fields and/or communication service interface parameters, and vice versa.

All of the FIS(ATIS) parameters were appropriately mapped in FIS Version 1. These parameters are de facto compatible in FIS Version 2.

Inspection of the text and prototype implementation show that the METAR parameters are mapped appropriately to PDU fields and/or Dialogue Service primitives. (d, g)

## 27	TVO 4

To determine if protocol errors in the peer application entity are correctly handled.

All aspects of the FIS protocol were implemented and executed in the prototyping exercise, including error handling. The new situations where a protocol error could be detected induced by Version 2 have been individually tested by interoperability testing. (d, g)

This validation objective may be considered as being achieved.

## 28	TVO 5

To determine if the SARPs are consistent with the Upper Layer architecture to the extent that this is a requirement, e.g. use of the Dialogue Service, application of the control function.

The SARPs invoke the ULCS in a manner correct and consistent with the ATN ULCS SARPs and the ATN Enhanced ULCS. (d, g)

This validation objective may be considered as being achieved.

## 29	TVO 6

To determine if the APDUs are correctly specified.

Compared to Version 1, the ASN.1 specification of Version 2 makes use of extensibility fields and types importation. The use of these features have been inspected and syntax-checked using an ASN.1 compiler. (f, g)

This validation objective may be considered as being achieved.

## 30	TVO 7

To determine if provision for QOS management has been addressed.

QOS management by the Version 1 FIS Application has been improved in Doc 9705 Ed. 1 / Amendment 1 and validated by various implementations (c,g).

The same mechanisms are used unchanged in version 2.

This validation objective may be considered as being achieved.

## 31	TVO 8

To determine if provision for future migration has been addressed.

Provisions for extension in Version 1 were validated when adding the METAR service in the FIS Application. The same provisions – now validated – can be used to add other FIS services in the future. However, some requirements need to be added in Version 1 to fully take into account the interoperability with version 2.

Migration to the Enhanced ULCS is intrinsically guaranteed by the ATN Upper Layers specifications.

This validation objective may be considered as being achieved.

## 32	TVO 9

To determine if efficiency requirements have been addressed, e.g. minimising size of data transfer, appropriate maintenance of dialogue.

The same mechanisms defined for Version 1 are used unchanged in Version 2 (use of PER, definition of PER-visible constraints, FIS contract multiplexing, etc…).

This validation objective may be considered as being achieved.

## 33	TVO 10

To determine that the functionality described in the technical provisions is implementable.

The prototype developed by a State has shown that the METAR functionality is implementable (d, g).

This validation objective may be considered as being achieved.

## 34	TVO 11

To determine that independent implementations built in accordance with the technical provisions will be able to interoperate.

Based on engineering judgement, independent implementations will interoperate. (g).

# 35	Conclusions

It is concluded that the enhanced technical provisions will be sufficiently validated for inclusion in ICAO Doc. 9705.