ATNP WG 3

Flimsy 8-6 Rev 1

CORRECTIONS TO AIR/GROUND SARPS, VERSION 3.0

Presented by M J A Asbury

Attached to this flimsy are the Corrections to the WORD 6 Version 3.0 of the CM, ADS & FIS SARPs, formatted as Appendix C, D & F of the appropriate ATNP/2 WP.

Although it is standard practice for ICAO papers only to have the page numbers in the footers, the title has also been included here due to the similarity of the material for each application.

(Post Meeting Note - CPDLC corrections amount to a virtual amendment on each page - they are available as the Appendix to W8WP17.DOC.)

APPENDIX C:	   CORRECTIONS TO WORD VERSION 3.0 OF THE

CONTEXT MANAGEMENT  (CM) SARPS

CORRECTIONS TO WORD VERSION 3.0 OF THE CONTEXT MANAGEMENT  (CM) SARPS

The following corrections are proposed for the Word Version 3.0 of the Context Management  (CM) SARPs.

1.	Delete Section 2.1.1.1

- Redline

2.	Modified Note 2 c) 1) of Section 2.1.1.1

- Redline

The ground initiates this function with a contact request specifying which ground system to logon with..

3.	Modified Section 2.1.2.1.1

- Redline

The CM-air-ASE and CM-ground-ASE version numbers shall both be set to one.

4.	Modified Section 2.1.2.2.1

- Redline

In the event of information input by the CM-user being incompatible with that able to be processed by the system, the CM-user shall be notified.

5.	Modified Section 2.1.2.2.2

- Redline

In the event of a CM-user invoking a CM service primitive when the CM-ASE is not in a state specified in  2.1.5, the CM-user shall be notified.

6.	Modified Section 2.1.3.1.1

- Redline

An implementation of either the CM ground based service or the CM air based service shall exhibit external behaviour consistent with having implemented a CM-ground-ASE, or CM-air-ASE respectively.

7.	Modified Notes 1, 6 and 8 of Section 2.1.3.1.1

- Redline

Note 1.— 2.1.3 defines the abstract service interface for the CM service.  The CM-ASE abstract service is described from the viewpoint of the CM-air-user, the CM-ground-user and the CM-service-provider.

Note 6.— The CM-ASE is the element in the communication system which executes the CM specific protocol.  In other words, it takes care of the CM specific service primitive sequencing actions, message creation, timer management, error and exception handling.

Note 8.— The DS interface is the interface between the CM-ASE and the part of CM-CF underneath the CM-ASE, and provides the dialogue service as defined in 4.2.

8.	Deleted Section 2.1.3.2.1

9.	Renumber 2.1.3.2.2 to 2.1.3.2.1

10.	Modified new Section 2.1.3.2.1

- Redline

The CM-ASE abstract service shall consist of a set of the following services as allowed by the subsetting rules in  2.1.8:

- CM-logon service as defined in 2.1.3.3,

- CM-update service as defined in 2.1.3.4,

- CM-contact service as defined in 2.1.3.5,

- CM-end service as defined in 2.1.3.6,

- CM-forward service as defined in 2.1.3.7,

- CM-user-abort service as defined in 2.1.3.8, and

- CM-provider-abort service as defined in 2.1.3.9.

11.	Add Note 5 to 2.1.3.2.1

Note 5. — An abstract syntax is a syntactical description of a parameter which does not imply a specific implementation.  Only when the CM-ASE maps a parameter onto an APDU field, or vice versa, is the abstract syntax of the parameter described by using the ASN.1 of 2.1.4 for this field.

12.	Modification to Tables 2.1.3-1, 2.1.3-2

- Redline

Aircraft Address

13.	Modification to Section 2.1.3.3.4

### Redline

### Aircraft Address

### 14.	Modification to the Note of Section 2.1.3.3.4

#### Redline

Note.— This parameter contains 24-bit aircraft address of the aircraft initiating the CM-logon service.

15.	Modification to Section 2.1.3.3.4.1

- Redline

#### The Aircraft Addressparameter value shall conform to the abstract syntax 24-bit aircraft address.

16.	Renumber Section 2.1.3.3.5.2 to 2.1.3.3.5.3

17.	Renumber Section 2.1.3.3.5.1 to 2.1.3.3.5.2,

18.	Add new 2.1.3.3.5.1

#### 2.1.3.3.5.1  When provided by the CM-ASE, the Version Number parameter shall conform to an abstract integer value from 1 to 255.

19.	Modification to Note 2 of 2.1.3.3.5.3

- Redline

Note 2.— The CM-air-ASE and CM-ground-ASE version numbers are both set to 1.

20.	Modification to Table 2.1.3-3

- Redline

Aircraft Address

21.	Modification to 2.1.3.4.2

- Redline

### Aircraft Address

22.	Modification to Note of 2.1.3.4.2

- Redline

Note.— This parameter contains the addressed aircraft’s 24-bit aircraft address.

23.	Modification to 2.1.3.4.2.1

- Redline

#### The Aircraft Address parameter value shall conform to the abstract syntax 24-bit aircraft address.

24.	Modification to 2.1.3.4.2.2

- Redline

#### If a CM dialogue does not exist when a CM-ground user invokes the CM-update service request, the CM-ground-user shall provide the Aircraft Address parameter value.

25.	Modification to Table 2.1.3-4

- Redline

| Aircraft Address | C |  |  |  |
| --- | --- | --- | --- | --- |

26.	Modification to 2.1.3.5.2

- Redline

### Aircraft Address

27.	Modification to Note of 2.1.3.5.2

- Redline

Note.— This parameter contains the addressed aircraft’s 24-bit aircraft address.

28.	Modification to 2.1.3.5.2.1

- Redline

#### The Aircraft Address parameter value shall conform to the abstract syntax 24-bit aircraft address.

29.	Modification to 2.1.3.5.2.2

- Redline

#### If a CM dialogue does not exist when a CM-ground user invokes the CM-contact service request, the CM-ground-user shall provide the Aircraft Address parameter value.

30.	Renumber 2.1.3.7.5 to 2.1.3.7.4.1.

31.	Renumber 2.1.3.7.6 to 2.1.3.7.5.

32.	Renumber 2.1.3.7.6.1 to 2.1.3.7.5.1.

33.	Renumber 2.1.3.7.6.2 to 2.1.3.7.5.2.

34.	Renumber 2.1.3.7.7 to 2.1.3.7.6.

35.	Renumber 2.1.3.7.7.1 to 2.1.3.7.6.1.

36.	Renumber 2.1.3.7.8 to 2.1.3.7.7.

37.	Renumber 2.1.3.7.8.1 to 2.1.3.7.7.1.

38.	Renumber 2.1.3.7.9 to 2.1.3.7.8.

39.	Renumber 2.1.3.7.9.1 to 2.1.3.7.8.1.

40.	Renumber 2.1.3.7.5.2 to 2.1.3.7.5.3.

41.	Renumber 2.1.3.7.5.1 to 2.1.3.7.5.2.

42.	Add new 2.1.3.7.5.1

2.1.3.7.5.1  When provided by the CM-ASE, the Version Number parameter shall conform to an abstract integer value from 1 to 255.

43.	Modified Note 2 of 2.1.3.7.5.3

- Redline

Note 2.— The sending CM-ground-ASE and receiving CM-ground-ASE version numbers are both set to 1.

44.	Modified 2.1.3.7.8.1

- Redline

#### The Result parameter shall conform to the ASN.1 abstract syntax CMForwardResponse.

45.	Renumber 2.1.4.1 to 2.1.4.2

46.	Renumber 2.1.4.1.1 to 2.1.4.2.1

47.	Add new 2.1.4.1

## 2.1.4.1  Encoding/decoding Rules

48.	Add new 2.1.4.1.1

### 2.1.4.1.1  A CM-air-ASE shall be capable of encoding CMAircraftMessage APDUs and decoding CMGroundMessage APDUs.

49.	Add new 2.1.4.1.2

### 2.1.4.1.2  A CM-ground-ASE shall be capable of encoding and decoding CMGroundMessage APDUs and decoding CMAircraftMessage ADPUs.

50.	Modified 2.1.4.2.1

- Redline

### The abstract syntax of the CM protocol data units shall comply with the description contained in the ASN.1 module CMMessageSetVersion1 (conforming to ISO/IEC 8824-1), as defined in 2.1.4.

51.	Modified ASN.1 for ‘APName’ as defined in 2.1.4.2.1

- Redline

AEQualifier ::= INTEGER (0..255)

--	ATN AE-Qualifier Numeric Values are described in 4

AEQualifierVersion ::= SEQUENCE

{

aeQualifier		AEQualifier,

apVersion	VersionNumber

}

AEQualifierVersionAddress ::= SEQUENCE

{

aeQualifier			AEQualifier,

apVersion		VersionNumber,

apAddress		APAddress

}

CMAbortReason ::= ENUMERATED

{

timer-expired				(0),

undefined-error				(1),

invalid-PDU				(2),

not-permitted-PDU			(3),

dialogue-acceptance-not-permitted		(4),

dialogue-end-not-accepted			(5),

communication-service-error		(6),

communication-service-failure		(7)

}

CMContactRequest ::= SEQUENCE

{

icaoFacilityDesignation		ICAOFacilityDesignation,

address				LongTsap

}

CMContactResponse ::= Response

CMForwardRequest ::= CMLogonRequest

CMForwardResponse ::= ENUMERATED

{

success					(0),

incompatible-version			(1),

service-not-supported			(2)

}

CMLogonRequest ::= SEQUENCE

{

aircraftFlightIdentification		[0]	AircraftFlightIdentification,

cMLongTSAP			[1]	LongTsap,

groundInitiatedApplications	[2]	SEQUENCE SIZE (1..256) OF 	AEQualifierVersionAddress												OPTIONAL,

airOnlyInitiatedApplications	[3]	SEQUENCE SIZE (1..256) OF	AEQualifierVersion													OPTIONAL,

icaoFacilityDesignation		[4]	ICAOFacilityDesignation			OPTIONAL,

airportDeparture			[5]	Airport					OPTIONAL,

airportDestination		[6]	Airport					OPTIONAL,

dateTimeDepartureETD		[7]	DateTime				OPTIONAL

}

CMLogonResponse ::= SEQUENCE

{

airInitiatedApplications		[0]	SEQUENCE SIZE (1..256) OF	AEQualifierVersionAddress											OPTIONAL,

groundOnlyInitiatedApplications	[1]	SEQUENCE SIZE (1..256) OF	AEQualifierVersion												OPTIONAL

}

CMUpdate ::= CMLogonResponse

Date ::= SEQUENCE

{

year		Year,

month		Month,

day		Day

}

DateTime ::= SEQUENCE

{

date               Date,

time               Time

}

Day ::= INTEGER (1..31)

--unit = Day, Range (1..31), resolution = 1

ICAOFacilityDesignation ::= IA5String (SIZE(8))

LongTsap ::= SEQUENCE

{

rDP			OCTET STRING (SIZE(5)),

shortTsap		ShortTsap

}

Month ::= INTEGER (1..12)

--unit = 1 Month, Range (1..12), resolution = 1

Response ::= ENUMERATED

{

contactSuccess			(0),

contactNotSuccessful		(1)

}

ShortTsap ::= SEQUENCE

{

aRS		[0]	OCTET STRING (SIZE(3))		OPTIONAL,

-- the aRS contains the ICAO 24 bit aircraft address when the ShortTsap belongs to an aircraft

-- or a ground address when the Short Tsap belongs to a ground system

locSysTsel	[1]	OCTET STRING  (SIZE(9..11))

}

Time ::= SEQUENCE

{

hours			Timehours,

minutes			Timeminutes

}

Timehours ::= INTEGER (0..23)

-- units = hour, range (0..23), resolution = 1 hour

Timeminutes ::= INTEGER (0..59)

-- units = minute, range (0..59), resolution = 1 minute

VersionNumber ::= INTEGER (1..255)

--VersionNumber 0 is reserved for the Dialogue Service

Year ::= INTEGER (1996..2095)

-- unit = Year, Range (1996..2095), resolution = 1

END

52.	Add new Note 3 to 2.1.5.1.1

Note 3. — More than one CM-logon attempt may be made for a given CM-contact request.  The number of attempts may be determined by local procedures.

53.	Delete 2.1.5.1.2

54.	Modified Note 4 to 2.1.5.1.1

- Redline

Note 4.— Primitives are processed in the order in which they are received.  See  4.3.3.

55.	Add new Note to 2.1.5.2.2,Table 2.1.5-1

Note.— The receipt of a CM-user-abort request, D-ABORT indication, or D-P-ABORT indication are also timer stop events.

56.	Modified 2.1.5.3.1.2

- Redline

Upon receipt of a PDU, if no actions are described for the arrival of that PDU when a CM-ASE is in a specific state, then that PDU is considered not permitted and exception handling procedures as described in 2.1.5.4.4 shall apply.

57.	Add new 2.1.5.3.1.3

2.1.5.3.1.3  If a PDU is received that cannot be decoded, then that PDU is considered an invalid PDU and exception handling procedures as described in 2.1.5.4.3 shall apply.

58.	Add new 2.1.5.3.1.4

2.1.5.3.1.4  If a PDU is not received where one is expected, then that PDU is considered an invalid PDU and exception handling procedures as described in 2.1.5.4.3 shall apply.

59.	Renumber 2.1.5.3.2.3.1.1 to 2.1.5.3.2.3.1

60.	Modified 2.1.5.3.2.3.1

- Redline

Upon receipt of a D-START confirmation, if the CM-air-ASE is in the LOGON  state and if the APDU contained in the D-START User Data parameter is a [CMLogonResponse] APDU or if the D-START User Data parameter is not present but the D-START DS User Version Number parameter is present,  the CM-air-ASE shall:

61.	Modified 2.1.5.3.2.6.1.1 b) 2)

- Redline

the CM-logon Aircraft Address parameter value as the D-START Calling Peer ID parameter value,

62.	Modified 2.1.5.3.3.2.1.2 a) 1)

- Redline

the D-START Calling Peer ID parameter value as the CM-logon service Aircraft Address parameter value,

63.	Modified 2.1.5.3.3.2.1.3 a) 1)

- Redline

the D-START Calling Peer ID parameter value as the CM-logon service Aircraft Address parameter value, and

64.	Modified 2.1.5.3.3.2.1

- Redline

Upon receipt of a D-START indication, if the CM-ground-ASE is in the IDLE state and the APDU contained in the D-START User Data parameter is a [CMLogonRequest] APDU and the abstract value of the D-START Calling Peer ID parameter is a 24 bit Aircraft Address then:

65.	Modified 2.1.5.3.3.2.2

- Redline

Upon receipt of a D-START indication, if the receiving CM-ground-ASE is in the IDLE state and the APDU contained in the D-START User Data parameter is a [CMForwardRequest] APDU and the abstract value of the D-START Calling Peer ID parameter is an 8 character ICAO Facility Designator then:

66.	Modified 2.1.5.3.3.7.1 b) 1)

- Redline

the CM-update Aircraft Address parameter value as the D-START Called Peer ID parameter value,

67.	Modified 2.1.5.3.3.8.1 b) 1)

- Redline

the CM-contact Aircraft Address parameter value as the D-START Called Peer ID parameter value,

68.	Modified 2.1.5.3.3.10.1 b) 5)

- Redline

the CMGroundMessage APDU as the D-START User Data parameter value,

69.	Modified 2.1.5.4.2.1

- Redline

Recommendation. — If a CM-ASE has an unrecoverable system error, the CM-ASE should:

70.	Modified 2.1.5.4.3.1

- Redline

If the User Data parameter of a D-START indication or D-DATA indication does not contain a valid PDU as defined in 2.1.5.3.1.3 and 2.1.5.3.1.4, the CM-ASE shall:

71.	Modified 2.1.5.4.3.2

- Redline

If the User Data parameter of a D-START confirmation does not contain a valid PDU as defined in 2.1.5.3.1.3 and 2.1.5.3.1.4, the CM-ASE shall:

72.	Modified 2.1.5.4.4.1

- Redline

If the User Data parameter of a D-START indication or D-DATA indication is a valid PDU, but is not a permitted PDU as defined within 2.1.5.3.1.2, the CM-ASE shall:

73.	Modified 2.1.5.4.4.2

- Redline

If the User Data parameter of a D-START confirmation is a valid PDU, but is not a permitted PDU as defined within 2.1.5.3.1.2, the CM-ASE shall:

74.	Modified Table 2.1.5-2,

Table Heading  - ABORT Events,

In Row 2 -  D-ABORT Indication Originator is “provider” and,

In Row 3 -  D-ABORT Indication Originator is “user” and,

In Row 4 - D-P-ABORT indication, and

In Row 7 - Tend Expires

In these four cases change Column 8 (END)

- Redline

- stop timer tend - IDLE

75.	Modified Table 2.1.5-2,

Table heading DIALOGUE Service Events, in Row 1 - D-START Indication Version Number is greater than the CM-ground-ASE version number, ground-ground forwarding supported, and

Table heading Abort Events, in Row 3 - D-ABORT Indication Originator is “user”,

Change Column 9 (FORWARD)

- Redline

cannot occur

76.	Modified 2.1.6.1.1

- Redline

The CM application shall use PER encoding as defined in ISO/IEC 8825-2, using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in  2.1.4.

77.	Modified 2.1.6.2.1.1

- Redline

Where dialogue service primitives, that is D-START, D-DATA, D-END, D-ABORT, and D-P-ABORT are described as being invoked in  2.1.5, the CM-ground-ASE and the CM-air-ASE shall exhibit external behavior consistent with the dialogue service, as described in 4.2, having been implemented and its primitives invoked.

78.	Deleted 2.1.6.3 and text immediately following

79.	Renumber 2.1.7.1.1.4 to 2.1.7.1.1.5

80.	Renumber 2.1.7.1.1.3 to 2.1.7.1.1.4

81.	Add  new 2.1.7.1.1.3

2.1.7.1.1.3  Upon receipt of a CM-logon service confirmation, the CM-air-user shall create the actual TSAP for each ground application information contained in the Logon Response based on the IDP and long TSAP for each application as defined in  2.1.4.

82.	Renumber 2.1.7.1.2.2 to 2.1.7.1.2.3

83.	Add new 2.1.7.1.2.2

2.1.7.1.2.2  Upon receipt of a CM-update service indication, the CM-air-user shall create the actual TSAP for each ground application information contained in the Update Information based on the IDP and long TSAP for each application as defined in  2.1.4.

84.	Add new 2.1.7.2.1

2.1.7.2.1  General CM-Ground-User Requirements

85.	Modified Note 1 of 2.1.7.2.1.2

- Redline

Note 1.— When a CM-ground-user invokes the CM-update service, CM-contact service, or CM-forward service and requires a particular class of communication service, it will set the class of communication service parameter to be the class of communication it requires.

86.	Modified Note 5 of 2.1.7.2.1.2

- Redline

Note 5.— Upon the initiation a CM-update service request or CM-contact service request, or upon receipt of a CM-logon service indication the ASE invocation correlation is based on the 24-bit aircraft address in the Aircraft Address primitive of the respective CM service.

APPENDIX D:   CORRECTIONS TO VERSION 3.0 OF THE AUTOMATIC DEPENDENT SURVEILLANCE (ADS) SARPS

CORRECTIONS TO VERSION 3.0 OF THE AUTOMATIC DEPENDENT SURVEILLANCE (ADS) SARPS

The following corrections are proposed for Version 3.0 of the automatic dependent surveillance (ADS) SARPs.

1.	Modified Section 2.2.1.1 Note 2 (c)

- Redline

c)	It will be necessary for an implementation to provide information which is both accurate and timely in the ADS reports, however, quantification of the age and accuracy of the information is beyond the scope of 2.2.1.

2.	Modified Section 2.2.1.2.2.1

- Redline

2.2.1.2.2.1  In the event of information input by the ADS-user being incompatible with that able to be processed by the system, the user shall be notified.

3.	Modified Section 2.2.1.2.2.2

- Redline

2.2.1.2.2.2  In the event of an ADS-user invoking an ADS service primitive, when the ADS-ASE is not in a state specified in 2.2.1.5, the user shall be notified.

4.	Modified Section 2.2.1.3.1

- Redline

2.2.1.3.1.1

2.2.1.3.1.1	An implementation of either the ADS ground based service or the ADS air based service shall exhibit external behaviour consistent with having implemented an ADS-ground-ASE, or ADS-air-ASE respectively.

5.	Modified Section 2.2.1.3.2.1

- Redline

2.2.1.3.2.1

2.2.1.3.2.1	The ADS-ASE abstract service shall consist of a set of the following services as allowed by the subsetting rules defined in 2.2.1.8:

a)	ADS-demand-contract  service as defined in section 2.2.1.3.4;

b)	ADS-event-contract  service as defined in section 2.2.1.3.5;

c)	ADS-periodic-contract  service as defined in section 2.2.1.3.6;

d)	ADS-report  service as defined in section 2.2.1.3.7;

e)	ADS-cancel service as defined in section 2.2.1.3.8;

f)	ADS-cancel-all-contracts service as defined in section 2.2.1.3.9;

g)	ADS-emergency-report  service as defined in section 2.2.1.3.10;

h)	ADS-modify-emergency-contract service as defined in section 2.2.1.3.11;

i)	ADS-cancel-emergency service as defined in section 2.2.1.3.12;

j)	ADS-user-abort service as defined in section 2.2.1.3.13;

k)	ADS-provider-abort service as defined in section 2.2.1.3.14.

6.	Modified Section 2.2.1.3.2.1 Note 5

- Redline

Note 5.— An abstract syntax is syntactical description of a parameter which does not imply a specific implementation. Only when the ADS-ASE maps a parameter onto an APDU field, or vice -versa, is the abstract syntax of the parameter described by using the ASN.1 of 2.2.1.4 for this field.

7.	Modified Section 2.2.1.3.4.4.1 Note 1

- Redline

Note 1.— Class of communication service parameter values are detailed in 1.2.

8.	Modified Section 2.2.1.3.5.4.1 Note 1

- Redline

Note 1.— Class of communication service parameter values are detailed in 1.2.

9.	Modified Section 2.2.1.3.6.3.1 Note 1

- Redline

Note 1.— Class of communication service parameter values are detailed in 1.2.

10.	Modified Section 2.2.1.4

- Renumber 2.2.1.4.1 to 2.2.1.4.2

- Renumber 2.2.1.4.1.1 to 2.2.1.4.2.1

- Redline

2.2.1.4.1	Encoding/Decoding Rules

2.2.1.4.1.1	An ADS-air-ASE shall be capable of encoding [ADSAircraftPDUs] APDUs and decoding [ADSGroundPDUs] APDUs.

2.2.1.4.1.2	An ADS-ground-ASE shall be capable of encoding [ADSGroundPDUs] APDUs and decoding [ADSAidcraftPDUs] APDUs.

11.	Modified Section 2.2.1.4.2.1

- Redline

2.2.1.4.2.1  The abstract syntax of the air-ground ADS protocol data units shall comply with the description contained in the ASN.1 module ADSMessageSetVersion1 (conforming to ISO/IEC 8824), as defined in 2.2.1.4.

12.	Modified Section 2.2.1.4.2.1 Note 1

- Redline

Note 1.— Different states have different requirements with respect to the units of measurement of some of the fields in the PDUs. The units described below have been defined to meet these differing requirements. Conversion to the units is made in the transmitting application and conversion from the units is made at the receiving application. The new units have been defined to be midway between the minimum resolution required by the two units used by different states.

13.	Modified Section 2.2.1.4.2 Ground-generated and Aircraft-generated message components - Protocol Data Units -  AbortReason

- Redline

AbortReason  ::= ENUMERATED

{

communications-service-failure (0),

unrecoverable-system-error (1),

invalid-PDU (2),

sequence-error (3),

timer-expiry (4),

cannot-establish-contact (5),

undefined-error (6),

dialogue-end-not-accepted (7),

unexpected-PDU (8),

decoding-error (9),

...

}

14.	Modified Section 2.2.1.4.2 Ground-generated and Aircraft-generated message components - Protocol Data Units - Event Contract::=SEQUENCE

- Redline

EventContract ::= SEQUENCE

{

lateral-deviation-change			[0]	LateralChange		OPTIONAL,

vertical-rate-change			[1]	VerticalRateChange	OPTIONAL,

altitude-range				[2]	AltitudeRange		OPTIONAL,

way-point-change			[3]	NULL			OPTIONAL,

air-speed-change				[4]	AirSpeedChange		OPTIONAL,

ground-speed-change			[5]	GroundSpeedChange	OPTIONAL,

heading-change				[6]	DegreesDirection		OPTIONAL,

extended-projected-profile-change		[7]	ExtendedProjectedProfileRequest

OPTIONAL,

fom-change				[8]	NULL			OPTIONAL,

track-angle-change			[9]	DegreesDirection		OPTIONAL,

altitude-change				[10]	AltitudeChange		OPTIONAL,

...

}

15.	Modified Section 2.2.1.4.2 Ground-generated and Aircraft-generated message components - Protocol Data Units - PeriodicContract::=SEQUENCE

- Redline

PeriodicContract ::= SEQUENCE

{

reporting-interval			[0]	ReportingInterval

DEFAULT {minutes-scale 5},

airframe-id-modulus			[1]	Modulus			OPTIONAL,

projected-profile-modulus			[2]	Modulus			OPTIONAL,

ground-vector-modulus			[3]	Modulus			OPTIONAL,

air-vector-modulus			[4]	Modulus			OPTIONAL,

weather-modulus				[5]	Modulus			OPTIONAL,

short-term-intent-modulus				[6]	ShortTermIntentModulus		OPTIONAL,

extended-projected-profile-modulus	[7]	ExtendedProjectedProfileModulus

OPTIONAL,

...

}

16.	Modified Section 2.2.1.4.2 -- Reports and their components - AirVector::=SEQUENCE

- Redline

AirVector ::= SEQUENCE

{

heading					[0]	DegreesDirection 	OPTIONAL,

air-speed				[1]	AirSpeed 		OPTIONAL,

vertical-rate				[2]	VerticalRateChange	OPTIONAL

}

17.	Modified Section 2.2.1.4.2 -- Reports and their components - GroundVector ::= SEQUENCE

- Redline

GroundVector ::= SEQUENCE

{

track				[0]	DegreesDirection 		OPTIONAL,

ground-speed			[1]	INTEGER (-110..4200) 		OPTIONAL,

-- units = speed-units (see Note 1)

-- range = -110 to + 4200 speed-units

vertical-rate			[2]	VerticalRateChange 		OPTIONAL

}

18.	Modified Section 2.2.1.4.2 -- Reports and their components - IntermediateIntent ::= SEQUENCE SIZE (0..7) OF SEQUENCE

- Redline

IntermediateIntent ::= SEQUENCE SIZE (0..7) OF SEQUENCE

{

distance					INTEGER (1..8000),

-- units = Nautical miles

-- range = 1 to 8000 nautical miles

track					DegreesDirection,

altitude					Altitude,

projected-time				ProjectionTime

}

-- IntermediateIntent indicates a set of way points between the current position and the

-- time indicated in the ShortTerm intent element.

19.	Modified Section 2.2.1.4.2 -- Reports and their components - Weather ::= SEQUENCE

- Redline

Weather ::= SEQUENCE

{

wind-speed			[0]	INTEGER (0..520) OPTIONAL,

-- units = speed-units (see Note 1)

-- range = 0 to 520 speed-units

wind-direction			[1]	INTEGER (1..360) OPTIONAL,

-- units = degrees true North

-- range= 1 to 360 degrees

temperature			[2]	INTEGER (-400..400) OPTIONAL,

-- units = 0.25 degrees Centigrate

-- range = -100 to 100 degrees C

turbulence			[3]	INTEGER (0..15) OPTIONAL

-- this is a place marker for a turbulence

-- index which is to be defined

}

20.	Modified Section 2.2.1.4.2 -- Components of Contracts - ShortTermIntentModulus ::= SEQUENCE

- Redline

ShortTermIntentModulus ::= SEQUENCE

{

intent-modulus				Modulus,

intent-projection-time			ProjectionTime

}

21.	Modified Section 2.2.1.4.2 -- Common components - Latitude ::= SEQUENCE

- Redline

Latitude ::= SEQUENCE

{

sign					Sign

degrees					INTEGER (0..90),

-- units = degrees

-- range = 0 degrees to 90 degrees

minutes					INTEGER (0..59),

-- units = minutes

-- range = 0 minutes to 59 minutes

tenth-seconds				INTEGER (0..599)

-- units = 0.1 seconds

-- range = 0 seconds to 59.9 seconds

}

22.	Modified Section 2.2.1.4.2 -- Common components - Longitude ::= SEQUENCE

- Redline

Longitude ::= SEQUENCE

{

sign					Sign

degrees					INTEGER (0..180),

-- units = degrees

-- range = 0 degrees to 180 degrees

minutes					INTEGER (0..59),

-- units = minutes

-- range = 0 minutes to 59 minutes

tenth-seconds				INTEGER (0..599)

-- units = 0.1 seconds

-- range = 0 seconds to 59.9 seconds

}

23.	Modified Section 2.2.1.5.1.1

- Redline

2.2.1.5.1.1  Only the sequence of primitives illustrated in figures 2.2.1.5-1 to 2.2.1.5-35 shall be permitted.

24.	Modified Section 2.2.1.5.1.1 Note 3

- Redline

Note 3.—Primitives are processed in the order in which they are received (see 4.3.3.1.2.4).

2.2.1.5.1.2

25.	Modified Section 2.2.1.5.1.1, Figure 2.2.1.5-28 - Changed t-EM-4 to t-EM-3

- Redline

Figure 2.2.1.5-28: Crossed air emergency cancellation and cancel all contracts

26.	Modified Section 2.2.1.5.1.1, Renumber Figure 2.2.1.5-29 to 2.2.1.5.31

- Redline

Figure 2.2.1.5-31: Air user abort service

27.	Modified Section 2.2.1.5.1.1, Renumber Figure 2.2.1.5-30 to 2.2.1.5-32

- Redline

Figure 2.2.1.5-32: Ground user abort service

28.	Modified Section 2.2.1.5.1.1, Renumber Figure 2.2.1.5-31 to 2.2.1.5-33

- Redline

Figure 2.2.1.5-33: Dialogue service provider abort service

29.	Modified Section 2.2.1.5.1.1, Renumber Figure 2.2.1.5-32 to 2.2.1.5-34

- Redline

Figure 2.2.1.5-34: Ground ASE abort

30.	Modified Section 2.2.1.5.1.1, Renumber Figure 2.2.1.5-33 to 2.2.1.5-35

- Redline

Figure 2.2.1.5-35: Air ASE abort

31.	Modified Section 2.2.1.5.1.1, Add Figure 2.2.1.5-29

- Redline

Figure 2.2.1.5-29: Crossed air emergency cancellation and modification of emergency contract with other contracts in place

32.	Modified Section 2.2.1.5.1.1, Add Figure 2.2.1.5-30

- Redline

Figure 2.2.1.5-30: Crossed air emergency cancellation and modification of emergency contract with no other contracts in place

33.	Modified Section 2.2.1.5.2.2 Table 2.2.1.5-1

- Redline

Table 2.2.1.5-1:  ADS Service Provider Timers

| ADS Service | Timer | Timer Value | Timer Start Event | Timer Stop Event |
| --- | --- | --- | --- | --- |
| ADS-demand-contract | t-DC-1 | 6 minutes | ADS-demand-contract request | ADS-demand-contract confirmation or ADS-report indication |
|  | t-DC-2 | 3 minutes 30 seconds | ADS-demand-contract confirmation | ADS-report indication |
| ADS-event-contract | t-EC-1 | 6 minutes | ADS-event-contract request | ADS-event-contract confirmation or ADS-report indication |
|  | t-EC-2 | 6 minutes | ADS-cancel request | ADS-cancel-contract confirmation |
| ADS-periodic-contract | t-PC-1 | 6 minutes | ADS-periodic-contract request | ADS-periodic-contract confirmation or ADS-report indication |
|  | t-PC-2 | reporting rate + 3 minutes | ADS-report indication or ADS-periodic-contract confirmation | ADS-report indication |
|  | t-PC-3 | 6 minutes | ADS-cancel request | ADS-cancel-contract confirmation |
| ADS emergency contract | t-EM-1 | reporting rate + 3 minutes | ADS-emergency-report indication | ADS-emergency-report indication |
|  | t-EM-2 | 6 minutes | ADS-modify-emergency-contract request | ADS-modify-emergency-contract request or ADS-emergency-report indication |
|  | t-EM-3 | 6 minutes | ADS-cancel-emergency request | Arrival of ADS-cancel-emergency PDU |
| General | t-LI-1 | 6 minutes | D-END request | D-END confirmation |

Note.— The receipt of ADS-user-abort request, D-ABORT indication or D-P-ABORT indication are also timer stop events.

34.	Modified Section 2.2.1.5.3

- Redline

2.2.1.5.3.3	Possible Errors Arising upon Receipt of an APDU

2.2.1.5.3.3.1	If an APDU is not received when one is required, or one is received in an inappropriate dialogue service primitive, then exception handling procedures as described in 2.2.1.5.4.3 shall apply.

2.2.1.5.3.3.2	Upon receipt of an APDU, if no actions are described for the arrival of that APDU when in a particular state, then exception handling procedures as described in 2.2.1.5.4.4 shall apply.

2.2.1.5.3.3.3	Upon receipt of an APDU but that can not be decoded, then exception handling procedures as described in 2.2.1.5.4.7 shall apply.

35.	Modified Section 2.2.1.5.3.16.3.1

- Redline

a)	Invoke D-END req with ADS-cancel-all-contracts-PDU in the user data,

b)	start the t-LI-1 timer, and

c)	enter the LI-G-END state.

36.	Modified Section 2.2.1.5.3.16.4.1

- Redline

a)	invoke D-DATA req with PDU in the user data,

b)	if the DC module is in the DC-G-IDLE state, and the EC module is in the EC-G-IDLE state, and the PC module is in the PC-G-IDLE state, and the ground EM module is in the EM-G-IDLE state, then:

1)	invoke D-END req with no user data,

2)	start the t-LI-1 timer, and

3)	enter the LI-G-END state; or

c)	otherwise, remain in the LI-G-ACTIVE state.

37.	Modified Section 2.2.1.5.3.16.5.1

- Redline

a)	Invoke D-ABORT request with the parameter values supplied,

b)	stop any timers, and

c)	enter the LI-G-IDLE state.

38.	Modified Section 2.2.1.5.3.16.8.1

- Redline

a)	stop the t-LI-1 timer,

b)	if the user data parameter value contains an ADS-positive-acknowledgment-PDU (cancel-all-contracts), then request the ground HI module to invoke ADS-cancel-all-contracts confirmation,

c)	request the DC, EC, PC and EM modules to stop operation, and

d)	enter the LI-G-IDLE state.

39.	Modified Section 2.2.1.5.3.16.11

- Redline

Upon expiry of the t-LI-1 timer, the ground LI module shall:

a)	request the air AB module to abort with reason timer-expiry, and

b)	remain in the same state.

40.	Modified Section 2.2.1.5.4.2.1

- Redline

2.2.1.5.4.2.1  Recommendation—. When any module has an unrecoverable system error, the module should request the air or ground AB module to abort with reason unrecoverable-system-error.

41.	Modified Section 2.2.1.5.4.3.1

- Redline

2.2.1.5.4.3.1  When the user data parameter value of a D-START indication is a valid APDU and is not an ADS-demand-contract-PDU, an ADS-event-contract-PDU or an ADS-periodic-contract-PDU, the air LI module shall request the AB module to abort with reason invalid-PDU.

42.	Modified Section 2.2.1.5.4.3.2

- Redline

2.2.1.5.4.3.2  When the user data parameter value of a D-START confirmation is a valid APDU and is not an ADS-demand-report-PDU, an ADS-event-report-PDU, an ADS-periodic-report-PDU, an ADS-positive-acknowledgment-PDU, and ADS-negative-acknowledgement-PDU or an ADS-noncompliance-notification-PDU the ground LI module shall request the AB module to abort with reason invalid-PDU.

43.	Modified Section 2.2.1.5.4.3.3

- Redline

2.2.1.5.4.3.3  When the user data parameter value of a D-DATA indication is a valid APDU and is not an ADS-demand-contract-PDU, an ADS-event-contract-PDU, an ADS-periodic-contract-PDU, an ADS-modify-emergency-contract-PDU, an ADS-cancel-emergency-acknowledgment-PDU, an ADS-cancel-all-contracts-PDU or an ADS-cancel-contract-PDU, the air LI module shall request the AB module to abort with reason invalid-PDU.

44.	Modified Section 2.2.1.5.4.3.4

- Redline

2.2.1.5.4.3.4  When the user data parameter value of a D-DATA indication is a valid APDU and is not an ADS-demand-report-PDU, an ADS-event-report-PDU, an ADS-periodic-report-PDU, an ADS-positive-acknowledgment-PDU, and ADS-negative-acknowledgement-PDU, an ADS-noncompliance-notification-PDU, an ADS-emergency-report-PDU or an ADS-cancel-emergency-PDU the ground LI module shall request the AB module to abort with reason invalid-PDU.

45.	Modified Section 2.2.1.5.4.3.6

- Redline

2.2.1.5.4.3.6  When the user data parameter value of a D-ABORT indication is a valid APDU and is not ADS-provider-abort-PDU, the air LI module or the ground LI module shall request the AB module to abort with reason invalid-PDU.

46.	Modified Section 2.2.1.5.4.7

- Redline

2.2.1.5.4.7	Decoding Error

2.2.1.5.4.7.1	When the air LI module or the ground LI module fails to decode an APDU, the LI module shall request the AB module to abort with reason decoding-error.

47.	Modified Section 2.2.1.6.1.1

- Redline

2.2.1.6.1.1  The ADS application shall use PER as defined in ISO/IEC 8825-2, using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in 2.2.1.4.

48.	Modified Section 2.2.1.6.2.1.1

- Redline

2.2.1.6.2.1.1  Where dialogue service primitives, that is D-START, D-END, D-ABORT, D-P-ABORT and D-DATA are described as being invoked in 2.2.1.5, the ADS-ground-ASE and the ADS-air-ASE shall exhibit external behaviour consistent with the dialogue service, as described in 4.2, having been implemented and its primitives invoked.

49.	Modified Section 2.2.1.6.2.2.3

- Redline

Note. — ATSC values are defined in 1.2.

50.	Deleted Sections 2.2.1.6.3, 2.2.1.6.3.1

2.2.1.6.3

2.2.1.6.3.1

51.	Modified Section 2. 2.1.7.4 Note 1

- Redline

Note 1.— 2.2.1.7.4 details the actions taken by the ADS-ground-user and the ADS-air-user in the establishment and operation of a periodic contract while no emergency contract exists..

52.	Modified Section 2.2.1.7.4.2.2.1

- Redline

a)	invoke ADS-periodic-contract response with the reply parameter set to positive acknowledgement, and

b)	if no emergency contract exists, send the first ADS-report request of the contract.

53.	Modified Section 2.2.1.7.4.2.2.2

- Redline

a)	invoke ADS-periodic-contract response with the reply parameter set to positive acknowledgement within 0.5 seconds, and

b)	if no emergency contract exists, send the first ADS-report request of the contract within 30 seconds from the receipt of the ADS-periodic-contract request.

54.	Modified Section 2.2.1.7.4.4.4 Note

- Redline

Note.— If an emergency contract is already in place, the periodic contract will be immediately suspended due to the provisions stated in 2.2.1.7.

55.	Modified Section 2.2.1.8.1.1 Table 2.2.1.8-2

- Redline

Table 2.2.1.8-2. ADS Protocol Options

|  | Status | Associated Predicate |
| --- | --- | --- |
| ADS-air-ASE | C.1 | ADS/air |
| ADS-ground-ASE | C.1 | ADS/ground |
| Demand contract supported | if (ADS/air) M, else C.2 | DC-FU |
| Event contract supported | if (ADS/air) M, else C.2 | EC-FU |
| Periodic contract supported | if (ADS/air) M, else C.2 | PC-FU |
| Emergency contract supported | if (EC-FU or PC-FU) M, else N/A | EM-FU |

C.1:  A conforming implementation will support one and only one of these two options.

C.2:  Optional; a conforming implementation will support at least one of the three options.

56.	Modified Section 2.2.2.2.2.1

- Redline

2.2.2.2.2.1  In the event of information input by the ARF-user being incompatible with that able to be processed by the system, the ARF-user shall be notified.

57.	Modified Section 2.2.2.2.2.2

- Redline

2.2.2.2.2.2  In the event of an ARF-user invoking an ADS Report Forwarding service primitive, when the ADS-RF-ASE is not in a state specified in 2.2.2.5, the ARF-user shall be notified.

58.	Modified Section 2.2.2.3.1.1

- Redline

2.2.2.3.1.1

2.2.2.3.1.1	An implementation of the ADS Report Forwarding service shall exhibit external behavior consistent with having implemented an ADS-RF-ASE.

59.	Delete Section 2.2.2.3.2.1, Renumber 2.2.2.3.2.2 to 2.2.2.3.2.1

- Redline

2.2.2.3.2.1	The ADS-RF-ASE abstract service shall consist of a set of the following services as allowed by the subsetting rules defined in 2.2.2.8:

- ADS-start-forward service as defined in 2.2.2.3.4;

- ADS-forward-report service as defined in 2.2.2.3.5;

- ADS-end-forward service as defined in 2.2.2.3.6;

- ADS- user-abort service as defined in 2.2.2.3.7;

- ADS-provider-abort service as defined in 2.2.2.3.8.

Note.— An abstract syntax is syntactical description of a parameter which does not imply a specific implementation. Only when the ADS-RF-ASE maps a parameter onto an APDU field, or vice -versa, is the abstract syntax of the parameter described by using the ASN.1 of 2.2.2.4 for this field.

60.	Modified Section 2.2.2.3.4.4.1 Note 1

- Redline

Note 1.— Class of communication service parameter values are detailed in 1.2.

61.	Renumber 2.2.2.4.1 to 2.2.2.4.2, 2.2.2.4.1.1 to 2.2.2.4.2.1

62.	Add 2.2.2.4.1, 2.2.2.4.1.1

- Redline

2.2.2.4.1	Encoding/Decoding Rules

2.2.2.4.1.1	An ADS-RF-ASE shall be capable of encoding and decoding [ADSRFPDUs] APDUs.

63.	Modified Section 2.2.2.4.2.1

- Redline

2.2.2.4.2.1	The abstract syntax of the ADS-RF protocol data units shall comply with the description contained in the ASN.1 module ADSRFMessageSetVersion1 (conforming to ISO/IEC 8824), as defined in 2.2.2.4.

64.	Modified Section 2.2.2.5.1.1 Note 3

- Redline

Note 3.—Primitives are processed in the order in which they are received (see 4.3.3.1.2.4).

65.	Modified Section 2.2.2.5.1.2

- Redline

2.2.2.5.1.2

66.	Modified Section 2.2.2.5.3.1

- Renumber 2.2.2.5.3.1.1 to 2.2.2.5.3.1.4

- Renumber 2.2.2.5.3.1.2 to 2.2.2.5.3.1.5

- Renumber 2.2.2.5.3.1.2.1 to 2.2.2.5.3.1.5.1

- Renumber 2.2.2.5.3.1.3 to 2.2.2.5.3.1.6

- Renumber 2.2.2.5.3.1.3.1 to 2.2.2.5.3.1.6.1

- Renumber 2.2.2.5.3.1.4 to 2.2.2.5.3.1.7

- Renumber 2.2.2.5.3.1.4.1 to 2.2.2.5.3.1.7.1

- Renumber 2.2.2.5.3.1.5 to 2.2.2.5.3.1.8

- Renumber 2.2.2.5.3.1.5.1 to 2.2.2.5.3.1.8.1

- Renumber 2.2.2.5.3.1.6 to 2.2.2.5.3.1.9

- Renumber 2.2.2.5.3.1.6.1 to 2.2.2.5.3.1.9.1

- Renumber 2.2.2.5.3.1.7 to 2.2.2.5.3.1.10.1

- Renumber 2.2.2.5.3.1.8 to 2.2.2.5.3.1.11.1

- Renumber 2.2.2.5.3.1.9 to 2.2.2.5.3.1.12.1

- Renumber 2.2.2.5.3.1.10 to 2.2.2.5.3.1.13

- Renumber 2.2.2.5.3.1.10.1 to 2.2.2.5.3.1.13.1

- Renumber 2.2.2.5.3.1.11 to 2.2.2.5.3.1.14

- Renumber 2.2.2.5.3.1.11.1 to 2.2.2.5.3.1.14.1

- Renumber 2.2.2.5.3.1.12 to 2.2.2.5.3.1.15

- Renumber 2.2.2.5.3.1.12.1 to 2.2.2.5.3.1.15.1

- Renumber 2.2.2.5.3.1.12.2 to 2.2.2.5.3.1.15.2

- Redline

2.2.2.5.3.1	If an APDU is not received when one is required, or one is received in an inappropriate dialogue service primitive, then exception handling procedures as described in 2.2.2.5.4.3 shall apply.

2.2.2.5.3.2	Upon receipt of an APDU, if no actions are described for the arrival of that APDU when in a particular state, then exception handling procedures as described in 2.2.2.5.4.4 shall apply.

2.2.2.5.3.3	Upon receipt of an APDU but that can not be decoded, then exception handling procedures as described in 2.2.2.5.4.6 shall apply.

67.	Modified Section 2.2.2.5.4.2.1

- Redline

2.2.2.5.4.2.1	Recommendation—. When the ADS-RF-ASE has an unrecoverable system error, it should:

68.	Modified Section 2.2.2.5.4.3.1

- Redline

2.2.2.5.4.3.1	When the user data parameter value of a D-START indication is a valid APDU and is not an aDS-forwarded-report-PDU,  or the user data parameter value of a D-START confirmation is present, or the user data parameter value of a D-DATA indication is not an aDS-forwarded-report-PDU, or the user data parameter of a D-END indication is present, or the user data parameter of a D-END confirmation is present, the ADS-RF-ASE shall:

69.	Modified Section 2.2.2.5.4.6

- Redline

2.2.2.5.4.6	Decoding Error

When the ADS-RF-ASE fails to decode an APDU, it shall:

a)	invoke D-ABORT with Originator parameter value DS user and user data parameter value aDS-provider-abort-PDU with value decoding-error,

b)	invoke ADS-provider-abort indication with reason decoding-error, and

c)	enter the RF-I-IDLE state.

70.	Modified Section 2.2.2.6.1.1

- Redline

2.2.2.6.1.1	The ADS application shall use PER as defined in ISO/IEC 8825-2, using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in 2.2.2.4.

71.	Modified Section 2.2.2.6.2.1.1

- Redline

2.2.2.6.2.1.1	Where dialogue service primitives, that is D-START, D-END, D-ABORT, D-P-ABORT and D-DATA are described as being invoked in 2.2.2.5, the ADS-ground-ASE and the ADS-air-ASE shall exhibit external behavior consistent with the dialogue service, as described in 4.2, having been implemented and its primitives invoked.

72.	Modified Section 2.2.2.6.2.2.3 Note

- Redline

Note. — ATSC values are defined in 1.2.

73.	Deleted Sections 2.2.2.6.3, 2.2.2.6.3.1

- Redline

APPENDIX F:   CORRECTIONS TO WORD VERSION 3.0 OF THE DATA LINK FLIGHT INFORMATION SERVICE (DFIS) SARPs

CORRECTIONS TO WORD VERSION 3.0 OF THE DATA LINK FLIGHT INFORMATION SERVICE (DFIS) STANDARDS AND RECOMMENDED PROCEDURES (SARPs)

The following corrections are proposed for the Word Version 3.0 of the FIS SARPs

1.	Added Note to  Section 2.4.1.1.2

·    Redline

Note.— The concept of FIS Demand Contract and the FIS Update Contract used in 2.4  is equivalent to the concept of FIS Demand Mode and FIS Contract Mode developed in the draft ICAO Manual of Air Traffic Services (ATS) Data Link Applications.

2.	Modified Section 2.4.2.2.1

- Redline

In the event of information input by the FIS-user being incompatible with that able to be processed by the system, the FIS-user shall be notified.

3.	Modified Section 2.4.2.2.2

- Redline

In the event of a FIS-user invoking a FIS service primitive when the FIS-ASE is not in a state specified in 2.4.5, the FIS-user shall be notified.

4.	Modified Section 2.4.3.1.1

- Redline

An implementation of either the FIS ground based service or the FIS air based service shall exhibit external behaviour consistent with having implemented a FIS-ground-ASE, or a FIS-air-ASE respectively.

5.	Modified Note 8 of 2.4.3.1.1

- Redline

The DS interface is the interface between the FIS-ASE and part of FIS-CF underneath the FIS-ASE, and provides the dialogue service as described in 4.2

6.	Deleted Section 2.4.3.2.1

- Redline

7.	Added Note to  2.4.3.2.2

·   Redline

Note 5. — An abstract syntax is a syntactical description of a parameter which does not imply a specific implementation. Only when the FIS- ASE maps a parameter onto an APDU field, or vice-versa, is the abstract syntax of the parameter described by using the ASN.1 of 2.4.4  for this field.

8.	Renumber Section 2.4.4.1 to 2.4.4.2

9.	Renumber Section 2.4.4.1.1 to 2.4.4.2.1

10.	Added Section 2.4.4.1

- Redline

2.4.4.1  Encoding/Decoding Rules

11.	Added Section 2.4.4.1.1

- Redline

2.4.4.1.1  A FIS-air-ASE shall be capable of encoding [FISDownlinkAPDU] APDUs and decoding [FISUplinkAPDU] APDUs.

12.	Added Section 2.4.4.1.2

- Redline

2.4.4.1.2  A FIS-ground-ASE shall be capable of encoding [FISUplinkAPDU] APDUs and decoding [FISDownlinkAPDU] APDUs.

13.	Modified Section 2.4.4.2.1

- Redline

The abstract syntax of the FIS protocol data units shall comply with the description contained in the ASN.1 module FISMessageSetVersion1 (conforming to ISO/IEC 8824-1]), as defined in 2.4.4.

14.	Deleted Section 2.4.5.1.2

- Redline

15.	Modified Note of  Section 2.4.5.1.1

- Redline

Note 3.— Primitives are processed in the order in which they are received (see 4.3.3).

16.	Modified Note of  2.4.5.2.2

- Redline

Note 1. —The t-inactivity timer value is set on configuration basis.

17.	Added Note 2 to 2.4.5.2.2

- Redline

Note 2.— The receipt of FIS-user-abort request, D-ABORT indication and D-P-ABORT indication are also timer stop event.

18.	Renumber 2.4.5.3.3 to 2.4.5.3.3.1

19.	Add Section 2.4.5.3.3

2.4.5.3.3  Possible Errors Arising from Receipt of an APDU

20.	Modified Section 2.4.5.3.3.1

- Redline

Upon receipt of an APDU, if no actions are described for the arrival of that APDU when in a particular state, then exception handling procedures as described in 2.4.5.4.3 shall apply.

21.	Added Section 2.4.5.3.3.2

- Redline

2.4.5.3.3.2  If an APDU is not received when one is required, then exception handling procedures as described in 2.4.5.4.2 shall apply.

22.	Added Section 2.4.5.3.3.3

- Redline

2.4.5.3.3.3 Upon receipt of an APDU but that cannot be decoded, then exception handling procedures as described in 2.4.5.4.4 shall apply.

23.	Modified Section 2.4.5.4.1.1

- Redline

Recommendation. — When any module has an unrecoverable system error, the module should:

- request the AB module to abort with reason "unrecoverable internal error", and

- enter its IDLE state.

24.	Modified Section 2.4.6.1.1

- Redline

2.4.6.1.1  The FIS application shall use PER as defined in ISO/IEC 8825-2, using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in 2.4.4.

25.	Modified Section 2.4.6.2.1.1

- Redline

2.4.6.2.1.1  Where dialogue service primitives, that is D-START, D-END, D-ABORT, D-P-ABORT, and D-DATA are described as being invoked in 2.4.5, the FIS-ground-ASE and the FIS-air-ASE shall exhibit external behavior consistent with the dialogue service, as described in 4.2, having been implemented and its primitives invoked.

26.	Modified  Section 2.4.6.2.2.1

- Redline

2.4.6.2.2.1  The application service priority for  ATIS shall have the abstract value of "Aeronautical Information Service messages".

27.	Deleted Section 2.4.6.3

- Redline

28.	Deleted Section 2.4.6.3.1

- Redline