ATNP/WG3

Gold Coast, Australia

13 February 1996

Flimsy 8

Change Request for Sub-volume V

1.  5.1.2   Internet Service Description

Note 1 -

a) stet

b) whether or not the expedited data option is required (it is not invoked in CNS/ATM-1);

c) the required residual error rate (RER) to determine whether or not the transport checksum is required non-use of the transport checksum is allowed (checksum is always selected in CNS/ATM-1).

DR Note -- CNS/ATM-1 applications require the transport checksum plus additional lower-layer integrity measures.

2.1  CNS/ATM-1 Security Label

6.2.2  The Security Function

6.2.2.1  Encoding of the Security Parameter

It may also be used to convey a security classification.

The value component of the CLNP Options Security Parameter shall be encoded as follows:

1.  The first octet shall always be encoded as [1100 0000] to indicate the Globally Unique Security Format [1000 0000] to indicate the Source Address Specific Security Format.

2.  The second octet shall contain the Security Tag.

Delete from 6.2.2.1 2. to 6.2.2.5.6 up to and including the Note.  Delete 6.2.2.5.8

Advantage: Minimal Encoding  15 -> 4 octets).

Disadvantage:  No extensibility to real security schema.

2.2   Extensible Security Label

6.2.2  The Security Function

6.2.2.1  Encoding of the Security Parameter

The value component of the CLNP Options Security Parameter shall be encoded as follows:

1.  The first octet shall always be encoded as [1100 0000] to indicate the Globally Unique Security Format [1000 0000] to indicate the Source Address Specific Security Format.

2.  The remaining octets shall containe the ATN security label encoded as the four two fields illustrated in Figure 6-1, and defined below.

Delete from 6.2.2.1 Note through 6.2.2.3. Delete the first two fields in Figure 6-1.

Advantage: Extensibility to real security schema.

Disadvantage:  More Encoding  15 -> 6 octets).