ATNP/WG3/SG1

WP/186

jj/03/98

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL(ATNP)

Working Group 3 - Applications and Upper Layers

SUBGROUP 1 - GROUND Applications SUBGROUP

Orlando, 11-12 March 1998 (thirteenth meeting)

Agenda Item 3 : ATS Message Handling Services

WP/186 : Analysis of Threats against the ATS Message Service

Presented by Jean-Marc Vacher (France)

Summary

The twelth WG3/SG1 meeting (Orlando, 14-16 January 1998) concluded that the existing analysis of risks concerning the AMHS was too general to allow the definition of an appropriate AMHS security approach.

This risk analysis was extracted from the ATN Security concept, elaborated by Eurocontrol, presented at ATNP/2 and distributed again to ATNP/WG3/SG1 as WP/164 in the eleventh WG3/SG1 meeting to serve as a working basis on the subject.

The goal of this paper is therefore to refine this risk analysis by an assessment of the impact of each type of attack, depending on the regarded message category.

This paper is a contribution to the production of sub-deliverable D111 ("Analysis of risks and threats against the ATS Message Service ") included in the SG1 Work Programme.

Table of contents

1. Introduction

2. Summary of identified threats and possible countermeasures

2.1 Attacks (general)

2.2 Security measures (general)

2.3 MHS-dedicated analysis

2.3.1 Impact

2.3.2 Attacks

3. AMHS Security Approach

3.1 Goal of the section

3.2 Scope of AMHS security

3.3 AMHS Security functions

3.4 Selection of a Security Class within the MHS ISPs

3.4.1 Description of security classes

3.4.2 Suitability of security classes for the AMHS

3.5 Definition of high level AMHS security functions

4. Recommendation

5. Appendix A : description of the referred MHS security EoS

References

[1]	ATN Overall Security Concept, Appendix to ATNP/2 WP/25 (previously elaborated by Eurocontrol under the aegis of ATNP/WG1)

[2]	ATNP/WG3/SG1 WP/164, Analysis of threats against the ATS Message Service - Jean-Marc Vacher, 11th SG1 meeting, Toulouse, 16-19 September 1997

[3]	ATNP/WG3/SG1 WP/155, SG1 Future Work Programme - Jean-Yves Piram, 11th SG1 meeting, Toulouse, 16-19 September 1997

[4]	ATNP/WG3/SG1 WP/176, Security Approach for the ATS Message Service - Jean-Marc Vacher, 12th SG1 meeting, Orlando, 14-16 January 1998

[5]	Draft report of the 12th ATNP/WG3/SG1 meeting (Orlando, 14-16 January 1998)

[6]	ICAO Annex 10, Vol. II

# Introduction

A first analysis of threats against the ATS Message Service has been provided to the ATNP/WG3/SG1 in the course of its 11th meeting, by means of WP/164 [2]. This was based on an earlier paper developed by Eurocontrol and named "ATN overall security framework" [1]. The 11th SG1 meeting noted the information provided in the paper and agreed to use it as a starting point for further work on AMHS security.

However, during the presentation of a WP proposing an AMHS security approach, in the 12th SG1 meeting, it became apparent that the risk analysis was not detailed enough to allow the definition of a security approach offering an appropriate protection against the identified risks. The meeting finally agreed that a more in-depth study should be undertaken to refine the threat analysis in relation with the nature of the exchanged messages.

The goal of this paper is to provide the subgroup with such a detailed threat analysis.

With respect to the agreed SG1 Work Programme [3], this paper is a contribution to the production of sub-deliverable D111 ("Analysis of risks and threats against the ATS Message Service") included in the Work Programme.

# Evaluation of identified threats

## Types of attacks

As stated in [1], Appendix D, section 1, the identified types of attack are listed in Table 1.

Table 1: Types of potential attacks against the AMHS

| Type of attack | Definition |
| --- | --- |
| Modifications | Modification to a message in transit. |
| Replay | Recording a valid message and playing it back at a later time. |
| Jamming | Insertion of noise or such disturbance on the transmission line to prevent the passage of messages. |
| Masquerade | Creating false messages (while impersonating an actual user). |
| Flooding | Sending so many messages that the correct messages are unable to get through. |
| Other DoS | Any other Denial of Service attack (such as physical damage to equipment). |

These "attacks" may be intentional or may in some cases result from technical malfunctions. However, the baseline is mostly that of an intentional attack.

## Risk

In the AMHS as it is currently specified, the threat associated with each type of attack may be classified with:

- a level of risk, which is defined here as the probability of occurrence of such an attack, and which is related to the "ease of attack";

- an impact, which is defined as the level of consequence of such an attack.

The level of risk does not depend at present upon the type of message, since all categories of messages are equally handled in the AMHS. The level of risk is therefore the same as the one evaluated in [1], which is recalled in Table 2.

Table 2 - Attacks to the MHS messaging system as a whole

|  | Air Links | Ground Links | Routers | Other Hosts/Users | Sys Man't & IDRP | MHS store/fwd |
| --- | --- | --- | --- | --- | --- | --- |
| Modifications |  | L | M |  |  | M |
| Replay |  | M | M |  | H | M |
| Jamming |  | M | M |  |  |  |
| Masquerade |  | M | M | H | H | M |
| Flooding |  | M | M | H | H | H |
| Other DoS |  | H | H |  | H | H |

## Impact

The impact of each of these types of attacks has been analysed depending on the category of messages which may be conveyed in the AMHS. It is possible that attacks are not directed to a particular category of messages. However, whatever the intention of the attacker is, the impact obviously ranks differently whether the attacked message is a distress message or an administrative one.

The basis for such an assessment is the categories of messages which can be handled by the AFTN, and consequently by the AMHS in the future. These categories are listed in ICAO Annex 10, Vol. II, section 4.4.1.1.3.

Obviously, such an assessment is subjective by essence. The goal is to provide a first written basis which WG3/SG1, with its extensive experience of AFTN, may comment and amend as appropriate until a consensus is reached.

The classification used to rank the impact is as follows:

H	high - immediate safety impact

M	medium - potential direct or indirect safety impact, but not time-critical and as such allowing for verification, correlation, etc. and/or for which alternative communications may be implemented. In cases where safety is not impacted, there is at least an impact on regularity and efficiency of air traffic

L	low - potential disturbance or loss of regularity and efficiency in air traffic, without any safety impact

-	none - no impact at all.

Table 3 - Ranking impact of attacks depending on message category

|  | Modification | Replay | Jamming | Masquerade | Flooding | Other DoS |
| --- | --- | --- | --- | --- | --- | --- |
| Distress Messages messages sent by mobile stations reporting that they are threatened by grave and imminent danger and all other messages relative to the immediate assistance required by the mobile station in distress | H | M | H | H | H | H |
| Urgency messages messages concerning the safety of a ship, aircraft or other vehicles, or of some person on board or within sight | H | M | H | H | H | H |
| Flight safety messages |  |  |  |  |  |  |
| Movement and control messages as defined in PANS-RAC (Doc 4444), Part VIII | M | M | M | M | M | M |
| Messages originated by an aircraft operating agency of immediate concern to aircraft in flight or preparing to depart | L | L | L | L | L | L |
| Meteorological messages restricted to SIGMET information, special air-reports and amended forecasts | M | L | M | L | M | M |
| Meteorological messages |  |  |  |  |  |  |
| Messages concerning forecasts, e.g. terminal aerodrome forecasts (TAFs), area and route forecasts | L | - | L | L | L | L |
| Messages concerning observations and reports, e.g. METAR, SPECI | L | - | L | L | L | L |

|  | Modification | Replay | Jamming | Masquerade | Flooding | Other DoS |
| --- | --- | --- | --- | --- | --- | --- |
| Flight regularity messages |  |  |  |  |  |  |
| Aircraft load messages required for weight and balance computation | L | - | L | L | L | L |
| Messages concerning changes in aircraft operating schedules | L | - | L | L | L | L |
| Messages concerning aircraft servicing | L | - | L | L | L | L |
| Messages concerning changes in collective requirements for passengers, crew and cargo covered by deviation from normal operating schedules | L | - | L | L | L | L |
| Messages concerning non-routine landings | M | - | L | L | L | L |
| Messages concerning pre-flight arrangements for air navigation services and operational serving for non-scheduled aircraft operations, e.g.overflight clearance requests | M | - | L | L | L | L |
| Messages originated by aircraft operating agencies reporting an aircraft arrival or departure | M | - | L | L | L | L |
| Messages concerning parts and materials urgently required for the operation of aircraft | L | - | L | L | L | L |
| Aeronautical information services (AIS) |  |  |  |  |  |  |
| Messages concerning NOTAMs | M | L | M | L | M | M |
| Messages concerning SNOWTAMs | M | L | M | L | M | M |
| Aeronautical administrative messages |  |  |  |  |  |  |
| messages regarding the operation or maintenance of facilities provided for the safety or regularity of aircraft operations | L | - | L | L | L | L |
| Messages concerning the functioning of aeronautical telecommunication services | L | - | L | L | L | L |
| Messages exchanged between civil aviation authorities relating to aeronautical services | L | - | L | L | L | L |

The following comments can be made about the rationale for the ranking above:

- modification generally ranks highest (or equal) because it affects a true message or true operational conditions, and thus information critical to the meaning/use of the message may be lost;

- replay generally ranks lowest because many systems or recipients would detect the inconsistence or irrelevance associated with a replayed message;

- jamming, flooding and other DoS generally rank identical as the highest impact for the message category, since they all may result in the loss or excessive reception delay for the considered message.

# Recommendation

- The subgroup is invited to comment, amend as appropriate and endorse the risk analysis detailed above. It is further invited to submit this analysis to ATNP/WG3 and ATNP/WG1 as the base risk analysis for the elaboration of the AMHS security approach.