ATNP WG-3

WP-5-32

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3

Gold Coast, Australia

February 5-14, 1996

MESSAGE RESPONSE REQUIREMENTS,

MODIFICATION FOR CPDLC SARP’s

(Presented by Mr. M J A Asbury)

(Prepared by Mr. G. Anderson)

This paper provides information relative to a modification to the CPDLC SARP’s as instructed by ADSP WG-B.

1. 	Introduction

1.1 	This paper provides information on the results of the ADSP WG-B subgroup meeting held in San Francisco, CA, USA from January 29-February 2,1996.

2. 	Discussion

2.1	The ADSP WG-B held a subgroup meeting for review of the comments received and progress work on the Draft ATS Data Link Services Applications Guidance Material to be presented to the ADSP Working Group of the Whole in Dakar, Senegal from March 19-29, 1996.

2.2	One of the items which is applicable to the ATNP WG-3 is the identification of the need for a modification of the “Y” or “YES”, response attribute category in the CPDLC message set.

2.3	In certain circumstances, there is an operational need for the allowance of more responses than heretofore allowed by the response attributes tables in the message set.  Due to this need, the decision was made to modify the response attributes allowed for uplink and downlink messages.

2.4	The relevant tables from the Draft ATS Data Link Services Applications Guidance Material are attached at Appendix A.

3.	Recommendation

3.1	ATNP WG-3 is invited to review and discuss the attached material and agree to its inclusion as a modification to the CPDLC SARP’s for the CNS/ATM-1 package.

#### APPENDIX A

Response Attribute Tables from the

Draft ATS Data Link Applications Guidance Material

#### Response

##### The response (RESP) attribute mandates response requirements for a given message element.  Response types are presented in Table (xx) for uplink messages and Table (xx) for downlink messages.

| Type | Response Required | Valid Responses | Precedence |
| --- | --- | --- | --- |
| W/U | Yes | WILCO, UNABLE, STANDBY permitted, LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 1 |
| A/N | Yes | AFFIRM, NEGATIVE, STANDBY permitted, LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 2 |
| R | Yes | ROGER, UNABLE, STANDBY permitted LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 3 |
| Y | Yes | Any CPDLC uplink message, LOGICAL ACKNOWLEDGMENT (only if required) | 4 |
| N | No, unless logical acknowledgment required | LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary, only when logical acknowledgment is required) | 5 |

Table X-X:  Response Attribute (Up-Link)

| Type | Response Required | Valid Responses | Precedence |
| --- | --- | --- | --- |
| Y | Yes | Any CPDLC downlink message, LOGICAL ACKNOWLEDGMENT (only if required), | 1 |
| N | No, unless logical acknowledgment required | LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary, only when logical acknowledgment is required) | 2 |

Table X-X:  Response Attribute (Down-Link)