## ADS-ASE State Tables

### Introduction

Note 1.— This section defines the state tables for the ADS-air-ASE and the ADS-ground ASE..

Note 2.—If the state tables shown in this section conflict with the textual statements made elsewhere in this SARPs, the textual statements take precedence.

### DC Module State Tables

| State Þ | DC-G-IDLE | DC-G-PENDING | DC-G-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Primitive Requests and Responses |  |  |  |
| ADS-demand-contract req | Send ADS-demand-contract-PDU Start t-DC-1 DC-G-PENDING | Not permitted | Not permitted |
| ADS downlink PDUs |  |  |  |
| ADS-demand-report-PDU (with positive acknowledgement) | request AB to abort | Stop t-DC-1 ADS-report ind DC-G-IDLE | request AB to abort |
| ADS-demand-report-PDU | request AB to abort | request AB to abort | stop t-DC-2 ADS-report ind DC-G-IDLE |
| ADS-negative-acknowledgement-PDU (demand contract) | request AB to abort | stop t-DC-1 ADS-demand-contract cnf DC-G-IDLE | request AB to abort |
| ADS-noncompliance-notification-PDU (demand-ncn) | request AB to abort | stop t-DC-1 ADS-demand-contract cnf start t-DC-2 DC-G-ACTIVE | request AB to abort |
| Requests from other modules |  |  |  |
| Request to stop operation | DC-G-IDLE | stop t-DC-1 DC-G-IDLE | stop t-DC-2 DC-G-IDLE |
| Timer expiry |  |  |  |
| t-DC-1 | cannot occur | request AB to abort | cannot occur |
| t-DC-2 | cannot occur | cannot occur | request AB to abort |

Table 5-10:  ADS ground DC module state table

| State Þ | DC-A-IDLE | DC-A-PENDING | DC-A-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Primitive Requests and Responses |  |  |  |
| ADS-demand-contract rsp (negative acknowledgement) | Not permitted | Send ADS-negative-acknowledgement-PDU DC-A-IDLE | Not permitted |
| ADS-demand-contract rsp (noncompliance notification) | Not permitted | Send ADS-noncompliance-notification-PDU DC-A-ACTIVE | Not permitted |
| ADS-report req (demand contract with positive acknowledgement) | Not permitted | Send ADS-demand-report-PDU DC-A-IDLE | Not permitted |
| ADS-report req (demand contract) | Not permitted | Not permitted | Send ADS-demand-report-PDU DC-A-IDLE |
| Requests from other modules |  |  |  |
| Requests to stop operation | DC-A-IDLE | DC-A-IDLE | DC-A-IDLE |
| ADS uplink PDUs |  |  |  |
| ADS-demand-contract-PDU | ADS-demand-contract ind DC-A-PENDING | request AB to abort | request AB to abort |

Table 5-11: ADS air DC module state table

### EC Module State Tables

| State Þ | EC-G-IDLE | EC-G-START-PENDING | EC-G-ACTIVE | EC-G-PENDING | EC-G-CANCEL |
| --- | --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |  |
| Primitive Requests and Responses |  |  |  |  |  |
| ADS-event-contract req | Send ADS-event-contract -PDU Start t-EC-1 EC-G-START-PENDING | Not permitted | Send ADS-event-contract -PDU Start t-EC-1 EC-G-PENDING | Not permitted | Not permitted |
| ADS-cancel req (event contract) | Not permitted | Not permitted | Send ADS-cancel-PDU Start t-EC-2 EC-G-CANCEL | Not permitted | Not permitted |
| ADS Aircraft PDUs |  |  |  |  |  |
| ADS-event-report-PDU (with positive acknowledgement) | request AB to abort | Stop t-EC-1 ADS-report ind EC-G-ACTIVE | request AB to abort | Stop t-EC-1 ADS-report ind EC-G-ACTIVE | request AB to abort |
| ADS-event-report-PDU | request AB to abort | request AB to abort | ADS-report ind EC-G-ACTIVE | ADS-report ind EC-G-PENDING | ADS-report ind EC-G-CANCEL |
| ADS-positive-acknowledgement-PDU (event-contract) | request AB to abort | stop t-EC-1 ADS-event-contract cnf EC-G-ACTIVE | request AB to abort | stop t-EC-1 ADS-event-contract cnf EC-G-ACTIVE | request AB to abort |
| ADS-positive-acknowledgement-PDU (cancel-contract - event) | request AB to abort | request AB to abort | request AB to abort | request AB to abort | Stop t-EC-2 ADS-cancel-contract cnf EC-G-IDLE |
| ADS-negative-acknowledgement-PDU (event-contract) | request AB to abort | stop t-EC-1 ADS-event-contract cnf EC-G-IDLE | request AB to abort | stop t-EC-1 ADS-event-contract cnf EC-G-ACTIVE | request AB to abort |
| ADS-noncompliance-notification-PDU (event-ncn) | request AB to abort | stop t-EC-1 ADS-event-contract cnf EC-G-ACTIVE | request AB to abort | stop t-EC-1 ADS-event-contract cnf EC-G-ACTIVE | request AB to abort |
| Requests from other modules |  |  |  |  |  |
| Requests to stop operation | EC-G-IDLE | stop t-EC-1 EC-G-IDLE | EC-G-IDLE | stop t-EC-1 EC-G-IDLE | stop t-EC-2 EC-G-IDLE |
| Timer expiry |  |  |  |  |  |
| t-EC-1 | cannot occur | request AB to abort | cannot occur | request AB to abort | cannot occur |
| t-EC-2 | cannot occur | cannot occur | cannot occur | cannot occur | request AB to abort |

Table 5-12: ADS ground EC module state table

| State Þ | EC-A-IDLE | EC-A-PENDING | EC-A-ACTIVE | EC-A-ACTIVE-PENDING |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| Primitive Requests and Responses |  |  |  |  |
| ADS-event-contract rsp (positive acknowledgement or noncompliance notification) | Not permitted | Send ADS-positive-acknowledgement-PDU or ADS-noncompliance-notification-PDU EC-A-ACTIVE | Not permitted | Send ADS-positive-acknowledgement-PDU or ADS-noncompliance-notification-PDU EC-A-ACTIVE |
| ADS-event-contract rsp (negative acknowldegement) | Not permitted | Send ADS-negative-acknowledgement-PDU EC-A-IDLE | Not permitted | Send ADS-negative-acknowledgement-PDU EC-A-ACTIVE |
| ADS-report req (with positive acknowledgement) (event contract) | Not permitted | Send ADS-event-report-PDU EC-A-ACTIVE | Not permitted | Send ADS-event-report-PDU EC-A-ACTIVE |
| ADS-report req (event contract) | Not permitted | Not permitted | Send ADS-event-report-PDU EC-A-ACTIVE | Not permitted |
| Requests from other modules |  |  |  |  |
| Requests to stop operation | EC-A-IDLE | EC-A-IDLE | EC-A-IDLE | EC-A-IDLE |
| ADS Ground PDUs |  |  |  |  |
| ADS-event-contract-PDU | ADS-event-contract ind EC-A-PENDING | request AB to abort | ADS-event-contract ind EC-A-ACTIVE-PENDING | request AB to abort |
| ADS-cancel-PDU (event contract) | request AB to abort | request AB to abort | ADS-cancel ind Send ADS-positive-acknowledgement EC-A-IDLE | request AB to abort |

Table 5-13: ADS air EC module state table

### PC Module State Tables

| State Þ | PC-G-IDLE | PC-G-START-PENDING | PC-G-ACTIVE | PC-G-PENDING | PC-G-CANCEL |
| --- | --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |  |
| Primitive Requests and Responses |  |  |  |  |  |
| ADS-periodic-contract req | Send ADS-periodic-contract -PDU Start t-PC-1 PC-G-START-PENDING | Not permitted | If emergency=FALSE then Stop t-PC-2 Send ADS-periodic-contract -PDU Start t-PC-1 PC-G-PENDING | Not permitted | Not permitted |
| ADS-cancel req | Not permitted | Not permitted | Stop t-PC-2 Send ADS-cancel-PDU Start t-PC-3 PC-G-CANCEL | Not permitted | Not permitted |
| ADS Aircraft PDUs |  |  |  |  |  |
| ADS-periodic-report-PDU (with positive acknowledgement) | request AB to abort | Stop t-PC-1 ADS-report ind If emergency=FALSE then Start t-PC-2 PC-G-ACTIVE | request AB to abort | Stop t-PC-1 ADS-report ind If emergency=FALSE then Start t-PC-2 PC-G-ACTIVE | request AB to abort |
| ADS-periodic-report-PDU (with no positive acknowledgement) | request AB to abort | request AB to abort | If emergency=FALSE then Stop t-PC-2 ADS-report ind If emergency=FALSE then Start t-PC-2 PC-G-ACTIVE | ADS-report ind PC-G-PENDING | ADS-report ind PC-G-CANCEL |
| ADS-positive-acknowledgement-PDU (periodic-contract) | request AB to abort | stop t-PC-1 ADS-periodic-contract cnf If emergency=FALSE then Start t-PC-2 PC-G-ACTIVE | request AB to abort | stop t-PC-1 ADS-periodic-contract cnf If emergency=FALSE then Start t-PC-2 PC-G-ACTIVE | request AB to abort |
| ADS-positive-acknowledgement-PDU (cancel-contract - periodic) | request AB to abort | request AB to abort | request AB to abort | request AB to abort | Stop t-PC-3 ADS-cancel-contract cnf PC-G-IDLE |
| ADS-negative-acknowledgement-PDU (periodic-contract) | request AB to abort | stop t-PC-1 ADS-periodic-contract cnf PC-G-IDLE | request AB to abort | stop t-PC-1 ADS-periodic-contract cnf If emergency=FALSE then Start t-PC-2 PC-G-ACTIVE | request AB to abort |
| ADS-noncompliance-notification-PDU | request AB to abort | stop t-PC-1 ADS-periodic-contract cnf If emergency=FALSE then Start t-PC-2 PC-G-ACTIVE | request AB to abort | stop t-PC-1 ADS-periodic-contract cnf If emergency=FALSE then Start t-PC-2 PC-G-ACTIVE | request AB to abort |
| Requests from other modules |  |  |  |  |  |
| Request to stop operation | set emergency=FALSE PC-G-IDLE | stop t-PC-1 set emergency=FALSE PC-G-IDLE | If emergency=FALSE then stop t-PC-2 set emergency=FALSE PC-G-IDLE | stop t-PC-1 set emergency=FALSE PC-G-IDLE | stop t-PC-3 set emergency=FALSE PC-G-IDLE |
| Request to suspend periodic contract | set emergency=TRUE | set emergency=TRUE | set emergency=TRUE stop t-PC-2 | set emergency=TRUE | set emergency=TRUE |
| Request to reinstate periodic contract | set emergency=FALSE | set emergency=FALSE | set emergency=FALSE start t-PC-2 | set emergency=FALSE | set emergency=FALSE |
| Timer expiry |  |  |  |  |  |
| t-PC-1 | cannot occur | request AB to abort | cannot occur | request AB to abort | cannot occur |
| t-PC-2 | cannot occur | cannot occur | request AB to abort | cannot occur | cannot occur |
| t-PC-3 | cannot occur | cannot occur | cannot occur | cannot occur | request AB to abort |

Table 5-14:  ADS-periodic-contract ground based state table

| State Þ | PC-A-IDLE | PC-A-PENDING | PC-A-ACTIVE | PC-A-ACTIVE-PENDING |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| Primitive Requests and Responses |  |  |  |  |
| ADS-periodic-contract rsp (positive acknowledgement or noncompliance notification) | Not permitted | Send ADS-positive-acknowledgement-PDU or ADS-noncompliance-notification-PDU PC-A-ACTIVE | Not permitted | Send ADS-positive-acknowledgement-PDU or ADS-noncompliance-notification-PDU PC-A-ACTIVE |
| ADS-periodic-contract rsp (negative acknowledgement) | Not permitted | Send ADS-negative-acknowledgement-PDU PC-A-IDLE | Not permitted | Send ADS-negative-acknowledgement-PDU PC-A-ACTIVE |
| ADS-report req - with positive acknowledgement (periodic contract) | Not permitted | Send ADS-periodic-report-PDU PC-A-ACTIVE | Not permitted | Send ADS-periodic-report-PDU PC-A-ACTIVE |
| ADS-report req (periodic contract) | Not permitted | Not permitted | Send ADS-periodic-report-PDU PC-A-ACTIVE | Not permitted |
| Requests from other modules |  |  |  |  |
| Requests to stop operation | PC-A-IDLE | PC-A-IDLE | PC-A-IDLE | PC-A-IDLE |
| ADS uplink PDUs |  |  |  |  |
| ADS-periodic-contract-PDU | ADS-periodic-contract ind PC-A-PENDING | request AB to abort | ADS-periodic-contract ind PC-A-ACTIVE-PENDING | request AB to abort |
| ADS-cancel-PDU (periodic contract) | request AB to abort | request AB to abort | ADS-cancel ind Send ADS-positive-acknowledgement PC-A-IDLE | request AB to abort |

Table 5-15: ADS air PC module state table

### EM Module State Tables

| State Þ | EM-G-IDLE | EM-G-ACTIVE | EM-G-MODIFY |
| --- | --- | --- | --- |
| Event ß |  |  |  |
| Primitive Requests and Responses |  |  |  |
| ADS-modify-emergency-contract req | Not permitted | Stop t-EM-1 Send ADS-modify-emergency-contract-PDU Start t-EM-2 EM-G-MODIFY | Not permitted |
| ADS Aircraft PDUs |  |  |  |
| ADS-emergency-report-PDU (with positive acknowledgement) | request AB to abort | request AB to abort | Stop t-EM-2 ADS-emergency-report ind Start t-EM-1 EM-G-ACTIVE |
| ADS-emergency-report-PDU | Suspend periodic contract ADS-emergency-report ind Start t-EM-1 EM-G-ACTIVE | Stop t-EM-1 ADS-emergency-report ind Start t-EM-1 EM-G-ACTIVE | ADS-emergency-report ind EM-G-MODIFY |
| ADS-cancel-emergency-PDU | request AB to abort | Stop t-EM-1 ADS-cancel-emergency ind Send ADS-cancel-emergency-acknowledgement-PDU Re-instate periodic contracts EM-G-IDLE | Stop t-EM-2 ADS-cancel-emergency ind Send ADS-cancel-emergency-acknowledgement-PDU Re-instate periodic contracts EM-G-IDLE |
| ADS-positive-acknowledgement-PDU (modify-emergency-contract) | request AB to abort | request AB to abort | Stop t-EM-2 ADS-modify-emergency-contract cnf Start t-EM-1 EM-G-ACTIVE |
| Requests from other modules |  |  |  |
| Requests to stop operation | EM-G-IDLE | stop t-EM-1 EM-G-IDLE | stop t-EM-2 EM-G-IDLE |
| Timer expiry |  |  |  |
| t-EM-1 | cannot occur | request AB to abort | cannot occur |
| t-EM-2 | cannot occur | cannot occur | request AB to abort |

Table 5-16: ADS ground EM module state table

| State Þ | EM-A-IDLE | EM-A-ACTIVE | EM-A-MODIFY | EM-A-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß |  |  |  |  |
| Primitive Requests and Responses |  |  |  |  |
| ADS-emergency-report req | Send ADS-emergency-report-PDU EM-A-ACTIVE | Send ADS-emergency-report-PDU EM-A-ACTIVE | Not permitted | Not permitted |
| ADS-emergency-report req (with positive acknowledgement) | Not permitted | Not permitted | Send ADS-emergency-report-PDU EM-A-ACTIVE | Not permitted |
| ADS-cancel-emergency req | Not permitted | Send ADS-cancel-emergency-PDU Start t-EM-3 EM-A-CANCEL | Not permitted | Not permitted |
| ADS-modify-emergency-contract rsp | Not permitted | Not permitted | Send ADS-negative-acknowledgement-PDU EM-A-ACTIVE | Not permitted |
| ADS Ground PDUs |  |  |  |  |
| ADS-cancel-emergency-acknowledgement-PDU | request AB to abort | request AB to abort | request AB to abort | Stop t-EM-3 EM-A-IDLE |
| ADS-modify-emergency-contract-PDU | request AB to abort | ADS-modify-emergency-contract ind EM-A-MODIFY | request AB to abort | EM-A-CANCEL |
| Requests from other modules |  |  |  |  |
| Requests to stop operation | EM-A-IDLE | EM-A-IDLE | EM-A-IDLE | stop t-EM-3 EM-A-IDLE |
| Timer expiry |  |  |  |  |
| t-EM-3 | cannot occur | cannot occur | cannot occur | request AB to abort |

Table 5-17: ADS air EM module state table

### LI Module State Tables

| State Þ | LI-G-IDLE | LI-G-START-I | LI-G-ACTIVE | LI-G-END-I | LI-START-R | LI-G-END-R |
| --- | --- | --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |  |  |
| Data and requests passed from other modules |  |  |  |  |  |  |
| ADS-demand-contract-request-PDU, ADS-event-contract-request-PDU, ADS-forward-contract-PDU or ADS-periodic-contract-request-PDU, | D-START req LI-G-START-I | Not permitted | D-DATA req LI-G-ACTIVE | Not permitted | Not permitted | Not permitted |
| ADS-cancel-all-contracts req or ADS-end-forward-service-PDU | Not permitted | Not permitted | D-END req LI-G-END-I | Not permitted | Not permitted | Not permitted |
| ADS-cancel-contract-PDU, ADS-modify-emergency-contract-PDU or ADS-cancel-emergency-acknowledgement-PDU | Not permitted | Not permitted | D-DATA req [1] | Not permitted | Not permitted | Not permitted |
| ADS-provider-abort-PDU | Not permitted | D-ABORT req LI-G-IDLE | D-ABORT req LI-G-IDLE | D-ABORT req LI-G-IDLE | D-ABORT req LI-G-IDLE | D-ABORT req LI-G-IDLE |
| ADS-forward-contract-response-PDU | Not permitted | Not permitted | Not permitted | Not permitted | D-START rsp LI-G-ACTIVE | Not permitted |
| Primitive Indications and Confirmations |  |  |  |  |  |  |
| D-START ind | pass user data to appropriate module LI-G-START-R | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START cnf | cannot occur | pass user data to appropriate module [1] | cannot occur | cannot occur | cannot occur | cannot occur |
| D-DATA ind | cannot occur | cannot occur | pass user data to appropriate module [1] | pass user data to appropriate module LI-G-END-I | cannot occur | cannot occur |
| D-END-ind | cannot occur | cannot occur | if ADS-end-forward-service-PDU, pass to HI module D-END rsp LI-G-IDLE | cannot occur | cannot occur | cannot occur |
| D-END cnf | cannot occur | cannot occur | cannot occur | LI-G-IDLE | cannot occur | cannot occur |
| D-ABORT ind or D-P-ABORT ind | cannot occur | pass to AB module LI-G-IDLE | pass to AB module LI-G-IDLE | pass to AB module LI-G-IDLE | pass to AB module LI-G-IDLE | pass to AB module LI-G-IDLE |

Table 5-18:  Ground ADS LI module state table

[1]	If DC, EC, PC, FC and EM modules are all in their idle state then

Invoke D-END req with no user data

LI-G-END-I

else

LI-G-ACTIVE

| State Þ | LI-A-IDLE | LI-A-START | LI-A-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Data and requests passed from other modules |  |  |  |
| ADS-demand-report-PDU, ADS-negative-acknowledgement-PDU, ADS-event-report-PDU, ADS-periodic-report-PDU, ADS-positive-acknowledgement-PDU, ADS-noncompliance-notification-PDU | Not permitted | D-START rsp LI-A-ACTIVE | D-DATA req LI-A-ACTIVE |
| ADS-cancel-emergency-PDU, ADS-emergency-report-PDU | Not permitted | Not permitted | D-DATA req LI-A-ACTIVE |
| Primitive Indications and Confirmations |  |  |  |
| D-START ind | pass to appropriate module LI-G-START | cannot occur | cannot occur |
| D-DATA ind | cannot occur | cannot occur | pass user data to appropriate module LI-A-ACTIVE |
| D-END ind | cannot occur | cannot occur | if ADS-cancel-all-contracts-PDU, pass to HI module D-END rsp LI-A-IDLE |
| D-ABORT ind or D-P-ABORT ind | cannot occur | pass to AB module LI-A-IDLE | pass to AB module LI-A-IDLE |

Table 5-19:  Air ADS LI module state table

### ADS FC ASE State Tables

| State Þ | FC-I-IDLE | FC-I-START | FC-I-ACTIVE | FC-I-END |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| Primitive Requests and Responses |  |  |  |  |
| ADS-start-forward req | D-START req start t-FC-1 FC-I-START | Not permitted | Not permitted | Not permitted |
| ADS-forward-report req | Not permitted | Not permitted | D-DATA req FC-I-ACTIVE | Not permitted |
| ADS-end-forward req | Not permitted | Not permitted | D-END req start t-FC-2 FC-I-END | Not permitted |
| Primitive Indications and Confirmations |  |  |  |  |
| D-START cnf accepted | Cannot occur | stop t-FC-1 ADS-start-forward cnf accepted FC-I-ACTIVE | Cannot occur | Cannot occur |
| D-START cnf rejected | Cannot occur | stop t-FC-1 ADS-start-forward cnf rejected FC-I-IDLE | Cannot occur | Cannot occur |
| D-END cnf accepted | Cannot occur | Cannot occur | Cannot occur | stop t-FC-2 FC-I-IDLE |
| D-END cnf rejected | Cannot occur | Cannot occur | Cannot occur | stop t-FC-2 D-ABORT req FC-I-IDLE |
| Timer Expiry |  |  |  |  |
| t-FC-1 | Cannot occur | D-ABORT req ADS-provider-abort ind FC-I-IDLE | Cannot occur | Cannot occur |
| t-FC-2 | Cannot occur | Cannot occur | Cannot occur | D-ABORT req ADS-provider-abort ind FC-I-IDLE |

Table 5-20: Initiating FC ASE state table

| State Þ | FC-R-IDLE | FC-R-START | FC-R-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Primitive Requests and Indications |  |  |  |
| ADS-start-forward reponse with Reply=“accepted” | Not permitted | D-START rsp accepted FC-R-ACTIVE | Not permitted |
| ADS-start-forward reponse with Reply=“rejected” | Not permitted | D-START rsp rejected FC-R-IDLE | Not permitted |
| Primitive Indications and Confirmations |  |  |  |
| D-START ind | ADS-start-forward ind FC-R-START | Cannot occur | Cannot occur |
| D-DATA ind | Cannot occur | Cannot occur | ADS-forward-report ind FC-R-ACTIVE |
| D-END ind | Cannot occur | Cannot occur | ADS-forward-end ind D-END rsp accepted FC-R-IDLE |
| D-ABORT ind with originator=“user” | cannot occur | ADS-user-abort ind FC-R-IDLE | ADS-user-abort ind FC-R-IDLE |
| D-ABORT ind with originator=“provider” or D-P-ABORT ind | cannot occur | ADS-provider-abort ind FC-R-IDLE | ADS-provider-abort ind FC-R-IDLE |

Table 5-21:  Responding FC ASE state table