# COMMUNICATION REQUIREMENTS

## Encoding Rules

### The ADS application shall use PER as defined in reference [2], using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in chapter 4 of this document, or a functionally equivalent means which provides the same result.

## Dialogue Service Requirements

### Primitive Requirements

#### Where dialogue service primitives, that is D-START, D-END, D-ABORT, D-P-ABORT and D-DATA are described as being invoked in chapter 5 of this document, the ADS-ground-ASE and the ADS-air-ASE shall exhibit external behaviour consistent with the dialogue service, as described in reference [4], having been implemented and its primitives invoked.

### Quality of Service Requirements

#### The application service priority for ADS shall have the abstract value of  “normal priority flight safety messages”.

#### The RER quality of service parameter of the D-START request shall be set to the abstract value of “low”.

Note 1.— The application service residual error rate for ADS is required to be 10-7 or better.

Note 2. — The RER takes into account non-delivery for messages over a given dialogue.

#### The ADS-ASE shall map the class of communication service abstract values to the ATSC routing class abstract value part of the D-START QOS parameter as presented in Table 6-1.

| Class of Communication Abstract Value | Routing Class Abstract Value |
| --- | --- |
| A | Traffic only follows Class A ATSC route(s) |
| B | Traffic only follows Class B ATSC route(s) |
| C | Traffic only follows Class C ATSC route(s) |
| D | Traffic only follows Class D ATSC route(s) |
| E | Traffic only follows Class E ATSC route(s) |
| F | Traffic only follows Class F ATSC route(s) |
| G | Traffic only follows Class G ATSC route(s) |
| H | Traffic only follows Class H ATSC route(s) |
| I | Traffic only follows Class I ATSC route(s) |
| J | Traffic only follows Class J ATSC route(s) |

Table 6-1:  Mapping between class of communication and routing class abstract values

Note. — ATSC values are defined in [5].

## ADS AE Control Function Requirements

### The AE-Qualifier for the ADS application shall be the IA5 character string “ADS”.