D-ATIS Glossary

Note:  This document should be read in conjunction with ICAO Annexes 3, 4, 5, 11, 14 and 15.

Aerodrome.  indicates the aerodrome for the D-ATIS information.

Air temperature.  indicates the outside air temperature representative of the runway(s).  Expressed as Temperature.

Altimeter setting.  indicates the QNH, and optionally the QFE for specified runway(s), for the aerodrome.

Approach area MET.  text providing information on significant meteorological phenomena in the approach area.

Arrival and/or Departure indicator.  indicates which type of D-ATIS information is provided (i.e. arrival, departure, or combined).

Arrival runways in use.  indicates landing runway(s).  Expressed as one or more of Runway, and may be further described as a runway which is circled to for landing.

ATIS designator.  provides a letter identifying the D-ATIS iteration.

Braking action.  indicates the braking action in case of contaminated runway conditions as Good, Medium to Good, Medium, Medium to Poor, and Poor, as well as associated explanatory text .  May be described as the full runway or by thirds of the runway.

CAVOK.  replaces Cloud, Visibility and Present Weather when the conditions specified in Annex 3 paragraph 4.13.2 are met.

Climb out area MET.  text providing information on significant meteorological phenomena in the climb out area.

Cloud.  provides the Cloud amount, Cloud height and Cloud type, where appropriate, of any cloud below 1 500 m (5 000 ft) or below the highest minimum sector level whichever is greater.  When the sky is obscured it is indicated by the characters “OBSCD”, and if the Vertical Visibility is available it will follow “OBSCD” with the characters “VV” and the vertical visibility value.

Cloud amount.  expressed as sky clear (SKC), few (FEW), scattered (SCT), broken (BKN), or overcast (OVC).

Cloud height.  indicates the height of the base of the clouds above ground level in meters or feet.

Cloud type.  identified only for cumulonimbus and towering cumulus.

Contract number.  identifying number of each contract.

Contract termination.  indicates that the D-ATIS contract is terminated.

Contract type.  Distinguishes between the types of D-ATIS provided (demand or contract).

Date.  provides date as Year, Month, and Day.

Day.  indicates day of month displayed as a two digit number.

Departure runways in use.  indicates departure runway(s).  Expressed as one or more of Runway.

Descriptor qualifier.  describes a present weather phenomenon as: shallow, partial, patches, low drifting, blowing, shower(s), thunderstorm, or freezing.

Dew point temperature.  indicates the outside dew-point temperature representative of the runway(s).  Expressed as Temperature.

Holding delay.  indicates an unplanned delay prior to the initiation of the next phase of flight.

Intensity qualifier.  describes a present weather phenomenon as: light, moderate, or heavy.

Month.  indicates month of a year displayed as a two digit number.

NOSIG.  contraction which indicates that the reported Present Weather is not expected to change or pass a specific threshold value.

Other essential operational information.  text providing operational information not covered by the other available parameters.

Present Weather.  Gives the present weather occurring at or near the aerodrome.  The present weather phenomena are selected from the Present weather type which is described by an Intensity qualifier, Proximity qualifier, or Descriptor qualifier.

Present weather type.  Identifies the present weather as precipitation (i.e. drizzle, rain, snow, snow grains, ice crystals, ice pellets, hail or small hail and/or snow pellets), obscuration (i.e. mist, fog, smoke, volcanic ash, widespread dust, sand or haze) or other weather phenomena (i.e. dust/sand whirls, squall, funnel clouds (tornado/waterspout), sandstorm or duststorm).

Proximity qualifier.  describes if a present weather phenomenon is occurring in the vicinity of the aerodrome.  Indicated with the characters “VC”.

Runway .  provides the identity of a runway using a Runway designator and optionally an additional Runway letter.

Runway arresting system condition.  describes the runway arresting system condition as either up, down, or unavailable.

Runway arresting system location.  describes the runway arresting system location as being at the arrival, departure, or both ends of the runway.

Runway designator.  identifies the runway by a two digit number.

Runway letter.  describes a runway as being LEFT LEFT, LEFT, LEFT CENTER, CENTER, RIGHT CENTER, RIGHT, RIGHT RIGHT.

RVR.  The range (in meters or feet* ) over which the pilot of an aircraft on the centre line of a runway can see the runway surface markings or the lights delineating the runway or identifying its centre line.  RVR values are representative of touchdown zone, mid-point and stop-end which are included when available, or reported as inoperative or not reported, when applicable.  Otherwise, only the value representative of the touchdown zone is provided.  RVR includes the Runway and, if required, the RVR Variation Qualifier.

RVR Variation Qualifier .  provides the lowest and the highest RVR (in meters or feet* ) separated by the designator “V” during varying RVR conditions.

Significant meteorological phenomena.  gives information on significant meteorological phenomena specific to Approach area MET, Take-off area MET, and/or Climb-out area MET.

Significant runway surface conditions.  describes the runway conditions as being Damp, Wet, Water Patches, Flooded, Snow, Wet Snow, Dry Snow, Slush, or Ice followed by the text to further explain details of the conditions, such as depth.  Expressed on a per Runway basis.

Sky clear.  describes sky conditions when there is no cloud, no restrictions on Vertical visibility, and CAVOK is not appropriate.  Expressed as “SKC”.

Specific ATIS instructions.  gives instruction to the pilot to acknowledge the receipt of the ATIS message upon initial contact.

Status of arresting system.  indicates when an arresting system constitutes a potential hazard.  Expressed as Runway, Runway arresting system location, and Runway arresting system condition.

Surface wind direction.  indicated in degrees magnetic.

Surface wind speed.  indicated in km per hour or knots.

Surface wind variations.  provides significant variations of wind direction and wind speed in terms of two extreme directions and maximum wind speed (gusts).  Expressed as Surface wind speed and two of Surface wind direction.  Indicated with the characters “VRB” before the directions or speed.

Surface wind.  indicated as surface wind direction, surface wind speed, and when appropriate, surface wind variations.

Take-off area MET.  text providing information on significant meteorological phenomena in the take-off area..

Temperature.  indicates the temperature in degrees Celsius, when available.

Time of observation.  time at which the meteorological observation was made.  Expressed in hours and minutes UTC.

Transition level.  indicates a change from the published transition level.  Expressed as flight level in hundreds of feet or tens of meters.

Trend type landing forecast.  a concise statement of the expected trend of the meteorological conditions at the aerodrome.

Type of Approach.  indicates the type of approach to be expected, as a named approach of ILS; ILS/DME; Localizer; Localizer Back Course; Localizer DME; NDB; NDB/DME; VOR; VOR/DME; VORTAC; TACAN; Precision GPS, Non Precision GPS; DME/ARC; Precision Approach Radar; Airport Surveillance Radar; Visual; Charted Visual Approach Procedure; LDA; FMS; Loran; RNAV; MLS, SRA, VDF, and/or a character string.

Vertical visibility.  defined as the vertical visual range (in meters or feet*) into an obscuring medium.  Included instead of Cloud amount, Cloud height and Cloud type when the sky is obscured.

Visibility.  expressed in km and meters or statute miles* .

Year.  gives year displayed as a two digit number.

*Unit of measurement not specified in Annex 5.