# MANAGED OBJECTS FOR ATN UPPER LAYERS

This chapter contains provisions for the set of managed objects (MOs) for all classes of ATN End systems for Upper Layers for ground-ground and air-ground communications.

It is likely that, given the minimal session and presentation protocol requirements specified in the ULCS SARPs, there will be no requirement for specific management of the session and presentation entities or SAPs.

It can be envisaged that there will be requirements for monitoring of associations within ACSE.  For example, logs of connection set-up delays and failure probabilities could be maintained.

ISO 10165-8 "Managed objects for supporting the upper layers" is under development within ISO, and will be assessed for utility within the ATN architecture.

Perhaps of more relevance to ATN operation would be monitoring within the ATN-App AE itself, rather than concentrating on ACSE.  Thus, the Dialogue Service provided by the CF in the ULCS SARPs could be monitored to provide statistics not only on connection set-up but potentially also on throughput rates per traffic type and communications class, and even on transit delays.

Thus, WG3/SG3 should specify candidate MOs and their attributes at the level of the Dialogue Service.