# MANAGED OBJECTS FOR ATN APPLICATIONS

Note 1.— This chapter contains provisions for the set of managed objects (MOs) for all classes of ATN End Systems for Applications for ground-ground and air-ground communications.

Note 2.— Systems management in the applications could be useful:

- to monitor the quality of service available to end-users,

- to convey alarms to notify equipment failures and other exception conditions

Note 3.— Analysis of the applications has determined where systems management could be useful in monitoring the QoS as seen by end-users.  For example, for ADS, statistical information could be gathered on the operation of ADS contracts (contract type, reporting rate, etc.) which would allow some optimisation of communications bandwidth.  For CM, the response time to complete a Logon operation at various ground centres can be monitored, and any anomalies identified.

Note 4.— Systems management may be utilised to convey alarms to notify equipment failures and other exception conditions.  For example. in ADS the validity and availability of data required in an ADS Report may be called into question if there is a failure after the contract has been agreed.  Systems management notifications could be used to report the failure, thus allowing remedial action to be taken.

<<< WARNING!  The MOs defined in this section are subject to an ongoing convergence process, and are still likely to change.  A later version of the MOs may be found in the WG3 WP presented by Frédéric Picard of STNA.  This document will be updated following review of that WP.

>>>

## Summary of managed object classes

The following set of managed object classes are defined for the ATN Application layer:

- applicationSubsystem

- aTNcMae

- aTNcMaeInstance

- aTNaDSae

- aTNaDSaeInstance

- aTNaRFae

- aTNaRFaeInstance

- aTNcPDLCae

- aTNcPDLCaeInstance

- aTNfISae

- aTNfISaeInstance

- aTNaIDCae (not yet defined),

- aTNaIDCaeInstance (not yet defined),

- aTNsMae (not yet defined), and

- aTNsMaeInstance (not yet defined).

## Containment hierarchy

The containment hierarchy is illustrated in Figure 6.7-1. Managed objects which can have multiple instances are illustrated by shadowed boxes.  These objects are defined in detail in the following sections.

The two levels of MOC identified in the containment hierarchy - i.e. AE MOC and AE Instance MOC - allows for distinguishing between management of the static aspects of the ATN application entities and dynamic aspects related to application association (e.g., per invocation).

## The applicationSubsystem managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | applicationSubsystem There shall be one such MO within an ATN system. It exists to provide a container for all ATN Application Layer specific MOs. This subclass of “Rec. X.723 ¦ ISO/IEC 10165-5:1995”: subsystem holds reference information about an ATN application subsystem. The applicationSubsystem MO can not be created or deleted explicitly by management operation. It exists inherently in an ATN system; created and deleted as part of system operation. | A | M |
|  | Naming attribute | SubsystemId | A | M |
|  | Superior in Naming Tree | ATNsystem |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | SubsystemId Naming attribute GraphicString Initial value = “ applicationSubsystem” | GET | A | M |

### Actions

None.

### Notifications

None.

## The aTNcMae managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | aTNcMae There shall be one such MO per ATN Application Subsystem. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about a CM ATN application entity. Its definition permits it to be created or deleted explicitly by management operation, but in some systems it will exist inherently and neither creation nor deletion by management operation will be possible | A | M |
|  | Naming attribute | CommunicationsEntityId | A | M |
|  | Superior in Naming Tree | applicationSubsystem |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | communicationsEntityId Naming attribute as defined in ISO/IEC 10165-5. GraphicString Initial value = “ aTNcMae” | GET | A | M |
|  | operationalState Operational state as defined in ISO/IEC 10164-2. ENUMERATED {disabled(0), enabled(1)} Note. value is 'disabled' for any ATN system not supporting the CM application entity. | GET | A | M |
|  | maxNumberOfCMAEInstances Maximum observed number of CM AE instances running in parallel. INTEGER{initial value=0} Rationale: interesting to monitor the workload the ATN end system in terms of CM activity. | GET | A | M |

Note: A number of configuration attributes are not proposed to be retained for standardisation in the ATN SARPs:

| AETtile CMASEVersion CMASEConfiguration CMPriority CMRER t-logon, t-update, t-contact, t-forward, t-end | The AE title of the local CM AE. The version of the CM protocol operated by the CM ASE entity. This parameter identifies as well the Application Context in use. The configuration supported by the ASE entity (identified by the subsetting rules of SARPs chapter 8). The application priority requested by the CM application for all messages. The Residual Error Rate requested by the CM application for all messages. CM Technical Timers |
| --- | --- |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | stateChange stateChange notification as defined in ISO/IEC 10165-2. Used to report the changes to the operationalState attribute. Rationale: it is a basic requirement for the manager to know whether a protocol entity is operational or not. Monitoring the operationalState may not be sufficient, since the polling period may not allow to detect problem quickly enough. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes. |  | M | M |

Note: A number of standard notifications are not proposed to be retained for standardisation in the ATN SARPs; the rationale is provided below:

| objectCreation | This notification allows the manager to dynamically discover that the managed system implements the MO, or to confirm a create operation, and allows to report initial MO attribute values. ATN systems are required to support one such MO. Manager are therefore assumed to a-priori know that one instance of this MO will exist. The stateChange notification will allow knowing when the MO is operational. No requirement for the logging of initial attribute values is identified for this MO. |
| --- | --- |

## The aTNcMaeInstance managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | aTNcMaeInstance This MO represents an instance of the CM AE. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about an instance of the CM ATN application entity. There may be multiple instances of these MOs for a CM AE. Each corresponds to dialogue established with a peer CM AE. A CM AE instance is created and deleted automatically as part of system operation. | A | M |
|  | Naming attribute | aEInstanceId | A | M |
|  | Superior in Naming Tree | aTNcMae |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | aEInstanceId Naming attribute. GraphicString | GET | A | M |
|  | remoteAETitle The AE Title identifying the remote CM AE in communication with the local CM AE instance. OBJECT IDENTIFIER Rationale: to keep trace of the identity of the peer CM ES. Needed for investigation of any potential problem. | GET | A | M |
|  | aTSCclassOfCommunicationService The ATSC class of communication service as requested by the initiator CM-user. ENUMERATED {'A'(0) to 'H'(7), no preference(8)} | GET | A | M |
|  | dialogueRole The role of the local AE instance during the establishment of the underlying dialogue. ENUMERATED {initiator(0), receptor(1)} | GET | A | M |
|  | terminationMode The way the local AE instance has been terminated. ENUMERATED {normal-termination (0), local-user-abort (1), remote-user-abort (2), local-AE-abort (3), remote-AE-abort (4)} Rationale: interesting for the off-line analysis of the ASE's behavior. | GET | A | M |
|  | aSEabortReason The reason of the dialogue abort, if aborted by an ASE. ENUMERATED {timer-expired (0), undefined-error(1), invalid-PDU (2), not-permitted-PDU (3), dialogue-acceptance-not-permitted (4), dialogue-end-not-permitted (5), communication-service-error (6), communication-service-failure (7), invalid-QOS-parameter (8)} Note. This attribute is associated with the localASEAbort event which generates a communication alarm notification. Note. This attribute is relevant when the dialogue is aborted by the local or the remote ASE. | GET | A | M |
|  | successfulCMLogonCounter Number of successful CM logon exchanges handled by the local CM AE instance (a cmLogonResponse APDU is sent or received). INTEGER {initial value=0} Rationale: it is of interest for accounting purpose to record the number and the type of service invocations by the CM-users. | GET | A | O |
|  | CMLogonIndicator Indicates whether the air-initiated CM logon has been rejected by the ground ASE for version incompatibility reason (D-START negative invoked or received). ENUMERATED {void(0), versionincompatibility(1)} Rationale: see above. | GET | A | O |
|  | successfulCMContactCounter Number of successful CM contact exchanges handled by the local AE instance (a cmContactResponse-contactSuccess APDU is sent or received). INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | unsuccessfulCMContactCounter Number of unsuccessful CM contact exchanges handled by the local AE instance (a cmContactResponse-contactNotSuccessful APDU is sent or received). INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | cMUpdateCounter Number of CM update exchanges performed by the local AE instance (a cmUpdate APDU is sent or received). INTEGER {initial value=0} Rationale: see above. Note. As this service can not be rejected by the peer, there is no counter for unsuccessful update. | GET | A | O |
|  | cMEndIndicator Indicates whether a CM end has been performed by the local AE instance (the D-END response/confirmation is sent/received). ENUMERATED {endnotperformed(0), endperformed(1)} Rationale: see above. Note. As this service can not be rejected by the peer, there is no indication for unsuccessful end. | GET | A | O |
|  | CMForwardResult Indicates the result of the CM forward. ENUMERATED {accept (0), version-incompatibility (1), forward-function-not-supported (2)} Rationale: see above. Note. This attribute is meaningless for air CM AE instances. | GET | A | O |
|  | roundTripDelayExceededIndicator The observed round trip delay exceeds the maximum delay authorized (for the requested class of communication). ENUMERATED {notexceeded(0), exceeded(1)} Note. This attribute is associated with the DelayExceeded event which generates a communication alarm notification. | GET | A | M |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | objectCreation Generated whenever an instance of the managed object class is created. The "sourceIndicator" parameter shall be set to the value 'resourceOperation'. None of the other optional parameters are used, with the exception of the "additionalInformation" field which contains the following parameters: - aEInstanceId, - remoteAETitle, - dialogueRole, - aTSCclassOfCommunicationService. Rationale: needed for the logging of every AE instance creation. |  | A | M |
|  | communicationAlarm Used to report a communication alarm condition in the operation of the aTNcMaeInstance managed object. It is used to report the following events: - localASEabort: generated when the ASE aborts the connection by itself. - Rationale: the exceptions detected by the ASEs are rare but are significant of a major problem in the ASE environment. - delayExceeded: generated when the observed delay for the round trip message exceeds the expected delay for the requested class. Rationale: these notifications inform the manager that the performances of the network have decreased. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes. |  | M | M |

## The aTNaDSae managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
| 1. | Managed Object Class | aTNaDSae There shall be one such MO per ATN Application Subsystem. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about an ADS ATN application entity. Its definition permits it to be created or deleted explicitly by management operation, but in some systems it will exist inherently and neither creation nor deletion by management operation will be possible | A | M |
| 2. | Naming attribute | CommunicationsEntityId | A | M |
| 3. | Superior in Naming Tree | applicationSubsystem |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | communicationsEntityId Naming attribute as defined in ISO/IEC 10165-5. GraphicString Initial value = “ aTNaDSae” | GET | A | M |
|  | operationalState Operational state as defined in ISO/IEC 10164-2 ENUMERATED {disabled(0), enabled(1)} Note. value is 'disabled' for any ATN system not supporting the ADS application entity. | GET | A | M |
|  | maxNumberOfADSAEInstances Maximum observed number of ADS AE instances running in parallel. INTEGER {initial value=0} Rationale: interesting to monitor the workload the ATN end system in terms of ADS activity. | GET | A | M |

Note: A number of configuration attributes are not proposed to be retained for standardisation in the ATN SARPs:

| AETitle ADSASEVersion ADSASEConfiguration ADSPriority ADSRER t-DC-1, t-DC-2, t-EC-1, t-EC-2, t-PC-1, t-PC-2, t-PC-3, t-EM-1, t-EM-2, t-EM-3, t-LI-1 | The AE title of the local ADS AE. The version of the ADS protocol operated by the ASE entity. This parameter identifies as well the Application Context in use. The configuration supported by the ASE entity (identified by the subsetting rules of the SARPs chapter 8). The application priority requested by the ADS application for all messages. The Residual Error Rate requested by the ADS application for all messages. ADS Technical Timers. |
| --- | --- |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | stateChange stateChange notification as defined in ISO/IEC 10164-2. Used to report the changes to the operationalState attribute. Rationale: it is a basic requirement for the manager to know whether a protocol entity is operational or not. Monitoring the operationalState may not be sufficient, since the polling period may not allow to detect problem quickly enough. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes. |  | M | M |

Note: A number of standard notifications are not proposed to be retained for standardisation in the ATN SARPs; the rationale is provided below:

| objectCreation | This notification allows the manager to dynamically discover that the managed system implements the MO, or to confirm a create operation, and allows to report initial MO attribute values. ATN systems are required to support one such MO. Manager are therefore assumed to a-priori know that one instance of this MO will exist. The stateChange notification will allow knowing when the MO is operational. No requirement for the logging of initial attribute values is identified for this MO. |
| --- | --- |

## The aTNaDSaeInstance managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
| 1. | Managed Object Class | aTNaDSaeInstance This MO represents an instance of the ADS ASE protocol machine. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about an instance of the ADS ATN application entity. There may be multiple instances of these MOs for a ADS ASE. Each corresponds to dialogue established with a peer ADS ASE. A ADS ASE instance is created and deleted automatically as part of system operation. | A | M |
| 2. | Naming attribute | aEInstanceId | A | M |
| 3. | Superior in Naming Tree | aTNaDSae |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | aEInstanceId Naming attribute GraphicString | GET | A | M |
|  | remoteAETitle The AE Title identifying the remote ADS AE in communication with the local ADS AE instance. OBJECT IDENTIFIER Rationale: to keep trace of the identity of the peer ES. Needed for investigation of any potential problem. | GET | A | M |
|  | aTSCclassOfCommunicationService The class of communication service as requested by the initiator ADS-user. ENUMERATED {'A'(0) to 'H'(7), no preference(8)} | GET | A | M |
|  | terminationMode The way the local AE instance has been terminated. ENUMERATED {normal-termination (0), local-user-abort (1), remote-user-abort (2), local-AE-abort (3), remote-AE-abort (4)} Rationale: interesting for the off-line analysis of the ASE's behavior. | GET | A | M |
|  | aSEabortReason The reason of the dialogue abort, if aborted. ENUMERATED {communications-service-failure (0), unrecoverable-system-error (1), invalid-PDU (2), sequence-error (3), timer-expiry (4), cannot-establish-contact (5), undefined-error (6), dialogue-end-not-accepted (7), unexpected-PDU (8), decoding-error (9), invalid-qos-parameter (10)} Note. This attribute is associated with the localASEAbort event which generates a communication alarm notification. Note. This attribute is relevant when the dialogue is aborted by the local or the remote ASE. | GET | A | M |
|  | aDSReportCounter Number of ADS reports sent or received by the local ASE Instance INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | successfulADSDemandCounter Number of successful ADS demand contracts fully supported and handled by the local ASE (an aDSreport with a positive acknowledgement is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | nonComplianceADSDemandCounter Number of ADS demand contracts partially supported and handled by the local ASE (a non compliance notification is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | unsuccessfulADSDemandCounter Number of unsuccessful ADS demand contracts handled by the local ASE (a negative acknowledgement is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | successfulADSEventCounter Number of successful ADS event contracts fully supported and handled by the local ASE (an aDSreport with a positive acknowledgement or a positive ADS-event-contract rsp/cnf is invoked or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | nonComplianceADSEventCounter Number of ADS event contracts partially supported and handled by the local ASE (a non compliance notification is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | unsuccessfulADSEventCounter Number of unsuccessful ADS event contracts handled by the local ASE (a negative acknowledgement is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | successfulADSPeriodicCounter Number of successful ADS periodic contracts fully supported and handled by the local ASE (an aDSreport with a positive acknowledgement or an positive ADS-periodic-contract rsp/cnf is invoked or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | nonComplianceADSPeriodicCounter Number of ADS periodic contracts partially supported and handled by the local ASE (a non compliance notification is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | unsuccessfulADSPeriodicCounter Number of unsuccessful ADS periodic contracts handled by the local ASE (a negative acknowledgement is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | aDSCancelContractCounter Number of ADS contracts cancellation handled by the local ASE (an ADS positive acknowledgement is sent or received after an ADS-cancel) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | aDSCancelAllContractsCounter Number of ADS contracts multiple cancellation handled by the local ASE (an ADS positive acknowledgement is sent or received after an ADS-cancel-all) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | aDSEmergencyContractsCounter Number of ADS emergency contracts handled by the local ASE (an initial ADS emergency report is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | successfulADSEmergencyModifyCounter Number of ADS emergency contract modifications handled by the local ASE (an ADS positive acknowledgement is sent or received after an ADS-modify-emergency-contract) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | unsuccessfulADSEmergencyModifyCounter Number of ADS emergency contract modifications handled by the local ASE (an ADS negative acknowledgement is sent or received after an ADS-modify-emergency-contract) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | aDSCancelEmergencyContractCounter Number of ADS emergency contract cancellations handled by the local ASE (an ADS cancel emergency acknowledgement is sent or received) INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | roundTripDelayExceededIndicator The observed round trip delay exceeds the maximum delay authorized (for the requested class of communication) ENUMERATED {notexceeded(0), exceeded(1)} Note. This attribute is associated with the DelayExceeded event which generates a communication alarm notification. | GET | A | M |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | objectCreation Generated whenever an instance of the managed object class is created. The "sourceIndicator" parameter shall be set to the value 'resourceOperation'. None of the other optional parameters are used, with the exception of the "additionalInformation" field which contains the following parameters: - aEInstanceId, - remoteAETitle, - aTSCclassOfCommunicationService. Rationale: needed for the logging of every AE instance creation. |  | A | M |
|  | communicationAlarm Used to report a communication alarm condition in the operation of the aTNaDSaeInstance managed object. It is used to report the following events: - localASEabort: generated when the ASE aborts the connection by itself. - Rationale: the exceptions detected by the ASEs are rare but are significant of a major problem in the ASE environment. - delayExceeded: generated when the observed delay for the round trip message exceeds the expected delay for the requested class. Rationale: these notifications informs the manager that the performances of the network have decreased. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes |  | M | M |

## The aTNaRFae managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
| 1. | Managed Object Class | aTNaRFae There shall be one such MO per ATN Application Subsystem. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about an ARF ATN application entity. Its definition permits it to be created or deleted explicitly by management operation, but in some systems it will exist inherently and neither creation nor deletion by management operation will be possible | A | M |
| 2. | Naming attribute | CommunicationsEntityId | A | M |
| 3. | Superior in Naming Tree | applicationSubsystem |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | communicationsEntityId Naming attribute as defined in ISO/IEC 10165-5. GraphicString Initial value = “ aTNaRFae” | GET | A | M |
|  | operationalState Operational state as defined in ISO/IEC 10164-2 ENUMERATED {disabled(0), enabled(1)} Note. value is 'disabled' for any ATN system not supporting the ARF application entity. | GET | A | M |
|  | maxNumberOfARFAEInstances Maximum observed number of ARF AE instances running in parallel. INTEGER {initial value=0} Rationale: interesting to monitor the workload the ATN end system in terms of ARF activity. | GET | A | M |

Note: A number of configuration attributes are not proposed to be retained for standardisation in the ATN SARPs:

| AETitle ARFASEVersion ARFASEConfiguration ARFPriority ARFRER t-RF-1, t-RF-2 | The AE title of the local AE. The version of the ARF protocol operated by the ASE entity. This parameter identifies as well the Application Context in use. The configuration supported by the ASE entity (identified by the subsetting rules of the SARPs chapter 8). The application priority requested by the ARF application for all messages. The Residual Error Rate requested by the ARF application for all messages. ARF technical timers. |
| --- | --- |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | stateChange stateChange notification as defined in ISO/IEC 10165-2. Used to report the changes to the operationalState attribute. Rationale: it is a basic requirement for the manager to know whether a protocol entity is operational or not. Monitoring the operationalState may not be sufficient, since the polling period may not allow to detect problem quickly enough. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes. |  | M | M |

Note: A number of standard notifications are not proposed to be retained for standardisation in the ATN SARPs; the rationale is provided below:

| objectCreation | This notification allows the manager to dynamically discover that the managed system implements the MO, or to confirm a create operation, and allows to report initial MO attribute values. ATN systems are required to support one such MO. Manager are therefore assumed to a-priori know that one instance of this MO will exist. The stateChange notification will allow knowing when the MO is operational. No requirement for the logging of initial attribute values is identified for this MO. |
| --- | --- |

## The aTNaRFaeInstance managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
| 1. | Managed Object Class | aTNaRFaeInstance This MO represents an instance of the ARF ASE protocol machine. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about an instance of the ARF ATN application entity. There may be multiple instances of these MOs for a ARF ASE. Each corresponds to dialogue established with a peer ARF ASE. A ARF ASE instance is created and deleted automatically as part of system operation. | A | M |
| 2. | Naming attribute | aEInstanceId | A | M |
| 3. | Superior in Naming Tree | aTNaRFae |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | aEInstanceId Naming attribute GraphicString | GET | A | M |
|  | remoteAETitle The AE Title identifying the remote ARF AE in communication with the local ARF AE instance. OBJECT IDENTIFIER Rationale: to keep trace of the identity of the peer ARF ES. Needed for investigation of any potential problem. | GET | A | M |
|  | aTSCclassOfCommunicationService The class of communication service as requested by the initiator ARF-user. ENUMERATED {'A'(0) to 'H'(7), no preference(8)} | GET | A | M |
|  | dialogueRole The role of the local AE instance during the establishment of the underlying dialogue. ENUMERATED {initiator(0), receptor(1)} | GET | A | M |
|  | terminationMode The way the local AE instance has been terminated. ENUMERATED {normal-termination (0), local-user-abort (1), remote-user-abort (2), local-AE-abort (3), remote-AE-abort (4)} Rationale: interesting for the off-line analysis of the ASE's behavior. | GET | A | M |
|  | aSEabortReason The reason of the dialogue abort, if aborted. ENUMERATED {communications-service-failure (0), unrecoverable-system-error (1), invalid-PDU (2), sequence-error (3), timer-expiry (4), cannot-establish-contact (5), undefined-error (6), dialogue-end-not-accepted (7), unexpected-PDU (8), decoding-error (9), invalid-qos-parameter (10)} Note. This attribute is associated with the localASEAbort event which generates a communication alarm notification. Note. This attribute is relevant when the dialogue is aborted by the local or the remote ASE. | GET | A | M |
|  | aDSForwardReportCounter Number of ADS forward reports sent or received by the local ASE Instance INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | ADSForwardResultIndicator Indicates whether the air-initiated ADS start forward has been rejected by the peer ASE for version incompatibility reason (D-START negative invoked or received). ENUMERATED {success(0), version-incompatibility(1)} Rationale: see above. | GET | A | O |
|  | roundTripDelayExceededIndicator The observed round trip delay exceeds the maximum delay authorized (for the requested class of communication) ENUMERATED {notexceeded(0), exceeded(1)} Note. This attribute is associated with the DelayExceeded event which generates a communication alarm notification. | GET | A | M |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | objectCreation Generated whenever an instance of the managed object class is created. The "sourceIndicator" parameter shall be set to the value 'resourceOperation'. None of the other optional parameters are used, with the exception of the "additionalInformation" field which contains the following parameters: - aEInstanceId, - remoteAETitle, - dialogueRole, - aTSCclassOfCommunicationService. Rationale: needed for the logging of every AE instance creation. |  | A | M |
|  | communicationAlarm Used to report a communication alarm condition in the operation of the aTNaRFaeInstance managed object. It is used to report the following events: - localASEabort: generated when the ASE aborts the connection by itself. - Rationale: the exceptions detected by the ASEs are rare but are significant of a major problem in the ASE environment. - delayExceeded: generated when the observed delay for the round trip message exceeds the expected delay for the requested class. Rationale: these notifications informs the manager that the performances of the network have decreased. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes |  | M | M |

## The aTNcPDLCae managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
| 1. | Managed Object Class | aTNcPDLCae There shall be one such MO per ATN Application Subsystem. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about a CPDLC ATN application entity. Its definition permits it to be created or deleted explicitly by management operation, but in some systems it will exist inherently and neither creation nor deletion by management operation will be possible | A | M |
| 2. | Naming attribute | CommunicationsEntityId | A | M |
| 3. | Superior in Naming Tree | applicationSubsystem |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | communicationsEntityId Naming attribute as defined in ISO/IEC 10165-5. GraphicString Initial value = “ aTNcPDLCae” | GET | A | M |
|  | operationalState Operational state as defined in ISO/IEC 10164-2 ENUMERATED {disabled(0), enabled(1)} Note. value is 'disabled' for any ATN system not supporting the CPDLC application entity. | GET | A | M |
|  | maxNumberOfCPDLCAEInstances Maximum observed number of CPDLC AE instances running in parallel. INTEGER {initial value=0} Rationale: interesting to monitor the workload the ATN end system in terms of CPDLC activity. | GET | A | M |

Note: A number of configuration attributes are not proposed to be retained for standardisation in the ATN SARPs:

| AETittle CPDLCASEVersion CPDLCASEConfiguration CPDLCPriority CPDLCRER CPDLC-start, CPDLC-forward, DSC-start | The AE title of the local AE The version of the CPDLC protocol operated by the ASE entity. This parameter identifies as well the Application Context in use. The configuration supported by the ASE entity (identified by the subsetting rules of the SARPs chapter 8). The application priority requested by the CPDLC application for all messages. The Residual Error Rate requested by the CPDLC application for all messages. CPDLC Technical Timers. |
| --- | --- |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | stateChange stateChange notification as defined in ISO/IEC 10165-2. Used to report the changes to the operationalState attribute. Rationale: it is a basic requirement for the manager to know whether a protocol entity is operational or not. Monitoring the operationalState may not be sufficient, since the polling period may not allow to detect problem quickly enough. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes. |  | M | M |

Note: A number of standard notifications are not proposed to be retained for standardisation in the ATN SARPs; the rationale is provided below:

| objectCreation | This notification allows the manager to dynamically discover that the managed system implements the MO, or to confirm a create operation, and allows to report initial MO attribute values. ATN systems are required to support one such MO. Manager are therefore assumed to a-priori know that one instance of this MO will exist. The stateChange notification will allow knowing when the MO is operational. No requirement for the logging of initial attribute values is identified for this MO. |
| --- | --- |

## The aTNcPDLCaeInstance managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
| 1. | Managed Object Class | aTNcPDLCaeInstance This MO represents an instance of the CPDLC AE protocol machine. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about an instance of the CPDLC ATN application entity. There may be multiple instances of these MOs for a CPDLC AE. Each corresponds to dialogue established with a peer CPDLC AE. A CPDLC AE instance is created and deleted automatically as part of system operation. | A | M |
| 2. | Naming attribute | aEInstanceId | A | M |
| 3. | Superior in Naming Tree | aTNcPDLCae |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | aEInstanceId Naming attribute GraphicString | GET | A | M |
|  | remoteAETitle The AE Title identifying the remote CPDLC AE in communication with the local CPDLC AE instance. OBJECT IDENTIFIER Rationale: to keep trace of the identity of the peer CPDLC ES. Needed for investigation of any potential problem. | GET | A | M |
|  | aTSCclassOfCommunicationService The class of communication service as requested by the initiator CPDLC-user. ENUMERATED {'A'(0) to 'H'(7), no preference(8)} | GET | A | M |
|  | dialogueRole The role of the local AE instance during the establishment of the underlying dialogue. ENUMERATED {initiator(0), receptor(1)} | GET | A | M |
|  | mode The mode of the local AE during the dialogue. ENUMERATED {cpdlc (0), dsc (1), forward (2)} Rationale: to keep trace of the type of CPDLC dialogue. | GET | A | M |
|  | terminationMode The way the local AE instance has been terminated. ENUMERATED {normal-termination (0), local-user-abort (1), remote-user-abort (2), local-AE-abort (3), remote-AE-abort (4)} Rationale: interesting for the off-line analysis of the ASE's behavior. | GET | A | M |
|  | aSEabortReason The reason of the dialogue abort, if aborted. ENUMERATED {communications-service-failure (0), unrecoverable-system-error (1), invalid-PDU (2), sequence-error (3), timer-expiry (4), cannot-establish-contact (5), undefined-error (6), dialogue-end-not-accepted (7), unexpected-PDU (8), decoding-error (9), invalid-qos-parameter (10), user-abort-undefined (11), no-message-id-available (12), duplicate-message-id (12), no-longer-NDA (13), CDA-abort (14), command-termination (15), invalid-response(16)} Note. This attribute is associated with the localASEAbort event which generates a communication alarm notification. Note. This attribute is relevant when the dialogue is aborted by the local or the remote ASE. | GET | A | M |
|  | cPDLCDialogueEstablishmentResult The result of the air-ground CPDLC/DSC dialogue establishment (a positive or a negative D-START res/cnf is invoked or received). ENUMERATED {success(0), failure(1)} Rationale: see above. Note. This attribute is meaningless when the mode attribute has the value "forward". | GET | A | O |
|  | downlinkMessageCounter Number of CPDLC Downlink messages sent or received by the local ASE Instance INTEGER {initial value=0} Rationale: see above. Note. This attribute is meaningless when the mode attribute has the value "forward" or the cPDLCDialogueEstablishmentResult attribute has the value "failure". | GET | A | O |
|  | uplinkMessageCounter Number of CPDLC Uplink messages sent or received by the local ASE Instance INTEGER {initial value=0} Rationale: see above. Note. This attribute is meaningless when the mode attribute has the value "forward" or the cPDLCDialogueEstablishmentResult attribute has the value "failure". | GET | A | O |
|  | CPDLCForwardResultIndicator Indicates whether the CPDLC forward has been rejected by the peer ASE for version incompatibility reason (D-START negative invoked or received). ENUMERATED {success(0), version-incompatibility(1), ground-forward-function-not-supported(2)}} Rationale: see above. | GET | A | O |
|  | roundTripDelayExceededIndicator The observed round trip delay exceeds the maximum delay authorized (for the requested class of communication) ENUMERATED {notexceeded(0), exceeded(1)} Note. This attribute is associated with the DelayExceeded event which generates a communication alarm notification. | GET | A | M |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | objectCreation Generated whenever an instance of the managed object class is created. The "sourceIndicator" parameter shall be set to the value 'resourceOperation'. None of the other optional parameters are used, with the exception of the "additionalInformation" field which contains the following parameters: - aEInstanceId, - remoteAETitle, - dialogueRole, - aTSCclassOfCommunicationService. Rationale: needed for the logging of every AE instance creation. |  | A | M |
|  | communicationAlarm Used to report a communication alarm condition in the operation of the aTNcPDLCaeInstance managed object. It is used to report the following events: - localASEabort: generated when the ASE aborts the connection by itself. - Rationale: the exceptions detected by the ASEs are rare but are significant of a major problem in the ASE environment. - delayExceeded: generated when the observed delay for the round trip message exceeds the expected delay for the requested class. Rationale: these notifications informs the manager that the performances of the network have decreased. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes. |  | M | M |

## The aTfISae managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
| 1. | Managed Object Class | aTNfISae There shall be one such MO per ATN Application Subsystem. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about a FIS ATN application entity. Its definition permits it to be created or deleted explicitly by management operation, but in some systems it will exist inherently and neither creation nor deletion by management operation will be possible | A | M |
| 2. | Naming attribute | CommunicationsEntityId | A | M |
| 3. | Superior in Naming Tree | applicationSubsystem |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | communicationsEntityId Naming attribute as defined in ISO/IEC 10165-5. GraphicString Initial value = “ aTNfISae” | GET | A | M |
|  | operationalState Operational state as defined in ISO/IEC 10164-2. ENUMERATED {disabled(0), enabled(1)} Note. value is 'disabled' for any ATN system not supporting the FIS application entity. | GET | A | M |
|  | maxNumberOfFISAEInstances Maximum observed number of FIS AE instances running in parallel. INTEGER {initial value=0} Rationale: interesting to monitor the workload the ATN end system in terms of FIS activity. | GET | A | M |

Note: A number of configuration attributes are not proposed to be retained for standardisation in the ATN SARPs:

| AETitle FISASEVersion FISASEConfiguration ATISPriority FISRER t-DC-1, t-DC-2, t-UC-1, t-UC-2, t-UC-3, t-CL-1, t-LI-1 | The AE title of the local AE. The version of the FIS protocol operated by the ASE entity. This parameter identifies as well the Application Context in use. The configuration supported by the ASE entity (identified by the subsetting rules of the SARPs chapter 8). The application priority requested by the FIS application for all messages. The Residual Error Rate requested by the FIS application for all messages. FIS Technical Timers. |
| --- | --- |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | stateChange stateChange notification as defined in ISO/IEC 10165-2. Used to report the changes to the operationalState attribute. Rationale: it is a basic requirement for the manager to know whether a protocol entity is operational or not. Monitoring the operationalState may not be sufficient, since the polling period may not allow to detect problem quickly enough. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes. |  | M | M |

Note: A number of standard notifications are not proposed to be retained for standardisation in the ATN SARPs; the rationale is provided below:

| objectCreation | This notification allows the manager to dynamically discover that the managed system implements the MO, or to confirm a create operation, and allows to report initial MO attribute values. ATN systems are required to support one such MO. Manager are therefore assumed to a-priori know that one instance of this MO will exist. The stateChange notification will allow knowing when the MO is operational. No requirement for the logging of initial attribute values is identified for this MO. |
| --- | --- |

## The aTNfISaeInstance managed object

### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
| 1. | Managed Object Class | aTNfISaeInstance This MO represents an instance of the FIS ASE protocol machine. This subclass of “Rec. X.721 ¦ ISO/IEC 10165-2:1992”: top holds reference information about an instance of the FIS ATN application entity. There may be multiple instances of these MOs for a FIS ASE. Each corresponds to dialogue established with a peer FIS ASE. A FIS ASE instance is created and deleted automatically as part of system operation. | A | M |
| 2. | Naming attribute | aEInstanceId | A | M |
| 3. | Superior in Naming Tree | aTNfISae |  |  |

### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | aEInstanceId Naming attribute GraphicString | GET | A | M |
|  | remoteAETitle The AE Title identifying the remote FIS AE in communication with the local FIS AE instance. OBJECT IDENTIFIER Rationale: to keep trace of the identity of the peer CPDLC ES. Needed for investigation of any potential problem. | GET | A | M |
|  | aTSCclassOfCommunicationService The class of communication service as requested by the initiator FIS-user. ENUMERATED {'A'(0) to 'H'(7), no preference(8)} |  |  |  |
|  | terminationMode The way the local AE instance has been terminated. ENUMERATED {normal-termination (0), local-user-abort (1), remote-user-abort (2), local-AE-abort (3), remote-AE-abort (4)} Rationale: interesting for the off-line analysis of the ASE's behavior. | GET | A | M |
|  | aSEabortReason The reason of the dialogue abort, if aborted. ENUMERATED {timer-expiration (0), protocol-error (1), sequence-error (2), decoding-error (3), unrecoverable-internal-error (4), invalid-contract-number (5), dialogue-end-not-supported (6), undefined (7), invalid-qos-parameter (8)} Note. This attribute is associated with the localASEAbort event which generates a communication alarm notification. Note. This attribute is relevant when the dialogue is aborted by the local or the remote ASE. | GET | A | M |
|  | fISReportCounter Number of FIS reports sent or received by the local ASE Instance. INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | successfulFISDemandCounter Number of successful FIS demand contracts fully supported and handled by the local ASE (an FISAccept APDU is sent or received after a FIS-demand-contract). INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | unsuccessfulFISDemandCounter Number of unsuccessful FIS demand contracts handled by the local ASE (a FISReject APDU is sent or received after a FIS-demand-contract). INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | successfulFISUpdateCounter Number of successful FIS update contracts fully supported and handled by the local ASE (an FISAccept APDU is sent or received after a FIS-update-contract). INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | unsuccessfulFISUpdateCounter Number of unsuccessful FIS update contracts handled by the local ASE (a FISReject APDU is sent or received after a FIS-update-contract). INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | fISCancelUpdateContractCounter Number of FIS update contracts cancellation handled by the local ASE (an FISCancelUpdateAccept APDU is sent or received). INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | fISCancelAllContractsCounter Number of FIS contracts multiple cancellation handled by the local ASE (an FISCancelContractsAccept APDU is sent or received). INTEGER {initial value=0} Rationale: see above. | GET | A | O |
|  | roundTripDelayExceededIndicator The observed round trip delay exceeds the maximum delay authorized (for the requested class of communication). ENUMERATED {notexceeded(0), exceeded(1)} Note. This attribute is associated with the DelayExceeded event which generates a communication alarm notification. | GET | A | M |

### Actions

None.

### Notifications

| Index | Notification Name (Description) |  | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | objectCreation Generated whenever an instance of the managed object class is created. The "sourceIndicator" parameter shall be set to the value 'resourceOperation'. None of the other optional parameters are used, with the exception of the "additionalInformation" field which contains the following parameters: - aEInstanceId, - remoteAETitle, - aTSCclassOfCommunicationService. Rationale: needed for the logging of every AE instance creation. |  | A | M |
|  | communicationAlarm Used to report a communication alarm condition in the operation of the aTNfISaeInstance managed object. It is used to report the following events: - localASEabort: generated when the ASE aborts the connection by itself. - Rationale: the exceptions detected by the ASEs are rare but are significant of a major problem in the ASE environment. - delayExceeded: generated when the observed delay for the round trip message exceeds the expected delay for the requested class. Rationale: these notifications inform the manager that the performances of the network have decreased. |  | A | M |
|  | objectDeletion Generated whenever an instance of the managed object class is deleted. The "sourceIndicator" parameter shall be set to the value ‘resourceOperation’. The "attributeList" parameter shall be used to report the values of the MO attributes. None of the other optional parameters are used. Rationale: needed for the logging of the actual value of the MO attributes |  | M | M |

## Managed Objects for Systems Management Applications

ISO 10165-9, "Systems management protocol machine managed objects" is under development within ISO, and should be assessed for utility within the ATN architecture.