# ACSE SPECIFICATION

Note. —  The ACSE requirements are described in many cases by means of completed protocol implementation conformance statement (PICS) proforma tables.  In such tables, the “Ref.” column contains a reference to the relevant section in the ACSE PICS proforma ISO/IEC 8650-2.

## Protocol details

The specification of the ACSE protocol supported shall([158])  be as defined in Table 4.6-1.

Table 4.6-1.  Identification of ACSE Protocol Specification

| Identification of Protocol Specification | ATN Support | Comments |
| --- | --- | --- |
| ISO/IEC 8650-1:1995 | M | See note |

Note. —  this is the second edition of the ACSE protocol specification.

## Protocol versions

The version of the ACSE protocol supported shall([159])  be as specified in Table 4.6-2.

Table 4.6-2.  Identification of ACSE Protocol version

| Ref. |  | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.4.2/1 | Version 1 | O.1 | M | A-V1 |
| A.A.4.2/2 | Version 2 | O.1 |  |  |

O.1:  The ISO PICS requires support of the implementation of only one version of the protocol to be described.

## Supported roles

### Association establishment

#### The supported roles for Association Establishment shall([160])  be as specified in Table 4.6-3.

Table 4.6-3.  ACSE Roles for Association Establishment

| Ref. | Capability | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.6.1/1 | Association initiator | O.2 | See text | A-CON_initiator |
| A.A.6.1/2 | Association responder | O.2 | See text | A-CON_responder |

O.2:  The ISO standard requires a conforming implementation to support at least one of the roles.

#### Either one or both of the ACSE roles “Association initiator” or “Association responder” shall([161])  be supported.

### Normal Release procedure

#### The supported roles for the Normal Release procedure shall([162])  be as specified in Table 4.6-4.

Table 4.6-4.  ACSE Roles for Normal Release

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.6.2/1 | Initiator | O | See text | A-REL_requestor |
| A.A.6.2/2 | Responder | O | See text | A-REL_acceptor |

#### Either one or both of the ACSE Normal Release roles “Initiator” or “Responder” shall([163])  be supported.

#### The ACSE Release Responder shall([164])  be allowed to give a negative response, despite the fact that the session Negotiated Release functional unit is not selected for the association.

Note. —  The above provision waives the ISO/IEC 8649 requirement that the Responder may give a negative response only if session Negotiated Release is selected.  This is possible because, for ATN, the ACSE release PDUs do not map directly to the Presentation release service; they are re-mapped by the CF to P-DATA.

### Abnormal Release procedure

The supported roles for the Abnormal Release procedure shall([165])  be as specified in Table 4.6-5.

Table 4.6-5.  ACSE Roles for Abnormal Release

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.6.3/1 | Initiator | M | M |  |
| A.A.6.3/2 | Responder | M | M |  |

## Protocol mechanisms

### The ACSE protocol mechanisms supported shall([166])  be as specified in Table 4.6-6.

Table 4.6-6.  ACSE Protocol Mechanisms Supported

| Ref. | Protocol Mechanism | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.7/1 | Normal mode | O.4 | M |  |
| A.A.7/2 | X.410-1984 mode | O.4 | X |  |
| A.A.7/2 | Rules for extensibility | M | M |  |
| A.A.7/4 | Supports operation of Session version 2 | O | M | S-O-SESS-V2 |

O.4:  The ISO standard requires either Normal mode or X.410-1984 mode or both to be supported.

### Extensibility and Encoding

#### For the purposes of this specification, the abstract syntax module defined in clause 9 of the ACSE protocol specification shall([167])  be augmented with the ASN.1 extensibility notation, as specified in ISO/IEC 8650-1 Amendment 2.

#### The system shall([168])  support that encoding which results from applying the ASN.1 packed encoding rules (basic, unaligned variant), as specified in ISO/IEC 8825-2, to the abstract syntax module specified in 4.6.4.2.1.

#### Packed encoding (basic, unaligned) shall([169])  be used for encoding all ACSE Protocol Control Information (PCI) for interchange.

## ACSE Functional units

The ACSE functional units selected shall([170])  be as specified in Table 4.6-7.

Table 4.6-7.  Selection of ACSE Functional Units

| Ref. | Role | ISO Status | ATN Support | Mnemonic |
| --- | --- | --- | --- | --- |
| A.A.8/1 | Normal mode | M | M |  |
| A.A.8/2 | Authentication | O | C1 | A-FU(AU) |

C1:  If the Dialogue Service user requires the use of the Security Requirements parameter of the D-START primitives, then M, else O.

## Supported APDUs

### The ACSE Protocol data units supported shall([171])  be as specified in Table 4.6-8.

Table 4.6-8  Supported ACSE Protocol Data Units

| Ref. | APDU |  | Sender |  |  |  | Receiver |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ISO Status |  | ATN Support |  | ISO Status |  | ATN Support |  |  |  |
| A.A.9/1 |  | AARQ |  | C1 |  | M |  | C2 |  | M |  |  |
| A.A.9/2 |  | AARE |  | C2 |  | M |  | C1 |  | M |  |  |
| A.A.9/3 |  | RLRQ |  | C3 |  | M |  | C4 |  | M |  |  |
| A.A.9/4 |  | RLRE |  | C4 |  | M |  | C3 |  | M |  |  |
| A.A.9/5 |  | ABRT |  | C5 |  | M |  | C5 |  | M |  |  |

C1:  if [A-CON_initiator] then M else N/A

C2:  if [A-CON_responder] then M else N/A

C3:  if [A-REL_requestor] then M else N/A

C4:  if [A-REL_acceptor] then M else N/A

C5:  if [S-O-SESS-V2] then M else N/A

### Supported APDU parameters

#### A-Associate-request (AARQ)

##### The parameters in the AARQ APDU shall([172])  be supported as specified in Table 4.6-9.

Table 4.6-9.  Supported AARQ Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.1/1 | Protocol Version | C6 | X | C2 | M |
| A.A.10.1/2 | Application Context Name | C1 | M | C2 | M |
| A.A.10.1/3 | Calling AP title | C6 | O | C2 | M |
| A.A.10.1/4 | Calling AE qualifier | C6 | O | C2 | M |
| A.A.10.1/5 | Calling AP invocation-identifier | C6 | O | C2 | M |
| A.A.10.1/6 | Calling AE invocation-identifier | C6 | O | C2 | M |
| A.A.10.1/7 | Called AP title | C6 | X | C2 | M |
| A.A.10.1/8 | Called AE qualifier | C6 | X | C2 | M |
| A.A.10.1/9 | Called AP invocation-identifier | C6 | X | C2 | See text |
| A.A.10.1/10 | Called AE invocation-identifier | C6 | X | C2 | See text |
| A.A.10.1/11 | ACSE-requirements | C8 | See text | C9 | M |
| A.A.10.1/12 | Authentication-mechanism name | C8 | X | C9 | N/A |
| A.A.10.1/13 | Authentication-value | C8 | See text | C9 | M |
| A.A.10.1/14 | Implementation information | C6 | X | C7 | O |
| A.A.10.1/15 | User information | C6 | M | C7 | M |

C1:  if [A-CON_initiator] then M else N/A

C2:  if [A-CON_responder] then M else N/A

C6:  if [A-CON_initiator] then O else N/A

C7:  if [A-CON_responder] then O else N/A

C8:  if [A-CON_initiator and A-FU(AU)] then M else N/A

C9:  if [A-CON_responder and A-FU(AU)] then M else N/A

##### The AARQ parameters “ACSE-Requirements” and “Authentication-value” shall([173])  be supported, for sending, only if the connection initiator role (A-CON_initiator) and the Authentication functional unit (A-FU(AU)) are supported.

Note.— The ATN specification is non-conformant to the ISO PICS proforma, in that the “Authentication-mechanism-name” parameter is not supported for sending.

##### The AARQ parameters “ACSE-Requirements” and “Authentication-value” shall([174])  be supported for receiving if the connection responder role (A-CON_responder) is supported, but are ignored if the Authentication functional unit (A-FU(AU)) is not supported by the responder.

Note.— The ATN specification is non-conformant to the ISO PICS proforma, in that the “Authentication-mechanism-name” parameter is “N/A”  for receiving, if the Authentication functional unit is selected, and “ACSE-requirements” and “Authentication-value” are “M” for receiving, even if the Authentication functional unit is not supported.

##### The AARQ parameters “Called AP invocation-identifier” and “Called AE invocation-identifier” shall([175])  be supported, for receiving, if the Association Responder role is supported.

#### A-Associate-response (AARE)

##### The parameters in the AARE APDU shall([176])  be supported as specified in Table 4.6-10.

Table 4.6-10.  Supported AARE Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.2/1 | Protocol Version | C7 | X | C1 | M |
| A.A.10.2/2 | Application Context Name | C2 | M | C1 | M |
| A.A.10.2/3 | Responding AP title | C7 | X | C1 | M |
| A.A.10.2/4 | Responding AE qualifier | C7 | X | C1 | M |
| A.A.10.2/5 | Responding AP invocation-identifier | C7 | X | C1 | M |
| A.A.10.2/6 | Responding AE invocation-identifier | C7 | X | C1 | M |
| A.A.10.2/7 | Result | C2 | M | C1 | M |
| A.A.10.2/8 | Result source - diagnostic | C10 | M | C11 | M |
| A.A.10.2/9 | ACSE-requirements | C9 | See text | C8 | See text |
| A.A.10.2/10 | Authentication-mechanism name | C9 | X | C8 | N/A |
| A.A.10.2/11 | Authentication-value | C9 | See text | C8 | See text |
| A.A.10.2/12 | Implementation information | C7 | X | C6 | O |
| A.A.10.2/13 | User information | C7 | M | C6 | M |

C1:  if [A-CON_initiator] then M else N/A

C2:  if [A-CON_responder] then M else N/A

C6:  if [A-CON_initiator] then O else N/A

C7:  if [A-CON_responder] then O else N/A

C8:  if [A-CON_initiator and A-FU(AU)] then M else N/A

C9:  if [A-CON_responder and A-FU(AU)] then M else N/A

C10:  if [A-CON_responder] then (if [A-FU(AU)] then M (with a value range of 0 to 14) else M (with a value range of 0 to 10) else N/A

C11:  if [A-CON_initiator] then (if [A-FU(AU)] then M (with a value range of 0 to 14) else M (with a value range of 0 to 10) else N/A

##### The AARE parameters “ACSE-Requirements”, Authentication-mechanism-name” and “Authentication-value” shall([177])  be supported, for sending, only if the connection responder role (A-CON_responder) and the Authentication functional unit (A-FU(AU)) are supported.

##### The AARE parameters “ACSE-Requirements” and “Authentication-value” shall([178])  be supported, for receiving, only if the connection initiator role (A-CON_initiator) and the Authentication functional unit (A-FU(AU)) are supported.

#### A-Release-request (RLRQ)

The parameters in the RLRQ APDU shall([179])  be supported as specified in Table 4.6-11.

Table 4.6-11.  Supported RLRQ Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.3/1 | Reason | C12 | M | C4 | M |
| A.A.10.3/2 | User information | C12 | M | C4 | M |

C4:  if [A-REL_acceptor] then M else N/A

C12:  if [A-REL_requestor] then O else N/A

#### A-Release-response (RLRE)

The parameters in the RLRE APDU shall([180])  be supported as specified in Table 4.6-12.

Table 4.6-12  Supported RLRE Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.4/1 | Reason | C13 | M | C3 | M |
| A.A.10.4/2 | User information | C13 | M | C3 | M |

C3:  if [A-REL_requestor] then M else N/A

C13:  if [A-REL_acceptor] then O else N/A

#### A-Abort (ABRT)

The parameters in the ABRT APDU shall([181])  be supported as specified in Table 4.6-13.

Table 4.6-13.  Supported ABRT Parameters

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Parameter | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.10.5/1 | Abort source | M | M | M | M |
| A.A.10.5/2 | Diagnostic | C14 | M | C14 | M |
| A.A.10.5/3 | User information | O | M | M | M |

C14:  if [A-FU(AU)] then M else N/A

### Supported parameter forms

#### AE title name form

The Application Entity Title parameter shall([182])  be supported in the forms specified in Table 4.6-14.

Table 4.6-14.  AE Title Name Form

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Syntax form | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.11.1/1 | Form 1 (Directory name) | O.5 | X | M | O |
| A.A.11.1/2 | Form 2 (Object identifier and integer) | O.5 | M | M | M |

O.5:  The ISO standard requires a conforming implementation to support at least one of the forms.

#### Authentication value form

##### The Authentication value parameter shall([183])  be supported in the forms specified in Table 4.6-15.

Table 4.6-15. Authentication Value Form

Prerequisite:  A-FU(AU)

|  |  | Sender |  | Receiver |  |
| --- | --- | --- | --- | --- | --- |
| Ref. | Authentication value form | ISO Status | ATN Support | ISO Status | ATN Support |
| A.A.11.2/1 | GraphicString | O.6 | See text | C14 | M |
| A.A.11.2/2 | BIT STRING | O.6 | See text | C14 | M |
| A.A.11.3/3 | EXTERNAL | O.6 | See text | C14 | M |
| A.A.11.4/4 | Other | O.6 | X | C14 | N/A |

O.6:  The ISO standard requires a conforming implementation to support at least one of the forms.

C14:  if [A-FU(AU)] then M else N/A

##### If the authentication functional unit is supported, at least one of the Authentication-value forms listed in Table 4.6-15 shall([184])  be implemented for sending.

## Mapping to the Presentation Service

The mapping of ACSE APDUs and parameters to presentation service primitives shall([185])  be performed by the CF as specified in 4.3, which takes precedence over the direct mapping defined in clause 8 of ISO/IEC 8650-1.