INTRODUCTION

APPENDIX E:  CORRECTIONS TO WORD VERSION 3.0

OF THE

CONTROLLER PILOT DATA LINK COMMUNICATION (CPDLC) SARPS

## Introduction

### This application allows data link communication between controllers and pilots.

### The CPDLC application provides the capability to establish, manage, and terminate CPDLC dialogues between ATS ground and aircraft system peers.  Once a dialogue is established, CPDLC provides for controller/pilot message exchange.

### The CPDLC application also provides the capability to establish, manage, and terminate CPDLC dialogues between two ATC ground system peers for the purpose of ground/ground forwarding of a CPDLC message.

- Note 1. — Structure:

- 2.3 1:  INTRODUCTION contains the structure of 2.3, and a summary of the functional capabilities of CPDLC.

- 2.3..2:  GENERAL REQUIREMENTS contains the CPDLC version number, and error processing requirements.

- 2.3. 3:  ABSTRACT SERVICE DEFINITION contains the description of the abstract service provided by the CPDLC Application Service Element (CPDLC-ASE).

- 2.3. 4:  FORMAL DEFINITION OF MESSAGES contains the formal definition of messages exchanged by CPDLC ASEs using Abstract Syntax Notation Number One (ASN.1).

- 2.3. 5: PROTOCOL DEFINITION describes the exchanges of messages allowed by the CPDLC protocol, as well as time constraints and CPDLC-ASE protocol descriptions and state tables.

- 2.3. 6:  COMMUNICATION REQUIREMENTS  contains the requirements that the CPDLC application imposes on the underlying communication system.

- 2.3. 7:  CPDLC USER REQUIREMENTS contains requirements imposed on the user of the CPDLC ASE service and message description tables.

- 2.3. 8:  SUBSETTING RULES defines the conformant subsets for the CPDLC-ASE.

- Note 2. — Functional Descriptions

- The Controller-Pilot Message Exchange Function defines a method for a controller and pilot to exchange messages via data link.  This function provides messages for the following :

  - general information exchange;

  - clearance

    - delivery,

    - request, and

    - response;

  - altitude/identity surveillance;

  - monitoring of current/planned position;

  - advisories

    - request and

    - delivery;

  - system management functions; and

  - emergency situations.

- The Transfer of Data Authority Function provides the capability for the current data authority to designate another ground system as the next data authority.  A CPDLC dialogue can be opened with or by the next data authority at a time before becoming the current data authority.  This capability is intended to prevent a loss of communication that would occur if the next data authority were prevented from actually setting up a dialogue with an aircraft until it became the current data authority.  The designation of a next data authority is accomplished using a CPDLC message.

- The Down Stream Clearance Function provides the capability for an aircraft to contact an air traffic service unit which is not the current data authority for the purpose of receiving a down stream clearance.  This information is exchanged using CPDLC message(s).

- The Ground Forward Function provides the capability for a ground system to forward information received in a CPDLC message to another ground system. The ground forwarding function can be used by the controlling data authority to forward an aircraft request to the next data authority, so that an aircraft does not need to issue the same request again.  This function can also be used by a downstream data authority to pass a message to a current data authority for transmission by the current data authority to an aircraft.  This information is exchanged using CPDLC message(s).  It is a one-way forwarding of information with an indication of success, failure or non-support  from the receiving ground system.

- Note 3. — See 2.3.7 for detailed CPDLC message intent/use descriptions.

# GENERAL REQUIREMENTS

## CPDLC ASE Version Number

### The CPDLC-air-ASE and CPDLC-ground-ASE version numbers shall both be set to one.

## Error Processing Requirements

### In the event of information input by the CPDLC-user being incompatible with that able to be processed by the system, the CPDLC-user shall be notified.

### In the event of a CPDLC-user invoking a CPDLC service primitive when the CPDLC-ASE is not in a state specified in 2.3.5, the CPDLC-user shall be notified.

# THE ABSTRACT SERVICE

## Service Description

### An implementation of either the CPDLC ground based service or the CPDLC air based service shall exhibit external behavior consistent with having implemented a CPDLC-ground-ASE, or CPDLC-air ASE respectively, with the following abstract service interface primitives, making them available to the CPDLC-ground-user or CPDLC-air-user respectively.

Note 1. — There is no requirement to implement the service in a CPDLC product; however, it is necessary to implement the ground based and air based system in such a way that it will be impossible to detect (from the peer system) whether or not an interface has been built.

### Note 2. — This chapter defines the abstract service interface for the CPDLC service.  The CPDLC-ASE abstract service is described in this chapter from the viewpoint of the CPDLC-air-user, the CPDLC-ground-user and the CPDLC-service-provider.

- Note 3. — This chapter defines the static behaviour (i.e., the format) of the CPDLC abstract service.  Its dynamic behaviour (i.e., how it is used) is described in 2.3.7.

- Note 4. — Figure 2.3.3-1 shows the functional model of the CPDLC Application. The functional modules identified in this model are the following :

- the CPDLC-user,

- the CPDLC Application Entity (CPDLC-AE) service,

- the CPDLC-AE,

- the CPDLC Control Function (CPDLC-CF),

- the CPDLC Application Service Element (CPDLC-ASE) service,

- the CPDLC-ASE, and

- the Dialogue Service (DS).

Figure 2.3.3-1  Functional Model of the CPDLC Application

- Note 4. — The CPDLC-user represents the operational part of the CPDLC system.  This user does not perform the communication functions but relies on a communication service provided to it via the CPDLC-AE through the CPDLC-AE service.  The individual actions possible through the  CPDLC-AE service called service primitives..

- Note 5. — The CPDLC-AE consists of several elements including the CPDLC-ASE and the CPDLC-CF.

- Note 6. — The CPDLC-ASE is the element in the communication system which executes the CPDLC specific protocol.  In other words, it takes care of the CPDLC specific service primitive sequencing , message creation, timer management, error and exception handling.

- Note 7. —This CPDLC-CF is responsible for mapping service primitives received from one element (such as the CPDLC-ASE and the CPDLC-user) to service primitives of other abstract elements..

- Note 8. — The CPDLC-ASE has two abstract boundaries with the CPDLC-CF:  the CPDLC-ASE service and the dialogue service.  The CPDLC-CF maps CPDLC-AE service primitives to CPDLC-ASE service primitives transparently and vice versa.  The CPDLC-CF maps dialogue service primitives to other abstract elements in the CPDLC-AE and the underlying communication service, and vice versa.].

## The CPDLC-ASE Abstract Service

### The CPDLC-ASE abstract service shall consist of a subset of the following services as allowed in 2.3.8:

- CPDLC-start service as defined in 2.3.3.5,

- DSC-start service as defined in 2.3.3.6,

- CPDLC-message service as defined in 2.3.3.7,

- CPDLC-end service as defined in  2.3.3.8,

- DSC-end service as defined in 2.3.3.9,

- CPDLC-forward service as defined in 2.3.3.10,

- CPDLC-user-abort service as defined in 2.3.3.11, and

- CPDLC-provider-abort service as defined in 2.3.3.12.

- Note. 1 — For a given primitive, the presence of each parameter is described by one of the following values in the parameter tables in 2.3.3.

| a) | blank | not present; |
| --- | --- | --- |
| b) | C | conditional upon some predicate explained in the text; |
| c) | C(=) | conditional upon the value of the parameter to the left being present, and equal to that value; |
| d) | M | mandatory; |
| e) | M(=) | mandatory, and equal to the value of the parameter to the left; |
| f) | U | user option. |

- Note 2. — The following abbreviations are used in this document:

- Req - request; data is input by CPDLC-user initiating the service to its respective ASE,

- Ind - indication; data is indicated by the receiving ASE to its respective CPDLC-user,

- Rsp - response; data is input by receiving CPDLC user to its respective ASE, and

- Cnf - confirmation; data is confirmed by the initiating ASE to its respective CPDLC-user.

- Note 3. — An unconfirmed service allows a message to be transmitted in one direction without providing a corresponding response.

- Note 4. — A confirmed service provides end-to-end confirmation that a message sent by one user was received by its peer user.

- Note 5. —An abstract syntax is a syntactical description of a parameter which does not imply a specific implementation.  Only when the CPDLC-ASE maps a parameter into an APDU field, or vise versa, the abstract syntax of the parameter is described by using the ASN.1 of 2.3.4 for this field.

## CPDLC-start Service

- Note 1. — The CPDLC-start service is used by the CPDLC-air-user or CPDLC-ground-user to establish a CPDLC dialogue.  It is a confirmed service.

- Note 2. — Once a CPDLC dialogue is established it remains open until explicitly closed.  (See CPDLC-end and CPDLC-abort services.)

### The CPDLC-start service shall contain the primitives and parameters as presented Table 2.3.3-1.

Table 2.3.3-1.  CPDLC-start Service Parameters

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| Called Peer Identifier | M |  |  |  |
| Calling Peer Identifier | M | M(=) |  |  |
| CPDLC Message | U | C(=) |  |  |
| Reject Reason |  |  | C | C(=) |
| Result |  |  | M | M(=) |
| Class of Communication Service | U | M |  |  |

### Called Peer Identifier

- Note 1. — If the service is ground initiated, this parameter contains the addressed aircraft’s 24-bit ICAO address.

- Note 2. — If the service is air initiated, this parameter contains the addressed ground system’s facility designation.

#### If the service is ground initiated, the Called Peer Identifier parameter value shall conform to the abstract syntax 24-bit aircraft-id.

#### If the service is air initiated, the Called Peer Identifier parameter value shall conform to the abstract syntax eight-character ICAO facility designator.

### Calling Peer Identifier

- Note 1. — If the service is ground initiated, this parameter contains the sending ground system’s facility designation.

- Note 2. — If the service is air initiated, this parameter contains the sending aircraft’s 24-bit ICAO address.

#### If the service is ground initiated, the Calling Peer Identifier parameter value shall conform to the abstract syntax eight-character ICAO facility designator.

#### If the service is air initiated, the Calling Peer Identifier parameter value shall conform to the abstract syntax 24-bit aircraft-id.

### CPDLC Message

- Note. — The CPDLC-user can use this parameter to send a CPDLC message to its peer user.

#### The CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCUplinkMessage, if supplied by the CPDLC-ground-user.

#### The CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCDownlinkMessage, if supplied by the CPDLC-air-user.

### Reject Reason

- Note. — This parameter is used to provide a reason for rejecting a CPDLC dialogue.

#### If, and only if, the CPDLC-user rejects the request to open a CPDLC dialogue, the CPDLC user shall provide a reason (a CPDLC message) for the rejection.

#### The Reject Reason parameter value shall conform to the ASN.1 abstract syntax ATCUplinkMessage if supplied by the CPDLC-ground-user.

#### The Reject Reason parameter value shall conform to the ASN.1 abstract syntax ATCDownlinkMessage if supplied by the CPDLC-air-user.

### Result

- Note. — This parameter is used to indicate whether or not a requested CPDLC dialogue is accepted.

#### This parameter shall have one of two abstract values: “accepted” or “rejected”.

### Class of Communication Service

- Note. — This parameter contains the value of the required class of communication service.  If not specified by the CPDLC -user, this indicates that there is no routing preference.

#### Where specified by the CPDLC-user, the Class of Communication Service parameter shall have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note. — Class of Communication service parameter values are detailed in 1.2.

## DSC-start Service

- Note 1. — The DSC-start service is  used to establish a DSC dialogue for the purpose of providing down stream clearances.  It is a confirmed service.

- Note 2. — Once a DSC dialogue is established it remains open until explicitly closed.  (See DSC-end and CPDLC-abort services.)

### The DSC-start service shall contain the primitives and parameters as presented Table 2.3.3-2.

Table 2.3.3-2.  DSC-start Service Parameters

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| ICAO Facility Designator | M |  |  |  |
| Aircraft Identifier | M | M(=) |  |  |
| CPDLC Message | U | C(=) |  |  |
| Reject Reason |  |  | C | C(=) |
| Result |  |  | M | M(=) |
| Class of Communication Service | U | M |  |  |

### ICAO Facility Designator

- Note. — This parameter contains the addressed ground system’s facility designation.

#### The ICAO Facility Designator parameter value shall conform to the abstract syntax eight-character ICAO facility designator.

### Aircraft Identifier

#### The Aircraft Identifier parameter value shall conform to the abstract syntax 24-bit aircraft-id.

- Note. — This parameter contains the aircraft’s 24 bit ICAO address.

### CPDLC Message

- Note. — The CPDLC-air-user can use this parameter to send a CPDLC message to a CPDLC-ground-user.

#### The CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCDownlinkMessage.

### Reject Reason

- Note. — The parameter is used to provide a reason for rejecting a DSC dialogue.

#### If, and only if, the CPDLC-ground-user rejects the request to open a DSC dialogue, the CPDLC-ground-user shall provide a reason (a CPDLC message) for the rejection.

#### The Reject Reason parameter shall conform to the ASN.1 abstract syntax ATCUplinkMessage.

### Result

- Note. — This parameter is used to indicate whether or not a requested DSC dialogue is accepted.

#### The Result parameter value shall have one of two abstract values: “accepted” or “rejected”.

### Class of Communication Service

- Note. — This parameter contains the value of the required class of communication service.  If not specified by the CPDLC-air-user, this indicates that there is no routing preference.

#### Where specified by the CPDLC-air-user, the Class of Communication Service parameter shall have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note. — Class of Communication service parameter values are detailed in 1.2.

## CPDLC-message Service

- Note. — The CPDLC-message service can be used for pilot/controller message exchange, once a dialogue is established.  It is an unconfirmed service.

### The CPDLC-message service shall contain the primitives and parameters as presented Table 2.3.3-3.

Table 2.3.3-3.  CPDLC-message Service Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| CPDLC Message | M | M(=) |

### CPDLC Message

- Note. — This parameter contains a CPDLC message.

#### If the CPDLC-message service is invoked by the CPDLC-ground-user, the CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCUplinkMessage.

#### If the CPDLC-message service is invoked by the CPDLC-air-user, the CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCDownlinkMessage.

## CPDLC-end Service

- Note. — The CPDLC-end service is used by the CPDLC-ground-user to end a CPDLC dialogue with a CPDLC-air-user.  It is a confirmed service.

### The CPDLC-end service shall contain the primitives and parameters as presented Table 2.3.3-4.

Table 2.3.3-4.  CPDLC-end Service Parameters

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| CPDLC Message | U | C(=) | U | C(=) |
| Result |  |  | M | M(=) |

### CPDLC Message

- Note.  — This parameter contains a CPDLC message.

#### The CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCUplinkMessage, if provided by the CPDLC-ground-user.

#### The CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCDownlinkMessage, if provided by the CPDLC-air-user.

### Result

- Note. — This parameter is used to indicate whether or not a request to terminate a CPDLC dialogue is accepted.

#### The Result parameter shall have one of two abstract values: “accepted” or “rejected”.

## DSC-end Service

- Note. — The DSC-end service is used by the DSC-air-user to end a DSC dialogue with a CPDLC-ground-user.  It is a confirmed service.

### The DSC-end service shall contain the primitives and parameters as presented Table 2.3.3-5.

Table 2.3.3-5.  DSC-end Service Parameters

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| CPDLC Message | U | C(=) | U | C(=) |
| Result |  |  | M | M(=) |

### CPDLC Message

- Note. — This parameter contains a CPDLC message.

#### The CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCUplinkMessage, if provided by the CPDLC-ground-user.

#### The CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCDownlinkMessage if provided by the CPDLC-air-user.

### Result

- Note. — This parameter is used to indicate whether or not a request to terminate a DSC dialogue is accepted.

#### The Result parameter shall have one of two abstract values: “accepted” or “rejected”.

## CPDLC-forward Service

- Note. — The CPDLC-forward service is used by a CPDLC-ground-user to send a CPDLC message to another CPDLC-ground-user.  Its primary use is for the forwarding of aircraft requests.

### If the CPDLC-forward service is supported by the receiving ground system and the sending CPDLC-ground-ASE and receiving CPDLC-ground-ASE version numbers are equal, the CPDLC-forward service shall contain the primitives and parameters as presented Table 2.3.3-6.

Table 2.3.3-6.  CPDLC-forward Service Parameters

(Service Supported, Versions Equal)

| Parameter Name | Req | Ind | Cnf |
| --- | --- | --- | --- |
| Called ICAO Facility Designator | M |  |  |
| Calling ICAO Facility Designator | M | M(=) |  |
| CPDLC Message | M | M(=) |  |
| Class of Communication Service | U |  |  |
| Result |  |  | M |

### If the CPDLC-forward service is not supported by the receiving ground system, or if the CPDLC-forward service is supported by the receiving ground system but the sending CPDLC-ground-ASE and receiving CPDLC-ground-ASE version numbers are not equal, the CPDLC-forward service shall contain the primitives and parameters as presented Table 2.3.3-7.

Table 2.3.3-7.  CPDLC-forward Service Parameters

(Service Not Supported or Versions Not Equal)

| Parameter Name | Req | Cnf |
| --- | --- | --- |
| Called ICAO Facility Designator | M |  |
| Calling ICAO Facility Designator | M |  |
| ASE Version Number |  | C |
| CPDLC Message | M |  |
| Class of Communication Service | U |  |
| Result |  | M |

### Called ICAO Facility Designator

Note. — This parameter contains the addressed ground system’s facility designation.

#### The Called ICAO Facility Designator parameter value shall conform to the abstract syntax eight-character ICAO facility designator.

### Calling ICAO Facility Designator

Note. — This parameter contains the sending ground system’s facility designation.

#### The Calling ICAO Facility Designator parameter value shall conform to the abstract syntax eight-character ICAO facility designator.

### ASE Version Number

- Note. — This parameter contains the version number of the CPDLC-ASE.

#### When provided by the CPDLC-ground-ASE, the ASE Version Number parameter shall conform to the abstract integer value in the range 1-255.

#### Only if the sending CPDLC-ground-ASE version number is not equal to  the receiving CPDLC-ground-ASE version number shall the receiving CPDLC-ground-ASE version number be confirmed to the sending CPDLC-ground-user.

- Note.— If the sending CPDLC-ground-ASE version number is the same as the receiving CPDLC-ground-ASE version number, the Version Number parameter is not present in the indication given to the receiving CPDLC-ground-user, nor in the confirmation to the sending CPDLC-ground-user.

### CPDLC Message

- Note. — The sending CPDLC-ground-user uses this parameter to forward a CPDLC message to another CPDLC-ground-user.

#### The CPDLC Message parameter value shall conform to the ASN.1 abstract syntax ATCForwardMessage, when supplied by the CPDLC-ground-user.

### Class of Communication Service

- Note. — This parameter contains the value of the required class of communication service.  If not specified by the CPDLC-ground-user, this indicates that there is no routing preference.

#### Where specified by the CPDLC-ground-user, the Class of Communication Service parameter shall have one of the following abstract values: “A”, “B”, “C”, “D”, “E”, “F”, “G”, or “H”.

- Note. — Class of Communication service parameter values are detailed in 1.2.

### Result

- Note. — This parameter contains the result of the CPDLC-forward service.  It will indicate success (service supported and matching versions), service unsupported, or version number incompatibility.

#### The Result parameter value shall conform to the ASN.1 abstract syntax ATCForwardResponse.

## CPDLC-user-abort Service

- Note.  — This service provides the capability for either the CPDLC-air-user or a CPDLC-ground-user to abort communication with its peer.  It can be invoked at any time the user is aware that the CPDLC service is in operation.  The CPDLC-user-abort service can be used for operational or technical reasons.  It is an unconfirmed service.  Messages in transit may be lost during this operation.

### The CPDLC-user-abort service shall contain the primitives and parameters as presented Table 2.3.3-8.

Table 2.3.3-8.  CPDLC-user-abort Service Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Reason | U | M |

### Reason

- Note. — This parameter is used to indicate a reason for aborting the CPDLC or DSC dialogue.

#### The Reason parameter value shall conform to the ASN.1 abstract syntax CPDLCUserAbortReason.

## CPDLC-provider-abort Service

- Note. — This service provides the capability for the CPDLC-service provider to inform its active users, that it can no longer provide the CPDLC service.  Messages in transit may be lost during this operation.

### The CPDLC-provider-abort service shall contain the primitives and parameters as presented Table 2.3.3-9.

Table 2.3.3-9.  CPDLC-provider-abort Service Parameters

| Parameter Name | Ind |
| --- | --- |
| Reason | M |

### Reason

- Note. — This parameter identifies the reason for the abort.

#### The Reason parameter shall conform to the ASN.1 abstract syntax CPDLCProviderAbortReason.

# FORMAL DEFINITIONS OF MESSAGES

## Encoding/Decoding Rules

### A CPDLC-air-ASE shall be capable of encoding AircraftPDUs APDUs and decoding GroundPDUs APDUs.

### A CPDLC-ground-ASE shall be capable of encoding and decoding GroundPDUs APDUs and decoding AircraftPDUs APDUs.

## CPDLC ASN.1 Abstract Syntax

### The abstract syntax of the CPDLC protocol data units shall comply with the description contained in the ASN.1 module CPDLCMessageSetVersion1 conforming to ISO/IEC 8824, as defined in this section.

CPDLCMessageSetVersion1 DEFINITIONS::=

BEGIN

-- ----------------------------------------------------------------------------------

-- Ground Generated Messages - Top level

-- ----------------------------------------------------------------------------------

GroundPDUs ::= CHOICE

{

abortUser	[0]	CPDLCUserAbortReason,

abortProvider	[1]	CPDLCProviderAbortReason,

startup		[2]	UplinkMessage,

send		[3]	ATCUplinkMessage,

forward		[4]	ATCForwardMessage,

forwardresponse	[5]	ATCForwardResponse,

...

}

UplinkMessage ::= CHOICE

{

noMessage		[0]	NULL,

aTCUplinkMessage	[1]	ATCUplinkMessage

}

ATCUplinkMessage ::= SEQUENCE

{

header			ATCMessageHeader,

elementIds		SEQUENCE SIZE (1..5) OF ATCUplinkMsgElementId

}

ATCForwardMessage ::= SEQUENCE

{

forwardHeader	ForwardHeader,

forwardMessage	ForwardMessage

}

ForwardHeader ::= SEQUENCE

{

dateTime		DateTimeGroup,

aircraftID		AircraftFlightIdentification,

aircraftdAdress		AircraftAddress

}

ForwardMessage ::= CHOICE

{

upElementIDs		[0]	SEQUENCE SIZE (1..5) OF ATCUplinkMsgElementId,

downElementIDs		[1]	SEQUENCE SIZE (1..5) OF ATCDownlinkMsgElementId

}

ATCForwardResponse ::= ENUMERATED

{

success			(0),

service-not-supported	(1),

version-not-equal		(2),

...

}

-- ----------------------------------------------------------------------------------

-- Aircraft Generated Messages - Top level

-- ----------------------------------------------------------------------------------

AircraftPDUs::= CHOICE

{

abortUser		[0]	CPDLCUserAbortReason,

abortProvider	[1]	CPDLCProviderAbortReason,

startdown	[2]	StartDownMessage,

send		[3]	ATCDownlinkMessage,

...

}

StartDownMessage ::= SEQUENCE

{

mode	Mode DEFAULT cpdlc,

startDownlinkMessage 	DownlinkMessage

}

Mode ::= ENUMERATED

{

cpdlc		(0),

dsc		(1)

}

DownlinkMessage ::=   CHOICE

{

noMessage			[0]	NULL,

aTCDownlinkMessage		[1]	ATCDownlinkMessage

}

ATCDownlinkMessage ::=   SEQUENCE

{

header			ATCMessageHeader,

elementIds		SEQUENCE SIZE (1..5) OF ATCDownlinkMsgElementId

}

-- ----------------------------------------------------------------------------------

-- Uplink and Downlink messages - Common Elements

-- ----------------------------------------------------------------------------------

ATCMessageHeader ::= SEQUENCE

{

messageIdNumber		[0]	MsgIdentificationNumber,

messageRefNumber		[1]	MsgReferenceNumber		OPTIONAL,

dateTime			[2]	DateTimeGroup,

logicalAck			[3]	LogicalAck			DEFAULT notRequired

}

MsgIdentificationNumber ::= INTEGER (0..63)

MsgReferenceNumber ::= INTEGER (0..63)

CPDLCUserAbortReason ::= ENUMERATED

{

undefined				(0),

no-message-identification-numbers-available 	(1),

duplicate-message-identification-numbers		(2),

no-longer-next-data-authority			(3),

current-data-authority-abort			(4),

commanded-termination				(5),

...

}

CPDLCProviderAbortReason ::= ENUMERATED

{

timer-expired					(0),

undefined-error					(1),

invalid-PDU					(2),

not-permitted-PDU				(3),

communication-service-error			(4),

communication-service-failure			(5),

...

}

LogicalAck ::= ENUMERATED

{

required				(0),

notRequired			(1)

}

-- ----------------------------------------------------------------------------------

-- Uplink message element

-- ----------------------------------------------------------------------------------

ATCUplinkMsgElementId ::= CHOICE

{

--	UNABLE					Urg(N)/Alr(L)/Resp(N )

uM0NULL					[0] NULL,

--	STANDBY					Urg(N)/Alr(L)/Resp(N )

uM1NULL					[1] NULL,

--	REQUEST DEFERRED				Urg(N)/Alr(L)/Resp(N )

uM2NULL					[2] NULL,

-- 	ROGER						Urg(N)/Alr(L)/Resp(N )

uM3NULL					[3] NULL,

--	AFFIRM 					Urg(N)/Alr(L)/Resp(N )

uM4NULL					[4] NULL,

--	NEGATIVE					Urg(N)/Alr(L)/Resp(N )

uM5NULL					[5] NULL,

--	EXPECT [level]					Urg(L)/Alr(L)/Resp( R )

uM6Level					[6] Level,

--	EXPECT CLIMB AT [time]			Urg(L)/Alr(L)/Resp( R )

uM7Time					[7] Time,

--	EXPECT CLIMB AT [position]			Urg(L)/Alr(L)/Resp( R )

uM8Position					[8] Position,

--	EXPECT DESCENT AT [time]			Urg(L)/Alr(L)/Resp( R )

uM9Time					[9] Time,

--	EXPECT DESCENT AT [position]			Urg(L)/Alr(L)/Resp( R )

uM10Position					[10] Position,

--	EXPECT CRUISE CLIMB AT [time]		Urg(L)/Alr(L)/Resp( R )

uM11Time					[11] Time,

--	EXPECT CRUISE CLIMB AT [position]		Urg(L)/Alr(L)/Resp( R )

uM12Position					[12] Position,

--	AT [time] EXPECT CLIMB TO [level]		Urg(L)/Alr(L)/Resp( R )

uM13TimeLevel					[13] TimeLevel,

--	AT [position] EXPECT CLIMB TO [level]		Urg(L)/Alr(L)/Resp( R )

uM14PositionLevel				[14] PositionLevel,

--	AT [time] EXPECT DESCENT TO [level]		Urg(L)/Alr(L)/Resp( R )

uM15TimeLevel					[15] TimeLevel,

--	AT [position] EXPECT DESCENT TO [level]	Urg(L)/Alr(L)/Resp( R )

uM16PositionLevel				[16] PositionLevel,

--	AT [time] EXPECT CRUISE CLIMB TO [level]	Urg(L)/Alr(L)/Resp( R )

uM17TimeLevel					[17] TimeLevel,

--	AT [position] EXPECT CRUISE CLIMB TO [level]														Urg(L)/Alr(L)/Resp( R )

uM18PositionLevel				[18] PositionLevel,

--	MAINTAIN [level]				Urg(N)/Alr(M)/Resp(W/U)

uM19Level					[19] Level,

--	CLIMB TO [level]				Urg(N)/Alr(M)/Resp(W/U)

uM20Level					[20] Level,

--	AT [time] CLIMB TO [level]			Urg(N)/Alr(M)/Resp(W/U)

uM21TimeLevel					[21] TimeLevel,

--	AT [position] CLIMB TO [level]			Urg(N)/Alr(M)/Resp(W/U)

uM22PositionLevel				[22] PositionLevel,

--	DESCEND TO [level]				Urg(N)/Alr(M)/Resp(W/U)

uM23Level 					[23] Level,

--	AT [time] DESCEND TO [level] 			Urg(N)/Alr(M)/Resp(W/U)

uM24TimeLevel 					[24] TimeLevel,

--	AT [position] DESCEND TO [level] 		Urg(N)/Alr(M)/Resp(W/U)

uM25PositionLevel 				[25] PositionLevel,

--	CLIMB TO REACH [level] BY [time]		Urg(N)/Alr(M)/Resp(W/U)

uM26LevelTime 					[26] LevelTime,

--	CLIMB TO REACH [level] BY [position]		Urg(N)/Alr(M)/Resp(W/U)

uM27LevelPosition 				[27] LevelPosition,

--	DESCEND TO REACH [level] BY [time]		Urg(N)/Alr(M)/Resp(W/U)

uM28LevelTime 					[28] LevelTime,

--	DESCEND TO REACH [level] BY [position]	Urg(N)/Alr(M)/Resp(W/U)

uM29LevelPosition 				[29] LevelPosition,

--	MAINTAIN BLOCK [level] TO [level]		Urg(N)/Alr(M)/Resp(W/U)

uM30LevelLevel 					[30] LevelLevel,

--	CLIMB TO AND MAINTAIN BLOCK [level] TO [level]													Urg(N)/Alr(M)/Resp(W/U)

uM31LevelLevel 					[31] LevelLevel,

--	DESCEND TO AND MAINTAIN BLOCK [level] TO [level]													Urg(N)/Alr(M)/Resp(W/U)

uM32LevelLevel 					[32] LevelLevel,

--	Reserved

uM33null					[33] NULL,

--	CRUISE CLIMB TO [level]			Urg(N)/Alr(M)/Resp(W/U)

uM34Level 					[34] Level,

--	CRUISE CLIMB ABOVE [level]			Urg(N)/Alr(M)/Resp(W/U)

uM35Level 					[35] Level,

--	EXPEDITE CLIMB TO [level]			Urg(U)/Alr(M)/Resp(W/U)

uM36Level 					[36] Level,

--	EXPEDITE DESCENT TO [level]			Urg(U)/Alr(M)/Resp(W/U)

uM37Level 					[37] Level,

--	IMMEDIATELY CLIMB TO [level]		Urg(D)/Alr(H)/Resp(W/U)

uM38Level 					[38] Level,

--	IMMEDIATELY DESCEND TO [level]		Urg(D)/Alr(H)/Resp(W/U)

uM39Level 					[39] Level,

--	IMMEDIATELY STOP CLIMB AT [level]		Urg(D)/Alr(H)/Resp(W/U)

uM40Level 					[40] Level,

--	IMMEDIATELY STOP DESCENT AT [level]	Urg(D)/Alr(H)/Resp(W/U)

uM41Level 					[41] Level,

--	EXPECT TO CROSS [position] AT [level] 		Urg(L)/Alr(L)/Resp( R )

uM42PositionLevel				[42] PositionLevel,

--	EXPECT TO CROSS [position] AT OR ABOVE [level]													Urg(L)/Alr(L)/Resp( R )

uM43PositionLevel				[43] PositionLevel,

--	EXPECT TO CROSS [position] AT OR BELOW [level]													Urg(L)/Alr(L)/Resp( R )

uM44PositionLevel 				[44] PositionLevel,

--	EXPECT TO CROSS [position] AT AND MAINTAIN [level]													Urg(L)/Alr(L)/Resp( R )

uM45PositionLevel 				[45] PositionLevel,

--	CROSS [position] AT [level]			Urg(N)/Alr(M)/Resp(W/U)

uM46PositionLevel 				[46] PositionLevel,

--	CROSS [position] AT OR ABOVE [level]		Urg(N)/Alr(M)/Resp(W/U)

uM47PositionLevel 				[47] PositionLevel,

--	CROSS [position] AT OR BELOW [level]		Urg(N)/Alr(M)/Resp(W/U)

uM48PositionLevel 				[48]PositionLevel,

--	CROSS [position] AT AND MAINTAIN [level]	Urg(N)/Alr(M)/Resp(W/U)

uM49PositionLevel 				[49] PositionLevel,

--	CROSS [position] BETWEEN [level] AND [level]														Urg(N)/Alr(M)/Resp(W/U)

uM50PositionLevelLevel				[50] PositionLevelLevel,

--	CROSS [position] AT [time]			Urg(N)/Alr(M)/Resp(W/U)

uM51PositionTime 				[51] PositionTime,

--	CROSS [position] AT OR BEFORE [time]		Urg(N)/Alr(M)/Resp(W/U)

uM52PositionTime 				[52] PositionTime,

--	CROSS [position] AT OR AFTER [time]		Urg(N)/Alr(M)/Resp(W/U)

uM53PositionTime 				[53] PositionTime,

--	CROSS [position] BETWEEN [time] AND [time]	Urg(N)/Alr(M)/Resp(W/U)

uM54PositionTimeTime				[54] PositionTimeTime,

--	CROSS [position] AT [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM55PositionSpeed 				[55] PositionSpeed,

--	CROSS [position] AT OR LESS THAN [speed]	Urg(N)/Alr(M)/Resp(W/U)

uM56PositionSpeed 				[56] PositionSpeed,

--	CROSS [position] AT OR GREATER THAN [speed]														Urg(N)/Alr(M)/Resp(W/U)

uM57PositionSpeed 				[57] PositionSpeed,

--	CROSS [position] AT [time] AT [level]		Urg(N)/Alr(M)/Resp(W/U)

uM58PositionTimeLevel				[58] PositionTimeLevel,

--	CROSS [position] AT OR BEFORE [time] AT [level]													Urg(N)/Alr(M)/Resp(W/U)

uM59PositionTimeLevel 				[59] PositionTimeLevel,

--	CROSS [position] AT OR AFTER [time] AT [level]														Urg(N)/Alr(M)/Resp(W/U)

uM60PositionTimeLevel				[60] PositionTimeLevel,

--	CROSS [position] AT AND MAINTAIN [level] AT [speed]													Urg(N)/Alr(M)/Resp(W/U)

uM61PositionLevelSpeed 				[61] PositionLevelSpeed,

--	AT [time] CROSS [position] AT AND MAINTAIN [level]													Urg(N)/Alr(M)/Resp(W/U)

uM62TimePositionLevel				[62] TimePositionLevel,

--	AT [time] CROSS [position] AT AND MAINTAIN [level] AT [speed]

--							Urg(N)/Alr(M)/Resp(W/U)

uM63TimePositionLevelSpeed			[63] TimePositionLevelSpeed,

--	OFFSET [distanceOffset] [direction] OF ROUTE	Urg(N)/Alr(M)/Resp(W/U)

uM64DistanceOffsetDirection			[64] DistanceOffsetDirection,

--	AT [position] OFFSET [distanceOffset] [direction] OF ROUTE

--							Urg(N)/Alr(M)/Resp(W/U)

uM65PositionDistanceOffsetDirection		[65] PositionDistanceOffsetDirection,

--	AT [time] OFFSET [distanceOffset] [direction] OF ROUTE													Urg(N)/Alr(M)/Resp(W/U)

uM66TimeDistanceOffsetDirection			[66] TimeDistanceOffsetDirection,

--	PROCEED BACK ON ROUTE			Urg(N)/Alr(M)/Resp(W/U)

uM67NULL 					[67] NULL,

--	REJOIN ROUTE BY [position]			Urg(N)/Alr(M)/Resp(W/U)

uM68Position 					[68] Position,

--	REJOIN ROUTE BY [time]			Urg(N)/Alr(M)/Resp(W/U)

uM69Time 					[69] Time,

--	EXPECT BACK ON ROUTE BY [position]		Urg(L)/Alr(L)/Resp( R )

uM70Position 					[70] Position,

--	EXPECT BACK ON ROUTE BY [time]		Urg(L)/Alr(L)/Resp( R )

uM71Time 					[71] Time,

--	RESUME OWN NAVIGATION			Urg(N)/Alr(M)/Resp(W/U)

uM72NULL 					[72] NULL,

--	[DepartureClearance]				Urg(N)/Alr(M)/Resp(W/U)

uM73DepartureClearance 				[73] DepartureClearance,

--	PROCEED DIRECT TO [position]			Urg(N)/Alr(M)/Resp(W/U)

uM74Position 					[74] Position,

--	WHEN ABLE PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM75Position 					[75] Position,

--	AT [time] PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM76TimePosition 				[76] TimePosition,

--	AT [position] PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM77PositionPosition 				[77] PositionPosition,

--	AT [level] PROCEED DIRECT TO [position]	Urg(N)/Alr(M)/Resp(W/U)

uM78LevelPosition 				[78] LevelPosition,

--	CLEARED TO [position] VIA [routeClearance]	Urg(N)/Alr(M)/Resp(W/U)

uM79PositionRouteClearance			[79] PositionRouteClearance,

--	CLEARED [routeClearance]			Urg(N)/Alr(M)/Resp(W/U)

uM80RouteClearance 				[80] RouteClearance,

--	CLEARED [procedureName]			Urg(N)/Alr(M)/Resp(W/U)

uM81ProcedureName 				[81] ProcedureName,

--	CLEARED TO DEVIATE UP TO [distanceOffset] [direction] OF ROUTE											Urg(N)/Alr(M)/Resp(W/U)

uM82DistanceOffsetDirection			[82] DistanceOffsetDirection,

--	AT [position] CLEARED [routeClearance]		Urg(N)/Alr(M)/Resp(W/U)

uM83PositionRouteClearance			[83] PositionRouteClearance,

--	AT [position] CLEARED [procedureName]		Urg(N)/Alr(M)/Resp(W/U)

uM84PositionProcedureName 			[84] PositionProcedureName,

--	EXPECT [routeClearance]				Urg(L)/Alr(L)/Resp( R )

uM85RouteClearance 				[85] RouteClearance,

--	AT [position] EXPECT [routeClearance] 		Urg(L)/Alr(L)/Resp( R )

uM86PositionRouteClearance			[86] PositionRouteClearance,

--	EXPECT DIRECT TO [position]			Urg(L)/Alr(L)/Resp( R )

uM87Position 					[87] Position,

--	AT [position] EXPECT DIRECT TO [position]	Urg(L)/Alr(L)/Resp( R )

uM88PositionPosition 				[88]PositionPosition,

--	AT [time] EXPECT DIRECT TO [position]		Urg(L)/Alr(L)/Resp( R )

uM89TimePosition 				[89] TimePosition,

--	AT [level] EXPECT DIRECT TO [position]	Urg(L)/Alr(L)/Resp( R )

uM90LevelPosition 				[90] LevelPosition,

--	HOLD AT [position] MAINTAIN [level] INBOUND TRACK [degrees][direction]

--	TURNS [legtype]					Urg(N)/Alr(M)/Resp(W/U)

uM91HoldClearance 				[91] HoldClearance,

--	HOLD AT [position] AS PUBLISHED MAINTAIN [level]													Urg(N)/Alr(M)/Resp(W/U)

uM92PositionLevel 				[92] PositionLevel,

--	EXPECT FURTHER CLEARANCE AT [time]	Urg(L)/Alr(L)/Resp( R )

uM93Time 					[93] Time,

--	TURN [direction] HEADING [degrees]		Urg(N)/Alr(M)/Resp(W/U)

uM94DirectionDegrees 				[94] DirectionDegrees,

--	TURN [direction] GROUND TRACK [degrees]	Urg(N)/Alr(M)/Resp(W/U)

uM95DirectionDegrees				[95] DirectionDegrees,

--	FLY PRESENT HEADING			Urg(N)/Alr(M)/Resp(W/U)

uM96NULL 					[96] NULL,

--	AT [position] FLY HEADING [degrees]		Urg(N)/Alr(M)/Resp(W/U)

uM97PositionDegrees 				[97] PositionDegrees,

--	IMMEDIATELY TURN [direction] HEADING [degrees]													Urg(D)/Alr(H)/Resp(W/U)

uM98DirectionDegrees 				[98] DirectionDegrees,

--	EXPECT [procedureName]			Urg(L)/Alr(L)/Resp(R)

uM99ProcedureName 				[99] ProcedureName,

--	AT [time] EXPECT [speed]			Urg(L)/Alr(L)/Resp(R)

uM100TimeSpeed 				[100] TimeSpeed,

--	AT [position] EXPECT [speed]			Urg(L)/Alr(L)/Resp(R)

uM101PositionSpeed 				[101] PositionSpeed,

--	AT [level] EXPECT [speed]			Urg(L)/Alr(L)/Resp(R)

uM102LevelSpeed 				[102] LevelSpeed,

--	AT [time] EXPECT [speed] TO [speed] 		Urg(L)/Alr(L)/Resp(R)

uM103TimeSpeedSpeed 				[103] TimeSpeedSpeed,

--	AT [position] EXPECT [speed] TO [speed]		Urg(L)/Alr(L)/Resp(R)

uM104PositionSpeedSpeed 			[104] PositionSpeedSpeed,

--	AT [level] EXPECT [speed] TO [speed]		Urg(L)/Alr(L)/Resp(R)

uM105LevelSpeedSpeed 				[105] LevelSpeedSpeed,

--	MAINTAIN [speed]				Urg(N)/Alr(M)/Resp(W/U)

uM106Speed 					[106] Speed,

--	MAINTAIN PRESENT SPEED			Urg(N)/Alr(M)/Resp(W/U)

uM107NULL 					[107] NULL,

--	MAINTAIN [speed] OR GREATER		Urg(N)/Alr(M)/Resp(W/U)

uM108Speed 					[108] Speed,

--	MAINTAIN [speed] OR LESS			Urg(N)/Alr(M)/Resp(W/U)

uM109Speed 					[109] Speed,

--	MAINTAIN [speed] TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM110SpeedSpeed 				[110] SpeedSpeed,

--	INCREASE SPEED TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM111Speed 					[111] Speed,

--	INCREASE SPEED TO [speed] OR GREATER  	Urg(N)/Alr(M)/Resp(W/U)

uM112Speed 					[112] Speed,

--	REDUCE SPEED TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM113Speed 					[113] Speed,

--	REDUCE SPEED TO [speed] OR LESS		Urg(N)/Alr(M)/Resp(W/U)

uM114Speed 					[114] Speed,

--	DO NOT EXCEED [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM115Speed 					[115] Speed,

--	RESUME NORMAL SPEED			Urg(N)/Alr(M)/Resp(W/U)

uM116NULL 					[116] NULL,

--	CONTACT [unitname] [frequency]			Urg(N)/Alr(M)/Resp(W/U)

uM117UnitNameFrequency 			[117] UnitNameFrequency,

--	AT [position] CONTACT [unitname] [frequency]														Urg(N)/Alr(M)/Resp(W/U)

uM118PositionUnitNameFrequency		[118] PositionUnitNameFrequency,

--	AT [time] CONTACT [unitname] [frequency]	Urg(N)/Alr(M)/Resp(W/U)

uM119TimeUnitNameFrequency			[119] TimeUnitNameFrequency,

--	MONITOR [unitname] [frequency]			Urg(N)/Alr(M)/Resp(W/U)

uM120UnitNameFrequency			[120] UnitNameFrequency,

--	AT [position] MONITOR [unitname] [frequency]														Urg(N)/Alr(M)/Resp(W/U)

uM121PositionUnitNameFrequency		[121] PositionUnitNameFrequency,

--	AT [time] MONITOR [unitname] [frequency]	Urg(N)/Alr(M)/Resp(W/U)

uM122TimeUnitNameFrequency 			[122] TimeUnitNameFrequency,

--	SQUAWK [code]				Urg(N)/Alr(M)/Resp(W/U)

uM123Code 					[123] Code,

--	STOP SQUAWK					Urg(N)/Alr(M)/Resp(W/U)

uM124NULL 					[124] NULL,

--	SQUAWK MODE CHARLIE			Urg(N)/Alr(M)/Resp(W/U)

uM125NULL					[125] NULL,

--	STOP SQUAWK MODE CHARLIE		Urg(N)/Alr(M)/Resp(W/U)

uM126NULL					[126] NULL,

--	REPORT BACK ON ROUTE			Urg(N)/Alr(M)/Resp( W/U )

uM127NULL 					[127] NULL,

--	REPORT LEAVING [level]			Urg(N)/Alr(M)/Resp(W/U )

uM128Level 					[128] Level,

--	REPORT MAINTAINING [level]			Urg(N)/Alr(M)/Resp(W/U)

uM129Level					[129] Level,

--	REPORT PASSING [position]			Urg(N)/Alr(M)/Resp( W/U )

uM130Position 					[130] Position,

--	REPORT REMAINING FUEL AND PERSONS ON BOARD													Urg(N)/Alr(M)/Resp(Y )

uM131NULL 					[131] NULL,

--	REPORT POSITION				Urg(N)/Alr(M)/Resp(Y)

uM132NULL					[132] NULL,

--	REPORT LEVEL				Urg(N)/Alr(M)/Resp(Y)

uM133NULL					[133] NULL,

--	REPORT [speedtype] [speedtype] [speedtype]SPEED													Urg(N)/Alr(M)/Resp(Y)

uM134SpeedTypeSpeedTypeSpeedType		[134] SpeedTypeSpeedTypeSpeedType,

--	CONFIRM ASSIGNED LEVEL			Urg(N)/Alr(M)/Resp(Y )

uM135NULL 					[135] NULL,

--	CONFIRM ASSIGNED SPEED			Urg(N)/Alr(M)/Resp(Y )

uM136NULL 					[136] NULL,

--	CONFIRM ASSIGNED ROUTE			Urg(N)/Alr(M)/Resp(Y )

uM137NULL 					[137] NULL,

--	CONFIRM TIME OVER REPORTED WAYPOINT 	Urg(N)/Alr(M)/Resp(Y)

uM138NULL 					[138] NULL,

--	CONFIRM REPORTED WAYPOINT		Urg(N)/Alr(M)/Resp(Y)

uM139NULL 					[139] NULL,

--	CONFIRM NEXT WAYPOINT			Urg(N)/Alr(M)/Resp(Y)

uM140NULL 					[140] NULL,

--	CONFIRM NEXT WAYPOINT ETA		Urg(N)/Alr(M)/Resp(Y)

uM141NULL 					[141] NULL,

--	CONFIRM ENSUING WAYPOINT		Urg(N)/Alr(M)/Resp(Y)

uM142NULL 					[142] NULL,

--	CONFIRM REQUEST				Urg(N)/Alr(M)/Resp(Y)

uM143NULL 					[143] NULL,

--	CONFIRM SQUAWK				Urg(N)/Alr(M)/Resp(Y)

uM144NULL 					[144] NULL,

--	REPORT HEADING				Urg(N)/Alr(M)/Resp(Y)

uM145NULL 					[145] NULL,

--	REPORT GROUND TRACK			Urg(N)/Alr(M)/Resp(Y)

uM146NULL 					[146] NULL,

--	REQUEST POSITION REPORT			Urg(N)/Alr(M)/Resp(Y )

uM147NULL 					[147] NULL,

--	WHEN CAN YOU ACCEPT [level]		Urg(N)/Alr(M)/Resp(Y)

uM148Level 					[148] Level,

--	CAN YOU ACCEPT [level] AT [position]		Urg(N)/Alr(M)/Resp(A/N)

uM149LevelPosition 				[149] LevelPosition,

--	CAN YOU ACCEPT [level] AT [time]		Urg(N)/Alr(M)/Resp(A/N)

uM150LevelTime 				[150] LevelTime,

--	WHEN CAN YOU ACCEPT [speed]		Urg(N)/Alr(M)/Resp(Y)

uM151Speed 					[151] Speed,

--	WHEN CAN YOU ACCEPT [distanceOffset] [direction] OFFSET												Urg(N)/Alr(M)/Resp(Y)

uM152DistanceOffsetDirection			[152] DistanceOffsetDirection,

--	ALTIMETER [altimeter]				Urg(N)/Alr(M)/Resp(R)

uM153Altimeter 					[153] Altimeter,

--	RADAR SERVICE TERMINATED		Urg(N)/Alr(M)/Resp(R)

uM154NULL					[154] NULL,

--	RADAR CONTACT [position]			Urg(N)/Alr(M)/Resp(R)

uM155Position 					[155] Position,

--	RADAR CONTACT LOST			Urg(N)/Alr(M)/Resp(R)

uM156NULL 					[156] NULL,

--	CHECK STUCK MICROPHONE [frequency]	Urg(U)/Alr(M)/Resp(N)

uM157Frequency 				[157] Frequency,

--	ATIS [atiscode]					Urg(N)/Alr(M)/Resp(R)

uM158AtisCode 					[158] ATISCode,

--	ERROR [errorInformation]			Urg(U)/Alr(M)/Resp(N)

uM159ErrorInformation				[159] ErrorInformation,

--	NEXT DATA AUTHORITY [facilitydesignation] 	Urg(L)/Alr(N)/Resp(N)

uM160FacilityDesignation 			[160] FacilityDesignation,

--	END SERVICE					Urg(L)/Alr(N)/Resp(N)

uM161NULL 					[161] NULL,

--	SERVICE UNAVAILABLE			Urg(L)/Alr(L)/Resp(N )

uM162NULL 					[162] NULL,

--	[facilitydesignation]				Urg(L)/Alr(N)/Resp(N)

uM163FacilityDesignation				[163] FacilityDesignation,

--	WHEN READY					Urg(L)/Alr(N)/Resp(N)

uM164NULL 					[164] NULL,

--	THEN						Urg(L)/Alr(N)/Resp(N)

uM165NULL 					[165] NULL,

--	DUE TO [traffictype]TRAFFIC			Urg(L)/Alr(N)/Resp(N)

uM166TrafficType 				[166] TrafficType,

--	DUE TO AIRSPACE RESTRICTION		Urg(L)/Alr(N)/Resp(N)

uM167NULL 					[167] NULL,

--	DISREGARD					Urg(N)/Alr(M)/Resp(R)

uM168NULL 					[168] NULL,

--	[freetext]					Urg(N)/Alr(L)/Resp(R)

uM169FreeText					[169] FreeText,

-- 	[freetext]					Urg(D)/Alr(H)/Resp(R)

uM170FreeText 					[170] FreeText,

--	CLIMB AT [verticalRate] MINIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM171VerticalRate				[171] VerticalRate,

--	CLIMB AT [verticalRate] MAXIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM172VerticalRate				[172] VerticalRate,

--	DESCEND AT [verticalRate] MINIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM173VerticalRate				[173] VerticalRate,

--	DESCEND AT [verticalRate] MAXIMUM		Urg(N)/Alr(M)/Resp(W/U)

uM174VerticalRate				[174] VerticalRate,

--	REPORT REACHING [level]			Urg(N)/Alr(M)/Resp(W/U)

uM175Level					[175] Level,

--	MAINTAIN OWN SEPARATION AND VMC	Urg(N)/Alr(M)/Resp(W/U)

uM176NULL					[176] NULL,

--	AT PILOTS DISCRETION			Urg(L)/Alr(L)/Resp(N)

uM177NULL					[177] NULL,

--	[freetext]					Urg(N)/Alr(L)/Resp(N)

uM178FreeText					[178] FreeText,

--	SQUAWK IDENT				Urg(N)/Alr(M)/Resp(W/U)

uM179NULL					[179] NULL,

--	REPORT REACHING BLOCK [level] TO [level]													Urg(N)/Alr(M)/Resp(W/U)

uM180LevelLevel				[180] LevelLevel,

--	REPORT DISTANCE [tofrom] [position]		Urg(N)/Alr(M)/Resp(Y)

uM181ToFromPosition				[181] ToFromPosition,

--	CONFIRM ATIS CODE				Urg(N)/Alr(M)/Resp(Y)

uM182NULL					[182] NULL,

--	[freetext]					Urg(N)/Alr(M)/Resp(N)

uM183FreeText					[183] FreeText,

--	AT [time] REPORT DISTANCE [tofrom] [position]	Urg(N)/Alr(M)/Resp(Y)

uM184TimeToFromPosition			[184] TimeToFromPosition,

--	AFTER PASSING [position] CLIMB TO [level]												Urg(N)/Alr(M)/Resp(W/U)

uM185PositionLevel				[185] PositionLevel,

--	AFTER PASSING [position] DESCEND TO [level]											Urg(N)/Alr(M)/Resp(W/U)

uM186PositionLevel				[186] PositionLevel,

--	[freetext]					Urg(L)/Alr(N)/Resp(N)

uM187FreeText					[187] FreeText,

--	AFTER PASSING [position] MAINTAIN [speed]	Urg(N)/Alr(M)/Resp(W/U)

uM188PositionSpeed				[188] PositionSpeed,

--	ADJUST SPEED TO [speed]			Urg(N)/Alr(M)/Resp(W/U)

uM189Speed					[189] NULL,

--	FLY HEADING [degrees]				Urg(N)/Alr(M)/Resp(W/U)

uM190Degrees					[190] Degrees,

--	ALL ATS TERMINATED			Urg(N)/Alr(M)/Resp(R)

uM191NULL					[191] NULL,

--	REACH [level] BY [time]				Urg(N)/Alr(M)/Resp(W/U)

uM192LevelTime				[192] LevelTime,

--	IDENTIFICATION LOST				Urg(N)/Alr(M)/Resp(R)

uM193NULL					[193] NULL,

--	[freetext]					Urg(N)/Alr(L)/Resp(Y)

uM194FreeText					[194] FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(R)

uM195FreeText					[195] FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(W/U)

uM196FreeText					[196] FreeText,

--	[freetext]					Urg(U)/Alr(M)/Resp(W/U)

uM197FreeText					[197] FreeText,

--	[freetext]					Urg(D)/Alr(H)/Resp(W/U)

uM198FreeText					[198] FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(W/U)

uM199FreeText					[199] FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(R)

uM200FreeText					[200] FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(W/U)

uM201FreeText					[201] FreeText,

--	[freetext]					Urg(D)/Alr(H)/Resp(W/U)

uM202FreeText					[202] FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(R)

uM203FreeText					[203] FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(Y)

uM204FreeText					[204] FreeText,

--	[freetext]					Urg(N)/Alr(M)/Resp(A/N)

uM205FreeText					[205] FreeText,

--	[freetext]					Urg(L)/Alr(N)/Resp(Y)

uM206FreeText					[206] FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(Y)

uM207FreeText					[207] FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(N)

uM208FreeText					[208] FreeText,

--	REACH [level] BY [position]			Urg(N)/Alr(M)/Resp(W/U)

uM209LevelPosition				[209] LevelPosition,

--	IDENTIFIED [position]				Urg(N)/Alr(M)/Resp(R)

uM210Position					[210] Position,

--	REQUEST FORWARDED			Urg(N)/Alr(L)/Resp(N)

uM211NULL					[211] NULL,

--	[facilitydesignation] ATIS [atiscode] CURRENT	Urg(N)/Alr(M)/Resp(R)

uM212FacilityDesignationATISCode		[212] FacilityDesignationATISCode,

--	[facilitydesignation] ALTIMETER [altimeter]	Urg(N)/Alr(M)/Resp(R)

uM213FacilityDesignationAltimeter		[213] FacilityDesignationAltimeter,

--	RUNWAY [runway] VISUAL RANGE [rvr]	Urg(N)/Alr(M)/Resp(R)

uM214RunwayRVR				[214] RunwayRVR,

--	TURN [degrees][direction]			Urg(N)/Alr(M)/Resp(W/U)

uM215DegreesDirection				[215] DirectionDegrees,

--	REQUEST FLIGHT PLAN			Urg(N)/Alr(M)/Resp(Y)

uM216NULL					[216] NULL,

--	REPORT ARRIVAL				Urg(N)/Alr(M)/Resp(Y)

uM217NULL					[217] NULL,

--	REQUEST ALREADY RECEIVED		Urg(L)/Alr(N)/Resp(N)

uM218NULL					[218] NULL,

--	STOP CLIMB AT [level]				Urg(U)/Alr(M)/Resp(W/U)

uM219Level					[219] Level,

--	STOP DESCENT AT [level]			Urg(U)/Alr(M)/Resp(W/U)

uM220Level					[220] Level,

--	STOPTURN HEADING [degrees]			Urg(U)/Alr(M)/Resp(W/U)

uM221Degrees					[221] Degrees,

--	NO SPEED RESTRICTION			Urg(L)/Alr(L)/Resp(R)

uM222NULL					[222] NULL,

--	REDUCE TO MINIMUM APPROACH SPEED	Urg(N)/Alr(M)/Resp(W/U)

uM223NULL					[223] NULL,

--	NO DELAY EXPECTED				Urg(N)/Alr(L)/Resp(R)

uM224NULL					[224] NULL,

--	DELAY NOT DETERMINED			Urg(N)/Alr(L)/Resp(R)

uM225NULL					[225] NULL,

--	EXPECTED APPROACH TIME [time]		Urg(N)/Alr(L)/Resp(R)

uM226Time					[226] Time,

--	LOGICAL ACKNOWLEDGMENT		Urg(N)/Alr(M)/Resp(N)

uM227NULL					[227] NULL,

--	REPORT ETA [position]				Urg(L)/Alr(L)/Resp(Y)

uM228Position					[228] Position,

--	REPORT ALTERNATE AERODROME		Urg(L)/Alr(L)/Resp(Y)

uM229NULL					[229] NULL,

--	IMMEDIATELY					Urg(D)/Alr(H)/Resp(N)

uM230NULL					[230] NULL,

--	STATE PREFERRED LEVEL			Urg(L)/Alr(L)/Resp(Y)

uM231NULL					[231] NULL,

--	STATE-TOP-OF-DESCENT			Urg(L)/Alr(L)/Resp(Y)

uM232NULL					[232] NULL,

--	USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED													Urg(N)/Alr(M)/Resp(N)

uM233NULL					[233] NULL,

--	FLIGHT PLAN NOT HELD			Urg(L)/Alr(L)/Resp(N)

uM234NULL					[234] NULL,

--	ROGER 7500					Urg(U)/Alr(H)/Resp(N)

uM235NULL					[235] NULL,

--	LEAVE CONTROLLED AIRSPACE		Urg(N)/Alr(M)/Resp(W/U)

uM236NULL					[236] NULL,

...

}

-- ----------------------------------------------------------------------------------

-- Downlink message element

-- ----------------------------------------------------------------------------------

ATCDownlinkMsgElementId ::= CHOICE

{

--	WILCO						Urg(N)/Alr(M)/Resp(N)

dM0NULL 					[0] NULL,

--	UNABLE					Urg(N)/Alr(M)/Resp(N)

dM1NULL 					[1] NULL,

--	STANDBY					Urg(N)/Alr(M)/Resp(N)

dM2NULL 					[2] NULL,

--	ROGER						Urg(N)/Alr(M)/Resp(N)

dM3NULL 					[3] NULL,

--	AFFIRM					Urg(N)/Alr(M)/Resp(N)

dM4NULL 					[4] NULL,

--	NEGATIVE					Urg(N)/Alr(M)/Resp(N)

dM5NULL 					[5] NULL,

--	REQUEST [level]				Urg(N)/Alr(L)/Resp(Y)

dM6Level 					[6] Level,

--	REQUEST BLOCK [level] TO [level]		Urg(N)/Alr(L)/Resp(Y)

dM7LevelLevel 				[7] LevelLevel,

--	REQUEST CRUISE CLIMB TO [level]		Urg(N)/Alr(L)/Resp(Y)

dM8Level 					[8] Level,

--	REQUEST CLIMB TO [level]			Urg(N)/Alr(L)/Resp(Y)

dM9Level					[9] Level,

--	REQUEST DESCENT TO [level]		Urg(N)/Alr(L)/Resp(Y)

dM10Level 					[10] Level,

--	AT [position] REQUEST CLIMB TO [level] 	Urg(N)/Alr(L)/Resp(Y)

dM11PositionLevel 				[11] PositionLevel,

--	AT [position] REQUEST DESCENT TO [level]	Urg(N)/Alr(L)/Resp(Y)

dM12PositionLevel 				[12] PositionLevel,

--	AT [time] REQUEST CLIMB TO [level]		Urg(N)/Alr(L)/Resp(Y)

dM13TimeLevel 				[13] TimeLevel,

--	AT [time] REQUEST DESCENT TO [level]	Urg(N)/Alr(L)/Resp(Y)

dM14TimeLevel				[14] TimeLevel,

--	REQUEST OFFSET [distanceOffset] [direction] OF ROUTE							Urg(N)/Alr(L)/Resp(Y)

dM15DistanceOffsetDirection			[15] DistanceOffsetDirection,

--	AT [position] REQUEST OFFSET [distanceOffset] [direction] OF ROUTE											Urg(N)/Alr(L)/Resp(Y)

dM16PositionDistanceOffsetDirection		[16] PositionDistanceOffsetDirection,

--	AT [time] REQUEST OFFSET [distanceOffset] [direction] OF ROUTE											Urg(N)/Alr(L)/Resp(Y)

dM17TimeDistanceOffsetDirection			[17] TimeDistanceOffsetDirection,

--	REQUEST [speed]				Urg(N)/Alr(L)/Resp(Y)

dM18Speed 					[18] Speed,

--	REQUEST [speed] TO [speed]			Urg(N)/Alr(L)/Resp(Y)

dM19SpeedSpeed 				[19] SpeedSpeed,

--	REQUEST VOICE CONTACT			Urg(N)/Alr(L)/Resp(Y)

dM20NULL 					[20] NULL,

--	REQUEST VOICE CONTACT [frequency]		Urg(N)/Alr(L)/Resp(Y)

dM21Frequency 					[21] Frequency,

--	REQUEST DIRECT TO [position]			Urg(N)/Alr(L)/Resp(Y)

dM22Position 					[22] Position,

--	REQUEST [procedureName]			Urg(N)/Alr(L)/Resp(Y)

dM23ProcedureName 				[23] ProcedureName,

--	REQUEST [routeClearance]			Urg(N)/Alr(L)/Resp(Y)

dM24RouteClearance 				[24] RouteClearance,

--	REQUEST [clearanceType] CLEARANCE		Urg(N)/Alr(L)/Resp(Y)

dM25ClearanceType 				[25] ClearanceType,

--	REQUEST WEATHER DEVIATION TO  [position] VIA [routeClearance]											Urg(N)/Alr(L)/Resp(Y)

dM26PositionRouteClearance 			[26] PositionRouteClearance,

--	REQUEST WEATHER DEVIATION UP  TO [distanceOffset] [direction] OF ROUTE										Urg(N)/Alr(L)/Resp(Y)

dM27DistanceOffsetDirection			[27] DistanceOffsetDirection,

--	LEAVING [level]				Urg(N)/Alr(L)/Resp(Y)

dM28Level 					[28] Level,

--	CLIMBING TO [level]				Urg(N)/Alr(M)/Resp(N)

dM29Level 					[29]Level,

--	DESCENDING TO [level]			Urg(N)/Alr(M)/Resp(N)

dM30Level 					[30] Level,

--	PASSING [position]				Urg(N)/Alr(M)/Resp(N)

dM31Position 					[31] Position,

--	PRESENT LEVEL [level]			Urg(N)/Alr(M)/Resp(N)

dM32Level 					[32] Level,

--	PRESENT POSITION [position]			Urg(N)/Alr(M)/Resp(N)

dM33Position 					[33] Position,

--	PRESENT SPEED [speed]			Urg(N)/Alr(M)/Resp(N)

dM34Speed 					[34]Speed,

--	PRESENT HEADING [degrees]			Urg(N)/Alr(M)/Resp(N)

dM35Degrees 					[35] Degrees,

--	PRESENT GROUND TRACK [degrees]		Urg(N)/Alr(M)/Resp(N)

dM36Degrees 					[36]Degrees,

--	LEVEL [level]					Urg(N)/Alr(M)/Resp(N)

dM37Level 					[37] Level,

--	ASSIGNED LEVEL [level]			Urg(N)/Alr(M)/Resp(N)

dM38Level 					[38] Level,

--	ASSIGNED SPEED [speed]			Urg(N)/Alr(M)/Resp(N)

dM39Speed 					[39] Speed,

--	ASSIGNED ROUTE [routeClearance]		Urg(N)/Alr(M)/Resp(N)

dM40RouteClearance 				[40] RouteClearance,

--	BACK ON ROUTE				Urg(N)/Alr(M)/Resp(N)

dM41NULL 					[41] NULL,

--	NEXT WAYPOINT [position]			Urg(N)/Alr(M)/Resp(N)

dM42Position 					[42] Position,

--	NEXT WAYPOINT ETA [time]			Urg(N)/Alr(M)/Resp(N)

dM43Time 					[43] Time,

--	ENSUING WAYPOINT [position]			Urg(N)/Alr(M)/Resp(N)

dM44Position 					[44] Position,

--	REPORTED WAYPOINT [position]		Urg(N)/Alr(M)/Resp(N)

dM45Position 					[45] Position,

--	REPORTED WAYPOINT [time]			Urg(N)/Alr(M)/Resp(N)

dM46Time 					[46] Time,

--	SQUAWKING [code]			Urg(N)/Alr(M)/Resp(N)

dM47Code 				[47] Code,

--	POSITION REPORT [positionreport]		Urg(N)/Alr(M)/Resp(N)

dM48PositionReport 				[48] PositionReport,

--	WHEN CAN WE EXPECT [speed]		Urg(L)/Alr(L)/Resp(Y)

dM49Speed 					[49] Speed,

--	WHEN CAN WE EXPECT [speed] TO [speed]	Urg(L)/Alr(L)/Resp(Y)

dM50SpeedSpeed 				[50] SpeedSpeed,

--	WHEN CAN WE EXPECT BACK ON ROUTE	Urg(L)/Alr(L)/Resp(Y)

dM51NULL 					[51] NULL,

--	WHEN CAN WE EXPECT LOWER LEVEL	Urg(L)/Alr(L)/Resp(Y)

dM52NULL 					[52] NULL,

--	WHEN CAN WE EXPECT HIGHER LEVEL	Urg(L)/Alr(L)/Resp(Y)

dM53NULL 					[53] NULL,

--	WHEN CAN WE EXPECT CRUISE CLIMB TO [level]													Urg(L)/Alr(L)/Resp(Y)

dM54Level 					[54] Level,

--	PAN PAN PAN					Urg(U)/Alr(H)/Resp(N)

dM55NULL 					[55] NULL,

--	MAYDAY MAYDAY MAYDAY			Urg(D)/Alr(H)/Resp(N)

dM56NULL 					[56] NULL,

--	[remainingFuel] OF FUEL REMAINING AND [personsonboard] PERSONS ON BOARD										Urg(U)/Alr(H)/Resp(N)

dM57RemainingFuelPersonsOnBoard 		[57] RemainingFuelPersonsOnBoard,

--	CANCEL EMERGENCY				Urg(U)/Alr(M)/Resp(N)

dM58NULL 					[58] NULL,

--	DIVERTING TO [position] VIA [routeClearance]	Urg(U)/Alr(H)/Resp(N)

dM59PositionRouteClearance 			[59] PositionRouteClearance,

--	OFFSETTING [distanceOffset] [direction] OF ROUTE													Urg(U)/Alr(H)/Resp(N)

dM60DistanceOffsetDirection 			[60] DistanceOffsetDirection,

--	DESCENDING TO [level]			Urg(U)/Alr(H)/Resp(N)

dM61Level 					[61] Level,

--	ERROR [errorInformation]			Urg(U)/Alr(L)/Resp(N)

dM62ErrorInformation 				[62] ErrorInformation,

--	NOT CURRENT DATA AUTHORITY		Urg(L)/Alr(L)/Resp(N)

dM63NULL 					[63] NULL,

--	[facilitydesignation]				Urg(L)/Alr(L)/Resp(N)

dM64FacilityDesignation 			[64] FacilityDesignation,

--	DUE TO WEATHER				Urg(L)/Alr(L)/Resp(N)

dM65NULL 					[65] NULL,

--	DUE TO AIRCRAFT PERFORMANCE		Urg(L)/Alr(L)/Resp(N)

dM66NULL 					[66] NULL,

--	[freetext]					Urg(N)/Alr(L)/Resp(N)

dM67FreeText 					[67] FreeText,

--	[freetext]					Urg(D)/Alr(H)/Resp(Y)

dM68FreeText 					[68] FreeText,

--	REQUEST VMC DESCENT			Urg(N)/Alr(L)/Resp(Y)

dM69NULL					[69] NULL,

--	REQUEST HEADING [degrees]			Urg(N)/Alr(L)/Resp(Y)

dM70Degrees  					[70] Degrees,

--	REQUEST GROUND TRACK [degrees]		Urg(N)/Alr(L)/Resp(Y)

dM71Degrees					[71] Degrees,

--	REACHING [level]				Urg(N)/Alr(M)/Resp(N)

dM72Level					[72] Level,

--	[versionnumber]					Urg(L)/Alr(L)/Resp(N)

dM73Versionnumber				[73] VersionNumber,

--	REQUEST TO MAINTAIN OWN SEPARATION AND VMC												Urg(L)/Alr(L)/Resp(Y)

dM74NULL					[74] NULL,

--	AT PILOTS DISCRETION			Urg(L)/Alr(L)/Resp(N)

dM75NULL					[75] NULL,

--	REACHING BLOCK [level] TO [level]	Urg(N)/Alr(N)/Resp(N)

dM76LevelLevel				[76] LevelLevel,

--	ASSIGNED BLOCK [level] TO [level]	Urg(N)/Alr(N)/Resp(N)

dM77LevelLevel				[77] LevelLevel,

--	AT [time] [distance] [tofrom] [position]		Urg(N)/Alr(N)/Resp(N)

dM78TimeDistanceToFromPosition		[78] TimeDistanceToFromPosition,

--	ATIS [atiscode]					Urg(N)/Alr(M)/Resp(N)

dM79AtisCode					[79] ATISCode,

--	DEVIATING [distanceOffset] [direction] OF ROUTE													Urg(N)/Alr(M)/Resp(N)

dM80DistanceOffsetDirection			[80] DistanceOffsetDirection,

--	WE CAN ACCEPT [level] AT [time]		Urg(N)/Alr(L)/Resp(N)

dM81LevelTime				[81] LevelTime,

--	WE CANNNOT ACCEPT [level]		Urg(N)/Alr(L)/Resp(N)

dM82Level					[82] Level,

--	WE CAN ACCEPT [speed] AT [time]		Urg(N)/Alr(L)/Resp(N)

dM83SpeedTime					[83] SpeedTime,

--	WE CANNNOT ACCEPT [speed]			Urg(N)/Alr(L)/Resp(N)

dM84Speed					[84] Speed,

--	WE CAN ACCEPT [distanceOffset] [direction] at[time]	Urg(N)/Alr(L)/Resp(N)

dM85DistanceOffsetDirectionTime			[85] DistanceOffsetDirectionTime,

--	WE CANNNOT ACCEPT [distanceOffset] [direction]													Urg(N)/Alr(L)/Resp(N)

dM86DistanceOffsetDirection			[86] DistanceOffsetDirection,

--	WHEN CAN WE EXPECT CLIMB TO [level]	Urg(L)/Alr(L)/Resp(Y)

dM87Level					[87] Level,

--	WHEN CAN WE EXPECT DESCENT TO [level]													Urg(L)/Alr(L)/Resp(Y)

dM88Level					[88] Level,

--	MONITORING[unitname] [frequency]		Urg(L)/Alr(M)/Resp(N)

dM89UnitnameFrequency			[89] UnitNameFrequency,

--	[freetext]					Urg(N)/Alr(M)/Resp(N)

dM90FreeText 					[90] FreeText,

--	[freetext]					Urg(N)/Alr(L)/Resp(Y)

dM91FreeText 					[91] FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(Y)

dM92FreeText 					[92] FreeText,

--	[freetext]					Urg(U)/Alr(H)/Resp(N)

dM93FreeText 					[93] FreeText,

--	[freetext]					Urg(D)/Alr(H)/Resp(N)

dM94FreeText 					[94] FreeText,

--	[freetext]					Urg(U)/Alr(M)/Resp(N)

dM95FreeText 					[95] FreeText,

--	[freetext]					Urg(U)/Alr(L)/Resp(N)

dM96FreeText 					[96] FreeText,

--	[freetext]					Urg(L)/Alr(L)/Resp(N)

dM97FreeText 					[97] FreeText,

--	[freetext]					Urg(N)/Alr(N)/Resp(N)

dM98FreeText 					[98] FreeText,

--	CURRENT DATA AUTHORITY			Urg(L)/Alr(L)/Resp(N)

dM99NULL 					[99] NULL,

--	LOGICAL ACKNOWLEDGMENT		Urg(N)/Alr(M)/Resp(N)

dM100NULL 					[100] NULL,

--	REQUEST END OF SERVICE			Urg(L)/Alr(L)/Resp(Y)

dM101NULL 					[101] NULL,

--	LANDING REPORT				Urg(N)/Alr(N)/Resp(N)

dM102NULL 					[102] NULL,

--	CANCELLING IFR				Urg(N)/Alr(N)/Resp(Y)

dM103NULL 					[103] NULL,

--	ETA[position][time]				Urg(L)/Alr(L)/Resp(N)

dM104PositionTime 				[104] PositionTime,

--	ALTERNATE AERODROME[airport]		Urg(L)/Alr(L)/Resp(N)

dM105Airport 					[105] Airport,

--	PREFERRED LEVEL[level]			Urg(L)/Alr(L)/Resp(N)

dM106Level 					[106] Level,

--	NOT AUTHORIZED NEXT DATA AUTHORITY	Urg(L)/Alr(L)/Resp(N)

dM107NULL					[107] NULL,

--	DE-ICING COMPLETE				Urg(L)/Alr(L)/Resp(N)

dM108NULL 					[108] NULL,

--	TOP OF DESCENT [time]			Urg(L)/Alr(L)/Resp(N),

dM109Time					[109] Time,

--	TOP OF DESCENT [position]			Urg(L)/Alr(L)/Resp(N),

dM110Position					[110] Position,

--	TOP OF DESCENT [time] [position]		Urg(L)/Alr(L)/Resp(N),

dM111TimePosition				[111] TimePosition,

--	SQUAWKING 7500				Urg(U)/Alr(H)/Resp(N),

dM112NULL					[112] NULL,

--	[speedType] [speedType] [speedType] SPEED [speed]													Urg(N)/Alr(L)/Resp(N),

dM113SpeedTypeSpeedTypeSpeedTypeSpeed														[113] SpeedTypeSpeedTypeSpeedTypeSpeed,

...

}

AircraftAddress::= BIT STRING (SIZE(24))

AircraftFlightIdentification ::= IA5String (SIZE (2..8))

Airport ::= IA5String (SIZE (4))

AirwayIdentifier ::= IA5String (SIZE (1..5))

AirwayIntercept ::= IA5String (SIZE (1..5))

Altimeter ::=  CHOICE

{

altimeterEnglish		[0]	AltimeterEnglish,

altimeterMetric		[1]	AltimeterMetric

}

AltimeterEnglish ::= INTEGER (2200..3200)

-- unit = Inches Mercury, Range (22.00 .. 32.00), resolution = 0.01

AltimeterMetric ::= INTEGER  (7500..12500)

-- unit = Hecto Pascal, Range (750.0..1250.0), resolution = 0.1

ATISCode ::= IA5String (SIZE (1))

ATWAlongTrackWaypoint ::= SEQUENCE

{

position 			[0]	Position,

aTWDistance 		[1]	ATWDistance,

speed 			[2]	Speed			OPTIONAL,

aTWLevels		[3]	ATWLevelSequence	OPTIONAL

}

ATWLevel ::= SEQUENCE

{

atw		ATWLevelTolerance,

level		Level

}

ATWLevelSequence ::= SEQUENCE SIZE (1..2) OF ATWLevel

ATWLevelTolerance ::= ENUMERATED

{

at		(0),

atorabove	(1),

atorbelow	(2)

}

ATWDistance ::= SEQUENCE

{

atwDistanceTolerance	ATWDistanceTolerance,

distance			Distance

}

ATWDistanceTolerance ::= ENUMERATED

{

plus		(0),

minus		(1)

}

ClearanceType ::= ENUMERATED

{

noneSpecified	(0),

approach	(1),

departure	(2),

further		(3),

start-up		(4),

pushback	(5),

taxi		(6),

take-off		(7),

landing		(8),

oceanic		(9),

en-route		(10),

downstream	(11),

...

}

Code ::=  SEQUENCE SIZE (4) OF CodeOctalDigit

CodeOctalDigit ::= INTEGER (0..7)

ControlledTime ::= SEQUENCE

{

time		Time,

timeTolerance	TimeTolerance

}

Date ::= SEQUENCE

{

year		Year,

month		Month,

day		Day

}

DateTimeGroup ::= SEQUENCE

{

date		Date,

timehhmmss	Timehhmmss

}

Day ::= INTEGER (1..31)

--unit = Day, Range (1..31), resolution = 1

DegreeIncrement ::= INTEGER (1..20)

--unit = Degree, Range (1..20), resolution = 1

Degrees ::=  CHOICE

{

degreesMagnetic		[0]	DegreesMagnetic,

degreesTrue 		[1]	DegreesTrue

}

DegreesMagnetic ::= INTEGER (1..360)

--unit = degree, Range (1..360), resolution = 1

DegreesTrue ::= INTEGER (1..360)

--unit = degree, Range (1..360), resolution = 1

DepartureClearance ::= SEQUENCE

{

aircraftFlightIdentification		[0]	AircraftFlightIdentification,

clearanceLimit			[1]	Position,

flightInformation			[2]	FlightInformation		OPTIONAL,

furtherInstructions		[3]	FurtherInstructions	OPTIONAL

}

DepartureMinimumInterval ::= INTEGER (1..150)

--unit = Minute, Range (0.1..15.0), resolution = 0.1

Direction ::= ENUMERATED

{

left			(0),

right			(1),

eitherSide		(2),

north			(3),

south			(4),

east			(5),

west			(6),

northEast  		(7),

northWest		(8),

southEast 		(9),

southWest		(10)

}

DirectionDegrees ::=  SEQUENCE

{

direction			Direction,

degrees 			Degrees

}

Distance ::= CHOICE

{

distanceNm 	[0]	DistanceNm,

distanceKm 	[1]	DistanceKm

}

DistanceKm ::= INTEGER (0..8000)

--	unit = Kilometer, Range (0..1000), resolution = 0.25

DistanceNm ::= INTEGER (0..9999)

--	unit = Nautical Mile, Range (0..999.9), resolution = 0.1

DistanceOffset ::= CHOICE

{

distanceOffsetNm	[0]	DistanceOffsetNm,

distanceOffsetKm 	[1]	DistanceOffsetKm

}

DistanceOffsetDirection ::=  SEQUENCE

{

distanceOffset 		DistanceOffset,

direction 		Direction

}

DistanceOffsetDirectionTime ::=  SEQUENCE

{

distanceOffsetDirection 	DistanceOffsetDirection,

time			Time

}

DistanceOffsetKm ::= INTEGER (1..500)

--	unit = Kilometer, Range (1..500), resolution = 1

DistanceOffsetNm ::= INTEGER (1..250)

--	unit = Nautical Mile, Range (1..250), resolution = 1

DistanceToNextPoint ::= CHOICE

{

distancetonextpointenglish		[0]	DistanceToNextPointEnglish,

distancetonextpointmetric		[1]	DistanceToNextPointMetric

}

DistanceToNextPointEnglish ::= INTEGER (1..10000)

--unit = Nautical Mile, Range (1..1000), resolution = 0.1

DistanceToNextPointMetric ::= INTEGER (1..20000)

--unit = Kilometer, Range (1..2000), resolution = 0.1

ErrorInformation ::=  ENUMERATED

{

unrecognizedMsgReferenceNumber	(0),

endServiceWithPendingMsgs 		(1),

logicalAcknowledgmentNotAccepted	(2),

moreThanOneNextDataAuthorityElement	(3),

insufficientResources	(4),

...

}

FacilityDesignation ::=  IA5String (SIZE (8))

FacilityFunction ::= ENUMERATED

{

center			(0),

approach 		(1),

tower			(2),

final			(3),

groundControl		(4),

clearanceDelivery	(5),

departure		(6),

control			(7)

}

FacilityDesignationAltimeter ::= SEQUENCE

{

facilityDesignation		UnitName,

altimeter			Altimeter

}

FacilityDesignationATISCode ::= SEQUENCE

{

facilityDesignation		UnitName,

aTISCode			ATISCode

}

FacilityIdentification ::=  CHOICE

{

facilityDesignation		[0]	FacilityDesignation,

facilityName		[1]	FacilityName

}

FacilityName ::= IA5String (SIZE (3..18))

FixName ::= IA5String (SIZE (1..5))

FlightInformation ::= CHOICE

{

routeOfFlight		[0]	RouteInformation,

levelsOfFlight		[1]	LevelsOfFlight,

routeAndLevels		[2]	RouteAndLevels

}

FreeText ::=  IA5String (SIZE (1..256))

Frequency ::=  CHOICE

{

frequencyhf		[0]	Frequencyhf,

frequencyvhf		[1]	Frequencyvhf,

frequencyuhf		[2]	Frequencyuhf,

frequencysatchannel	[3]	Frequencysatchannel,

frequencyvhfchannel	[4]	Frequencyvhfchannel

}

Frequencyhf ::= INTEGER (2850..28000)

--	unit = Kilohertz, Range (2850..28000), resolution = 1

Frequencysatchannel ::= NumericString (SIZE (12))

--	Frequencysatchannel corresponds to a 12 digit telephone number

Frequencyuhf ::= INTEGER  (9000..15999)

--	unit = Megahertz, Range (225.000..399.975), resolution = 0.025

Frequencyvhf ::= INTEGER (14000..17000)

--	unit = Megahertz, Range (117.00000..138.00000), resolution = 0.025

Frequencyvhfchannel ::= INTEGER (23600..27600)

--	Range (118.000..138.000), resolution = 0.005

FurtherInstructions ::= SEQUENCE

{

code		[0]	Code			OPTIONAL,

frequencyDeparture	[1]	UnitNameFrequency		OPTIONAL,

clearanceExpiryTime	[2]	Time				OPTIONAL,

airportDeparture		[3]	Airport				OPTIONAL,

airportDestination	[4]	Airport				OPTIONAL,

timeDeparture		[5]	TimeDeparture			OPTIONAL,

runwayDeparture		[6]	Runway				OPTIONAL,

revisionNumber		[7]	RevisionNumber			OPTIONAL,

aTISCode		[8]	ATISCode			OPTIONAL

}

Holdatwaypoint ::= SEQUENCE

{

position				[0]	Position,

holdatwaypointspeedlow		[1]	Speed		OPTIONAL,

aTWlevel			[2]	ATWLevel	OPTIONAL,

holdatwaypointspeedhigh		[3]	Speed		OPTIONAL,

direction				[4]	Direction	OPTIONAL,

degrees				[5]	Degrees		OPTIONAL,

eFCtime				[6]	Time		OPTIONAL,

legtype				[7]	LegType	OPTIONAL

}

HoldClearance ::=  SEQUENCE

{

position 			[0]	Position,

level			[1]	Level,

degrees			[2]	Degrees,

direction			[3]	Direction,

legType			[4]	LegType		OPTIONAL

}

InterceptCourseFrom ::= SEQUENCE

{

fromSelection		InterceptCourseFromSelection,

degrees			Degrees

}

InterceptCourseFromSelection ::= CHOICE

{

publishedIdentifier 		[0]	PublishedIdentifier,

latitudeLongitude 		[1]	LatitudeLongitude,

placeBearingPlaceBearing 	[2]	PlaceBearingPlaceBearing,

placeBearingDistance 		[3]	PlaceBearingDistance

}

Icing ::= ENUMERATED

{

trace		(0),

light		(1),

moderate	(2),

severe		(3)

}

Latitude ::= SEQUENCE

{

latitudeDegrees		[0]	LatitudeDegrees,

minutesLatLon		[1]	MinutesLatLon		OPTIONAL,

secondsLatLon		[2]	SecondsLatLon		OPTIONAL,

latitudeDirection		[3]	LatitudeDirection

}

LatitudeDegrees  ::= INTEGER (0..90000)

-- unit = Degree, Range (0..90), resolution = 0.001

LatitudeDirection ::= ENUMERATED

{

north		(0),

south		(1)

}

LatitudeLongitude ::= SEQUENCE

{

latitude		Latitude,

longitude	Longitude

}

LatitudeReportingPoints ::= SEQUENCE

{

latitudeDirection		LatitudeDirection,

latitudeDegrees		LatitudeDegrees

}

LatLonReportingPoints ::= CHOICE

{

latitudeReportingPoints		[0]	LatitudeReportingPoints,

longitudeReportingPoints		[1]	LongitudeReportingPoints

}

LegDistance ::= CHOICE

{

legDistanceEnglish		[0]	LegDistanceEnglish,

legDistanceMetric		[1]	LegDistanceMetric

}

LegDistanceEnglish ::= INTEGER (0..50)

-- unit = Nautical Mile, Range (0..50), resolution = 1

LegDistanceMetric ::= INTEGER (1..128)

-- unit = Kilometer, Range (1..128), resolution = 1

LegTime ::= INTEGER (0..10)

--unit = Minute, Range (0..10), resolution = 1

LegType ::= CHOICE

{

legDistance	[0]	LegDistance,

legTime		[1]	LegTime

}

Level ::= CHOICE

{

noLevel		[0]	NULL,

singleLevel		[1]	LevelType,

blockLevel		[2]	SEQUENCE SIZE (2) OF LevelType

}

LevelType ::=  CHOICE

{

levelFeet		[0]	LevelFeet,

levelMeters		[1]	LevelMeters,

levelFlightLevel	[2]	LevelFlightLevel,

levelFlightLevelMetric	[3]	LevelFlightLevelMetric

}

LevelLevel ::=  SEQUENCE SIZE (2) OF Level

LevelFlightLevel ::= INTEGER (30..700)

--unit = Level (100 Feet), Range (030..700), resolution = 1

LevelFlightLevelMetric ::= INTEGER (100..2500)

--unit = Level (10 Meters), Range (100..2500), resolution = 1

LevelPosition ::= SEQUENCE

{

level			Level,

position			Position

}

LevelProcedureName ::= SEQUENCE

{

level			Level,

procedureName		ProcedureName

}

LevelFeet ::=  INTEGER (-600..70000)

--unit = Feet, Range (-600..70000), resolution = 10

LevelMeters ::= INTEGER (-30..25000)

--unit = Meter, Range (-30..25000), resolution = 1

LevelSpeed ::= SEQUENCE

{

level			Level,

speed			Speed

}

LevelSpeedSpeed ::= SEQUENCE

{

level			Level,

speeds			SpeedSpeed

}

LevelTime ::= SEQUENCE

{

level			Level,

time			Time

}

LevelsOfFlight ::= CHOICE

{

level			[0]	Level,

procedureName		[1]	ProcedureName,

levelProcedureName	[2]	LevelProcedureName

}

Longitude ::= SEQUENCE

{

longitudeDegrees		[0]	LongitudeDegrees,

minutesLatLon		[1]	MinutesLatLon		OPTIONAL,

secondsLatLon		[2]	SecondsLatLon		OPTIONAL,

longitudeDirection	[3]	LongitudeDirection

}

LongitudeDegrees ::= INTEGER (0..180000)

--unit = Degree, Range (0..180), resolution = 0.001

LongitudeDirection ::= ENUMERATED

{

east		(0),

west		(1)

}

LongitudeReportingPoints ::= SEQUENCE

{

longitudeDirection	LongitudeDirection,

longitudeDegrees		LongitudeDegrees

}

MinutesLatLon ::= INTEGER (0..5999)

--unit = Minute, Range (0.. 59.99), resolution = 0.01

Month ::= INTEGER (1..12)

--unit = 1 Month, Range (1..12), resolution = 1

Navaid ::=  IA5String (SIZE (1..4) )

PersonsOnBoard ::= INTEGER (1..1024)

PlaceBearing ::= SEQUENCE

{

fixName			[0]	FixName,

latitudeLongitude		[1]	LatitudeLongitude	OPTIONAL,

degrees			[2]	Degrees

}

PlaceBearingDistance ::= SEQUENCE

{

fixName			[0]	FixName,

latitudeLongitude		[1]	LatitudeLongitude	OPTIONAL,

degrees			[2]	Degrees,

distance			[3]	Distance

}

PlaceBearingPlaceBearing ::= SEQUENCE SIZE (2) OF PlaceBearing

PointLevel ::= SEQUENCE

{

levelFlightLevel	[0]	LevelFlightLevel,

aTWLevelTolerance	[1]	ATWLevelTolerance	OPTIONAL

}

PointLevelBlock ::= SEQUENCE SIZE (2) OF LevelFlightLevel

PointDetail ::= SEQUENCE

{

latitudeLongitude	[0]	LatitudeLongitude,

fixName			[1]	FixName		OPTIONAL,

truetrackangle		[2]	DegreesTrue		OPTIONAL,

distancetonextpoint	[3]	DistanceToNextPoint	OPTIONAL,

pointLevel		[4]	PointLevel		OPTIONAL,

pointLevelBlock	[5]	PointLevelBlock	OPTIONAL

}

Position ::=  CHOICE

{

fixName			[0]	FixName,

navaid			[1]	Navaid,

airport			[2]	Airport,

latitudeLongitude		[3]	LatitudeLongitude,

placeBearingDistance	[4]	PlaceBearingDistance

}

PositionLevel ::= SEQUENCE

{

position			Position,

level			Level

}

PositionLevelLevel ::= SEQUENCE

{

position			Position,

levels      		LevelLevel

}

PositionLevelSpeed ::= SEQUENCE

{

positionlevel		PositionLevel,

speed			Speed

}

PositionDegrees ::=  SEQUENCE

{

position			Position,

degrees			Degrees

}

PositionDistanceOffsetDirection ::=  SEQUENCE

{

position			Position,

distanceOffsetDirection	DistanceOffsetDirection

}

PositionUnitNameFrequency ::= SEQUENCE

{

position			Position,

unitname		UnitName,

frequency		Frequency

}

PositionPosition ::=  SEQUENCE SIZE (2) OF Position

PositionProcedureName ::= SEQUENCE

{

position			Position,

procedureName		ProcedureName

}

PositionReport ::= SEQUENCE

{

positioncurrent		 	[0]	Position,

timeatpositioncurrent 		[1]	Time,

level				[2]	Level,

fixnext				[3]	Position		OPTIONAL,

timeetaatfixnext			[4]	Time		OPTIONAL,

fixnextplusone			[5]	Position		OPTIONAL,

timeetaatdestination		[6]	Time		OPTIONAL,

remainingFuel			[7]	RemainingFuel	OPTIONAL,

temperature			[8]	Temperature	OPTIONAL,

winds	 			[9]	Winds		OPTIONAL,

turbulence			[10]	Turbulence	OPTIONAL,

icing				[11]	Icing		OPTIONAL,

speed				[12]	Speed		OPTIONAL,

speedground			[13]	SpeedGround	OPTIONAL,

verticalChange			[14]	VerticalChange	OPTIONAL,

trackAngle			[15]	Degrees		OPTIONAL,

heading				[16]	Degrees		OPTIONAL,

distance				[17]	Distance		OPTIONAL,

supplementaryInformation		[18]	FreeText	OPTIONAL,

reportedWaypointPosition		[19]	Position		OPTIONAL,

reportedWaypointTime		[20]	Time		OPTIONAL,

reportedWaypointLevel		[21]	Level		OPTIONAL

}

PositionRouteClearance ::= SEQUENCE

{

position			Position,

routeClearance		RouteClearance

}

PositionSpeed ::= SEQUENCE

{

position			Position,

speed			Speed

}

PositionSpeedSpeed ::= SEQUENCE

{

position			Position,

speeds			SpeedSpeed

}

PositionTime ::= SEQUENCE

{

position			Position,

time			Time

}

PositionTimeLevel ::= SEQUENCE

{

positionTime		PositionTime,

level			Level

}

PositionTimeTime ::= SEQUENCE

{

position			Position,

times 	 		TimeTime

}

Procedure ::= IA5String (SIZE (1..20))

ProcedureName ::= SEQUENCE

{

type			[0]	ProcedureType,

procedure		[1]	Procedure,

transition		[2]	ProcedureTransition	OPTIONAL

}

ProcedureTransition ::= IA5String (SIZE (1..5))

ProcedureType ::= ENUMERATED

{

arrival			(0),

approach		(1),

departure		(2)

}

PublishedIdentifier ::= SEQUENCE

{

fixName			[0]	FixName,

latitudeLongitude	[1]	LatitudeLongitude	OPTIONAL

}

RemainingFuel ::= Time

RemainingFuelPersonsOnBoard ::=  SEQUENCE

{

remainingFuel	RemainingFuel,

personsOnBoard	PersonsOnBoard

}

ReportingPoints ::= SEQUENCE

{

latLonReportingPoints	[0]	LatLonReportingPoints,

degreeIncrement		[1]	DegreeIncrement			OPTIONAL

}

RevisionNumber ::= INTEGER (1..16)

RouteAndLevels ::= SEQUENCE

{

routeOfFlight		RouteInformation,

levelsOfFlight		LevelsOfFlight

}

RouteClearance ::= SEQUENCE

{

airportDeparture			[0]	Airport				OPTIONAL,

airportDestination		[1]	Airport				OPTIONAL,

runwayDeparture			[2]	Runway				OPTIONAL,

procedureDeparture		[3]	ProcedureName			OPTIONAL,

runwayArrival			[4]	Runway				OPTIONAL,

procedureApproach		[5]	ProcedureName			OPTIONAL,

procedureArrival			[6]	ProcedureName			OPTIONAL,

airwayIntercept			[7]	AirwayIntercept 			OPTIONAL,

routeInformations			[8]	SEQUENCE SIZE (1..128)

OF RouteInformation		OPTIONAL,

routeInformationAdditional	[9]	RouteInformationAdditional	OPTIONAL

}

RouteInformation ::= CHOICE

{

publishedIdentifier 		[0]	PublishedIdentifier,

latitudeLongitude 		[1]	LatitudeLongitude,

placeBearingPlaceBearing 	[2]	PlaceBearingPlaceBearing,

placeBearingDistance 		[3]	PlaceBearingDistance,

airwayIdentifier 			[4]	AirwayIdentifier,

trackDetail 			[5]	TrackDetail

}

RouteInformationAdditional ::= SEQUENCE

{

aTWAlongTrackWaypoints	[0]	SEQUENCE SIZE (1..8) OF ATWAlongTrackWaypoint  OPTIONAL,

reportingpoints			[1]	ReportingPoints					       OPTIONAL,

interceptCourseFroms		[2]	SEQUENCE SIZE (1..4) OF InterceptCourseFrom	       OPTIONAL,

holdAtWaypoints			[3]	SEQUENCE SIZE (1..8) OF Holdatwaypoint	       OPTIONAL,

waypointSpeedLevels		[4]	SEQUENCE SIZE (1..32) OF WaypointSpeedLevel     OPTIONAL,

rTARequiredTimeArrivals	[5]	SEQUENCE SIZE (1..32) OF RTARequiredTimeArrival													       OPTIONAL

}

RTARequiredTimeArrival ::= SEQUENCE

{

position			[0]	Position,

rTATime		[1]	RTATime,

rTATolerance		[2]	RTATolerance		OPTIONAL

}

RTATime ::= SEQUENCE

{

time			Time,

timeTolerance		TimeTolerance

}

RTATolerance ::= INTEGER (1..150)

--unit= Minute, Range (0.1..15.0), resolution = 0.1

Runway ::= SEQUENCE

{

direction		RunwayDirection,

configuration		RunwayConfiguration

}

RunwayDirection ::= INTEGER (1..36)

RunwayConfiguration ::= ENUMERATED

{

left		(0),

right		(1),

center		(2),

none		(3)

}

RunwayRVR ::= SEQUENCE

{

runway			Runway,

rVR			RVR

}

RVR ::= CHOICE

{

rVRFeet 	[0]	RVRFeet,

rVRMeters 	[1]	RVRMeters

}

RVRFeet ::= INTEGER (0..6100)

-- unit = Feet, Range (0..6100), resolution = 1

RVRMeters ::= INTEGER (0..1500)

-- unit = Meters (0..1500), resolution = 1

SecondsLatLon ::= INTEGER (0..59)

--unit = Second, Range (0.. 59), resolution = 1

Speed ::=  CHOICE

{

speedIndicated 		[0]	SpeedIndicated,

speedIndicatedMetric 	[1]	SpeedIndicatedMetric,

speedTrue 		[2]	SpeedTrue,

speedTrueMetric 		[3]	SpeedTrueMetric,

speedGround 		[4]	SpeedGround,

speedGroundMetric 	[5]	SpeedGroundMetric,

speedMach 		[6]	SpeedMach

}

SpeedIndicated ::= INTEGER (0..400)

-- unit = Knots, Range (0..400), resolution = 1

SpeedIndicatedMetric ::= INTEGER (0..800)

-- unit = Kilometers/Hour, Range (0..800), resolution = 1

SpeedGround ::= INTEGER (-50..2000)

-- unit = Knots, Range (-50..2000), resolution = 1

SpeedGroundMetric ::= INTEGER (-100..4000)

-- unit = Kilometers/Hour, Range (-100..4000), resolution = 1

SpeedMach ::= INTEGER (50..400)

-- unit = Mach Range (0.5 to 4.0), resolution = 0.001

SpeedSpeed ::=  SEQUENCE SIZE (2) OF Speed

SpeedTime ::= SEQUENCE

{

speed			Speed,

time			Time

}

SpeedTrue ::= INTEGER (7..70)

-- unit = Knots, Range (70..700), resolution = 10

SpeedTrueMetric ::= INTEGER (10..137)

-- unit = Kilometers/Hour, Range (100..1370), resolution = 10

SpeedType ::= ENUMERATED

{

noneSpecified	(0),

indicated	(1),

true		(2),

ground		(3),

mach		(4),

approach	(5),

cruise		(6),

minimum	(7),

maximum	(8),

...

}

SpeedTypeSpeedTypeSpeedType ::= SEQUENCE SIZE (3) OF SpeedType

SpeedTypeSpeedTypeSpeedTypeSpeed ::= SEQUENCE

{

speedTypes	SpeedTypeSpeedTypeSpeedType,

speed		Speed

}

Temperature ::= INTEGER (-100..100)

-- unit = Degree Centigrade, Range (-100..100), resolution = 1

Time ::= SEQUENCE

{

hours			TimeHours,

minutes			TimeMinutes

}

TimeLevel ::= SEQUENCE

{

time		Time,

level		Level

}

TimeDeparture ::= SEQUENCE

{

timeDepartureAllocated		[0]	Time				OPTIONAL,

timeDepartureControlled		[1]	ControlledTime			OPTIONAL,

timeDepartureClearanceExpected	[2]	Time				OPTIONAL,

departureMinimumInterval	[3]	DepartureMinimumInterval	OPTIONAL

}

TimeDistanceOffsetDirection ::= SEQUENCE

{

time			Time,

distanceOffsetDirection	DistanceOffsetDirection

}

TimeDistanceToFromPosition ::=  SEQUENCE

{

time			Time,

distance			Distance,

tofrom			ToFrom,

position			Position

}

Timehhmmss ::= SEQUENCE

{

hoursminutes		Time,

seconds			TimeSeconds

}

TimeHours ::= INTEGER (0..23)

-- unit = Hour, Range (0..23), resolution = 1

TimeUnitNameFrequency ::=  SEQUENCE

{

time			Time,

unitName		UnitName,

frequency		Frequency

}

TimeMinutes ::= INTEGER (0..59)

-- unit = Minute, Range (0..59), resolution = 1

TimePosition ::=  SEQUENCE

{

time			Time,

position			Position

}

TimePositionLevel ::=    SEQUENCE

{

timeposition		TimePosition,

level			Level

}

TimePositionLevelSpeed ::= SEQUENCE

{

timeposition		TimePosition,

levelspeed		LevelSpeed

}

TimeSeconds ::= INTEGER (0..59)

-- unit = Second, Range (0..59), resolution = 1

TimeSpeed ::=  SEQUENCE

{

time			Time,

speed			Speed

}

TimeSpeedSpeed ::=  SEQUENCE

{

time			Time,

speedspeed	 	SpeedSpeed

}

TimeTime ::= SEQUENCE SIZE (2) OF Time

TimeToFromPosition ::= SEQUENCE

{

time			Time,

tofrom			ToFrom,

position			Position

}

TimeTolerance ::= ENUMERATED

{

at		(0),

atorafter		(1),

atorbefore	(2)

}

ToFrom ::= ENUMERATED

{

to		(0),

from		(1)

}

ToFromPosition ::=  SEQUENCE

{

toFrom		ToFrom,

position		Position

}

TrackDetail ::= SEQUENCE

{

trackName		TrackName,

latitudeLongitudes	SEQUENCE SIZE (1..128) OF LatitudeLongitude

}

TrackName ::= IA5String (SIZE (3..6))

TrafficType ::= ENUMERATED

{

noneSpecified		(0),

oppositeDirection		(1),

sameDirection		(2),

converging		(3),

crossing			(4)

}

Turbulence ::= ENUMERATED

{

light		(0),

moderate	(1),

severe		(2)

}

UnitName ::=   SEQUENCE

{

facilityId		FacilityIdentification,

facilityFunction	FacilityFunction

}

UnitNameFrequency ::=  SEQUENCE

{

unitName			UnitName,

frequency			Frequency

}

VersionNumber ::=  INTEGER (0..15)

VerticalChange ::= SEQUENCE

{

direction			VerticalDirection,

rate			VerticalRate

}

VerticalDirection ::= ENUMERATED

{

up	(0),

down	(1)

}

VerticalRate ::=  CHOICE

{

verticalRateEnglish 		[0]	VerticalRateEnglish,

verticalRateMetric 		[1]	VerticalRateMetric

}

VerticalRateEnglish ::= INTEGER (0..3000)

-- unit = Feet/Minute, Range (0..30000), resolution = 10

VerticalRateMetric ::= INTEGER (0..1000)

-- unit = Meters/Minute, Range (0..10000), resolution = 10

WaypointSpeedLevel ::= SEQUENCE

{

position 			[0]	Position,

speed			[1]	Speed			OPTIONAL,

aTWLevels		[2]	ATWLevelSequence	OPTIONAL

}

WindDirection ::= INTEGER (1..360)

-- unit = Degree, Range (1..360), resolution = 1

Winds ::= SEQUENCE

{

direction		WindDirection,

speed			WindSpeed

}

WindSpeed ::= CHOICE

{

windSpeedEnglish	[0]	WindSpeedEnglish,

windSpeedMetric		[1]	WindSpeedMetric

}

WindSpeedEnglish ::= INTEGER (0..255)

-- unit = Knot, Range (0..255), resolution = 1

WindSpeedMetric ::= INTEGER (0..511)

-- unit = Kilometer/Hour, Range (0..511), resolution = 1

Year ::= INTEGER (1996..2095)

-- unit = Year, Range (1996..2095), resolution = 1

END

# PROTOCOL DEFINITION

## Sequence Rules

### With the exception of abort primitives, only the sequence of primitives illustrated in figures 2.3.5-1 to 2.3.5-18  shall be permitted.

- Note 1. — The following figures define the valid sequences of primitives that are possible to be invoked during the operation of the CPDLC application.  It shows the relationship in time between the service request and the resulting indication, and if applicable, the subsequent response and resulting confirmation.

- Note 2. — Abort primitives may interrupt and terminate any of the normal message sequences outlined below.

- Note 3.— Primitives are processed in the order received.  See 4.4.3.

Figure 2.3.5-1.  Sequence Diagram for CPDLC-start Service

Air Initiated

Figure 2.3.5-2.  Sequence Diagram for CPDLC-start Service

Ground Initiated

Figure 2.3.5-3.  Sequence Diagram for DSC-start Service

Figure 2.3.5-4.  Sequence Diagram for CPDLC-message Service

Air Initiated

Figure 2.3.5-5. Sequence Diagram for CPDLC-message Service

Ground Initiated

Figure 2.3.5-6.  Sequence Diagram for CPDLC-end Service

Figure 2.3.5-7.  Sequence Diagram for DSC-end Service

Figure 2.3.5-8.  Sequence Diagram for CPDLC-forward Service

Ground Forwarding Supported, ASE Version Numbers the Same

Figure 2.3.5-9.  Sequence Diagram for CPDLC-forward Service

Ground Forwarding Not Supported, or

Ground Forwarding Supported and ASE Version Numbers Not the Same

Figure 2.3.5-10.  Sequence Diagram for CPDLC-user-abort Service

CPDLC-Air-User Initiated

Figure 2.3.5-11.  Sequence Diagram for CPDLC-user-abort Service

CPDLC-Ground-User Initiated

Figure 2.3.5-12.  Sequence Diagram for CPDLC-provider-abort Service

Dialogue Service Abort

Figure 2.3.5-13.  Sequence Diagram for CPDLC-provider-abort Service

CPDLC-Air-ASE Abort

Figure 2.3.5-14.  Sequence Diagram for CPDLC-provider-abort Service

CPDLC-Ground-ASE Abort

Figure 2.3.5-15.  Sequence Diagram for CPDLC-user-abort Service

Sending CPDLC-Ground-User Initiated

- .

Figure 2.3.5-16.  Sequence Diagram for CPDLC-provider-abort Service

Dialogue Service Abort

Figure 2.3.5-17.  Sequence Diagram for CPDLC-provider-abort Service

Receiving CPDLC-Ground-ASE Abort

Figure 2.3.5-18.  Sequence Diagram for CPDLC-provider-abort Service

Sending CPDLC-Ground-ASE Abort

## CPDLC Service Provider Timers

### A CPDLC-ASE shall be capable of detecting when a timer expires.

- Note 1. — Table 2.3.5-1 lists the time constraints related to the CPDLC application.  Each time constraint requires a timer to be set in the CPDLC protocol machine.

Note 2. — If the timer expires before the final event has occurred, a CPDLC-ASE takes appropriate action as defined in 2.3.5.4.1.

### Recommendation. — The timer values should be as indicated in Table 2.3.5-1.

Table 2.3.5-1.  CPDLC Service Provider Timers

| CPDLC Service | Timer | Timer Value | Timer Start Event | Timer Stop Event |
| --- | --- | --- | --- | --- |
| CPDLC-start | tstart | 6 minutes | D-START request | D-START confirmation |
| DSC-start | tstart | 6 minutes | D-START request | D-START confirmation |
| CPDLC-forward | tstart | 6 minutes | D-START request | D-START confirmation |

Note. — The receipt of CPDLC-user-abort requests, D-ABORT Indications, or D-P-ABORT Indications are also timer stop events.

## CPDLC-Air-ASE Protocol Description

### Introduction

#### If no actions are described for a CPDLC service primitive when a CPDLC-air-ASE is in a specific state, then the invocation of that service primitive shall be prohibited while the CPDLC-air-ASE is in that state.

#### Upon receipt of a PDU, if no actions are described for the arrival of that PDU when a CPDLC-air-ASE is in a specific state, then that PDU is considered not permitted, and exception handling procedures as described in 2.3.5.4.4 shall apply.

#### If a PDU is received that cannot be decoded, then exception handling procedures as described in 2.3.5.4.3 for invalid PDU shall apply.

#### If a PDU is not received when one is required, then exception handling as described in 2.3.5.4.3 shall apply.

Note 1. — The states defined for the CPDLC-air-ASE are the following.

- IDLE

- START-REQ,

- START-IND,

- DIALOGUE, and

- END.

Note 2. — The CPDLC-air-user is an active user from:

- the time it has invoked the CPDLC-start service request until:

  - receipt of a CPDLC-start service confirmation with Result parameter equal to the abstract value “rejected”, or

  - invocation of a CPDLC-end service response with the Result parameter set to the abstract value “accepted”, or

  - invocation of a CPDLC-user-abort service request, or

  - receipt of CPDLC-user-abort service indication, or

  - receipt of a CPDLC-provider-abort service indication; or

- the time it has received the CPDLC-start service indication until:

  - invocation of a CPDLC-start service response with Result parameter equal to the abstract value “rejected”, or

  - invocation of a CPDLC-end service response with the Result parameter set to the abstract value “accepted”, or

  - invocation of a CPDLC-user-abort service request, or

  - receipt of CPDLC-user-abort service indication, or

  - receipt of a CPDLC-provider-abort service indication; or

- the time it has invoked the DSC-start service request until:

  - receipt of a DSC-start service confirmation with Result parameter equal to the abstract value “rejected”, or

  - receipt of a DSC-end service confirmation with Result parameter equal to the abstract value “accepted”, or

  - invocation of a CPDLC-user-abort service request, or

  - receipt of CPDLC-user-abort service indication, or

  - receipt of a CPDLC-provider-abort service indication.

#### On initiation the CPDLC-air-ASE shall be in the IDLE state.

- Note. — The CPDLC-air-ASE contains a Boolean called DSC.  DSC has the abstract value “true” when the dialogue is a DSC dialogue, and has the abstract value “false” otherwise.

#### On the initiation of a CPDLC-air-ASE, DSC shall be set to the abstract value “false”.

### D-START Indication

#### Upon receipt of a D-START indication, if the CPDLC-air-ASE is in the IDLE state and the D-START User Data parameter contains a GroundPDUs [UplinkMessage] APDU, the CPDLC-air-ASE shall:

- Invoke CPDLC-start service indication containing the following:

  - the D-START Calling Peer ID parameter value as the CPDLC-start service Calling Peer Identifier parameter value,

  - the D-START QOS Routing Class parameter value as the CPDLC-start service Class of Communication parameter value,

  - if the GroundPDUs [UplinkMessage] APDU contained in the D-START User Data parameter is an ATCUplinkMessage, set the GroundPDUs APDU-element as the CPDLC-start service CPDLC Message parameter value, and

- Enter the START-IND state.

### D-START Confirmation

#### Upon receipt of a D-START confirmation, if the CPDLC-air-ASE is in the START-REQ state and the D-START Result parameter has the abstract value “accepted” and DSC has the abstract value “false” and D-START User Data parameter is not provided, the CPDLC-air-ASE shall:

- Stop timer tstart,

- Invoke CPDLC-start service confirmation with the abstract value “accepted” as the CPDLC-start service Result parameter value,

- Enter the DIALOGUE state.

#### Upon receipt of a D-START confirmation, if the CPDLC-air-ASE is in the START-REQ state and the D-START Result parameter has the abstract value “rejected (permanent)” and the D-START Reject Source parameter has the abstract value “DS user” and DSC has the abstract value “false” and if the D-START User Data parameter is provided, the User Data parameter contains a GroundPDUs [ATCUplinkMessage] APDU, the CPDLC-air-ASE shall:

- Stop timer tstart,

- Invoke CPDLC-start service confirmation containing the following:

  - if the D-START User Data parameter is provided, the APDU contained in the D-START User Data parameter as the CPDLC-start service Reject Reason parameter value, and

  - the abstract value “rejected” as the CPDLC-start service Result parameter value, and

- Enter the IDLE state.

#### Upon receipt of a D-START confirmation, if the CPDLC-air-ASE is in the START-REQ state and the D-START Result parameter has the abstract value “accepted” and DSC has the abstract value “true” and D-START User Data parameter is not provided, the CPDLC-air-ASE shall:

- Stop timer tstart,

- Invoke DSC-start service confirmation with the abstract value “accepted” as the DSC-start service Result parameter value,

- Enter the DIALOGUE state.

#### Upon receipt of a D-START confirmation, if the CPDLC-air-ASE is in the START-REQ state and the D-START Result parameter has the abstract value “rejected (permanent)” and the D-START Reject Source parameter has the abstract value “DS user”, and DSC has the abstract value “true”, and if the D-START User Data parameter is provided, the User Data parameter contains a GroundPDUs [ATCUplinkMessage] APDU, the CPDLC-air-ASE shall:

- Stop timer tstart,

- Invoke DSC-start service confirmation containing the following:

  - if the D-START User Data parameter is provided, the APDU contained in the D-START User Data parameter as the CPDLC-start service Reject Reason parameter value, and

  - the abstract value “rejected” as the DSC-start service Result parameter value,

- Set DSC to the abstract value “false”, and

- Enter the IDLE state.

### D-DATA Indication

#### Upon receipt of a D-DATA indication, if the CPDLC-air-ASE is in the DIALOGUE state and the APDU contained in the D-DATA User Data parameter is a GroundPDUs [ATCUplinkMessage] APDU, the CPDLC-air-ASE shall:

- Invoke CPDLC-message service indication with the APDU contained in the D-DATA User Data parameter as the CPDLC-message service CPDLC Message parameter value, and

- Remain in the DIALOGUE state.

#### Upon receipt of a D-DATA indication, if the CPDLC-air-ASE is in the END state and DSC has the abstract value of “true” and the APDU contained in the D-DATA User Data parameter is a GroundPDUs [ATCUplinkMessage] APDU, the CPDLC-air-ASE shall:

- Invoke CPDLC-message service indication with the APDU contained in the D-DATA User Data parameter as the CPDLC-message service CPDLC Message parameter value, and

- Remain in the END state.

### D-END Indication

#### Upon receipt of a D-END indication, if the CPDLC-air-ASE is in the DIALOGUE state, and DSC has the abstract value “false”, and if the D-END User Data parameter is provided, the User Data parameter contains a GroundPDUs [ATCUplinkMessage] APDU, the CPDLC-air-ASE shall:

- Invoke CPDLC-end service indication with the APDU contained in the D-END User Data parameter as the CPDLC-message service CPDLC Message parameter value, if provided, as the CPDLC-end service CPDLC Message parameter value, and

- Enter the END state

### D-END Confirmation

#### Upon receipt of a D-END confirmation, if the CPDLC-air-ASE is in the END state and the abstract the D-END Result parameter has the abstract value “accepted” and DSC has the abstract value “true” and if the D-END User Data parameter is provided, the User Data parameter contains a GroundPDUs [ATCUplinkMessage] APDU, the CPDLC-air-ASE shall:

- Invoke DSC-end service confirmation with:

  - if the D-END User Data parameter is provided, the APDU contained in the D-END User Data parameter as the DSC-end service CPDLC Message parameter value, and

  - the abstract value “accepted” as the CPDLC-end service Result parameter value,

- Set DSC to the abstract value “false”, and

- Enter the IDLE state.

#### Upon receipt of a D-END confirmation, if the CPDLC-air-ASE is in the END state and the D-END Result parameter has the abstract value “rejected”, and DSC has the abstract value “true”, and if the D-END User Data parameter is provided, the User Data parameter contains a GroundPDUs [ATCUplinkMessage] APDU, the CPDLC-air-ASE shall:

- Invoke DSC-end service confirmation with:

  - if the D-END User Data parameter is provided, the APDU contained in the D-END User Data parameter as the DSC-end service CPDLC Message parameter value, and

  - the abstract value “rejected” as the CPDLC-end service Result parameter value:

- Enter the DIALOGUE state.

### CPDLC-start Service Request

#### Upon receipt of a CPDLC-start service request, if the CPDLC-air-ASE is in the IDLE state, the CPDLC-air-ASE shall:

- Create an AircraftPDUs APDU with a StartDownMessage APDU element containing:

  - the abstract value “cpdlc” as the mode,

  - if provided, the CPDLC Message parameter as the DownlinkMessage, or

  - else, NULL as the DownlinkMessage,

- Invoke D-START request with the following:

  - the CPDLC-start service Called Peer Identifier parameter value as the D-START Called Peer ID parameter value,

  - the CPDLC-start service Calling Peer Identifier parameter value as the D-START Calling Peer ID parameter value,

  - the D-START Quality of Service parameters set as follows:

    - if provided, the CPDLC-start service Class of Communication parameter value as the D-START QOS Routing Class parameter value, or

    - The abstract value of “high priority flight safety messages”, as the D-START QOS Priority parameter value, and

    - The abstract value of “low” as the D-START QOS Residual Error Rate parameter value, and

  - the APDU as the D-START User Data parameter value;

- Start timer tstart, and

- Enter the START-REQ state.

### CPDLC-start Service Response

#### Upon receipt of a CPDLC-start service response, if the CPDLC-air-ASE is in the START-IND state and the CPDLC-start service Result parameter has the abstract value “accepted” and the CPDLC-start service Reject Reason parameter is not provided, and DSC has the abstract value “false”, the CPDLC-air-ASE shall:

- Invoke D-START response with the abstract value “accepted” as the D-START Result parameter value, and

- Enter the DIALOGUE state.

#### Upon receipt of a CPDLC-start service response, if the CPDLC-air-ASE is in the START-IND state, and the CPDLC-start service Result parameter has the abstract value “rejected” and DSC has the abstract value “false”, the CPDLC-air-ASE shall:

- If the CPDLC-start service Reject Reason parameter is provided, create an AircraftPDUs APDU with an ATCDownlinkMessage APDU element based on the Reject Reason parameter,

- Invoke D-START response with the following:

  - if created, the APDU as the D-START User Data parameter, and

  - the abstract value “rejected (permanent)” as the D-START Result parameter value, and

- Enter the IDLE state.

### DSC-start Service Request

#### Upon receipt of a DSC-start service request, if the CPDLC-air-ASE is in the IDLE state, the CPDLC-air-ASE shall:

- Create an AircraftPDUs APDU with a StartDownMessage APDU element containing:

  - the abstract value “dsc” as the mode,

  - if provided, the CPDLC Message parameter as the DownlinkMessage, or

  - else, NULL as the DownlinkMessage,

- Invoke D-START request with the following:

  - the DSC-start service ICAO Facility Designator parameter value as the D-START Called Peer ID parameter value,

  - the DSC-start service Aircraft Identifier parameter value as the D-START Calling Peer ID parameter value,

  - Set the D-START Quality of Service parameters as follows:

    - Class of Communication parameter from the DSC-start service request if provided,

    - The abstract value of “high priority flight safety messages”, as the D-START QOS Priority parameter value, and

    - The abstract value of “low” as the D-START QOS Residual Error Rate parameter value, and

  - the APDU as the D-START User Data parameter value;

- Set DSC to the abstract value “true”,

- Start timer tstart, and

- Enter the START-REQ state.

### CPDLC-message Service Request

#### Upon receipt of a CPDLC-message service request, if the CPDLC-air-ASE is in the DIALOGUE state, the CPDLC-air-ASE shall:

- Create an AircraftPDUs APDU with an ATCDownlinkMessage APDU-element based on the CPDLC-message service CPDLC Message parameter,

- Invoke D-DATA request with the APDU as the D-DATA User Data parameter value, and

- Remain in the DIALOGUE state.

#### Upon receipt of a CPDLC-message service request, if the CPDLC-air-ASE is in the END state and DSC has the abstract value “false”, the CPDLC-air-ASE shall:

- Create an AircraftPDUs APDU with an ATCDownlinkMessage APDU-element based on the CPDLC-message service CPDLC Message parameter,

- Invoke D-DATA request with the APDU as the D-DATA User Data parameter value, and

- Remain in the END state.

### CPDLC-end Service Response

#### Upon receipt of a CPDLC-end service response, if the CPDLC-air-ASE is in the END state, and the CPDLC-end service Result parameter has the abstract value “accepted” and DSC has the abstract value “false”, the CPDLC-air-ASE shall:

- Create an AircraftPDUs APDU with an ATCDownlinkMessage APDU-element based on the CPDLC-end service CPDLC Message parameter, if provided,

- Invoke D-END response with the following:

  - if created, the APDU as the D-END User Data parameter value, and

  - the abstract value “accepted”, as the D-END Result parameter value, and

- Enter the IDLE state.

#### Upon receipt of a CPDLC-end service response, if the CPDLC-air-ASE is in the END state, and the CPDLC-end service Result parameter has the abstract value “rejected” and DSC has the abstract value “false”, the CPDLC-air-ASE shall:

- Create an AircraftPDUs APDU with an ATCDownlinkMessage APDU-element based on the CPDLC-end service CPDLC Message parameter, if provided,

- Invoke D-END response with the following:

  - if created, the APDU as the D-END User Data parameter value, and

  - the abstract value “rejected”, as the D-END Result parameter value, and

- Enter the DIALOGUE state

### DSC-end Service Request

#### Upon receipt of a DSC-end service request, if the CPDLC-air-ASE is in the DIALOGUE state and DSC has the abstract value “true”, the CPDLC-air-ASE shall:

- Create an AircraftPDUs APDU with an ATCDownlinkMessage APDU-element based on the DSC-end service CPDLC Message parameter, if provided,

- Invoke D-END request with the APDU as the D-END User Data parameter value, if provided, and

- Enter the END state.

### CPDLC-user-abort Service Request

#### Upon receipt of a CPDLC-user-abort service request, if the CPDLC-air-ASE is not in the IDLE state, the CPDLC-air-ASE shall:

- Stop any timer,

- If the CPDLC-user-abort service Reason parameter is provided, create an AircraftPDUs APDU with a CPDLCUserAbortReason APDU-element based on the CPDLC-user-abort service Reason parameter,

- Else create an AircraftPDUs APDU with a CPDLCUserAbortReason [undefined] APDU-element

- Invoke D-ABORT request with the following:

  - the D-ABORT Originator parameter set to the abstract value “user”, and

  - the APDU as the D-ABORT User Data parameter value, and

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### D-ABORT Indication

#### Upon receipt of a D-ABORT indication, if the CPDLC-air-ASE is not in the IDLE state, and the D-ABORT Originator parameter is “user” and the D-ABORT User Data parameter contains a GroundPDUs [CPDLCUserAbortReason] APDU, the CPDLC-air-ASE shall:

- Stop any timer,

- If the CPDLC-air-user is an active user, invoke CPDLC-user-abort service indication with the APDU contained in the D-ABORT User Data parameter as the CPDLC-user-abort service Reason parameter value,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

#### Upon receipt of a D-ABORT indication, if the CPDLC-air-ASE is not in the IDLE state, and if the D-ABORT Originator parameter is “provider” and if the D-ABORT User Data parameter is provided, the D-ABORT User Data parameter contains a GroundPDUs [CPDLCProviderAbortReason] APDU, the CPDLC-air-ASE shall:

- Stop any timer,

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the D-ABORT User Data parameter as the CPDLC-provider-abort service Reason parameter value, if provided,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### D-P-ABORT Indication

#### Upon receipt of a D-P-ABORT indication, if the CPDLC-air-ASE is not in the IDLE state, the CPDLC-air-ASE shall:

- Stop any timer,

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the CPDLC-provider-abort service Reason parameter set to the abstract value “communication-service-failure”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

## CPDLC-Air-ASE Exception Handling

### A Timer Expires

#### If a CPDLC-air-ASE detects that a timer has expired, that CPDLC-air-ASE shall:

- Interrupt any current activity,

- Create an AircraftPDUs APDU with a CPDLCProviderAbortReason [timer-expired] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “timer-expired” as the CPDLC-provider abort service Reason parameter value,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### Unrecoverable System Error

#### If a CPDLC-air-ASE has an unrecoverable system error, the CPDLC-air-ASE shall:

- Stop any timer,

- Create an AircraftPDUs APDU with a CPDLCProviderAbortReason [undefined-error] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “undefined-error” as the CPDLC-provider abort service Reason parameter value,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### Invalid PDU

#### If the User Data parameter of a D-END confirmation with Result parameter set to the abstract value “rejected”, or if the User Data parameter of a D-START indication, a D-DATA indication, or a D-END indication, does not contain a valid PDU, the CPDLC-air-ASE shall:

- Stop any timer,

- Create an AircraftPDUs APDU with a CPDLCProviderAbortReason [invalid-PDU] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “invalid-PDU” as the CPDLC-provider abort service Reason parameter value,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

#### If the User Data parameter of a D-START confirmation with Result set to the abstract value “rejected (permanent)”, or a D-END confirmation with Result set to the abstract value “accepted”, is not a valid PDU then the CPDLC-air-ASE shall:

- Stop any timer,

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the CPDLC-provider-abort service Reason parameter set to the abstract value “invalid-PDU”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### Not Permitted PDU

#### If the User Data parameter of a D-START indication or D-DATA indication is a valid PDU, but is not a PDU for which action is described within a given state  in 2.3.5.3, the CPDLC-air-ASE shall:

- Stop any timer,

- Create an AircraftPDUs APDU with a CPDLCProviderAbortReason [not-permitted-PDU] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “not-permitted-PDU” as the CPDLC-provider abort service Reason parameter value ,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”,  and

- Enter the IDLE state.

#### If the User Data parameter of a D-START confirmation is a valid PDU, but is not a permitted PDU as defined in 2.3.5.3, the CPDLC-air-ASE shall:

- If the D-START Result parameter is set to the abstract value “accepted”, then

  - Stop any timer,

  - Create an AircraftPDUs APDU with a CPDLCProviderAbortReason [not-permitted-PDU] APDU message element,

  - Invoke D-ABORT request with:

    - the abstract value “provider” as the D-ABORT Originator parameter value, and

    - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “not-permitted-PDU” as the CPDLC-provider-abort service Reason parameter value,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### D-START Confirmation Result or Reject Source Not as Expected

#### If a D-START confirmation Result parameter has the abstract value of “rejected (transient)” or if the Reject Source parameter has the abstract value of “DS provider”, the CPDLC-air-ASE shall:

- Stop any timer,

- If the CPDLC-air-user is an active user, invoke CPDLC-provider-abort service indication with the CPDLC-provider-abort service Reason parameter set to the abstract value “communication-service-error”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”,  and

- Enter the IDLE state.

## CPDLC-Ground-ASE Protocol Description

### Introduction

#### If no actions are described for a CPDLC service primitive when a CPDLC-ground-ASE is in specific state, then the invocation of that service primitive shall be prohibited while the CPDLC-ground-ASE is in that state.

#### Upon receipt of a PDU, if no actions are described for the arrival of that PDU when a CPDLC-ground-ASE is in a specific state, then that PDU is considered not permitted, and exception handling procedures as described in 2.3.5.6.4 shall apply.

#### If a PDU is received that cannot be decoded, then exception handling procedures as described in 2.3.5.6.3 for invalid PDU shall apply.

#### If a PDU is not received when one is required, then exception handling as described in 2.3.5.6.3 shall apply.

Note 1. — The states defined for the CPDLC-ground-ASE are the following.

- IDLE

- START-REQ,

- START-IND,

- DIALOGUE,

- END, and

- FORWARD.

Note 2. — The CPDLC-ground-user is an active user from:

- the time it has invoked the CPDLC-start service request until:

  - receipt of a CPDLC-start service confirmation with Result parameter equal to the abstract value “rejected”, or

  - receipt of a CPDLC-end service confirmation with the Result parameter equal to the abstract value “accepted”, or

  - invocation of a CPDLC-user-abort service request, or

  - receipt of a CPDLC-user-abort service indication, or

  - receipt of a CPDLC-provider-abort service indication; or

- the time it has received the CPDLC-start service indication until:

  - invocation of a CPDLC-start service response with Result parameter set to the abstract value “rejected”, or

  - receipt of a CPDLC-end service confirmation with the Result parameter equal to the abstract value “accepted”, or

  - invocation of a CPDLC-user-abort service request, or

  - receipt of CPDLC-user-abort service indication, or

  - receipt of a CPDLC-provider-abort service indication; or

- the time it has received the DSC-start service indication until:

  - invocation of a DSC-start service response with Result parameter equal to the abstract value “rejected”, or

  - invocation of a CPDLC-user-abort service request, or

  - receipt of CPDLC-user-abort service indication, or

  - receipt of a CPDLC-provider-abort service indication; or

- the time it has invoked the CPDLC-forward service request until:

  - receipt of a CPDLC-forward service confirmation,

  - invocation of a CPDLC-user-abort service request, or

  - receipt of CPDLC-user-abort service indication, or

  - receipt of a CPDLC-provider-abort service indication.

#### On initiation the CPDLC-ground-ASE shall be in the IDLE state.

- Note. — The CPDLC-ground-ASE contains a Boolean called DSC.  DSC has the abstract value “true” when the dialogue is a DSC dialogue, and has the abstract value “false” otherwise.

#### On the initiation of a CPDLC-ground-ASE, DSC shall be set to the abstract value “false”.

### D-START Indication

#### Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the abstract value of the D-START Calling Peer ID parameter is a 24 bit Aircraft Address and the D-START User Data parameter contains an AircraftPDUs [StartDownMessage] APDU with the APDU-element mode “cpdlc”, the CPDLC-ground-ASE shall:

- Invoke CPDLC-start service indication containing the following:

  - the D-START Calling Peer ID parameter value as the CPDLC-start service Calling Peer Identifier parameter value,

  - the D-START QOS Routing Class parameter value as the CPDLC-start service Class of Communication parameter value,

  - if the AircraftPDUs APDU-element contained in the D-START User Data parameter is an ATCDownlinkMessage, set the AircraftPDUs APDU-element as the CPDLC-start service CPDLC Message parameter value, and

- Enter the START-IND state.

#### Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the abstract value of the D-START Calling Peer ID parameter is a 24 bit Aircraft Address and the D-START User Data parameter contains an AircraftPDUs [StartDownMessage]APDU with the APDU-element mode “dsc”, the CPDLC-ground-ASE shall:

- Invoke DSC-start service indication containing the following:

  - the D-START Calling Peer ID parameter value as the DSC-start service Aircraft Identifier parameter value,

  - the D-START QOS Routing Class parameter value as the CPDLC-start service Class of Communication parameter value,

  - if the APDU AircraftPDUs APDU contained in the D-START is an ATCDownlinkMessage, set the AircraftPDUs APDU as the DSC-start service CPDLC Message parameter value,

- Set DSC to “true”, and

- Enter the START-IND state.

#### Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the abstract value of the D-START Calling Peer ID parameter is an 8 character ICAO Facility Designator and the D-START User Data parameter contains a GroundPDUs [ATCForwardMessage] APDU and the CPDLC-ground-ASE supports the CPDLC-forward service, the CPDLC-ground-ASE shall:

- If the D-START DS User Version Number parameter value is equal to the CPDLC-ground-ASE version number:

  - Invoke CPDLC-forward service indication containing the following:

    - the D-START Calling Peer ID parameter value as the CPDLC-forward service Calling ICAO Facility Designator parameter value,

    - set the D-START GroundPDUs APDU-element as the CPDLC-forward service CPDLC Message parameter value, and

  - Create a GroundPDUs APDU with an ATCForwardResponse [success] APDU element,

  - Invoke D-START response with the following:

    - the APDU as the D-START User Data parameter value, and

    - the abstract value “rejected (permanent)” as the D-START Result parameter value, and

  - Remain in the IDLE state.

- If the D-START DS User Version Number parameter value is not equal to the CPDLC-ground-ASE version number:

  - Create a GroundPDUs APDU with an ATCForwardResponse [version-not-equal] APDU element,

  - Invoke D-START response with the following:

    - the CPDLC-ground-ASE version number as the D-START DS User Version Number parameter value,

    - the APDU as the D-START User Data parameter value, and

    - the abstract value “rejected (permanent)” as the D-START Result parameter value, and

  - Remain in the IDLE state.

#### Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the abstract value of the D-START Calling Peer ID parameter is an 8 character ICAO Facility Designator and the D-START User Data parameter contains a GroundPDUs APDU and the APDU element is an ATCForwardMessage and the CPDLC-ground-ASE does not support the CPDLC-forward service, the CPDLC-ground-ASE shall:

- Create a GroundPDUs APDU with an ATCForwardResponse [service-not-supported] APDU element,

- Invoke D-START response with the following:

  - the APDU as the D-START User Data parameter value, and

  - the abstract value “rejected (permanent)” as the D-START Result parameter value, and

- Remain in the IDLE state.

### D-START Confirmation

#### Upon receipt of a D-START confirmation, if the CPDLC-ground-ASE is in the START-REQ state and if the D-START Result parameter has the abstract value “accepted”, and DSC has the abstract value of “false” and D-START User Data parameter is not provided, the CPDLC-ground-ASE shall:

- Stop timer tstart,

- Invoke CPDLC-start service confirmation containing the abstract value “accepted” as the CPDLC-start service Result parameter value, and

- Enter the DIALOGUE state.

#### Upon receipt of a D-START confirmation, if the CPDLC-ground-ASE is in the START-REQ state and the D-START Result parameter has the abstract value “rejected (permanent)” and the D-START Reject Source parameter has the abstract value “DS user” and DSC has the abstract value “false” and if the D-START User Data parameter is provided, the User Data parameter contains a AircraftPDUs [ATCDownlinkMessage] APDU, the CPDLC-ground-ASE shall:

- Stop timer tstart,

- Invoke CPDLC-start service confirmation containing the following:

  - if the D-START User Data parameter is provided, the APDU contained in the D-START User Data parameter as the CPDLC-start service Reject Reason parameter value, and

  - the abstract value “rejected” as the CPDLC-start service Result parameter value, and

- Enter the IDLE state.

#### Upon receipt of a D-START confirmation, if the CPDLC-ground-ASE is in the FORWARD state and if the D-START Result parameter has the abstract value “rejected (permanent)” and the Reject Source parameter has the abstract value “DS user” and the D-START User Data parameter contains a GroundPDUs [ATCForwardResponse] APDU, the CPDLC-ground-ASE shall:

- If the D-START DS User Version Number parameter value is equal to the CPDLC-ground-ASE version number:

  - Stop timer tstart,

  - Invoke CPDLC-forward service confirmation with the D-START GroundPDUs APDU-element as the CPDLC-forward service Result parameter value, and

  - Enter the IDLE state.

- If the D-START DS User Version Number parameter value is not equal to the CPDLC-ground-ASE version number:

  - Stop timer tstart,

  - Invoke CPDLC-forward service confirmation with the following:

    - the D-START GroundPDUs APDU-element as the CPDLC-forward service Result parameter value,

    - the D-START DS User Version Number parameter value as the CPDLC-forward service ASE Version Number parameter value, and

  - Enter the IDLE state.

### D-DATA Indication

#### Upon receipt of a D-DATA indication, if the CPDLC-ground-ASE is in the DIALOGUE state and the APDU contained in the D-DATA User Data parameter is a AircraftPDUs [ATCDownlinkMessage] APDU, the CPDLC-ground-ASE shall:

- Invoke CPDLC-message service indication with the APDU contained in the D-DATA User Data parameter as the CPDLC-message service CPDLC Message parameter value, and

- Remain in the DIALOGUE state.

#### Upon receipt of a D-DATA indication, if the CPDLC-ground-ASE is in the END state and DSC has the abstract value “false” and the APDU contained in the D-DATA User Data parameter is an AircraftPDUs [ATCDownlinkMessage] APDU, the CPDLC-ground-ASE shall:

- Invoke CPDLC-message service indication with the APDU contained in the D-DATA User Data parameter as the CPDLC-message service CPDLC Message parameter value, and

- Remain in the END state.

### D-END Indication

#### Upon receipt of a D-END indication, if the CPDLC-ground-ASE is in the DIALOGUE state, and DSC has the abstract value “true”, and if the D-END User Data parameter is provided, the User Data parameter contains an AircraftPDUs [ATCDownlinkMessage] APDU, the CPDLC-ground-ASE shall:

- Invoke DSC-end service indication with the APDU contained in the D-END User Data parameter as the DSC-end service CPDLC Message parameter value, if provided, and

- Enter the END state

### D-END Confirmation

#### Upon receipt of a D-END confirmation, if the CPDLC-ground-ASE is in the END state and the D-END Result parameter has the abstract value “accepted” and DSC has the abstract value “false” and if the D-END User Data parameter is provided, the User Data parameter contains an AircraftPDUs [ATCDownlinkMessage] APDU, the CPDLC-ground-ASE shall:

- Invoke CPDLC-end service confirmation with:

  - The APDU contained in the D-END User Data parameter as the CPDLC-end service CPDLC Message parameter value, if provided, and

  - The abstract value “accepted” as the CPDLC-end service Result parameter value, and

- Enter the IDLE state.

#### Upon receipt of a D-END confirmation, if the CPDLC-ground-ASE is in the END state and the D-END Result has the abstract value “rejected” and DSC has the abstract value “false”, and if the D-END User Data parameter is provided, the User Data parameter contains an AircraftPDUs [ATCDownlinkMessage] APDU, the CPDLC-ground-ASE shall:

- Invoke CPDLC-end service confirmation with:

  - The APDU contained in the D-END User Data parameter as the CPDLC-end service CPDLC Message parameter value, if provided, and

  - The abstract value “rejected” as the CPDLC-end service Result parameter value, and

- Enter the DIALOGUE state.

### CPDLC-start Service Request

#### Upon receipt of a CPDLC-start service request, if the CPDLC-ground-ASE is in the IDLE state, the CPDLC-ground-ASE shall:

- Create a GroundPDUs APDU with an UplinkMessage APDU element containing:

  - if provided, the CPDLC Message parameter as the UplinkMessage, or

  - else, NULL as the UplinkMessage,

- Invoke D-START request with the following:

  - the CPDLC-start service Called Peer Identifier parameter value as the D-START Called Peer ID parameter value,

  - the CPDLC-start service Calling Peer Identifier parameter value as the D-START Calling Peer ID parameter value,

  - the D-START Quality of Service parameters set as follows:

    - if provided, the CPDLC-start service Class of Communication parameter value as the D-START QOS Routing Class parameter value, or

    - The abstract value of “high priority flight safety messages”, as the D-START QOS Priority parameter value, and

    - The abstract value of “low” as the D-START QOS Residual Error Rate parameter value, and

  - the APDU as the D-START User Data parameter value;

- Start timer tstart, and

- Enter the START-REQ state.

### CPDLC-start Service Response

#### Upon receipt of a CPDLC-start service response, if the CPDLC-ground-ASE is in the START-IND state and the CPDLC-start service Result parameter has the abstract value “accepted” and the CPDLC-start service Reject Reason parameter is not provided, and DSC has the abstract value “false”, the CPDLC-ground-ASE shall:

- Invoke D-START response with the abstract value “accepted” as the D-START Result parameter value, and

- Enter the DIALOGUE state.

#### Upon receipt of a CPDLC-start service response, if the CPDLC-ground-ASE is in the START-IND state and the CPDLC-start service Result parameter has the abstract value “rejected” and DSC has the abstract value “false”, the CPDLC-ground-ASE shall:

- If the CPDLC-start service Reject Reason parameter is provided, create a GroundPDUs APDU with an ATCUplinkMessage APDU element based on the Reject Reason parameter,

- Invoke D-START response with the following:

  - The APDU as the D-START User Data parameter value; if the Reject Reason parameter was provided, and

  - the abstract value “rejected (permanent)”as the D-START Result parameter value, and

- Enter the IDLE state.

### DSC-start Service Response

#### Upon receipt of a DSC-start service response, if the CPDLC-ground-ASE is in the START-IND state and the DSC-start service Result parameter has the abstract value “accepted” and DSC has the abstract value “true”, the CPDLC-ground-ASE shall:

- Invoke D-START response with the abstract value “accepted” as the D-START Result parameter value, and

- Enter the DIALOGUE state.

#### Upon receipt of a DSC-start service response, if the CPDLC-ground-ASE is in the START-IND state and the DSC-start service Result parameter has the abstract value “rejected” and DSC has the abstract value “true”, the CPDLC-ground-ASE shall:

- If the DSC-start service Reject Reason parameter is provided, create a GroundPDUs APDU with an ATCUplinkMessage APDU element based on the Reject Reason parameter,

- Invoke D-START response with the following:

  - The APDU element as D-START User Data parameter value; if the Reject Reason parameter was provided, and

  - the abstract value “rejected (permanent)”as the D-START Result parameter value,

- Set DSC to the abstract value “false”, and

- Enter the IDLE state.

### CPDLC-message Service Request

#### Upon receipt of a CPDLC-message service request, if the CPDLC-ground-ASE is in the DIALOGUE state, the CPDLC-ground-ASE shall:

- Create a GroundPDUs APDU with an ATCUplinkMessage APDU-element based on the CPDLC-message service CPDLC Message parameter,

- Invoke D-DATA request with the APDU as the D-DATA User Data parameter value, and

- Remain in the DIALOGUE state.

#### Upon receipt of a CPDLC-message service request, if the CPDLC-ground-ASE is in the END state and DSC has the abstract value “true”, the CPDLC-ground-ASE shall:

- Create a GroundPDUs APDU with an ATCUplinkMessage APDU-element based on the CPDLC-message service CPDLC Message parameter,

- Invoke D-DATA request with the APDU as the D-DATA User Data parameter value, and

- Remain in the END state.

### CPDLC-end Service Request

#### Upon receipt of a CPDLC-end service request, if the CPDLC-ground-ASE is in the DIALOGUE state and DSC has the abstract value “false”, the CPDLC-ground-ASE shall:

- Create a GroundPDUs APDU with an ATCUplinkMessage APDU-element based on the CPDLC-end service CPDLC Message parameter, if provided,

- Invoke D-END request with the APDU as the D-END User Data parameter value, if provided and,

- Enter the END state.

### DSC-end Service Response

#### Upon receipt of a DSC-end service response, if the CPDLC-ground-ASE is in the END state and DSC has the abstract value “true”, and the DSC-end service Result parameter has the abstract value “accepted”, the CPDLC-ground-ASE shall:

- Create a GroundPDUs APDU with an ATCUplinkMessage APDU-element based on the DSC-end service CPDLC Message parameter , if provided,

- Invoke D-END response with the following:

  - the APDU as the D-END User Data parameter; if provided, and

  - the abstract value “accepted” as the D-END Result parameter value,

- Set DSC to the abstract value “false”, and

- Enter the IDLE state.

#### Upon receipt of a DSC-end service response, if the CPDLC-ground-ASE is in the END state and DSC has the abstract value “true”, and the DSC-end service Result parameter has the abstract value “rejected”, the CPDLC-ground-ASE shall:

- Create a GroundPDUs APDU with an ATCUplinkMessage APDU-element based on the DSC-end service CPDLC Message parameter , if provided,

- Invoke D-END response with the following:

  - the APDU as the D-END User Data parameter; if provided, and

  - the abstract value “rejected” as the D-END Result parameter value, and

- Enter the DIALOGUE state.

### CPDLC-forward Service Request

#### Upon receipt of a CPDLC-forward service request, if the CPDLC-ground-ASE is in the IDLE state, the CPDLC-ground-ASE shall:

- Create a GroundPDUs APDU with an ATCForwardMessage APDU element based on the CPDLC-forward service CPDLC Message parameter,

- Invoke D-START request with the following:

  - the CPDLC-forward service Called ICAO Facility Designator parameter value as the D-START Called Peer ID parameter value,

  - the CPDLC-start service Calling ICAO Facility Designator parameter value as the D-START Calling Peer ID parameter value,

  - the D-START Quality of Service parameters set as follows:

    - if provided, the CPDLC-start service Class of Communication parameter value as the D-START QOS Routing Class, or

    - The abstract value of “high priority flight safety messages”, as the D-START QOS Priority parameter value, and

    - The abstract value of “low” as the D-START QOS Residual Error Rate parameter value, and

  - the APDU as the D-START User Data parameter value;

- Start timer tstart, and

- Enter the FORWARD state.

### CPDLC-user-abort Service Request

#### Upon receipt of a CPDLC-user-abort service request, if the CPDLC-ground-ASE is not in the IDLE state, the CPDLC-ground-ASE shall:

- Stop any timer,

- If the CPDLC-user-abort service Reason parameter is provided, create a GroundPDUs APDU with a CPDLCUserAbortReason APDU-element based on the CPDLC-user-abort service Reason parameter,

- Else create a GroundPDUs APDU with a CPDLCUserAbortReason [undefined] APDU-element

- Invoke D-ABORT request with the following:

  - the D-ABORT Originator parameter set to the abstract value “user”, and

  - the APDU as the D-ABORT User Data parameter value, and

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### D-ABORT Indication

#### Upon receipt of a D-ABORT indication, if the CPDLC-ground-ASE is not in the IDLE state and the D-ABORT Originator parameter is “user” and the D-ABORT User Data parameter contains an AircraftPDUs [CPDLCUserAbortReason] APDU, the CPDLC-ground-ASE shall:

- Stop any timer,

- If the CPDLC-ground-user is an active user, invoke CPDLC-user-abort service indication with the APDU contained in the D-ABORT User Data parameter as the CPDLC-user-abort service Reason parameter value,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

#### Upon receipt of a D-ABORT indication, if the CPDLC-ground-ASE is not in the IDLE state , and if the D-ABORT Originator parameter is “provider” and if the D-ABORT User Data parameter is provided, the D-ABORT User Data parameter contains either an AircraftPDUs [CPDLCProviderAbortReason] APDU or a GroundPDUs [CPDLCProviderAbortReason] APDU , the CPDLC-ground-ASE shall:

- Stop any timer,

- If the CPDLC-ground-user is an active user, invoke CPDLC-provider-abort service indication with the D-ABORT User Data parameter as the CPDLC-provider-abort service Reason parameter value, if provided,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### D-P-ABORT Indication

#### Upon receipt of a D-P-ABORT indication, if the CPDLC-ground-ASE is not in the IDLE state, the CPDLC-ground-ASE shall:

- Stop any timer,

- If the CPDLC-ground-user is an active user, invoke CPDLC-provider-abort service indication with the CPDLC-provider-abort service Reason parameter set to the abstract value “communication-service-failure”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

## CPDLC-Ground-ASE Exception Handling

### A Timer Expires

#### If a CPDLC-ground-ASE detects that a timer has expired, that CPDLC-ground-ASE shall:

- Interrupt any current activity,

- Create a GroundPDUs APDU with a CPDLCProviderAbortReason [timer-expired] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-ground-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “timer-expired” as the CPDLC-provider abort service Reason parameter value”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”,  and

- Enter the IDLE state.

### Unrecoverable System Error

#### If a CPDLC-ground-ASE has an unrecoverable system error, the CPDLC-ground-ASE shall:

- Stop any timer,

- Create a GroundPDUs APDU with a CPDLCProviderAbortReason [undefined-error] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-ground-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “undefined-error” as the CPDLC-provider abort service Reason parameter value”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”,  and

- Enter the IDLE state.

### Invalid PDU

#### If the User Data parameter of a D-END confirmation with Result parameter set to the abstract value “rejected”, a D-START indication, a D-DATA indication, or a D-END indication, does not contain a valid PDU, the CPDLC-ground-ASE shall:

- Stop any timer,

- Create a GroundPDUs APDU with a CPDLCProviderAbortReason [invalid-PDU] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-ground-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “invalid-PDU” as the CPDLC-provider abort service Reason parameter value,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

#### If the User Data parameter of a D-START confirmation with Result set to the abstract value “rejected (permanent)”, or a D-END confirmation with Result set to the abstract value “accepted”, is not a valid PDU the CPDLC-ground-ASE shall:

- Stop any timer,

- If the CPDLC-ground-user is an active user, invoke CPDLC-provider-abort service indication with the CPDLC-provider-abort service Reason parameter set to the abstract value “invalid-PDU”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”,  and

- Enter the IDLE state.

### Not Permitted PDU

#### If the User Data parameter of a D-START indication or D-DATA indication is a valid PDU, but is not a permitted PDU for which action is described within a given state as defined in 2.3.5.5, the CPDLC-ground-ASE shall:

- Stop any timer,

- Create a GroundPDUs APDU with a CPDLCProviderAbortReason [not-permitted-PDU] APDU message element,

- Invoke D-ABORT request with:

  - the abstract value “provider” as the D-ABORT Originator parameter value, and

  - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-ground-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “not-permitted-PDU” as the CPDLC-provider abort service Reason parameter value”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”,  and

- Enter the IDLE state.

#### If the User Data parameter of a D-START confirmation is a valid PDU, but is not a permitted PDU for which action is described within a given state as defined in 2.3.5.5, the CPDLC-ground-ASE shall:

- Stop any timer,

- If the D-START Result parameter is set to the abstract value “accepted”, then

  - Create a GroundPDUs APDU with a CPDLCProviderAbortReason [not-permitted-PDU] APDU message element,

  - Invoke D-ABORT request with:

    - the abstract value “provider” as the D-ABORT Originator parameter value, and

    - the APDU as the D-ABORT User Data parameter value, and

- If the CPDLC-user is an active user, invoke CPDLC-provider-abort service indication with the abstract value “not-permitted-PDU” as the CPDLC-provider-abort service Reason parameter value,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

### D-START Confirmation Result or Reject Source Not as Expected

#### If a D-START confirmation Result parameter has the abstract value of “rejected (transient)” or if the Reject Source parameter has the abstract value of “DS provider”, the CPDLC-ground-ASE shall:

- Stop any timer,

- if the CPDLC-ground-user is an active user, invoke CPDLC-provider-abort service indication with the CPDLC-provider-abort service Reason parameter set to the abstract value “communication-service-error”,

- If DSC has the abstract value “true”, set DSC to the abstract value “false”, and

- Enter the IDLE state.

## CPDLC ASE State Tables

### Priority

#### If the state tables shown for the CPDLC-air-ASE and the CPDLC-ground-ASE shown below conflict with textual statements made elsewhere in this document, the textual statements shall take precedence.

Note 1.— In the following state tables, the statement “cannot occur” means that if the implementation conforms to the SARPs, it is impossible for this event to occur. If the event does occur, this implies that there is an error in the implementation. If such a situation is detected, it is suggested that the ASE aborts with the error “unrecoverable system error”.

Note 2.— In the following state tables, the statement “not permitted” means that the implementation must prevent this event from occurring through some local means. If the event does occur this implies that there is an error in the implementation. If such a situation is detected, it is suggested that the ASE performs a local rejection of the request rather than aborting the dialogue.

Table 2.3.5-2.  CPDLC-Air-ASE State Table

| STATE Þ EVENT ß | IDLE |  | START-REQ |  | START-IND |  | DIALOGUE |  | END |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dialogue Service Events |  |  |  |  |  |  |  |  |  |
| D-START Indication APDU = UplinkMessage | - CPDLC-start indication - START-IND |  | cannot occur |  | cannot occur |  | cannot occur |  | cannot occur |
| D-START Confirmation Result “accepted”, DSC = “false” No User Data | cannot occur |  | - Stop timer tstart - CPDLC-start confirmation - DIALOGUE |  | cannot occur |  | cannot occur |  | cannot occur |
| D-START Confirmation Result “rejected (permanent)” and Reject Source “DS user”, DSC=“false” if User Data, APDU = ATCUplinkMessage | cannot occur |  | - Stop timer tstart - CPDLC-start confirmation - IDLE |  | cannot occur |  | cannot occur |  | cannot occur |
| D-START Confirmation Result “accepted”, DSC = “true” No User Data | cannot occur | - Stop timer tstart - DSC-start confirmation, - DIALOGUE |  | cannot occur |  | cannot occur |  | cannot occur |  |
| D-START Confirmation Result “rejected (permanent)” and Reject Source “DS user”, DSC= “true” if User Data, APDU = ATCUplinkMessage | cannot occur |  | - Stop timer tstart - DSC-start confirmation - Set DSC = “false” - IDLE |  | cannot occur |  | cannot occur |  | cannot occur |
| D-DATA Indication APDU = ATCUplinkMessage | cannot occur |  | cannot occur |  | cannot occur |  | - CPDLC-message indication - DIALOGUE |  | - if DSC=“true” - CPDLC-message indication - END - else not permitted |
| - D-END Indication: DSC=“false" if User Data, APDU = ATCUplinkMessage | cannot occur |  | cannot occur |  | cannot occur |  | - CPDLC-end indication - END |  | cannot occur |
| D-END Confirmation: DSC=“true” Result “accepted” if User Data, APDU = ATCUplinkMessage | cannot occur |  | cannot occur |  | cannot occur |  | cannot occur |  | - DSC-end confirmation - Set DSC “false” - IDLE |
| D-END Confirmation: DSC=“true”, Result “rejected” if User Data, APDU = ATCUplinkMessage | cannot occur |  | cannot occur |  | cannot occur |  | cannot occur |  | - DSC-end confirmation - DIALOGUE |
| CPDLC-User Events |  |  |  |  |  |  |  |  |  |
| CPDLC-start Request | - D-START request - Start timer tstart - START-REQ |  | not permitted |  | not permitted |  | not permitted |  | not permitted |
| CPDLC-start Response DSC=“false” Result “accepted” | not permitted |  | not permitted |  | - D-START response - DIALOGUE |  | not permitted |  | not permitted |
| CPDLC-start Response DSC=“false” Result “rejected” | not permitted |  | not permitted |  | - D-START response - IDLE |  | not permitted |  | not permitted |
| DSC-start Request | - D-START request - set DSC =“true” - Start timer tstart - START-REQ |  | not permitted |  | not permitted |  | not permitted |  | not permitted |
| CPDLC-message Request | not permitted |  | not permitted |  | not permitted |  | - D-DATA request - DIALOGUE |  | - if DSC=“false” - D-DATA request - END - else not permitted |
| - CPDLC-end Service Response DSC = “false” Result “accepted” | cannot occur |  | cannot occur |  | cannot occur |  | not permitted |  | - D-END response - IDLE |
| CPDLC-end Service Response DSC = “false” Result “rejected” | cannot occur |  | cannot occur |  | cannot occur |  | not permitted |  | - D-END response - DIALOGUE |
| DSC-end Request: DSC = “true” | not permitted |  | not permitted |  | not permitted |  | - D-END request - END |  | not permitted |
| ABORT Events |  |  |  |  |  |  |  |  |  |
| CPDLC-user-abort Request | not permitted |  | - Stop timer tstart, if set - D-ABORT request - If DSC = “true”, set DSC = “false” - IDLE |  | - Stop timer tstart, if set - D-ABORT request - If DSC = “true”, set DSC = “false” - IDLE |  | - D-ABORT request - If DSC = “true”, set DSC = “false” - IDLE |  | - D-ABORT request - If DSC = “true”, set DSC = “false” - IDLE |
| D-ABORT Indication Originator “user” | cannot occur |  | - Stop timer Tstart, if set - If active user: CPDLC-user-abort indication - If DSC = “true”, set DSC = “false” - IDLE |  | - Stop timer Tstart, if set - If active user: CPDLC-user-abort indication - If DSC = “true”, set DSC = “false” - IDLE |  | - If active user: CPDLC-user-abort indication - If DSC = “true”, set DSC = “false” - IDLE |  | - If active user: CPDLC-user-abort indication - If DSC = “true”, set DSC = “false” - IDLE |
| D-ABORT Indication Originator “provider” | cannot occur |  | - Stop timer Tstart, if set - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC “false” - IDLE |  | - Stop timer Tstart, if set - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC “false” - IDLE |  | - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC “false” - IDLE |  | - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC “false” - IDLE |
| D-P-ABORT indication | cannot occur |  | - Stop timer tstart, if set - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE |  | - Stop timer tstart, if set - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE |  | - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE |  | - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE |
| Tstart Expires | cannot occur |  | - D-ABORT request - CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE |  | cannot occur |  | cannot occur |  | cannot occur |

Table 2.3.5-3.  CPDLC-Ground-ASE State Table

| STATE Þ EVENT ß | IDLE | START-REQ | START-IND | DIALOGUE | END | FORWARD |
| --- | --- | --- | --- | --- | --- | --- |
| Dialogue Service Events |  |  |  |  |  |  |
| D-START Indication APDU Aircraft mode “cpdlc” | - CPDLC-start indication - START-IND | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Indication APDU Aircraft mode “dsc” | - DSC-start indication, - Set DSC =“true” - START-IND | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Indication APDU Forward version equal and function supported | - CPDLC-forward indication - D-START response - IDLE | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Indication APDU Forward version not equal or function not supported | - D-START response - IDLE | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Confirmation Result “accepted” DSC = “false” | cannot occur | - Stop timer tstart - CPDLC-start confirmation - DIALOGUE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Confirmation Result “rejected (permanent)” and Reject Source “DS user” DSC=“false” | cannot occur | - Stop timer tstart - CPDLC-start confirmation - IDLE | cannot occur | cannot occur | cannot occur | - Stop timer tstart - CPDLC-forward confirmation - IDLE |
| D-START Confirmation Result “accepted” DSC =“true” | cannot occur | - Stop timer tstart - DSC-start confirmation, - DIALOGUE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START Confirmation Result “rejected (permanent)” and Reject Source “DS user” DSC = “true” | cannot occur | - Stop timer tstart - DSC-start confirmation, - Set DSC = “false” - IDLE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-DATA Indication | cannot occur | cannot occur | cannot occur | - CPDLC-message indication - DIALOGUE | - if DSC=“false” - CPDLC-message indication - END - else not permitted | cannot occur |
| D-END Indication: DSC=“true” | cannot occur | cannot occur | cannot occur | - DSC-end indication - END | cannot occur | cannot occur |
| D-END Confirmation: DSC = “false” Result “accepted” | cannot occur | cannot occur | cannot occur | cannot occur | - CPDLC-end confirmation - IDLE | cannot occur |
| D-END Confirmation: DSC = “false” Result “rejected” | cannot occur | cannot occur | cannot occur | cannot occur | - CPDLC-end confirmation - DIALOGUE | cannot occur |
| CPDLC-User Events |  |  |  |  |  |  |
| CPDLC-start Request | - D-START request - Start timer tstart - START-REQ | not permitted | not permitted | not permitted | not permitted | not permitted |
| CPDLC-start Response DSC = “false” Result “accepted” | not permitted | not permitted | - D-START response - DIALOGUE | not permitted | not permitted | not permitted |
| CPDLC-start Response DSC = “false” Result “rejected” | not permitted | not permitted | - D-START response - IDLE | not permitted | not permitted | not permitted |
| DSC-start Response DSC = “true” Result “accepted” | not permitted | not permitted | - D-START response - DIALOGUE | not permitted | not permitted | not permitted |
| DSC-start Response DSC = “true” Result “rejected” | not permitted | not permitted | - D-START response - Set DSC= “false” - IDLE | not permitted | not permitted | not permitted |
| CPDLC-message Request | not permitted | not permitted | not permitted | - D-DATA request - DIALOGUE | - if DSC=“true” - D-DATA request - END - else not permitted | not permitted |
| CPDLC-end Request: DSC = “false” | not permitted | not permitted | not permitted | - D-END request - END | not permitted | not permitted |
| DSC-end Service Response DSC = “true” Result “accepted” | cannot occur | cannot occur | cannot occur | not permitted | - D-END response - Set DSC = “false” - IDLE | not permitted |
| DSC-end Service Response DSC = “true” Result “rejected” | cannot occur | cannot occur | cannot occur | not permitted | - D-END response - DIALOGUE | not permitted |
| CPDLC-forward Request | - D-START request - Start timer tstart - FORWARD | not permitted | not permitted | not permitted | not permitted | not permitted |
| ABORT Events |  |  |  |  |  |  |
| CPDLC-user-abort Request | not permitted | - Stop timer tstart, if set - D-ABORT request - If DSC = “true”, set DSC = “false” - IDLE | - Stop timer tstart, if set - D-ABORT request - If DSC = “true”, set DSC = “false” - IDLE | - D-ABORT request - If DSC = “true”, set DSC = “false” - IDLE | - D-ABORT request - IDLE - If DSC = “true”, set DSC = “false” | - Stop timer tstart; if set - D-ABORT request - IDLE |
| D-ABORT Indication Originator “provider” | cannot occur | - Stop timer Tstart, if set - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - Stop timer Tstart, if set - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - Stop timer Tstart, if set - If active user: CPDLC-provider-abort indication - IDLE |
| D-ABORT Indication Originator “user” | cannot occur | - Stop timer Tstart, if set - If active user: CPDLC-user-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - Stop timer Tstart, if set - If active user: CPDLC-user-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - If active user: CPDLC-user-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - If active user: CPDLC-user-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - Stop timer Tstart, if set - If active user: CPDLC-user-abort indication - IDLE |
| D-P-ABORT indication | cannot occur | - Stop timer tstart, if set - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - Stop timer tstart, if set - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - If active user: CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | - Stop timer tstart, if set - If active user: CPDLC-provider-abort indication - IDLE |
| Tstart Expires | cannot occur | - D-ABORT request - CPDLC-provider-abort indication - If DSC = “true”, set DSC = “false” - IDLE | cannot occur | cannot occur | cannot occur | - D-ABORT request - CPDLC-provider-abort indication - IDLE |

# COMMUNICATION REQUIREMENTS

## Encoding Rules

### The CPDLC application shall use PER as defined in ISO/IEC 8825-2, using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in 2.3.4.

## Dialogue Service Requirements

### Primitive Requirements

#### Where dialogue service primitives, that is D-START, D-DATA, D-END, D-ABORT, and D-P-ABORT are described as being invoked in 2.3.5, the CPDLC-ground-ASE and the CPDLC-air-ASE shall exhibit external behavior consistent with the dialogue service, as described in 4.2 having been implemented and its primitives invoked.

### Quality-of-Service Requirements

#### The application service priority for CPDLC shall have the abstract value of “normal priority flight safety messages”.

#### The RER Quality of Service Parameter of the D-START shall be set to the abstract value of “low”.

#### The CPDLC-ASE shall map the CPDLC-start service or DSC-start service Class of Communication parameter abstract value to the ATSC routing class abstract value part of the D-START QOS parameter as presented in Table 2.3.6-1.

Table 2.3.6-1.  Mapping Between Class of Communication and Routing Class Abstract Values

| Class of Communication Abstract Value | Routing Class Abstract Value |
| --- | --- |
| A | Traffic follows Class A ATSC route(s) |
| B | Traffic follows Class B ATSC route(s) |
| C | Traffic follows Class C ATSC route(s) |
| D | Traffic follows Class D ATSC route(s) |
| E | Traffic follows Class E ATSC route(s) |
| F | Traffic follows Class F ATSC route(s) |
| G | Traffic follows Class G ATSC route(s) |
| H | Traffic follows Class H ATSC route(s) |

- Note. — ATSC values are defined in 1.2.

# CPDLC USER REQUIREMENTS

## General

- Note 1. — Requirements imposed on CPDLC-user concerning CPDLC messages and interfacing with the CPDLC-ASEs are presented in this chapter.

- Note 2. — Where reference is made to the “CPDLC-user”, this implies both the CPDLC-air-user and the CPDLC-ground-user.

- Note 3. — A CPDLC message is:

- what the CPDLC-user provides the CPDLC-service as the CPDLC Message or Reject Reason parameter, when invoking a CPDLC service request or response primitive, or

- what the CPDLC-user receives in the same parameters from the CPDLC service indication or confirmation primitives.

- Note 4. —The terms CPDLC message, message, uplink message and downlink message are used interchangeably, and equate to a CPDLC message.  When the terms “send” and “transmit” are used this means that the CPDLC-user has invoked a CPDLC service request or response primitive.  When the term “receive” is used this means that a CPDLC indication or confirmation primitive parameter containing a CPDLC message has been provided by the CPDLC service.

### General CPDLC Service Requirements

#### A CPDLC-ground-user shall invoke CPDLC-start service, DSC-start service, CPDLC-message service, CPDLC-end service, and DSC-end service only when communicating with a CPDLC-air-user.

#### A CPDLC-ground-user shall invoke CPDLC-forward service, only when communicating with another CPDLC-ground-user.

- Note. 1. — When a CPDLC-user invokes the CPDLC-start service, the DSC-start service, or the CPDLC-forward service and requires a particular class of communication service, the CPDLC-user sets the Class of Communication Service parameter.

- Note 2. — When a CPDLC-user does not require a particular class of communication, the user does not set the Class of Communication Service parameter.

## CPDLC Message Generation Requirements

- Note 1. — A response message is a message which is a reply to a received message.  It contains a message reference number identical to the message identification number of the message to which it refers.  Only response messages contain a message reference number.

- Note 2. — Message response attributes dictate a) if a response is required or prohibited; b) if a response is required, dictate the permitted response messages.

Note 3. — A closure response message is a reply to a message or series of messages which terminates a sequence of message exchanges.  However due to the multiple element capability of a CPDLC message, a closure message may contain message element(s) in addition to the required closure message element that initiate a new sequence of messages.

### Message Composition

#### A CPDLC message shall be composed of a message header, and from one to five message elements.

#### For air/ground messages, the message header shall be composed of a message identification number, a message reference number, if required, a time stamp, and a logical acknowledgment requirement (optional).

#### For ground/ground messages, the message header shall be composed of a time stamp, the aircraft flight identification, and the airframe identification to which the message refers.

#### A message element shall consist of a message element identifier, data as indicated by the specified message element, and associated message element attributes.

#### For each CPDLC message the CPDLC-user sends air/ground it shall provide the following information:

- a message identification number,

- a message reference number only if the message is a response message,

- date and time,

- a logical acknowledgment indication, if required,

- from one to five message element identifiers, and

- data as required for each message element identification included.

#### For each CPDLC message the CPDLC-user sends ground/ground it shall provide the following information:

- the aircraft identification to which the ground/ground message refers,

- date and time,

- from one to five message element identifiers, and

- data as required for each message element identification included.

### Message Identification Number

- Note 1. — A message identification number pertains to a single peer to peer dialogue.

- Note 2. — Message identification numbers used by a CPDLC ground system for uplink messages to an aircraft have no relationship to the message identification numbers used by the same ground system another aircraft.

- Note 3. — Similarly, message identification numbers used by a CPDLC aircraft for downlink messages to a CPDLC ground system have no relationship to the message identification numbers used by the same aircraft with another ground system.

- Note 4. — There is no relationship between message identification numbers assigned and managed by a the CPDLC ground system and those message identification numbers assigned and managed by the aircraft.

#### The message identification number provided by the CPDLC-user shall be different from any other message identification number currently in use.

#### A message identification number shall be deemed currently in use until:

- if the message does not allow a response, the message is sent, or

- if the message requires a response, the closure response is received.

#### If a CPDLC or DSC dialogue is terminated, all message identification numbers pertaining to that dialogue shall be considered available.

### Message Elements That Cannot Be Combined With Other Message Elements in a Message

#### The ERROR [error information] message element (uplink message element 159, and downlink message 62) shall be sent only as a single message element CPDLC message.

#### The LOGICAL ACKNOWLEDGMENT message element (uplink message element 227 and downlink message element 100) shall be sent only as a single message element CPDLC message.

#### The NEXT DATA AUTHORITY [facility designation] message element (uplink message element 160) shall be sent only as a single message element CPDLC message.

## CPDLC Message Receipt Requirements

### Message Attributes

- Note 1. — Message attributes dictate certain message handling requirements for the CPDLC-user receiving a message.  Each CPDLC message has Urgency, Alert, and Response attributes.

- Note 2. — Message element attribute table entries are listed in order of precedence (i.e., a precedence value of 1 is highest followed by 2, etc.).

#### When a message contains a single message element, the message attributes shall be the message element attributes.

#### When a message contains multiple message elements, the highest precedence message element attribute for each attribute type associated with any element in the message shall be the message attribute for each attribute type for the entire message.

### Urgency Requirements

- Note. — The Urgency (URG) attribute delineates the queuing requirements for received messages that are displayed to the end-user.  The same Urgency attribute types are used for both air/ground and ground/ground messages.

#### Each message element shall have associated Urgency attributes with precedence as defined in table 2.3.7-1.

Table 2.3.7-1.  Urgency Attribute (Air/Ground and Ground/Ground)

| Type | Description | Precedence |
| --- | --- | --- |
| D | Distress | 1 |
| U | Urgent | 2 |
| N | Normal | 3 |
| L | Low | 4 |

#### When a CPDLC-user queues received messages, messages with the highest Urgency type shall be placed at the beginning of the queue.

#### When a CPDLC-user queues received messages, messages with the same Urgency type shall be queued in order of receipt.

### Alerting Requirements

- Note. — The alert (ALRT) attribute delineates the type of end-user alerting required by the CPDLC-user upon message receipt.  The same Alert attribute types are used for both air/ground and ground/ground messages.

#### Each message element shall have associated Alert attributes with precedence as defined in table 2.3.7-2.

Table 2.3.7-2.  Alert Attribute (Air/Ground and Ground/Ground)

| Type | Description | Precedence |
| --- | --- | --- |
| H | High | 1 |
| M | Medium | 2 |
| L | Low | 3 |
| N | No alerting required | 4 |

#### Upon receipt of a CPDLC message, the CPDLC-user shall provide one of three distinct alerts as determined by the received message alert attribute.

### Response Attribute

- Note. — The response (RESP) attribute mandates CPDLC-user response requirements for a given message element. Response message attribute only apply to air/ground messages.

#### Each uplink message element shall have associated Response attributes with precedence as defined in table 2.3.7-3.

Table 2.3.7-3.  Response Attribute (Up-Link)

| Type | Response Required | Valid Responses Description | Precedence |
| --- | --- | --- | --- |
| W/U | Yes | Response required: WILCO, UNABLE, STANDBY permitted, LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 1 |
| A/N | Yes | Response required: AFFIRM, NEGATIVE, STANDBY permitted, LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 2 |
| R | Yes | Response required: ROGER, UNABLE, STANDBY permitted LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary) | 3 |
| Y | Yes | Any CPDLC downlink message, LOGICAL ACKNOWLEDGMENT (only if required), | 4 |
| N | No, unless logical acknowledgment required | LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary, only when logical acknowledgment is required) | 5 |

#### Each downlink message element shall have associated Response attributes with precedence as defined in table 2.3.7-4.

- Table 2.3.7-4.  Response Attribute (Down-Link)

| Type | Response Required | Valid Responses Description | Precedence |
| --- | --- | --- | --- |
| Y | Yes | Any CPDLC uplink message, LOGICAL ACKNOWLEDGMENT (only if required) | 1 |
| N | No, unless logical acknowledgment required | LOGICAL ACKNOWLEDGMENT (only if required), ERROR (if necessary, only when logical acknowledgment is required) | 2 |

### CPDLC/DSC Distinction

#### Upon receipt of a CPDLC message the CPDLC-user shall provide a distinction between CPDLC messages received from the Current Data Authority and those received from a Downstream Data Authority.

### Air/Ground - Ground/Ground Distinction

#### Upon receipt of a CPDLC message the CPDLC-user shall provide a distinction between CPDLC messages received from an aircraft and those received from another ground system.

### Logical Acknowledgment Prohibited

#### Upon receipt of the CPDLC message USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED the CPDLC-air-user shall be prohibited from requiring a logical acknowledgment for any message sent for the duration of the CPDLC or DSC dialogue.

#### If the CPDLC-ground-user receives a CPDLC message requiring a logical acknowledgment where the use of logical acknowledgment has been prohibited as above, the CPDLC-ground-user shall invoke the CPDLC-message service with a message containing the ERROR [errorinformation] message element with the [logicalAcknowledgmentNotAccepted]value as the CPDLC message parameter and discard the content of the received message.

### Message Reference Numbers

#### If a received message requires a response, the CPDLC-user shall provide a message reference number for each response message sent.

#### The message reference number shall be identical to the message identification number of the received message to which it refers.

### Message Response Requirements

#### Recommendation. — A message sequence initiated by data link should be closed by data link.

#### Recommendation. — If a message sequence exchange initiated by data link is subsequently closed by voice, local procedures should be in place to ensure deletion of outstanding data link messages requiring closure.

#### A CPDLC-user shall only be permitted to respond to a received message in its entirety.

#### Only one closure response shall be permitted for a given message.

#### If the CPDLC-air user has not issued a DSC-end service request primitive, or if the CPDLC-air-user has issued a DSC-end service request primitive for which a DSC-end service confirmation primitive has been received with the Result parameter containing the abstract value “rejected” then, if a message is received that requires a response, the CPDLC-air-user shall either:

- send any permitted response messages and then send a closure response message, or

- send a closure response message.

#### If the CPDLC-ground-user has not issued a CPDLC-end service request primitive, or if the CPDLC-ground-user has issued a CPDLC-end service request primitive for which a CPDLC-end service confirmation primitive has been received with the Result parameter containing the abstract value “rejected” then, if a message is received that requires a response, the CPDLC-air-user shall either:

- send any permitted response messages and then send a closure response message, or

- send a closure response message.

#### For a given message, once the CPDLC-user has sent the closure response message, no other response messages shall be sent referring to the given message.

#### When a message is received by the CPDLC-user requiring a logical acknowledgment response, the CPDLC-user shall respond with either a CPDLC message containing a LOGICAL ACKNOWLEDGMENT message element, or a message containing an ERROR message element.

#### A logical acknowledgment response message, if required, shall be sent prior to sending any other related response message(s), except a response message containing an ERROR message element.

#### When the CPDLC-air-user receives a message with a W/U RESP attribute, the only permitted responses shall be messages that contain a LOGICAL ACKNOWLEDGMENT (if required), STANDBY, WILCO, UNABLE, or ERROR message element.

#### When the CPDLC-air-user receives a message with a W/U RESP attribute, the closure response message shall contain at least a WILCO, UNABLE, or ERROR message element.

#### When the CPDLC-air-user receives a message with a A/N RESP attribute, the only permitted responses shall be messages that contain a LOGICAL ACKNOWLEDGMENT (if required), STANDBY, AFFIRM, NEGATIVE, or ERROR message element.

#### When the CPDLC-air-user receives a message with a A/N RESP attribute, the closure response message shall contain at least a AFFIRM, NEGATIVE, or ERROR message element.

#### When the CPDLC-air-user receives a message with a R RESP attribute, the only permitted responses shall be messages that contain a LOGICAL ACKNOWLEDGMENT (if required), STANDBY, ROGER, or ERROR message element.

#### When the CPDLC-air-user receives a message with a R RESP attribute, the closure response message shall contain at least a ROGER or ERROR message element.

#### When the CPDLC-air-user receives a message with a Y RESP attribute, a LOGICAL ACKNOWLEDGMENT only when requested, and all other CPDLC messages shall be permitted as a response message.

#### When the CPDLC-air-user receives a message with a Y RESP attribute, the first response message sent that does not contain a STANDBY or LOGICAL ACKNOWLEDGMENT shall constitute the closure response message.

#### When the CPDLC-ground-user receives an air/ground message with a Y RESP attribute, a LOGICAL ACKNOWLEDGMENT, only when requested and all other CPDLC messages shall be permitted as a response message.

#### When the CPDLC-ground-user receives an air/ground message with a Y RESP attribute, the first response message sent that does not contain a STANDBY, REQUEST DEFERRED, or LOGICAL ACKNOWLEDGMENT message element shall constitute the closure response message.

#### When the CPDLC-air-user receives a message with a N RESP attribute, but requiring a logical acknowledgment, the only permitted response shall be a message that contains a LOGICAL ACKNOWLEDGMENT or ERROR message element.  This response message is the closure message.

#### When the CPDLC-ground-user receives an air/ground message with a N RESP attribute, but requiring a logical acknowledgment the only permitted response shall be a message that contains a LOGICAL ACKNOWLEDGMENT or ERROR message element. This response message is the closure message.

#### When the CPDLC-ground-user receives a ground/ground message the ground-user shall be prohibited from generating a ground/ground response message.

Note. — Ground/ground forwarding of messages is a one-way exchange on a one message per dialogue basis.  There are no message identification or message reference numbers contained in the header of a ground/ground message.

### Error Conditions

#### Duplicate Message Identification Numbers

##### If a CPDLC message is received containing an identification number identical to that of an identification number currently in use the CPDLC-user shall invoke the CPDLC-user-abort request service with a CPDLC message containing the CPDLCUserAbortReason with value [duplicate-message-identification-number] as the Reason parameter.

#### Invalid Reference Number

##### If the CPDLC-user receives a message containing a message reference number which is not identical to any message identification number currently in use, the CPDLC-user shall:

- invoke CPDLC-message request with the ERROR [errorinformation] message element with the [unrecognizedMsgReferenceNumber] value as the CPDLC Message parameter, and

- disregard the received message.

#### No Available Message Identification Numbers

##### If the CPDLC-user attempts to send a CPDLC message and all message identification numbers are currently in use, the CPDLC-user shall invoke the CPDLC-user-abort request with a CPDLC message containing the CPDLCUserAbortReason with the value [no-message-identification-numbers-available] as the Reason parameter.

#### Insufficient Resources

##### If the CPDLC-user receives a message and has insufficient resources to handle the message, the CPDLC-user shall:

- invoke CPDLC-message request with the ERROR [errorinformation] message element with the [insufficientResources] value as the CPDLC Message parameter, and

- disregard the received message.

#### ERROR Message Response

##### If the CPDLC-user sends a message containing the ERROR message element instead of the expected response message, the ERROR message shall contain the received message identification number as the message reference number.

##### If an ERROR message is sent in response to a CPDLC message the received CPDLC message shall be disregarded.

## CPDLC-air-user Requirements

### The CPDLC-start Service

#### Invoking the CPDLC-start request

##### If there is no CPDLC service, the only CPDLC service primitives the CPDLC-air-user shall be permitted to invoke are the CPDLC-start request or the DSC-start request.

##### The CPDLC-air-user shall only be permitted to invoke the CPDLC-start request with:

- any ground system, if there is no existing CPDLC service for the CPDLC-air-user, or

- the Next Data Authority, if the CPDLC-air-user has received a message from the Current Data Authority designating a Next Data Authority.

##### If a CPDLC-air-user has invoked a CPDLC-start request, the CPDLC-air-user shall be prohibited from invoking any CPDLC-service primitive pertaining to the ground system addressed in the CPDLC-start service, except the CPDLC-user-abort request, until after it has received a CPDLC-start confirmation.

#### Receipt of a CPDLC-start Indication and Invoking CPDLC-start Response

##### Upon receipt of a CPDLC-start service indication, the CPDLC-user shall invoke a CPDLC-start service response within 0.5 seconds.

##### Upon receipt of a CPDLC-start indication the CPDLC-air-user shall invoke the CPDLC-start response, with the response parameters set as follows:

- The Result parameter to the abstract value of “accepted” if:

  - There is no existing CPDLC service, or

  - CPDLC service exists and the request is from either the Current Data Authority or Next Data Authority,

- Else set the Result parameter is set to the abstract value “rejected” and the Reject Reason to a CPDLC message with the message element NOT AUTHORIZED NEXT DATA AUTHORITY.

##### If a CPDLC-start indication is received from either the Current Data Authority or the Next Data Authority, and this results in a second CPDLC dialogue being established with a given ground system, the CPDLC-air-user shall invoke the CPDLC-user-abort request primitive for the first connection with that ground system.

##### If the CPDLC-air-user sets the CPDLC-response Result parameter to the abstract value “rejected” any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter shall be disregarded.

##### If the CPDLC-air-user sets the CPDLC-response Result parameter to the abstract value “accepted” and the request is from the Current Data Authority any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter shall be processed.

##### If the CPDLC-air-user sets the CPDLC-response Result parameter to the abstract value “accepted” and the request is from the Next Data Authority any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter shall be disregarded.

##### If the CPDLC-air-user sets the CPDLC-response Result parameter to the abstract value “accepted” the CPDLC-air-user shall:

- Establish an association between a CPDLC-ASE invocation and a ground system facility designation contained in CPDLC-start indication Calling Peer Identifier parameter,

- If there is no Current Data Authority, associate this CPDLC-ASE invocation with the Current Data Authority, or

- If the facility designation contained CPDLC-start indication Calling Peer Identifier parameter is the Next Data Authority associate the CPDLC-ASE invocation with the Next Data Authority.

##### If CPDLC-start indication has been received, the CPDLC-air-user shall be prohibited from invoking any CPDLC-service primitive with this ground system, except the CPDLC-user-abort request, until after it has invoked the CPDLC-start response.

#### Receipt of a CPDLC-start confirmation

##### If a CPDLC-start confirmation has been received with a Result parameter containing the abstract value “accepted” the CPDLC-air-user shall:

- Establish an association between a CPDLC-ASE invocation and a ground system  facility designation contained in CPDLC-start request Called Peer Identifier parameter,

- If there is no Current Data Authority, associate the CPDLC-ASE invocation with the Current Data Authority, or

- If the  facility designation contained CPDLC-start indication Called Peer Identifier parameter is the Next Data Authority associate the CPDLC-ASE invocation with the Next Data Authority.

### The DSC-start Service

#### Invoking the DSC-start request

##### Only a CPDLC-air-user shall be permitted to invoke the DSC-start service request primitive.

##### A CPDLC-air-user shall only be permitted to invoke the DSC-start-service request primitive if the CPDLC-air-user has no existing DSC dialogue.

##### If a CPDLC-air-user has invoked a DSC-start request, the CPDLC-air-user shall be prohibited from invoking any CPDLC-service primitive with this ground system, except the CPDLC-user-abort request, until after it has received a DSC-start confirmation.

#### Receipt of a DSC-start Indication and Invoking a DSC-start Response

##### Upon receipt of a DSC-start indication, the CPDLC-ground-user shall invoke a DSC-start response within 0.5 seconds.

##### Upon receipt of a DSC-start indication, the CPDLC-ground-user shall invoke a DSC-start response, with the response parameters set as follows:

- the Result parameter set to the abstract value “accepted” or “rejected”, and

- if, and only if,  the Result parameter is set to the abstract value “rejected”, then set the Reject Reason parameter to a CPDLC message with the message element SERVICE UNAVAILABLE.

#### Receipt of a DSC-start confirmation

##### If a DSC-start confirmation has been received with a Result parameter containing the abstract value “accepted” the CPDLC-air-user shall:

- Establish an association between a CPDLC-ASE invocation and the ground system facility designation contained in DSC-start request ICAO Facility Designator parameter,

- Associate the CPDLC-ASE invocation with a Downstream Data Authority.

### The CPDLC-message Service

#### Receipt of a CPDLC-message Indication

##### Upon receipt of a CPDLC-message indication, if the indication is from the Current Data Authority or a Downstream Data Authority the CPDLC-air-user shall process the CPDLC message contained in the CPDLC Message parameter.

##### If a CPDLC-message indication is received from the Current Data Authority containing at least the uplink message element NEXT DATA AUTHORITY, and having no more than one NEXT DATA AUTHORITY message elements, indicating that the specified facility designation is the Next Data Authority the CPDLC-air-user shall do the following in the order listed:

- Check that there is no other Next Data Authority already established;

- if there is, invoke CPDLC-user-abort request with the established Next Data Authority with the Reason parameter set to CPDLCUserAbortReason value [no-longer-next-data-authority]; and

- Then designate the ground system indicated in the CPDLC message from the CPDLC-message indication as the Next Data Authority.

##### If a CPDLC-message indication is received from the Current Data Authority containing more than one NEXT DATA AUTHORITY uplink message elements, the CPDLC-air-user shall disregard the Next Data Authority designations, and invoke CPDLC-message service request with the ERROR [errorinformation] message element with the [moreThanOneNextDataAuthorityElement] value as the CPDLC Message parameter value.

##### If a CPDLC-message indication is received containing at least the uplink message element NEXT DATA AUTHORITY, and it is not from the Current Data Authority the message shall be disregarded.

##### Upon receipt of a CPDLC-message indication, if the indication is not from the Current Data Authority or a Downstream Data Authority, the CPDLC-air-user shall:

- Invoke CPDLC-message service request with the CPDLC Message parameter containing a message with the message element NOT CURRENT DATA AUTHORITY, and

- Disregard the received message contained in the CPDLC-message indication CPDLC Message parameter.

### The CPDLC-end Service

#### The CPDLC-end Service Request

##### The CPDLC-air-user shall be prohibited from invoking the CPDLC-end request.

#### Receipt of a CPDLC-end Indication and Invoking a CPDLC-end Response

##### If a CPDLC-end indication is received but it is not from the Current Data Authority the CPDLC-air-user shall:

- Invoke CPDLC-end response with

  - the CPDLC Message parameter containing a message with the message element NOT CURRENT DATA AUTHORITY, and

  - the Result parameter set to the abstract value “rejected”, and

- Disregard any message provided in the CPDLC-end indication CPDLC Message parameter.

##### If a CPDLC-end indication is received from the Current Data Authority and the CPDLC-air-user has sent a message that requires a response for which it has not received a closure response then the CPDLC-air-user shall:

- Invoke CPDLC-end response with:

  - the CPDLC Message parameter containing a CPDLC ATCDownlinkMessage with the ERROR [errorinformation] message element with the [endServiceWithPendingMsgs] value, and

  - the Result parameter set to the abstract value “rejected”, and

- Disregard any message provided in the CPDLC-end indication CPDLC Message parameter.

##### If a CPDLC-end indication is received from the Current Data Authority and the CPDLC-air-user has received a message for which a response is required, and it has not yet sent the closure response to that message then the CPDLC-air-user shall:

- Invoke CPDLC-end response with:

  - the CPDLC Message parameter containing a CPDLC ATCDownlinkMessage with the ERROR [errorinformation] message element with the [endServiceWithPendingMsgs] value, and

  - the Result parameter set to the abstract value “rejected”, and

- Disregard any message provided in the CPDLC-end indication CPDLC Message parameter.

##### If a CPDLC-end indication is received that is from the Current Data Authority and the CPDLC Message parameter contains a message requiring a response, the CPDLC-air-user shall:

- If responding with any permitted CPDLC message response that is not the closure response, invoke CPDLC-message request with the response message as the CPDLC Message parameter, then

- If desired, invoke CPDLC-message request with the closure response message as the CPDLC Message parameter, then

- Invoke CPDLC-end response with:

  - the CPDLC Message parameter as the CPDLC closure response if not already sent, and

  - the Result parameter set to the abstract value “rejected” or “accepted”.

##### If a CPDLC-end indication is received that is from the Current Data Authority without a CPDLC Message parameter or with a CPDLC Message parameter containing a message with the response attribute N and not requiring a logical acknowledgment, the CPDLC-air-user shall invoke CPDLC-end response with:

- the CPDLC Message parameter with a CPDLC message if desired, and

- the Result parameter set to the abstract value “rejected” or “accepted”.

##### Upon invoking CPDLC-end response with Result parameter set to “accepted”, the CPDLC-air-user shall:

- delete any association with a ground system and Current Data Authority, and

- if a ground system is designated as Next Data Authority and an association with a CPDLC-ASE exists, replace the Next Data Authority association with a Current Data Authority association, or

- if a ground system is designated as Next Data Authority and no association with a CPDLC-ASE exists, delete Next Data Authority association with any ground system.

##### If the CPDLC-air-ASE associated with the Current Data Authority ceases to exist for any reason other than in response to a CPDLC-end request as specified above, any existing Next Data Authority designation and/or association shall cease to exist.

### The DSC-end Service

#### The DSC-end Request

##### Only the CPDLC-air-user shall be permitted to invoke the DSC-end request.

##### If a CPDLC-air user has invoked a DSC-end service request primitive, the air-user shall be prohibited from invoking a any CPDLC service primitive with this ground system (except the CPDLC-user-abort request primitive)  until it receives a DSC-end service confirmation primitive.

### The CPDLC-user-abort Service

#### Issuing a CPDLC-user-abort Request

##### The CPDLC-air-user shall have the capability to invoke CPDLC-user-abort request with the Reason parameter set to CPDLCUserAbortReason value [commanded-termination].

#### Receipt of a CPDLC-abort Indication

##### If the CPDLC-air-user receives a CPDLC-user-abort indication from the Current Data Authority or a CPDLC-provider-abort indication that causes the ASE invocation associated with the Current Data Authority to cease to exist, the CPDLC-air-user shall:

- Delete any association of a ground system to a Current Data Authority,

- If a ground system is designated as Next Data Authority and an association with a CPDLC-ASE exists, invoke CPDLC-user-abort request with the Reason parameter set to the value [current-data authority-abort], and

- Delete any association of a ground system to a Next Data Authority.

##### If the CPDLC-air-user receives a CPDLC-user-abort indication from the Next Data Authority or receives a CPDLC-provider-abort indication that causes the ASE invocation associated with the Next Data Authority to cease to exist, the CPDLC-air-user shall continue to maintain the association of the ground system to the Next Data Authority.

## CPDLC-Ground-User Requirements

### The CPDLC-start Service

#### Invoking the CPDLC-start request

##### If there is no CPDLC service, the only CPDLC service primitives the CPDLC-ground-user shall be permitted to invoke are the CPDLC-start-request or the CPDLC-forward request.

##### If a CPDLC-ground-user has invoked a CPDLC-start request, the CPDLC-ground-user shall be prohibited from invoking any CPDLC-service primitive, except the CPDLC-user-abort request with that aircraft, until after it has received a CPDLC-start confirmation.

#### Receipt of a CPDLC-start Indication and Invoking CPDLC-start Response

##### If a CPDLC-start indication is received from an aircraft with which the ground system currently has a CPDLC dialogue, the CPDLC-ground-user shall:

- invoke the CPDLC-start response with the Result parameter set to the abstract value “accepted”, and

- invoke the CPDLC-user-abort request for the first CPDLC dialogue with that aircraft.

##### The CPDLC-ground-user shall be prohibited from invoking the CPDLC-start response unless and until it has received a CPDLC-start indication.

##### If the CPDLC-ground-user sets the CPDLC-start response Result parameter to the abstract value “rejected” then the Reject Reason shall be an uplink message containing only the SERVICE UNAVAILABLE message element.

##### If the CPDLC-ground-user sets the CPDLC-start response Result parameter to the abstract value “rejected” the CPDLC-ground-user shall disregard any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter.

##### If the CPDLC-ground-user sets the CPDLC-response Result parameter to the abstract value “accepted” any CPDLC message contained in the CPDLC-start indication CPDLC Message parameter shall be processed.

##### If the CPDLC-ground-user sets the CPDLC-response Result parameter to the abstract value “accepted” the CPDLC-ground-user shall establish an association between a CPDLC-ASE invocation and a 24 bit aircraft address contained in CPDLC-start indication Calling Peer Identifier parameter.

##### If CPDLC-start indication has been received, the CPDLC-ground-user shall be prohibited from invoking any CPDLC-service primitive, except the CPDLC-user-abort request with that aircraft, until after it has invoked the CPDLC-start response.

#### Receipt of a CPDLC-start confirmation

##### If a CPDLC-start confirmation has been received with a Result parameter containing the abstract value “accepted” the CPDLC-ground-user shall establish an association between a CPDLC-ASE invocation and a 24 bit aircraft address contained in CPDLC-start request Called Peer Identifier parameter.

### The DSC-start Service

#### Receipt of a DSC-start Indication and Invoking DSC-start Response

##### The CPDLC-ground-user shall be prohibited from invoking the DSC-start response unless and until it has received a DSC-start indication.

##### If a DSC-start indication is received from an aircraft with which the ground system currently has a DSC dialogue, the CPDLC-ground-user shall:

- invoke the DSC-start response with the Result parameter set to the abstract value “accepted”, and

- invoke the CPDLC-user-abort request for the first DSC dialogue with that aircraft.

##### If the CPDLC-ground-user sets the DSC-start response Result parameter to the abstract value “rejected” then the Reject Reason shall be an uplink message containing only the SERVICE UNAVAILABLE message element.

##### If the CPDLC-ground-user sets the DSC-start response Result parameter to the abstract value “rejected” the CPDLC-ground-user shall disregard any CPDLC message contained in the DSC-start indication CPDLC Message parameter.

##### If the CPDLC-ground-user sets the DSC-start response Result parameter to the abstract value “accepted” any CPDLC message contained in the DSC-start indication CPDLC Message parameter shall be processed.

##### If the CPDLC-ground-user sets the DSC-start response Result parameter to the abstract value “accepted” the CPDLC-ground-user shall establish an association between a CPDLC-ASE invocation and a 24 bit aircraft address contained in the DSC-start indication Aircraft Identifier parameter.

##### If DSC-start indication has been received, the CPDLC-ground-user shall be prohibited from invoking any CPDLC-service primitive, except the CPDLC-user-abort request, until after it has invoked the DSC-start response.

### The CPDLC-end Service

#### The CPDLC-end Request

##### Only the CPDLC-ground-user shall be permitted to invoke the CPDLC-end request.

##### The CPDLC-ground-user shall be prohibited from invoking the CPDLC-end request if:

- it has sent a message that requires a response for which it has not received a closure response, or

- it has received a message for which a response is required, and it has not yet sent the closure response.

##### If a CPDLC-ground user has invoked a CPDLC-end service request primitive, the ground-user shall be prohibited from invoking any CPDLC service primitive with this aircraft, except the CPDLC-user-abort request primitive, until after It has received a CPDLC-end service confirmation primitive.

### The DSC-end Service

#### Receipt of a DSC-end Indication and Invoking DSC-end Response

##### The CPDLC-ground-user shall be prohibited from invoking the DSC-end response unless and until it has received a DSC-end indication.

##### If a DSC-end indication is received and the CPDLC-ground-user has sent a message that requires a response for which it has not received a closure response then the CPDLC-ground-user shall:

- Invoke DSC-end response with:

  - the CPDLC Message parameter containing a CPDLC ATCDownlinkMessage with the ERROR [errorinformation] message element with the [endServiceWithPendingMsgs] value, and

  - the Result parameter set to the abstract value “rejected”, and

- Disregard any message provided in the DSC-end indication CPDLC Message parameter.

##### If a DSC-end indication is received and the CPDLC-ground-user has received a message for which a response is required, and it has not yet sent the closure response to that message then the CPDLC-ground-user shall:

- Invoke DSC-end response with:

  - the CPDLC Message parameter containing a CPDLC ATCDownlinkMessage with the ERROR [errorinformation] message element with the [endServiceWithPendingMsgs] value, and

  - the Result parameter set to the abstract value “rejected”, and

- Disregard any message provided in the DSC-end indication CPDLC Message parameter.

##### If a DSC-end indication is received and the CPDLC Message parameter contains a message requiring a response, the CPDLC-ground-user shall:

- If responding with any permitted CPDLC message response that is not the closure response, invoke CPDLC-message request with the response message as the CPDLC Message parameter, then

- If desired, invoke CPDLC-message request with the closure response message as the CPDLC Message parameter, then

- Invoke DSC-end response with:

  - the CPDLC Message parameter as the CPDLC closure response if not already sent, and

  - the Result parameter set to the abstract value “rejected” or “accepted”.

##### If a DSC-end indication is received without a CPDLC Message parameter, or with a CPDLC Message parameter containing a message with the response attribute N and not requiring a logical acknowledgment, the CPDLC-ground-user shall invoke DSC-end response with:

- the CPDLC Message parameter with a CPDLC message if desired, and

- the Result parameter set to the abstract value “rejected” or “accepted”.

### The CPDLC-forward Service

#### Invoking the CPDLC-forward request

##### Only the CPDLC-ground-user shall be permitted to invoke the CPDLC-forward request.

### The CPDLC-user-abort Service

#### Issuing a CPDLC-user-abort Request

##### The CPDLC-ground-user shall have the capability to invoke CPDLC-user-abort request with the Reason parameter set to CPDLCUserAbortReason value [commanded-termination].

## Message Intent

### Purpose

- Note. —  2.3.7.6  contains the message set for CPDLC.  Message attributes, message presentation guidance, and data structure presentation guidance are presented.  The actual information exchanged between an aircraft and ground peer or a ground and ground peer CPDLC applications is defined in 2.3.4; however, 2.3.4 does not mandate any particular method for presenting this information.  The presentation of information to the controller and aircraft crew is a local implementation.  The message presentation recommendations contained in Tables 2.3.7-5 to 2.3.7-28 are one possible means of presenting the information.  These recommendations are generally consistent with current ICAO practices for displaying ATC information.

### Uplink message elements shall comply with the intent, use, and element attributes as presented in Tables 2.3.7-5 to 2.3.7-16.

Table 2.3.7-5.  Responses/Acknowledgments (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 0 | Indicates that ATS cannot comply with the request. | UNABLE | N | M | N |
| 1 | Indicates that ATS has received the message and will respond. | STANDBY | N | L | N |
| 2 | Indicates that ATS has received the request but it has been deferred until later. | REQUEST DEFERRED | N | L | N |
| 3 | Indicates that ATS has received and understood the message. | ROGER | N | L | N |
| 4 | Yes. | AFFIRM | N | L | N |
| 5 | No. | NEGATIVE | N | L | N |
| 235 | Notification of receipt of unlawful interference message. | ROGER 7500 | U | H | N |
| 211 | Indicates that the ATS has received the request and has passed it to the Next Control Authority. | REQUEST FORWARDED | N | L | N |
| 218 | Indicates to the pilot that the request has already been received on the ground. | REQUEST ALREADY RECEIVED | L | N | N |

Table 2.3.7-6.  Vertical Clearances (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 6 | Notification that an level change instruction should be expected. | EXPECT [level] | L | L | R |
| 7 | Notification that an instruction should be expected for the aircraft to commence climb at the specified time. | EXPECT CLIMB AT [time] | L | L | R |
| 8 | Notification that an instruction should be expected for the aircraft to commence climb at the specified position. | EXPECT CLIMB AT [position] | L | L | R |
| 9 | Notification that an instruction should be expected for the aircraft to commence descent at the specified time. | EXPECT DESCENT AT [time] | L | L | R |
| 10 | Notification that an instruction should be expected for the aircraft to commence descent at the specified position. | EXPECT DESCENT AT [position] | L | L | R |
| 11 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time. | EXPECT CRUISE CLIMB AT [time] | L | L | R |
| 12 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position. | EXPECT CRUISE CLIMB AT [position] | L | L | R |
| 13 | Notification that an instruction should be expected for the aircraft to commence climb at the specified time to the specified level. | AT [time] EXPECT CLIMB TO [level] | L | L | R |
| 14 | Notification that an instruction should be expected for the aircraft to commence climb at the specified position to the specified level. | AT [position] EXPECT CLIMB TO [level] | L | L | R |
| 15 | Notification that an instruction should be expected for the aircraft to commence descent at the specified time to the specified level. | AT [time] EXPECT DESCENT TO [level] | L | L | R |
| 16 | Notification that an instruction should be expected for the aircraft to commence descent at the specified position to the specified level. | AT [position] EXPECT DESCENT TO [level] | L | L | R |
| 17 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified time to the specified level. | AT [time] EXPECT CRUISE CLIMB TO [level] | L | L | R |
| 18 | Notification that an instruction should be expected for the aircraft to commence cruise climb at the specified position to the specified level. | AT [position] EXPECT CRUISE CLIMB TO [level] | L | L | R |
| 19 | Instruction to maintain the specified level. | MAINTAIN [level] | N | M | W/U |
| 20 | Instruction that a climb to a specified level is to commence and the level is to be maintained when reached. | CLIMB TO [level] | N | M | W/U |
| 21 | Instruction that at the specified time, a climb to the specified level is to commence and once reached the specified level is to be maintained. | AT [time] CLIMB TO [level] | N | M | W/U |
| 22 | Instruction that at the specified position, a climb to the specified level is to commence and once reached the specified level is to be maintained. | AT [position] CLIMB TO [level] | N | M | W/U |
| 185 | Instruction that after passing the specified position, a climb to the specified level is to commence and once reached the specified level is to be maintained. | AFTER PASSING [position] CLIMB TO [level] | N | M | W/U |
| 23 | Instruction that a descent to a specified level is to commence and the level is to be maintained when reached. | DESCEND TO AND MAINTAIN [level] | N | M | W/U |
| 24 | Instruction that at a specified time a descent to a specified level is to commence and once reached the specified level is to be maintained. | AT [time] DESCEND TO AND MAINTAIN [level] | N | M | W/U |
| 25 | Instruction that at the specified position a descent to the specified level is to commence and when the specified level is reached it is to be maintained. | AT [position] DESCEND TO AND MAINTAIN [level] | N | M | W/U |
| 186 | Instruction that after passing the specified position, a descent to the specified level is to commence and once reached the specified level is to be maintained. | AFTER PASSING [position] DESCEND TO AND MAINTAIN [level] | N | M | W/U |
| 26 | Instruction that a climb is to commence at a rate such that the specified level is reached at or before the specified time. | CLIMB TO REACH [level] BY [time] | N | M | W/U |
| 27 | Instruction that a climb is to commence at a rate such that the specified level is reached at or before the specified position. | CLIMB TO REACH [level] BY [position] | N | M | W/U |
| 28 | Instruction that a descent is to commence at a rate such that the specified level is reached at or before the specified time. | DESCEND TO REACH [level] BY [time] | N | M | W/U |
| 29 | Instruction that a descent is to commence at a rate such that the specified level is reached at or before the specified position. | DESCEND TO REACH [level] BY [position] | N | M | W/U |
| 192 | Instruction that a change of level is to continue, but at a rate such that the specified level is reached at or before the specified time. | REACH [level] BY [time] | N | M | W/U |
| 209 | Instruction that a change of level is to continue, but at a rate such that the specified level is reached at or before the specified position | REACH [level] BY [position] | N | M | W/U |
| 30 | A level within the defined vertical range specified is to be maintained. | MAINTAIN BLOCK [level] TO [level] | N | M | W/U |
| 31 | Instruction that a climb to an level within the vertical range defined is to commence. | CLIMB TO AND MAINTAIN BLOCK [level] TO [level] | N | M | W/U |
| 32 | Instruction that a descent to an level within the vertical range defined is to commence. | DESCEND TO AND MAINTAIN BLOCK [level] TO [level] | N | M | W/U |
| 34 | A cruise climb is to commence and continue until the specified level is reached. | CRUISE CLIMB TO [level] | N | M | W/U |
| 35 | A cruise climb can commence once above the specified level. | CRUISE CLIMB ABOVE [level] | N | M | W/U |
| 219 | Instruction to stop the climb below the previously assigned level. | STOP CLIMB AT [level] | U | M | W/U |
| 220 | Instruction to stop the descent above the previously assigned level. | STOP DESCENT AT [level] | U | M | W/U |
| 36 | The climb to the specified level should be made at the aircraft’s best rate. | EXPEDITE CLIMB TO [level] | U | M | W/U |
| 37 | The descent to the specified level should be made at the aircraft’s best rate. | EXPEDITE DESCENT TO [level] | U | M | W/U |
| 38 | Urgent instruction to immediately climb to the specified level. | IMMEDIATELY CLIMB TO [level] | D | H | W/U |
| 39 | Urgent instruction to immediately descend to the specified level. | IMMEDIATELY DESCEND TO [level] | D | H | W/U |
| 40 | Urgent instruction to immediately stop a climb once the specified level is reached. | IMMEDIATELY STOP CLIMB AT [level] | D | H | W/U |
| 41 | Urgent instruction to immediately stop a descent once the specified level is reached. | IMMEDIATELY STOP DESCENT AT [level] | D | H | W/U |
| 171 | Instruction to climb at not less than the specified rate. | CLIMB AT [vertical rate] MINIMUM | N | M | W/U |
| 172 | Instruction to climb at not above the specified rate. | CLIMB AT [vertical rate] MAXIMUM | N | M | W/U |
| 173 | Instruction to descend at not less than the specified rate. | DESCEND AT [vertical rate] MINIMUM | N | M | W/U |
| 174 | Instruction to descend at not above the specified rate. | DESCEND AT [vertical rate] MAXIMUM | N | M | W/U |
| 33 | Reserved. |  |  |  |  |

Table 2.3.7-7.  Crossing Constraints (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 42 | Notification that an level change instruction should be expected which will require the specified position to be crossed at the specified level. | EXPECT TO CROSS [position] AT [level] | L | L | R |
| 43 | Notification that an level change instruction should be expected which will require the specified position to be crossed at or above the specified level. | EXPECT TO CROSS [position] AT OR ABOVE [level] | L | L | R |
| 44 | Notification that an level change instruction should be expected which will require the specified position to be crossed at or below the specified level. | EXPECT TO CROSS [position] AT OR BELOW [level] | L | L | R |
| 45 | Notification that an level change instruction should be expected which will require the specified position to be crossed at the specified level which is to be maintained subsequently. | EXPECT TO CROSS [position] AT AND MAINTAIN [level] | L | L | R |
| 46 | The specified position is to be crossed at the specified level. This may require the aircraft to modify its climb or descent profile. | CROSS [position] AT [level] | N | M | W/U |
| 47 | The specified position is to be crossed at or above the specified level. | CROSS [position] AT OR ABOVE [level] | N | M | W/U |
| 48 | The specified position is to be crossed at or below the specified level. | CROSS [position] AT OR BELOW [level] | N | M | W/U |
| 49 | Instruction that the specified position is to be crossed at the specified level and that level is to be maintained when reached. | CROSS [position] AT AND MAINTAIN [level] | N | M | W/U |
| 50 | The specified position is to be crossed at an level between the specified levels. | CROSS [position] BETWEEN [level] AND [level] | N | M | W/U |
| 51 | The specified position is to be crossed at the specified time. | CROSS [position] AT [time] | N | M | W/U |
| 52 | The specified position is to be crossed at or before the specified time. | CROSS [position] AT OR BEFORE [time] | N | M | W/U |
| 53 | The specified position is to be crossed at or after the specified time. | CROSS [position] AT OR AFTER [time] | N | M | W/U |
| 54 | The specified position is to be crossed at a time between the specified times. | CROSS [position] BETWEEN [time] AND [time] | N | M | W/U |
| 55 | The specified position is to be crossed at the specified speed and the specified speed is to be maintained until further advised. | CROSS [position] AT [speed] | N | M | W/U |
| 56 | The specified position is to be crossed at a speed equal to or less than the specified speed and the specified speed or less is to be maintained until further advised. | CROSS [position] AT OR LESS THAN [speed] | N | M | W/U |
| 57 | The specified position is to be crossed at a speed equal to or greater than the specified speed and the specified speed or greater is to be maintained until further advised. | CROSS [position] AT OR GREATER THAN [speed] | N | M | W/U |
| 58 | The specified position is to be crossed at the specified time and the specified level. | CROSS [position] AT [time] AT [level] | N | M | W/U |
| 59 | The specified position is to be crossed at or before the specified time and at the specified level. | CROSS [position] AT OR BEFORE [time] AT [level] | N | M | W/U |
| 60 | The specified position is to be crossed at or after the specified time and at the specified level. | CROSS [position] AT OR AFTER [time] AT [level] | N | M | W/U |
| 61 | Instruction that the specified position is to be crossed at the specified level and speed and the level and speed are to be maintained. | CROSS [position] AT AND MAINTAIN [level] AT [speed] | N | M | W/U |
| 62 | Instruction that at the specified time the specified position is to be crossed at the specified level and the level is to be maintained. | AT [time] CROSS [position] AT AND MAINTAIN [level] | N | M | W/U |
| 63 | Instruction that at the specified time the specified position is to be crossed at the specified level and speed and the level and speed are to be maintained. | AT [time] CROSS [position] AT AND MAINTAIN [level] AT [speed] | N | M | W/U |

Table 2.3.7-8.  Lateral Offsets (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 64 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction. | OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 65 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified position. | AT [position] OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 66 | Instruction to fly a parallel track to the cleared route at a displacement of the specified distance in the specified direction and commencing at the specified time. | AT [time] OFFSET [distance offset] [direction] OF ROUTE | N | M | W/U |
| 67 | The cleared flight route is to be rejoined. | PROCEED BACK ON ROUTE | N | M | W/U |
| 68 | The cleared flight route is to be rejoined at or before the specified position. | REJOIN ROUTE BY [position] | N | M | W/U |
| 69 | The cleared flight route is to be rejoined at or before the specified time. | REJOIN ROUTE BY [time] | N | M | W/U |
| 70 | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified position. | EXPECT BACK ON ROUTE BY [position] | L | L | R |
| 71 | Notification that a clearance may be issued to enable the aircraft to rejoin the cleared route at or before the specified time. | EXPECT BACK ON ROUTE BY [time] | L | L | R |
| 72 | Instruction to resume own navigation following a period of tracking or heading clearances. May be used in conjunction with an instruction on how or where to rejoin the cleared route. | RESUME OWN NAVIGATION | N | M | W/U |

Table 2.3.7-9.  Route Modifications (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 73 | Notification to the aircraft of the instructions to be followed from departure until the specified clearance limit. | [departure clearance] | N | M | W/U |
| 74 | Instruction to proceed directly from its present position to the specified position. | PROCEED DIRECT TO [position] | N | M | W/U |
| 75 | Instruction to proceed, when able, directly to the specified position. | WHEN ABLE PROCEED DIRECT TO [position] | N | M | W/U |
| 76 | Instruction to proceed, at the specified time, directly to the specified position. | AT [time] PROCEED DIRECT TO [position] | N | M | W/U |
| 77 | Instruction to proceed, at the specified position, directly to the next specified position. | AT [position] PROCEED DIRECT TO [position] | N | M | W/U |
| 78 | Instruction to proceed, upon reaching the specified level, directly to the specified position. | AT [level] PROCEED DIRECT TO [position] | N | M | W/U |
| 79 | Instruction to proceed to the specified position via the specified route. | CLEARED TO [position] VIA [route clearance] | N | M | W/U |
| 80 | Instruction to proceed via the specified route. | CLEARED [route clearance] | N | M | W/U |
| 81 | Instruction to proceed in accordance with the specified procedure. | CLEARED [procedure name] | N | M | W/U |
| 236 | Instruction to leave controlled airspace. | LEAVE CONTROLLED AIRSPACE | N | M | W/U |
| 82 | Approval to deviate up to the specified distance from the cleared route in the specified direction. | CLEARED TO DEVIATE UP TO [distance offset] [direction] OF ROUTE | N | M | W/U |
| 83 | Instruction to proceed from the specified position via the specified route. | AT [position] CLEARED [route clearance] | N | M | W/U |
| 84 | Instruction to proceed from the specified position via the specified procedure. | AT [position] CLEARED [procedure name] | N | M | W/U |
| 85 | Notification that a clearance to fly on the specified route may be issued. | EXPECT [route clearance] | L | L | R |
| 86 | Notification that a clearance to fly on the specified route from the specified position may be issued. | AT [position] EXPECT [route clearance] | L | L | R |
| 87 | Notification that a clearance to fly directly to the specified position may be issued. | EXPECT DIRECT TO [position] | L | L | R |
| 88 | Notification that a clearance to fly directly from the first specified position to the next specified position may be issued. | AT [position] EXPECT DIRECT TO [position] | L | L | R |
| 89 | Notification that a clearance to fly directly to the specified position commencing at the specified time may be issued. | AT [time] EXPECT DIRECT TO [position] | L | L | R |
| 90 | Notification that a clearance to fly directly to the specified position commencing when the specified level is reached may be issued. | AT [level] EXPECT DIRECT TO [position] | L | L | R |
| 91 | Instruction to enter a holding pattern with the specified characteristics at the specified position and level. | HOLD AT [position] MAINTAIN [level] INBOUND TRACK [degrees] [direction] TURNS [leg type] | N | M | W/U |
| 92 | Instruction to enter a holding pattern with the published characteristics at the specified position and level. | HOLD AT [position] AS PUBLISHED MAINTAIN [level] | N | M | W/U |
| 93 | Notification that an onwards clearance may be issued at the specified time. | EXPECT FURTHER CLEARANCE AT [time] | L | L | R |
| 94 | Instruction to turn left or right as specified onto the specified heading. | TURN [direction] HEADING [degrees] | N | M | W/U |
| 95 | Instruction to turn left or right as specified onto the specified track. | TURN [direction] GROUND TRACK [degrees] | N | M | W/U |
| 215 | Instruction to turn a specified number of degrees left or right. | TURN [degrees][direction] | N | M | W/U |
| 190 | Instruction to fly on the specified heading. | FLY HEADING [degrees] | N | M | W/U |
| 96 | Instruction to continue to fly on the current heading. | CONTINUE PRESENT HEADING | N | M | W/U |
| 97 | Instruction to fly on the specified heading from the specified position. | AT [position] FLY HEADING [degrees] | N | M | W/U |
| 221 | Instruction to stop turn at the specified heading prior to reaching the previously assigned heading. | STOP TURN HEADING [degrees] | U | M | W/U |
| 98 | Instruction to turn immediately left or right as specified onto the specified heading. | IMMEDIATELY TURN [direction] HEADING [degrees] | D | H | W/U |
| 99 | Notification that a clearance may be issued for the aircraft to fly the specified procedure. | EXPECT [procedure name] | L | L | R |

Table 2.3.7-10.  Speed Changes (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 100 | Notification that a speed instruction may be issued to be effective at the specified time. | AT [time] EXPECT [speed] | L | L | R |
| 101 | Notification that a speed instruction may be issued to be effective at the specified position. | AT [position] EXPECT [speed] | L | L | R |
| 102 | Notification that a speed instruction may be issued to be effective at the specified level. | AT [level] EXPECT [speed] | L | L | R |
| 103 | Notification that a speed range instruction may be issued to be effective at the specified time. | AT [time] EXPECT [speed] TO [speed] | L | L | R |
| 104 | Notification that a speed range instruction may be issued to be effective at the specified position. | AT [position] EXPECT [speed] TO [speed] | L | L | R |
| 105 | Notification that a speed range instruction may be issued to be effective at the specified level. | AT [level] EXPECT [speed] TO [speed] | L | L | R |
| 106 | The specified speed is to be maintained. | MAINTAIN [speed] | N | M | W/U |
| 188 | After passing the specified position the specified speed is to be maintained. | AFTER PASSING [position] MAINTAIN [speed] | N | M | W/U |
| 107 | The present speed is to be maintained. | MAINTAIN PRESENT SPEED | N | M | W/U |
| 108 | The specified speed or a greater speed is to be maintained. | MAINTAIN [speed] OR GREATER | N | M | W/U |
| 109 | The specified speed or a lesser speed is to be maintained. | MAINTAIN [speed] OR LESS | N | M | W/U |
| 110 | A speed with the specified range is to be maintained. | MAINTAIN [speed] TO [speed] | N | M | W/U |
| 111 | The present speed is to be increased to the specified speed and maintained until further advised. | INCREASE SPEED TO [speed] | N | M | W/U |
| 112 | The present speed is to be increased to the specified speed or greater, and maintained at or above the specified speed until further advised. | INCREASE SPEED TO [speed] OR GREATER | N | M | W/U |
| 113 | The present speed is to be reduced to the specified speed and maintained until further advised. | REDUCE SPEED TO [speed] | N | M | W/U |
| 114 | The present speed is to be reduced to the specified speed or less and maintained at or below the specified speed until further advised. | REDUCE SPEED TO [speed] OR LESS | N | M | W/U |
| 115 | The specified speed is not to be exceeded. | DO NOT EXCEED [speed] | N | M | W/U |
| 116 | Notification that the aircraft need no longer comply with the previously issued speed restriction. | RESUME NORMAL SPEED | N | M | W/U |
| 189 | The present speed is to be changed to the specified speed. | ADJUST SPEED TO [speed] | N | M | W/U |
| 222 | Notification that the aircraft may keep its preferred speed without restriction. | NO SPEED RESTRICTION | L | L | R |
| 223 | Instruction to reduce present speed to the minimum safe approach speed | REDUCE TO MINIMUM APPROACH SPEED | N | M | W/U |

Table 2.3.7-11.  Contact/Monitor/Surveillance Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 117 | The ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | CONTACT [unitname] [frequency] | N | M | W/U |
| 118 | At the specified position the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | AT [position] CONTACT [unitname] [frequency] | N | M | W/U |
| 119 | At the specified time the ATS unit with the specified ATS unit name is to be contacted on the specified frequency. | AT [time] CONTACT [unitname] [frequency] | N | M | W/U |
| 120 | The ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | MONITOR [unitname] [frequency] | N | M | W/U |
| 121 | At the specified position the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | AT [position] MONITOR [unitname] [frequency] | N | M | W/U |
| 122 | At the specified time the ATS unit with the specified ATS unit name is to be monitored on the specified frequency. | AT [time] MONITOR [unitname] [frequency] | N | M | W/U |
| 123 | The specified code (SSR code) is to be selected. | SQUAWK [code] | N | M | W/U |
| 124 | The SSR transponder responses are to be disabled. | STOP SQUAWK | N | M | W/U |
| 125 | The SSR transponder responses should include level information. | SQUAWK MODE CHARLIE | N | M | W/U |
| 126 | The SSR transponder responses should no longer include level information. | STOP SQUAWK MODE CHARLIE | N | M | W/U |
| 179 | The ‘ident’ function on the SSR transponder is to be actuated. | SQUAWK IDENT | N | M | W/U |

Table 2.3.7-12.  Report/Confirmation Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 127 | Instruction to report when the aircraft is back on the cleared route. | REPORT BACK ON ROUTE | N | L | W/U |
| 128 | Instruction to report when the aircraft has left the specified level. | REPORT LEAVING [level] | N | L | W/U |
| 129 | Instruction to report when the aircraft is in level flight at the specified level. | REPORT MAINTAINING [level] | N | L | W/U |
| 175 | Instruction to report when the aircraft has reached the specified level. | REPORT REACHING [level] | N | L | W/U |
| 180 | Instruction to report when the aircraft is within the specified vertical range. | REPORT REACHING BLOCK [level] TO [level] | N | L | W/U |
| 130 | Instruction to report when the aircraft has passed the specified position. | REPORT PASSING [position] | N | L | W/U |
| 181 | Instruction to report the present distance to or from the specified position. | REPORT DISTANCE [to/from] [position] | N | M | Y |
| 184 | Instruction to report at the specified time the distance to or from the specified position. | AT TIME [time] REPORT DISTANCE [to/from] [position] | N | L | Y |
| 228 | Instruction to report the estimated time of arrival at the specified position | REPORT ETA [position] | L | L | Y |
| 131 | Instruction to report the amount of fuel remaining and the number of persons on board. | REPORT REMAINING FUEL AND PERSONS ON BOARD | U | M | Y |
| 132 | Instruction to report the present position. | REPORT POSITION | N | M | Y |
| 133 | Instruction to report the present level. | REPORT PRESENT LEVEL | N | M | Y |
| 134 | Instruction to report the requested speed. | REPORT [speed type] [speed type] [speed type] SPEED | N | M | Y |
| 135 | Instruction to confirm and acknowledge the currently assigned level. | CONFIRM ASSIGNED LEVEL | N | L | Y |
| 136 | Instruction to confirm and acknowledge the currently assigned speed. | CONFIRM ASSIGNED SPEED | N | L | Y |
| 137 | Instruction to confirm and acknowledge the currently assigned route. | CONFIRM ASSIGNED ROUTE | N | L | Y |
| 138 | Instruction to confirm the previously reported time over the last reported waypoint. | CONFIRM TIME OVER REPORTED WAYPOINT | N | L | Y |
| 139 | Instruction to confirm the identity of the previously reported waypoint. | CONFIRM REPORTED WAYPOINT | N | L | Y |
| 140 | Instruction to confirm the identity of the next waypoint. | CONFIRM NEXT WAYPOINT | N | L | Y |
| 141 | Instruction to confirm the previously reported estimated time at the next waypoint. | CONFIRM NEXT WAYPOINT ETA | N | L | Y |
| 142 | Instruction to confirm the identity of the next but one waypoint. | CONFIRM ENSUING WAYPOINT | N | L | Y |
| 143 | The request was not understood. It should be clarified and resubmitted. | CONFIRM REQUEST | N | L | Y |
| 144 | Instruction to report the selected code (SSR). | CONFIRM SQUAWK | N | L | Y |
| 145 | Instruction to report the present heading. | REPORT HEADING | N | M | Y |
| 146 | Instruction to report the present ground track. | REPORT GROUND TRACK | N | M | Y |
| 182 | Instruction to report the identification code of the last ATIS received. | CONFIRM ATIS CODE | N | L | Y |
| 147 | Instruction to make a position report. | REQUEST POSITION REPORT | N | M | Y |
| 216 | Instruction to file a flight plan. | REQUEST FLIGHT PLAN | N | M | Y |
| 217 | Instruction to report that the aircraft has landed. | REPORT ARRIVAL | N | M | Y |
| 229 | Instruction to report the preferred alternate aerodrome for landing. | REPORT ALTERNATE AERODROME | L | L | Y |
| 231 | Instruction to indicate the pilot’s preferred level. | STATE PREFERRED LEVEL | L | L | Y |
| 232 | Instruction to indicate the pilot’s preferred time and/or position to commence descent to the aerodrome of intended arrival. | STATE TOP OF DESCENT | L | L | Y |

Table 2.3.7-13.  Negotiation Requests (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 148 | Request for the earliest time at which the specified level can be accepted. | WHEN CAN YOU ACCEPT [level] | N | L | Y |
| 149 | Instruction to report whether or not the specified level can be accepted at the specified position. | CAN YOU ACCEPT [level] AT [position] | N | L | A/N |
| 150 | Instruction to report whether or not the specified level can be accepted at the specified time. | CAN YOU ACCEPT [level] AT [time] | N | L | A/N |
| 151 | Instruction to report the earliest time when the specified speed can be accepted. | WHEN CAN YOU ACCEPT [speed] | N | L | Y |
| 152 | Instruction to report the earliest time when the specified offset track can be accepted. | WHEN CAN YOU ACCEPT [distance offset] [direction] OFFSET | N | L | Y |

Table 2.3.7-14.  Air Traffic Advisories (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 153 | ATS advisory that the altimeter setting should be the specified setting. | ALTIMETER [altimeter] | N | L | R |
| 213 | ATS advisory that the specified altimeter setting relates to the specified facility. | [facility designation] ALTIMETER [altimeter] | N | L | R |
| 154 | ATS advisory that the radar service is terminated. | RADAR SERVICE TERMINATED | N | L | R |
| 191 | ATS advisory that the aircraft is entering airspace in which no air traffic services are provided and all existing air traffic services are terminated. | ALL ATS TERMINATED | N | M | R |
| 155 | ATS advisory that radar contact has been established at the specified position. | RADAR CONTACT [position] | N | M | R |
| 156 | ATS advisory that radar contact has been lost. | RADAR CONTACT LOST | N | M | R |
| 210 | ATS advisory that the aircraft has been identified on radar at the specified position. | IDENTIFIED [position] | N | M | R |
| 193 | Indication that radar identification has been lost. | IDENTIFICATION LOST | N | M | R |
| 157 | A continuous transmission is detected on the specified frequency. Check the microphone button. | CHECK STUCK MICROPHONE [frequency] | U | M | N |
| 158 | ATS advisory that the ATIS information identified by the specified code is the current ATIS information. | ATIS [atis code] | N | L | R |
| 212 | ATS advisory that the specified ATIS information at the specified airport is current. | [facility designation] ATIS [atis code] CURRENT | N | L | R |
| 214 | ATS advisory that indicates the RVR value for the specified runway. | RUNWAY [runway] VISUAL RANGE [rvr] | N | M | R |
| 224 | ATS advisory that no delay is expected. | NO DELAY EXPECTED | N | L | R |
| 225 | ATS advisory that the expected delay has not been determined. | DELAY NOT DETERMINED | N | L | R |
| 226 | ATS advisory that the aircraft may expect to be cleared to commence its approach procedure at the specified time. | EXPECTED APPROACH TIME [time] | N | L | R |

Table 2.3.7-15.  System Management Messages (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 159 | A system generated message that the ground system has detected an error. | ERROR [error information] | U | M | N |
| 160 | Notification to the avionics that the next data authority is the specified ATSU. | NEXT DATA AUTHORITY [facility designation] | L | N | N |
| 161 | Notification to the avionics that the data link connection with the current data authority is being terminated. | END SERVICE | L | N | N |
| 162 | Notification that the ground system does not support this message. | SERVICE UNAVAILABLE | L | L | N |
| 234 | Notification that the ground system does not have a flight plan for that aircraft. | FLIGHT PLAN NOT HELD | L | L | N |
| 163 | Notification to the pilot of an ATSU identifier. | [facility designation] | L | N | N |
| 227 | Confirmation to the aircraft system that the ground system has received the message to which the logical acknowledgment refers and found it acceptable for display to the responsible person. | LOGICAL ACKNOWLEDGMENT | N | M | N |
| 233 | Notification to the pilot that messages sent requiring a logical acknowledgment will not be accepted by this ground system. | USE OF LOGICAL ACKNOWLEDGMENT PROHIBITED | N | M | N |

Table 2.3.7-16.  Additional Messages (uplink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 164 | The associated instruction may be complied with at any future time. | WHEN READY | L | N | N |
| 230 | The associated instruction is to be complied with immediately. | IMMEDIATELY | D | H | N |
| 165 | Used to link two messages, indicating the proper order of execution of clearances/ instructions. | THEN | L | N | N |
| 166 | The associated instruction is issued due to traffic considerations. | DUE TO [traffic type] TRAFFIC | L | N | N |
| 167 | The associated instruction is issued due to airspace restrictions. | DUE TO AIRSPACE RESTRICTION | L | N | N |
| 168 | The indicated communication should be ignored. | DISREGARD | U | M | R |
| 176 | Notification that the operator is responsible for maintaining separation from other traffic and is also responsible for maintaining Visual Meteorological Conditions. | MAINTAIN OWN SEPARATION AND VMC | N | M | W/U |
| 177 | Used in conjunction with a clearance/instruction to indicate that the operator may execute when prepared to do so. | AT PILOTS DISCRETION | L | L | N |
| 169 |  | [free text] | N | L | R |
| 170 |  | [free text] | D | H | R |
| 194 |  | [free text] | N | L | Y |
| 178 |  | [free text] | N | L | N |
| 195 |  | [free text] | L | L | R |
| 196 |  | [free text] | N | M | W/U |
| 197 |  | [free text] | U | M | W/U |
| 198 |  | [free text] | D | H | W/U |
| 199 |  | [free text] | N | M | W/U |
| 200 |  | [free text] | L | L | R |
| 201 |  | [free text] | N | M | W/U |
| 202 |  | [free text] | D | H | W/U |
| 203 |  | [free text] | N | M | R |
| 204 |  | [free text] | N | M | Y |
| 183 |  | [free text] | N | M | N |
| 205 |  | [free text] | N | M | A/N |
| 206 |  | [free text] | L | N | Y |
| 187 |  | [free text] | L | N | N |
| 207 |  | [free text] | L | L | Y |
| 208 |  | [free text] | L | L | N |

### Downlink message elements shall comply with the intent, use, and element attributes as presented in Tables 2.3.7-17 to 2.3.7-28.

Table 2.3.7-17.  Responses (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 0 | The instruction is understood and will be complied with. | WILCO | N | M | N |
| 1 | The instruction cannot be complied with. | UNABLE | N | M | N |
| 2 | Wait for a reply. | STANDBY | N | M | N |
| 3 | Message received and understood. | ROGER | N | M | N |
| 4 | Yes. | AFFIRM | N | M | N |
| 5 | No. | NEGATIVE | N | M | N |

Table 2.3.7-18.  Vertical Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 6 | Request to fly at the specified level. | REQUEST [level] | N | L | Y |
| 7 | Request to fly at an level within the specified vertical range interval. | REQUEST BLOCK [level] TO [level] | N | L | Y |
| 8 | Request to cruise climb to the specified level. | REQUEST CRUISE CLIMB TO [level] | N | L | Y |
| 9 | Request to climb to the specified level. | REQUEST CLIMB TO [level] | N | L | Y |
| 10 | Request to descend to the specified level. | REQUEST DESCENT TO [level] | N | L | Y |
| 11 | Request that at the specified position a climb to the specified level be approved. | AT [position] REQUEST CLIMB TO [level] | N | L | Y |
| 12 | Request that at the specified position a descent to the specified level be approved. | AT [position] REQUEST DESCENT TO [level] | N | L | Y |
| 13 | Request that at the specified time a climb to the specified level be approved. | AT [time] REQUEST CLIMB TO [level] | N | L | Y |
| 14 | Request that at the specified time a descent to the specified level be approved. | AT [time] REQUEST DESCENT TO [level] | N | L | Y |
| 69 | Request that a descent be approved on a see-and-avoid basis. | REQUEST VMC DESCENT | N | L | Y |

Table 2.3.7-19.  Lateral Off-Set Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 15 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved. | REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |
| 16 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified position. | AT [position] REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |
| 17 | Request that a parallel track, offset from the cleared track by the specified distance in the specified direction, be approved from the specified time. | AT [time] REQUEST OFFSET [distance offset] [direction] OF ROUTE | N | L | Y |

Table 2.3.7-20.  Speed Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 18 | Request to fly at the specified speed. | REQUEST [speed] | N | L | Y |
| 19 | Request to fly within the specified speed range. | REQUEST [speed] TO [speed] | N | L | Y |

Table 2.3.7-21.  Voice Contact Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 20 | Request for voice contact. | REQUEST VOICE CONTACT | N | L | Y |
| 21 | Request for voice contact on the specified frequency. | REQUEST VOICE CONTACT [frequency] | N | L | Y |

Table 2.3.7-22.  Route Modification Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 22 | Request to track from the present position direct to the specified position. | REQUEST DIRECT TO [position] | N | L | Y |
| 23 | Request for the specified procedure clearance. | REQUEST [procedure name] | N | L | Y |
| 24 | Request for a route clearance. | REQUEST CLEARANCE [route clearance] | N | L | Y |
| 25 | Request for a clearance. | REQUEST [clearance type] CLEARANCE | N | L | Y |
| 26 | Request for a weather deviation to the specified position via the specified route. | REQUEST WEATHER DEVIATION TO [position] VIA [route clearance] | N | M | Y |
| 27 | Request for a weather deviation up to the specified distance off track in the specified direction. | REQUEST WEATHER DEVIATION UP TO [distance offset] [direction] OF ROUTE | N | M | Y |
| 70 | Request a clearance to adopt the specified heading. | REQUEST HEADING [degrees] | N | L | Y |
| 71 | Request a clearance to adopt the specified ground track. | REQUEST GROUND TRACK [degrees] | N | L | Y |

Table 2.3.7-23.  Reports (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 28 | Notification of leaving the specified level. | LEAVING [level] | N | L | N |
| 29 | Notification of climbing to the specified level. | CLIMBING TO [level] | N | L | N |
| 30 | Notification of descending to the specified level. | DESCENDING TO [level] | N | L | N |
| 31 | Notification of passing the specified position. | PASSING [position] | N | L | N |
| 78 | At the specified time, the aircraft’s position was as specified. | AT [time] [distance] [to/from] [position] | N | L | N |
| 32 | Notification of the present level. | PRESENT LEVEL[level] | N | L | N |
| 33 | Notification of the present position. | PRESENT POSITION [position] | N | L | N |
| 34 | Notification of the present speed. | PRESENT SPEED [speed] | N | L | N |
| 113 | Notification of the requested speed type. | [speed type] [speed type] [speed type] SPEED [speed] | N | L | N |
| 35 | Notification of the present heading in degrees. | PRESENT HEADING [degrees] | N | L | N |
| 36 | Notification of the present ground track in degrees. | PRESENT GROUND TRACK [degrees] | N | L | N |
| 37 | Notification that the aircraft is maintaining the specified level. | LEVEL [level] | N | L | N |
| 72 | Notification that the aircraft has reached the specified level. | REACHING [level] | N | L | N |
| 76 | Notification that the aircraft has reached an level within the specified vertical range. | REACHING BLOCK [level] TO [level] | N | L | N |
| 38 | Read-back of the assigned level. | ASSIGNED LEVEL[level] | N | M | N |
| 77 | Read-back of the assigned vertical range. | ASSIGNED BLOCK [level] TO [level] | N | M | N |
| 39 | Read-back of the assigned speed. | ASSIGNED SPEED [speed] | N | M | N |
| 40 | Read-back of the assigned route. | ASSIGNED ROUTE [route clearance] | N | M | N |
| 41 | The aircraft has regained the cleared route. | BACK ON ROUTE | N | M | N |
| 42 | The next waypoint is the specified position. | NEXT WAYPOINT [position] | N | L | N |
| 43 | the ETA at the next waypoint is as specified. | NEXT WAYPOINT ETA [time] | N | L | N |
| 44 | The next but one waypoint is the specified position. | ENSUING WAYPOINT [position] | N | L | N |
| 45 | Clarification of previously reported waypoint passage. | REPORTED WAYPOINT [position] | N | L | N |
| 46 | Clarification of time over previously reported waypoint. | REPORTED WAYPOINT [time] | N | L | N |
| 47 | The specified code has been selected | SQUAWKING [ code] | N | L | N |
| 48 | Position report. | POSITION REPORT [position report] | N | M | N |
| 79 | The code of the latest ATIS received is as specified. | ATIS [atis code] | N | L | N |
| 89 | The specified ICAO unit is being monitored on the specified frequency. | MONITORING [unitname] [frequency] | U | M | N |
| 102 | Used to report that an aircraft has landed. | LANDING REPORT | N | N | N |
| 104 | Notification of estimated time of arrival at the specified position. | ETA [position][time] | L | L | N |
| 105 | Notification of the alternative aerodrome for landing. | ALTERNATE AERODROME [airport] | L | L | N |
| 106 | Notification of the preferred level | PREFERRED LEVEL [level] | L | L | N |
| 109 | Notification of the preferred time to commence descent for approach | TOP OF DESCENT [time] | L | L | N |
| 110 | Notification of the preferred position to commence descent for approach | TOP OF DESCENT [position] | L | L | N |
| 111 | Notification of the preferred time and position to commence descent for approach | TOP OF DESCENT [time] position] | L | L | N |

Table 2.3.7-24.  Negotiation Requests (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 49 | Request for the earliest time at which a clearance to the specified speed can be expected. | WHEN CAN WE EXPECT [speed] | L | L | Y |
| 50 | Request for the earliest time at which a clearance to a speed within the specified range can be expected. | WHEN CAN WE EXPECT [speed] TO [speed] | L | L | Y |
| 51 | Request for the earliest time at which a clearance to regain the planned route can be expected. | WHEN CAN WE EXPECT BACK ON ROUTE | L | L | Y |
| 52 | Request for the earliest time at which a clearance to descend can be expected. | WHEN CAN WE EXPECT LOWER LEVEL | L | L | Y |
| 53 | Request for the earliest time at which a clearance to climb can be expected. | WHEN CAN WE EXPECT HIGHER LEVEL | L | L | Y |
| 54 | Request for the earliest time at which a clearance to cruise climb to the specified level can be expected. | WHEN CAN WE EXPECT CRUISE CLIMB TO [level] | L | L | Y |
| 87 | Request for the earliest time at which a clearance to climb to the specified level can be expected. | WHEN CAN WE EXPECT CLIMB TO [level] | L | L | Y |
| 88 | Request for the earliest time at which a clearance to descend to the specified level can be expected. | WHEN CAN WE EXPECT DESCENT TO [level] | L | L | Y |

Table 2.3.7-25.  Emergency Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 55 | Urgency prefix. | PAN PAN PAN | U | H | N |
| 56 | Distress prefix. | MAYDAY MAYDAY MAYDAY | D | H | N |
| 112 | Indicates specifically that the aircraft is being subjected to unlawful interference. | SQUAWKING 7500 | U | H | N |
| 57 | Notification of fuel remaining and number of persons on board. | [remaining fuel] OF FUEL REMAINING AND [persons on board] PERSONS ON BOARD | U | H | N |
| 58 | Notification that the pilot wishes to cancel the emergency condition. | CANCEL EMERGENCY | U | M | N |
| 59 | Notification that the aircraft is diverting to the specified position via the specified route. | DIVERTING TO [position] VIA [route clearance] | U | H | N |
| 60 | Notification that the aircraft is deviating the specified distance in the specified direction off the cleared route and maintaining a parallel track. | OFFSETTING [distance offset] [direction] OF ROUTE | U | H | N |
| 61 | Notification that the aircraft is descending to the specified level. | DESCENDING TO [level] | U | H | N |
| 80 | Notification that the aircraft is deviating from the cleared route by the specified distance in the specified direction. | DEVIATING [distance offset] [direction] OFF ROUTE | U | H | N |

Table 2.3.7-26.  System Management Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 62 | A system generated message that the avionics has detected an error. | ERROR [error information] | U | L | N |
| 63 | A system generated denial to any CPDLC message sent from a ground facility that is not the Current Data Authority. | NOT CURRENT DATA AUTHORITY | L | L | N |
| 99 | A system generated message to inform a ground facility that it is now the Current Data Authority | CURRENT DATA AUTHORITY | L | L | N |
| 107 | A system generated message sent to a ground system that tries to connect to an aircraft when a current data authority has not designated the ground system as the NDA. | NOT AUTHORIZED NEXT DATA AUTHORITY | L | L | N |
| 64 | Notification to the ground system that the specified ATSU is the current data authority. | [facility designation] | L | L | N |
| 73 | A system generated message indicating the software version number. | [version number] | L | L | N |
| 100 | Notification to the ground system that the aircraft system has received the message to which the logical acknowledgment refers. | LOGICAL ACKNOWLEDGMENT | N | M | N |

Table 2.3.7-27.  Additional Messages (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 65 | Used to explain reasons for aircraft operator’s message. | DUE TO WEATHER | L | L | N |
| 66 | Used to explain reasons for aircraft operator’s message. | DUE TO AIRCRAFT PERFORMANCE | L | L | N |
| 74 | States a desire by the aircraft operator to provide his/her own separation and remain in VMC. | REQUEST TO MAINTAIN OWN SEPARATION AND VMC | L | L | Y |
| 75 | Used in conjunction with another message to indicate that the operator wishes to execute request when the air crew is prepared to do so. | AT PILOTS DISCRETION | L | L | N |
| 101 | Allows the aircraft operator to indicate a desire for termination of CPDLC service with the current data authority. | REQUEST END OF SERVICE | L | L | Y |
| 103 | Allows the aircraft operator to indicate that he has canceled IFR flight plan. | CANCELLING IFR | N | L | Y |
| 108 | Notification that de-icing action has been completed. | DE-ICING COMPLETE | L | L | N |
| 67 |  | [free text] | N | L | N |
| 68 |  | [free text] | D | H | Y |
| 90 |  | [free text] | N | M | N |
| 91 |  | [free text] | N | L | Y |
| 92 |  | [free text] | L | L | Y |
| 93 |  | [free text] | U | H | N |
| 94 |  | [free text] | D | H | N |
| 95 |  | [free text] | U | M | N |
| 96 |  | [free text] | U | L | N |
| 97 |  | [free text] | L | L | N |
| 98 |  | [free text] | N | N | N |

Table 2.3.7-28.  Negotiation Responses (downlink)

|  | Message Intent/Use | Message Element | URG | ALRT | RESP |
| --- | --- | --- | --- | --- | --- |
| 81 | We can accept the specified level at the specified time. | WE CAN ACCEPT [level] AT [time] | L | L | N |
| 82 | We cannot accept the specified level. | WE CANNOT ACCEPT [level] | L | L | N |
| 83 | We can accept the specified speed at the specified time. | WE CAN ACCEPT [speed] AT [time] | L | L | N |
| 84 | We cannot accept the specified speed. | WE CANNOT ACCEPT [speed] | L | L | N |
| 85 | We can accept a parallel track offset the specified distance in the specified direction at the specified time. | WE CAN ACCEPT [distance offset] [direction] at [time] | L | L | N |
| 86 | We cannot accept a parallel track offset the specified distance in the specified direction. | WE CANNOT ACCEPT [distance offset] [direction] | L | L | N |

## Parameter Value Unit, Range, and Resolution

### A CPDLC-user shall interpret CPDLC message element variables unit, range, and resolution as defined in 2.3.4.

# SUBSETTING RULES

## General

- Note. — This chapter specifies conformance requirements which all implementations of the CPDLC protocol obey.

### An implementation of either the CPDLC ground based service or the CPDLC air based service claiming conformance to 2.3 shall support the CPDLC protocol features as shown in the tables below:

- Note. — The ‘status’ column indicates the level of support required for conformance to the CPDLC-ASE protocol described in this part.  The values are as follows:

| - ‘M’ | - mandatory support is required, |
| --- | --- |
| - ‘O’ | - optional support is permitted for conformance to the CPDLC protocol, |
| - ‘N/A’ | - the item is not applicable, and |
| - ‘C.n’ | - the item is conditional where n is the number which identifies the condition which is applicable. The definitions for the conditional statements used in this chapter are written under the tables in which they first appear. |

Table 2.3.8-1.  Protocol Versions Implemented

|  | Status | Associated Predicate |
| --- | --- | --- |
| Version 1 | M | none |

Table 2.3. 8-2.  CPDLC Protocol Options

|  | Status | Associated Predicate |
| --- | --- | --- |
| CPDLC-air-ASE | C.1 | CPDLC/air |
| CPDLC-ground-ASE | C.1 | CPDLC/ground |
| DSC function supported | if (CPDLC/air) O, else M | DSC-FU |
| DSC function supported by CPDLC-ground-user | if (CPDLC/ground) O, else N/A | DSC-USER |
| Forward function supported by initiating user | if (CPDLC/ground) O, else N/A | FWD-INIT |
| Forward function supported by receiving user | if (CPDLC/ground) O, else N/A | FWD-USER |

C.1: a conforming implementation will support one and only one of these two options.

- Table 2.3.8-3.  CPDLC-air-ASE Conformant Configurations

|  | - List of Predicates | - Functionality Description |
| --- | --- | --- |
| - I. | - CPDLC/air | - a CPDLC-air-ASE supporting just the core* CPDLC functionality (no downstream clearance capability) |
| - II. | - CPDLC/air + DSC-FU | - a CPDLC-air-ASE supporting the core CPDLC functionality and the downstream clearance capability (complete CPDLC-air-ASE functionality) |
|  |  | - * the core CPDLC functionality is defined as support for the CPDLC-start, message, end services plus abort services. |

- Table 2.3.8-4.  CPDLC-ground-ASE Conformant Configurations

|  | - List of Predicates | - Functionality Description |
| --- | --- | --- |
| - I. | - CPDLC/ground | - a CPDLC-ground-ASE supporting the core CPDLC functionality plus: - functionality for receiving and CPDLC-ground-user rejecting a request for a DSC dialogue - functionality for receiving and indicating that the forward function is not supported |
| - II. | - CPDLC/ground + DSC-FU + DSC-USER | - a CPDLC-ground-ASE supporting the core CPDLC functionality and downstream clearance function plus - functionality for receiving and indicating that the ground forward function is not supported |
| - III. | - CPDLC/ground + DSC-FU + FWD-INIT | - a CPDLC-ground-ASE supporting the core CPDLC functionality plus: - functionality for receiving and CPDLC-ground-user rejecting a request for a DSC dialogue - functionality for receiving and indicating that the ground forward function is not supported - functionality for supporting the capability to initiate the ground forwarding of CPDLC messages |
| - IV. | - CPDLC/ground + DSC-FU + DSC-USER + FWD-INIT | - a CPDLC-ground-ASE supporting the core CPDLC functionality and downstream clearance function plus - functionality for receiving and indicating that the ground forward function is not supported - functionality for supporting the capability to initiate the ground forwarding of CPDLC messages |
| - V. | - CPDLC/ground + DSC-FU + FWD-USER | - a CPDLC-ground-ASE supporting the core CPDLC functionality plus: - functionality for receiving and CPDLC-ground-user rejecting a request for a DSC dialogue - functionality for CPDLC-ground user support for the receipt of CPDLC ground forward messages |
| - VI. | CPDLC/ground + DSC-FU + DSC-USER + FWD-USER | - a CPDLC-ground-ASE supporting the core CPDLC functionality and downstream clearance function plus - functionality for CPDLC-ground user support for the receipt of CPDLC ground forward messages |
| - VII. | - CPDLC/ground + DSC-FU + FWD-INIT + FWD-USER | - a CPDLC-ground-ASE supporting the core CPDLC functionality plus: - functionality for receiving and CPDLC-ground-user rejecting a request for a DSC dialogue - full CPDLC ground forwarding functionality (initiating and user receiving) |
| - VIII. | - CPDLC/ground + DSC-FU + DSC-USER + FWD-INIT + FWD-USER | - a CPDLC-ground-ASE supporting the core CPDLC functionality and downstream clearance function plus - full CPDLC ground forwarding functionality (initiating and user receiving) (complete CPDLC-ground-ASE functionality) |

Table 2.3. 8-5.  Supported CPDLC Service Primitives

|  | Sending (req,[cnf]) | Receiving (ind, [rsp]) |
| --- | --- | --- |
| CPDLC-start service | M | M |
| CPDLC-message service | M | M |
| CPDLC-end service | if (CPDLC/ground) M, else N/A | if (CPDLC/air) M, else N/A |
| DSC-start service | if (CPDLC/air and DSC-FU) M, else N/A | if (CPDLC/ground) M, else N/A |
| DSC-end service | if (CPDLC/air and DSC-FU) M, else N/A | if (CPDLC/ground and DSC-USER) M, else N/A |
| CPDLC-forward service | if (FWD-INIT) M, else N/A | if (FWD-USER) M, else N/A |
| CPDLC-user-abort | M | M |
| CPDLC-provider-abort | N/A | M |

## Table 2.3. 8-6.  Supported CPDLC APDUs

|  | Sender | Receiver |
| --- | --- | --- |
| [GroundPDUs] startup | if (CPDLC/ground) M, else N/A | if (CPDLC/air) M, else N/A |
| [GroundPDUs] send | if (CPDLC/ground) M, else N/A | if (CPDLC/air) M, else N/A |
| [GroundPDUs] forward | if (FWD-INIT) M, else N/A | if (CPDLC/ground) M, else N/A |
| [GroundPDUs] forwardresponse | if (CPDLC/ground) M, else N/A | if (FWD-INIT) M, else N/A |
| [GroundPDUs] abort | if (CPDLC/ground) M, else N/A | if (CPDLC/air or FWD-INIT) M, else N/A |
| [AircraftPDUs] startdown | if (CPDLC/air) M, else N/A | if (CPDLC/ground) M, else N/A |
| [AircraftPDUs] send | if (CPDLC/air) M, else N/A | if (CPDLC/ground) M, else N/A |
| [AircraftPDUs] abort | if (CPDLC/air) M, else N/A | if (CPDLC/ground) M, else N/A |