|  | ATNP/WG3/SG3 (Architecture) Version DRAFT 1.1 (Word 6) 22 June 1998 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Sub Group 3 (Upper Layer Architecture)

ATN Systems Management

Sub-Volume 6 of ATNP Manual

Prepared by:		ATNP/WG3/SG3

SUMMARY

This is the draft ATN Systems Management provision for inclusion as Sub-Volume 6 of the ICAO Manual prepared by the ICAO Aeronautical Telecommunication Network Panel (ATNP).

This working draft is an update based on comments from WG3 at its meeting in Rio in March 1998.  It should be noted that the whole area of MIB standardisation is under review, and the provisions in this draft may change fundamentally when the Concept of Operations for ATN systems management stabilises.  The Working Group is invited to review this document and to provide comments for inclusion in the next version.

CONFIGURATION SHEET

Title :		ATN Systems Manament -

Sub-Volume 6 of ATNP Manual

Version :	DRAFT 1.1 (Word 6)

Date:		22 June 1998

Contact:	Sub-Volume editor:  Tony.Kerr@level-7.co.uk

SG3 chairman:  Stephen.Van.Trees@faa.dot.gov

Status:		Draft

Change History :

| Version | Description | Affected Parts | Date |
| --- | --- | --- | --- |
| 0.1 | Initial outline for SG3 review | All | 07/10/97 |
| 0.2 | Minor updates from WG3/SG3 Toulouse meeting. Presented at WG3-11 Redondo Beach, October 1997 | All | 24/10/97 |
| 1.0 | First substantive version. Input to WG3-12, Rio de Janeiro, March 1998 | All | March 1998 |
| 1.1 | Updated working draft incorporating editing instructions from WG3-12. Input to WG3-13, Utrecht, June 1998 | All | June 1998 |

Preface

This working draft has been formatted as Sub-Volume 6 of the detailed ATN technical provisions.  For this reason, section numbering starts at 6.0.

This draft represents work in progress within the ICAO ATNP Working Groups and should not be taken as a stable set of requirements.  It should be noted that the whole area of MIB standardisation is under review, and the provisions in this draft may change fundamentally when the Concept of Operations for ATN systems management stabilises.

This draft is based on the following assumptions:

- that System Management (including in scope both Network Management, Applications and higher level functions) will be essential for world-wide ATN operation

- - that cross-domain management will be required, and therefore SARPs are required to ensure interworking between management domains.  Within domains, system management is a local issue.

- that system management data traffic will flow over the air-ground data link, if not in the short term then at some time in the future.  The management protocol must therefore not preclude such traffic.

- that a flexible, extensible System Management infrastructure is needed, as it is not possible to predict all future System and Network Management scenarios.

- that a Concept of Operations for ATN Systems Management will be defined, and this will specify the operational requirements more closely.

This Working Draft is structured such that the draft technical provisions are presented in the style of ICAO SARPs in the first part of the document, and the draft technical Guidance Material is presented in the second part (section 6.10 onwards).  As this document reaches maturity, the Guidance Material will be removed for inclusion in the Comprehensive ATN Manual (CAMAL).  Note that the Guidance Material included here is technical material which relates directly to the technical provisions - it does not include the Operational Requirements nor the Concept of Operations, which are assumed to be defined elsewhere.

Cross-references:

[1] Draft Sub-Volume 1 and Core Guidance Material

[2] Draft ATNSI CONOPS

[3] Draft ACI/ProATN Convergent MIB

Table of Contents

6.1. INTRODUCTION

6.1.1 Scope and Objectives

6.1.2 Structure of ATN Systems Management Specification

6.1.3 Systems Management Functionality

6.2. NAMING AND ADDRESSING PROVISIONS

6.3. ATN SYSTEMS MANAGEMENT COMMUNICATION PROFILE

6.3.1 Introduction

6.3.2 The SM Application Model

6.3.3 Profile for SM Stack

6.3.4 Encoding Requirements

6.3.5 Mapping to Dialogue Service

6.4. MANAGER-TO-MANAGER COMMUNICATIONS PROFILE

6.5. SYSTEMS MANAGEMENT PROFILES FOR MANAGEMENT FUNCTIONS

6.6. MANAGEMENT INFORMATION GENERAL PROVISIONS

6.6.1 Symbols, abbreviations and terms

6.6.2 Global Containment Tree for One System

6.6.3 “System” MO Classes

6.7. MANAGED OBJECTS FOR ATN APPLICATIONS

6.7.1 Summary of managed object classes

6.7.2 Containment hierarchy

6.7.3 The applicationSubsystem managed object

6.7.4 The aTNcMae managed object

6.7.5 The aTNcMaeInstance managed object

6.7.6 The aTNaDSae managed object

6.7.7 The aTNaDSaeInstance managed object

6.7.8 The aTNaRFae managed object

6.7.9 The aTNaRFaeInstance managed object

6.7.10 The aTNcPDLCae managed object

6.7.11 The aTNcPDLCaeInstance managed object

6.7.12 The aTfISae managed object

6.7.13 The aTNfISaeInstance managed object

6.7.14 Managed Objects for Systems Management Applications

6.8. MANAGED OBJECTS FOR ATN UPPER LAYERS

6.9. MANAGED OBJECTS FOR ATN ICS

6.9.1 Introduction

6.9.2 Elements of Management Information for ATN Transport Layer

6.9.3 Summary of managed objects

6.9.4 Containment hierarchy

6.9.5 The aTNtransportSubsystem managed object

6.9.6 The aTNtransportEntity managed object

6.9.7 The aTNclmodeTPM managed object

6.9.8 The aTNcomodeTPM managed object

6.9.9 The aTNtransportConnection managed object

6.9.10 Elements of Management Information for ATN Network Layer

6.9.11 Summary of managed objects

6.9.12 Containment hierarchy

6.9.13 The aTNnetworkSubsystem managed object

6.9.14 The aTNnetworkEntity managed object

6.9.15 The aTNcLNS managed object

6.9.16 The aTNlinkage managed object

6.9.17 The aTNmobileConnection managed object

6.9.18 The aTNfIB managed object

6.9.19 The aTNidrpConfig managed object

6.9.20 The aTNadjacentBIS managed object

6.9.21 The aTNrIB managed object

6.9.22 The aTNiSSME managed object

6.9.23 The aTNmobileAdjacency managed object

6.9.24 The x25PLE-DTE managed object

6.9.25 The virtualCall-DTE managed object

6.10. GUIDANCE MATERIAL FOR ATN SYSTEMS MANAGEMENT

6.10.1 Communications Profile

6.10.2 Interworking Provisions

6.10.3 Dual-Stack Managers

6.10.4 Integration of Manager Products with ATN Agents

6.10.5 Rationale for ATN Systems Management Architecture

6.10.6 The ATN Upper Layer Stack

6.10.7 Porting OSI SM Protocols to the ATN Upper Layer Environment

6.10.8 Conclusion and Recommendations

6.10.9 Comparison Between ISP 11183-1 and the ATN Upper Layer Profile

6.10.10 Alternative Internal Architectures for the System Management Application

6.11. REFERENCES

6.12. Issues / Work In Progress

6.0	ATN SYSTEMS MANAGEMENT PROVISIONS

# INTRODUCTION

Editor’s note.— WG3/SG3 was tasked with producing validated SARPs and Guidance for ATN Systems Management before ATNP/3.  Discussions in WG1 and WG3 sub-groups identified the need for a separate Sub-Volume dedicated to Systems Management provisions.

Note.— Chapter 1 contains introductory material and an overview of the Sub-Volume structure.  There are no requirements or recommendations (shalls or shoulds) in this chapter.

## Scope and Objectives

The minimum requirements for ATN systems management are specified here.

## Structure of ATN Systems Management Specification

There are several distinct aspects to the ATN systems management requirements specified here.  This specification is structured as follows:

- Introduction (6.1) describes the purpose and structure of the ATN Systems Management specification, and the background to the functionality defined herein.

- Naming and Addressing Provisions (6.2) specifies the requirements for navigating the Management Information Base and identifying particular attributes within individual Managed Object (MO) instances, or groups of MOs.  It also assigns Application Entity Qualifiers to systems management applications.

- ATN Systems Management Communication Profile (6.3) specifies provisions for an efficient profile for general ATN systems management (Manager to Agent) communications, using the ULCS and ICS communication services.

- Manager-to-Manager Communications Profile (6.4) specifies the architecture and provisions for peer to peer Manager to Manager communications including “bulk or file transfer”.

- Systems Management Profiles for Management Functions (6.5) specifies the provisions for Systems Management Application functionality required in ATN systems to support Performance assessment, Accounting information and Fault detection (with Configuration and Security support as needed).  The scope includes secure systems management application exchanges and access control to systems management resources.

- Management Information General Provisions (6.8) specifies common provisions and conventions which are used in 6.7, 6.8 and 6.9.

- Managed Objects for ATN Applications (6.7) specifies the provisions for MOs for ground-ground and air-ground ATN Applications, including provisions for MOs for ATN systems management applications (CMIP).

- Managed Objects for ATN Upper Layers (6.8) specifies the provisions for MOs for ATN upper layers (Session, Presentation, ACSE, ATN-App-AE).

- Managed Objects for ATN ICS (6.9) specifies the provisions for the set of MOs required for management of the ATN Internet Communications Service.

Editor’s note.— MO specifications may move to the Sub-Volume which specifies the relevant functionality, i.e. ICS MOs would belong in Sub-Volume 5, but for now they are specified here as a convenient repository which does not destabilise existing SARPs.

Editor’s note.— This working draft also includes draft guidance material in sections 6.10 onwards.  This is mostly technical guidance rather than operational.  It includes a description of requirements for accommodating and interworking with various other systems management approaches that may be adopted internally within management domains.  One aspect of this is the specification of standard APIs which are independent of any particular technology.

## Systems Management Functionality

ATN systems management is based on the ISO/IEC and ITU-T international standards for OSI management.

Editor’s note.— This section will provide an overview of ATN systems management functionality, i.e. the management framework and what can be exchanged between Manager and Agent Processes.  Such an overview currently resides in the draft Guidance Material for Core/SV1 Systems Management - WG1-10 WP 16.

# NAMING AND ADDRESSING PROVISIONS

Editor’s note.— To be defined.  This chapter will contain SARPs text on Managed Object addressing and registration requirements.  It will specify the requirements for navigating the Management Information Base and identifying particular attributes within individual Managed Object (MO) instances, or groups of MOs.  It also assigns Application Entity Qualifiers to systems management applications.  Presentation context identifiers also need to be assigned.

# MANAGER-TO-MANAGER COMMUNICATIONS PROFILE

Note 1.—  There are requirements for Manager applications in different management domains to be able to exchange management information on a peer-to-peer basis.  This chapter specifies the architecture and requirements for peer to peer Manager to Manager communications including “bulk or file transfer” based on existing provisions for ULCS (4), ICS (5) and/or AMHS (2.3.2).

Editor’s note.—  Discussions to date suggest that there are no special profile requirements for Manager to Manager communications.  The lightweight profile defined in 6.3 will fulfil all requirements, with one of the managers taking a “supra-manager” role and the other taking the Agent role for a given instance of communication.

The information exchanged between Managers is likely to be limited to cross-domain statistical or aggregate information e.g. for accounting purposes.  Work is ongoing to define MOs to support this level of communication, in the context of a Concept of Operations for ATN System Management.

Low volume Manager-to-Manager communication shall be acheived by one of the Managers adopting the Agent role for a particular interchange.  Thus Manager implementations shall support both Manager and Agent roles.

Editor’s note.—  The details of the format and content of the management information to be exchanged are not yet known, and in any case are likely to evolve over time.  The requirement is therefore for a flexible, general-purpose interchange mechanism, which will allow manager applications to identify the information content and take appropriate action depending upon procedures which will be defined as required.

The protocols and MOs used by commercial implementations (e.g. H-P OpenView and Bull ISM) which implement Manager to Manager communications should be investigated.

It may be that a more general-purpose peer-to-peer communication mechanism is required, to enable the transparent interchange of management (or other) information between systems.

The only general-purpose end-to-end interchange protocol in the current ATN SARPs is the AMHS application.  However, this is a store-and-forward application, and is based on "full" OSI upper layers and Basic Encoding Rules.

There may therefore be a future requirement to specify an efficient lightweight protocol, making use of the efficient ULCS profile, for the interchange of management information.  Indeed, the protocol would be general-purpose and not restricted to management operations.

Such a service could be based on the existing Dialogue Service specified in the ULCS SARPs.  It would entail the specification of a simple pass-through ASE, and definition of the service offered to the AE-User.

Figure 3-1.  ASE for Peer-to-Peer Communications

High volume Manager-to-Manager communication shall be acheived by a bulk transfer protocol

There is likely to be a requirement for a bulk transfer protocol, for example to transfer log files to a management application, or to download configuration files to a managed system.  Such a protocol should be highly reliable, allow interruptions by users, and run in the background with priority such as not to interfere with other ATN usage (except in the case of management operations critical to the correct functioning of the ATN).  Again, AMHS might provide the only solution required.  Alternatively, there are numerous standard bulk transfer mechanisms, including well-proven file transfer protocols such as FTAM and FTP.  A profile to map one of these protocols to the ATN transport service could be developed.  There are no plans to do this at present.

# SYSTEMS MANAGEMENT PROFILES FOR MANAGEMENT FUNCTIONS

Note 1.—  This chapter contains provisions for the Systems Management Application functionality required in ATN systems (standard ISO 10164 or other) required to support Performance assessment, Accounting and Fault detection (with Configuration and Security support as needed) in ATN systems for Manager to Manager and Manager to Agent.

Editor’s note.—  International standardised profiles (ISPs) exist for OSI systems management functions.  The AOM 2xx profiles should be analysed in the context of requirements (currently BIS/ES/Subnet).  It is necessary to assess the suitability of these profiles to satisfy identified functional requirements for ATN systems management, and to select those profiles necessary to support such requirements.

It is also required to develop SARPs for secure Systems Management application exchanges and access control to Systems management resources (e.g. applicability of Access control 10164-9 Managed Objects for access control).

# MANAGED OBJECTS FOR ATN ICS

Note 1.— For OSI lower layers, there exist a number of international standards which specify MOs for layer management.  These standardised MOs have been adapted and extended for ATN management.

Editor’s note.— The MOs defined here may move to the ICS Sub-Volume in due course, but for now this chapter serves as a "home" for these systems management requirements without disrupting the CNS/ATM-1 SARPs.

## Introduction

# Issues / Work In Progress

- Is it really necessary to standardise MOs at the level given in this working draft?  It may be desireable to adopt a common framework in order to reduce procurement and deployment costs, but should States be mandated to build the specified GDMO MIB?  The only MOs essential to standardise are those used in management exchanges between administrative domains.  Such communication is likely to be “manager-to-manager”.

- Statistical / aggregate MOs for Manager - Manager communication need to be defined.  Dependency on the immature CONOPS work.  WG1/SG3/WP6-4 “Operational Concepts on System Management for the European ATN” by S.Tamalet makes a start on this topic at a high level.

- None of the MOs defined to date can be considered as stable.  Detailed technical review is needed.  Also consistency check with ACI/ProATN Convergent MIB.

- There may be significant bandwidth savings if the CMIP APDUs were augmented with PER-visible constraints and extensibility markers.  The resulting abstract syntax would be input to the ISO/IEC and ITU standardisation process.  Studies of encoded CMIP PDUs are in progress.

- MO classes related to ground-ground applications and systems management application are not currently specified in this document.

- Does there need to be a separate containment tree per class of Router?

- PDR 97120001 - how to name multiple Agents in an End System, or any Agent in a non-ICAO named location?

- What does the distinguished name of “system” look like?

- How should the requirements for “full stack” management, such as ATSMHS be documented. ATSMHS will make use of ISO standards and profiles (see WG3/IP12-06)
For example: 
ISO/IEC 11588-3 Information technology - Message Handling Systems (MHS) management - Part 3: Logging information
ISO/IEC 11588-8 Information technology - Message Handling Systems (MHS) management - Part 8: Message Transfer Agent management.

- What are the requirements for subnetwork management - there are no MOs currently defined at this level.

- Coding examples of CMIP / ROSE / PER to be developed for a typical CMIP exchange, for Guidance Material.

- Some MOs for Security are specified in WG3/WP 12-25 “ATN Upper Layers Security” by G. Mittaux-Biron.  These need to be incorporated into SV6 once stable.