# ATN SYSTEMS MANAGEMENT COMMUNICATION PROFILE

## Lightweight CMIP Profile

Note 1.— This chapter specifies requirements for an efficient CMIP profile for general ATN systems management (Manager to Agent) communications.  It is not applicable to “full stack” applications such as ATSMHS, where a conventional full stack CMIP profile is more appropriate.

Note 2.— For efficient use of air-ground data links, and to avoid multiple protocol stacks in ATN systems, the CMIP profile is based on the ULCS and ICS SARPs.  The profile specified here references the international standardised profile (ISP) AOM 12, modified to take account of the null-encoding session and presentation layer protocols, and ACSE APDUs encoded for transfer using the Packed Encoding Rules of ASN.1.

Note 3.— The protocol profile  includes Transport and lower layers, and this is required to be ICS SARPs compatible.  ATN-specific transport layer parameters are specified (traffic type, communications class, transport priority and integrity requirements).

## The SM Application Model

Most of the existing ATN application SARPs were specified to make use of the Dialogue Service, which is defined in the ULCS SARPs.  The Dialogue Service hides the ACSE and Presentation services from the application ASEs, and is provided by the control function (CF).  The "Lower CF," which supports the Dialogue Service, is fully specified in the ULCS SARPs.

The architecture, as applied to ATN systems management, is illustrated in Figure 6.3-1.

Figure 6.3-1.   Use of the CNS/ATM-1 Dialogue Service

The CF provides a "pass-through" service to the SM Application User, allowing the SM Application User to invoke CMISE services and also to establish and release associations.

For modelling purposes, a "conceptual SM ASO" envelopes CMISE and ROSE and invokes association establishment and termination services on behalf of the Application User.  Thus, the Dialogue service is used to establish, release and abort the application-association with the peer SMAE and to exchange SM information when requested by CMISE and ROSE.  The CMIS service is provided unchanged to the SM-users as part of the SM service.

The CMIS standard states that the user of the CMISE service uses ACSE services for the establishment and release of associations.  The CF maps such ACSE service invocations by the CMISE user onto appropriate Dialogue Service requests.  For example, when the CMISE user requests an association, the CF constructs the A-ASSOCIATE user information, adds the requiored D-START parameters, and invokes the D-START service.  Thus, from the point of view of CMISE / ROSE, an “implicit start” service is provided.

The actions of the "conceptual SM ASO"are specified in Sub-Volume 6 of the ATN SARPs, in particular the actions to handle the primitive exchanges:

- between the SM-users and CMISE,

- between the SM-users and the Dialogue Service Provider, and

- between ROSE and Dialogue Service Provider.

In particular, the specification is responsible for re-mapping the Presentation service primitives (P-DATA request and indication) used by ROSE at its lower interface to the Dialogue service interface, and also for mapping ACSE service invocations by the CMISE user onto appropriate Dialogue Service requests.

The provisions in section 6.3 equate to the specification of the "conceptual SM ASO".

The definition of the "conceptual SM ASO" is required only for modelling purposes.  It avoids any need to modify the existing ULCS SARPs, which assume a one-to-one service mapping between the application ASE and the AE.  The SM-User is provided with a service consisting of all the CMIS service primitives, plus a pass-through to the D-START, D-END and D-ABORT services.  There is no requirement to implement any physical entity corresponding to the "conceptual SM ASO."

Note.— The main problem of this architecture when used for the ATN air-ground application specifications was the induced complexity of the App-ASE protocol, because the states of the underlying dialogue (e.g. pending establishment, established, pending release, collision) should be handled by the ASE protocol itself.  For the SMAE, CMISE and ROSE assume the association established and invoke only a data transfer primitive.  This problem is therefore not encountered for the specification of the SM application.

## Profile for SM Stack

### The complete communication requirements between Manager and Agent for ATN systems management shall be as specified here, taken together with the profile defined in the ULCS SARPs and the connection-mode Transport service defined in the ICS SARPs.

### The complementary communications interactions between CMISE-service-users within two end Management Information systems, with scope as shown in Figure 6.3-2, shall comply with the provisions specified here.

Figure 6.3-2.  Scope of the SM Communications Profile

### The supporting stack shall be as specified in the international standards indicated in Table 6.3-1, subject to the constraints and options specified in this profile.

Table 6.3-1.  Profile supporting stack

| Application Layer | ISO 9595, 9596-1 (CMIS, CMIP v2) ISO 9072-1, 9072-2 (ROSE) ISO 8649, 8650-1 Amd.1 (ACSE ed.2) ULCS SARPs (CF, encoding) |
| --- | --- |
| Presentation Layer | ISO 8822, 8823-1 Amd.1 (Service, "Fast Byte" protocol) ISO 8824, 8825-2 (ASN.1, PER) |
| Session Layer | ISO 8826, 8327-1 Amd.1 (Service, "Fast Byte" protocol) |

### The communication profile for ATN system management shall be as specified in ISO/IEC ISP 11183-2, with modifications as specified in Table 6.3-2.

Table 6.3-2 - Modifications to ISP 11183-2

| ISP 11183-1 clause | Modification for ATN Profile |
| --- | --- |
| 1.5, Table 1 | replace the table with Table 6.3-1 in this document |
| 5 (Conformance to AOM 12) | replace "and also in ISO/IEC ISP 11183-1" with "and also in the ULCS SARPs profile" |
| 5.1 | delete "ISO/IEC ISP 11183-1 and". |
| 5.3 | delete Note 2. |
| 5.3 | delete final paragraph. |
| 5.4 | replace "shall support the mapping of ROSE APDUs only onto the P-DATA Presentation service" with "shall support the mapping of ROSE APDUs only onto the D-DATA Dialogue service". |
| A.1 | This clause allows non-conformant implementations to list the non-supported mandatory capabilities. For the SM SARPs, non-compliance is not permitted. Therefore the following provision is required: "All mandatory capabilities of ISO/IEC ISP 11183-2 as modified here shall be implemented." |
| A.2.1 | replace "association" with "dialogue" throughout this clause, as the CMISE services are mapped to the ULCS Dialogue service, and not directly to ACSE. |
| A.2.3, Table A.3 | replace "(in AARQapdu)" with "(in D-START Request and Indication User-Data)". |
| A.2.3, Table A.4 | replace "(in AAREapdu)" with "(in D-START Response and Confirmation User-Data)". |
| A.2.3, Table A.3 and A.4 | redefine profile support for userInfo parameter in CMIPUserInfo as "out of scope". |
| Clause A.2.4, Table A.5 | profile support for userInfo parameter in CMIPAbortInfo is changed to "out of scope". |
| Clause A.3.2, Table A.13a and A.13b, index A.13a.1, A.13a.2, A.13b.1, A.13b.2 | replace "See ISP 11183-1, 8.3" with "(3)", and insert new note after table: (3) A sender shall not encode values of greater than 2**31-1 or less than -2**31. A receiver shall be able to decode at least values in the range -2**31 to 2**31-1. |
| Clause A.3.2, Table A.13a, index A.13a.10, A.13a.11, A.13a.12 | profile support of the INTEGER form of actionType, attributeId and eventType is changed from "i" to "m". |
| Table A.123 and A.124 | delete note referring to ISP 11183-1. |

Note 1.— Access Control parameters are outside the scope of this profile.

Note 2.— The format of the information contained in the CMIP PDUs "userInfo" in CMIPUserInfo and CMIPAbortIInfo definitions is outside the scope of AOM 12.

## Encoding Requirements

### The abstract syntax of the management information conveyed in CMIP PDUs shall be as defined in the ATN MIB specification.

### The encoding of management information for interchange shall be realised using the basic, unaligned variant ASN.1 Packed Encoding Rules.

### Implementations shall support the transfer syntax derived from the encoding rules specified in ISO/IEC 8825-2 and named { joint-iso-itu-t asn1 (1) packed-encoding (3) basic (0) unaligned (1) } for the purpose of generating and interpreting CMIP PDUs as defined in ISO/IEC 9596-1 by the abstract syntax "CMIP-PCI".

Note.— The above requirement is equivalent to specifying that all CMISE and ROSE APDUs are encoded using the basic, unaligned variant ASN.1 Packed Encoding Rules.  It replaces the requirement in clause 8.1 of ISO/IEC 9596-1 that "the implementation shall support the transfer syntax derived from the encoding rules specified in ISO/IEC 8825 and named { joint-iso-ccitt asn1(1) basic-encoding(1) } for the purpose of generating and interpreting CMIP PDUs as defined by the abstract syntax "CMIP-PCI".

## Mapping to Dialogue Service

### ROSE service primitives shall map to the D-DATA request / indication primitives of the Dialogue Service defined in the ULCS SARPs.

Note.— The above requirement replaces the ISO 9072-2 mapping to P-DATA request / indication primitives.

### When a CMISE-service-user requires to open an association for the exchange of CMISE / ROSE APDUs, the following sequence of events shall occur:

- A connection is established using the D-START service (and not the A-ASSOCIATE service as specified in ISO/IEC 9596-1).  The CMIPUserInfo maps to the D-START request User Data.

- On receiving a D-START indication containing User Data, the peer CMIPM and CMISE-service-user analyse the CMIPUserInfo as specified in ISO/IEC 9596-1 A.2.2.

- If the dialogue parameters are acceptable, the receiving CMISE-service-user and CMIPM construct the CMIPUserInfo required for the response and invoke a positive D-START response primitive, with the CMIPUserInfo as User-Data.

- If the dialogue parameters are not acceptable, the receiving CMISE-service-user and/or CMIPM invoke a negative D-START response primitive, with the constructed CMIPUserInfo, if any, as User-Data.

- If the initiating CMISE-service-user receives a negative D-START confirmation, no association has been established.

- If the initiating CMISE-service-user receives a positive D-START confirmation, an association has been established and the peer CMISE-service-users can exchange management protocol data units.

### When a CMISE-service-user requires the orderly termination of an association betwewen peer application entities, the following sequence of events shall occur:

- A D-END request primitive is invoked by the release initiator.

- On receiving a D-END indication, the release responder invokes a positive D-END response, which will close the connection.

- On receiving a positive D-END confirmation, the association ceases to exist.

### When a CMISE-service-user requires the abrupt termination of the association betwewen peer application entities, the following sequence of events shall occur:

- A D-ABORT request primitive is invoked by the release initiator, with the Originator parameter set to “User” and no User Data parameter.

- On receiving a D-ABORT indication, the Abort indication with Originator parameter is passed to the CMISE-User.

- The association ceases to exist.

### When the association betwewen peer application entities is terminated by the loss of the underlying communications connection, the following sequence of events shall occur:

- On receiving a D-P-ABORT indication, the Abort indication is passed to the CMISE-User.

- The association ceases to exist.

### Mapping to Dialogue Service Parameters

#### The D-START Routing Class QoS parameter shall be set as specified in Table 6.3-3.

Table 6.3-3

| Abstract Class of Communication | Routing Class Value (Hex) |
| --- | --- |
| ATN Systems Management Communications | 60 |

#### The D-START Priority QoS parameter shall be set as specified in Table 6.3-4.

Table 6.3-4

| Abstract Priority Value | QoS Priority Value (Decimal) |
| --- | --- |
| Network / Systems Management | 14 |