ATNP/CCB WP/11-__

ATNP/WG3 WP/18-32

29/11/99

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL(ATNP)

CONFIGURATION CONTROL BOARD – 11th meeting

WG 3 - Applications and Upper Layers – 18th meeting

Tokyo, 1st December – 3rd December 1999 (eighteenth meeting)

SME3 CCB Report

Presented by Jean-Marc Vacher (Sub-Volume 3 SME)

Summary

This paper provides a summary status of PDRs raised against the Sub-Volume III SARPs since the Phuket WGW/1 meeting.

Table of contents

1.	Introduction	2

2.	Summary of ATSMHS PDRs	2

3.	Summary of AIDC PDRs	3

4.	Recommendation	4

5.	ATTACHMENT A : Details of Sub-Volume 3 PDRs	4

________________

# Introduction

The goal of this paper is to provide the CCB / Working Group / Subgroup with the current status of the Sub-Volume 3 PDRs.

# Summary of ATSMHS PDRs

The following table lists all PDRs raised against the ATSMHS SARPs (Doc 9705, Sub-Volume 3, Chapter 1) since their approval at the Phuket ATNP WGW/1 meeting.

| Number | Name | Status (post CCB/9) | Comments |
| --- | --- | --- | --- |
| 97060014 | ATSMHS / use of implicit-conversion flag and EITS | RESOLVED | Included in Edition 1 |
| 97060015 | ATSMHS / conversion of forwarded messages | RESOLVED | Included in Edition 1 |
| 97060016 | ATSMHS / MHS priority and ATS-Message-Priority mismatch | RESOLVED | Included in Edition 1 |
| 97060017 | ATSMHS / prohibited character check in converted AMHS messages | FORWARDED | to be addressed in Edition 2 of Guidance Material |
| 97060018 | ATSMHS / erroneous cross-references to Sub-Volume 5 | RESOLVED | Included in Edition 1 |
| 97060019 | ATSMHS / recommendation on report generation | RESOLVED | Included in Edition 1 |
| 97100040 | ATSMHS / editorial corrections | RESOLVED | Included in Edition 2 |
| 98030005 | ATSMHS / Year 2000 dependency | RESOLVED | Included in Edition 2 |

# Summary of AIDC PDRs

The following table lists all PDRs raised against the AIDC SARPs (Doc 9705, Sub-Volume 3, Chapter 2) since their approval at the Phuket ATNP WGW/1 meeting.

| Number | Title | Status (post-CCB/9) | Comments |
| --- | --- | --- | --- |
| 97060020 | AIDC / Errors and inconsistencies in AIDC Abstract Definition | RESOLVED | Included in Edition 1 |
| 97060021 | AIDC / Provision of the Called ICAO Facility Designation when Info-transfer-request is invoked outside a dialogue | RESOLVED | Included in Edition 1 |
| 97060022 | AIDC / construction of the calling end-system AP-title | RESOLVED | Included in Edition 1 |
| 97060023 | AIDC / ASN.1 correction : Airport | RESOLVED | Included in Edition 1 |
| 97060024 | AIDC / sequencing table inconsistent with state tables | RESOLVED | Included in Edition 1 |
| 97100004 | AIDC / Definition of releaseIndicator | RESOLVED | Included in Edition 1 |
| 97100005 | AIDC / reference error in SARPs | RESOLVED | Included in Edition 1 |
| 97100015 | CPDLC/AIDC Airway Name | RESOLVED | Included in Edition 1 |
| 97100016 | CPDLC/AIDC VHF Frequency/ Frequencyvhfchannel | RESOLVED | Included in Edition 1 |
| 97100027 | AIDC / Year 2000 dependency | RESOLVED | Included in Edition 1 |
| 97100033 | AIDC / ASN.1 message type and Abort issue | RESOLVED | Included in Edition 1 |
| 97100042 | AIDC / AIDC Abstract Definition | RESOLVED | Included in Edition 1 |
| 97100043 | AIDC / Transfer Control Information parameter | RESOLVED | Included in Edition 1 |
| 97100044 | AIDC / Departure Airport | RESOLVED | Included in Edition 1 |
| 97100045 | AIDC / AircraftIdentification | RESOLVED | Included in Edition 1 |
| 97100046 | AIDC / BeaconCode | RESOLVED | Included in Edition 1 |
| 98030002 | AIDC / Control Function | RESOLVED | Included in Edition 1 |
| 98030003 | AIDC / Control Function | RESOLVED | Included in Edition 1 |
| 98040005 | all / ICAO V2.2 problems | RESOLVED | Included in Edition 1 |
| 98050019 | CPDLC/AIDC problems with ICAO V2.2 CPDLC SARPs | RESOLVED | Included in Edition 1 |

| 98090006 | AIDC / AIDC UCF Indication | WITHDRAWN |  |
| --- | --- | --- | --- |
| 98090009 | AIDC / AIDC PM variable names | RESOLVED | Included in Edition 2 |
| 99080002 | AIDC / Transfer Control protocol states | REJECTED |  |
| 99080003 | AIDC / Provider Abort indication parameters | RESOLVED | For inclusion in Edition 3 |
| 99100001 | AIDC / Bad started timer | Rec. : ACCEPT/ RESOLVE | For inclusion in Edition 3 |
| 99100002 | AIDC / Coord-Start Service Bad vr1/vs1 | Rec. : ACCEPT/ RESOLVE | For inclusion in Edition 3 |
| 99110001 | AIDC / Bad timers 1CT/2CT | Rec. : ACCEPT/ RESOLVE | For inclusion in Edition 3 |
| 99110002 | AIDC / Coord-end incomplete text | Rec. : ACCEPT/ RESOLVE | For inclusion in Edition 3 |
| 99110003 | AIDC / Latitude Semantics | Rec. : | For inclusion in Edition 3 |
| 99110004 | AIDC / ASN.1 of FrequencyVHF | Rec. : REJECT |  |

# Recommendation

The CCB is invited to note the information provided.

# ATTACHMENT A : Details of Sub-Volume 3 PDRs

PDR 99100001

PDR 99100002

PDR 99110001

PDR 99110002

PDR99 110003

PDR 99110004

Title : ICAO 9705 - Bad started timer

PDR Reference:                            99100001

Originator Reference:                     -

SARPs Document Reference:

SARPs Sub_Volume III Status:              SUBMITTED

Impact:                                   B (Bug)

PDR Revision Date:

PDR Submission Date:                      18/10/99

Submitting State/Organization:            CENA/CHARME

Project Submitting Author Name:           Mathieu JEAN

Submitting Author E-mail Address:         jean@cenatls.cena.dgac.fr

Submitting Author Supplemental

Contact Information:

SARPs Date:                               ICAO 9705 Edition 2

SARPs Language:                           English

Summary of Defect:

In the section 3.2.6.1.33.2.1 b) 2) i) C), there is the sentence :

if the variable vr1=info-trans, then start the timer t1NC.

I think it is a mistake. According to the others sections : 3.2.6.1.33.2.1 b) 3) C), 3.2.6.1.34.2.1 c) 1) i) C), 3.2.6.1.34.2.1 c) 2) i) C),

when vr1 or vs1 have the value info-trans, it is always an Info-Transfer timer (t1IN or t2IN) which is started and not a Notifying-Coordinating timer (t1NC).

Assigned SME:           Sub-Volume III SME

Proposed SARPs amendment:

1) section 3.2.6.1.33.2.1 b) 2) i) C), replace :

"if the variable vr1=info-trans, then start the timer t1NC."

with:

"if the variable vr1=info-trans, then start the timer t1IN."

2) Table 3.2.6-4 (State Table), cell located in row AIDC-ucf REQ and column "IDLE", fourth level-1 bullet, replace:

". if c1 & c32 {

. start t1NC

-> IDLE } "

with

". if c1 & c32 {

. start t1IN

-> IDLE } "

SME Recommendation to CCB:   ACCEPT / RESOLVE

CCB Decision: (CCB/11)               ?

Title : ICAO 9705 - Coord-Start service : Bad vr1/vs1

PDR Reference:                            99100002

Originator Reference:                     -

SARPs Document Reference:                 SARPs Sub_Volume III

Status:                                   SUBMITTED

Impact:

PDR Revision Date:

PDR Submission Date:                      28/10/99

Submitting State/Organization:            CENA/CHARME Project

Submitting Author Name:                   Mathieu JEAN

Submitting Author E-mail Address:         jean@cenatls.cena.dgac.fr

Submitting Author Supplemental

Contact Information:

SARPs Date:                               ICAO 9705 Edition 2

SARPs Language:                           English

Summary of Defect:

In the sections 3.2.6.1.7.2.1. and 3.2.6.1.8.2.1., it is specified that the value vr1/vs1 is set to 'back'.

I think vr1 and vs1 must be initialized with 'coord-start'.

For example, when the ASE receives a Coord-Start Ind, vr1 is set to 'back', but on the next User-Cnf-Req (section 3.2.6.1.33.2.1. b) 2) i) or ii) ), there is just a check on vr1 with 'notify', 'coord-start', 'info-trans' and 'end'. But no check on 'back'.

In the same way, when the ASE receives a Coord-Start Req, vs1 is set to 'back', but on the next User-Cnf-Ind (section 3.2.6.1.34.2.1 c) 1) i) ), there is just a check on vs1 with 'notify', 'coord-start', 'info-trans' and 'end'.

The CHARME implementation detects a protocol error.

Additional comments:

Indeed this is a defect of the SARPs. The objective of vr1 = back must have been the case when the state is TRANSFERRED; in that case 'back' means 'go to BACKWARD-COORDINATING on receipt of matching ucf.req/ind'. However the separate value 'back' does not seem to be necessary since the transition to BACKWARD-COORDINATING can be inferred from state = TRANSFERRED when ucf is received.

Proposed SARPs amendment:

1. Section 3.2.6.1.7.2.1 h) Replace :

h) set the variable vs1 = back and vs2 = Msg Number

with :

h) set the variable vs1 = coord-start and vs2 = Msg Number

2. Section 3.2.6.1.8.2.1. h) Replace :

h) set the variable vr1 = back and vs2 = Msg Number

with :

h) set the variable vr1 = coord-start and vs2 = Msg Number

3. Section 3.2.6.1.33.2.1 b) 2) viii) A) Replace :

A)      if the variable vr1 = back, then enter the BACKWARD COORDINATING state.

with :

A)      if the variable vr1 = coord-start, then enter the BACKWARD COORDINATING state.

4. Section 3.2.6.1.34.2.1 c) 1) vii) A) Replace :

A)      if the variable vs1 = back, then enter the state BACKWARD COORDINATING

with :

A)      if the variable vs1 = coord-start, then enter the state BACKWARD COORDINATING

5. Table 3.2.6-4 (State Table), cell located in row AIDC-ucf REQ and column "TRANSFERRING", second level-1 bullet, replace:

". if c1 & c26 => BACKWARD-COORDINATING"

with

". if c1 & c6 => BACKWARD-COORDINATING"

6. Table 3.2.6-4 (State Table), cell located in row "rcv AIDC-ucf-REQ pdu" and column "TRANSFERRING", second level-1 bullet, replace:

". if c1 & c27 => BACKWARD-COORDINATING"

with

". if c1 & c7 => BACKWARD-COORDINATING"

SME Recommendation to CCB:   ACCEPT / RESOLVE

CCB Decision: (CCB/11)               ?

Title : ICAO 9705 - Figure 3.2.10-10 - Bad timers 1CT/2CT

PDR Reference:                           99110001

Originator Reference:                    -

SARPs Document Reference:                SARPs Sub_Volume III

Status:                                  SUBMITTED

Impact: 				 B

PDR Revision Date:

PDR Submission Date:                     02/11/99

Submitting State/Organization:           CENA/CHARME Project

Submitting Author Name:                  Mathieu JEAN

Submitting Author E-mail Address:        jean@cenatls.cena.dgac.fr

Submitting Author Supplemental

Contact Information:

SARPs Date:                              ICAO 9705 Edition 2

SARPs Language:                          English

Summary of Defect:

In the figure 3.2.10-10, when a positive User-Cnf Request is sent after a Coordinate-End Indication, the timer t2CT is set.

When the User-Cnf Indication is received, the timer t1CT is set.

I think it is a mistake and the both timers have to be reversed on the figure.

In the section 3.2.6.1.33.2.1 b) 2) iii) A) I), the timer t1CT is set when a User-Cnf Request is sent in the NEGOCIATING state with 'vr1 = coord-end' In the section 3.2.6.1.34.2.1 c) 1) ii) B), the timer t2CT is set when a User-Cnf Indication is received in the NEGOCIATING state with 'vs1 = coord-end'.

Proposed SARPs amendment:

Figure 3.2.10-10 :

reverse the both timers 1CT/2CT on the figure.

SME Recommendation to CCB:   ACCEPT / RESOLVE

CCB Decision: (CCB/11)               ?

Title : ICAO 9705 - Coord-End : incomplete text.

PDR Reference:                            99110002

Originator Reference:                     -

SARPs Document Reference:                 SARPs Sub_Volume III

Status:                                   SUBMITTED

Impact: 				  B (Bug)

PDR Revision Date:

PDR Submission Date:                      03/11/99

Submitting State/Organization:            CENA/CHARME Project

Submitting Author Name:                   Mathieu JEAN

Submitting Author E-mail Address:         jean@cenatls.cena.dgac.fr

Submitting Author Supplemental

Contact Information:

SARPs Date:                               ICAO 9705 Edition 2

SARPs Language:                           English

Summary of Defect:

According to the section 3.2.6.1.10.2.1 f) 2) (Coord-End Indication), the variable vre could receive the value accept when the Result parameter has the value 'accept'.

When the Result parameter has the value 'reject', there is no sentence to set the variable vre with reject.

This incomplete text raises an error because after receiving a negative Coord-End indication, the user sends a User-Cnf request. According to the section 3.2.6.1.33.2.1 b) 2) iii) A), vre has not the value 'reject' and the section II) is not executed.

Assigned SME:        			   Sub-Volume III SME

Proposed SARPs amendment:

Replace :

"2)  if the AIDC-crd-end Result parameter has the value "accept" then set

the variable vre = accept; and"

with :

"2)  if the AIDC-crd-end Result parameter has the value "accept" then set

the variable vre = accept;

3)  if the AIDC-crd-end Result parameter has the value "reject" then set

the variable vre = reject; and"

SME Recommendation to CCB:   ACCEPT / RESOLVE

CCB Decision: (CCB/11)               ?

Title: 					AIDC / ASN.1 Semantics of Latitude

PDR Reference: 				99110003

Originator Reference: 			ONS-SV3-07

SARPs Document Reference: 		3.2.1.1 page III-230

Status: 					SUBMITTED

PDR Revision Date:

PDR Submission Date:  			09 Nov. 1999

Submitting State/Organization:  	USA

Submitting Author Name: 		Moulton, James

Submitting Author E-mail Address: 	moulton@ons.com

Submitting Author

Supplemental Contact Information:	22636 Glenn Drive Suite 305

Sterling, VA 20164 USA

SARPs Date: 				Doc. 9705

SARPs Language:  				English

Summary of Defect:  The ASN.1 for longitude and latitude include a definition of degrees with a resolution of .001.  Optionally, minutes and seconds may be specified.  There is no discussion of the semantics of decimal degrees and minutes/seconds.

Assigned SME:				Sub-Volume III SME

Proposed SARPs amendment:

OPTION 1 (provided by PDR author)

Make degrees with a resolution of 1 and add a choice for the remainder of the

resolution range:

Latitude::=SEQUENCE

{

latitudeDegrees [0]     LatitudeDegrees latitudePartDegrees [1] LatitudePartDegrees latitudeDirection   [2] LatitudeDirection

}

LatitudeDegrees::= INTEGER(0..90)

unit = Degree, Range (0..90) resolution 1 LatitudePartDegrees::= CHOICE

{

thousandths      [0] Thousandths

latitudeMinSec   [1] LatitudeMinSec

}

Thousandths::=INTEGER(0..1000)

unit = thousandths (.001) Range 0..1000

LatitudeMinSec ::= SEQUENCE

{

latitudeMinutes    [0] DegreeMinutes OPTIONAL latitudeSeconds    [1] DegreeSeconds OPTIONAL

}

OPTION 2 (preferred by SME)

Check consistency with ADSP Documentation. If a change is needed, preference should be given if possible to an alignment with the syntax used in CPDLC.

SME Recommendation to CCB: <resolve, reject, forward>

CCB Decision:

Title: 					AIDC / ASN.1 of FrequencyVHF

PDR Reference: 				99110004

Originator Reference: 			ONS-SV3-08

SARPs Document Reference: 		3.2.1.1 page III-244

Status: 					SUBMITTED

PDR Revision Date:

PDR Submission Date:  			09 Nov. 99

Submitting State/Organization:  	USA

Submitting Author Name: 		Moulton, James

Submitting Author E-mail Address: 	moulton@ons.com

Submitting Author

Supplemental Contact Information:	22636 Glenn Drive Suite 305

Sterling, VA 20164 USA

SARPs Date: 				Doc. 9705

SARPs Language:  				English

Summary of Defect:  The ASN.1 for FrequencyVHF does not correspond to the voice frequency range.  In AIDC, this construct is used in TransferComm and it encompasses the entire voice band. The resolution in use today is 8.33Mz

Assigned SME:				Sub-Volume III SME

Proposed SARPs amendment:

FrequencyVHF ::INTEGER(0..22)

unit = 118MHz +n*8.33Khz separation (22 channels)

SME Comment:

This subject has been discussed at considerable length on the basis of PDR 97100016 (CPDLC/AIDC VHFFrequency/ VHFFrequencyChannel). The definitions were then aligned between CPDLC and AIDC, and due consideration was given to the 8.33 spacing issue.

The AIDC ASN.1 definition is fully aligned on the resolution of PDR 97100016. Looking further at the subject in CPDLC, it appears that the ASN.1 definition uses a different terminology (in CPDLC the ASN.1 type is FrequencyVHF instead of FrequencyVHFChannel for AIDC). However the definition and range are identical in both sub-volumes. In conclusion, there might be a need for an editorial update in either AIDC or CPDLC, but not for a change in the ASN.1 syntax.

SME Recommendation to CCB: 		REJECT

CCB Decision: (CCB/11)			?