# INTRODUCTION

## Scope and Objectives

### This Sub-Volume contains Standards and Recommended Practices (SARPs) for the initial version of the ATN Upper Layers (UL)-.

### The UL specification in this Sub-Volume supports all current ATN applications except the ATS Message Application defined in Sub-Volume 3.  This specification is designed to optimise the use of communications bandwidth, and consequently restricts the functionality available from the OSI Session and Presentation layers.

### These SARPs address the ATN requirements for Session Layer (Layer 5), Presentation Layer (Layer 6), and a part of the Application Layer (Layer 7) of the OSI reference model.  Figure 4.1-1 shows a conceptual view of the scope of these SARPs.  The remaining part of the Application Layer is the province of the individual ATN application SARPs (i.e.  the ADS, CM, CPDLC and FIS(ATIS) SARPs for air-ground applications, and the ICC (AIDC) SARPs for ground-ground applications).

Figure 4.1-1.  Conceptual view of the scope of the UL SARPs

## Background

### The communication aspects of the ATN applications are modelled as Application Entities (AEs) (see section 4.1.4.2).  Figure 4.1-2 illustrates an example of the application layer structure for the Air/Ground applications.

Figure 4.1-2.  Conceptual view of Application Layer

### These SARPs define a profile for the protocols in the upper layers, an AE structure and a number of common application services.

## Structure of Sub-Volume

### Chapter 4.1: INTRODUCTION contains the Sub-Volume’s purpose and structure, and a background to the functionality defined herein.

### Chapter 4.2: DIALOGUE SERVICE DESCRIPTION describes the abstract service which is defined for application SARPs to refer to in order to provide a common communications service.

### Chapter 4.3: APPLICATION ENTITY (AE) DESCRIPTION describes the Application Entity and specifies the Control Function (CF) which co-ordinates the operation of the various Application Service Elements (ASEs).  It also describes the names which are assigned to various upper layer entities.

### Chapter 4.4: SESSION LAYER REQUIREMENTS describes the requirements for the OSI Session Layer, in the form of a Profile Requirements List (PRL).

### Chapter 4.5: PRESENTATION LAYER REQUIREMENTS describes the requirements for the OSI Presentation Layer, in the form of a PRL.

### Chapter 4.6: ACSE SPECIFICATION describes the requirements for the Association Control Service Element (ACSE).

## Upper Layer Functionality

### Upper Layer Profile Overview

#### These SARPs specify a profile for the connection oriented protocols of Session layer, Presentation layer and the Association Control Service Element (ACSE).

#### The Session portion of the profile specified in these SARPs is based on the efficiency enhancements to the Session protocol which are standardised in ISO/IEC 8327-1 Amendment 2.

#### The Presentation portion of the profile specified in these SARPs is based on the efficiency enhancements to the Presentation protocol which are standardised in ISO/IEC 8823-1 Amendment 2.

#### As a consequence of using the Session and Presentation protocol efficiency enhancements, the protocol control information transferred by these protocols amounts to two octets in each direction during the connection establishment phase, and zero octets at all other times.

#### The ACSE portion of the profile specified in these SARPs is based on ISO/IEC 8650-1, including the extensibility notation as specified as Amendment 2 to that standard.

Note. — The ISO definition of the PER-visible extensibility markers is the subject of a Draft Amendment (DAM).

### Application Entity (AE) Structure

#### The AE structure specified within these SARPs is based on the application layer structure defined in ISO/IEC 9545, where the concepts of Application Service Element (ASE), Application Service Object (ASO) and Control Function (CF) are defined.

#### Figure 4.1-3 shows the generic structure of an AE with arrows representing the abstract service boundaries of the various elements.  The “upper” service boundary is the abstract service provided by an ASE to its user(s).  The “lower” service boundary is the abstract service which is provided to the ASE by the CF.

#### The ASE is an element engineered to perform a required task.  ISO/IEC 9545 describes how two or more ASEs may be combined, together with a CF to co-ordinate their operation to form an ASO.  In turn, an ASO may be combined with other ASOs or ASEs with a CF to form larger ASOs.  The AE is the outermost ASO.

Figure 4.1-3.  Generic Application Entity structure

### Application Services

#### These SARPs specify various abstract services.  The services are provided at abstract service boundaries.  The abstract service provided by the AE to the Application-user (i.e.  the service provided at the upper boundary of the AE) is specified in chapter 4.3.  In the AE structure specified in this Sub-Volume, this service is a pass-through to the ATN-App ASE.

#### For each of the current ATN applications a specific ASE exists, and is specified in the relevant ATN Application Sub-Volume.  The generic name “ATN-App ASE” is used for these specific ASEs.

#### Figure 4.1-4 shows the AE structure which is used to model the ATN applications.  This is described in detail in chapter 4.3.

Figure 4.1-4.  ATN-specific AE structure

#### The Dialogue Service (DS) as defined in chapter 4.2 of these SARPs is the abstract service which the ATN-App ASEs use to interact with the communication facilities defined in these SARPs.  That is, the DS is the combination of specific internal primitives made available by the CF at the lower boundary of the ATN ASE/ASO - it is the application’s “world view”.  In order to provide this service, the CF uses the services of ACSE.