| RTCA SC-189/EUROCAE WG-53 Position Paper Safety and Interoperability for Air Traffic Services Communication |  |  |  |  |
| --- | --- | --- | --- | --- |
| General Information |  |  |  |  |
| Position Paper Number: Call 206 227 2763 to get #: Leave fax # PSG1201A | Revision: e.g. 3 A | Date: e.g. 11 Nov 94 11 Apr 97 | Comments Due: 06 Jul 97 |  |
| Author/Address/Fax/Voice/Internet: Rob Mead Eurocontrol, PETAL-II Rue de la Fusee 96 B-1130 Brussels Belgium Voice: +32 2 729-3082 FAX: +32 2 729-9087 Internet: mead.rob@eurocontrol.be | Send Comments to (Include Name/Address/Fax/Voice/Internet): Rob Mead Eurocontrol, PETAL-II Rue de la Fusee 96 B-1130 Brussels Belgium Voice: +32 2 729-3082 FAX: +32 2 729-9087 Internet: mead.rob@eurocontrol.be |  |  |  |
| Position Paper Title: Adoption of FANS-1/A Connection Initiation Procedures During Transition Period |  |  |  |  |
| Abstract: In FANS-1/A, CPDLC (TWDL) connections are initiated by ground systems (i.e. the CPDLC connection request is sent by the ATS facility, and confirmed by the aircraft). In CNS/ATM-1, CPDLC connections can be intitiated by either the air or ground systems (i.e. the CPDLC connection request can be sent by either the ATS facility or the aircraft), and a mainstream operational proposal recommends that the aircraft initiates the request. If guidance is not published on CNS/ATM-1 systems, the potential exists for: a) interoperability problems for “dual stack” ground systems, which could be asked to request connections for FANS-1/A systems, and await requests from CNS/ATM-1 aircraft. b) potential interoperability problems for “single stack” CNS/ATM-1 airborne and ground systems, which could simply wait for the other partner to request a connection, or could require additional logic for the airborne and ground systems to deal with e.g. crossing connection requests. Both the above situations can be expected to intrude on controller and aircrew procedures, and increase costs for e.g. training and systems, thereby reducing overall benefits and use. It is proposed that SC189/WG53: Recomend that the FANS-1/A connection process be established as an internation standard for at least the FANS-1/A & ATN transtion period, to ease the operational transition problems surrounding transfer of communications and abnormal modes. Ground systems should routinely initiate CPDLC connection requests, and in a CNS/ATM-1 environment ground systems should have priority (i.e. in the case of crossing requests, ground systems would reject an airborne request; aircraft systems will always accept ground requests). NOTE: This recommendation does not apply to the CNS/ATM-1 Downstream Clearance service (i.e. CPDLC with an ATS facility other than the Current Data Authority), currently required as always requiring air-initiation. |  |  |  |  |
| Action taken: This paper was agreed by SG-1 at its March 1997, for consideration for FANS-1/A, Version 3.0 |  |  |  | Resolution Required By: |
| Key words (Optional): |  |  |  |  |

| Status | Individual Proposal |  |  | Sub-Group Agreement | Plenary Agreement |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Internal Use Only |  |  | Closed | SC-189 Co-chair prop. |  |
| Distribution |  |  |  |  |  |  |
| Name |  | Org | Internet |  |  | Fax |
| Dave Allen |  | BCAG | David.L.Allen@Boeing.Com |  |  | +01-425-294-2007 |
| Martine Blaize |  | DGAC | BLAIZE_Martine@STNA.DGAC.Fr |  |  | +33-5-62-14-58-53 |
| Serge Bagieu |  | Aerospatiale | Serge.Bagieu@Avions.Aerospatiale.Fr |  |  | +33-5-61-93-80-90 |
| Tom Kraft |  | FAA | Tom.Kraft@FAA.DOT.Gov |  |  | +01-425-227-1181 |
| Peter Skaves |  | FAA | Peter.Skaves@FAA.DOT.Gov |  |  | +01-425-227-1181 |
| Anne-Sophie Luce SG-1 |  | AIRSYS | LUCE@Thomson.SDC.Fr |  |  | +33-1-40-84-1859 |
| Tony Martin SG-1 |  | AJM Cnslt | AJMartin@Accessone.Com |  |  | +01- 206-868-7297 |
| Emmanuel Chenevier SG-2 |  | DGAC | CHENEVIER_Emmanuel@ccmail.DGAC.Fr |  |  | +33-62-14-53-53 |
| Elizabeth Noon SG-1 |  | FAA | elizabeth.noon@faa.dot.gov |  |  | +01-425-227-1181 |
| Mark Joseph SG-2 |  | MITRE | MJ@MITRE.Org |  |  | +01-703-883-1330 |
| Kent Loving - SG-3 |  | BCAG | Kent.Loving@Boeing.Com |  |  | +01-425-342-6078 |
| Marc Barrere - SG-3 |  | Aerospatiale | Marc.Barrere@avions.aerospatiale.fr |  |  | +33-5-61-93-07-29 |