| RTCA SC-189/EUROCAE WG-53 Position Paper Air Traffic Services Safety and Interoperability Requirements |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General Information |  |  |  |  |  |  |  |  |  |  |
| Position Paper Number: Call 206 227 2763 to get #: Leave fax # P/SG2/2 |  |  |  |  | Revision: e.g. 3 5 | Date: e.g. 11 Nov 94 23 May 97 |  |  | Comments Due: 15 July 97 |  |
| Author/Address/Fax/Voice/Internet: Mark Joseph CAASD/MITRE 1820 Dolly Madison Blvd. McLean, VA 22102 Voice: 703-883-5254 FAX: 703-883-1330 Internet: mj@mitre.org |  |  |  |  | Send Comments to (Include Name/Address/Fax/Voice/Internet): Author |  |  |  |  |  |
| Position Paper Title: Characteristics of the CNS/ATM Operational Environment for Air Traffic Services (ATS) that Use Data Communications |  |  |  |  |  |  |  |  |  |  |
| Abstract: This position paper identifies the characteristics of the CNS/ATM operational environment expected for the use of data communications relevant for near term separation minima reductions and procedural changes as circumscribed by the Terms of Reference for RTCA SC-189/EUROCAE WG-53. This paper is a product of the Safety Objectives Group (SG-2) which is responsible for developing the safety objectives, associated substantiation, and the guidelines to qualify implemented systems to meeting the safety objectives. |  |  |  |  |  |  |  |  |  |  |
| Action taken: Update of ‘Environment Characteristics Template’ position paper based on SG2 review at the 11-13 March 1997 meeting, for SG2 approval. |  |  |  |  |  |  |  |  |  | Resolution Required By: |
| Key words (Optional): RTCA SC-189/EUROCAE WG-53, Environment Characteristics Template |  |  |  |  |  |  |  |  |  |  |
| Status | Individual Proposal |  |  | Sub-Group Agreement |  |  | Plenary Agreement |  |  |  |
|  | Internal Use Only |  |  | Closed |  |  | SC-189 Co-chair prop. |  |  |  |
| Distribution |  |  |  |  |  |  |  |  |  |  |
| Name |  | Org | Internet |  |  |  |  | Fax |  |  |
| Tom Kraft |  | FAA | Tom.Kraft@FAA.DOT.Gov |  |  |  |  | +01-206-227-1181 |  |  |
| Peter Skaves |  | FAA | Peter.Skaves@FAA.DOT.Gov |  |  |  |  | +01-206-227-1181 |  |  |
| Dave Allen |  | BCAG | David.L.Allen@Boeing.Com |  |  |  |  | +01-206-294-2007 |  |  |
| Serge Bagieu |  | Aerospatiale | Serge.Bagieu@avions.aerospatiale.fr |  |  |  |  | 33-1-5-61-93-80-06 |  |  |
| Martine Blaize |  | DGAC | BLAIZE_Martine@ccmail.DGAC.Fr |  |  |  |  | +33-5-62-14-58-53 |  |  |
| Matthias Queck |  | DASA | Matthias.Queck@Airbus.De |  |  |  |  | +49-40-74-37-37-82 |  |  |
| Anne-Sophie Luce SG-1 |  | AIRSYS | LUCE@Thomson.SDC.Fr |  |  |  |  | +33-1-40-84-1859 |  |  |
| Tony Martin SG-1 |  | AJM Cnslt | AJMartin@Accessone.Com |  |  |  |  | +01- 206-868-7297 |  |  |
| Emmanuel Chenevier SG-2 |  | DGAC | CHENEVIER_Emmanuel@ccmail.DGAC.Fr |  |  |  |  | +33-62-14-53-53 |  |  |
| Mark Joseph SG-2 |  | MITRE | MJ@MITRE.Org |  |  |  |  | +01-703-883-1330 |  |  |
| Kent Loving - SG-3 |  | BCAG | Kent.Loving@Boeing.com |  |  |  |  | +01-2060342-6078 |  |  |
| Marc Barrere - SG-3 |  | Aerospatiale | Marc.Barrere@avions.aerospatiale.fr |  |  |  |  | +33-5-61-93-07-29 |  |  |

| RTCA SC-189/EUROCAE WG-53 Comment |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Position Paper Information (To be completed by Position Paper Author and attached to position paper) |  |  |  |  |  |  |  |  |  |  |
| Position Paper Number: Call 206 227 2763 to get #: Leave fax # P/SG2/2 |  |  |  |  | Revision: e.g. 3 5 |  | Date: e.g. 11 Nov 94 23-May-97 |  |  | Comments Due: 15 July 97 |
| Author/Address/Fax/Voice/Internet: Mark Joseph CAASD/MITRE 1820 Dolly Madison Blvd. McLean, VA 22102 Voice: 703-883-5254 FAX: 703-883-1330 Internet: mj@mitre.org |  |  |  |  | Send Comments to (Include Name/Address/Fax/Voice/Internet): Author |  |  |  |  |  |
| Position Paper Title: Characteristics of the CNS/ATM Operational Environment for Air Traffic Services (ATS) that Use Data Communications |  |  |  |  |  |  |  |  |  |  |
| Comment Information (To be completed by Comment Author) |  |  |  |  |  |  |  |  |  |  |
| Comment Author (Include Name/Address/Fax/Voice/Internet): |  |  |  |  | Comment Date: e.g., 3 Apr 96 |  | Type of Comment (Check all that apply): Response Requested Disagreement Clarification Additional Material |  |  |  |
| Comment (Attach pages as necessary): |  |  |  |  |  |  |  |  |  |  |
| Proposed Solution (Attach pages as necessary) |  |  |  |  |  |  |  |  |  |  |
| Status of Comment/Action Taken (To be completed by position paper author or delegate) |  |  |  |  |  |  |  |  |  |  |
| Assigned to: | SG-1 | SG-2 | SG-3 | Staff |  | CAG |  | Other | Comment Number: C- |  |
| Status/Action Taken: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1. Introduction

1.1 Environment Definition

1.2 Guidance for Usage

1.3 Environment Characteristics Coordination

1.4 Hierarchical Environment Characteristics Template

2. Airspace Characteristics

2.1 Airspace Configuration Characteristics

2.1.1 Separation Minima

2.1.2 Route Configuration

2.1.3 Route Structure Complexity

2.1.4 Type of Control

2.1.5 CNS Coverage

2.1.6 Type of Airspace

2.1.7 Other

2.2 Traffic Characteristics

2.2.1 Traffic/Schedule Demand

2.2.2 Aircraft Mix

3.0 Operations Description

3.1 Operational Scenario List

3.2 Procedure Description

4.0 Required Functional Capabilities

4.1 Functional Allocation to Segments

4.1.1 Aircraft Functional Capabilities

4.1.2 ATS Functional Capabilities

4.1.3 CSP LCD Functional Capabilities

4.1.4 AOC LCD Functional Capabilities

4.2 Identification of Standards and Deviations

4.3  Inter-segment Behavioral Assumptions

5.0 System Technical Characteristics

5.1 Required Technical Function

5.1.1 Communication Function

5.1.2 Navigation Function

5.1.3 Surveillance Function

5.2 Required Technical Performance

5.2.1 Required Total System Performance

- Required Control Performance

Appendix A	environment CHARACTERISTICS Template ApplicatioN

Appendix B	DETAILED Airspace Configuration Characteristics

Characteristics of the Communication, Navigation, and Surveillance (CNS)/Air Traffic Management (ATM)

Operational Environment for Air Traffic Services (ATS)

that Use Data Communications

# 1.	Introduction

## 1.1	Environment Definition

This document defines the characteristics of the Communication, Navigation, and Surveillance (CNS)/Air Traffic Management (ATM) operational environments for Air Traffic Service (ATS) that use data communications.  The term Ôoperational environmentÕ as used herein is defined by those characteristics which form the descriptive basis of operations that use data communications relevant for assessing the safety of operations for current conditions or conditions under which data communications will be used as a prerequisite for separation minimum reductions, and procedural enhancements.  The environment definition is fundamental to the process of developing safety objectives. The operational environment definition could be augmented with characteristics for use in evaluating ATS efficiency, or user benefit capture; however these characteristics are not addressed here.

## 1.2	Guidance for Usage

It is the purpose of this document to provide the necessary template for use by Regional Planning Groups (RPGs), Civil Aviation Authority (CAAs), ATS providers, and airspace users,  in compiling needed information for the identification and elaboration of safety objectives.

This template is provided to facilitate the end-to-end safety assessment process.  It:

- Organizes the necessary data in a consistent manner and framework;

- Serves as a measure of the comprehensiveness of the particular environment definition; and

- Provides a boundary to those characteristics of the environment which are believed to be pertinent to a safety assessment of an ATS using data communications.

This is not to say that an environment definition cannot be presented in a different form or that this template guarantees a complete description.

## 1.3	Environment Characteristics Coordination

In the introduction of any new operational capability, such as data communications, implementing entities have the joint objectives of ensuring global safety, interoperability, and acceptance.  The operational environment characteristics assumed for the purposes of safety assessment must also be consistent with those defined by the regional coordinating groups, and the International Civil Aviation Organization (ICAO) operational panels, technical panels, and RPGs.

## 1.4	Hierarchical Environment Characteristics Template

The operating environment can be characterized in terms of airspace characteristics, system technical characteristics, required functional characteristics, and operations description. A top level representation of the scope of this work is given in Figure 1:  Environment Characteristics. Sections 2 through 5 outline the template, and near term examples of its application are given in Appendix A.

# 2.	Airspace Characteristics

Airspace characteristics can be further categorized as airspace configuration characteristics and traffic characteristics.  These in turn can be subdivided as indicated in Figure 2, and described below.

Figure 1.  Environment Characteristics

Figure 2.  Airspace Characteristics

## 2.1	Airspace Configuration Characteristics

This environment characteristics template highlights the airspace configuration characteristics most pertinent for the safety assessment associated with ATS which use data communications. These characteristics include, separation minima, route configuration, route complexity, type of control, CNS coverage, sectorization, special use airspace definition, and topography considerations. A more complete list of airspace configuration parameters is included in Appendix B for reference.

### 2.1.1	Separation Minima

Specification of separation minima includes specification of lateral, longitudinal, vertical, and composite minima. [Detailed description of the separation minima in effect must be provided as it is a key element of the safety assessment of the end-to-end system. Any linkage to other characteristics must be indicated by explicit traceability to those characteristics of the environment. This includes any prerequisites for the implementation of separation minima which are defined through other characteristics of the environment.]

### 2.1.2	Route Configuration

The route configuration is specified through identification of fixed track, flexible tracks defined on a daily or more frequent basis, and the potential for random route clearances within the airspace.

### 2.1.3	Route Structure Complexity

The number of aircraft that may be safely handled by the sector controller is affected by route structure complexity which affects the potential for conflict between aircraft and the frequency of flight plan change requests.  Conflicts may arise through overtakes, at merges, at route crossings, through deviations from the planned route of flight, or through cleared or uncleared maneuvers.  Route structure complexity may be categorized as high, medium, and low complexity.

### 2.1.4	Type of Control

Control type may be specified as procedural, tactical, or mixed procedural and tactical.

### 2.1.5	CNS Coverage

CNS coverage is specified through the geographic description of its extent for each of its components; communications coverage, navigation coverage, and surveillance coverage.

### 2.1.6	Type of Airspace

Airspace type may be airport surface, approach/departure, terminal area, en route, off-shore, or oceanic and remote airspace.

### 2.1.7	Other

#### 2.1.7.1	Sectorization

Definition of sectors within Flight Information Regions (FIRs) that have multiple sectors is described through the coordinates of the vertices of the polygons which form the sectors, and through the specification of the times during which this subdivision of the FIR is valid.

#### 2.1.7.2	Special Use Airspace (SUA)

Definition of Special Use Airspace (SUA), is in terms of geographic and temporal extent, and management and coordination details between SUA authority and ATS provider.

#### 2.1.7.3	Topography

## Topographic consideration include constraints affect the implementation of CNS capabilities, as well as minimum safe altitude area definition.

## 2.2	Traffic Characteristics

### 2.2.1	Traffic/Schedule Demand

Traffic growth will generate the need for increased throughput on some routes, even as the creation of new routes may tend to reduce growth rates on other existing routes.  New routes will be defined to accommodate new airports and associated city pairs.  Demand increases within a region or on a route may be characterized by increases corresponding to airport capacity coming on-line. Metrics characterizing traffic density include track occupancy and sector traffic density.

#### 2.2.1.1	Throughput

The target throughput which can be accommodated subject to an acceptable distribution of system induced delay is the primary metric class associated with a route, route system, gateway fix, sector, or in general airspace resource. Achievable throughput is dependent on traffic mix, airspace configuration, and required total system performance parameters.

#### 2.2.1.2	Track Occupancy

Daily/hourly peak, average, and standard deviation of [airspace resource] throughput are more specific metrics which characterize the airspace resource.

#### 2.2.1.3	Sector Traffic Density

Sector traffic density can be characterized by maximum instantaneous aircraft count, and maximum daily aircraft count. Traffic tends to peak at desired travel times, and thus daily averages are not alone indicative of the peak sector traffic density. Sector traffic density affects collision risk and controller workload.  Traffic density is characterized as follows:

Low		5 or fewer aircraft per sector

Medium	5 to 14 aircraft per sector

High		15 or more aircraft per sector

### 2.2.2	Aircraft Mix

Aircraft mix includes equipment mix and aircraft performance mix as follows.

#### 2.2.2.1	Equipment

Traffic is in general thought of as being a mix of equipped and unequipped aircraft  including General Aviation, Military, and Airline Operator traffic. Equipment may be diverse in communication capabilities, and navigation capabilities. Equipment diversity increases the complexity of the ATS in handling the traffic safely.

#### 2.2.2.2	Aircraft Performance

Independent of equipment mix [though potentially highly correlated] is aircraft performance mix defined here in terms of speed, altitude, and climb/descent performance mix.

##### 2.2.2.2.1	Speed

Speed mix refers to the optimal and maximum operating speed characteristic of the aircraft on a route.  Aircraft of differing speed characteristics present the possibility of changing separations as a function of time, and therefore presents a control problem should a faster aircraft be assigned a slot on a route behind a slower aircraft.

##### 2.2.2.2.2	Altitude

Altitude mix refers to the capability of the various aircraft on the route at various cruising altitude.  Aircraft have differing capabilities which allow them to achieve optimum performance at different cruising levels.  To the extent there is altitude preference diversity on a given route aircraft may be separated more easily in this dimension.  Otherwise a form of horizontal separation must be applied.

##### 2.2.2.2.3	Climb/Descent

Aircraft types differ in climb/descent performance.  In the en route phase of flight this parameter may be of use in optimizing aircraft sequencing.  In the Terminal Management Area (TMA), it influences the determination of Top of Descent (TOD).

# 3.0	Operations Description

The Operations description as input to the safety assessment consists of a list of operational scenarios, and descriptions of procedures which relate to each human within operations, e.g., controller, and flight crew.  This includes aircraft, ATM, Aeronautical Operational Control (AOC), and Communications Service Provider (CSP) components; and their interactions in the use of data communications.  An example of a the Operations Description elements of a particular instance of the environment characteristics template are depicted in Figure A-1.

## 3.1	Operational Scenarios List

Operational scenarios which are relevant to data communication implementation cover the proposed overall operation using data communication and the intended use of the individual data communication functions. These operational scenarios could include AFN threads, CPDLC threads, and ADS threads. An example operational scenario list is given in Appendix A.

## 3.2	Procedure Description

Procedure  description  for the purpose of safety assessment is taken to be those regional or FIR specific procedures for the identified airspace. These may include, for example, the following: Primary means CPDLC, HF backup, VHF voice, Satellite voice, ADS use as primary means of surveillance, etc. These procedures must cover both the nominal situations and the non-nominal, including failure situations. An example list of procedures is given in Appendix A.

In addition, any aircrew, controller, traffic management specialist, and AOC procedures which may have safety objective relevance as identified in the functional hazard assessment where procedural mitigation of a hazard is possible, are required.

# 4.0	Required Functional Capabilities

While a globally integrated and seamless ATC environment is an operational objective, in the near term the environment can be characterized by functional capability variations, in terms of aircraft, and ground system (FIR) functional capabilities.  These capabilities are those required for maintaining the airspace operation, as it pertains to CNS/ATM, in a safe manner. For the purposes of this template, these should generally be closely related to data communications.

There are also numerous possibilities of interaction between equipped and non-equipped entities, as well as the possibility of differences in functional allocation to segment, interface implementation, and inter-segment behavioral assumptions.  For clarity, these possibilities must be articulated within the FIR or regional environment description.

The required functional capabilities portion of the environment characteristics template is shown in Figure 3.

Figure 3.  Required Functional Capabilities

## 4.1	Functional Allocation to Segments

### 4.1.1	Aircraft Functional Capabilities

Within the context of a fleet mix of equipped aircraft, performance differences over the range of traffic characteristics,  may create potential complexities to which the ATC system must evolve solutions. In general the aircraft functional capabilities must support the operational capabilities or behaviors to:

- Maintain aircraft flight path in accordance with its flight plan [navigation, guidance and control capabilities consistent with airspace requirements].

- Receive and acknowledge clearance information from ATC [communications functionality and performance consistent with procedures and minima].

- Inform ATC and AOC of the aircraft state and intent [reporting functionality and performance consistent with procedures and minima].

In addition, requirements may drive the need for other functional capabilities, which should also be defined such as the capability to;

- Receive and acknowledge reroute information from AOC [autoload capabilities]

- Provide automated collision avoidance (ACAS/TCAS)

Documented and validated aircraft functional allocations and inter-segment assumptions are required.

### 4.1.2	ATS Functional Capabilities

For the purposes of this template Regional Planning Groups must define a lowest common denominator (LCD) set of operational/functional capabilities provided, or supported by the end-to-end system (including ground system automation and personnel) which enables the safe implementation of planned procedure and minima changes across FIR boundaries. This set of capabilities enables the controller to effectively monitor the traffic under his/her control within reasonable workload limits; and may be realized through a number of possible combinations of automation tools, display, and manual methods. ATS functional capabilities to be included in consideration of safety objectives are at a minimum the capabilities to;

- Inform controllers of the state and intent of aircraft under their control

- Assure separation between aircraft consistent with minima

- Coordinate control responsibilities and clearances (includes interfacility communication)

- Distribute information to aircraft/pilots (include adverse weather or conditions)

- Monitor aircraft conformance to its published flight plan (includes surveillance of flights)

- Maintain records of clearance sequences

Beyond minimum capabilities a desire for higher levels of service may drive requirements for capabilities to;

- Provide dynamic reroutes for wind optimized aircraft trajectories

- Provide means to measure and document the components of achieved total system performance including, achieved navigation performance, gross navigation errors, achieved communication performance, achieved monitoring performance, and achieved control performance.

### 4.1.3	CSP LCD Functional Capabilities

Documented and validated Communications Service Provider Segment functional allocations, and inter-segment assumptions are required.

### 4.1.4	AOC LCD Functional Capabilities

Documented and validated AOC functional allocations and inter-segment assumptions are required to support defined procedures for DARPS or reduced separation minima within the scope of this work are required.

## 4.2	Identification of Standards and Deviations

Industry standards which form the basis of the implementation must be identified, as well as any known deviations from the standards whether functional or performance deviations. The use of industry standards and if necessary, the deviations from those standards, to describe the above functional capabilities is encouraged.

## 4.3 	Inter-segment Behavioral Assumptions

Expected nominal segment behavior as well as behavior in the event of possible segment failures are described here for identified non-nominal conditions.

# 5.0	System Technical Characteristics

The description of system technical characteristics consists of the specification of required technical function, and required technical performance as shown. The system technical characteristics portion of the environment characteristics template is outlined in Figure 4.

## 5.1	Required Technical Function

### 5.1.1	Communication Function

Required functional characteristics associated with the communication function are specified here.  These may include, requirements for voice backup, medium diversity, time stamp, addressability, address control, and transparency of data transfer; as identified in the draft Required Communications Performance (RCP) Minimum Avionics System Performance Standard (MASPS).

### 5.1.2	Navigation Function

Required navigation function relevant for data communications operations is specified here in accordance with  the functional requirements defined in the RNP MASPS (RTCA DO-236) (Systems Features and Capabilities Section) such as position initialization, and path selection via data link uplink.

### 5.1.3	Surveillance Function

# Required surveillance function is specified here and may be characterized by radar or non-radar, and/or dependent surveillance which may be automated or non-automated.

Figure 4.  System Technical Characteristics

## 5.2	Required Technical Performance

Required Technical Performance characteristics are given in terms of  required total system performance (RTSP) and required control performance.  RTSP for the traffic, airspace, and procedures is defined in terms of RCP, required navigation performance (RNP), and required monitoring performance (RMP).

### 5.2.1	Required Total System Performance

#### 5.2.1.1	RCP

RCP is specified in terms of delay, integrity, and availability parameters. For particular instances to be qualified the term Installed Communication Performance (ICP) may be more correct here for the end-to-end system [as defined in the RCP MASPS draft version 9.0]

#### 5.2.1.2	RNP

Required navigation performance may be specified in terms of accuracy, integrity, availability and continuity parameters. Gross Navigation Error (GNE) rate also characterizes the navigation performance of a population of aircraft in a statistical sense. RNP or Available Navigation Performance (ANP) not to exceed a given RNP with a given probability, is specified for an airspace, route, procedure, or operation.

#### 5.2.1.3	RMP

RMP requires as a minimum specification of position reporting rates, and position report latency, as a function of separation minimum and procedure implementation.

### 5.2.2	Required Control Performance

This section requires specification of characteristics for both Aircraft/Pilot, and Ground System/Controller performance.

#### 5.2.2.1	Aircraft/Pilot Performance

This section must include pilot reaction, and aircraft performance related parameters associated with pilot actuated avoidance maneuvers, such as maximum time to achieve altitude separation.

#### 5.2.2.2	Ground System/Controller Performance

##### 5.2.2.2.1	Decision Support Tools

This section requires the identification of required controller tools as a function of traffic densities expected, and procedures to be implemented. Examples are, distance measurement tools, and conflict probe capabilities.

##### 5.2.2.2.2	Proficiency

This section requires the specification of training appropriate to the procedures for data communication under all circumstances experienced in the airspace. Controller decision and execution time should be considered under this heading.

# References

- Characterization of Airspace Environments Basis for Hazard Analysis, David L. Allen

- South Pacific Operation Manual, ISPACG

- Air Traffic Services System Requirements and Objectives, Boeing Commercial Aircraft Group

- Minimum Aviation System Performance Standards (MASPS):Required Communication Performance for Air Traffic Services (DRAFT 9.0)

- ICAO DOC 7030 Regional Supplementary Procedures

- Minimum Aviation System Performance Standards: Required Navigation performance for Area Navigation, DO-236, January 27, 1997

Glossary

ACARS	Aircraft Communications Addressing and Reporting Systems

ADS		Automatic Dependent Surveillance

AFN		ATS Facilities Notification Function

ANP		Available Navigation Performance

AOC		Aeronautical Operational Control

ATM		Air Traffic Management

ATS		Air Traffic Service

CAA		Civil Aviation Authority

CNS		Communication, Navigation, and Surveillance

CPDLC	Controller Pilot Data Link Communication

CSP		Communication Service Provider

DARPS	Dynamic Aircraft Route Planning System

FANS		Future Air Navigation System

FIR		Flight Information Region

HF		High Frequency

ICAO		International Civil Aviation Organization

ICP		Installed Communication Performance

ID		Identification

LCD		Lowest Common Denominator

NM		nautical miles

MASPS	Minimum Aviation System Performance Standard

RCP		Required Communications Performance

RNP		Required Navigation Performance

RMP		Required Monitoring Performance

RPGs		Regional Planning Groups

RTA		Required Time of Arrival

RTSP		Required Total System Performance

SATCOM	Satellite Communications

SUA		Special Use Airspace

TCAS		Traffic Alert and Collision Avoidance System

TMA		Terminal Management Area

TOP		Top of Descent

VHF		Very High Frequency

Wx		Weather

APPENDIX A

environment CHARACTERISTICS Template Application

A.1	ICAO Regional Applicability

Examples of applicability from the  ICAO PAC, and NAT regions are discussed here, though it is expected that the template will have global application. The effectiveness with which regional differences may be identified and accommodated will be influenced by the existence of material provided by each Regional Planning Groups (RPGs). The form of the template is shown in table A-1.  It is then shown abbreviated to highlight a distinguishing characteristics subset in tables A-2 through A-7.

A.2	Regional Application of Near-Term Initiative Summary

The following paragraphs list the more prominent initiatives by region of application and provides a table of environment characteristics as they are currently understood. The tables are not all inclusive, but are rather illustrative subsets.

A.2.1	RHSM in the Pacific

RHSM enabling capabilities for the ground system and aircraft have been considered and evaluated for four initiatives:  50 NM lateral in the Pacific, 50 NM longitudinal in the Pacific, 30 NM lateral in the Pacific, and 30 NM longitudinal in the Pacific.

A.2.1.1		50 NM LATERAL IN THE PACIFIC

Application of 50 NM lateral in the Pacific requires a population of RNP-10 aircraft as verified by State of Registry or State of Operator. Guidance for approval or aircraft for RNP-10 airspace or route has been published in FAA Order 8400.12, Required Navigation Performance 10 (RNP-10) Operational Approval, which requires cross-track and along track positioning error <  10 NM (95%).  Consistent with the notion of ÔRNP’ 50 NM lateral based on RNP-10 does not require the use of satellite navigation. ICAO Document 9613-AN/937, Manual on Required Navigation Performance, contains the fundamental concepts for the application of RNP within an airspace. However, aircraft approved to utilize GPS as a primary means of navigation for oceanic and remote operations IAW FAA Order 8110.60 are considered to meet RNP-10 requirements of  FAA Order 8400.12.  FAA Order 8110.60 in turn specifies dual GPS equipment, and  an approved fault detection and exclusion (FDE) availability prediction program must be used [with a lateral separation protected area of 20 NM, yielding an acceptable FDE outage time of 34 minutes]. Refer to FAA Order 8400.12. for details of other means of demonstrating RNP-10 capability for approval. For aircraft with INS equipment RNP-10 approval is defined in terms of a maximum stage length without update.

In terms of procedure definition, for 50 lateral implementation the preflight phase considerations include state approval for operations, pilot and dispatcher study of the route of flight to determine any special navigation, communication or surveillance requirements, review of track messages, en route charts, NOTAMs, and flight planning material; to insure that RNP and equipment requirements for the filed route of flight and any planned alternate have been met. The latter to include any required communication equipment for the route. The pilot annotates Item 10 of the ATC flight plan with the letter ÒRÓ, if an RNP is specified on the route and the aircraft conforms to the requirements of the route or airspace. Prior  to entry onto the route of flight the pilot ensures that the navigation equipment is operational (e.g., prior to the oceanic airspace gateway fix).

| 1.0 Environment Characteristics |  |  |  |
| --- | --- | --- | --- |
| 2.0 Airspace Characteristics | 3.0 Operations Description | 4.0 Required Functional Capabilities | 5.0 System Technical Characteristics |
| 2.1 Airspace Configuration 2.2.1 Separation minima 2.2.2 Route configuration 2.2.3 Route Structure Complexity 2.2.4 Type of control 2.2.5 CNS coverage 2.2.6 Type of airspace 2.2.7 Other | 3.1 Operational scenario list (see figure A-1) 3.1.1 AFN Threads 3.1.2 CPDLC Threads 3.1.3 ADS Threads 3.1.4 Other Threads | 4.1 Functional Allocation to Segment 4.1.1 Aircraft 4.1.2 ATS 4.1.3 CSP 4.1.4 AOC | 5.1 Required technical function 5.1.1 Communication function 5.1.2 Navigation function 5.1.3 Surveillance function |
| 2.2 Traffic Characteristics 2.2.1 Traffic/Schedule Demand 2.2.1.1 Throughput 2.2.1.2 Track Occupancy 2.2.1.3 Sector Traffic Density - Aircraft mix - Equipment - Aircraft Performance | 3.2 Procedures Description (See Figure A-1) 3.2.1 Primary means CPDLC 3.2.2 Primary means ADS 3.2.3 DARPS 3.2.4 Separation minima reduction In-trail climb/descent ACAS/TCAS Wx Deviation Transition between environments with different units of measurement | 4.2 Interface ID & deviations (A.4.2) | 5.2 Required technical performance 5.2.1 RTSP 5.2.1.1 RCP 5.2.1.2 RNP 5.2.1.3 RMP - Required control performance - Aircraft/Pilot - Ground System/ Controller Decision support tools Proficiency |
|  |  | 4.3 Inter-segment behavior assumptions (A.4.3) |  |

Table A-1 Environment Characteristics

| Airspace Characteristics 50 NM lateral |  |  |
| --- | --- | --- |
| Operations Description | Required Functional Capabilities | Required Total System Performance |
| - Procedural RNAV - Procedures for 50 lateral implementation - Wx deviation procedure tailored for severe weather areas | - HF voice - DCPC* may be needed in some areas (e.g., severe weather areas) *CPDLC in U.S. FIRs | RCP - Delay (110) - Integrity (10-04) - Availability (.9-.99) RNP - RNP-10 - Cross-track positioning error < 10 NM (95%) - Along-track positioning error < 10 NM (95%) - Dual navigation systems of requisite integrity, and availability RMP - Pilot position reports at mandatory waypoints |

Table A-2 50 NM Lateral Environment Characteristics Subset

A.2.1.2		50 NM LONGITUDINAL IN THE PACIFIC

| Airspace Characteristics 50 NM longitudinal [Limited use and extended use phases] |  |  |
| --- | --- | --- |
| Operations Description | Required Functional Capabilities | Required Total System Performance |
| - Procedural RNAV - Speed control techniques - Distance verification with respect to common waypoint @ 30 minute intervals - HF voice backup - Satellite voice backup (if available) - ACAS/TCAS (if available) | - DCPC* - Application of minimum facilitated by distance verification controller tools. - ADS reports and distance verification (required for extended use phase of implementation) - See table of interface definitions and exceptions - *CPDLC in U.S. FIRs | RCP - Delay (55 seconds) - Integrity (10-05) - Availability (.99) RNP (RNP-10) - Common time reference (In practice GPS usage if ADS reports taken at different times are used for separation) RMP - Position reports @ 30 minute intervals |

Table A-3 50 NM Longitudinal Environment Characteristics Subset

Enabling capabilities for 50 NM longitudinal are still under evaluation and may change over time, for example to arguments supporting a broader application then presently supported  (i.e., for a small fraction of the aircraft in the airspace.) The environment is assumed to be procedural RNAV and distance verification neednÕt be automated to meet the requirements of the DOC 7030 amendment to the regional supplementary procedures, though in practice controller workload issues may dictate that automation capabilities exist before any extensive application.

A.2.1.3		30 NM LATERAL IN THE PACIFIC

Though initial evaluations have been conducted, and enabling capabilities identified, it should be noted that the enabling capabilities for 30 NM lateral are still under evaluation and may change over time. At this separation minimum, automatic position reporting is listed as a prerequisite, and the transition to an ADS environment must be made. ADS is used as the primary means of surveillance.

| Airspace Characteristics 30 NM Lateral |  |  |
| --- | --- | --- |
| Operations Description | Required Functional Capabilities | Required Total System Performance |
| - ADS Environment | - DCPC* (Voice or CPDLC) - ADS - Distance verification - Conformance monitoring - Conflict probe - Flight plan autoload *CPDLC in U.S. FIRs | RCP - Delay (55 seconds) - Integrity (10-06) - Availability (.999) RNP - RNP-4 - Common reference time RMP - ADS position reports @ 15 minute intervals |

Table A-4 30 NM Lateral Environment Characteristics Subset

A.2.1.4		30 NM LONGITUDINAL IN THE PACIFIC

Though initial evaluations have been conducted, and enabling capabilities identified, it should be noted that the enabling capabilities for 30 NM longitudinal are still under evaluation and may change over time.

| Airspace Characteristics 30 NM longitudinal |  |  |
| --- | --- | --- |
| Operations Description | Required Functional Capabilities | Required Total System Performance |
| - ADS Environment | - DCPC* - Automated distance verification @ 15 minute intervals based on ADS periodic reports - Conformance monitoring - Conflict probe *CPDLC in U.S. FIRs | RCP - Delay (55 seconds) - Integrity (10-06) - Availability (.999) RNP - RNP-4 - Common time reference RMP - ADS position reports @ 15 minute intervals |

Table A-5 30 NM Longitudinal Environment Characteristics Subset

A.2.1.5		IN-TRAIL CLIMB/DESCENT IN THE PACIFIC

This initiative provides procedural mechanism whereby an aircraft desiring an altitude climb/descent may do so with as little as 15 NM longitudinal separation from a leading aircraft. Aircraft flying behind and 2000 feet above or below the other aircraft along the same oceanic route may request a climb or descent through the altitude of the lead aircraft as long as the distance between them is 15 NM, as observed on the ACAS/TCAS, and the ground speed closure rate is 20 knots or less.

| Airspace Characteristics Equipment mix - Applicable to ACAS/TCAS equipped aircraft only |  |  |
| --- | --- | --- |
| Operations Description | Required Functional Capabilities | Required Total System Performance |
| - In-Trail Climb/Descent procedure | - HF air to ground - Air to air VHF - ACAS/TCAS which meets minimum adequacy requirements | - RCP for HF air to ground |

Table A-6 In-Trail Climb/Descent Environment Characteristics Subset

A.2.1.6		DARP IN THE SOUTH PACIFIC

The DARP initiative provides for the capability to update/revise the route of flight while an aircraft or a group of aircraft are en route.  It  is planned in three phases;

- ATS supplied single group re-routes. [This phase is planned for implementation in 1997 in the Pacific and is described in the SPOM Part 2.]

- User supplied individual original track and single re-routes: between nominated city pairs, anywhere in the region

- User supplied individual original track and multiple re-route anywhere in the region

| Airspace Characteristics Equipment mix FANS 1/A capable or equivalent aircraft |  |  |
| --- | --- | --- |
| Operations Description | Required Functional Capabilities | Required Total System Performance |
| - Procedural RNAV - DARP procedures | - CPDLC - Wx forecast updates - Track generation & distribution - ATS Interfacility Data Communications (AIDC) - FMS Autoload - Required Time of Arrival [If procedurally required] | RCP - Delay (55 seconds) - Integrity (10-05) - Availability (.9-.99) RNP - RNP-10 or 4 RMP ADS(optional) |

Table A-7 DARP Environment Characteristics Subset

A.2.2	RHSM IN THE NAT

Initiatives discussed for relatively near term separation minima implementation in the NAT are

- 7 minutes longitudinal

- 30 NM lateral

- 5 minutes longitudinal

Environment characteristics for the sum of these initiatives may be the equivalent of those outlined for the 30 longitudinal table above, however, the time based longitudinal separation minimum, suggests potential design differences in controller tools.

A.3	Operations Description Elements for FANS 1/A

An example of a the Operations Description elements of a particular instance of the environment characteristics template are depicted in Figure A-1. It shows both the operational scenario list, and the procedure definition portion of the template, in a form potentially suitable for operation using FANS 1/A equipment, and ARINC 622 communications.

Figure A-1.  Operations Description

### A.3.1.1	AFN Threads

This section provides a list of required AFN threads, such as those listed in Figure A1.

### A.3.1.2	CPDLC Threads

This section provides a list of required CPDLC threads, such as those listed in Figure A1.

### A.3.1.3	ADS Threads

This section provides a list of required ADS threads, such as those listed in Figure A1.

### A.3.2.1	Primary Means CPDLC

Where CPDLC is used as the primary means of communication, procedures for its use as such are required.  Voice communication is considered a back-up to primary means CPDLC.  This may include use of Very High Frequency (VHF), Direct High Frequency (HF), HF through a third party, and Satellite Communications (SATCOM) voice.

#### A.3.2.1.1	HF Backup

Procedures which specify the means and conditions under which to revert to backup voice communication are required, as well as the procedures required to assure that the HF backup capability can be switched to in a timely manner when needed.  Normally the procedure to revert to the backup will be indicated through the expiration of a message assurance timers either on board the aircraft or on the ground.

#### A.3.2.1.2	Satellite Voice

Procedures for the use of satellite voice when available as a backup for primary use data communications are required. For example, initial satellite voice service in effect in Oakland (ZOA) provides direct satellite voice service between aircraft and FAA Ocean Centers, i.e., a central answering point on the oceanic control room floor. Aircraft can call center through any GES, in distress, urgency, or exceptional situations only. Center can call aircraft via COMSAT priority calling service if ; aircraft is logged onto POR or AOR-W, aircraft telephone number is in Inmarsat data base or advised by crew. Center is restricted to ÒFlight SafetyÓ, and ÒMeteorological/Flight RegularityÓ priorities.

#### A.3.2.1.3	VHF Voice

Procedures for the use of VHF voice when available as a backup for primary use data communications are required.

### A.3.2.2	ADS use as Primary Means for Surveillance

Procedures for the use of ADS are required consistent with the instance of its use. Procedures for its use as primary means of surveillance are required.  When ADS is used as the primary means of surveillance backup procedures are also required to revert to other means if there is a failure in the end-to-end system.  Procedures for reversion to the alternative reporting methods such as HF, VHF, or satellite voice pilot reports, or CPDLC reporting methods are required.

### A.3.2.3	Dynamic Aircraft Route Planning System (DARPS)

Procedures for the use of Dynamic Aircraft Route Planning System (DARPS) are required. The DARP initiative provides for the capability to update/revise the route of flight while an aircraft or a group of aircraft are en route.  DARP phase 1 operations as described, for example, in the SPOM requires that;

- Only capable aircraft shall be permitted to fly the DARP flex track re-route, while all others shall be constrained to ATS fixed routes,

- All flight plans and change messages shall be sent to all ATS units en route

- Successive aircraft on the DARP flex tracks shall accept the re-route that the lead aircraft is using

- The pilot shall request the re-route using the CPDLC downlink message 24, and the controller delivers the re-route clearance via the uplink message 80

- The ATS provider in whose airspace the DARP begins shall assure that each aircraft is recleared.

### A.3.2.4	Separation Minima Reduction

Procedures required for implementation of a particular reduced minimum, consist of;

- Preflight procedures

- Procedures prior to entry into designated airspace

- Procedures for application of the minima

- Transition procedures, and,

- Contingency procedures.

The general context of the procedural definition within the template are the rules derived from procedural separation based ATC in which;

- In general, an ATC clearance is requested before deviating from an ATC approved flight plan

- In the event of air/ground communications failure aircraft movements are assumed to be in accordance with published procedures.

- Minima in effect are consistent with procedures for voice usage, or, if reduced minima are in effect which require specified levels or elements of RCP, RNP, and RMP,  as prerequisites for possible ATC intervention via data communications, then procedural methods for reverting to voice usage must be available.

- Special procedures used for reduced minimum application must incorporate safety features so that the procedure may be safely terminated if the procedure prerequisites are no longer in effect.

With the implementation of reliable data communications and associated improvements in dependent surveillance near-tactical intervention becomes possible and the associated procedural enhancements will enable the desired minimum reductions. There are variations possible in the specification of procedural context which have been identified in the analytical materials supporting reduced separation minima, such as given in the following paragraphs.

In a procedural RNAV environment aircraft are asked to report their position with respect to a common waypoint, leading aircraft reporting first. Separation exists if the difference in the reported distances is equal or greater than the appropriate minimum. This context assumes the possibility of controller intervention, on a near-tactical basis.  This is the context in which CPDLC position reports are used to determine aircraft separation IAW the DOC 7030 Regional Supplementary Procedures, approved for 50 longitudinal

In an ADS environment, separation may be established by providing a reference display of aircraft positions (traffic situation display) which allows the controller to assess the distance between aircraft at prescribed intervals.  Separation exists if the observed longitudinal separation on the display, or associated alphanumeric table, is equal or greater than the applied minimum.  As above this context assumes the possibility of controller intervention, on a near-tactical basis.

#### A.3.2.4.1	In-Trail Climb/Descent

In-trail climb/descent procedures since they enable a longitudinal minimum reduction beyond those expected in general use are required as part of this category.

#### A.3.2.4.2	Airborne Collision Avoidance System (ACAS)/Traffic Alert and Collision Avoidance Systems (TCAS)

Procedures which specify the proper domain use of ACAS/TCAS (if available) are required. For example, The SPOM [Part 4] contains guidance for the use of TCAS II in Pacific oceanic airspace. While TCAS II provides a large measure of collision risk reduction, not all aircraft operating in Pacific oceanic airspace are so equipped. While current arguments for separation minima reductions do not rely on  (or Ôtake credit forÕ) the use  of TCAS II the additional safety improvement deriving from its use has been clearly demonstrated analytically and in practice.

#### A.3.2.4.3	Weather (Wx) Deviation

Wx deviation procedures are required to be consistent with safety of operations in the defined airspace, reduced minima, and the required communication capabilities.  Adverse weather may affect safety margins by forcing lateral deviations when to do so would cause separations below minima.  Procedures must include methods of proceeding with or without benefit of ATC clearance.

#### A.3.2.4.4	Transition Between Environments with Different Units of Measurement

Procedures which specify transition procedures between areas that are applying different units of measurement (e.g. metres/feet,WGS84/ and other reference system, meters/kilometers/NM, altimeter settings, etc.) are required.

#### A.4		Required Functional Capabilities Examples for FANS 1/A

[Note: Paragraph numbers correlate with document section numbers]

#### A.4.1.2.1	AFN LCD Functions

The AFN logon serves the purposes of providing the ATS with the applications supported on-board the aircraft, their associated version numbers, and in the Future Air Navigation System (FANS) environment the Aircraft Communications Addressing and Reporting System (ACARS) addresses for the aircraft. It provides ATS with the flight identification (ID), and the registration number of the aircraft which can then be correlated with the corresponding flight plan data. In the absence of logon information there can be no CPDLC connection. The South Pacific Operations Manual (SPOM), reference [2], defines pilot initiated logon, and ATS initiation (manual or automatic) on address forwarding.

#### A.4.1.2.2	ADS Use as Primary Means of Surveillance LCD Functions

ADS report utilization and ground system implementation may differ from FIR to FIR due to the fact that the ADS architecture will support a number of different operational concepts and corresponding system implementation.  ADS provides the capability to report at mandatory reporting points, at periodic intervals specified by ground contract, and upon altitude or lateral deviation events. On-demand reports provide the capability for the ground to request a single position report, and intent data,  when needed to resolve uncertainty in the position of an aircraft. ADS report utilization and ground system implementation may differ from FIR to FIR due to the fact that the ADS architecture will support a number of similar operational concepts and corresponding system implementations with respect to the use of waypoints, lateral and altitude deviation events, periodic position reports, and on-demand position reports.

The implementation of an automation based ADS environment for separation minimum of 30 lateral (or 30/30) requires RNP-4, and common time reference in accordance with the current arguments justifying the application of the reduced minima. It should be noted that the use of ADS reports within an operations concept does not mean of necessity that it is an ÔADS environment.Õ

#### A.4.1.2.3	CPDLC for Primary Means of Communication LCD Functions

LCD functional capabilities for primary means of communication use of CPDLC must be defined.

#### A.4.1.2.4	Voice Backup Communication LCD Functions

Voice backup may include VHF, Direct HF, HF through a third party, and SATCOM voice. Backup voice communication and the procedures by which the controller or pilot reverts to voice communication are assumed to be a part of the system capability in all cases.

#### A.4.1.2.5	Ground-Ground ATS Communication LCD Functions

Ground-ground ATS communications functions as required to support defined procedures for DARPS or reduced separation minima are required. Plans for these may currently differ from FIR to FIR. Inter-facility assumptions with regard to behavior in the event of loss of function are required.

#### A.4.1.2.6 - 4.1.2.n Other ATS LCD Functions

Other ATS communications functions as identified by the region or CAA as LCD are documented in additional sections added to the environment characteristics template here.

A.4.2	Identification of Standards and Deviations

From reference [2] industry standards which define the baseline for the FANS 1/A equivalent ATS functions, including the ACARS datalink are incorporated by reference, e.g. ;

- ARINC 429, Mark 33 Digital Information Transfer System

- ARINC 429, Supplement 13, Williamsburg Protocol

- ARINC 618, Air-Ground Character Oriented Protocol Specification

- ARINC 619, ACARS Protocols for Avionics End Systems

- ARINC 620, Supplement 1, Datalink Ground System Standard and Interface Specification

- ARINC 622-1, Supplement 1, ATS Datalink Applications over ACARS Air-Ground Network

- ARINC 702, Flight Management Computer System

- ARINC 724B, Aircraft Communications Addressing and Reporting System

- ARINC 741, Aviation Satellite Communications System: Part 1 Supplement 6, Aircraft Installation Provisions and Part 2 Supplement 3, System Design and Equipment

- ARINC 716, Supplement 2 Aircraft VHF Communications Transceiver

- ARINC 745-2, Supplement 2 Automatic Dependent Surveillance

- RTCA DO-212, MOPS for ADS (developed by RTCA SC-170)

- RTCA DO-219, MOPS for ATC Communications Application (developed by RTCA SC-169)

- ADS/ATS Data Link Applications Guidance Material, ICAO ADS Panel

A.4.3	Inter-segment Behavioral Assumptions

Behaviors of the segments under the conditions of failures of segments must be defined and the safety critical aspects identified.  This may take the form of safety objectives which define the expected behavior of a segment with respect to another segment. These objectives are documented in a series of tables as depicted in Table A-8, which provides guidance for the software requirements, design, and development. They are tracked through system development to assure that they are not violated. There are six such table representing the possible assumptions associated with the three segments; Aircraft, Ground, and Communications Service Provider.

| Function | Aircraft Assumptions of Ground System Function | Safety Concern |
| --- | --- | --- |
| {Function Name} | {Behavioral assumption assertion} | {Identification of the safety concern with respect to this function/assumption} |
| … | … | … |
| … | … | … |

Table A-8 Inter-segment Behavioral Assumptions

APPENDIX B

Detailed Airspace Configuration Characteristics

B.1	General

A more comprehensive template may include the following paragraphs for completeness in the specification of airspace configuration characteristics.

B.1.1		Aircraft instruments and equipment requirements

The aircraft instruments and equipment requirements must be in accordance with the provisions included in the ICAO Annex 6 - Operation of Aircraft, Part I - International Commercial Air Transport - Aeroplanes, Chapter 6 (Aeroplane Instruments, Equipment and Flight Documents)  and Chapter 7 (Aeroplane Communication and Navigation Equipment).

In addition to that, the aircraft operating in the described FIR must adhere to any other specific requirement valid for this FIR as established by the appropriate authority or based on a Regional Air Navigation Agreement and is mentioned as such in the ICAO DOC 7030 - Regional Supplementary Procedures

B.1.2		Differences from ICAO Standards, Recommended Practices and Procedures

This paragraph must include all the differences applied in that FIR from the ICAO SARPs relating to the Air Navigation System.

B.1.3.	Measuring system

Units of measurement:	A table of units of measurement referring at least to:

- distances used in navigation;

- altitudes, elevations and heights;

- horizontal speed, including wind speed;

- vertical speed;

- wind direction;

- visibility (including RVR);

- altimeter setting;

- temperature;

- weight;

- time and time reference.

Time System:	The time system information has to refer at least to:

- reporting time procedure;

- summer time period;

- winter time period;

- local time.

Geographic reference datum:	The geographic reference datum must include at least the following  information:

- the reference system utilized for indicating geographical coordinates 		   (latitude and longitude);

- the area of application of the reference system.

B.1.4.	List of radio-navigation aids (only for continental airspace)

The list of radio-navigation aids must include the identification, the station name, the type of equipment, the purpose (aerodrome, approach, en-route, etc.)

B.1.5.	Aeronautical Information Services (optional)

This part has to include the list of the AIS Documents issued for this FIR, means of publication, amendments, cancellations.

B.1.6.	Air Traffic Services

Area of Responsibility:	This part describes the area of responsibility for the provisions of air traffic services of the corresponding ATS Authority.

Types of Services:	This part includes the complete list of Air Traffic Services provided within the FIR, the name of the ATS Units/type of Air Traffic Service, the area of responsibility of each ATS unit, the communication frequencies used by each ATS unit, any other relevant information.

Coordination between operators and ATS:	This part includes the differences (if any) between the coordination procedures mentioned in ICAO Annex 11, para 2.15. and ICAO DOC 4444, PANS-RAC, Part VIII, para. 2.1.1.4. and 2.1.1.5.

B.1.7.	Meteorological Services

Area of Responsibility:	This part describes the area of responsibility for the provision of meteorological services of the corresponding aeronautical meteorological Authority.

Meteorological observations and reports:	This part must include at least the following elements:

- name of the meteorological stations;

- type and frequency of the observations;

- type of MET reports and the supplementary information included;

- hours of information;

- specific climatological information.

Types of Services:	This part includes the complete list of aeronautical meteorological services provided within the FIR and the flight documentation that is made available for different types of flights (e.g. IFR/VFR, international/domestic, etc.).

Aircraft reports:	This part describes the location of the ATS reporting points that are requiring also the transmission of meteorological aircraft reports (ATS/MET reporting points)

Automated meteorological services:	This part describes the Automated Meteorological Services provided within the FIR by giving at least the following information:

- name of station;

- identification;

- frequency;

- broadcast period;

- hours of service;

- FIR/CTA/Aerodrome served

- information content and format;

- other communication means available (phone, fax, telex, etc.) - if the case

B.2.	Airspace Description

B.2.1.	General

Minimum safe height:	This part includes the general specifications concerning the minimum safe height and must include at least the following information:

- minimum safe height (s);

- area(s) of applicability (e.g. mountainous areas, plain areas, densely populated areas, etc.).

Visual Flight Rules (if necessary):	This part includes the conditions of visibility and distance from clouds described per airspace classes.

Instrumental Flight Rules:	This part includes at least the following information:

- a reference to B.1.1.

- a reference to the “Procedures” part in this document;

- general minimum flight levels and the areas of applicability;

- cruising levels.

ATS Airspace Classification:	This part includes at least the following information:

- the classes of ATS airspaces;

- type of flights (IFR/VFR);

- service provided;

- VMC visibility and distance from cloud minima;

- speed limitations;

- radio-communication requirements;

- ATC clearances.

Holding patterns:	This part includes at least the following information:

- Flight Levels;

- speed limitations/categories of aircraft/flight level;

- normal met conditions;

- turbulence conditions.

Radar services (if any)

- Radar ATS units, areas of responsibility, operational frequencies;

- Radar stations, locations and coverage;

- SSR code assignment (including emergency procedures).

B.2.2.	ATS Airspace Description

FIR, UIR, CTA, TMA, CTR:	This part includes at least the following information:

- the type and the name of the airspace concerned (FIR, UIR, CTA, TMA, CTR), lateral and vertical limits, class;

- unit(s) providing service(s);

- callsign, languages, area and conditions of use, hours of service;

- frequency(ies) used and purpose.

Delegation of Air Traffic Services/Delegation of Airspace:	This part includes at least the following information:

- the type and the name of the airspace concerned (FIR, UIR, CTA, TMA, CTR), lateral and vertical limits, class;

- unit(s) providing service(s);

- callsign, languages, area and conditions of use, hours of service;

- frequency(ies) used and purpose.

ATS Routes (the same applies to SIDs/STARs):	This part includes at least the following information:

- route designator;

- RNP value/route segments;

- name and coordinates of significant points/route;

- magnetic track/route segments;

- VOR Radial,

- route segments distances;

- upper/lower limits/route segments;

- minimum flight altitude/route segments;

- airspace classification;

- lateral limits/route segments;

- direction of cruising levels/route segments.

RNAV ATS Routes (the same applies to SIDs/STARs):	This part includes at least the following information:

- route designator;

- RNP value/route segments;

- name and coordinates of significant points/route;

- Way-Point ident of VOR/DME;

- bearing and distance from VOR/DME;

- elevation of the DME Antennas

- great circle distances/route segments;

- upper/lower limits/route segments;

- minimum flight altitude/route segments;

- airspace classification;

- lateral limits/route segments;

- direction of cruising levels/route segments.

En-route Holding:	This part includes at least the following information:

- holding ID/FIX/Way-Point;

- in-bound tracks (magnetic tracks)

- maximum IAS;

- minimum/maximum holding level;

- time or distance out-bound;

- ATS unit responsible and frequency.

Radio-navigation aids/special navigation systems (for continental airspace only):	This part includes at least the following information:

- type and name of the station;

- identification;

- frequencies;

- hours of operation;

- coordinates;

- coverage.

Prohibited, Restricted and Danger Areas:	This part includes at least the following information:

- identification, name and lateral limits;

- upper/lower limit;

- type of activity, restriction, nature of hazard, risk of interception,

Military exercise and training areas:	This part includes at least the following information:

- name and lateral limits;

- system/means of activation/announcement for civilian flights;

- time of activation.

Air navigation obstacles (en-route):	This part includes at least the following information:

- designation of the obstacle;

- type of obstacle;

- coordinates;

- elevation/height from ground;

- obstacle lights.

Note: As a general comment on Prohibited, Restricted and Danger Areas and on Military exercise and training areas, we have to take into account that in Europe and USA we have some special understanding on these issues that is not yet reflected in the ICAO documentation:

- in Europe we have Flexible Use of Airspace that defines the following airspaces:

- CDR (Conditional Routes) categories 1,2,3

- Temporary Segregated Areas (TSAs)

- Cross-Border Areas (CBAs).

TSAs and CBAs can be described like the PAs, RAs or DAs. CDRs can be described like an ATS  route with the addition of the time period when the utilization of this route is possible.

- in USA you have also a different approach to the common use of the National Airspace System that, probably, does not fit with the ICAO description, but we can find the appropriate means of describing it by using the same template.

As of 9 October 1996, as reported at C/SOIT by F. Mackowick.

Most crucial from the safety perspective, for consideration as part of a baseline concept of operations,  is the fact that ADS periodic reports mitigate a number failure effects for aircraft equipped with ADS including; coordination errors  at FIR boundaries, flight data corruption resulting in incorrect flight strips, conflict probe results corruption, loss or corruption of waypoint event reports, loss or corruption of deviation event reports, and corruption of wind forecast data leading to incorrect trajectory estimates.