| RTCA SC-189/EUROCAE WG-53 Position Paper Safety and Interoperability for Air Traffic Services Communication |  |  |  |  |
| --- | --- | --- | --- | --- |
| General Information |  |  |  |  |
| Position Paper Number: Call 206 227 2763 to get #: Leave fax # PSG1102A | Revision: e.g. 3 A | Date: e.g. 11 Nov 94 07 Feb 97 | Comments Due: 06 Jul 97 |  |
| Author/Address/Fax/Voice/Internet: Rob Mead Eurocontrol, PETAL-II Rue de la Fusee 96 B-1130 Brussels Belgium Voice: +32 2 729-3082 FAX: +32 2 729-9087 Internet: mead.rob@eurocontrol.be | Send Comments to (Include Name/Address/Fax/Voice/Internet): Rob Mead Eurocontrol, PETAL-II Rue de la Fusee 96 B-1130 Brussels Belgium Voice: +32 2 729-3082 FAX: +32 2 729-9087 Internet: mead.rob@eurocontrol.be |  |  |  |
| Position Paper Title: Mandatory Data Elements in CMA and AFN |  |  |  |  |
| Abstract: European Flight Data Processing Systems (FDPS) require the following data elements to correlate datalink addresses with ground held flight plans: a) Flight ID (the flight’s callsign, as filed in Field 7 of the ICAO flight plan); b) Departure aerodrome; c) Destination aerodrome; d) Estimated Off Blocks Time. The requirement for this data is apparently not limited to Europe, based on FAA concerns for, and subsequent rejection of, using AFN as a log-on mechanism for their ARINC 623 CPC specification. The specific reasons surrounding ATM’s use of these elements for flight plan correlation are available upon request. For the purposes of this paper, it is sufficient to say that there is no currently agreed mechanism for replacing these elements, and their use for flight plan correlation can be expected to continue for the foreseeable future. Failure to recognize this fact, and to provide the corresponding data at log-on, will delay the operational implementation of air/ground datalink. CNS/ATM-1’s DLIC/CMA allows optional use of fields ‘b’ - ‘d’, above. FANS-1/A’s AFN does not allow use of the elements ‘b’ - ‘d’, and does not clearly mandate use of ICAO Flight IDs. It is proposed that the SC189/WG53 recommend that: 1) data elements ‘b’ through ‘d’, inclusive, be made mandatory for CNS/ATM-1 CMA. 2) data elements ‘b’ through ‘d’, inclusive, be made mandatory for FANS-1/A. 3) the term ‘Flight ID’ be standardized for use in all ATM applications as the ICAO Flight ID, as filed in Field 7 of the ICAO flight plan. |  |  |  |  |
| Action taken: |  |  |  | Resolution Required By: |
| Key words (Optional): |  |  |  |  |

| Status | Individual Proposal |  |  | Sub-Group Agreement | Plenary Agreement |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Internal Use Only |  |  | Closed | SC-189 Co-chair prop. |  |
| Distribution |  |  |  |  |  |  |
| Name |  | Org | Internet |  |  | Fax |
| Dave Allen |  | BCAG | David.L.Allen@Boeing.Com |  |  | +01-425-294-2007 |
| Martine Blaize |  | DGAC | BLAIZE_Martine@STNA.DGAC.Fr |  |  | +33-5-62-14-58-53 |
| Serge Bagieu |  | Aerospatiale | Serge.Bagieu@Avions.Aerospatiale.Fr |  |  | +33-5-61-93-80-90 |
| Tom Kraft |  | FAA | Tom.Kraft@FAA.DOT.Gov |  |  | +01-425-227-1181 |
| Peter Skaves |  | FAA | Peter.Skaves@FAA.DOT.Gov |  |  | +01-425-227-1181 |
| Anne-Sophie Luce SG-1 |  | AIRSYS | LUCE@Thomson.SDC.Fr |  |  | +33-1-40-84-1859 |
| Tony Martin SG-1 |  | AJM Cnslt | AJMartin@Accessone.Com |  |  | +01- 206-868-7297 |
| Emmanuel Chenevier SG-2 |  | DGAC | CHENEVIER_Emmanuel@ccmail.DGAC.Fr |  |  | +33-62-14-53-53 |
| Elizabeth Noon SG-1 |  | FAA | elizabeth.noon@faa.dot.gov |  |  | +01-425-227-1181 |
| Mark Joseph SG-2 |  | MITRE | MJ@MITRE.Org |  |  | +01-703-883-1330 |
| Kent Loving - SG-3 |  | BCAG | Kent.Loving@Boeing.Com |  |  | +01-425-342-6078 |
| Marc Barrere - SG-3 |  | Aerospatiale | Marc.Barrere@avions.aerospatiale.fr |  |  | +33-5-61-93-07-29 |