| RTCA SC-189/EUROCAE WG-53 Position Paper Safety and Interoperability for Air Traffic Services Communication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Position Paper Number: Call 206 227 2763 to get #: Leave fax # PSG1007C |  |  |  |  | Revision: C | Date: 05 Jun 97 |  |  | Comments Due: 07 Jul 97 |  |  |  |  |  |
| Author/Address/Fax/Voice/Internet: Lionel Bertin Aerospatiale B0403 316, route de Bayonne 31060 Toulouse Cedex 03, FRANCE Voice: 33 5 61 93 62 93 FAX: 33 5 61 93 80 06 Internet: lionel.bertin@avions.aerospatiale.fr |  |  |  |  | Send Comments to (Include Name/Address/Fax/Voice/Internet): Author |  |  |  |  |  |  |  |  |  |
| Position Paper Title: FANS A/1 deviations to ADS standards. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Abstract: - This position paper is a proposed draft of part of the section 5 ‘Applications’ of the SG-1 Interoperability document. - Version C of this paper addresses ADS and reflects works of 4th SG-1 meeting. - Final section 5 will address AFN and CPDLC too. - ADS part: it refers to DO-212, with the same terminology (ADSF, ADSP,...) and the same kind of wording. However, a cross reference to ARINC 745-2 is provided. - Note that some ToBeDefined and ToBeConfirmed remain in the Guidance Material part. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Action taken: Aircraft manufacturers: to provide requested information. ATS system manufacturers: to validate all common deviations and to decide whether specific deviations are acceptable or raise interoperability issues. |  |  |  |  |  |  |  |  |  | Resolution Required By: |  |  |  |  |
| Key words (Optional): SG-1, FANS A/1 Interoperability, ADS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Status | Individual Proposal |  |  | Sub-Group Agreement |  |  |  | Plen Agreement |  |  |  |  |  |  |
|  | Internal Use Only |  |  | Closed |  |  |  | Co-chair prop. |  |  |  |  |  |  |
| Distribution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  | Org | Internet |  |  |  | Fax |  |  |  |  |  |  |  |
| Dave Allen |  | BCAG | David.L.Allen@Boeing.Com |  |  |  | +01-425-294-2007 |  |  |  |  |  |  |  |
| Martine Blaize |  | DGAC | BLAIZE_Martine@STNA.DGAC.Fr |  |  |  | +33-5-62-14-58-53 |  |  |  |  |  |  |  |
| Serge Bagieu |  | Aerospatiale | Serge.Bagieu@Avions.Aerospatiale.Fr |  |  |  | +33-5-61-93-80-90 |  |  |  |  |  |  |  |
| Tom Kraft |  | FAA | Tom.Kraft@FAA.DOT.Gov |  |  |  | +01-425-227-1181 |  |  |  |  |  |  |  |
| Peter Skaves |  | FAA | Peter.Skaves@FAA.DOT.Gov |  |  |  | +01-425-227-1181 |  |  |  |  |  |  |  |
| Anne-Sophie Luce SG-1 |  | AIRSYS | LUCE@Thomson.SDC.Fr |  |  |  | +33-1-40-84-1859 |  |  |  |  |  |  |  |
| Tony Martin SG-1 |  | AJM Cnslt | AJMartin@Accessone.Com |  |  |  | +01- 206-868-7297 |  |  |  |  |  |  |  |
| Emmanuel Chenevier SG-2 |  | DGAC | CHENEVIER_Emmanuel@ccmail.DGAC.Fr |  |  |  | +33-62-14-53-53 |  |  |  |  |  |  |  |
| Elizabeth Noon SG-1 |  | FAA | elizabeth.noon@faa.dot.gov |  |  |  | +01-425-227-1181 |  |  |  |  |  |  |  |
| Mark Joseph SG-2 |  | MITRE | MJ@MITRE.Org |  |  |  | +01-703-883-1330 |  |  |  |  |  |  |  |
| Kent Loving - SG-3 |  | BCAG | Kent.Loving@Boeing.Com |  |  |  | +01-425-342-6078 |  |  |  |  |  |  |  |
| Marc Barrere - SG-3 |  | Aerospatiale | Marc.Barrere@avions.aerospatiale.fr |  |  |  | +33-5-61-93-07-29 |  |  |  |  |  |  |  |

Introduction:

From an air/ground point of view, the ADS application is described in DO-212, section 2.2.1. This document focuses on this section although others (introduction, tests, ...) may need to be changed.

Subsections which are not mentioned in the tables hereafter are considered implicitly ‘as specified’.

Content of the ‘Type’ column

deviation: the text in the column ‘Description’ shall replace the text in the part of the DO-212 specified in the 1st column. When ‘Not applicable’, the part of the DO-212 specified in the 1st column shall be deleted.

addition: the text in the column ‘Description’ shall be added to the text in the part of the DO-212 specified in the 1st column.

addition, (item added): the text in the column ‘Description’ shall be added as a new item to the section of the DO-212 specified in the 1st column.

clarification: the text in the column ‘Description’ does not replace existing sections of DO-212.

option: the text in the column ‘Description’ refers to specific aircraft implementations.

Content of the ‘Description’ column

AI: applies to Airbus aircraft.

B: applies to Boeing aircraft (747-400).

ADSF: airborne ADS application.

ADSP: ground (ATC) ADS application.

ADS-1 Review of the DO-212

This section documents common deviations/additions/clarifications to DO-212.

| DO212 section | ARINC 745-2 | Type | Description |
| --- | --- | --- | --- |
| All |  | clarification | Notes do not supersede any requirements. They are for information only. |
| 2.2.1 ADSF Operation, item a | 3.1.1, 2nd para | clarification | Aircraft Intent Groups are optional. Note: optional means that ADSF may or may not implement the Aircraft Intent Groups. When not implemented, ADSF will sent a Non-Compliance Notification message (see section 2.2.1.4.3) when any request for these groups is received from an ADSP. |
| 2.2.1.1 Application Interface |  | deviation | The ADS application shall interface with the interface described in ARINC 622-2, section 4.2: Support for Bit-Oriented Applications - ADS Provisions. Note: Disconnect requests mentioned hereafter are provided by this interface. |
| 2.2.1.1.1.1 Connection Establishment item b | 3.2.2, 7th para | deviation | The ADSF shall allow the establishment of at least four connections with no transport priority. |
| 2.2.1.1.1.2 Connection Denial item a | 3.2.2 | deviation | Not applicable |
| 2.2.1.1.1.2 item b |  | deviation | When the ADSF receives an ADS service request while the maximum number of ADS ground connections are established, the ADSF shall deny service by sending a Disconnect Request reason code 1. Application data uplinked on a connection that has been denied service shall not be processed by the ADSF. |
| 2.2.1.2 Processing of Uplinks item a | 3.1.3, 3rd para | deviation | The ADSF shall be capable of processing an uplink containing: one Periodic Contract request, one Event Contract request, one Demand Contract request, one Cancel Contract request, one Cancel Emergency Mode request and one Cancel All Contracts And Terminate Connection request. Note: the maximum size of the uplink described above is 52 octets. |
| Table 2-1 ADS Input Message Parameters | Attachment 8-2, Figure 6 | deviation | The MSB of the Vertical Rate Change Threshold, the Ceiling Altitude and Floor Altitude fields shall be marked as a sign bit. |
| 2.2.1.2.1.item d | 3.2.2, 14th para | deviation | The ADSF shall support a periodic reporting interval of 64 seconds on any or all connections. If a periodic reporting interval which is less than 64 seconds is requested, the ADSF shall respond with a Non-compliance Notification and shall establish a periodic contract with a 64-seconds reporting period. |
| 2.2.1.2.1.item g | 3.2.2.1, 6th para | addition | If the answer to the contract request is Negative Acknowledgement, the ADSF shall not modify the contract in effect. If the answer to the contract request Non Compliance Notification, the ADSF shall modify the contract in effect, on the basis of the elements of the request that can be complied with. |
| 2.2.1.2.1.item g | 3.2.2.1, 6th para | addition | The contract number shall be modified based upon the new contract number. |
| 2.2.1.2.1. item i (added) | 3.2.2.1, 3rd para | addition | A demand contract request shall not cancel or modify (as described in section g) a periodic contract already established. |
| 2.2.1.2.4 Cancel All Contracts and Terminate Connection Request |  | addition | The ADSF shall send the Disconnect Request reason code 8. |
| 2.2.1.2.5 Cancel emergency Mode Request item a | 3.2.2.4 | addition | the 'ADS Contract Request Number' field shall be filled in with the number of the periodic contract to be established after transition to normal mode. |
| 2.2.1.2.5. item c (added) | 3.2.2.4 | addition | When the ADSF receives a Cancel Emergency Mode Request and an emergency periodic contract exists, then the reporting rate and the on-request groups are retained and applied to the normal periodic contract to be established. |
| 2.2.1.3.1.1 Create Emergency Mode Indication item b | 3.2.5 | deviation | The default emergency contract shall be defined as a periodic contract containing the Basic ADS Group, and Flight Identification and Earth Reference Groups included with every fifth report, beginning with the first emergency report sent. When a normal periodic contract exists before the creation of emergency mode, its reporting rate is retained and applied to the emergency periodic contract. When an emergency mode is created, the first emergency periodic report shall be sent immediately. |
| 2.2.1.3.1.2. item b (added) | 3.2.2.4 | addition | When an emergency periodic contract exists, the transition shall be accomplished by changing the ADS report tag to be non-emergency (tag 7) and retaining the reporting rate and the on-request groups and applying them to the normal periodic contract. |
| 2.2.1.3.1.2 item c (added) | 3.2.5, 6th para | addition | When no emergency periodic contract exists on a connection which is in emergency mode, the transition shall not create a normal periodic contract. |
| 2.2.1.3.1.1 item e (added) | 3.2.5, 6th para | addition | If a new connection is established while the ADSF is in emergency mode, then an event contract request during connection establishment shall result in an event contract only. No default emergency periodic shall be established. |
| 2.2.1.3.2 Context Management Interface |  | deviation | ADSFs that meet the requirements defined by this MOPS shall be version 01. |
| Fig 2-13 Predicted Route Group | Att 8-2 | deviation | The ETA field shall be an Estimated Time to Go (ETG) from aircraft position to the next waypoint. |
| Fig 2-14 Intermediate Projected Intent Group | Att 8-2 | deviation | Validity and sign bits are in reverse order in the Intermediate Intent Group. |
| Table 2-4 ADS Output | Att 4-8 | deviation | The unit of the Altitude field shall be feet. |
| Message Parameters |  | deviation | Valid range for Distance shall be 0 - 8191.750 nm. Default value for Distance shall be 8191.875 nm. |
|  |  | deviation | Valid range for ETA shall be 0 - 16382 seconds. Default value for ETA shall be 16383 seconds. |
|  |  | deviation | Valid range for Projected Time shall be 0 - 16382 seconds. Default value for Projected Time shall be 16383 seconds. |
|  |  | deviation | NOTE (1) The default values shown here does not necessarily reflect a coding of all ‘ones’. |
| 2.2.1.4 item c | 3.4 | addition | A maximum of 10 Intermediate Projected Intent points shall be reported. |
| 2.2.1.4.2 Negative Acknowledgement Message item c (added) |  | addition | When the ADS Contract Number misses in a request, the ADSF shall respond with a Negative Acknowledgement reason 9, which contains 254 as a ‘default’ ADS Contract Number. |
| Table 2-6 |  | deviation | Reason 7 is used for Ôundefined request tagÕ, i.e. a request tag other than 1, 2, 6, 7, 8 and 9. |
| 2.2.1.4.3 item d | 3.2.6, 5th para | deviation | When the ADSF uses the Non-compliance Notification message to indicate that it is not going to support a reporting interval of less than the minimum reporting interval (for the time being , 64 seconds) on one connection, the reporting rate shall be contained in octet four, the first bit of octet five shall be set to zero, and the second bit of octet five shall be set to one. The remaining bits of octet five shall not be used. A reporting rate of 64 seconds shall be applied when a Non-Compliance Notification message containing the reporting rate is sent. Figure 2-18 illustrates an example of the Non-Compliance Notification message. |
| 2.2.1.5 Abnormal Connection Termination item a |  | clarification | A ‘disconnect indication’ refers to a message from any other parts of the avionics (not from the ground). |

ADS 2 - Guidance Material

This section documents:

1) common avionics’ behaviour,

2) differences and unique implementations.

3) issues.

| DO212 section | ARINC 745-2 | Type | Description |
| --- | --- | --- | --- |
| 2.2.1.1.1.1 Connection Establishment item b |  | deviation | Issue: there is no way to distinguish an ADS connection with an ATS ADSP from an ADS connection with any other entity. |
| 2.2.1.2 Processing of Uplinks item a | 3.1.3, 3rd para | deviation | Action item AI, B: to describe the behaviour of their avionics when an uplink exceeding the content described in row ‘2.2.1.2 item a’ of table above is received |
| 2.2.1.2 item c |  | clarification | AI: if the first ADS uplink received from an ADSP contains only one ADS request and that request is invalid, the ADSF will: - establish an ADS connection with that ADSP, - send the appropriate Negative Acknowledgement message, - set the contract inactivity timer. B: if the first ADS uplink received from an ADSP contains only one ADS request and that request is invalid, the ADSF will: - establish an ADS connection with that ADSP, - send the appropriate Negative Acknowledgement message, - consider the ADS connection is closed (no Disconnect Request will be sent to the ADSP). |
| 2.2.1.2 item d (Note) |  | clarification | AI: any downlinking of an Acknowledgement message or Non-Compliance Notification message will reset the counter of consecutive Non-Acknowledgement messages. B: any downlink other than Non-Acknowledgement message will reset the counter of consecutive Non-Acknowledgement messages. |
| Table 2-2 Valid Uplink Tag Values |  | option | Aircraft Intent groups will be provided. |
| 2.2.1.2.5. item c (added) |  | addition | AI: When the ADSF receives a Cancel Emergency Mode Request and an emergency periodic contract does not exist, then a normal periodic contract will be established with the default reporting rate (64 sec) and no on-request group. B: When the ADSF receives a Cancel Emergency Mode Request and an emergency periodic contract does not exist, then no normal periodic contract is established. |
| 2.2.1.2.5.item d (added) |  | addition | AI: The first normal periodic report will be sent immediately. B: The first normal periodic report will be sent with respect to the period. |
| 2.2.1.3.1.1 item d (added) | 3.2.5, 6th para | addition | AI: If a new connection is established while the ADSF is in emergency mode, then a normal demand contract request during connection establishment will result in a Negative Acknowledgement (with reason 4) of the contract request. B: If a new connection is established while the ADSF is in emergency mode, then a normal demand contract request during connection establishment will result in a Negative-Acknowledgement (with reason 4) of the contract request. However, a default emergency contract will be applied to this connection. Therefore, the response to the uplink request will be a Negative Acknowledgement message and an emergency periodic report. |
| 2.2.1.3.1.2 Cancel Emergency Mode Indication | 3.2.5, 5th para | deviation | AI: When the ADSF receives an indication (from the board) to cancel emergency mode, a Cancel Emergency Mode Indication shall be sent immediately and the ADSF will transition each connection in emergency mode to normal mode. B: As specified. |
| 2.2.1.4 Message Generation |  | deviation | last two sentences not applicable and to be replaced by: The ADSF will include within the Basic ADS Group the position information provided by the navigation system actually navigating the system. |
| Fig 2-7 Basic ADS Group | Att 9 | clarification | Note: Because of interface differences on different aircraft and different implementations of the TCAS discrete outputs, the TCAS Health bit does not give a reliable and consistent indication and should not be used for ATS purposes. |
| Fig 2-7 Basic ADS Group | Att 9 | clarification | B: The Navigation System Redundancy bit is set to one when the position from two or more Inertial Reference Unit is being used by the flight management function. |
| Fig 2-11 Airframe Identification Group | Att 8-2 | option | The ADSF will not provide the Airframe Identification Group in ADS reports. |
| Fig 2-13 Predicted Route Group | Att 8-2 | clarification | ATC mandatory waypoints and pilot inserted waypoints will be reported in the Predicted Route Group. |
| Fig 2-14 Intermediate Projected | Att 8-2 | clarification | ATC mandatory waypoints, pilot inserted and computed waypoints will be reported in the Intermediate Projected Intent Group. |
| Intent Group |  | deviation | The Time field will be reported as an Estimated Time to Go (ETG) from the present position and along the flight plan. |
| Fig 2-15 Fixed Intent | Att 8-2 | clarification | The fixed projected point is the predicted aircraft location along the flight plan at the end of the requested time interval. |
| Group |  | clarification | If the predicted aircraft location at the requested time is beyond the end of route, then the last waypoint will be reported as the fixed projected point. |
| Table 2-4 ADS Output Message Parameters | Att 4-8 | addition | AI: When the time stamp is invalid, the field will be filled in with an out-of-range value (4095.875) as a default value. B: When the time stamp is invalid, the field will be filled in with 0 as a default value. Note: for both avionics, when the time stamp is invalid, the FOM will be set to 0 too (as specified). |
| 2.2.1.4 item h | 3.2.3, 3rd para | deviation | AI: The initial response to a valid Periodic Contract Request will be an Acknowledgement or a Non-Compliance notification message. This response and the first periodic report will be sent in separate uplinks. Subsequent periodic reports will be sent at the requested reporting interval, with on-request groups included at their specified interval. B: As specified. |
| 2.2.1.4 item i | 3.2.3, 3rd para | deviation | AI: The initial response to a valid Demand Contract Request will be an Acknowledgement or a Non-Compliance notification message. This response and the demand report will be sent in separate uplinks. B: As specified. |
| 2.2.1.4 item l | 3.2.3, 6th para | deviation | Not applicable. Issue: is it possible to have a common understanding of ‘output from the physical media’ when avionics architectures are different ? Issue: according to DO-212, default values shall be provided in ADS reports when data are invalid. This may happen when Intent groups are requested because they require avionics computations. What is the the balance between ADSF response time requirement (5 sec in DO-212) and avionics computation time ? This is a performance issue too: what means an ADSF response time requirement of 5 sec compared to total transit time of 60 sec ? |
| 2.2.1.4 item m |  | clarification | The altitudes in ADS reports will be uncorrected altitudes. The vertical rates in ADS reports will be inertial. |
| 2.2.1.4 item n | 3.4.5, 5th para | clarification | As specified, when offset operation is managed through FMS. |
| 2.2.1.4 item n | 3.4.5, 5th para | clarification | AI: when offset operation is managed through FMS, the Aircraft Intent Groups will contain the position information relative to the offset path, not the original flight plan. B: when offset operation is managed through FMS, the Aircraft Intent Groups will contain the position information relative to the original flight plan, not the offset route. |
|  | 3.4.5, 5th para | clarification | When offset operation is managed through FMS, a Lateral Deviation event will be triggered if: - an Event contract containing the Lateral Deviation Event is active, and - the offset distance is greater than the Lateral Deviation Threshold. The report will be generated at the time the offset is activated. |
| 2.2.1.4 item n | 3.4.5, 5th para | clarification | AI: when offset operation is not managed through FMS, a waypoint will not be sequenced if the cross-track error to this waypoint is greater than 7.5 nmi (TBC). Then: - a Lateral Event report will be triggered if appropriate, - but a Waypoint Change Event report will not. B: when offset operation is not managed through FMS, a waypoint will not be sequenced if the cross-track error to this waypoint is greater than 21 nmi. Action item AI, B: provide complementary information. |
| 2.2.1.4 item n | 3.4.5, 5th para | clarification | when offset operation is not activated through the Flight Management System, data of Predicted Route and Intent Groups will be based on assumptions that may prove to be inadequate on the long term. However, those groups will still be sent even if they do not reflect the pilot intent. |
| 2.2.1.4 item o (added) |  | addition | B: When the aircraft is about to enter a Holding pattern, Next and Next+1 waypoints will be reported as the entry waypoint. When the aircraft is flying the Holding pattern, the Next waypoint will be reported as the exit waypoint and the Next+1 waypoint will be reported as the subsequent waypoint in the flight plan. ADS data will be based on the assumption that the aircraft will fly the Holding pattern once. Action item AIprovide a description of their ADSF behaviour when the aircraft is in a Hold. |
| 2.2.1.4.3 Non-Compliance Notification message |  |  | In addition to the case where a reporting interval of less than the minimum reporting interval, a Non-compliance Notification message will be sent when an ADS contract request contains: - an on-request group tag or an event tag which is undefined (less than 10 or greater than 21). Then only data contained before the undefined tag in the uplink message will be processed. The remaining data (including following requests in this uplink message) will be ignored. - the Airframe Identification Group tag (17). Then all other data contained in the request will be processed. In both cases, ADS reports based on the data processed will be provided. |
| 2.2.1.5 item c |  | deviation | AI: At flight completion, the ADSF shall cancel all contracts and terminate all connections by sending disconnect requests reason code 2. B: At flight completion, the ADSF shall cancel all contracts and terminate all connections by sending disconnect requests reason code 8. |

Other issues and information:

1- Intent groups (namely Intermediate Projected Intent and Fixed Intent groups) contains ATC-mandatory, pilot-defined and avionics-computed waypoints. Following are drawbacks of the definition of Intent groups:

- an ATC-mandatory waypoint may not be reported if a speed change, a track change or an altitude change does not occur at this waypoint,

- avionics computations to fill in these groups may take longer than the response time required. Therefore default values would be provided instead of the expected data.

2 - ‘Deferred’ clearance

A ‘deferred’ CPDLC clearance (e.g. AT [time] CLIMB TO AND MAINTAIN [altitude]) will not be reflected in ADS Predicted Route and Intent Groups until it is executed.

(valid for AI)

3 - ADS reports ahead of schedule

ATS system designers must be aware that ADS periodic reports may be received earlier than expected. This will occur in certain cases of ADSF shutdowns, where characteristics of contracts established at the time of the shutdown are retained but the ADSF periodic contract timer is reset.

(valid for AI)

4 - ADS Contract Number 254:

ATS sytem designers must be aware that 254 is a contract number used by the avionics under certain conditions (refer to row ‘2.2.1.4.2 item c’ of first table). Therefore this number should not be used in an uplink ADS request.

5 - Cross-track deviation:

Lateral cross-track deviation computed by the aircraft Area Navigation (RNAV) function should always be with respect to the active flight plan leg and the curved transition path between legs. The active leg may be straight, or curved. examples of curved legs include fixed radius legs (RF), holding patterns and procedure turns. For active legs defined by headingonly, lateral cross-track deviation should not be computed.

When an offset path is activated in the RNAV function, the lateral cross-track deviation will immediately change to be with respect to the new offset path.