|  | ATNP/WG3/SG3 (Architecture) Version 3.0 19 April 1996 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Sub Group 3 (Upper Layer Architecture)

Draft SARPs and Guidance Material for ATN Upper Layers for the CNS/ATM-1 Package

Sub-Volume 4 of CNS/ATM-1 SARPs

Prepared by:		ATNP/WG3/SG3

SUMMARY

This document is the draft of the SARPs for ATN Upper Layers in the CNS/ATM-1 Package Data Link Applications.

CONFIGURATION SHEET

Title :		Draft SARPs for the ATN CNS/ATM-1Package

Sub-Volume 4:  ATN Upper Layer SARPs and Guidance Material

Version :	3.0

Date:		19 April 1996

Contact:	Airservices Australia, Eurocontrol, Federal Aviation Administration,

Direction Generale de l’Aviation Civile

Document editor:  kerr@level-7.co.uk

SG3 chairman:  vantrees@sed.stel.com

Status:	Draft

Change History :

| Version | Description | Affected Parts | Date |
| --- | --- | --- | --- |
| 0.2 | Major Restructure |  | August 1995 |
| 0.3 | Dialogue Service Updated |  | August 1995 |
| 1.0 | Banff Proposed Draft |  | October 1995 |
| 1.1 | Final Banff Draft |  | October 1995 |
| 1.2 | Updated at Bracknell editing meeting |  | December 1995 |
| 1.3 | Output of Boston SG3 meeting |  | December 1995 |
| 2.0 | Output of Brisbane WG3 meeting |  | 21 Mar 1996 |
|  | Clarify g-g applicability | §1.1.2 |  |
|  | Replace FIS with FIS(ATIS) | §1.1.3, 2.2, 3.2.3.5 |  |
|  | Add new AE ICC(AIDC) to Fig 1.2 | §1.2.1 |  |
|  | Add naming to scope of chapter 3 | §1.3.3 |  |
|  | Delete description of Annex A | §1.3.7 |  |
|  | Add ICC, ICD, PDV, AIDC, ITU-T | §1.4.1.1 |  |
|  | Add PDV definition, clarify RER, add note that ICAO may not count non-delivered messages | §1.4.2.1 |  |
|  | Tidy Table 1.1 | §1.4.3.2 |  |
|  | Add note to ISO efficiency references. Update PDAM references to latest DAMs. | §1.5.1 |  |
|  | Add Session, Presentation and ACSE PICS proformas to list of references. | §1.5.1 |  |
|  | Delete incorrect note: extensibility markers are not yet standardised in X.227. | §1.6.1.5 |  |
|  | (DR 1) Remove CDSE references | Fig 1.4, Fig 3.1 , §3.1.1 |  |
|  | Numbering of notes | §2.2, 2.2.1, 3.2.2.2, 3.2.5 |  |
|  | Add explanation to “too large on receive” | §2.2.3 |  |
|  | Change QOS support from C(=) to M on D-START Confirmation in Table 2.4 | §2.3.2.2 |  |
|  | (DR 2) Remove 4-character restriction on facility designator | §2.3.2.3, 2.3.2.4 |  |
|  | Add range 1..255 to Version Number in D-START | §2.3.2.5 |  |
|  | Delete statement about QOS parameter in Confirmation primitive being conditional. Add statement that QOS on Response must be lower or equal to QOS on Indication. | §2.3.2.7 |  |
|  | Add note referring to Sub-Volume 5 Table 6-1 & align ATSC routing class values with that table. Add ATSC Class I and J. Clarify that default applies to both ATSC and AOC. | §2.3.2.7.1 |  |
|  | Clarify RER explanatory note. | §2.3.2.7.3 |  |
|  | Add note 5 to clarify scope of CF | §3.1.1 |  |
|  | Merge naming and addressing requirements and notes from Annex A to body of SARPs. Delete Annex A. | §3.2.1, 3.2.2, 3.2.3, 3.2.5, Annex A |  |
|  | Add naming hierarchy diagram as guidance (Fig 3.2), add g-g AE qualifiers, indicate Version 1 is used in package 1. | §3.2.1.1 |  |
|  | Delete AMHS and Type B from AE-Qualifier table, leaving values 5 and 7 as reserved. | §3.2.3.5 |  |
|  | (DR 7) Add SMA to AE-Qualifier table, value 8 | §3.2.3.5 |  |
|  | Clarify encoding of end-system-id | §3.2.4 |  |
|  | Add note that value 0 is reserved for CF use | §3.2.5.3 |  |
|  | Add reference to Chapter 5 for Null Encoding | §3.2.6 |  |
|  | QOS in Table 3.7: Replace “Not used” with ref. to new subsection. Delete bullet j. | §3.3.3.2.2.1 |  |
|  | Add new subsection on QOS mapping to lower layers. | §3.3.3.2.3 |  |
|  | QOS in Table 3.8: Replace “Not used” with ref. to new subsection. Delete bullet g. | §3.3.3.3.2.1 |  |
|  | QOS in Table 3.13: Replace “local means” with ref. to new subsection | §3.3.4.1.2.1 |  |
|  | Add new subsection on QOS mapping from lower layers. | §3.3.4.1.3 |  |
|  | QOS in Tables 3.14 and 3.15: Replace “local means” with ref. to new subsection | §3.3.4.2.2.1 |  |
|  | (DR 3) Replace ASSOCIATE PENDING with ASSOCIATION PENDING | §3.3.5.2.1.1, 3.3.6.2.1.1 |  |
|  | Add clause (b) stating change of state. | §3.3.5.2.2.1 |  |
|  | Add note to say that Ref. column refers to PICS entries. | §4, 5, 6 |  |
|  | Consistent case for ATN Support table entries. | §4, 5, 6 |  |
|  | Reword option definitions from ISO PICS so that ISO Status does not imply ATN requirements. | Tables 4.1, 4.2, 4.4, 4.5, 4.6, 4.8, 5.1, 5.4, 5.5, 6.2, 6.3, 6.6, 6.14, 6.15 |  |
|  | (DR 4) Add explicit requirements to Presentation and Session to implement ITU-T amendments and defect report resolutions. | §4.3.2, 5.1.2 |  |
|  | Replace “Note n” in ATN support column with “- (Note n)”, to show support level “not applicable” | Tables 4.5, 4.7, 4.8, 5.8 |  |
|  | Reword explanatory notes on ATN compliance so that notes do not contain requirements | Tables 4.5, 4.7, 4.8, 5.8 |  |
|  | Replace irequestor with requestor | Tables 4.5, 4.6, 5.6, 5.7 |  |
|  | Correct table heading | Table 4.8 |  |
|  | Add new section on mapping to ATN Internet, to explain mapping of traffic type to security label, non-use of transport expedited, etc. | §4.9 |  |
|  | Notes in italics, separate from requirements paragraphs, sequentially numbered per requirements paragraph. | §5.2.3, 4.3.4, 6.2.1 |  |
|  | Split sentence containing 2 requirements into separate sentences | §5.3.2 |  |
|  | Correct Ref. column entries in ACSE tables | §6 |  |
|  | Make paragraph into note | §6.1 |  |
|  | Remove reference in section heading | §6.2, 6.3 |  |
|  | Change explicit references to [xx] | §6.5.2 |  |
|  | Split paragraph into 2 requirements | §6.5.2.2 |  |
|  | Reword “requires to use” | §6.6.1 |  |
|  | Delete second Refs. column | Table 6.8 |  |
|  | Reword “Note x” AS “cX”, as this is a conditional ATN requirement, not a note | Tables 6.9, 6.10, 6.15 |  |
|  | (DR 5) Make ATN support of Directory Name optional, not prohibited | Table 6.14 |  |
| 3.0 (proposed) | WG3 meeting, Brussels |  | 09 Apr 1996 |
|  | (DR 9) Replace “corresponding SARPs” with “ICC (AIDC) SARPs” | §1.1.3 |  |
|  | (DR 9) Note expanded to give rationale | §1.4.2.1.v) |  |
|  | (DR 11) Reference to ISO/IEC 10731 added. (DR 27, 30) Symbol O added to Table 1.1 | §1.4.3.2 |  |
|  | (DR 21) Refer to ISO documents rather than ITU-T as the base standards for efficiency enhancements | §1.5.1, 1.6.1.2, 1.6.1.3, 4.2.1, 4.3.1, 4.3.2, 4.5.1, 5.1.1, 5.1.2, 5.3, 5.4.2, 5.5.2.1 |  |
|  | (DR 13) Clarification of possible events after D-END Request issued | §2.3.4.2, 2.3.4.3 |  |
|  | (Editorial) Guidance made into new Note 1. Subsequent Notes renumbered. | §3.1.1 |  |
|  | (DR 15, 18) New note 5 added to explain the implicit assumption of ASE atomicity in the model. | §3.1.1 |  |
|  | (DR 22) In Fig 3.2, “ait” replaced with “ati” | §3.2.1.1 |  |
|  | (DR 10) Clarification of naming. “atn-facility-designator” and “atn-aircraft-id” nodes renamed “atn-end-system-ground” and “atn-end-system-air”, respectively. Forward references to §3.2.4 inserted. | §3.2.1.1, 3.2.1.2, 3.2.2.2, 3.2.2.3, 3.2.3.6 |  |
|  | (DR 10) Note 2 added for further guidance. | §3.2.4 |  |
|  | (Editorial) Reference to Table 3.4 made explicit. Bullet style corrected. Shall statements split out into new § 3.3.1.2.2, 3.3.1.2.3. Subsequent paras. renumbered. Subsections renumbered in §3.3.1.2.x (previously §3.3.1.x). | §3.3.1.2.1 |  |
|  | (Editorial) Subsections renumbered to clarify context (previously §3.3.1.x). | §3.3.1.2.7, 3.3.1.2.8, 3.3.1.2.9 |  |
|  | (DR 15, 18) New para. added to explain the implicit assumption of ASE atomicity in the model. | §3.3.1.2.4 |  |
|  | (DR 20) Predicates added to cells D-START rsp/STA1 and D-END rsp/STA3. | Table 3.4 |  |
|  | (DR 16) Correction to cell D-DATA req/STA3, new predicate p2 defined. | Table 3.4, §3.3.1.2.7 |  |
|  | (Editorial) Guidance made into Note 2. | §3.3.2 |  |
|  | (DR 19) Editorial - explicit context added to all requirements statements (“shall” clauses). | §3.3.x.x |  |
|  | (DR 23) Misleading “look up” replaced with “retrieve” | §3.3.3.2.2.1.e) |  |
|  | (Editorial) Double “shall” statements reworded. | §3.3.3.2.2.1.f), 3.3.3.3.2.1.c) |  |
|  | (DR 25) Editorial - meaning of “positive” and “negative” A-ASSOCIATE response made explicit; redundant “shall”s removed. | §3.3.3.3.2.1.e), 3.3.3.3.2.1.f) |  |
|  | (DR 16) D-DATA Request allowed for Release Responder when in RELEASE PENDING state. | §3.3.3.6.1.1 |  |
|  | (Editorial) Make explicit that CF remains in same state after D-DATA Request | §3.3.3.6.2.1.c) |  |
|  | (DR 26) Clarify behaviour if Version Number is zero. | §3.3.4.1.2.1.a), 3.3.4.2.2.1.a) |  |
|  | (Editorial) Bullet style corrected. | §3.3.5.x, 3.3.6.x |  |
|  | (Editorial) Headings renumbered for consistency with chapters 5 & 6 | §4.6.x. 4.7 |  |
|  | (Editorial) A-REL_initiator/responder replaced with A-REL_requestor/acceptor (bug in ISO PICS) | §6.4.2.1 |  |
|  | (DR 14) Release rejection allowed without negotiated release FU. New §6.8 added to make mapping of ACSE to Presentation service via the CF explicit. | §6.4.2.2, 6.8 |  |
|  | (DR 29) If Security Requirements not required, then Authentication is “O” (previously “X”) | §6.6.1 |  |
| 3.0 | Output from WG3 meeting, Brussels |  | 19 Apr 1996 |
|  | Editorial: consistent style “Sub-Volume N”; references changed from [x] to ISO/IEC number. Format of notes, table and figure captions. Font changed to Times New Roman. | All |  |
|  | (DR 23) References to Facility Designator syntax added. | §1.5.1, 3.2.4 |  |
|  | (DR 36) Redundant “shall”s removed, clarification that DS is not required to be realised. | §2.2.1, 2.3.4.1, 3.3.6.5.2.1 |  |
|  | Editorial: context of “shall” statements made explicit. | §2.3.2.3, 2.3.2.4, 2.3.4.1, 2.3.4.4, 2.3.5.1, 3.2.6.3, 3.2.6.4, 5.2.1, 5.2.2, 6.4.2.1, 6.7.2.1.1, 6.7.2.2.1, 6.7.3.2.1 |  |
|  | (DR 37) Different “No Preference” values for ATSC and AOC in Routing Class parameter. ATSC made default. | §2.3.2.7.1 |  |
|  | Editorial: clarification that only external behaviour is mandated. | §3.1.1 |  |
|  | (DR 40) AP Title identifies application category rather than whole end system. Note moved ahead of text. | §3.2.2.1 |  |
|  | (DR 31) AE Qualifier table updated for MHS values AMS(7), GWB(8), AUA(9). SMA value changed from 8 to 5. Figure 3.2 aligned. | §3.2.3.5, Fig 3.2 |  |
|  | (DR 32) AE Qualifier table: ICC renamed IDC | §3.2.3.5 |  |
|  | (DR 43) Clarification of error conditions in State Table | §3.3.1.2.5-7 |  |
|  | (DR 34) “ISO Status” column added | Tables 3.7, 3.8, 3.9, 3.10, 3.11, 3.12, 3.16 |  |
|  | (DR 38) Cross-reference corrected | §3.3.3.6.2.1 |  |
|  | (DR 39) CF is initiator, not responder | §3.3.4.2.2.1.e |  |
|  | (DR 33) Reference to “M” values in Table 4.5 removed, notes extended to explain why Session Orderly Release role is not required. PRL conformance in §4, 5, 6 aligned. | §4.1.1, 4.3.1, , 4.4.1.1, 4.4.2.1, 4.4.3.1, 4.5.1.1, 5.1.1, 6.5.1, 6.7.1 |  |
|  | Editorial: security parameter diagram reformatted | §4.6.6 |  |

Table of Contents

1. INTRODUCTION

1.1 Scope and Objectives

1.2 Background

1.3 Structure of Sub-Volume

1.4 Explanation of Terms

1.5 References

1.6 Upper Layer Functionality

2. DIALOGUE SERVICE DESCRIPTION

2.1 Introduction to the Dialogue Service

2.2 DS Functionality

2.3 Service Description

3. APPLICATION ENTITY (AE) DESCRIPTION

3.1 Introduction

3.2 Application Level Naming and Context Definition

3.3 Control Function Specification

4. SESSION LAYER REQUIREMENTS

4.1 Protocol versions implemented

4.2 Session Functional units

4.3 Protocol mechanisms

4.4 Supported Roles

4.5 Supported SPDUs

4.6 Mapping to the ATN Internet Transport Service

5. PRESENTATION LAYER REQUIREMENTS

5.1 Protocol mechanisms

5.2 Use of null encoding protocol option

5.3 Mapping of Presentation Primitives to the Null Encoding option

5.4 Functional units

5.5 Elements of procedure

5.6 Structure and encoding of PPDUs

5.7 Pass through to Session functional units

6. ACSE SPECIFICATION

6.1 Classification of requirements

6.2 Protocol details

6.3 Protocol versions

6.4 Supported roles

6.5 Protocol mechanisms

6.6 ACSE Functional units

6.7 Supported APDUs

6.8 Mapping to the Presentation Service