ATNP/WG3/WP14-__

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

Working Group 3 (Applications and Upper Layers)

Albuquerque (U.S.A.), 8th – 11th December 1998

Proposed CPDLC Provisions

To Support Package-2 Security Services

Presented by: F. Picard

SYSMANDCIL7_005SYSMANDCIL7_005SUMMARY

This WP provides a summary of the security mechanisms of the ATN Upper Layer Architecture proposed by WG3/SG3 to support the security. All security functions are actually provided by a new component of the Dialogue Service Provider, the Security Application Service Object (Security ASO) except the negotiation of the level of security needed on a dialogue which is left to the dialogue service users.

This WP proposes a way to modify the current specifications of the ASEs likely to have security requirements (i.e. CM, CPDLC and ADS). CPDLC is taken as an example.

The modification of CM to support the exchange of application-related security information is outside the scope of this WP.

Reference:

[1]	Secured ATN Dialogue Service. WG3/SG3 – G. Mittaux-Biron.

# Introduction

# Background

## The ATN Security Framework

### Overview

The operational requirements for ADS and CPDLC identify the need for security measures to be taken with respect to information flowing between end systems. Risk analysis and studies have shown these ATN applications are vulnerable to several threats, namely message modification, message replay, masquerade and denial of service attacks.

The ATN security architecture provides two main security services providing efficient countermeasures to these threats: peer entities strong authentication and data integrity checking.

The ATN security architecture is a public-key cryptosystem based on the use of a pair of keys (one private and one public) by each of the communicating AEs. Both keys can be used for encipherment, with the private key to be used to decipher if the public key was used, and the public key being used to decipher if the private key was used. By signing the message sent to the peer AE with its private key (known only by itself), the sender AE is identified by any remote AE owning the sender AE's public key. The signature – computed from the contents of the data sent and containing unique identifier and a timestamp – provides the guarantee that the message has not been replayed or modified during the transfer.

Public keys are made available as certificates. A certificate authenticated a trusted party establish the link between a Public Key and a particular user. The efficiency of the security system relies on the control of the way the keys and certificates are created, authenticated, distributed and updated. The key/certificate management is performed by the Public Key Infrastructure (PKI). The CM Application could be integrated in the PKI for the exchange of keys with the aircraft.

The scenario assumed for the exchange of keys and the establishment of secured dialogues between the air and the ground ASEs is the following:

- The aircraft is supposed to have the public key of the initial ground CM. The air CM sends a CM-logon request signed with the aircraft private key.

- The ground retrieves through the ATN directory service the public key of the aircraft and authenticates the aircraft as the originator of the message.

- The ground-CM sends a CM-logon response to the aircraft signed with the CM private key and containing the public keys of the ground ATN applications (ADS, CPDLC and FIS).

- Using the ground CM's public key, the air CM authenticates the received CM-logon response and stores the public keys of the ground ATN applications.

- For air-initiated applications, e.g. CPDLC, the air CPDLC establishes a dialogue with a signature created with the aircraft private key. The ground already knowing the aircraft's public key (see 2.) can authenticate the sender. In return, the ground CPDLC sends a signature created with the ground CPDLC's private key. The aircraft uses the CPDLC's public key received in the Logon response to authenticate the ground CPDLC.

- For ground-initiated applications, e.g. ADS, the ground ADS establishes a dialogue with a signature created with the ground ADS private key. The aircraft uses the ADS's public key received in the Logon response to authenticate the ground ADS. In return, the air ADS sends a signature created with the aircraft's private key. The ground already knowing the aircraft's public key (see 2.) can authenticate the sender.

The document deals with item 5 and 6, i.e. the way the ATN Application ASE establish a secured dialogue. The way the security information is passed by CM is outside the scope of this document.

### Establishment of a secured dialogue

#### Model

The functional model of the ATN applications within the Secured ATN Upper Layer Architecture is showed in Figure 1.

#### Application Entity Security Functions

Security services are provided transparently by the Secure Dialogue Service Provider (SDSP) to the ATN Application Service Elements. The only requirements put on the ASE when using the secured dialogue service are the following:

- Supply the desired level of security on a dialogue basis:

- No security,

- Peer Entities Authentication during dialogue establishment and

- No data integrity check, or

- Integrity check of data sent by the dialogue initiator only,

- Integrity check of data sent by the dialogue responder only,

- Integrity check of all data sent on the dialogue.

- Negotiate the level of security. An ATN Application Service Entity on behalf of its user must be able to negotiate with the remote entity the level of security services which will be operated on the dialogue being established. In other words, the dialogue responder can downgrade the level of security proposed by the dialogue initiator.

#### Security ASO

All the other functions are under the responsibility of the Security ASO.

During the dialogue lifetime, the Security ASO:

- takes part to the negotiation of the level of security on the current dialogue. In some circumstances, the Security ASOs involved in the communication can not provide the requested level of security,

- Generates the signature of the messages based on the knowledge of the private key and hash and signature algorithms and checks the signatures of the received messages based on the knowledge of the public key and hash and signature algorithms. This function is local to the Security ASO.

As side activities, the Security ASO is responsible for the following:

- Manage and keep secret the private key,

- For each peer user to authenticate, retrieves the needed certificates and authenticates the peer user's public key.

These activities requires specific security services and protocols between the ATN end systems and the security systems as the Certificate Authority (CA).

## The Secure ATN Upper Layers

### The Secure Dialogue Service

#### Service

The secured dialogue D-START service is identical to the Package-1 dialogue D-START service with the exception of the Security Requirements parameter which should be defined as follows:

|  | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| Security Requirements | U | C(<=) | C(<=) | C(<=) |

This modification of the D-START service does not impact the current specification of the ATN ASEs.

The Secured Dialogue Service Users are expected to use this parameter to negotiate the level of security on the dialogue being established.

#### Secured Dialogue Protocol

During the establishment of a secured dialogue:

- The initiating Security ASO checks that it can comply with the requested level of security. The resulting level of security is passed to the peer in the A-ASSOCIATE request authenticationMechanismName parameter.

- The initiating Security ASO creates the following security PDUs (they are passed in the A-ASSOCIATE request authenticationValue parameter):

- An Authentication Security PDU containing the following fields (the recipient being the dialogue responder):

- The security token in clear (algorithm identifier, name of the intended recipient, timestamp, token identifier),

- The certification path needed to validate the public key of the initiator,

- The security token signature (created with the algorithm identified in the token and the private key of the initiator),

- If initiator's data integrity check is requested, a Security Signature PDU containing the data signature (using the default algorithm or the negotiated one).

- If the initiator wants to negotiate the algorithms for future dialogues, a Security Negotiation PDU identifying the algorithms used for signing the security token and the algorithms used for signing the data,

- The receiving Security ASO checks that it can comply with the requested level of security. The resulting level of security is passed to the local user which can downgrade it and is send back in the A-ASSOCIATE response authenticationMechanismName parameter.

- The receiving Security ASO creates the following security PDUs (they are passed in the A-ASSOCIATE response authenticationValue parameter):

- An Authentication Security PDU containing the same fields as above (the recipient being the dialogue initiator),

- If responder's data integrity check is requested, a Security Signature PDU containing the data signature (using the default algorithm or the negotiated one).

- If the responder wants to negotiate the algorithms for future dialogues, a Security Negotiation PDU identifying the algorithms used for signing the security token and the algorithms used for signing the data,

During the data transfer phase of a secured dialogue:

- The sender Security ASO creates the following security PDUs (they are passed in the P-DATA request user data parameter):

- If data integrity check is requested, a Security Signature PDU containing

- the data in clear, and

- the data signature (using the default algorithm or the negotiated one).

For the time being, there is no description in [1] of the actions taken by the Security ASO when a security attack (data modification, invalid signature) is detected.

# Proposed Changes in CPDLC to provide Secured CPDLC links

The proposal is to modify the CPDLC ASEs in such a way the processing expected from the Secured Dialogue Service User be performed by the CPDLC-users. This means that the CPDLC ASEs act as a pass-through as far Security parameters are concerned. The level of security is therefore requested and negotiated by the CPDLC-users.

Annex A provides the changes required to be made to the current CPDLC specifications to support the security pass-through function. All the added functions are optional, in order to guarantee the full interoperability with the Package-1 version.

In summary, the changes are the following:

- In chapter 3: adding a Security Requirement parameter to the CPDLC-start and DSC-start services,

- In chapter 5: Whenever the Security Requirement parameter is supplied by the DSP in the D-START primitives, pass it unchanged to the CPDLC-user. Whenever the Security Requirement parameter is supplied by the CPDLC-user in the CPDLC-start and DSC-start primitives, pass it unchanged to the DSP.

# Conclusion

WG3/SG2 is invited to review this material for inclusion in Package-2 SARPs specification.

This facility will be used by CM is the default algorithms used for CM are not suitable for other applications. For instance, CM could use a powerful algorithm (producing traffic and time overhead) and the other applications could use a lighter algorithm.