SARPs Version Number	ADS V3.0

Defect Report Number	ADS-0056

Originator	ATNP/WG3/SG2

Originator’s Reference (where applicable)

Date	28/8/96

Location in SARPs	2.2.1.5.4, 2.2.1.5.3.2, 2.2.1.5.3.3

2.2.2.5.4, 2.2.2.5.3.2, 2.2.2.5.3.3

Status (OPEN, CLOSED or REJECTED)	CLOSED

Problem

The definition of the invalid PDU and sequence error is not consistent with those definitions in the other air-ground application SARPs.

Editor’s Proposed Solution (optional)

Agreed Solution

In 2.2.1.5.3.2 & 2.2.1.5.3.3, delete:

### In 2.2.1.5.3, if no actions are described for an ADS service primitive in a particular state, then the invocation of that service primitive shall be prohibited in that state.

### Upon receipt of a PDU, if no actions are described for the arrival of that PDU when in a particular state, then exception handling procedures as described in 2.2.1.5.5.4 shall apply.

and replace with:

### If an APDU is not received when one is required, or one is received in an inappropriate dialogue service primitive, then exception handling procedures as described in 2.2.1.5.4.3 shall apply.

### Upon receipt of an APDU, if no actions are described for the arrival of that APDU when in a particular state, then exception handling procedures as described in 2.2.1.5.4.4 shall apply.

### Upon receipt of an APDU but that can not be decoded, then exception handling procedures as described in 2.2.1.5.4.5 shall apply.

Add 2.2.1.5.4.5

When the air LI module or the ground LI  module fails to decode an APDU, the LI module shall request the AB module to abort with reason decoding-error.

In 2.2.1.5.4.3, for all occurances of the phrase

does not contain

delete

... does not contain ...

and replace with:

... is a valid APDU and is not ...

In the ASN.1 in 2.2.1.4, in the definition of AbortReason add:

decoding-error (9),

Make similar changes in 2.2.2

27/8/96