SARPs Version Number	ADS V3.0

Defect Report Number	ADS-0048

Originator	Thompson

Originator’s Reference (where applicable)	12

Date	23/8/96

Location in SARPs	2.2.1.5.3.15

Status (OPEN, CLOSED or REJECTED)	CLOSED

Problem

The t-LI-1 timer is neither set nor tested.

Editor’s Proposed Solution (optional)

Agreed Solution

In 2.2.1.5.3.15.3 delete

##### If in the LI-G-ACTIVE state, the ground LI module shall:

- Invoke D-END req with ADS-cancel-all-contracts-PDU in the user data and

- enter the LI-G-END state.

and replace with

##### If in the LI-G-ACTIVE state, the ground LI module shall:

- Invoke D-END req with ADS-cancel-all-contracts-PDU in the user data,

- start the t-LI-1 timer, and

- enter the LI-G-END state.

In 2.2.1.5.3.15.4.1 delete

- invoke D-END req with no user data, and

- enter the LI-G-END state; or

and replace with:

- invoke D-END req with no user data,

- start the t-LI-1 timer, and

- enter the LI-G-END state; or

In 2.2.1.5.3.15.5.1 delete

- Invoke D-ABORT request with the parameter values supplied, and

- enter the LI-G-IDLE state.

and replace with

- Invoke D-ABORT request with the parameter values supplied,

- stop any timers, and

- enter the LI-G-IDLE state.

In 2.2.1.5.3.15.8.1 delete

- if the user data parameter value contains an ADS-positive-acknowledgment-PDU (cancel-all-contracts), then request the ground HI module to invoke ADS-cancel-all-contracts confirmation,

- request the DC, EC, PC and EM modules to stop operation, and

- enter the LI-G-IDLE state.

and replace with

- stop the t-LI-1 timer,

- if the user data parameter value contains an ADS-positive-acknowledgment-PDU (cancel-all-contracts), then request the ground HI module to invoke ADS-cancel-all-contracts confirmation,

- request the DC, EC, PC and EM modules to stop operation, and

- enter the LI-G-IDLE state.

Add 2.2.1.5.3.15.11:

Upon expiry of the t-LI-1 timer, the ground LI module shall:

- request the air AB module to abort with reason timer-expiry, and

- remain in the same state.

27/8/96