SARPs Version Number	ADS V 3.0

Defect Report Number	ADS-0057

Originator	Eurocontrol TES

Originator’s Reference (where applicable)	tes-ads-054

Date	17 Sept 1996

Location in SARPs	2.2.1.3.4

2.2.1.3.5

2.2.1.3.6

2.2.1.7.1.5.3

Status (OPEN, CLOSED or REJECTED)	CLOSED

Problem

If the ADS-air-user is already working at maximum capacity (e.g. 4 connections already) 2.2.1.7.1.5.3 requires the ADS-air-user to send to the ground an indication of the existing ICAO facility designators. However, in the ADS-demand-contract, ADS-event-contract and ADS-periodic-contract primitives, there is no indication of which ICAO facility designator is using the service. Therefore the ADS-air-user has no knowledge of which facility is using it.

Editor’s Proposed Solution (optional)

In tables 2.2.1.3-1 to 2.2.1.3.-6 insert one of the following two lines (as appropriate):

| ICAO facility designator | C | C(=) |  |  |
| --- | --- | --- | --- | --- |

| ICAO facility designator | C | C(=) |
| --- | --- | --- |

In sections 2.2.1.3.4 to 2.2.1.3.6 add the following paragraphs:

2.2.1.3.X.X	ICAO facility designator

Note.— This parameter contains the 8 character ICAO facility designator of the ICAO facility which is initiating the contract.

2.2.1.3.X.X.1	The ICAO facility designator parameter value shall conform to an abstract value corresponding to an 8 character ICAO facility designator.

2.2.1.3.X.X.2	The ICAO facility designator parameter value shall be provided when the ADS-ground-user has no other contracts in place with the aircraft.

In section 2.2.1.5.3.6.2.1, 2.2.1.5.3.8.2.1 and 2.2.1.5.3.10.2.1, point b) after "aircraft identifier parameter value", add

, ICAO facility designator parameter

In section 2.2.1.5.3.15.2.1, in Table 2.2.1.5-61, delete the line:

| Calling peer Id | Not used |
| --- | --- |

and replace with:

| Calling peer Id | ICAO facility designator parameter value from contract request |
| --- | --- |

In section 2.2.1.5.3.16.5 a), after "user data" add

and the calling peer id parameter value

In sections 2.2.1.5.3.7.5.1 in Table 2.2.1.5-15, 2.2.1.5.3.9.7.2 in Table 2.2.1.5-30, and in 2.2.1.5.3.11.6.2 in Table 2.2.1.5-46 add the following line:

| ICAO facility designator | Calling peer id, if provided by the air LI module |
| --- | --- |

Agreed Solution

Proposed solution agreed. Also add to 2.2.1.3.X.X.2

Note: If contracts are currently in place, the facility designator parameter is not used by the ADS-service-provider