SARPs Version Number 				ADS ICAO V1.0

Defect Report Number 				ADS-0081

Originator					Jane Hamelink / Dung Nguyen

Originator's Reference (where applicable)

Date						11 February 1997

Location in SARPs				2.2.1.7.3

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				11 February 1997

Fixed in 					ICAO V1.1

Problem

There is no description of the heading change event.

Agreed Solution

Add the following requirement:

2.2.1.7.3.5.13	When heading-change is provided in the ADS-event-contract contract details parameter, and not indicated in the noncompliance notification if sent, then for the duration of the event contract, whenever the aircraft's heading differs negatively or positively from the value transmitted in the previous ADS report containing an air-vector element by an amount exceeding the value of the heading-change element specified in the event contract request, then the ADS-air-user shall invoke ADS-report request including the air-vector element in the report details parameter.