SARPs Version Number 				ADS ICAO V1.0

Defect Report Number 				ADS-0080

Originator					Jane Hamelink / Dung Nguyen

Originator's Reference (where applicable)

Date						11 February 1997

Location in SARPs				2.2.1.7

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				11 February 1997

Fixed in 					ICAO V1.1

Problem

There is a requirement that the aircraft has only on periodic contract (& event, & emergency). This is not possible to manage easily if two connections are set up between the same aircraft and ground system. Therefore, the accepted method is to allow only one connection between aircraft and ground. The resolution to ADS-0057 has required the first (and only the first) contract (i.e. the one that creates the connection) to include the ICAO facility designator of the sender. The words chosen in the solution to this defect have fortuitously prevented the ground system setting up two connections.

The problem remains that the aircraft does not check that a new connection is not from an existing ground system.

Agreed Solution

Add an additional requirement for the ADS-air-user.

2.2.1.7.1.5.4	If the ICAO facility designation parameter is provided in an ADS-demand-contract indication, an ADS-event-contract indication or an ADS-periodic-contract indication, and if this ICAO facility designation is equal to the ICAO facility designations of any other ground system with which the aircraft has one or more contracts, the ADS-air-user shall invoke ADS-user-abort in place of the normal response.

Note.- The intention is that the new connection will be aborted; the existing connection and all the contracts on it will be retained.