SARPs Version Number 				ADS 3.0 /  28-06-1996

Defect Report Number 				ADS-0067

Originator					TES (EUROCONTROL)

Originator's Reference (where applicable)		tes-ads-071

Date						9 December 1996

Location in SARPs				2.2.1.5.4.3.5

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				18/12/96

Fixed in 					ICAO V1.1

Problem

Requirement 2.2.1.5.4.3.5 is wrong since the ADS-air-ASE shall be able to receive a D-END indication primitive containing as UserData parameter an ADS-cancel-all-PDU.

It is proposed to rephrase the requirement as follows:

When the user data parameter value of a D-END indication is present but does not contain an ADS-cancel-all-contracts-PDU, the air LI module shall request the AB module to abort with reason invalid-PDU.

Note. The reference to the ground ASE is not required any more since the D-END service is only initiated by the ground-ASE (therefore the D-END indication cannot be received by the ground ASE).

Agreed Solution

Proposed solution accepted.