SARPs Version Number 				ADS 3.0 /  28-06-1996

Defect Report Number 				ADS-0068

Originator					TES (EUROCONTROL)

Originator's Reference (where applicable)		tes-ads-072

Date						9 December 1996

Location in SARPs				2.2.1.5.4.3

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				18/12/96

Fixed in 					ICAO V1.1

Problem

A new requirement describing the situation of an Invalid PDU should be added to cover the reception of a D-END confirmation containing an invalid PDU.

The proposed requirement is as follows:

When the user data parameter value of a D-END confirmation is present but does not contain an ADS-positive-acknowledgement-PDU(cancel-all-contracts), the ground LI module shall request the AB module to abort with reason invalid-PDU.

Editor's Proposed Solution (optional)

Proposed solution agreed.